AT Commands Interface Guide

4118395

7.0

August 10, 2017

# AirPrime HL76xx

Important Notice

Due to the nature of wireless communications, transmission and reception of data can never be guaranteed. Data may be delayed, corrupted (i.e., have errors) or be totally lost. Although significant delays or losses of data are rare when wireless devices such as the Sierra Wireless modem are used in a normal manner with a well-constructed network, the Sierra Wireless modem should not be used in situations where failure to transmit or receive data could result in damage of any kind to the user or any other party, including but not limited to personal injury, death, or loss of property. Sierra Wireless accepts no responsibility for damages of any kind resulting from delays or errors in data transmitted or received using the Sierra Wireless modem, or for failure of the Sierra Wireless modem to transmit or receive such data.

Safety and Hazards

Do not operate the Sierra Wireless modem in areas where cellular modems are not advised without proper device certifications. These areas include environments where cellular radio can interfere such as explosive atmospheres, medical equipment, or any other equipment which may be susceptible to any form of radio interference. The Sierra Wireless modem can transmit signals that could interfere with this equipment. Do not operate the Sierra Wireless modem in any aircraft, whether the aircraft is on the ground or in flight. In aircraft, the Sierra Wireless modem MUST BE POWERED OFF. When operating, the Sierra Wireless modem can transmit signals that could interfere with various onboard systems.

Some airlines may permit the use of cellular phones while the aircraft is on the ground and the door is open. Sierra Wireless modems may be used at this time.

The driver or operator of any vehicle should not operate the Sierra Wireless modem while in control of a vehicle. Doing so will detract from the driver or operator’s control and operation of that vehicle. In some states and provinces, operating such communications devices while in control of a vehicle is an offence.

Limitations of Liability

This manual is provided “as is”.  Sierra Wireless makes no warranties of any kind, either expressed or implied, including any implied warranties of merchantability, fitness for a particular purpose, or noninfringement.  The recipient of the manual shall endorse all risks arising from its use.

The information in this manual is subject to change without notice and does not represent a commitment on the part of Sierra Wireless. SIERRA WIRELESS AND ITS AFFILIATES SPECIFICALLY DISCLAIM LIABILITY FOR ANY AND ALL DIRECT, INDIRECT, SPECIAL, GENERAL, INCIDENTAL, CONSEQUENTIAL, PUNITIVE OR EXEMPLARY DAMAGES INCLUDING, BUT NOT LIMITED TO, LOSS OF PROFITS OR REVENUE OR ANTICIPATED PROFITS OR REVENUE ARISING OUT OF THE USE OR INABILITY TO USE ANY SIERRA WIRELESS PRODUCT, EVEN IF SIERRA WIRELESS AND/OR ITS AFFILIATES HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES OR THEY ARE FORESEEABLE OR FOR CLAIMS BY ANY THIRD PARTY.

Notwithstanding the foregoing, in no event shall Sierra Wireless and/or its affiliates aggregate liability arising under or in connection with the Sierra Wireless product, regardless of the number of events, occurrences, or claims giving rise to liability, be in excess of the price paid by the purchaser for the Sierra Wireless product.

Patents

This product may contain technology developed by or for Sierra Wireless Inc.

This product includes technology licensed from QUALCOMM®.

This product is manufactured or sold by Sierra Wireless Inc. or its affiliates under one or more patents licensed from MMP Portfolio Licensing.

Copyright

© 2017 Sierra Wireless. All rights reserved.

Trademarks

Sierra Wireless®, AirPrime®, AirLink®, AirVantage®, WISMO®, ALEOS® and the Sierra Wireless and Open AT logos are registered trademarks of Sierra Wireless, Inc. or one of its subsidiaries.

Watcher® is a registered trademark of NETGEAR, Inc., used under license.

Windows® and Windows Vista® are registered trademarks of Microsoft Corporation.

Macintosh® and Mac OS X® are registered trademarks of Apple Inc., registered in the U.S. and other countries.

QUALCOMM® is a registered trademark of QUALCOMM Incorporated.  Used under license.

Other trademarks are the property of their respective owners.

Contact Information

| Sales information and technical support, including warranty and returns | Web: sierrawireless.com/company/contact-us/ Global toll-free number: 1-877-687-7795 6:00 am to 5:00 pm PST |
| --- | --- |
| Corporate and product information | Web: sierrawireless.com |

Document History

| Version | Date | Updates |
| --- | --- | --- |
| 1.0 | February 03, 2016 | Creation |
| 1.1 | February 15, 2016 | Updated 5.36 +KSLEEP Command: Power Management Control for UART |
| 2.0 | April 20, 2016 | Added: - 2.11 &C Command: Set Data Carrier Detect (DCD) Function Mode - 2.12 &D Command: Set Data Terminal Ready (DTR) Function Mode - 2.14 &W Command: Save Stored Profile - 2.15 &V Command: Display Current Configuration - 2.16 &K Command: Flow Control Option - 2.17 &S Command: DSR Option - 3.15 +CMUX Command: Multiplexing Mode - 5.16 +KCELL Command: Cell Environment Information - 5.21 +KSIMDET Command: SIM Detection - 5.31 +KTEMPMON Command: Temperature Monitor - 5.36 +KSLEEP Command: Power Management Control for UART - 5.45 +KRIC Command: Ring Indicator Control - 5.47 +KUSBCOMP Command: Set USB Composition - 5.48 +WMUSBVCC Command: USB VCC Detection Setting - 9 Audio Commands - 12 Protocol Specific Commands - 13 AVMS Commands - 14 Test Commands - 18.2.5 FTP Reply Codes - 18.2.6 AVMS Error Codes - 18.5 TCP Commands Examples - 18.6 UDP Commands Examples - 18.7 FTP Commands Examples - 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table |
|  | May 16, 2016 | Added: - 5.49 +KLTEMUTE Command: Mute LTE TX - 5.50 +KSYNC Command: Application Synchronization Signal - 12.13 HTTP Client Specific Commands - 12.14 HTTPS Client Specific Commands - 12.15 SSL Certificate Manager - 18.2.7 Error Case Examples - 18.8 HTTP Commands Examples |
|  | June 02, 2016 | Added 5.51 +KLTEPARAM Command: LTE Parameters |
|  | June 29, 2016 | Updated: - 14.1 +WMTXPOWER Command: Test RF Tx - 14.2 +WMRXPOWER Command: Test RF Rx |
| 2.1 | July 04, 2016 | Added: - 5.52 +KBND Command: Current Networks Band Indicator - 5.53 +KSRAT Command: Set Radio Access Technology - 5.54 *PSRDBS Command: Change Frequency Band |
| 3.0 | August 11, 2016 | Added: - 5.55 +CMEC Command: Mobile Equipment Control Mode - 5.56 +CPOF Command: Power Off - 5.57 +KGSMAD Command: GSM/LTE Antenna Detection - 5.58 +KSREP Command: Mobile Start-up Reporting - 5.59 +WMANTSEL Command: Select Main / Diversity Antenna for LTE - 12.9 SSL Configuration - 17 M2M Service Optimization Commands |
|  |  | Updated: - 5.4 +CIND Command: Indicator Control - 5.7 +CMER Command: Mobile Equipment Event Reporting - 5.51 +KLTEPARAM Command: LTE Parameters - 12.10.1 +KTCPCFG Command: TCP Connection Configuration 12.13.1 +KHTTPCFG Command: HTTP Connection Configuration |
|  | August 15, 2016 | Updated 6.8 +CPOL Command: Preferred PLMN List |
|  | September 05, 2016 | Removed HL7618 support from: - 12.14 HTTPS Client Specific Commands - 12.15 SSL Certificate Manager |
|  | September 09, 2016 | Added: - 5.60 +KSIMSEL Command: SIM Selection - 5.61 +BOOTDWLCFG Command: Boot Configuration for Firmware Download |
|  | September 21, 2016 | Added: - 5.62 +CALA Command: Set Alarm - 5.63 +CALD Command: Delete Alarm - 5.64 +KCCINFO Command: Camped Cell Information |
|  |  | Updated: - 5.51 +KLTEPARAM Command: LTE Parameters - 10.19 +CGREG Command: GPRS Network Registration Status |
|  | October 06, 2016 | Updated: - 5.4 +CIND Command: Indicator Control - 5.7 +CMER Command: Mobile Equipment Event Reporting - 5.52 +KBND Command: Current Networks Band Indicator - 5.53 +KSRAT Command: Set Radio Access Technology - 5.54 *PSRDBS Command: Change Frequency Band |
| 4.0 | November 11, 2016 | Added: - HL7648, HL7650 and HL7688 support - 9.3 +CLVL Command: Loudspeaker Volume Level - 9.4 +KECHO Command: Echo Cancellation - 9.5 +KNOISE Command: Echo Suppression - 9.6 +KPC Command: Peak Compressor - 9.7 +KST Command: Side Tone - 9.8 +KVGR Command: Receive Gain Selection - 9.9 +KVGT Command: Transmit Gain Selection - 9.10 +VGR Command: Receive Gain Selection - 9.11 +VGT Command: Transmit Gain Selection - 9.12 +VIP Command: Initialize Voice Parameters - 9.13 +CODECINFO Command: Display Audio Codec Information |
| 4.0 | November 11, 2016 | Added: - 9.14 +KSRAP Command: Save or Restore Audio Parameters - 9.15 +WVR Command: Voice Codec Selection - 9.16 +VTD Command: Tone Duration - 9.17 +VTS Command: DTMF and Tone Generation |
|  |  | Updated: - 5.16 +KCELL Command: Cell Environment Information - 5.52 +KBND Command: Current Networks Band Indicator - 5.53 +KSRAT Command: Set Radio Access Technology - 5.54 *PSRDBS Command: Change Frequency Band - 14.1 +WMTXPOWER Command: Test RF Tx - 14.2 +WMRXPOWER Command: Test RF Rx |
| 5.0 | February 13, 2017 | Added: - HL7618RD support - 3.28 B Command: Data Rate Selection - 3.29 S2 Command: Set Character for the Escape Sequence (Data to Command Mode) - 3.30 S3 Command: Command Line Termination Character - 3.31 S10 Command: Automatic Disconnect Delay - 3.32 S11 Command: DTMF Dialing Speed - 5.67 +CCED Command: Cell Environment Description - 6.15 +KAAT Command: GPRS Automatic Attach - 17.10 +MSOFACTORYPOLICY Command: Factory Policy |
|  |  | Updated: - 2.11 &C Command: Set Data Carrier Detect (DCD) Function Mode - 2.12 &D Command: Set Data Terminal Ready (DTR) Function Mode - 2.14 &W Command: Save Stored Profile - 2.15 &V Command: Display Current Configuration - 2.16 &K Command: Flow Control Option - 2.17 &S Command: DSR Option - 2.18 IPR Command: Set Fixed Local/DTE Rate - 3.15 +CMUX Command: Multiplexing Mode - 3.19 &R Command: RTS and CTS Option - 3.24 S5 Command: Write Command Line Editing Character - 3.25 S6 Command: Pause before Blind Dialing - 3.26 S8 Command: Comma Dial Modifier Time - 4.1 D Command: Dial Number - 5.16 +KCELL Command: Cell Environment Information - 5.49 +KLTEMUTE Command: Mute LTE TX - 5.50 +KSYNC Command: Application Synchronization Signal - 5.51 +KLTEPARAM Command: LTE Parameters - 6.4 +CNUM Command: Subscriber Number - 8.9 +CNMI Command: New Message Indication - 9.1 +KPCMCFG Command: Configure PCM Digital Audio - 9.4 +KECHO Command: Echo Cancellation - 10.2 +CGACT Command: Activate or Deactivate PDP Context - 10.5 +CGTFT Command: Traffic Flow Template |
| 5.0 | February 13, 2017 | Updated: - 10.18 +CGEQNEG Command: 3G Negotiated Quality of Service Profile - 10.19 +CGREG Command: GPRS Network Registration Status - 10.24 +WPPP Command: PDP Context Authentication Configuration - 12.7.1 +KCNXCFG Command: GPRS Connection Configuration - 12.13.1 +KHTTPCFG Command: HTTP Connection Configuration - 13.6 +WDSI Command: Device Services Indication - 13.8 +WDSS Command: Device Services Session - 14.1 +WMTXPOWER Command: Test RF Tx - 14.2 +WMRXPOWER Command: Test RF Rx - 17 M2M Service Optimization Commands - 18.2.7 Error Case Examples |
|  |  | Deleted 13.1 +WDSA Command: Change Account for DM Connection |
| 5.1 | March 03, 2017 | Deleted: - 10.23 +XCEDATA Command: Establish ECM Data Connection - 17.10 +MSOFACTORYPOLICY Command: Factory Policy |
| 6.0 | May 17, 2017 | Added: - 5.67 +WESHDOWN Command: Emergency Shutdown - 5.68 +KMCLASS Command: Change GPRS and EGPRS Multislot Class |
|  |  | Updated: - 2.10 X Command: Result Code Selection and Call Progress Monitoring Control - 2.16 &K Command: Flow Control Option - 2.18 IPR Command: Set Fixed Local/DTE Rate - 3.23 N Command: Negotiate Handshake Option - 3.24 S5 Command: Write Command Line Editing Character - 3.27 W Command: Extended Result Code - 3.28 B Command: Data Rate Selection - 3.29 S2 Command: Set Character for the Escape Sequence (Data to Command Mode) - 5.16 +KCELL Command: Cell Environment Information - 5.35 +XCELLINFO Command: Provide Cell Information - 5.50 +KSYNC Command: Application Synchronization Signal - 5.53 +KSRAT Command: Set Radio Access Technology - 5.54 *PSRDBS Command: Change Frequency Band - 5.59 +WMANTSEL Command: Select Main / Diversity Antenna for LTE - 5.67 +CCED Command: Cell Environment Description - 9.12 +VIP Command: Initialize Voice Parameters - 9.13 +CODECINFO Command: Display Audio Codec Information - 10.18 +CGEQNEG Command: 3G Negotiated Quality of Service Profile - 12.12.4 +KFTPSND Command: Send FTP Files - 12.13.1 +KHTTPCFG Command: HTTP Connection Configuration - 12.13.2 +KHTTPCNX Command: Start the HTTP Connection - 12.14.1 +KHTTPSCFG Command: HTTPS Connection Configuration |
| 6.0 | May 17, 2017 | Updated: - 14.1 +WMTXPOWER Command: Test RF Tx - 14.2 +WMRXPOWER Command: Test RF Rx - Table 9 Non-Generic Error Case Examples |
| 6.1 | May 25, 2017 | Updated: - 4.8 +CEER Command: Extended Error Report - 18.2.2 CEER Error Codes |
| 6.2 | June 26, 2017 | Updated: - 3.15 +CMUX Command: Multiplexing Mode - 3.22 \N Command: Data Transmission Mode - 3.28 B Command: Data Rate Selection - 4.8 +CEER Command: Extended Error Report - 5.34 +XDATACHANNEL Command: Configure Data Channel - 5.35 +XCELLINFO Command: Provide Cell Information - 6.10 +CREG Command: Network Registration - 8.15 +CSDH Command: Show Text Mode Parameters - 17 M2M Service Optimization Commands - 18.4 GSM 27.010 Multiplexing Protocol |
| 7.0 | August 01, 2017 | Updated: - 2.12 &D Command: Set Data Terminal Ready (DTR) Function Mode - 3.2 Z Command: Reset and Restore User Configuration - 3.18 +KODIS Command: Access ODIS Information - 5.6 +CFUN Command: Set Phone Functionality - 5.16 +KCELL Command: Cell Environment Information - 5.19 +KADC Command: Analog Digital Converter - 5.47 +KUSBCOMP Command: Set USB Composition - 5.53 +KSRAT Command: Set Radio Access Technology - 6.7 +COPS Command: Operator Selection - 6.14 +CEMODE Command: UE Modes of Operation for EPS - 9 Audio Commands - 10.19 +CGREG Command: GPRS Network Registration Status - 11.7 *PSSTKI Command: SIM Toolkit Configuration - 12.8.3 +KIPOPT Command: General Options Configuration - 12.10.2 +KTCPCNX Command: Start TCP Connection - 12.10.4 +KTCPSND Command: Send Data through a TCP Connection - 12.10.11 +KTCPSTART Command: Start a TCP Connection in Direct Data Flow12.11.3 +KUDPSND Command: Send Data through a UDP Connection - 12.12.4 +KFTPSND Command: Send FTP Files - 12.13.3 +KHTTPHEADER Command: Set the HTTP Request Header - 12.14.3 +KHTTPSHEADER Command: Set the HTTPS Request Header - 12.14.4 +KHTTPSGET Command: Get Information from HTTPS Server - 12.15.1 +KCERTSTORE Command: Store Root CA and Local Certificates to Internal Storage - 12.15.2 +KPRIVKSTORE Command: Store Private Key Associated to a Local Certificate |
| 7.0 | August 01, 2017 | Updated: - 18.2.3 CMS Error Codes - 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table |
|  |  | Deleted 5.67 +CCED Command: Cell Environment Description |

Contents

1.	Introduction	21

1.1.	Reference Configuration	21

1.2.	AT Command Principles	21

1.2.1.	Parameters	22

1.2.2.	Answers and Responses	22

1.2.3.	Multiple AT Commands on the Same Command Line	22

1.2.4.	AT Commands on Separate Lines	23

1.3.	Unsolicited Result Codes (URCs)	23

1.4.	PDP Context Usage	23

1.5.	SMS Commands	24

1.6.	Document Modification	24

1.7.	Abbreviations	24

2.	V25ter AT Commands	29

2.1.	+++ Command: Switch from Data Mode to Command Mode	29

2.2.	A/ Command: Repeat Previous Command Line	29

2.3.	O Command: Switch from Command Mode to Data Mode	29

2.4.	E Command: Enable Echo Command	30

2.5.	Q Command: Set Result Code Presentation Mode	30

2.6.	S0 Command: Set Number of Rings before Automatic Call Answering	31

2.7.	S4 Command: Set Response Formatting Character	31

2.8.	S7 Command: Set Delay for Connection Completion	32

2.9.	V Command: TA Response Format	32

2.10.	X Command: Result Code Selection and Call Progress Monitoring Control	33

2.11.	&C Command: Set Data Carrier Detect (DCD) Function Mode	33

2.12.	&D Command: Set Data Terminal Ready (DTR) Function Mode	34

2.13.	&F Command: Restore Factory Settings	34

2.14.	&W Command: Save Stored Profile	35

2.15.	&V Command: Display Current Configuration	35

2.16.	&K Command: Flow Control Option	36

2.17.	&S Command: DSR Option	37

2.18.	IPR Command: Set Fixed Local/DTE Rate	37

2.19.	L Command: Monitor Speaker Loudness	38

2.20.	M Command: Monitor Speaker Mode	38

3.	General AT Commands	39

3.1.	I Command: Request Identification Information	39

3.2.	Z Command: Reset and Restore User Configuration	42

3.3.	+CGMI Command: Request Manufacturer Identification	42

3.4.	+CGMM Command: Request Model Identification	42

3.5.	+CGMR Command: Request Revision Identification	43

3.6.	+CGSN Command: Request Product Serial Number Identification (IMEI)	44

3.7.	+KGSN Command: Request Product Serial Number and Software Version	44

3.8.	+HWREV Command: Request Hardware Revision	45

3.9.	+CSCS Command: Set TE Character Set	46

3.10.	+CIMI Command: Request International Mobile Subscriber Identity	47

3.11.	+GMI Command: Request Manufacturer Identification	47

3.12.	+GMM Command: Request Model Identification	48

3.13.	+GMR Command: Request Revision Identification	48

3.14.	+GSN Command: Request Product Serial Number (IMEI)	49

3.15.	+CMUX Command: Multiplexing Mode	49

3.16.	+GCAP Command: Request Complete TA Capability List	51

3.17.	+WIMEI Command: IMEI Write and Read	51

3.18.	+KODIS Command: Access ODIS Information	52

3.19.	&R Command: RTS and CTS Option	53

3.20.	+FMI Command: Request Manufacturer Identification	53

3.21.	+FMM Command: Request Model Identification	54

3.22.	\N Command: Data Transmission Mode	54

3.23.	N Command: Negotiate Handshake Option	55

3.24.	S5 Command: Write Command Line Editing Character	55

3.25.	S6 Command: Pause before Blind Dialing	56

3.26.	S8 Command: Comma Dial Modifier Time	56

3.27.	W Command: Extended Result Code	57

3.28.	B Command: Data Rate Selection	57

3.29.	S2 Command: Set Character for the Escape Sequence (Data to Command Mode)	57

3.30.	S3 Command: Command Line Termination Character	58

3.31.	S10 Command: Automatic Disconnect Delay	58

3.32.	S11 Command: DTMF Dialing Speed	59

4.	Call Control Commands	60

4.1.	D Command: Dial Number	60

4.2.	D> Command: Direct Dialing from Phonebook	61

4.3.	+CHUP Command: Hang up Call	61

4.4.	+CR Command: Service Reporting Control	62

4.5.	+CRC Command: Set Cellular Result Codes for Incoming Call Indication	62

4.6.	+CSTA Command: Select Type of Address	63

4.7.	+CMOD Command: Call Mode	64

4.8.	+CEER Command: Extended Error Report	64

4.9.	+CSNS Command: Single Numbering Scheme	65

4.10.	+CBST Command: Select Bearer Service Type	66

5.	Mobile Equipment Control and Status Commands	68

5.1.	+CAMM Command: Accumulated Call Meter (Maximum)	68

5.2.	+CCWE Command: Call Meter Maximum Event	68

5.3.	+CCLK Command: Real Time Clock	69

5.4.	+CIND Command: Indicator Control	70

5.5.	+CLAC Command: List Available AT Commands	71

5.6.	+CFUN Command: Set Phone Functionality	71

5.7.	+CMER Command: Mobile Equipment Event Reporting	73

5.8.	+CMEE Command: Report Mobile Termination Error	75

5.9.	+CCID Command: Request SIM Card Identification	76

5.10.	+FMR Command: Request Revision Identification	77

5.11.	+CPIN Command: Enter Pin	77

5.12.	+CPIN2 Command: Enter Pin2	78

5.13.	+CPUC Command: Price per Unit and Currency	79

5.14.	+CPAS Command: Phone Activity Status	80

5.15.	+CSQ Command: Signal Quality	80

5.16.	+KCELL Command: Cell Environment Information	81

5.17.	+KGPIO Command: Hardware IO Control	83

5.18.	+KGPIOCFG Command: GPIO Configuration	85

5.19.	+KADC Command: Analog Digital Converter	86

5.20.	+CSIM Command: Generic SIM Access	87

5.21.	+KSIMDET Command: SIM Detection	88

5.22.	+CLAN Command: Read Language	89

5.23.	+CCHO Command: Open Logical Channel	89

5.24.	+CCHC Command: Close Logical Channel	90

5.25.	+CGLA Command: Generic UICC Logical Channel Access	90

5.26.	+CRLA Command: Restricted UICC Logical Channel Access	91

5.27.	+CUAD Command: UICC Application Discovery	92

5.28.	+CRSM Command: Restricted SIM Access	92

5.29.	+CEAP Command: EAP Authentication	94

5.30.	+CERP Command: EAP Retrieve Parameters	95

5.31.	+KTEMPMON Command: Temperature Monitor	95

5.32.	+CTZU Command: Automatic Time Zone Update	97

5.33.	+CTZR Command: Time Zone Reporting	97

5.34.	+XDATACHANNEL Command: Configure Data Channel	98

5.35.	+XCELLINFO Command: Provide Cell Information	100

5.36.	+KSLEEP Command: Power Management Control for UART	101

5.37.	+HBHV Command: Configure General System Behavior	103

5.38.	+CIREP Command: IMS Network Reporting	104

5.39.	+CIREG Command: Registration Information	105

5.40.	+GST Command: General System Status Information	106

5.41.	+CESQ Command: Extended Signal Quality	107

5.42.	+XCSQ Command: Radio Signal Strength and Quality with URC Support	108

5.43.	+XCESQ Command: Extended Signal Quality with URC Support	109

5.44.	+WEXTCLK Command: External Clocks Setting	111

5.45.	+KRIC Command: Ring Indicator Control	112

5.46.	+CPWROFF Command: Switch MS Off	113

5.47.	+KUSBCOMP Command: Set USB Composition	114

5.48.	+WMUSBVCC Command: USB VCC Detection Setting	115

5.49.	+KLTEMUTE Command: Mute LTE TX	116

5.50.	+KSYNC Command: Application Synchronization Signal	118

5.51.	+KLTEPARAM Command: LTE Parameters	119

5.52.	+KBND Command: Current Networks Band Indicator	121

5.53.	+KSRAT Command: Set Radio Access Technology	122

5.54.	*PSRDBS Command: Change Frequency Band	123

5.55.	+CMEC Command: Mobile Equipment Control Mode	124

5.56.	+CPOF Command: Power Off	124

5.57.	+KGSMAD Command: GSM/LTE Antenna Detection	125

5.58.	+KSREP Command: Mobile Start-up Reporting	126

5.59.	+WMANTSEL Command: Select Main / Diversity Antenna for LTE	127

5.60.	+KSIMSEL Command: SIM Selection	129

5.61.	+BOOTDWLCFG Command: Boot Configuration for Firmware Download	131

5.61.1.	Description	131

5.61.2.	Syntax	133

5.62.	+CALA Command: Set Alarm	135

5.63.	+CALD Command: Delete Alarm	136

5.64.	+KCCINFO Command: Camped Cell Information	137

5.65.	+CALM Command: Alert Sound Mode	138

5.66.	+CRSL Command: Ringer Sound Level	139

5.67.	+WESHDOWN Command: Emergency Shutdown	140

5.68.	+KMCLASS Command: Change GPRS and EGPRS Multislot Class	141

6.	Network Service Related Commands	143

6.1.	+CAOC Command: Advice of Charge	143

6.2.	+CUSD Command: Unstructured Supplementary Service Data	143

6.3.	+CLCK Command: Facility Lock	144

6.4.	+CNUM Command: Subscriber Number	146

6.5.	+COLP Command: Connected Line Identification Presentation	146

6.6.	+COPN Command: Read Operator Name	147

6.7.	+COPS Command: Operator Selection	148

6.8.	+CPOL Command: Preferred PLMN List	149

6.9.	+CPWD Command: Change Password	150

6.10.	+CREG Command: Network Registration	151

6.11.	+CSSN Command: Supplementary Service Notification	152

6.12.	+CPLS Command: Select Preferred PLMN List	154

6.13.	+CEREG Command: EPS Network Registration Status	154

6.14.	+CEMODE Command: UE Modes of Operation for EPS	155

6.15.	+KAAT Command: GPRS Automatic Attach	156

7.	Phone Book Management	158

7.1.	+CPBF Command: Find Phonebook Entries	158

7.2.	+CPBR Command: Read Current Phonebook Entries	159

7.3.	+CPBS Command: Select Phonebook Memory Storage	160

7.4.	+CPBW Command: Write Phonebook Entry	161

7.5.	+PBREADY URC: Phonebook Ready	163

8.	SMS Commands	164

8.1.	Parameters Definition	164

8.1.1.	Message Storage Parameters	164

8.1.2.	Message Data Parameters	165

8.2.	+CMGD Command: Delete Message	166

8.3.	+CMGF Command: Set Message Format	167

8.4.	+CMGL Command: List Messages	168

8.5.	+CMGR Command: Read Message	168

8.6.	+CMGS Command: Send Message	169

8.7.	+CMGW Command: Write Message to Memory	170

8.8.	+CMSS Command: Send Message from Storage	171

8.9.	+CNMI Command: New Message Indication	171

8.10.	+CSCB Command: Select Cell Broadcast Message Type	173

8.11.	+CSCA Command: Service Center Address	174

8.12.	+CSMP Command: Set Text Mode Parameters	174

8.13.	+CSMS Command: Select Message Service	175

8.14.	+CPMS Command: Preferred Message Storage	175

8.15.	+CSDH Command: Show Text Mode Parameters	176

8.16.	+XCMGS3GPP2 Command: Send 3GPP2 SMS Message	177

8.17.	+XCMT3GGP2 Command: Enable or Disable the 3GPP2 MT SMS URC	178

9.	Audio Commands	179

9.1.	+KPCMCFG Command: Configure PCM Digital Audio	179

9.2.	+WMAUDIOLOOP Command: Audio Test	181

9.3.	+CLVL Command: Loudspeaker Volume Level	182

9.4.	+KECHO Command: Echo Cancellation	183

9.5.	+KNOISE Command: Echo Suppression	184

9.6.	+KPC Command: Peak Compressor	186

9.7.	+KST Command: Side Tone	187

9.8.	+KVGR Command: Receive Gain Selection	188

9.9.	+KVGT Command: Transmit Gain Selection	189

9.10.	+VGR Command: Receive Gain Selection	190

9.11.	+VGT Command: Transmit Gain Selection	191

9.12.	+VIP Command: Initialize Voice Parameters	192

9.13.	+CODECINFO Command: Display Audio Codec Information	193

9.14.	+KSRAP Command: Save or Restore Audio Parameters	195

9.15.	+WVR Command: Voice Codec Selection	196

9.16.	+VTD Command: Tone Duration	197

9.17.	+VTS Command: DTMF and Tone Generation	198

10.	Packet Domain Commands	199

10.1.	+CGATT Command: PS Attach or Detach	199

10.2.	+CGACT Command: Activate or Deactivate PDP Context	199

10.3.	+CGANS Command: PDP Context Activation Manual Response	200

10.4.	+CGCMOD Command: Modify PDP Context	201

10.5.	+CGTFT Command: Traffic Flow Template	201

10.6.	+CGCLASS Command: GPRS Mobile Station Class	203

10.7.	+CGDCONT Command: Define PDP Context	204

10.8.	+CGDSCONT Command: Define Secondary PDP Context	206

10.9.	+CGDATA Command: Enter Data State	207

10.10.	+CGED Command: GPRS Cell Environment	207

10.11.	+CGEREP Command: Packet Domain Event Reporting	216

10.12.	+CGAUTO Command: Automatic Response	217

10.13.	+CGPADDR Command: Show PDP Address	218

10.14.	+CGQMIN Command: Quality of Service Profile (Minimum)	219

10.15.	+CGEQMIN Command: 3G Quality of Service Profile (Minimum)	220

10.16.	+CGQREQ Command: Request Quality of Service Profile	222

10.17.	+CGEQREQ Command: 3G Request Quality of Service Profile	223

10.18.	+CGEQNEG Command: 3G Negotiated Quality of Service Profile	225

10.19.	+CGREG Command: GPRS Network Registration Status	227

10.20.	+CGSMS Command: Select Service for MO SMS Messages	229

10.21.	+CRLP Command: Select Radio Link Protocol	229

10.22.	+XDNS Command: Dynamic DNS Request	230

10.23.	+CGPIAF Command: Printing IP Address Format	231

10.24.	+WPPP Command: PDP Context Authentication Configuration	232

11.	SIM Application Toolkit Commands	234

11.1.	+STKPRO Command: Display List of Supported Proactive Commands	234

11.2.	+STKTR Command: Enter Response	237

11.3.	+STKENV Command: Send a SIM APPL TK Envelope Command	240

11.4.	+STKPROF Command: Terminal Profile Data	241

11.5.	+STKCC Notification: SIM – APPL – TK Call Control	241

11.6.	+STKCNF Notification: SIM – APPL – TK Proactive Session Status	242

11.7.	*PSSTKI Command: SIM Toolkit Configuration	242

12.	Protocol Specific Commands	246

12.1.	Preliminary Comments	246

12.2.	IP Address Format in AT Commands	246

12.3.	Session ID	246

12.4.	Connection of PDP Contexts	246

12.5.	Buffer Length of AT Commands	246

12.6.	Parameter Format of AT Commands	247

12.7.	Connection Configuration	247

12.7.1.	+KCNXCFG Command: GPRS Connection Configuration	247

12.7.2.	+KCNXTIMER Command: Connection Timer Configuration	248

12.7.3.	+KCNXPROFILE Command: Current Profile Connection Configuration	249

12.7.4.	+KCGPADDR Command: Display PDP Address	250

12.7.5.	+KCNX_IND Notification: Connection Status Notification	251

12.7.6.	+KCNXUP Command: Bring the PDP Connection Up	251

12.7.7.	+KCNXDOWN Command: Bring the PDP Connection Down	252

12.8.	Common Configuration	253

12.8.1.	+KPATTERN Command: Custom End of Data Pattern	253

12.8.2.	+KURCCFG Command: Enable or Disable the URC from Protocol Commands	254

12.8.3.	+KIPOPT Command: General Options Configuration	255

12.9.	SSL Configuration	257

12.9.1.	+KSSLCRYPTO Command: Cipher Suite Configuration	257

12.9.2.	+KSSLCFG Command: SSL Configuration	258

12.10.	TCP Specific Commands	260

12.10.1.	+KTCPCFG Command: TCP Connection Configuration	260

12.10.2.	+KTCPCNX Command: Start TCP Connection	261

12.10.3.	+KTCPRCV Command: Receive Data through a TCP Connection	262

12.10.4.	+KTCPSND Command: Send Data through a TCP Connection	263

12.10.5.	+KTCPCLOSE Command: Close Current TCP Operation	264

12.10.6.	+KTCPDEL Command: Delete a Configured TCP Session	264

12.10.7.	+KTCP_SRVREQ Notification: Incoming Client Connection Request	265

12.10.8.	+KTCP_DATA Notification: Incoming Data through a TCP Connection	266

12.10.9.	+KTCP_IND Notification: TCP Status	267

12.10.10.	+KTCPSTAT Command: Get TCP Socket Status	267

12.10.11.	+KTCPSTART Command: Start a TCP Connection in Direct Data Flow	268

12.10.12.	+KTCP_ACK Notification: Status Report for Latest TCP Data	269

12.10.13.	+KTCPACKINFO Command: Poll ACK Status for the Latest Data	269

12.11.	UDP Specific Commands	270

12.11.1.	+KUDPCFG Command: UDP Connection Configuration	270

12.11.2.	+KUDPRCV Command: Receive Data through a UDP Connection	272

12.11.3.	+KUDPSND Command: Send Data through a UDP Connection	273

12.11.4.	+KUDPCLOSE Command: Close Current UDP Operation	274

12.11.5.	+KUDPDEL Command: Delete a Configured UDP Session	275

12.11.6.	+KUDP_IND Notification: UDP Status	275

12.11.7.	+KUDP_DATA Notification: Incoming Data through a UDP Connection	275

12.12.	FTP Client Specific Commands	276

12.12.1.	+KFTPCFG Command: FTP Configuration	276

12.12.2.	+KFTPCNX Command: Start FTP Connection	278

12.12.3.	+KFTPRCV Command: Receive FTP Files	279

12.12.4.	+KFTPSND Command: Send FTP Files	280

12.12.5.	+KFTPDEL Command: Delete FTP Files	281

12.12.6.	+KFTP_IND Notification: FTP Status	282

12.12.7.	+KFTPCLOSE Command: Close Current FTP Connection	283

12.12.8.	+KFTPCFGDEL Command: Delete a Configured FTP Session	283

12.13.	HTTP Client Specific Commands	284

12.13.1.	+KHTTPCFG Command: HTTP Connection Configuration	284

12.13.2.	+KHTTPCNX Command: Start the HTTP Connection	285

12.13.3.	+KHTTPHEADER Command: Set the HTTP Request Header	286

12.13.4.	+KHTTPGET Command: Get HTTP Server Information	287

12.13.5.	+KHTTPHEAD Command: Get HTTP Headers	288

12.13.6.	+KHTTPPOST Command: Perform HTTP Post	288

12.13.7.	+KHTTPCLOSE Command: Close an HTTP Connection	289

12.13.8.	+KHTTPDEL Command: Delete a Configured HTTP Session	290

12.13.9.	+KHTTP_IND Notification: HTTP Status	290

12.14.	HTTPS Client Specific Commands	291

12.14.1.	+KHTTPSCFG Command: HTTPS Connection Configuration	291

12.14.2.	+KHTTPSCNX Command: Start HTTPS Connection	293

12.14.3.	+KHTTPSHEADER Command: Set the HTTPS Request Header	294

12.14.4.	+KHTTPSGET Command: Get Information from HTTPS Server	294

12.14.5.	+KHTTPSHEAD Command: Retrieve HTTP Headers	295

12.14.6.	+KHTTPSPOST Command: Send Data to HTTPS Server	296

12.14.7.	+KHTTPSCLOSE Command: Close an HTTPS Connection	297

12.14.8.	+KHTTPSDEL Command: Close an HTTPS Connection	298

12.14.9.	+KHTTPS_IND Notification: HTTPS Status	298

12.15.	SSL Certificate Manager	299

12.15.1.	+KCERTSTORE Command: Store Root CA and Local Certificates to Internal Storage		299

12.15.2.	+KPRIVKSTORE Command: Store Private Key Associated to a Local Certificate			300

12.15.3.	+KCERTDELETE Command: Delete Local Certificate from the Index	301

12.15.4.	+KPRIVKDELETE Command: Delete Private Key from the Index	302

13.	AVMS Commands	303

13.1.	+WDSC Command: Device Services Configuration	303

13.2.	+WDSD Command: Device Services Local Download	305

13.3.	+WDSE Command: Device Services Error	306

13.4.	+WDSF Command: Device Services Fallback	307

13.5.	+WDSG Command: Device Services General Status	308

13.6.	+WDSI Command: Device Services Indication	309

13.7.	+WDSR Command: Device Services Reply	312

13.8.	+WDSS Command: Device Services Session	313

13.9.	+WDSM Command: Manage Device Services	316

14.	Test Commands	318

14.1.	+WMTXPOWER Command: Test RF Tx	318

14.2.	+WMRXPOWER Command: Test RF Rx	321

15.	NV Related Commands	325

15.1.	Auto Generation of NV Backup Files	325

15.2.	Auto Recovery from Backup NV Files	325

15.3.	+NVBU Command: NV Backup Status and Control	326

15.4.	+NVBU_IND Notification: NV Backup Status Notification	328

16.	Board Support Commands	330

16.1.	+WCARRIER Command: Show Carrier Name	330

17.	M2M Service Optimization Commands	331

17.1.	+MSOSTATUS Command: Operating Status	331

17.2.	+MSORTCSTATUS Command: Display Trust RTC Status	332

17.3.	+MSOPOLICY Command: Update MSO Policies	332

17.4.	+MSORETRYINFO Command: Read Retry Information	334

17.5.	+MSOMONITOR Command: Monitoring Status Control	335

17.6.	+MSOMONITORVALUE Command: Read Monitored Data	335

17.7.	+MSOEVTLOGSTATUS Command: Event Log Status	337

17.8.	+MSOEVTLOGPUSH Command: Event Log Push	338

17.9.	+MSOEVTLOG Command: Read Event Log	338

18.	Appendix	340

18.1.	Result Codes and Unsolicited Messages	340

18.2.	Error Codes	341

18.2.1.	CME Error Codes	341

18.2.2.	CEER Error Codes	343

18.2.2.1.	SS Error Codes	348

18.2.3.	CMS Error Codes	353

18.2.4.	GPRS Error Codes	354

18.2.5.	FTP Reply Codes	355

18.2.6.	AVMS Error Codes	356

18.2.7.	Error Case Examples	356

18.3.	Commands without Pin Code Requirement	363

18.4.	GSM 27.010 Multiplexing Protocol	364

18.5.	TCP Commands Examples	365

18.5.1.	Client Mode	365

18.5.2.	Server Mode	366

18.5.3.	Polling for the Status of a Socket	367

18.5.4.	End to End TCP Connection	368

18.5.5.	Error Case for End to End TCP Connection	369

18.5.6.	Use Cases for AT+KTCPACKINFO and <URC-ENDTCP-enable> Option	369

18.5.6.1.	<URC-ENDTCP-enable> is Disabled (default setting)	369

18.5.6.2.	<URC-ENDTCP-enable> is Enabled	370

18.6.	UDP Commands Examples	371

18.6.1.	Client Mode	371

18.6.2.	Server Mode	372

18.6.3.	Use Cases for KTCP_DATA and KUDP_DATA	373

18.6.3.1.	KTCP_DATA and KUDP_DATA without Data Auto Retrieval – Client Mode	373

18.6.3.2.	KTCP_DATA and KUDP_DATA without Data Auto Retrieval – Server Mode	373

18.6.3.3.	KTCP_DATA and KUDP_DATA with Data Auto Retrieval – Client Mode	374

18.6.3.4.	KTCP_DATA and KUDP_DATA with Data Auto Retrieval – Server Mode	375

18.7.	FTP Commands Examples	375

18.7.1.	Client Mode	375

18.7.2.	"FTP Resume" Use Case	376

18.7.2.1.	Resume Feature when Transmitting Data to Serial Link	376

18.7.2.2.	Use Case when FTP Server does not Support the Resume Feature	377

18.8.	HTTP Commands Examples	377

18.9.	Switch Data/Command Mode DTR +++ ATO Behavior Table	379

List of Tables

Table 1.	Types of Extended AT Commands	22

Table 2.	Time Out Values	133

Table 3.	Extended Time Out Values	133

Table 4.	Tag 128 MN_GENERAL_PROBLEM with Causes	358

Table 5.	Tag 129 MN_INVOKE_PROBLEM with Causes	358

Table 6.	Tag 130 MN_RETURN_RESULT_PROBLEM with Causes	358

Table 7.	Tag 131 MN_RETURN_ERROR_PROBLEM with Causes	358

Table 8.	Generic Error Case Examples	362

Table 9.	Non-Generic Error Case Examples	364

Table 10.	Internet Error Case Examples	365

# Introduction

This document presents the AT Command Set for the AirPrime HL76xx series of embedded modules. AirPrime HL76xx variants covered in this manual are:

HL7618

HL7618RD

HL7648

HL7650

HL7688

HL7690

HL7692

## Reference Configuration

Reference Configuration

The User Equipment (UE) consists of the mobile equipment (ME) and the (U)SIM messages may be stored in either, but the present document does not distinguish between messages stored in the (U)SIM or in the ME. The management of message storage in the two parts of the UE is a matter for the UE implementation.

## AT Command Principles

The "AT" or "at" prefix must be set at the beginning of each line. To terminate a command line, a <CR> character must be inserted.

Commands are usually followed by a response that includes ‘<CR><LF><response><CR><LF>’. Throughout this document, only the responses are indicated, the <CR> and <LF> characters are omitted intentionally.

Four kinds of extended AT commands are implemented as listed in the table below.

Types of Extended AT Commands

| Command Type | Syntax | Definition |
| --- | --- | --- |
| Test Command | AT+CXXX=? | The equipment returns the list of parameters and values ranges set with the corresponding Write command or by internal processes |
| Read Command | AT+CXXX? | This command returns the currently set value of parameters |
| Write Command | AT+CXXX=<…> | This command sets user-related parameter values |
| Execution command | AT+CXXX | The execution command reads non-variable parameters affected by internal processes in the equipment |

### Parameters

In this document, the default parameters are underlined and the optional parameters are enclosed in square brackets.

Optional parameters or sub-parameters can be omitted unless they are followed by other parameters. A parameter in the middle of a string can be omitted by replacing it with a comma.

When the parameter is a character string, the string must be enclosed in quotation marks.

All space characters will be ignored when using strings without quotation marks.

### Answers and Responses

There is always an answer sent by the TA to an AT Command line (except the very special case of a TA setup for no answer, see ATQ).

The answer is always terminated by an indication of success or failure. However, regarding the setup of the TA (by AT Commands), the message may be different.

Classical messages	OK or ERROR

Extended Error message (see AT+CMEE)	+CME ERROR: <n>

(See Appendix for the different values for <n>)

Numeric Mode (see ATV)	<n> with: <n> = 0  OK or <n> is an error code

### Multiple AT Commands on the Same Command Line

You may enter several AT commands on the same line. This eliminates the need to type the "AT" or "at" prefix before each command and to wait for the answer for each command. The main advantage is to avoid losing bandwidth on the link between DTE and the Module.

There is no separator between two basic commands but a semi-colon character is necessary between two extended commands (prefix +).The command line buffer accepts a maximum of 391 characters. If this number is exceeded none of the commands will be executed and TA returns ERROR.

If a command is not supported, then the treatment of the line is stopped (i.e. the following ones are not treated) and an error message is returned.

Example:

Command:	ATZE1+CBST=7,0,1;+CBST?

Answer:	+CBST=7,0,1

OK

### AT Commands on Separate Lines

When you enter a series of AT commands on separate lines, it is strongly advised to leave a pause between the preceding and the following command until the final answer (OK or Error message) appears. This avoids sending too many AT commands at a time without waiting for a response for each.

## Unsolicited Result Codes (URCs)

Unsolicited result codes (URCs) are sent simultaneously to all the channels (USB/UART) configured in AT commands mode.

URCs are not sent to channels configured in Data/NMEA/Traces modes.

In sleep mode URCs wake up the module and are sent to the AT commands channels.

## PDP Context Usage

This section is only applicable to HL7618 and HL7618RD.

PDP context IDs (CIDs) are designed with the following purposes:

CID=1 with APN=VZWIMS is reserved for IMS in Verizon (SMS over IMS)

CID=2 with APN=VZWADMIN is reserved for Verizon Administration (SIM provision, OMADM, etc.)

CID=3 with APN=VZWINTERNET is the default Internet APN in Verizon

CID=4 with APN=VZWAPP is the default application APN in Verizon

CID=5 is reserved for Sierra Wireless AirVantage (AVMS)

Note that:

CID=1 to CID=4 are managed by Verizon OMADM administration. These APNs may be updated by the VZW OMADM server, e.g. after server’s initialized DM session, or after a SIM card change.

CID=3 to CID=10 are intended for customer use if another APN/PDP context is needed (e.g. APN for private network). Note that:

CID=3 or 4 may be updated by the VZW OMADM server

CID=5 is reserved for AVMS, but can be used by customer if AVMS feature is not used

It is recommended to use CID=6 to CID=10 in customer application if another APN/PDP context is needed

CID=11 to CID=20 are disabled (related AT commands return error response) as maximum PDP context (APN parameter list) is 10 entries for Verizon data retry restriction. These CIDs can only be used properly provided that their IP-type and APN are the same as one configured in CID=1 to CID=10.

CID=1 or 2 and CID=11-20 are locked by AT+HBHV=2 (related AT commands return error response).

Two failed activation attempts (+CGACT, DUT, etc.) due to invalid/incorrect APN blocks the corresponding CID from further attempts until the module reboots, due to Verizon data retry restriction. If the CID is blocked, AT+CGACT returns +CME ERROR: 4 until the module reboots.

## SMS Commands

This section is only applicable to HL7618 and HL7618RD.

SMS is sent over IMS in the Verizon network using 3GPP2 SMS PDU format and protocol. Generally, 3GPP AT commands do not work with 3GPP2 SMS, but additional support is added in the HL7618 and HL7618RD to convert input SMS from 3GPP format to 3GPP2 format. This allows the use of 3GPP AT commands like +CMGS, +CNMI and +CMGD to send, show or delete SMS messages as if the SMS is sent, received or stored in 3GPP SMS PDU format.

However, +XCMGS3GPP2 and +XCMT3GGP2 are still available for sending and receiving SMS messages in 3GPP2 SMS PDU format.

## Document Modification

The commands described in this document are only to be used for usual AT commands use.

The information provided for the commands are subject to change without notice.

## Abbreviations

| Abbreviation | Definition |
| --- | --- |
| ACM | Accumulated Call Meter |
| ADC | Analog Digital Converter |
| ADN | Abbreviated Dialing Number (Phonebook) |
| AMR | Adaptive Multi-Rate |
| AMR-FR | AMR Full Rate (full rate speech version 3) |
| AMR-HR | AMR Half Rate (half rate speech version 3) |
| AOC | Advice Of Charge |
| APN | Access Point Name |
| ARN | Address Resolution Protocol |
| ARFCN | Absolute Radio Frequency Channel Number |
| ASCII | American Standard Code for Information Interchange |
| AT | ATtention; Hayes Standard AT command Set |
| BCCH | Broadcast Channel |
| BER | Bit Err Rate |
| BM | Broadcast Message Storage |
| CBM | Cell Broadcast Message |
| CB | Cell Broadcast |
| CCK | Corporate Control Key |
| CCM | Current Call Meter |
| CHV | Card Holder Verification |
| CHAP | Challenge handshake Authentication Protocol |
| CI | Cell Identifier |
| CLI | Client Line Identification |
| CNL | Cooperative Network List |
| CODEC | Coder Decoder |
| COLP | Connected Line Identification Presentation |
| CPHS | Common PCN Handset Specification |
| CPU | Central Processing Unit |
| CSD | Circuit Switched Data |
| CSP | Customer Service Profile |
| CTM | Cellular Text telephone Modem |
| CTS | Clear To Send signal |
| CUG | Closed User Group |
| DAC | Digital to Analog Converter |
| DCS | Digital Cellular System |
| DCE | Data Circuit Equipment |
| DCD | Data Carrier Detect |
| DLC | Data Link Connection |
| DLCI | Data Link Connection Identifier |
| DM | Device Management |
| DNS | Domain Name System |
| DSR | Data Set Ready |
| DTE | Date Terminal Equipment |
| DTMF | Dual Tone Multi-Frequency |
| DTR | Data Terminal Ready |
| ECC | Emergency Call Codes |
| ECM | Error Correction Mode |
| ECT | Explicit Call Transfer |
| EDGE | Enhanced Data rates for GSM Evolution |
| EEPROM | Electrically Erasable Programming Only Memory |
| EF | Elementary Files |
| EFR | Enhanced Full Rate (full rate speech version 2) |
| EGPRS | Enhanced GPRS |
| ENS | Enhanced Network Selection |
| E-ONS | Enhanced Operator Name Service |
| ERMES | European Radio Messaging System |
| ETSI | European Telecommunications Standards Institute |
| FD | FIFO depth |
| FDN | Fixed Dialing Number (Phonebook) |
| FR | Full Rate (full rate speech version 1) |
| GERAN | GSM EDGE Radio Access Network |
| GPIO | General Purpose Input Output |
| GPRS | General Packet Radio Service |
| GSM | Global System for Mobile communication |
| HDLC | High-level Data Link Control |
| HFR | High Frequency Regeneration |
| HLR | Home Location Register |
| HR | Half Rate (half rate speech version 1) |
| ID | IDentifier |
| IETF | Internet Engineering Task Force |
| IMEI | International Mobile Equipment Identity |
| IMSI | International Mobile Subscriber Identity |
| IN/OUT/IN_OUT | In, out or In Out |
| I/O | Input/Output |
| IP | Internet Protocol |
| LAC | Local Area Code |
| LED | Light Emitting Diode |
| LND | Last Number Dialed |
| LP | Language Preferred |
| LPI | Lines Per Inch |
| M | Mandatory |
| MCC | Mobile Country Code |
| ME | Mobile Equipment |
| MMI | Man Machine Interface |
| MNC | Mobile Network Code |
| MNP | Microcom Networking Protocol |
| MO | Mobile Originated |
| MOC | Mobile Originated Call (outgoing call) |
| MS | Mobile Station |
| MSB | Most Significant Bit |
| MSISDN | Mobile Station International ISDN Number |
| MT | Mobile Terminal |
| MTC | Mobile Terminated Call (incoming call) |
| N.A. | Not applicable |
| NCK | Network Control Key |
| NITZ | Network Information and Time Zone |
| NSCK | Network Subset Control Key |
| NTC | Negative Temperature Coefficient |
| N.U. | Not used |
| O | Optional |
| OA | Outgoing Access |
| OPL | Operator PLMN List |
| OS | Operating System |
| OTA | Over the Air |
| PAD | Portable Application Description |
| PAP | Password Authentication Protocol |
| PC | Personal Computer |
| PCCP | PC character set Code Page |
| PCK | Personalization Control Key |
| PCL | Power Control Level |
| PCM | Protection Circuit Module |
| PCN | Personal Communication Network |
| PCS 1900 | Personal Communication Service |
| PDP | Packet Data Protocol |
| PDU | Protocol Description Unit |
| PIN | Personal Identification Number |
| PLMN | Public Land Mobile Networks |
| PNN | PLMN Network Name |
| PPP | Point-to-Point Protocol/Peer to Peer |
| PSTN | Public Switched Telephone Network |
| PTS | Product Technical Specification |
| PUCT | Price per Unit and Currency Table |
| PUK | PIN Unlock Key |
| PWM | Pulse Width Modulation |
| QoS | Quality of Service |
| RAM | Random Access Memory |
| RDMS | Remote Device Management Services |
| RI | Ring Indicator |
| RIL | Radio Interface Layer |
| RLP | Radio Link Protocol |
| RSSI | Received Signal Strength Indication |
| RTS | Ready To Send signal |
| RX | Reception |
| SAP | Service Access Point |
| SC | Service Center |
| SDU | Service Data Unit |
| SIM | Subscriber Information Module |
| SMSR | Short Message Status Report |
| SMS | Short Message Service |
| SS | Supplementary Services |
| SPCK | Service Provider Control Key |
| SPN | Service Provider Name |
| STK | SIM ToolKit |
| SVN | Software Version Number |
| TA | Terminal Adaptor |
| TBF | Temporary Block Flow |
| TE | Terminal Equipment |
| TTY | TeleTYpe |
| TON/NPI | Type Of Number/Numbering Plan Identification |
| TX | Transmission |
| UART | Universal Asynchronous Receiver Transmitter |
| UCS2 | Universal Character Set 2 Character table (2-byte coding) |
| UDUB | User Determined User Busy |
| UIH | Unnumbered Information with Header check |
| USB | Universal Serial Bus |
| USSD | Unstructured Supplementary Service Data |

# V25ter AT Commands

## +++ Command: Switch from Data Mode to Command Mode

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax +++ | Response OK |
| Reference V.25Ter | Notes - This command is only available during data mode. The +++ character sequence suspends the data flow over the AT interface and switches to command mode. This allows entering AT commands while maintaining the data connection to the remote device. - To return to data mode, use the ATO[n] command. - Line needs one second silence before and one second after (do not end with terminating character). - The "+" character may be changed with the ATS2 command (see following chapters). - The +++ characters are not transmitted in the data flow. |

## A/ Command: Repeat Previous Command Line

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax A/ | Response Depends on the previous command |
| Reference V.25Ter | Notes Line does not need to end with terminating character. |

## O Command: Switch from Command Mode to Data Mode

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax ATO[<n>] | Response TA returns to data mode from command mode: CONNECT <text> If connection is not successfully resumed: NO CARRIER Parameter <n> 0 Switch from command mode to data mode 1 – 200 Session ID |
| Reference V.25Ter | Notes ATO is the alternative command to the +++ escape sequence described in section 2.1. When a data call has been established and TA is in command mode, ATO causes the TA to resume the data connection and return to data mode. |

## E Command: Enable Echo Command

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax ATE[<value>] | Response OK or +CME ERROR: <err> Parameter <value> 0 Echo OFF 1 Echo ON |
| Notes | - This setting determines whether or not the TA echoes characters received from TE during the command state. - <value> is saved in non-volatile memory per AT port over module reboot. |

## Q Command: Set Result Code Presentation Mode

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax ATQ[<n>] | Response OK (if <n> = 0) Nothing (if <n> = 1) Parameter <n> 0 Result codes transmitted by TA 1 No result codes transmitted by TA |
| Notes | - Specifies whether or not the TA transmits any result code to the TE. Information text transmitted in response is not affected by this setting. - <n> is saved in non-volatile memory per AT port over module reboot. |

## S0 Command: Set Number of Rings before Automatic Call Answering

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Read command Syntax ATS0? | Response <n> OK |
| Write command Syntax ATS0=<n> | Response OK Parameter <n> 0 Automatic answering deactivated 1 – 255 Number of rings before automatically answering |
| Notes | In data mode (after any CONNECT) automatic call answering does not work; that means that incoming calls are not automatically answered during data mode. |

## S4 Command: Set Response Formatting Character

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Read command Syntax ATS4? | Response <n> OK |
| Write command Syntax ATS4=<n> | Response OK Parameter <n> 10 Response formatting character <LF>: line feed |
| Notes | This parameter determines the character recognized by TA to terminate answer line (10 = <LF> by default); it cannot be changed. |

## S7 Command: Set Delay for Connection Completion

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Read command Syntax ATS7? | Response <n> OK |
| Write command Syntax ATS7=<n> | Response OK Parameter <n> 1 – 255 Number of seconds to wait for connection completion |

## V Command: TA Response Format

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax ATV[value] | Response In case of information responses the format is: for V0: <text><CR><LF> for V1: <CR><LF><text><CR><LF> In case of result codes the format is: for V0: <numeric code><CR> for V1: <CR><LF><verbose code><CR><LF> or +CME ERROR: <err> Parameter <value> 0 Short result code format: <numeric code> 1 Long result code format: <verbose code> |
| Notes | <n> is saved in non-volatile memory per AT port over module reboot. |

## X Command: Result Code Selection and Call Progress Monitoring Control

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Write command Syntax ATX[<value>] | Response OK or +CME ERROR: <err> Parameter <value> 0 or omitted CONNECT result code only returned, dial tone and busy detection are both disabled 1 CONNECT<text> result code only returned, dial tone and busy detection are both disabled 2 CONNECT<text> result code returned, dial tone detection is enabled, busy detection is disabled 3 CONNECT<text> result code returned, dial tone detection is disabled, busy detection is enabled 4 CONNECT<text> result code returned, dial tone and busy detection are both enabled |
| Notes | - This command defines the result code to be returned, as well as sets the dial tone or busy detection features. - <value> is saved in non-volatile memory per AT port over module reboot. |

## &C Command: Set Data Carrier Detect (DCD) Function Mode

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax AT&C<value> | Response OK Parameter <value> 0 DCD line is always active 1 DCD line is active in the presence of data carrier only |
| Reference V.25Ter | Notes DCD/AT&C is only applicable to the USB AT port; it has no effect on UART1. |

## &D Command: Set Data Terminal Ready (DTR) Function Mode

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax AT&D<value> | Response OK or +CME ERROR: <err> Parameter <value> 0 TA ignores status on DTR 1 DTR drops from active to inactive. Change to command mode while retaining the connected data call 2 DTR drops from active to inactive. Disconnect data call, change to command mode. Auto-answer is off during DTR inactive state |
| Reference V.25Ter | Notes - This command only applies to data calls. - DTR/AT&D is only applicable to the USB AT port; it has no effect on UART1. |

## &F Command: Restore Factory Settings

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax AT&F[<value>] | Response OK Parameter <value> 0 or Omitted Restore STORED PROFILE 0 and 1 to factory settings |
| Reference V.25Ter | Notes This command also restores the factory settings to the active profile. |
| Examples | AT&F OK AT&F0 OK AT&F1 ERROR |

## &W Command: Save Stored Profile

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax AT&W[<value>] | Response OK Parameters <value> 0 or Omitted Save in STORED PROFILE 0 1 Save in STORED PROFILE 1 |
| Reference V.25Ter | Notes - This command saves the current configuration in a non-erasable place. - &R, S05, S06 and S08 have no effect. Their parameters are not saved in non-volatile memory. |
| Examples | AT&W // Save current configuration to Profile 0 OK AT&W0 // Save current configuration to Profile 0 OK AT&W1 // Save current configuration to Profile 1 OK |

## &V Command: Display Current Configuration

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax AT&V[<value>] | Response ACTIVE PROFILE: <current configuration> STORED PROFILE 0: <user default configuration> STORED PROFILE 1: <manufacturer configuration> OK Parameter <value> 0 Profile number This command indicates the result of certain actions as shown below: Active Profile Stored profile Default Settings ATZ AT&F AT&W |
| Reference Sierra Wireless Proprietary | Notes - At startup, the latest profile stored with AT&W is restored to the Active profile (no restoration if AT&W has not been used). - The configuration is a text string on multiple lines as shown in the example below. This string may vary depending on the manufacturer, the product and the user setup. - Registers S05, S06 and S08 have no effect. They are only implemented for compliance with V.25ter. Their parameters are always read as values 8, 2 and 2 respectively. - &R has no effect and it is not defined in the V.25ter specification. It is only implemented here for compatibility purposes. Its parameter is always read as 1. |
| Example | AT&V ACTIVE PROFILE: E1 Q0 V1 X0 &C1 &D1 &R:1 &S0 &K0 +IPR:115200 +FCLASS0 S00:0 S01:0 S04:10 S05:8 S06:2 S07:255 S08:2 STORED PROFILE 0: E1 Q0 V1 X0 &C1 &D1 &R:1 &S0 &K3 +IPR:115200 +FCLASS0 S00:0 S01:0 S04:10 S05:8 S06:2 S07:255 S08:2 STORED PROFILE 1: E1 Q0 V1 X0 &C1 &D1 &R:1 &S0 &K3 +IPR:115200 +FCLASS0 S00:0 S01:0 S04:10 S05:8 S06:2 S07:255 S08:2 OK |

## &K Command: Flow Control Option

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax AT&K[<mode>] | Response OK Parameter <mode> 0 or omitted Disable all flow control 3 Enable bi-directional hardware flow control |
| Reference V.25ter | Notes - Use AT&V0 to display the current flow control setting. - Sierra Wireless recommends the use of hardware flow control. - AT&K3 hardware flow control is only effective for UART1 and +KSLEEP=2 (UART always ON); it has no effect on the USB AT port. - This command is not supported in MUX mode; flow control is not supported in the DLC channel. |

## &S Command: DSR Option

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Write command Syntax AT&S [<override>] | Response OK Parameter <override> 0 or omitted DSR signal is always ON (0 is the default value) 1 DSR signal is always OFF |
| Reference V.25ter | Notes This is a dummy command and has no effect on the DSR signal. |

## IPR Command: Set Fixed Local/DTE Rate

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+IPR=? | Response With Autobaud: +IPR: (list of supported auto detectable <baud_rate>s)[,(list of fixed only <baud_rate>s)] OK Without Autobaud: +IPR: ()[,(list of fixed only <baud_rate>s)] OK |
| Read command Syntax AT+IPR? | Response +IPR: <baud_rate> OK |
| Write command Syntax AT+IPR= <baud_rate> | Response OK or +CME ERROR: <err> Parameter <baud_rate> 115200 (default value) For HL7618, HL7618RD, HL7648, HL7688, HL7690 and HL7692: 300, 1200, 2400, 4800, 9600, 19200, 38400, 57600, 230400, 460800, 500000, 750000, 921600, 1843200, 3000000, 3250000, 6000000 For HL7650: 0 (autobaud), 300, 1200, 2400, 4800, 9600, 19200, 38400, 57600, 230400, 460800, 500000, 750000, 921600, 1843200, 3000000, 3250000, 3686400, 4000000, 6000000 |
| Notes | - Not all listed rates may be available as they depend on the target. - The full range of data rate values may be reduced depending on hardware or other criteria. - <baud_rate> is saved in non-volatile memory per AT port over module reboot when AT&W is entered. It is not automatically saved after a +IPR write command. - After autobaud is activated on a UART port, the next AT command can be entered at a different speed. +IPR is then set to the speed of this command. Autobaud is then automatically deactivated. - When autobaud is activated on a USB COM port, any speed provided by the USB driver is accepted. AT+IPR? responds with +IPR: 0 regardless of USB speed used. |

## L Command: Monitor Speaker Loudness

For HL7648 and HL7688 only.

| HL7648 and HL7688 |  |
| --- | --- |
| Write command Syntax ATL [<volume>] | Response OK Parameter <volume> 0 – 9 |
| Notes | The responses of this command are compliant with the recommendation but this command has no effect. |

## M Command: Monitor Speaker Mode

For HL7648 and HL7688 only.

| HL7648 and HL7688 |  |
| --- | --- |
| Write command Syntax ATM[<mode>] | Response OK Parameter <mode> 0 – 65535 |
| Notes | The responses of this command are compliant with the recommendation but this command has no effect. |

# General AT Commands

## I Command: Request Identification Information

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax ATI[<value>] | Response If <value> = 0 or omitted: <model> OK If <value> = 1: <short version name> OK If <value> = 3: <version name> OK If <value> = 4: <fuse state> OK If <value> = 9: <version name> <model> <short version name> <chipset> <fuse state> <build date & time> <source rev> OK If <value> = 10: Modem-Firmware: <version name> <model> <short version name> <chipset> <fuse state> <build date & time> <source rev> Primary-Boot: <version name> <build date & time> <source rev> Secondary-Boot: <version name> <build date & time> <source rev> Update-Agent: <version name> <build date & time> <source rev> 4G-Firmware: <4G FW version name> 3G-Firmware: <3G FW version name> OK Parameters <model> Model identifier <version name> Firmware version string HL7618, HL7618RD, HL7688, HL7690 and HL7692 follow the format: AHL75xx_TEST.0.0.141506 <…> (test firmware) AHL75xx.1.0.141506.<…> (official firmware) HL7648 and HL7650 follow the format: <prefix>.<major no.>.<minor no.>.<baseline no.>.<date-time>.<integration> of size: <variable, up to 32 characters>.<2digits>.<2digits >.<6digits >.<12digits>.<2digits> Note that test firmware will have <major no.>.<minor no.> = 00.00. For example: SWIMCB71XX-G.00.00.163500.201609231719.01 (test firmware) SWIMCB71XX-G.01.00.163500.201609231719.01 (official firmware) <short version name> Firmware version string in short format (without date and time) For example: HL75xx_TEST.0.0 (test firmware) HL75xx.1.0 (official firmware) <4G FW version name> 4G Firmware version string <3G FW version name> 3G Firmware version string <chipset> Chipset name <build date & time> Firmware build time in format YYYY-MM-DD HH:MM:SS <source rev> Source code revision in version control <fuse state> Fuse state information FUSED Fused module with secure boot NON-FUSED Non-fused module |
| Reference V.25ter | Notes - ATI3 is identical to AT+GMR and AT+CGMR. - ATI0 and ATI are identical to AT+GMM and AT+CGMM. |
| Examples | ATI HL7618 //When using an HL7618 module OK ATI0 HL7618 //When using an HL7618 module OK # For fused modules ATI4 FUSED OK # For non-fused modules ATI4 NON-FUSED OK # Examples on a test firmware for HL7648/HL7650 ATI1 SWIMCB71XX-G.00.00.163500 OK ATI3 SWIMCB71XX-G.00.00.163500.201609261356.01 OK # Example of a test firmware with TEST as the version name ATI3 BHL7618_TEST.0.0.154401.201511132200.x7120_2 OK # Examples on official firmware ATI1 HL7618.3.0 //When using an HL7618 module OK ATI3 BHL7618.3.0.154401.201511132200.x7120_2 OK |

## Z Command: Reset and Restore User Configuration

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax ATZ[<value>] | Response OK or +CME ERROR: <err> Parameter <value> 0 Reset and restore user configuration with profile 0 1 Reset and restore user configuration with profile 1 |

## +CGMI Command: Request Manufacturer Identification

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGMI=? | Response OK |
| Execute command Syntax AT+CGMI | Response (manufacturer identification text) OK |
| Reference [27.007] § 5.1 | Note This command is identical to AT+GMI. |
| Example | AT+CGMI Sierra Wireless OK |

## +CGMM Command: Request Model Identification

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGMM=? | Response OK |
| Execute command Syntax AT+CGMM | Response <mode> OK Parameter <model> Model identifier |
| Reference [27.007] § 5.2 | Note This command is identical to AT+GMM, ATI and ATI0. |
| Example | AT+CGMM HL7618 //When using an HL7618 module OK |

## +CGMR Command: Request Revision Identification

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGMR=? | Response OK |
| Execute command Syntax AT+CGMR | Response (model revision identification text) OK |
| Reference [27.007] § 5.3 | Note This command is identical to ATI3 and AT+GMR. |
| Examples | AT+CGMR AHL7618_TEST.0.0.153200.201508220500.x7120_1 // test HL7618 firmware OK AT+CGMR AHL7618.1.0.153200.201508220500.x7120_1 // official HL7618 firmware OK |

## +CGSN Command: Request Product Serial Number Identification (IMEI)

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGSN=? | Response OK |
| Execute command Syntax AT+CGSN | Response <IMEI> (identification text for determination of the individual ME) OK |
| Reference V.25ter | Notes - This command is identical to AT+GSN. - This command can work with or without a SIM. - See also AT+KGSN. |

## +KGSN Command: Request Product Serial Number and Software Version

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KGSN=? | Response +KGSN: (list of supported <number type>s) OK |
| Write command Syntax AT+KGSN= <number type> | Response If <number type> = 0: +KGSN: <IMEI> OK If <number type> = 1: +KGSN: <IMEISV> OK If <number type> = 2: +KGSN: <IMEISV_STR> OK If <number type> = 3: +KGSN: <FSN> OK If <number type> = 4: +KGSN: <FSN-BB> OK Parameters <IMEI> 15 digits IMEI (8 digits for TAC + 6 digits for SNR + 1 check digit) <IMEISV> 16 digits IMEISV (8 digits for TAC + 6 digits for SNR + 2 SVN digits) <IMEISV_STR> Formatted string; <15 digits>-<Check digit> SV: <Software version> <FSN> 14 digits Serial Number <FSN-BB> 16 digits Serial Number + BB |
| Reference Sierra Wireless Proprietary | Notes This command has been developed to provide the IMEI SV and Serial Number through an AT command and it can work without a SIM. |
| Examples | AT+KGSN=0 +KGSN: 351578000023006 OK AT+KGSN=1 +KGSN: 3515780000230001 OK AT+KGSN=2 +KGSN: 35157800002300-6 SV:01 OK AT+KGSN=3 +KGSN: 0123456789ABCD OK AT+KGSN=4 +KGSN: 0123456789ABCD01 OK |

## +HWREV Command: Request Hardware Revision

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+HWREV=? | Response OK |
| Read command Syntax AT+HWREV? | Response Hardware revision: X.Y OK Parameter X.Y These are the HH numbers in FSN (returned by TTYWWDNNNNPPHH-BB) |
| Reference Sierra Wireless Proprietary | Notes This command works with or without a SIM. |
| Example | Assuming FSN=TTYWWDNNNNPP01-BB AT+HWREV? Hardware revision: 0.1 OK |

## +CSCS Command: Set TE Character Set

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CSCS=? | Response +CSCS: (list of supported <vail>s) OK |
| Read command Syntax AT+CSCS? | Response +CSCS: <vail> OK or +CME ERROR: <err> |
| Write command Syntax AT+CSCS= [<vail>] | Response OK or +CME ERROR: <err> Parameter <vail> "GSM" GSM default alphabet (3GPP TS 23.038) "HEX" Character strings only consist of hexadecimal numbers from 00 to FF. For example, “032FE6” equals three 8-bit characters with decimal values 3, 47 and 230. No converstions to the original MT character set shall be done "IRA" International reference alphabet (ITU-T T.50) "UCS2" 16-bit universal multiple-octet coded character set (ISO/IEC 10646) |
| Notes | <vail> is saved in non-volatile memory per AT port over module reboot. |

## +CIMI Command: Request International Mobile Subscriber Identity

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CIMI=? | Response OK |
| Execute command Syntax AT+CIMI | Response <IMSI> OK or +CME ERROR: <err> Parameter <IMSI> International Mobile Subscriber Identity |

## +GMI Command: Request Manufacturer Identification

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+GMI=? | Response OK |
| Execute command Syntax AT+GMI | Response (manufacturer identification text) OK |
| Reference [27.007] § 5.1 | Note This command is identical to AT+CGMI. |
| Example | AT+GMI Sierra Wireless OK |

## +GMM Command: Request Model Identification

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+GMM=? | Response OK |
| Execute command Syntax AT+GMM | Response <model> OK Parameter <mode> Model identifier |
| Reference [27.007] § 5.2 | Note This command is identical to AT+CGMM, ATI and ATI0. |
| Example | AT+GMM HL7618 //When using an HL7618 module OK |

## +GMR Command: Request Revision Identification

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+GMR=? | Response OK |
| Execute command Syntax AT+GMR | Response (model revision identification text) OK |
| Reference [27.007] § 5.3 | Note This command is identical to ATI3 and AT+CGMR. |
| Examples | AT+CGMR AHL7618_TEST.0.0.153200.201508220500.x7120_1 // test HL7618 firmware OK AT+CGMR AHL7618.1.0.153200.201508220500.x7120_1 // official HL7618 firmware OK |

## +GSN Command: Request Product Serial Number (IMEI)

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+GSN=? | Response OK |
| Execute command Syntax AT+GSN | Response <IMEI> (identification text for determination of the individual ME) OK |
| Reference V.25ter | Notes - This command is identical to AT+CGSN. - This command can work with or without a SIM. - See also AT+KGSN. |

## +CMUX Command: Multiplexing Mode

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CMUX=? | Response +CMUX: (list of supported <mode>s),(list of supported <subset>s),(list of supported <port_speed>s),(list of supported <N1>s),(list of supported <T1>s),(list of supported <N2>s),(list of supported <T2>s),(list of supported <T3>s),(list of supported <k>s) OK |
| Read command Syntax AT+CMUX? | Response +CMUX: <mode>,<subset>,<port_speed>,<N1>,<T1>,<N2>,<T2>,<T3>,<k> OK or +CME ERROR: <err> OK |
| Write command Syntax AT+CMUX= <mode> [,<subset> [,<port_speed> [,<N1>[,<T1> [,<N2>[,<T2> [,<T3>[,<k>]]]]]]]] | Response OK or +CME ERROR: <err> OK Parameters <mode> Multiplexer transparency mechanism 0 Basic option 1 Advanced option (not supported) <subset> 0 UIH frames used only 1 UI frames used only (not supported) 2 I frames used only (not supported) <port_speed> Transmission rate 1 9 600 bit/s 2 19 200 bit/s 3 38 400 bit/s 4 57 600 bit/s 5 115 200 bit/s 6 230 400 bit/s 7 1 Mbit/s <N1> 1 – 1509 Maximum frame size. Default value = 31 (64 if Advanced option is used) <T1> 1 – 255 Acknowledgement time in units of ten milliseconds. Default value = 10 (100 ms) <N2> 0 – 5 Maximum number of re-transmissions. Default value = 3 <T2> 2 – 255 Response time for the multiplexer control channel in units of ten milliseconds. Default value = 30 (300 ms). Note that <T2> must be longer than <T1>. <T3> 1 – 255 Wake up response timer in seconds. Currently not supported; in case of read command, 0 is returned. <k> 1 – 7 Window size for advanced operation with error recovery options. Currently not supported; in case of read command, 0 is returned. |
| Notes | - This command enables the multiplexing protocol control channel as defined in GSM07.10. - The AT command sets parameters for the Control Channel. If parameters are left out the default values are used. If no autobauding is supported, a customer related interface speed is pre-selected. The final response code OK or CME ERROR: <err> is returned using the old interface speed; the parameters become active only after sending OK. - The "+++" escape sequence is not supported in the DLC port in CMUX mode. Alternatively, DTR can be used to switch from data mode to command mode, or use another DLC port to send AT commands. - The module handles the frame data step by step in CMUX mode. If there are any wrong data in the frame, e.g. wrong CRC, nothing will be returned to the terminal, and the module will wait for a valid frame data. - If the AT+CFUN command is entered with <rst>=1, all open CMUX channels will be closed and the module will reset. - There is no activity timeout to return to AT mode after entering MUX mode. - MUX DLC ports are not persistent over power cycles. After a power cycle, DLC ports need to be re-established. - When an established MT call is hanged up from the caller side, NO CARRIER will only be sent to the port on which the call was established (i.e. the port on which ATD/ATA was sent). |

## +GCAP Command: Request Complete TA Capability List

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax AT+GCAP | Response +GCAP: list of <name>s OK |
| Example | +GCAP:+FCLASS,+CGSM OK |

## +WIMEI Command: IMEI Write and Read

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+WIMEI=? | Response OK |
| Read command Syntax AT+WIMEI? | Response +WIMEI: <IMEI> OK |
| Write command Syntax AT+WIMEI= <IMEI> | Response +WIMEI: <IMEI> OK Parameter <IMEI> 14 or 15-digit IMEI as defined in GSM 23.003 |
| Notes | - The default IMEI is 012345678901237. - The write command can only be used once for IMEI programming. - The IMEI to be written must be different from the default IMEI. - If a 14-digit IMEI is entered, the 15th checksum digit is automatically calculated. - The NV backup of the static calibrated NV partition which stores the IMEI is automatically updated after successfully executing the write command (i.e. backup is updated when OK is returned). |
| Examples | at+wimei? +WIMEI: 012345478901237 // Default IMEI OK at+wimei=354610060035829 // Enter 15-digit IMEI OK at+wimei? +WIMEI: 354610060035829 OK at+wimei=35461006003582 // Enter 14digit IMEI OK at+wimei? +WIMEI: 354610060035829 OK |

## +KODIS Command: Access ODIS Information

For HL7648 and HL7688 only.

| HL7648 and HL7688 |  |
| --- | --- |
| Test command Syntax AT+KODIS=? | Response OK |
| Read command Syntax AT+KODIS? | Response +KODIS: <index>,“<hostMan>”,“<hostMod>”,“<hostSwV>”,“<hostID>” OK |
| Write command Syntax AT+KODIS= <index>, <hostMan>, <hostMod>, <hostSwV>, <hostID> | Response OK or +CME ERROR: <err> Parameters <index> Index number of the following parameters <hostMan> Host manufacturer of ODIS node (ATT) <hostMod> Host model of ODIS node (ATT) <hostSwV> Host software version of ODIS node (ATT) <hostID> Host ID of ODIS node (ATT) |
| Reference Sierra Wireless Proprietary | Notes - This command is used for modifying host device details required by specific ODIS test cases in AT&T. - The maximum number of characters in the parameters listed above is 31. Characters beyond the maximum limit will be ignored. |
| Examples | at+kodis? +KODIS: 1,"HMAN1","HMOD1","HSW1","HUID1" OK at+kodis=1,"HostMan","HostMode","01.00","HostID" OK at+kodis? +KODIS: 1,"HostMan","HostMode","01.00","HostID" OK |

## &R Command: RTS and CTS Option

For HL7618, HL7618RD, HL7648, HL7688, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7648, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Write command Syntax AT&R <option> | Response OK Parameter <option> 1 In sync mode, CTS is always ON (RTS transitions are ignored). In async mode, CTS will only drop if required by the flow control |
| Notes | This command has no effect and is not defined in the V.25ter specification; it was only implemented for compatibility reasons. Parameters are ignored and are not saved in non-volatile memory. |

## +FMI Command: Request Manufacturer Identification

For HL7648 and HL7688 only.

| HL7648 and HL7688 |  |
| --- | --- |
| Test command Syntax AT+FMI=? | Response OK |
| Execute command Syntax AT+FMI | Response (manufacturer identification text) OK |
| Reference [27.007] § 5.1 | Example AT+FMI Sierra Wireless OK |

## +FMM Command: Request Model Identification

For HL7648 and HL7688 only.

| HL7648 and HL7688 |  |
| --- | --- |
| Test command Syntax AT+FMM=? | Response OK |
| Execute command Syntax AT+FMM | Response <model> OK Parameter <model> Model identifier |
| Reference [27.007] § 5.2 | Example AT+FMM HL7688 OK |

## \N Command: Data Transmission Mode

For HL7648, HL7688, HL7690 and HL7692 only.

| HL7648, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax AT\N<x> | Response OK Parameter <x> 0 Transparent mode 4, 6 RLP mode (non-transparent) |
| Notes | This command has no effect and is not defined in the V.25ter specification; it was only implemented for compatibility reasons. Parameters are ignored and are not saved in non-volatile memory. |

## N Command: Negotiate Handshake Option

For HL7648, HL7688, HL7690 and HL7692 only.

| HL7648, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax ATN[<option>] | Response OK Parameter <option> 0 – 9 |
| Notes | This command has no effect and is not defined in the V.25ter specification; it was only implemented for compatibility reasons. Parameters are ignored and are not saved in non-volatile memory. |

## S5 Command: Write Command Line Editing Character

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Read command Syntax ATS5? | Response <n> OK |
| Write command Syntax ATS5=<n> | Response OK Parameters <n> 8 Only 8 (backspace) is supported |
| Reference V.25Ter | Notes This command has no effect and was only implemented to comply with V.25ter. Parameters are ignored and are not saved in non-volatile memory. |

## S6 Command: Pause before Blind Dialing

For HL7618, HL7618RD, HL7648, HL7688, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7648, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Write command Syntax ATS6=<time> | Response OK Parameters <time> 0 – 999 |
| Reference V.25ter | Notes This command has no effect and was only implemented to comply with V.25ter. Parameters are ignored and are not saved in non-volatile memory. |

## S8 Command: Comma Dial Modifier Time

For HL7618, HL7618RD, HL7648, HL7688, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7648, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Read command Syntax ATS8? | Response <time> OK |
| Write command Syntax ATS8=<time> | Response OK Parameters <time> 0 – 255 |
| Reference V.25ter | Notes This command has no effect and was only implemented to comply with V.25ter. Parameters are ignored and are not saved in non-volatile memory. |

## W Command: Extended Result Code

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Write command Syntax ATW<mode> | Response OK Parameter <mode> 0 or Omitted Only CONNECT will be shown 1 CONNECT<connection speed> will be shown |
| Notes | This command has no effect and is not defined in the V.25ter specification; it was only implemented for compatibility reasons. Parameters are ignored and are not saved in non-volatile memory. |

## B Command: Data Rate Selection

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax ATB<rate> | Response OK Parameter <rate> 0 – 99 Data rate |
| Reference V.25Ter | Notes - This command has no effect and was only implemented for compliance with V.25ter. Parameters are ignored and are not saved in non-volatile memory. - Refer to AT+CBST regarding CSD data rate. (CSD is not applicable to the HL7650.) |

## S2 Command: Set Character for the Escape Sequence (Data to Command Mode)

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Read command Syntax ATS2? | Response <n> OK |
| Write command Syntax ATS2=<n> | Response OK Parameter <n> Only 43 ("+") is supported |
| Reference V.25Ter | Notes - This command has no effect and was only implemented for compliance with V.25ter. Parameters are ignored and are not saved in non-volatile memory - <n> will not be shown by the AT&V command. |

## S3 Command: Command Line Termination Character

For HL7618, HL7618RD, HL7648, HL7650, HL7688 and HL7690 only.

| HL7618, HL7618RD, HL7648, HL7650, HL7688 and HL7690 |  |
| --- | --- |
| Read command Syntax ATS3? | Response <n> OK |
| Write command Syntax ATS3=<n> | Response OK Parameters <n> 13 Command line termination character <CR>: carriage return |
| Reference V.25Ter | Notes This command has no effect and was only implemented for compliance with V.25ter. Parameters are ignored and are not saved in non-volatile memory. |

## S10 Command: Automatic Disconnect Delay

For HL7618, HL7618RD, HL7648, HL7650, HL7688 and HL7690 only.

| HL7618, HL7618RD, HL7648, HL7650, HL7688 and HL7690 |  |
| --- | --- |
| Read command Syntax ATS10? | Response <time> OK |
| Write command Syntax ATS10=<time> | Response OK Parameter <time> 1 – 254 Number of tenths of a second of delay |
| Reference V.25Ter | Notes This command has no effect and was only implemented for compliance with V.25ter. Parameters are ignored and are not saved in non-volatile memory. |

## S11 Command: DTMF Dialing Speed

For HL7618, HL7618RD, HL7648, HL7650, HL7688 and HL7690 only.

| HL7618, HL7618RD, HL7648, HL7650, HL7688 and HL7690 |  |
| --- | --- |
| Write command Syntax ATS11=<time> | Response OK Parameter <time> 0 – 999 |
| Reference V.25Ter | Notes This command has no effect and was only implemented for compliance with V.25ter. Parameters are ignored and are not saved in non-volatile memory. |

# Call Control Commands

## D Command: Dial Number

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax ATD=? | Response 1 2 3 4 5 6 7 8 9 0 * # + A B C D P T W , @ ! OK |
| Read command Syntax ATD? | Response 1 2 3 4 5 6 7 8 9 0 * # + A B C D P T W , @ ! OK |
| Execute command Syntax ATD[<n>] | Response OK If successfully connected CONNECT Connection has been established RING The DCE has detected an incoming call signal from the network NO CARRIER The connection cannot be established BUSY Engaged (busy) signal detected NO ANSWER If no hang up is detected after a fixed network timeout CONNECT <data rate> Same as CONNECT but includes the data rate RING CTM The MS has detected an incoming CTM call signal from the network; this code is proprietary CONNECT FAX Same as CONNECT but includes the indication related to a fax call Parameter <n> String of dialing digits and optionally V.25ter modifiers (dialing digits): 0-9, * , #, +, A, B, C, D, P, T, W, ,, @, ! (maximum length: 20 digits) |
| Reference V.25Ter | Notes - This command may generally be aborted when receiving an ATH command during execution. - Response “OK” may arrive just after the ATD command or after the call is active (see AT+COLP). - <n> is ignored when it is set to ",", "T", "!", "W" or "@" - When an established MT call is hanged up from the caller side, NO CARRIER will only be sent to the port on which the call was established (i.e. the port on which ATD was sent). |
| Examples | ATD*99***3# CONNECT ~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~ NO CARRIER ATD=? 1 2 3 4 5 6 7 8 9 0 * # + A B C D P T W , @ ! OK |

## D> Command: Direct Dialing from Phonebook

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax ATD><str> ATD>[<mem>] <n> | Response See ATD Parameters <str> Alphanumeric field (if possible all available memories should be searched for correct entry) <mem> Memory storage ("ME", "SM", etc.) <n> Entry location |
| Notes | For memory storage locations, see AT+CPBS. |

## +CHUP Command: Hang up Call

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CHUP=? | Response OK |
| Execute command Syntax AT+CHUP | Response OK or +CME ERROR: <err> |
| Notes | This command hangs up waiting/active MT calls and MO calls. |

## +CR Command: Service Reporting Control

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CR=? | Response +CR: (list of supported <mode>s) OK |
| Read command Syntax AT+CR? | Response +CR: <mode> OK |
| Write command Syntax AT+CR= [<mode>] | Response OK or +CME ERROR: <err> Parameters <mode> 0 Disables reporting 1 Enables reporting <serv> ASYNC Asynchronous transparent SYNC Synchronous transparent REL ASYNC Asynchronous non-transparent REL SYNC Synchronous non-transparent GPRS [<L2P>] GPRS |
| Notes | The optional <L2P> proposes a layer 2 protocol to use between the MT and the TE. It is defined in +CGDATA command. |

## +CRC Command: Set Cellular Result Codes for Incoming Call Indication

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CRC=? | Response +CRC: (list of supported <mode>s) OK |
| Read command Syntax AT+CRC? | Response +CRC:<mode> OK |
| Write command Syntax AT+CRC= [<mode>] | Response OK or +CME ERROR: <err> Parameter <mode> 0 Disable extended format 1 Enable extended format |
| Unsolicited Notification | Response +CRING: <type> Paramerter <type> ASYNC [,<priority>[,<subaddr>,<satype>]] Asynchronous transparent SYNC [,<priority>[,<subaddr>,<satype>]] Synchronous transparent REL ASYNC [,<priority>[,<subaddr>,<satype>]] Asynchronous non transparent REL SYNC [,<priority>[,<subaddr>,<satype>]] Synchronous non transparent CTM [,<priority>[,<subaddr>,<satype>]] Incoming CTM call CTM2 [,<priority>[,<subaddr>,<satype>]] Incoming CTM call at line 2 GPRS <PDP_type>, <PDP_addr>[, [<L2P>][,<APN>]] GPRS network request for PDP context activation <priority> (Optional) EMLPP priority level of the incoming call by paging, notification or setup message. <subaddr> String type subaddress of format specified by <satype> <satype> Type of subaddress octet in integer format <PDP_type>, <PDP_addr>, <APN> As defined in AT+CGDCONT command <L2P> (Optional) proposes a layer 2 protocol to use between the MT and the TE. |

## +CSTA Command: Select Type of Address

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CSTA=? | Response +CSTA: (list of supported <type>s) OK |
| Read command Syntax AT+CSTA? | Response +CSTA: <type> OK |
| Write command Syntax AT+CSTA= <type> | Response OK or +CME ERROR: <err> Parameter <type> 129 Dial string begins with a digit, or is a local number 145 Dial string includes international access code character "+" |
| Notes | <type> is saved in non-volatile memory over module reboot. |

## +CMOD Command: Call Mode

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CMOD=? | Response +CMOD: (list of supported <mode>s) OK |
| Read command Syntax AT+CMOD? | Response +CMOD: <mode> OK |
| Write command Syntax AT+CMOD= [<mode] | Response OK or +CME ERROR: <err> Parameter <mode> 0 Single mode |

## +CEER Command: Extended Error Report

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CEER=? | Response OK |
| Write command Syntax AT+CEER | Response +CEER: <category>[,<cause>,<descriptions>] OK Parameters <category> "No report available" "CC setup error" "CC modification error" "CC release" "SM attach error" "SM detach" "SM activation error" "SM deactivation" "SS network error cause" "SS network reject cause" "SS network GSM cause" <cause> Digit representing the error cause sent internally or by the network. Refer to 18.2.2 CEER Error Codes for more information. <description> Verbose string containing the textual representation of <cause>. Refer to 18.2.2 CEER Error Codes for more information. |

## +CSNS Command: Single Numbering Scheme

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CSNS=? | Response +CSNS: (list of supported <mode>) OK |
| Read command Syntax AT+CSNS? | Response +CSNS:<mode> OK |
| Write command Syntax AT+CSNS= [<mode>] | Response OK Parameter <mode> 4 Data |

## +CBST Command: Select Bearer Service Type

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CBST=? | Response +CBST: (list of supported <speed>s),(list of supported <name>s),(list of supported <ce>s) OK |
| Read command Syntax AT+CBST? | Response +CBST: <speed>,<name>,<ce> OK |
| Write command Syntax AT+CBST= [<speed> [,<name>[,<ce>]]] | Response OK or CME ERROR: <err> Parameters <speed> 0 Autobauding (automatic selection of the speed; this setting is possible in case of 3.1 kHz modem and non-transparent service) 4 2400 bps (V.22bis) 5 2400 bps (V.26ter) 6 4800 bps (V.32) 7 9600 bps (V.32) 12 9600 bps (V.34) 14 14400 bps (V.34) 15 19200 bps (V.34) 16 28800 bps (V.34) 17 33600 bps (V.34) 39 9600 bps (V.120) 43 14400 bps (V.120) 47 19200 bps (V.120) 48 28800 bps (V.120) 49 38400 bps (V.120) 50 48000 bps (V.120) 51 56000 bps (V.120) 68 2400 bps (V.110 or X.31 flag stuffing) 70 4800 bps (V.110 or X.31 flag stuffing) 71 9600 bps (V.110 or X.31 flag stuffing) 75 14400 bps (V.110 or X.31 flag stuffing) 79 19200 bps (V.110 or X.31 flag stuffing) 80 28800 bps (V.110 or X.31 flag stuffing) 81 38400 bps (V.110 or X.31 flag stuffing) 82 48000 bps (V.110 or X.31 flag stuffing) 83 56000 bps (V.110 or X.31 flag stuffing; this setting can be used in conjunction with asynchronous non-transparent UDI or RDI service to get FTM) 84 64000 bps (X.31 flag stuffing; this setting can be used in conjunction with asynchronous non-transparent UDI service to get FTM) 115 56000 bps (bit transparent) 116 64000 bps (bit transparent) 120 32000 bps (PIAFS32k) 121 64000 bps (PIAFS64k) 130 28800 bps (multimedia) 131 32000 bps (multimedia) 132 33600 bps (multimedia) 133 56000 bps (multimedia) 134 64000 bps (multimedia) <name> 0 Data circuit asynchronous (UDI or 3.1 kHz modem) 1 Data circuit synchronous (UDI or 3.1 kHz modem) 4 Data circuit asynchronous (RDI) 5 Data circuit synchronous (RDI) <ce> 0 Transparent 1 Non-transparent 2 Both, transparent preferred 3 Both, non-transparent preferred |

# Mobile Equipment Control and Status Commands

## +CAMM Command: Accumulated Call Meter (Maximum)

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CAMM=? | Response OK |
| Read command Syntax AT+CAMM? | Response +CAMM: <acmmax> OK |
| Write command Syntax AT+CAMM= [<acmmax> [,<passwd>]] | Response OK or +CME ERROR: <err> Parameters <acmmax> String type containing the accumulated call meter maximum value coded in hexadecimal format. Value 0 disables the ACMmax feature <passwd> SIM PIN2 |

## +CCWE Command: Call Meter Maximum Event

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CCWE=? | Response +CCWE: (list of supported <mode>s) OK |
| Read command Syntax AT+CCWE? | Response +CCWE: <mode> OK |
| Write command Syntax AT+CCWE= <mode> | Response OK or +CME ERROR: <err> Parameter <mode> 0 Disable the call meter warning event 1 Enable the call meter warning event |

## +CCLK Command: Real Time Clock

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CCLK=? | Response OK |
| Read command Syntax AT+CCLK? | Response +CCLK: <time> or +CME ERROR: <err> |
| Write command Syntax AT+CCLK= <time> | Response OK or +CME ERROR: <err> Parameter <time> String type value; format is "yy/MM/dd,hh:mm:ss+/-TZ", where characters indicate year (last two digits), month, day, hour, minutes, seconds and time zone (optional). |
| Notes | Year must be 2004 or later. |

## +CIND Command: Indicator Control

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CIND=? | Response +CIND: ("battchg",(0-5)),("signal",(0-5)),("service",(0-1)),("message",(0-1)), ("call",(0-1)),("roam",(0-1)),("smsfull",(0-1)) OK |
| Read command Syntax AT+CIND? | Response +CIND: <battchg>,<signal>,<service>,<call>,<smsfull> OK Parameters <battchg> 0 – 5 Battery level 0 Lowest level 4 Highest level 5 Battery is charging (not supported) <signal> 0 – 5 Signal quality level 0 Lowest level signal 5 Highest level signal <service> Network service availability 0 Network service is not available 1 Network service is available <message> Message reception 0 No message is received 1 Message is received <call> Calling in progress 0 Service is not available 1 Service is available <roam> Roaming indicator 0 Home network 1 Roaming <smsfull> SMS memory storage 0 Memory available 1 Memory full |
| Reference Sierra Wireless Proprietary | Notes - This command can be used without a SIM. - <message> and <smsfull> are only supported for memory 3 with “SM” and “ME” storage type. If a different storage type is used with memory 3, <message> and <smsfull> parameters are always 0. |
| Examples | // Test command AT+CIND=? +CIND: ("battchg",(0-5)),("signal",(0-5)),("service",(0-1)),("message",(0-1)),("call",(0-1)),("roam",(0-1)),("smsfull",(0-1)) OK // Read command AT+CIND? +CIND: 0,1,1,0,0,0,0 // Indicates signal level = 1 and service is available OK |

## +CLAC Command: List Available AT Commands

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax AT+CLAC | Response <AT command 1> [<CR><LF><AT command 2>[..]] OK or +CME ERROR: <err> Parameter <AT command> AT command (including the prefix “AT”) |
| Notes | This command provides the AT Command list available for the user. |

## +CFUN Command: Set Phone Functionality

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CFUN=? | Response +CFUN: (list of supported <fun>s), (list of supported <rst>s) OK or +CME ERROR: <err> |
| Read command Syntax AT+CFUN? | Response +CFUN: <power_mode>,<STK_mode> or +CME ERROR: <err> |
| Write command Syntax AT+CFUN=<fun> [,<rst>] | Response OK or +CME ERROR: <err> Parameters <fun> 0 Switch off MS 1 Full functionality 4 Disable both phone’s transmit and receive RF circuits; airplane mode 5 Fast detach 6 Enable SIMTK and fetching of proactive commands 7 Disable SIMTK and enable fetching of proactive commands 8 Disable fetching of proactive commands 15 Perform hidden reset (reset MS without resetting SIM) 16 Simulate reset (reset MS including SIM) 27 Perform SIM reset and restore previous PIN validation state (reset SIM without resetting MS) 31 Disable single stack with an option to power off/power on single (U)SIM card 32 Disable all stacks with an option to power off/power on single (U)SIM card 33 Enable single stack with an option to reset (U)SIM card 34 Enable all stacks with an option to reset (U)SIM cards 35 Power off/power on single (U)SIM card 36 Power off/power on all (U)SIM cards 37 Reset single stack 38 Reset all stacks Note that when <fun> = 0, 15 or 16, the OK response may be missed due to race conditions, as MT may switch off by the time the OK response is triggered. <rst> Reset value If <fun> = 1 or 4: 0 Do not reset MT before resetting it to <fun> power level 1 Reset MT before setting it to <fun> power level If <fun> = 31, 32, 35 or 36: 0 SIM is switched OFF 1 SIM is switched ON If <fun> = 33 or 34: 0 SIM reset not needed 1 SIM reset needed If <fun> = 27 0 Hidden SIM reset (MS is not informed of SIM reset) 1 Normal SIM reset (MS is informed of SIM reset) <power_mode> 1 MS is switched ON 2 Invalid mode 4 Airplane mode <STK_mode> 0 Inactive state 6 Enable the SIM-toolkit interface and fetching of proactive commands by SIM-APPL from the SIM card 7 Disable the SIM-toolkit interface and enable fetching of proactive commands by SIM-APPL from the SIM card 8 Disable fetching of proactive commands by SIM-APPLU from the SIM card |

## +CMER Command: Mobile Equipment Event Reporting

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CMER=? | Response +CMER: (list of supported <mode>s),(list of supported <keyp>s),(list of supported <disp>s),(list of supported <ind>s),(list of supported <bfr>s) OK |
| Read command Syntax AT+CMER? | Response +CMER: <mode>,<keyp>,<disp>,<ind>,<bfr> OK |
| Write command Syntax AT+CMER= [<mode>[,<keyp>[,<disp>[,<ind> [,<bfr>]]]]] | Response OK or +CME ERROR: <err> Parameters <mode> 0 Buffer unsolicited result codes in the TA; if TA result code buffer is full, codes can be buffered in some other place or the oldest ones can be discarded 1 Discard unsolicited result codes when the TA-TE link is reserved (e.g. in on-line data mode); otherwise forward them directly to the TE 2 Buffer unsolicited result codes in the TA when the TA-TE link is reserved (e.g. in online data mode) and flush them to the TE after reservation; otherwise forward them directly to the TE <keyp> 0 No keypad event reporting <disp> 0 No display event reporting <ind> 0 No indicator event reporting 1 Indicator event reporting using result code +CIEV: <ind>,<value>. <ind> indicates the indicator order number (as specified for +CIND) and <value> is the new value of the indicator. Only indicator events which are not caused by +CIND shall be indicated by the TA to the TE. <bfr> 0 TA buffer of unsolicited result codes defined within this command is cleared when <mode>=1 or 2 is entered. 1 TA buffer of unsolicited result codes defined within this command is flushed to the TE when <mode>=1 or 2 is entered (OK response shall be given before flushing the codes) |
| Unsolicited Notification | Responses - +CIEV: 1,(0-5) indicates the battery charging level - +CIEV: 2,(0-5) indicates the received signal level - +CIEV: 3,(0-1) indicates the network service status - +CIEV: 4,(0-1) indicates the message status - +CIEV: 5,(0-1) indicates the active call status - +CIEV: 6,(0-1) indicates the roaming status - +CIEV: 7,(0-1) indicates the sms full status Refer to +CIND for more information regarding indicator control. |
| Reference Sierra Wireless Proprietary | Notes This command can be used without a SIM. |
| Example | at+cmer=? +CMER: (1-2),0,0,(0-1),(0-1) OK at+cmer=2,,,1 OK # mode =2: enable indication if AT link is available # ind = 1: enable indicator event report (+CIND) at+cmer? +CMER: 2,0,0,1,0 OK # +CMER setting can be preserved after boot at+cfun=1,1 OK at+cmer? +CMER: 2,0,0,1,0 OK # roaming status = 0 update on registration status change +CIEV: 6,0 +PBREADY # enable +CMER <mode> = 0 buffering at+cmer=0 OK at+cfun=4 OK at+cfun=1 OK # wait for registration, one +CIEV: 6 should be buffered, some +CGEV buffered at+creg? +CREG: 0,1 OK # buffered +CIEV is flushed with <bfr>=1 and <mode>=2 at+cmer=2,,,,1 OK +CIEV: 6,0 at+cmer=0 OK at+cfun=4 OK at+cfun=1 OK # wait for registration, one +CIEV: 6 should be buffered, some +CGEV buffered at+creg? +CREG: 0,1 OK # buffered +CIEV is cleared with <bfr>=1 and <mode>=2 at+cmer=2,,,,0 OK |

## +CMEE Command: Report Mobile Termination Error

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CMEE=? | Response +CMEE: (list of supported <n>s) OK |
| Read command Syntax AT+CMEE? | Response +CMEE: <n> OK |
| Write command Syntax AT+CMEE=[<n>] | Response OK Parameter <n> 0 Disable +CME ERROR: <err> result code and use ERROR instead 1 +CME ERROR: <err> result code and use numeric <err> values 2 +CME ERROR: <err> result code and use verbose <err> values |
| Notes | <n> is saved in non-volatile memory per AT port over module reboot. |

## +CCID Command: Request SIM Card Identification

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CCID=? | Response OK |
| Read command Syntax AT+CCID? | Response +CCID: <ICCID> OK or +CME ERROR: <err> |
| Execute command Syntax AT+CCID | Response +CCID: <ICCID> OK or +CME ERROR: <err> Parameter <ICCID> Integrated Circuit Card ID of the SIM card |

## +FMR Command: Request Revision Identification

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+FMR=? | Response OK |
| Execute command Syntax AT+FMR | Response <revision>,<sv> OK or +CME ERROR: <err> Parameters <revision> Revised version from IMEISV of the mobile station <sv> Software version from IMEISV of the mobile station |

## +CPIN Command: Enter Pin

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CPIN=? | Response OK |
| Read command Syntax AT+CPIN? | Response +CPIN: <code> OK or +CME ERROR: <err> |
| Write command Syntax AT+CPIN=<pin> [,<newpin>] | Response OK or +CME ERROR: <err> Parameters <code> Values when queried using the read command READY MT is not pending for any password SIM PIN MT is waiting for SIM PIN to be given SIM PUK MT is waiting for SIM PUK to be given SIM PIN2 MT is waiting SIM PIN2 to be given (this <code> is recommended to be returned only when the last executed command resulted in PIN2 authentication failure (i.e. +CME ERROR: 17); if PIN2 is not entered right after the failure, it is recommended that MT does not block its operation) SIM PUK2 MT is waiting SIM PUK2 to be given (this <code> is recommended to be returned only when the last executed command resulted in PUK2 authentication failure (i.e. +CME ERROR: 18); if PUK2 and new PIN2 are not entered right after the failure, it is recommended that ME does not block its operation). PH-NET PIN MT is waiting for the network personalization password to be given PH-NET PUK MT is waiting network personalization unblocking password to be given PH-NETSUB PIN MT is waiting network subset personalization password to be given PH-NETSUB PUK MT is waiting network subset personalization unblocking password to be given PH-SP PIN MT is waiting service provider personalization password to be given PH-SP PUK MT is waiting service provider personalization unblocking password to be given PH-CORP PIN MT is waiting corporate personalization password to be given PH-CORP PUK MT is waiting corporate personalization unblocking password to be given <pin>, <newpin> String type values |

## +CPIN2 Command: Enter Pin2

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CPIN2=? | Response OK |
| Read command Syntax AT+CPIN2? | Response +CPIN:code OK or +CME ERROR: <err> |
| Write command Syntax AT+CPIN2= <puk2/oldpin2> [,<newpin2>] or AT+CPIN2= <oldpin2> | Response OK or +CME ERROR: <err> Parameters <puk2/oldpin2>, <newpin2> String type values <code> READY MT is not pending for any password SIM PIN2 MT is waiting for SIM PIN2 to be given (this "code" is recommended to be returned only when the last executed command resulted in PIN2 authentication failure (i.e. +CME ERROR: 17); if PIN2 is not entered right after the failure, it is recommended that MT does not block its operation) SIM PUK2 MT is waiting for SIM PUK2 to be given (this "code" is recommended to be returned only when the last executed command resulted in PUK2 authentication failure (i.e. +CME ERROR: 18); if PUK2 and new PIN2 are not entered right after the failure, it is recommended that MT does not block its operation) |

## +CPUC Command: Price per Unit and Currency

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CPUC=? | Response OK |
| Read command Syntax AT+CPUC? | Response +CPUC: <currency>,<ppu> OK |
| Write command Syntax AT+CPUC= <currency>, <ppu> [,<passwd>] | Response OK or +CME ERROR: <err> Parameters <currency> String type containing the three-character currency code (e.g. GBP, EUR) <ppu> String type containing the price per unit; dot is used as a decimal separator <passwd> String type containing SIM PIN2 |

## +CPAS Command: Phone Activity Status

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CPAS=? | Response +CPAS: (list of supported <pas>es) OK or +CME ERROR: <err> |
| Execute command Syntax AT+CPAS | Response +CPAS: <pas> OK or +CME ERROR: <err> Parameter <pas> 0 Ready (ME allows commands from TA/TE) 1 Unavailable (ME does not allow commands from TA/TE) 2 Unknown (ME is not guaranteed to respond to instructions) 3 Ringing (ME is ready for commands from TA/TE, but the ringer is active) 4 Call in progress (ME is ready for commands from TA/TE, but a call is in progress) 5 Asleep (ME is unable to process commands from TA/TE because it is in a low function-ality state) |

## +CSQ Command: Signal Quality

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CSQ=? | Response +CSQ: (list of supported <rssi>s),(list of supported <ber>s) OK |
| Execute command Syntax AT+CSQ | Response +CSQ: <rssi>,<ber> or +CME ERROR: <err> Parameters <rssi> Received signal strength indication 0 -113 dBm or less 1 – 30 -111 to -53 dBm 31 -51 dBm or greater 99 Not known or not detectable <ber> Integer type; channel bit error rate (in percent) 0 – 7 As RXQUAL values in the table in 3GPP TS 45.008 [20] subclause 8.2.4 99 Not known or not detectable |
| Notes | - For LTE, <rssi> is scaled from the current radio signal strength (RSRP) value of the serving cell. RSRP is defined according to 3GPP TS 36.133 section 9.1.4, from -140 dBm to -44 dBm with 1 dB resolution. - For LTE, <ber> is scaled to 0 – 7 from RSRQ signal quality 34 – 0. RSRQ is defined according to specification 3GPP 36.133 section 9.1.7, from -19.5 dBm to -3 dBm with 0.5 dB resolution. |

## +KCELL Command: Cell Environment Information

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KCELL=? | Response +KCELL: (list of supported <revision>s) OK |
| Read command Syntax AT+KCELL? | Response OK |
| Write command Syntax AT+KCELL= <revision> | Response For GSM cells: +KCELL: <nbGSMcells>[,<cell_typei>,<ARFCNi>,<BSICi>,<PLMNi>,<LACi>, <GSM_CIi>,<RSSIi>,<GSM_TA>][,<cell_typei>,<ARFCNi>,<BSICi>,<PLMNi>, <LACi>,<CIi>,<RSSIi>][…]]] For UMTS cells: +KCELL: <nbUMTScells>[,<cell_typek>,<dl_UARFCNk>,<PLMNk>,<LACk>, <UMTS_CIk>,<scrambling_codek>,<rscpk>,<ecnok>[,<pathlossk>]][…]]] For LTE cells: +KCELL: <nbLTEcells>[,<cell_type>,<PLMN>,<LTE_CI>,<PhyCellInd>, <trackingAreaCode>,<RSRPResult>,<RSRQResult>,<LTE_TA>][<cell_type>,[[Earfcn>, [<PhyCellID>,[<RSRPResult>,[<RSRQResult>]]]]][…]] OK Parameters <revision> Reserved for future development (only 0 for the moment) <nbGSMcells> 0 i 7 Number of base stations available <cell_type> 0 GSM serving cell 1 GSM neighbor cell 2 UMTS serving cell 3 UMTS neighbor cell 4 UMTS detected cell 5 LTE serving cell 6 LTE neighbor cell <ARFCN> 0 – 1023 Absolute Radio Frequency Channel Number in decimal format <BSIC> 0 – 63 Base Station Identity Code in 6 bits decimal format <PLMN> PLMN identifiers (3 bytes) in hexadecimal format, made of MCC (Mobile Country Code), and MNC (Mobile Network Code) <LAC> Location Area in hexadecimal format, 4 digits <GSM_CI> Cell ID, 4 hexadecimal digits, e.g. ABCD <RSSI> 0 – 63 Received signal level of the BCCH carrier. The indicated value is an offset which should be added to –110 dBm to get a value in dBm. See the formula specified in TS 05.08 Radio Subsystem Link Control <GSM_TA> 0 – 63 Timing advance; only available for serving cell 255 Not available (there is no active CS/PS connection) <nbUMTScells> 0 k 25 Number of UMTS base stations available <dl_UARFCN> DL UARFCN of serving cell in decimal format. The range can be found at 3GPP TS 25.101 <UMTS_CI> Cell ID, 8 hexadecimal digits, 32 bits <scrambling code> 0 – 511 Downlink scrambling code in decimal format <rscp> 0 – 91 Received Signal Code Power. The power level in one chip 255 Invalid/default value <ecno> 0 – 24 Ratio of energy per modulating bit to the noise spectral density. This is the cell quality and is equal to RSCP/RSSI Energy per chip/noise 255 Invalid/default value <pathloss> 46 dB to 158 dB Path loss in decimal format 255 Not available <nbLTEcells> 0 k 33 Number of LTE base stations available <LTE_CI> Cell Identity in 8 hexadecimal digits with length = 28 bits. (Ref: 3GPP TS 36.331, 6.3.4, CellIdentity IE) <PhyCellInd> 0 – 503 Physical Cell ID (Ref: 3GPP TS 36.331, 6.3.4, PhysCellId IE) <TrackingAreacode> Tracking Area Code (Ref: 3GPP TS 36.331, 6.3.4, Tracking AreaCode IE) Integer type with length = 16 bits <RSRPResult> 0 – 97 Reference Signal Received Power (Ref: 3GPP TS 36.331, 6.3.5, RSRP-Range IE) <RSRQResult> 0 – 34 Reference Signal Received Quality (Ref: 3GPP TS 36.331, 6.3.5, RSRQ-Range IE) <LTE_TA> 0 – 1282 Timing advance (as per [3GPP 36.321]) <Earfcn> 0 – 0xFFFF Carrier frequency of the neighbor cell designated by the EUTRA Absolute Radio Frequency Channel Number (EARFCN) (Ref: 3GPP TS 36.101, 5.7.3) <PhyCellInd> 0 – 503 Physical Cell ID (Ref: 3GPP TS 36.331, 6.3.4, PhysCellId IE) |
| Reference Sierra Wireless Proprietary | Notes - This command provides information related to the network environment and can be used, for example, for localization calculation. - This command can only be used with a SIM. The cell information can only be retrieved when the UE stays in attached mode. - If no cell information is provided but the radio access technology is supported, +KCELL:0 is returned. |

## +KGPIO Command: Hardware IO Control

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KGPIO=? | Response +KGPIO: (list of supported <IO>s),(list of supported <cde>s) OK |
| Read command Syntax AT+KGPIO? | Response OK |
| Write command Syntax AT+KGPIO=<IO>,<cde> | Response If <cde> = 2: +KGPIO: <IO>, <current_value> OK else: OK Parameters <IO> 1 – 8, 10, 11, 13 – 15 Selected IO <cde> 0 Reset the selected IO 1 Set the selected IO 2 Request the current value of the IO <current_value> 0 GPIO is Low 1 GPIO is High |
| Reference Sierra Wireless Proprietary | Notes - The current configuration is saved in non-volatile memory over module reboot. - Check the configuration of +KGPIOCFG when +CME ERROR: 3 is issued. - GPIO 3 is used for SIM detection by default; it cannot be reconfigured. - The test command AT+KGPIO=? returns a dynamic list of supported GPIOs. GPIOs assigned to a specific purpose are not listed. - This command can be used without a SIM. |
| Examples | Make GPIO1 output high/low level AT+KGPIOCFG=1,0,2 //Configure GPIO1 as output mode; <pull mode> must //be "no pull" OK AT+KGPIO=1,1 //Set GPIO1 OK AT+KGPIO=1,0 //Reset GPIO1 OK Define input/output mode for GPIO1 AT+KGPIOCFG=1,1,0 //Configure GPIO1 as input mode;<pull mode> is “pull down ” OK AT+KGPIO=1,2 //Request the current value of GPIO1 +KGPIO: 1,1 //Value is HIGH for GPIO1 OK at+kgpio=? +KGPIO: (1,2,4,5,6,7,8,10,11,13,14,15),(0-2) OK at+kgpio=9,1 //Set GPIO9, and it should return ERROR +CME ERROR: 3 |

## +KGPIOCFG Command: GPIO Configuration

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KGPIOCFG= ? | Response +KGPIOCFG: (list of supported <n>s),(list of supported <dir>s), (list of supported <pull mode>s) OK |
| Read command Syntax AT+KGPIOCFG? | Response +KGPIOCFG: <n>,<dir>,<pull mode>[<CR><LF> +KGPIOCFG: <n>,<dir>,<pull mode> [...]] OK |
| Write command Syntax AT+KGPIOCFG =<n>,<dir>,<pull mode> | Response OK Parameters <n> 1 – 8, 10, 11, 13 –15 GPIO number <dir> Direction 0 Output 1 Input <pull mode> 0 Pull down. Internal pull-down resistor available. Only used in input mode 1 Pull up. Internal pull up resistor available. Only used in input mode 2 No pull. Internal pull up/down resistor NOT available. Only used in output mode |
| Reference Sierra Wireless Proprietary | Notes - This command provides configuration for +KGPIO command. - The current configuration is saved in non-volatile memory before a reset. - By default, GPIO 3 is used by SIM detection; it cannot be reconfigured. - Pull down/up mode provides a stable input level. - Commands AT+KGPIOCFG=? and AT+KGPIOCFG? return a dynamic list of supported GPIOs. GPIOs assigned to a specific purpose are not listed. - This command can be used without a SIM. |
| Examples | at+kgpiocfg=1,0,0 // When setting GPIO1 as Output, with incorrect <pull mode> ERROR at+kgpiocfg=1,0,1 // When setting GPIO1 as Output, with incorrect <pull mode> ERROR at+kgpiocfg=1,0,2 // When setting GPIO1 as Output, with correct <pull mode> OK at+kgpiocfg=1,1,0 // When setting GPIO1 as Input, with pull down OK at+kgpiocfg=1,1,1 // When setting GPIO1 as Input, with pull up OK at+kgpiocfg=1,1,2 // When setting GPIO1 as Input, with incorrect <pull mode> ERROR at+kgpiocfg=? +KGPIOCFG: (1,2,4,5,6,7,8,10,11,13,14,15),(0-1),(0-2) OK at+kgpiocfg? // GPIO 9 is not available for use +KGPIOCFG: 1,0,2 +KGPIOCFG: 2,0,2 +KGPIOCFG: 4,0,2 +KGPIOCFG: 5,0,2 +KGPIOCFG: 6,0,2 +KGPIOCFG: 7,0,2 +KGPIOCFG: 8,0,2 +KGPIOCFG: 10,0,2 +KGPIOCFG: 11,0,2 +KGPIOCFG: 13,0,2 +KGPIOCFG: 14,0,2 +KGPIOCFG: 15,0,2 OK at+kgpiocfg=9,1,0 // When setting GPIO9, it returns ERROR +CME ERROR: 3 |

## +KADC Command: Analog Digital Converter

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KADC=? | Response +KADC: (list of supported <Meas id>s),(list of supported <Meas time>s) OK |
| Read command Syntax AT+KADC= <Meas id>, <Meas time> | Response +KADC: <Meas result>,<Meas id>,<Meas time>[,<Temperature>] Parameters <Meas id> Measurement ID 0 VBATT – “VBATT” voltage 1 VCOIN – “BAT_RTC” backup battery voltage 2 THERM – Connected to RT400 (the thermistor on board which is located close to the 26MHz VCTCXO) 3 Reserved 4 ADC0 (not supported on the HL7688) 5 Reserved 6 Reserved 7 ADC1 <Meas time> Measurement time 1 During TX 2 Far from TX 3 No constraint <Meas result> Measurement result is in µV <Temperature> Temperature in degrees Celsius |
| Reference Sierra Wireless Proprietary | Notes - 10 bits converter. - VBATT does not support no constraint measurement time. - This command can be used without a SIM. - Available range for voltage input are as follows: <Meas id>; Range (V). VBATT; 3.2 - 4.5. VCOIN; 0 - 1.8. THERM; 0 - 1.2. ADC0; 0 - 1.2. ADC1; 0 - 1.2. |

## +CSIM Command: Generic SIM Access

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CSIM=? | Response OK |
| Write command Syntax AT+CSIM= <length>, <command> | Response +CSIM: <length>,<response> OK or +CME ERROR: <err> Parameters <length> Integer type; length of the characters that are sent to TE in <command> or <response> <command> Command passed on by MT to the SIM in hexadecimal format <response> Response to the command passed on by the SIM to the MT in hexadecimal format |

## +KSIMDET Command: SIM Detection

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KSIMDET=? | Response +KSIMDET: (list of supported <mod>s) OK |
| Read command Syntax AT+KSIMDET? | Response +KSIMDET: <mod> OK |
| Write command Syntax AT+KSIMDET= <mod> | Response OK Parameter <mod> 0 Disable SIM detection 1 Enable SIM detection |
| Notes | - If a change in the SIM status is detected, the module is notified by URC +SIM: <status>, where <status> = 0 means the SIM is extracted and <status> = 1 means the SIM is inserted. - This command can be used without a SIM. - <mod> setting is kept even after the module reboots. |
| Examples | <A SIM card is inserted> AT+KSIMDET? // read current setting +KSIMDET: 1 OK +SIM: 0 // Active SIM card is removed +SIM: 1 // Active SIM card is inserted AT+KSIMDET=? // check supported setting +KSIMDET: (0-1) OK AT+KSIMDET=0 // disable SIM detection OK <No URC indication when SIM card is removed or inserted> AT+KSIMDET? // read current setting +KSIMDET: 0 OK <Reboot module> AT+KSIMDET? // read current setting +KSIMDET: 0 OK |

## +CLAN Command: Read Language

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CLAN=? | Response OK |
| Read command Syntax AT+CLAN? | Response +CLAN: <ln> Parameter <ln> Two letter abbreviation of the language. The language codes, as defined in ISO 639, consists of two characters, e.g. "sv", "en" etc. |

## +CCHO Command: Open Logical Channel

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CCHO=? | Response OK |
| Write command Syntax AT+CCHO= <dfname> | Response <session_id> OK or +CME ERROR: <err> Parameters <dfname> DF name coded on 1 to 16 bytes that references to all selectable application in the UICC <session_id> Session ID to be used in order to target a specific application on the smart card using logical channels mechanism (string without double quotes that represents a decimal value). |
| Notes | The +CCHO execute command gives the <session_id> when it receives SIM application response status words as shown below: - ‘90’ ‘00’ – normal ending of the command - ‘91’ ‘XX’ – normal ending of the command with extra information from the proactive UICC containing a command for the terminal.length ‘XX’ of the response data - ‘92’ ‘XX’ – normal ending of the command with extra information concerning an ongoing data transfer session |

## +CCHC Command: Close Logical Channel

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CCHC=? | Response OK |
| Write command Syntax AT+CCHC= <session_id> | Response OK or +CME ERROR: <err> Parameter <session_id> Session ID to be used in order to target a specific application on the smart card using logical channels mechanism (string without double quotes that represents a decimal value). |

## +CGLA Command: Generic UICC Logical Channel Access

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Write command Syntax AT+CGLA= <sessionid>, <length>, <command> | Response +CGLA: <length>,<response> OK or +CME ERROR: <err> Parameters <sessionid> Integer type; used as the identifier of the session to be used in order to send the APDU commands to the UICC. It is mandatory in order to send commands to the UICC when targeting applications on the smart card using a logical channel other than the default channel (channel "0"). <length> Integer type; length of the characters that are sent to TE in <command> or <response> (two times the actual length of the command or response). <command> Command passed on by the MT to the UICC in the format as described in 3GPP TS 31.101 in hexadecimal format (refer to +CSCS). <response> Response to the command passed on by the UICC to the MT in the format as described in 3GPP TS 31.101 in hexadecimal format (refer to +CSCS). |

## +CRLA Command: Restricted UICC Logical Channel Access

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Write command Syntax AT+CRLA= <sessionid>, <command> [,<file id>[,<P1>, <P2>,<P3> [,<data> [,<pathid>]]]]> | Response +CRLA: <sw1>,<sw2>[,<response>] OK or +CME ERROR: <err> Parameters <sessionid> Integer type which identifies the session to be used in order to send the APDU commands to the UICC. It is mandatory in order to send commands to the UICC when targeting applications on the smart card using a logical channel other than the default channel (channel "0"). <command> 176 READ BINARY 178 READ RECORD 192 GET RESPONSE 214 UPDATE BINARY 220 UPDATE RECORD 242 STATUS 219 SET DATA All other values are reserved <fileid> Integer type that identifies the elementary datafile on SIM. Mandatory for every <command> except STATUS. <P1>, <P2>, <P3> Integer type; parameters passed on by the MT to the UICC. These parameters are mandatory for every command, except GET RESPONSE and STATUS. <data> Information which shall be written to the SIM in hexadecimal format <pathid> String type containing the path of an elementary file on the UICC in hexadecimal format <sw1>, <sw2> Integer type; information from the UICC about the execution of the actual command. These parameters are delivered to the TE in both cases, on successful or failed execution of the command <response> Response of a successful completion of the command previously issued in hexadecimal format. STATUS and GET RESPONSE returns data, which gives information about the current elementary datafield. This information includes the type of file and its size (refer to 3GPP TS 31.101). After READ BINARY, READ RECORD or RETRIEVE DATA command the requested data will be returned. |
| Notes | By using this command instead of generic UICC access command, +CGLA, the TE application has an easier but more limited access to the UICC database. |

## +CUAD Command: UICC Application Discovery

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CUAD=? | Response OK |
| Execute command Syntax AT+CUAD | Response <response> OK or +CME ERROR: <err> Parameter <response> Content of the EFDIR. String type in hexadecimal format. |

## +CRSM Command: Restricted SIM Access

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CRSM=? | Response OK |
| Write command Syntax AT+CRSM= <command> [,<fileid>[,<P1>, <P2>,<P3> [,<data> [,<pathid>]]]] | Response +CRSM: <sw1>,<sw2>[,<response>] OK or +CME ERROR: <err> Parameters <command> 176 READ BINARY 178 READ RECORD 192 GET RESPONSE 214 UPDATE BINARY 220 UPDATE RECORD 242 STATUS <fileid> Integer type; this is the identifier of an elementary data file on the SIM. Mandatory for every command except STATUS. 28423 IMSI file (6F07) 28473 ACM file (6F39) 28481 PUKT file (6F41) 28482 SMS file (6F42) <P1>, <P2>, <P3> Integer type defining the request. These parameters are mandatory for every command, except GET RESPONE and STATUS. The values are described in GSM 51.011 <data> Information which shall be written to the SIM (hexadecimal character format; refer +CSCS) <sw1>, <sw2> Integer type containing SIM information 0x90 0x00 Normal entry of the command 0x9F 0xXX Length XX of the response data 0x92 0x0X Update successful but after using an internal retry routine X times 0x92 0x40 Memory problem 0x94 0x00 No EF selected 0x94 0x02 Out of range (invalid address) 0x94 0x04 File ID not found; pattern not found 0x94 0x08 File is inconsistent with the command 0x98 0x02 No CHV initialized 0x98 0x04 Access cond. Not fullfiled / unsuccessful CHV verify / authentication failed 0x98 0x08 In contradiction with CHV status 0x98 0x10 In contradiction with invalidation status 0x98 0x40 Unsucc. CHV-verif. Or UNBLOCK CHF / CHV blocked /UNBL.blocked 0x98 0x50 Increase can not be performed. Maximum value reached 0x61 0xXX SW2 indicates the number of response bytes still available. Use Get Response to access this data. 0x62 0xXX Warning - state unchanged 0x62 0x00 Warning - no information provided 0x62 0x81 Warning - part of returned data may be corrupt 0x62 0x82 Warning - end of file/record reached (bad cmd) 0x62 0x83 Warning - selected file invalidated 0x62 0x84 Warning - bad file control information format 0x63 0xXX Warning - state unchanged 0x63 0x00 Warning - no information provided 0x63 0x81 Warning - file filled up with last write 0x63 0xCx Warning - counter value is x 0x64 0xXX Error - state unchanged 0x65 0xXX Error - state changed 0x65 0x00 Error - no information provided 0x65 0x81 Error - memory failure 66 xx Security Error 0x66 0xXX Security Error 0x67 0xXX Incorrect parameter P3 0x68 0xXX Check Error - CLA function not supported 0x68 0x00 Check Error - no information provided 0x68 0x81 Check Error - logical channel not supported 0x68 0x82 Check Error - secure messaging not supported 0x69 0xXX Check Error - command not allowed 0x69 0x00 Check Error - no information provided 0x69 0x81 Check Error - command incompatible with file structure 0x69 0x82 Check Error - security status not satisfied 0x69 0x83 Check Error - authentication method blocked 0x69 0x84 Check Error - referenced data invalidated 0x69 0x85 Check Error - conditions of use not satisfied 0x69 0x86 Check Error - command not allowed (no current EF) 0x69 0x87 Check Error - expected SM data objects missing 0x69 0x88 Check Error - SM data objects incorrect 0x6A 0xXX Check Error - wrong parameters 0x6A 0x00 Check Error - no information provided 0x6A 0x80 Check Error - incorrect parameters in data field 0x6A 0x81 Check Error - function not supported 0x6A 0x82 Check Error - file not found 0x6A 0x83 Check Error - record not found 0x6A 0x84 Check Error - not enough memory space in the file 0x6A 0x85 Check Error - Lc vailable on with TLV structure 0x6A 0x86 Check Error - vailable on parameters P1-P2 0x6A 0x87 Check Error - Lc vailable on with P1-P2 0x6A 0x88 Check Error - referenced data not found 0x6B 0xXX Incorrect parameter P1 or P2 0x6C 0xXX Check Error - wrong length - xx is the correct length 0x6D 0xXX Unknown instruction code given in the command 0x6E 0xXX Wrong instruction class given in the command 0x6F 0xXX Technical problem with no diagnostic given <response> Response of successful completion of the command previously issued in hexadecimal character format; refer to +CSCS. STATUS and GET RESPONSE returns data, which gives information about the current elementary datafield. This information includes the type of file and its size (refer to GSM 51.011 [28]). After READ BINARY or READ RECORD commands, the requested data will be returned. <response> is not returned after a successful UPDATE BINARY or UPDATE RECORD command. <pathid> String type that contains the path of an elementary file on the SIM/USIM in hexadecimal format as defined in ETSI TS 102 221 (e.g. "7F205F70" in SIM and USIM case). |
| Notes | By using this command instead of generic SIM access command, +CSIM, the DTE application has an easier but more limited accessto the SIM database. |

## +CEAP Command: EAP Authentication

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Write command Syntax AT+CEAP= <dfname>, <EAPMethod>, <EAP packet data>[,<DFeap>] | Response +CEAP: <EAPsessionid>,<EAP packet response> OK or +CME ERROR: <err> Parameters <dfname> String type in hexadecimal format. All selectable applications are represented in the UICC by an AID coded on 1 to 16 bytes. <EAPMethod> String type in hexadecimal format. The value range for 1-byte format and for 8 bytes expanded format is defined in RFC 3748. <EAP packet data> String type in hexadecimal format <DFeap> String type in hexadecimal format <EAPsessionid> 1 – 4294967295 Identifier of the EAP session to be used in order to retrieve the EAP parameters with +CERP command. <EAP packet response> String type in hexadecimal format |

## +CERP Command: EAP Retrieve Parameters

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax AT+CERP= <EAPsessionid>, <EAPparameter> | Response +CERP: <EAP parameter response> OK or +CME ERROR: <err> Parameters <EAPparameter> 1 Keys 2 Status 3 Identity 4 Pseudonym <EAPsessionid> 1 – 4294967295 Identifier of the EAP session to be used to retrieve the EAP parameters corresponding to an active EAP session. <EAP parameter response> String type in hexadecimal format |

## +KTEMPMON Command: Temperature Monitor

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KTEMPMON=? | Response +KTEMPMON: (list of supported <mod>s),(list of supported <temperature>s),(list of supported <urcMode>s),(list of supported <action>s),(list of supported <hystTime>s),(list of supported <repGPIO>s) OK |
| Read command Syntax AT+KTEMPMON? | Response +KTEMPMON: <mod>,<temperature>,<urcMode>,<action>,<hystTime>,<repGPIO> OK |
| Write command Syntax AT+KTEMPMON=<mod>, [<temperature> [,<urcMode> [,<action> [,<hystTime> [,<repGPIO>]]]]] | Response +KTEMPMON: <level>,<value> OK Parameters <mod> 0 Disable the module’s internal temperature monitor 1 Enable the module’s internal temperature monitor <temperature> Temperature limit before the module acts as defined by <action>. Default value: 0 <urcMode> 0 Disables the presentation of the temperature monitor URC 1 Enables the presentation of the temperature monitor URC <action> 0 No action 1 Automatic shut-down when the temperature is beyond <temperature> 2 The output pin <repGPIO> is tied HIGH when <temperature> is reached; when the temperature is normal the output pin <repGPIO> is tied LOW. Note that if this parameter is required, it is mandatory to set the <repGPIO> parameter. <hyst_time> 0 – 255 Hysteresis time in seconds. Action will only happen if <temperature> is maintained for at least as long as this period. This parameter is mandatory if <action> is not zero. Default value: 30. <repGPIO> 1 – 8, 10, 11, 13 – 15 Defines which GPIO is used as output pin. This parameter is mandatory only if <action>=2 is required. Default value: 6. |
| Notes | - When the module’s internal temperature reaches either operating or extreme levels; the unsolicited message is in the format: +KTEMPMEAS: <level>,<value> where: <level> is the threshold level: -2 Extreme temperature lower bound (-40°C) -1 Operating temperature lower bound (-20°C) 0 Normal temperature 1 Operating temperature upper bound (+55°C) 2 Extreme temperature upper bound (+85°C) <value> is the actual temperature expressed in degrees Celsius - Due to temperature measurement uncertainty, there is a tolerance of ± 2°C. - Check available GPIOs with +KGPIOCFG when using this command. |

## +CTZU Command: Automatic Time Zone Update

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CTZU=? | Response +CTZU: (list of supported <onoff>s) OK |
| Read command Syntax AT+CTZU? | Response +CTZU: <onoff> OK |
| Write command Syntax AT+CTZU =<onoff> | Response OK or +CME ERROR: <err> Parameter <onoff> 0 Disable automatic time zone update via NITZ 1 Enable automatic time zone update via NITZ |
| Notes | - <onoff> is saved in non-volatile memory over module reboot. - CTZU (onoff=1) is enabled by default for proper Verizon Administration (SIM provision, OMADM, etc.) |

## +CTZR Command: Time Zone Reporting

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CTZR=? | Response +CTZR: (list of supported <onoff>s) OK |
| Read command Syntax AT+CTZR? | Response +CTZR: <onoff> OK |
| Write command Syntax AT+CTZR =<onoff> | Response OK or +CME ERROR: <err> Parameter <onoff> 0 Disable time zone change event reporting 1 Enable time zone change event reporting |
| Unsolicited Notification | Response +CTZV: <tz>,<time> XNITZINFO: <timzone_variance>,<time> +CTZDST: <dst> Parameters <tz> Integer value indicating the time zone <time> String type value in format “YY/MM/dd,hh:mm:ss” wherein the characters indicate year, month, date, hour, minutes and seconds. <dst> Daylight sabings time value 0 Disable time zone change event reporting and URC +XNITZINFO, +CTZDST 1 Enable time zone change event reporting and URC +XNITZINFO, +CTZDST <timzone_variance> String of format “GMT+HH:MM” or “GMT-HH:MM” (for example, GMT+5:30) |
| Reference [27.007] §8.41 | Notes - The Time Zone reporting is not affected by the Automatic Time Zone setting command +CTZU - If the reporting is enabled the MT returns the unsolicited result code +CTZV: <tz> whenever the time zone is changed - <onoff> is saved in non-volatile memory per AT port over module reboot |

## +XDATACHANNEL Command: Configure Data Channel

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ XDATACHANNEL=? | Response +XDATACHANNEL: (list of <mode>s),(list of <csd_gprs_flag>s),(list of <connect_flag>s),(list of supported <cid>s) OK |
| Write command Syntax AT+ XDATACHANNEL=<mode>, <csd_gprs_flag>,<ctrl_tid_path>, <tid_path> [,<connect_flag> [,<cid>]] | Response OK or +CME ERROR: <err> Parameters <mode> 0 Disable routing 1 Enable routing 2 Query current setting for the channel where the command is executed (other parameters will be ignored) <csd_gprs_flag> 0 Configure channel for a CSD connection 1 Configure channel for a GPRS connection <ctrl_tid_path> Terminal for which the data routing mechanism shall be enabled in string format (e.g.: “/mux/5”) <tid_path> Terminal to which a data call shall be routed in string format (e.g.: “/mux/5”) <connect_flag> 0 No reporting on the data channel (neither CONNECT nor NO CARRIER) 1 Reporting on the data channel enabled (CONNECT and NO CARRIER) 2 Reporting on the control channel enabled (CONNECT and NO CARRIER) <cid> Numeric parameter which specifies a particular PDP contect definition (see the +CGDCONT and +CGDSCONT commands) |
| Notes | - The control channel must be in OPEN state when the +XDATACHANNEL command is sent. - +XDATACHANNEL settings will only apply while control channel DLC is OPEN and will be reset as soon as DLC is closed. - When this command is sent with <cid> parameter, then the data channel (<tid_path>) must be in OPEN state and the given <cid> should already be defined. - If the <cid> is deleted or undefined, the XDATACHANNEL settings pertaining to the <cid> are not retained. - Connection must be established (start and stop) through <ctrl_tid_path> for data to be properly routed. - +XDATACHANNEL query (mode=2) does not return the <cid> associated with the control channel, as the data routing of a control channel can be configured for multiple <cid>s. - CSD is not allowed on the HL7650. |

## +XCELLINFO Command: Provide Cell Information

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+XCELLINFO =? | Response +XCELLINFO: (range of <mode>s) OK |
| Read command Syntax AT+XCELLINFO? | Response +XCELLINFO: <mode>,<type>,<MCC>,<MNC>,<LAC>,<CI>,<RxLev> [,<t_advance>] OK or +XCELLINFO: <mode>,<type>,<MCC>,<MNC>,<LAC>,<CI>,<scrambling_code>, <dl_frequency>,<rscp>,<ecn0>,<pathloss> OK or +XCELLINFO: <mode>,<type>,[[<Earfcn>,[<PhyCellID>,[<RSRPResult>, [<RSRQResult>]]]]] OK or +XCELLINFO:<mode><type><MCC>,<MNC>,<CI>,<PhyCellInd>,<TrackingAreaCode>, <RSRPResult>,<RSRQResult>,<TA> OK |
| Write command Syntax AT+XCELLINFO=<mode> | Response OK or +CME ERROR: <err> Parameters <mode> 0 Disable periodic reporting 1 Enable reporting 2 Currently not used (for backward compatibility) <type> 2 UMTS sercing cell 3 UMTS neighbor cell 4 UMTS detected cell 5 LTE serving cell 6 LTE neighbor cell <RxLev> See command +CGED <t_advance> Signal strength; only valid for the serving cell <MCC> 0 – 999 Mobile country code <MNC> 0 – 999 Mobile network code <CI> Cell identity, 28-bit integer type <PhysCellId> 0 – 503 Physical cell ID <TrackingAreaCode> Tracking area code, 16-bits integer type <RSRPResult> 0 – 97 Reference signal received power <RSRQPResult> 0 – 34 Reference signal reference quality <TA> 0 – 1282 Timing advance <Earfcn> Carrier frequency of the neighbor cell designated by the EUTRA absolute radio frequency <PhyCellID> 0 – 503 Physical cell ID of the neighbor cell <RSRPResult> 0 – 97 Average RSRP of the neighbor cell <RSRQResult> 0 – 34 Average RSRQ of the neighbor cell |
| Unsolicited Notification | Response for UMTS cells: +XCELLINFO:<type>,<MCC>,<MNC>,<LAC>,<CI>,<scrambling_code>, <dl_frequency>,<rscp>,<ecn0>,<pathloss> Response for LTE serving cell: +XCELLINFO:<type><MCC>,<MNC>,<CI>,<PhyCellInd>,<TrackingAreaCode>, <RSRPResult>,<RSRQResult>,<TA> Response for LTE neighbor cell: +XCELLINFO: <type>,[[<Earfcn>,[<PhyCellID>,[< RSRPResult>,[<RSRQResult>]]]]] |

## +KSLEEP Command: Power Management Control for UART

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KSLEEP=? | Response +KSLEEP: (list of supported <mngt>s) OK |
| Read command Syntax AT+KSLEEP? | Response +KSLEEP: <mngt> OK |
| Write command Syntax AT+KSLEEP= <mngt> | Response OK Parameter <mngt> 0 The UART doesn’t go to sleep mode when the DTR is active (low level). The DTR must be active to send AT commands. 1 The UART decides by itself (internal timing) when it will go to sleep mode, and it will be woken up with a character 2 The UART never goes in sleep mode regardless of the DTR state |
| Reference Sierra Wireless Proprietary | Notes - The current configuration is kept in non-volatile memory over module reboot. - This command only controls UART power management, and does not affect the USB AT command port. - This command can be used without a SIM. - When AT+KSLEEP=1 and the module is in sleep mode, the user needs to input a character to wake the module up. After which, AT commands can be input normally. |
| Example | AT+KSLEEP=? +KSLEEP: (0-2) OK AT+KSLEEP? +KSLEEP: 2 OK AT+KSLEEP=0 // Change settings to mode 0 OK AT+KSLEEP? +KSLEEP: 0 OK AT+KSLEEP=2 // Change settings to mode 2 OK AT+KSLEEP? +KSLEEP: 2 OK |

## +HBHV Command: Configure General System Behavior

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+HBHV=? | Response +HBHV: (0,2,3),(0,1) +HBHV: 1,(0-2) … OK |
| Read command Syntax AT+HBHV? | Response +HBHV: 0,<ppp_dun_mode> +HBHV: 1,<omadm_reg_mode>,<omadm_reg_state> +HBHV: 2,<pdp_unlock_mode> +HBHV: 3,<show_orig_apn> OK |
| Write command Syntax AT+HBHV=0, <ppp_dun_ mode> AT+HBHV=1, <omadm_reg_ mode> AT+HBHV=2, <pdp_unlock_ mode> AT+HBHV=3, <show_orig_ apn> | Response OK Parameters <ppp_dun_mode> PPP dial-up networking behavior 0 PDP context is brought up after LCP negotiation 1 PDP context is brought up before LCP negotiation <omadm_reg_mode> OMADM client boostrapping behavior 0 Disables boostrapping initiated by the client 1 Enables boostrappining initiated by the client on the next successful registration if the module’s IMEI is used for the first time and no server initatiated session has happened before 2 Enables boostrapping initiates by the client on the next successful registration regardless of the above-mentioned criteria. <omadm_reg_state> Boostrap registration state 0 The client hasn’t been boostrapped yet (no server initiated session has happened before) 1 The client has been boostrapped before with a successful server initiated session <pdp_unlock_mode> PDP unlock mode For HL7618 Verizon and HL7618RD modules: 0 Protects the reserved PDP contexts (1, 2, 11-20) from being modified accidentally 1 Unlocks the protection on the reserved PDP contexts For HL7690 and HL7692 modules: 0 Protects the reserved PDP context (1) from being modified accidentally 1 Unlocks the protection on the reserved PDP context <show_orig_apn> Enables showing the original APN saved in non-volatile memory (updated by AT+CGDCONT=…); this is effective for PDP context 1 (LTE default bearer) with PDP context reading (AT+CGDCONT?) 0 Disabled. Shows APN given by the network (e.g. "ltemobile.apn.mnc720.mcc302.gprs", "vzwims.mnc480.mcc311.gprs") 1 Enabled. Shows the original APN saved in non-volatile memory |
| Notes | - <omadm_reg_mode> only affects HL7618 Verizon and HL7618RD modules. - <omadm_reg_mode> will automatically be changed from “2” to “1” after the server initialized session was successfully processed. - The HL7618 and HL7618RD’s OMADM client uses customized bootstrapping defined in the OMADM specification, i.e. with Verizon OMADM server connection configurations preloaded. However, the module still needs to be bootstrapped before the OMADM client can work with the Verizon OMADM server. There are two mechanisms to complete this bootstrapping: - Boostrapping initiated by the server – the IMEI/IMSI are pre-registered to Verizon’s OMADM databse and boostrapping is initiated automatically by the OMADM server through a DM session. - Boostrapping initiated by the client – the module initiates a DM session to the Verizon OMADM server that performs the bootstrapping. The default option <omadm_reg_mode>=1 enables the module to perform boostrapping automatically via mechanism 2. |

## +CIREP Command: IMS Network Reporting

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CIREP=? | Response +CIREP: (list of supported <reporting>s) OK |
| Read command Syntax AT+CIREP? | Response +CIREP: <reporting>,<nwimsvops> OK or +CME ERROR: <err> |
| Write command Syntax AT+CIREP= <reporting> | Response OK or +CME ERROR: <err> Parameters <reporting> 0 Disable reporting 1 Enable reporting <nwimsvops> Gives the last IMS Voice over PS session (IMSVOPS) supported indication received from network 0 IMSVOPS support indication is not received from network, or is negative 1 IMSVOPS support indication as received from network is possible <srvcch> SRVCC handover information 0 PS to CS SRVCC handover has started in the CS domain ("Handover Command" indicating SRVCC received) 1 PS to CS SRVCC handover successful ("Handover Complete" sent) 2 PS to CS SRVCC handover cancelled ("Handover Failure" sent) 3 PS to CS SRVCC handover, general non-specific failure |
| Unsolicited Notification | Response +CIREPI: <nwimsvops> +CIREPH: <srvcch> |
| Notes | - <reporting> is saved in non-volatile memory per AT port over module reboot. - <srvcch>=3, general non-specific failure, may be used, for example in the case of handover cancellation as specified in 3GPP TS 24.301 subclause 6.6.2. |

## +CIREG Command: Registration Information

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CIREG=? | Response +CIREG: (list of supported <n>s) OK |
| Read command Syntax AT+CIREG? | Response +CIREP: <n>,<reg_info>[,<ext_info>] OK |
| Write command Syntax AT+CIREG=<n> | Response OK or +CME ERROR: <err> Parameters <n> Enables or disables reporting of changes in the MT’s IMS registration information 0 Disable reporting 1 Enable reporting (parameter <reg_info>) 2 Enable extended reporting (parameter <reg_info> and <ext_info>) <reg_info> Indicates IMS registration status 0 Not registered 1 Registered <ext_info> Numeric value in hexadecimal format. It is the sum of hexadecimals values, each respresenting a particular IMS capability of the MT. This parameter is not present if the IMS registration status is “not registered” 1 RTP-based transfer of voice 2 SMS using IMS functionality 5 Both RTP-based transfer of voice according to MMTEL and SMS using IMS functionality can be used |
| Unsolicited Notification | Response +CIREGU: <reg_info>[,<ext_info>] |
| Notes | <n> is saved in non-volatile memory per AT port over module reboot |

## +GST Command: General System Status Information

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+GST=? | Response +GST: (list of supported <mode>s) OK |
| Read command Syntax AT+GST? | Response … (display all responses of <mode>s) OK |
| Write command Syntax AT+GST=<mode> | Response For <mode>=0: … (display all responses of <mode>s) OK For <mode>=1: +GST: <rtc_time>,<up_time> OK For <mode>=2: +GST: <port device string> OK Parameters <mode> 0 Display all status information 1 Display the RTC time in seconds since 1970 Jan 1, and system boot up time in seconds 2 Display module port device string (e.g. /USBCDC/0) <rtc_time> RTC time in seconds since 1970 Jan 1 <up_time> System boot up time in seconds <port device string> String type; unique AT port device string e.g. “/USBCDC/0” /USBCDC/0 ACM0 AT port /USBCDC/2 ACM2 AT port |

## +CESQ Command: Extended Signal Quality

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CESQ=? | Response +CESQ: (list of supported <rxlev>s),(list of supported <ber>s),(list of supported <rscp>s), (list of supported <ecno>s),(list of supported <rsrq>s),(list of supported <rsrp>s) OK |
| Execute command Syntax AT+CESQ | Response +CESQ: <rxlev>,<ber>,<rscp>,<ecno>,<rsrq>,<rsrp> OK Parameters <rxlev> Integer type; received signal strength level (see 3GPP TS 45.008 [20] subclause 8.1.4) 0 rssi < -110 dBm 1 -110 dBm ≤ rssi < -109 dBm 2 -109 dBm ≤ rssi < -108 dBm … 61 -50 dBm ≤ rssi < -49 dBm 62 -49 dBm ≤ rssi < -48 dBm 63 -48 dBm ≤ rssi 99 not known or not detectable <ber> Integer type; channel bit error rate (in percent) 0 – 7 As RXQUAL values in the table in 3GPP TS 45.008 [20] subclause 8.2.4 99 Not known or not detectable <rscp> Integer type; received signal code power (see 3GPP TS 25.133 [95] subclause 9.1.1.3 and 3GPP TS 25.123 [96] subclause 9.1.1.1.3) 0 rscp < -120 dBm 1 -120 dBm ≤ rscp < -119 dBm 2 -119 dBm ≤ rscp < -118 dBm … 94 -27 dBm ≤ rscp < -26 dBm 95 -26 dBm ≤ rscp < -25 dBm 96 -25 dBm ≤ rscp 255 Not known or not detectable <ecno> Integer type; ratio of the received energy per PN chip to the total received power spectral density (see 3GPP TS 25.133 [95] subclause) 0 Ec/Io < -24 dB 1 -24 dB ≤ Ec/Io < -23.5 dB 2 -23.5 dB ≤ Ec/Io < -23 dB … 47 -1 dB ≤ Ec/Io < -0.5 dB 48 -0.5 dB ≤ Ec/Io < 0 dB 49 0 dB ≤ Ec/Io 255 Not known or not detectable <rsrq> Integer type; reference signal received quality (see 3GPP TS 36.133 [96] subclause 9.1.7) 0 rsrq < -19.5 dB 1 -19.5 dB ≤ rsrq < -19 dB 2 -19 dB ≤ rsrq < -18.5 dB … 32 -4 dB ≤ rsrq < -3.5 dB 33 -3.5 dB ≤ rsrq < -3 dB 34 -3 dB ≤ rsrq 255 Not known or not detectable <rsrp> Integer type; reference signal received power (see 3GPP TS 36.133 [96] subclause 9.1.4) 0 rsrp < -140 dBm 1 -140 dBm ≤ rsrp < -139 dBm 2 -139 dBm ≤ rsrp < -138 dBm … 95 -46 dBm ≤ rsrp < -45 dBm 96 -45 dBm ≤ rsrp < -44 dBm 97 -44 dBm ≤ rsrp 255 Not known or not detectable |
| Notes | - If the current serving cell is not a GERAN cell, <rxlev> and <ber> are set to value 99. - If the current serving cell is not a UTRA FDD or UTRA TDD cell, <rscp> is set to 255. - If the current serving cell is not a UTRA FDD cell, <ecno> is set to 255. - If the current serving cell is not an E-UTRA cell, <rsrq> and <rsrp> are set to 255. |

## +XCSQ Command: Radio Signal Strength and Quality with URC Support

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+XCSQ=? | Response +XCSQ: (list of supported <n>s) OK |
| Read command Syntax AT+XCSQ? | Response +XCSQ: <n>,<rssi>,<ber> OK |
| Write command Syntax AT+XCSQ=<n> | Response OK or +CME ERROR: <err> Parameters <n> 0 Disable radio signal strength and quality indication URC 1 Enable radio signal strength and quality indication URC <rssi> Radio signal strength indication 0 -113 dBm or less 1 – 30 -111 to -53 dBm 31 -51 dBm or greater 99 Not known or not detectable <ber> Received signal quality. Range of values = 0 – 34 accoding to specification 3GPP 36.133 section 9.1.7 |
| Unsolicited Notification | Response +XCSQ: <rssi>,<ber> |

## +XCESQ Command: Extended Signal Quality with URC Support

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+XCESQ=? | Response +XCESQ: (list of supported <n>s),(list of supported <rxlev>s),(list of supported <ber>s),(list of supported <rscp>s),(list of supported <ecno>s),(list of supported <rsrq>s),(list of supported <rsrp>s),(list of supported <rssnr>s) OK |
| Read command Syntax AT+XCESQ? | Response +XCESQ: <n>,<rxlev>,<ber>,<rscp>,<ecno>,<rsrq>,<rsrp>,<rssnr> OK |
| Write command Syntax AT+XCESQ= [<n>] | Response OK or +CME ERROR: <err> Parameters <rxlev> Integer type; received signal strength level (see 3GPP TS 45.008 [20] subclause 8.1.4) 0 rssi < -110 dBm 1 -110 dBm ≤ rssi < -109 dBm 2 -109 dBm ≤ rssi < -108 dBm … 61 -50 dBm ≤ rssi < -49 dBm 62 -49 dBm ≤ rssi < -48 dBm 63 -48 dBm ≤ rssi 99 Not known or not detectable <ber> Integer type; channel bit error rate (in percent) 0 – 7 As RXQUAL values in the table in 3GPP TS 45.008 [20] subclause 8.2.4 99 Not known or not detectable <rscp> Integer type; received signal code power (see 3GPP TS 25.133 [95] subclause 9.1.1.3 and 3GPP TS 25.123 [96] subclause 9.1.1.1.3) 0 rscp < -120 dBm 1 -120 dBm ≤ rscp < -119 dBm 2 -119 dBm ≤ rscp < -118 dBm … 94 -27 dBm ≤ rscp < -26 dBm 95 -26 dBm ≤ rscp < -25 dBm 96 -25 dBm ≤ rscp 255 Not known or not detectable <ecno> Integer type; ratio of the received energy per PN chip to the total received power spectral density (see 3GPP TS 25.133 [95] subclause) 0 Ec/Io < -24 dB 1 -24 dB ≤ Ec/Io < -23.5 dB 2 -23.5 dB ≤ Ec/Io < -23 dB … 47 -1 dB ≤ Ec/Io < -0.5 dB 48 -0.5 dB ≤ Ec/Io < 0 dB 49 0 dB ≤ Ec/Io 255 Not known or not detectable <rsrq> Integer type; reference signal received quality (see 3GPP TS 36.133 [96] subclause 9.1.7) 0 rsrq < -19.5 dB 1 -19.5 dB ≤ rsrq < -19 dB 2 -19 dB ≤ rsrq < -18.5 dB … 32 -4 dB ≤ rsrq < -3.5 dB 33 -3.5 dB ≤ rsrq < -3 dB 34 -3 dB ≤ rsrq 255 Not known or not detectable <rsrp> Integer type; reference signal received power (see 3GPP TS 36.133 [96] subclause 9.1.4) 0 rsrp < -140 dBm 1 -140 dBm ≤ rsrp < -139 dBm 2 -139 dBm ≤ rsrp < -138 dBm … 95 -46 dBm ≤ rsrp < -45 dBm 96 -45 dBm ≤ rsrp < -44 dBm 97 -44 dBm ≤ rsrp 255 Not known or not detectable <rssnr> Integer type; radio signal strength noise ration value -100 RSSNR ≤ -50 dB -99 -50 dB < RSSNR ≤ -49.5 dB -98 -49.5 dB < RSSNR ≤ -49 dB … -1 -1 dB < RSSNR ≤ -0.5 dB 0 -0.5 dB < RSSNR ≤ 0 dB 1 0 dB < RSSNR ≤ 0.5 dB … 98 49 dB ≤ RSSNR < 49.5 dB 99 49.5 dB ≤ RSSNR < 50 dB 100 50 dB ≤ RSSNR 255 Not known or not detectable |
| Unsolicited Notification | Response +XCESQI: <rxlev>,<ber>,<rscp>,<ecno>,<rsrq>,<rsrp>,<rssnr> |
| Notes | - If the current serving cell is not a GERAN cell, <rxlev> and <ber> are set to value 99. - If the current serving cell is not a UTRA FDD or UTRA TDD cell, <rscp> and <ecno> are set to 255. - If the current serving cell is not an E-UTRA cell, <rsrq>, <rsrp> and <rssnr> are set to 255. |

## +WEXTCLK Command: External Clocks Setting

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+WEXTCLK=? | Response +WEXTCLK: (list of supported <output>s),(list of supported <status>es) OK |
| Read command Syntax AT+WEXTCLK? | Response +WEXTCLK: <output>,<status> +WEXTCLK: <output>,<status> OK |
| Write command Syntax AT+WEXTCLK= <output>, <status> | Response +WEXTCLK: <output>,<status> OK Parameters <output> 0 32kHz output (32K_CLKOUT) 1 26MHz output (26M_CLKOUT) <status> 0 Disabled 1 Enabled |
| Notes | - This command allows generating 32 kHz and 26 MHz on the output clock pins of the module. - Parameters are saved in non-volatile memory. - This command is available when the module has finished its initialization. - This command can be used without a SIM. |

## +KRIC Command: Ring Indicator Control

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KRIC=? | Response +KRIC: (list of supported <masks>es),(list of supported <shape>s) OK |
| Read command Syntax AT+KRIC? | Response +WEXTCLK: <masks>,<shape> OK |
| Write command Syntax AT+KRIC= <masks> [,<shape>] | Response OK Parameters <masks> Use of RI signal 0x00 RI is not used 0x01 RI is activated on incoming calls (+CRING, RING) 0x02 RI is activated on SMS (+CMT, +CMTI) 0x04 RI is activated on SMS-CB (+CBM, +CBMI) 0x08 RI is activated on USSD (+CUSD) 0x10 RI is activated on network state (+CIEV) <shape> Signal shape (only available for incoming calls) 0 Repeat pulses. The total length of the pulse is equivalent to the transfer of the RING or CRING notification 1 Always active. The signal is set to be active during the whole incoming call notification |
| Reference Sierra Wireless Proprietary | Notes - The current configuration is kept in non-volatile memory after a reset. - For SMS and other unsolicited messages, only one pulse is set regardless of <shape>. - The width of the pulse is 1s. For repeated pulse on incoming calls, pulse width is 1s, and then rest for 4 seconds, and then repeated. - This command should not be used during an incoming call, SMS, SMSCB, USSD, etc. - This command can be used without a SIM. - If <shape> is omitted, the previously saved value will be used. |
| Examples | AT+KRIC=? +KRIC: (0-31),(0-1) OK AT+KRIC? +KRIC: 15,0 OK AT+KRIC=1,1 //RI is always activated on incoming calls OK AT+KRIC? +KRIC: 1,1 OK AT+KRIC=2 //RI is activated on SMS OK AT+KRIC? +KRIC: 2,1 OK |

## +CPWROFF Command: Switch MS Off

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CPWROFF=? | Response OK |
| Execute command Syntax AT+CPWROFF [=<mode>] | Response OK or +CME ERROR: <error> Parameter <mode> Power down mode 1 Fast power down mode |
| Notes | - Not specifying a parameter value for the execute command will perform normal IMSI detach before powering down. - <mode>=1 will perform fast power down (~100 to 300 ms) without an IMSI detach request being sent to the network. |

## +KUSBCOMP Command: Set USB Composition

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KUSBCOMP=? | Response +KUSBCOMP: (list of supported <mode>s) OK |
| Read command Syntax AT+KUSBCOMP? | Response +KUSBCOMP: <mode> OK |
| Write command Syntax AT+KUSBCOMP=<mode> | Response OK Parameter <mode> 0 3 CDC-ACM and 4 NCM, (VID: 0x0807 PID: 0x0443) NCM0 – NCM Network interface NCM1 – NCM Network interface NCM2 – NCM Network interface NCM3 – NCM Network interface USB0 – AT / modem port USB1 – Traces port USB2 – AT / modem port 1 7 CDC-ACM, (VID: 0x1519 PID: 0x0020) USB0 – AT / modem port USB1 – Traces port USB2 – AT / modem port USB3 – AT / modem port USB4 – AT / modem port USB5 – reserved port USB6 – reserved port 2 1 MBIM and 3 CDC-ACM, (VID: 0x0807 PID: 0x0911) MBIM0 – MBIM Network interface USB0 – AT / modem port USB1 – Traces port USB2 – AT / modem port |
| Notes | - The current configuration is kept in non-volatile memory. - New configuration will only be activated after the module reboots. - The factory preset value of <mode> is 0. - This command can be used without a SIM. - Configuration is not updated when <mode> = 2 if a firmware downgrade is performed. |
| Examples | AT+KUSBCOMP=0 OK AT+KUSBCOMP=3 ERROR AT+KUSBCOMP? +KUSBCOMP: 0 OK AT+KUSBCOMP=1 OK AT+CFUN=1,1 // Reboot the module to take effect. The new mode is effective // with the USB bus re-enumerated. OK <<<< module reboots >>>> |

## +WMUSBVCC Command: USB VCC Detection Setting

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+WMUSBVCC=? | Response +WMUSBVCC: (list of supported <mode>s) OK |
| Read command Syntax AT+WMUSBVCC? | Response +WMUSBVCC: <mode> OK |
| Write command Syntax AT+WMUSBVCC=<mode> | Response OK Parameter <mode> 0 USB detection if Vbus > 4.75V 1 USB detection if Vbus > 2.5V (e.g., for PC mini-card applications) |
| Reference Sierra Wireless Proprietary | Notes - <mode> is saved to non-volatile memory over module reset. - This command can be used without a SIM. |
| Examples | AT+WMUSBVCC=? +WMUSBVCC: (0-1) OK AT+WMUSBVCC? +WMUSBVCC: 0 OK AT+WMUSBVCC=0 // Change setting to mode 0 OK AT+WMUSBVCC? +WMUSBVCC: 0 OK AT+WMUSBVCC=1 // Change setting to mode 1 OK AT+WMUSBVCC? +WMUSBVCC: 1 OK |

## +KLTEMUTE Command: Mute LTE TX

For HL7650, HL7690 and HL7692 only.

| HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KLTEMUTE=? | Response +KLTEMUTE: (list of supported <mode>s),(list of supported <duration>s), (list of supported <ind>s) OK or +CME ERROR: <err> |
| Read command Syntax AT+KLTEMUTE? | Response +KLTEMUTE: <mode>,<duration>,<ind> OK or +CME ERROR: <err> |
| Write command Syntax AT+KLTEMUTE=<mode> [,<duration> [,<ind>]] | Response OK or +CME ERROR: <err> Parameters <mode> Enable or Disable LTE TX mute 0 The feature is deactivated, the LTE transmit power emission is unmuted 1 The feature is activated and the LTE transmit power emission is currently muted <duration> Mute duration (only used when <mode>=1) Range: 5s – 120s; default value = 30 <ind> Unsolicited result code mode 0 Disable LTE TX mute unsolicited result code 1 Disable LTE TX mute unsolicited result code +KLTEMUTE: <mode> |
| Unsolicited Notification | Response +KLTEMUTE: <state>,<duration> 1 // start LTE mute with duration <state> 0 // stop LTE mute |
| Reference Sierra Wireless Proprietary | Notes - If AT+KLTEMUTE=1 is resent within <duration> after it has already been sent, then it will not take any effect. The <duration> timer will not be reactivated and will continue to run as expected. - If AT+KLTEMUTE=0 is sent within <duration> after AT+KLTEMUTE=1 is sent, then the <duration> timer will be killed and mute will be deactivated. - If AT+KLTEMUTE=0 is sent while mute is not activated, then the AT command does not take any effect. - When the feature is activated, the <mode> parameter will be automatically reset to 0 after the <duration> mute timer expires. - At module power up, by default, <mode> is always 0. - This command can only be used with a SIM. |
| Examples | AT+KLTEMUTE=? +KLTEMUTE: (0-1),(5-120),(0-1) OK AT+KLTEMUTE? // Read the current settings +KLTEMUTE: 0,30,0 OK AT+KLTEMUTE=1,40,1 // Activate LTE TX mute during 40s OK +KLTEMUTE: 1 // LTE TX mute is started AT+KLTEMUTE? +KLTEMUTE: 1, 40,1 OK |

## +KSYNC Command: Application Synchronization Signal

For HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KSYNC=? | Response +KSYNC: (list of supported <mode>s),(list of supported <IO>s),(range of <Duty Cycle>), (range of <Pulse Duration>) OK |
| Read command Syntax AT+KSYNC? | Response +KSYNC: <mode>,<IO>,<Duty Cycle>,<Pulse Duration> OK |
| Write command Syntax AT+KSYNC= <mode>[,<IO> [,<Duty Cycle> [,<Pulse Duration>]]] | Response OK Parameters <mode> Operation mode 0 Disable the generation of synchronization signal 1 Manage the generation of signal according to <Duty Cycle> and <Pulse Duration>. The waveform of the signal is controlled only by these two parameters; network status would not affect the waveform 2 Manage the generation of signal according to CS network registration status; PERMANENTLY OFF Not register/Initialization/Register denied/no SIM card 600 ms ON / 600ms OFF Not registered but searching 75 ms ON / 3s OFF Right connected to the network <Duty Cycle> and <Pulse Duration> are not used in mode 2 For backward compatibility, HL7618/90 does not support CS network registration. 3 Manage the generation of signal according to PS network registration status; OFF Not registered/Initialzation/Registered denied/no SIM card ON Registered to the network <IO> 1 – 8, 10, 11, 13 – 15 GPIO used as output <Duty Cycle> 1 – 100 In percent; only effective when <mode>=1 Default value = 50 <Pulse Duration> 10 – 65535 In milliseconds; only effective when <mode>=1 Default value = 1000 |
| Notes | - Parameter settings are automatically saved in non-volatile memory. - <Duty Cycle> and <Pulse Duration> can be configured regardless of <mode>. - Refer to +KGPIOCFG for multiplexed functions of GPIOs. - GPIOs may be already used by SIM detection, temperature monitoring, etc. Check with other related commands such as +KSIMDET, +KTEMPMON, etc. prior to using this command. - This command can be used without a SIM. - This command will force the GPIO pins as output, regardless of AT+KGPIOCFG configuration. - Only 1 GPIO signal can be generated at any time. - The minimum LED ON/OFF cycle is 5ms due to the precision of the timer; this feature cannot be used if either the LED ON/OFF cycle is less than 5ms. “LED ON cycle” is <Pulse Duration>*<Duty Cycle> while “LED OFF cycle” is <Pulse Duration> - “LED ON cycle”. - <mode>=2 is kept for compatibility with other HL series LTE-only products which do not support CS, e.g. HL7618, HL7618RD and HL7690. |
| Examples | AT+KSYNC=1,1,50,2000 // Generate signal with 50% duty cycle and 2000 ms pulse // duration on GPIO1 OK AT+KSYNC=1,2,50,2000 // Generate signal with 50% duty cycle and 2000 ms pulse // duration on GPIO2 OK // Previous signal on GPIO1 will be stopped AT+KSYNC=0,2 // Disable signal generation OK AT+KSYNC=2,1 // Generate signal on GPIO1 according to the CS network // registration status OK AT+KSYNC=3,1 // Generate signal on GPIO1 according to the PS network // registration status OK |

## +KLTEPARAM Command: LTE Parameters

For HL7650, HL7690 and HL7692 only.

| HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KLTEPARAM=? | Response +KLTEPARAM: (list of supported <mode>s) OK or +CME ERROR: <err> |
| Read command Syntax AT+KLTEPARAM? | Response +KLTEPARAM: <mode>,<qrxlevmin>,<T3402-dur>,<T3402-stat>,<T3412-dur>, <T3412-stat> OK or +CME ERROR: <err> |
| Write command Syntax AT+KLTEPARAM=<mode> | Response OK or +CME ERROR: <err> Parameters <mode> URC reporting mode 0 Disable URC reporting 1 Enable URC reporting <qrxlevmin> -70 to 22 qRxLevMin in dBm. Default value = 32767 This parameter is omitted if it is not available <T3402-dur> T3402 duration in ms. Default value = 720000 (12 min) This parameter is omitted if it is not available <T3402-stat> T3402 timer status. This parameter is omitted if it is not available 0 Stopped 1 Running <T3412-dur> T3412 duration in ms. Default value = 3240000 (54 min) This parameter is omitted if it is not available <T3412-stat> T3412 timer status. This parameter is omitted if it is not available 0 Stopped 1 Running |
| Unsolicited Notification | Response +KLTEPARAM: <qrxlevmin>,<T3402-dur>,<T3402-stat>,<T3412-dur>,<T3412-stat> |
| Reference Sierra Wireless Proprietary | Notes - The qrxlevmin is defined according to 3GPP specification TS 36.304 section 5.2.4.7. - The EMM timer T3402 and T3412 are defined according to 3GPP specification TS 24.301 section 5.3.5 – 5.3.6. - <mode> is reset to 0 automatically after reboot. - The command can only be used with a SIM. - Parameter values are only available after the module is registered to the network. - URC is presented when the value is updated by the network. |
| Examples | AT+KLTEPARAM=? +KLTEPARAM: (0-1) OK AT+KLTEPARAM? // Read the current settings when the // module is registered to the network +KLTEPARAM: 0,-60,720000,0,3240000,1 OK AT+KLTEPARAM=1 // Enable URC message OK +KLTEPARAM: -60,720000,0,3240000,1 // URC message AT+KLTEPARAM? // Read command when the module is not // registered to the network +KLTEPARAM: 0,,,,, OK |

## +KBND Command: Current Networks Band Indicator

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KBND=? | Response +KBND: (list of supported <bnd>s) OK |
| Read command Syntax AT+KBND? | Response +KBND: <bnd> OK Parameter <bnd> Band in hexadecimal format 0x00000000 Not available 0x00000002 GSM 900 MHz (HL7692 only) 0x00000004 DCS 1800 MHz (HL7692 only) 0x00000010 UMTS Band I (2100 MHz) (HL7650 only) 0x00000020 UMTS Band II (1900 MHz) (HL7688 only) 0x00000040 UMTS Band V (850 MHz) (HL7650 and HL7688) 0x00000100 UMTS Band VIII (900 MHz) (HL7650 only) 0x00000800 LTE Band 2 (1900 MHz) (HL7648 and HL7688) 0x00001000 LTE Band 3 (1800 MHz) (HL7650, HL7690 and HL7692) 0x00002000 LTE Band 4 (1700 MHz) (HL7618, HL7618RD, HL7648 and HL7688) 0x00004000 LTE Band 5 (850 MHz) (HL7650 and HL7688) 0x00010000 LTE Band 13 (700 MHz) (HL7618, HL7618RD and HL7688) 0x00020000 LTE Band 17 (700MHz) (HL7688 only) 0x00040000 LTE Band 28 (700 MHz) (HL7650 only) 0x00800000 LTE Band 8 (900MHz) (HL7650, HL7690 and HL7692) 0x01000000 LTE Band 20 (800MHz) (HL7690 and HL7692) 0x02000000 LTE Band 12 (700 MHz) (HL7648 only) |
| Reference Sierra Wireless Proprietary | Notes - This command returns the GSM, UMTS or LTE band that the module is currently using. - This command cannot be used without a SIM. |

## +KSRAT Command: Set Radio Access Technology

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KSRAT=? | Response +KSRAT: (list of supported <mode>s) OK |
| Read command Syntax AT+KSRAT? | Get current band Response +KSRAT: <mode> OK |
| Write command Syntax AT+KSRAT= <mode> | Set current mode Response OK Parameter <mode> 1 GSM only 2 UMTS only 5 LTE only 6 Search for LTE first then UMTS 7 Search for LTE first then UMTS 8 Search for GSM first then LTE 9 Search for LTE first then GSM |
| Reference Sierra Wireless Proprietary | Notes - This command can be used without a SIM. - <mode> is automatically stored in persistent memory. - Settings take effect immediately. - The HL7650 and HL7688 support both UMTS and LTE; the HL7618, HL7618RD, HL7648 and HL7690 only support LTE; and the HL7692 supports both GSM and LTE. - Setting the <mode> of +KSRAT automatically corrects the <band> of *PSRDBS if the two values conflict with each other. For example, when the <band> of *PSRDBS is set to GSM only, changing the <mode> of +KSRAT to LTE only will also correct <band> of *PSRDBS to the original or all LTE bands of *PSRDBS. |

## *PSRDBS Command: Change Frequency Band

| HL7618, HL7618RD, Hl7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT*PSRDBS=? | Response *PSRDBS: (list of supported <mode>s), (list of supported <band>s) OK |
| Read command Syntax AT*PSRDBS? | Response *PSRDBS: <band> OK |
| Write command Syntax AT*PSRDBS= <mode>,<band> | Response OK Parameters <mode> 0 Set <band> at next switch on 1 Set <band> immediately <band> Bit field type parameter. To set several bands, sum up the values 2 GSM 900 MHz (HL7692 only) 8 DCS 1800 MHz (HL7692 only) 32 UMTS Band I (2100 MHz) (HL7650 only) 64 UMTS Band II (1900 MHz) (HL7688 only) 128 UMTS Band V (850 MHz) (HL7650 and HL7688) 512 UMTS Band VIII (900 MHz) (HL7650 only) 4096 LTE Band 2 (1900 MHz) (HL7648 and HL7688) 8192 LTE Band 3 (1800 MHz) (HL7650, HL7690 and HL7692) 16384 LTE Band 4 (1700 MHz) (HL7618, HL7618RD, HL7648 and HL7688) 32768 LTE Band 5 (850 MHz) (HL7650 and HL7688) 131072 LTE Band 13 (700 MHz) (HL7618, HL7618RD and HL7688) 262144 LTE Band 17 (700MHz) (HL7688 only) 524288 LTE Band 28 (700 MHz) (HL7650 only) 16777216 LTE Band 8 (900 MHz) (HL7650, HL7690 and HL7692) 33554432 LTE Band 20 (800 MHz) (HL7690 and HL7692) 67108864 LTE Band 12 (700 MHz) (HL7648 only) |
| Reference Sierra Wireless Proprietary | Notes - Selection can be one or more (up to two) GSM bands, one or more (up to three) UMTS bands, and one or more (up to five) LTE bands. - Setting the <band> of *PSRDBS automatically corrects the <mode> of +KSRAT if the two values conflict with each other. For example, when the <mode> of +KSRAT is GSM only, changing the <band> of *PSRDBS to LTE band only will also correct the <mode> of +KSRAT to LTE only. Likewise, when the <mode> of +KSRAT is GSM only, changing the <band> of *PSRDBS to LTE+GSM bands will also correct <mode> of +KSRAT to dual LTE and GSM mode. |

## +CMEC Command: Mobile Equipment Control Mode

For HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CMEC=? | Response +CMEC: (list of supported <keyp>s),(list of supported <disp>s),(list of supported <ind>s) OK |
| Read command Syntax AT+CMEC? | Response +CMEC: <keyp>,<disp>,<ind> OK |
| Write command Syntax AT+CMEC= [<keyp>[,<disp> [,<ind>]]] | Response OK Parameters <keyp> 0 Keypad management, not significant (no keypad) <disp> 0 Display management, not significant (no display) <ind> 0 Only the ME can set the status of its indicators (command +CIND can only be used to read the indicators) |
| Notes | This command has no effect and was only implemented for compatibility purposes. Parameters are ignored and are not saved in non-volatile memory. |

## +CPOF Command: Power Off

For HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax AT+CPOF | Response OK |
| Notes | - This command powers the module off. It is equivalent to AT+CFUN=0. - “OK” is immediately returned after the power off sequence is started. |

## +KGSMAD Command: GSM/LTE Antenna Detection

For HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KGSMAD=? | Response +KGSMAD: (list of supported <mod>s),(list of supported <urcmode>s),(list of supported <interval>s),(list of supported <detGPIO>s),(list of supported <repGPIO>s) OK |
| Read command Syntax AT+KGSMAD? | Response +KGSMAD: <mod>,<urcmode>,<interval>,<detGPIO>,<repGPIO> OK |
| Write command Syntax AT+KGSMAD= <mod>, [<urcmode> [,<interval> [,<detGPIO> [,<repGPIO>]]]] | Response OK Parameters <mod> 0 Disable antenna detection 1 Periodic antenna detection 2 Instantaneous antenna detection <urcmode> URC presentation mode. This is only applicable if <mod>=1 0 Disable the presentation of antenna detection URC 1 Enable the presentation of antenna detection URC <interval> 45 – 3600 Interval between two detections, in seconds. This is only applicable if <mod>=1. Default value = 120 <detGPIO> 1 – 8, 10, 11, 13 –15 GPIO to be used as input by the antenna detection algorithm. Default value = 5 <repGPIO> 1 – 8, 10, 11, 13 –15 GPIO to be used as output by the antenna detection algorithm to report the antenna’s condition. This is only applicable if <mod>=1. Default value = 7 |
| Notes | - <repGPIO> is set to LOW when the antenna is connected, set to HIGH otherwise. - If the antenna detection algorithm detects a change in the antenna status, the module is notified by URC +KGSMAD: <presence> where <presence>: 0 - antenna connected 1 - antenna connector short circuited to ground 2 - antenna connector short circuited to power 3 - antenna not detected (open) - GPIOs may be already used by +KSIMDET, +KGNSSAD, +KSYNC, and +KTEMPMON; use +KGPIOCFG to check if a GPIO is already used by another interface/command prior to using this command. - Instantaneous activation doesn’t affect a periodic activation that has already been started. |

## +KSREP Command: Mobile Start-up Reporting

For HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KSREP=? | Response +KSREP: (list of supported <mode>s) OK |
| Read command Syntax AT+KSREP? | Response +KSREP: <mode>,<stat>,<PB ready> OK |
| Write command Syntax AT+KSREP= <mode> | Response OK or +CME ERROR: <err> Parameters <mode> Unsolicited result code mode 0 Disable the start-up URC 1 Enable the start-up URC <stat> Module status 0 The module is ready to receive commands for the TE. No access code is required 1 The module is waiting for an access code. Use AT+CPIN? to determine the code 2 The SIM card is not present 3 The module is in “SIM lock” state 4 Unrecoverable error 5 Unknown state <PB ready> Phone book status 0 Phone book is not ready 1 Phone book is ready for read and write |
| Unsolicited Notification | Response +KSUP: <stat> |
| Reference Sierra Wireless Proprietary | Notes - URC +KSUP: <stat> will only be displayed once after reboot if <mode>=1. - If <mode>=0, +PBREADY and +SIM URC notifications will not be sent at the start-up process. However, they will still be sent afterwards during normal module operation. - This command can be used without a SIM. - <mode> is saved in non-volatile memory. |
| Example | // SIM Card is inserted AT+KSREP? +KSREP: 1,0,1 // <mode>=1. The module and phone book are ready OK AT+KSREP=? +KSREP: (0-1) OK AT+KSREP=0 // Set mode to 0 OK AT+KSREP? +KSREP: 0,0,1 // Mode is changed to 0 and save to non-volatile memory OK // Reboot the module AT+KSREP? +KSREP: 0,0,1 // Mode=0 which is restored from non-volatile memory OK // SIM card is not inserted // Reboot the module +SIM: 0 // URC after reboot +KSUP: 2 // Start-up report shows that the SIM is not present AT+KSREP? +KSREP: 1,2,0 // SIM is not present and the phone book is not ready OK +SIM: 1 // Insert SIM card +PBREADY // Phone Book is ready AT+KSREP? +KSREP: 1,0,1 // Start-up reporting is enabled. Both module and phone book // are ready OK |

## +WMANTSEL Command: Select Main / Diversity Antenna for LTE

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+WMANTSEL =? | Response +WMANTSEL: (list of supported <MODE>s) OK |
| Read command Syntax AT+WMANTSEL? | Response +WMANTSEL: <MODE> OK |
| Write Command Syntax AT+WMANTSEL=<MODE> | Response OK Parameter <MODE> Mode of operations for main and diversity antennas For LTE-only variants, <MODE> is coded as a single decimal number <digit-L>; while for variants that support both LTE and UMTS, <MODE> is coded as a 2-digit BCD number [<digit-U>]<digit-L> <digit-L> Digit for LTE 0 Use main and diversity antenna on LTE 1 Only use main antenna on LTE 2 Only use diversity antenna on LTE <digit-U> Digit for UMTS 0 Use main and diversity antenna on UMTS 1 Only use main antenna on UMTS 2 Only use diversity antenna on UMTS |
| Reference Sierra Wireless Proprietary | Notes - This command works with or without a SIM. - <MODE> is stored in non-volatile memory using the AT&W command. - This command should be issued when the device is deregistered from the network; settings will be effective the next time the module registers to the network. |
| Examples | // For variants that only support LTE (e.g. HL7690 and HL7692) at+wmantsel? +WMANTSEL: 0 OK at+cops=2 // Deregister from network OK at+wmantsel=1 // Only select only main antenna OK at+cops=0 // Re-register to network OK at+cops=2 // Deregister from network OK at+wmantsel=2 // Only select diversity antenna OK at+cops=0 // Re-register to network OK // For variants that support both UMTS and LTE (e.g. HL7650 and HL7688) at+wmantsel? // Read command +WMANTSEL: 00 OK at+wmantsel=? // Test command +WMANTSEL: (0-2,00-02,10-12,20-22) OK at+wmantsel=11 // Write command to enable main antenna only for both UMTS // and LTE OK at+wmantsel? // Read back the inputted value +WMANTSEL: 11 OK |

## +KSIMSEL Command: SIM Selection

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KSIMSEL=? | Response +KSIMSEL: (list of supported <mode>s),(list of supported <GPIO>s) OK |
| Read command Syntax AT+KSIMSEL? | Response +KSIMSEL: <mode>[,<GPIO>[,<sim_used>]] OK |
| Write command Syntax AT+KSIMSEL= <mode> [, <GPIO>] | Response OK If <mode> = 4: +KSIMSEL: <mode>,<sim1_pres>,<sim2_pres> OK Parameters <mode> SIM selection mode 0 SIM selection disable 1 Force to select the 1st external SIM. The 2nd external SIM presence will be ignored. 2 Force to select the 2nd external SIM. The 1st external SIM presence will be ignored. 3 Select the 1st external SIM if present, else select the 2nd external SIM if present. 4 Read SIM cards presence status <GPIO> 1 – 8, 10, 11, 13 – 15 GPIO to be used for external SIM selection. Default value = 6. If the value is omitted, the previously configured GPIO will be used. <sim_used> 1 The 1st external SIM currently used 2 The 2nd external SIM currently used <sim1_pres> 0 The 1st external SIM is not present 1 The 1st external SIM is present <sim2_pres> 0 The 2nd external SIM is not present 1 The 2nd external SIM is present |
| Notes | - The firmware supports DSSS – Dual SIM Single Standby. This means that only one SIM can be set as active at a time. - <GPIO> would be low leveled for enabling the 1st external SIM, whereas <GPIO> would be high leveled for enabling the 2nd external SIM. - <sim_used> information is only available when <mode> = 3. - Response [+KSIMSEL: 4,<sim1_pres>,<sim2_pres>] is only available when <mode> = 4. - This command can be used without a SIM. - Parameters <mode> and <GPIO> are saved in non-volatile memory over module reboot. - When SIM select feature is disabled, only the 1st external SIM interface is available and the dedicated GPIO is free for customer use via +KGPIO. - When <mode>=3, SIM selection is performed immediately after the user enters the AT+KSIMSEL command. No SIM selection is performed for SIM insertion or SIM removal afterwards. - Module reboot is needed when the <mode> setting is changed from enabled (<mode> = 1 or 2 or 3) to disabled (<mode> = 0) and vice versa. |
| Examples | AT+KSIMSEL=? // test command +KSIMSEL: (0-4),(1-8,10-11,13-15) OK AT+KSIMSEL? // check current setting +KSIMSEL: 1,6 // 1st SIM active and GPIO 6 is used for SIM selection OK AT+KSIMSEL=2,6 // force to select the 2nd external SIM OK AT+KSIMSEL? +KSIMSEL:2,6 // 2nd SIM active and GPIO 6 is used for SIM selection OK AT+KSIMSEL=1 // force to select the 1st external SIM OK AT+KSIMSEL? +KSIMSEL:1,6 // 1st SIM active and GPIO 6 is used for SIM selection OK AT+KSIMSEL=0 // Disable SIM select functionality OK AT+KSIMSEL=3,6 // Enable SIM select functionality. SIM selection will be // performed. SIM slot status = the 1st SIM is present, the 2nd // SIM is absent OK AT+KSIMSEL? +KSIMSEL: 3,6,1 // SIM selection performed. GPIO 6 is used as selection pin // and the 1st external SIM is currently activated OK AT+KSIMSEL=0 // Disable SIM select functionality OK AT+KSIMSEL=3 // Re-enable SIM select functionality. SIM selection will be // performed. SIM slot status = the 1st SIM is absent, the 2nd // SIM is present OK AT+KSIMSEL? +KSIMSEL: 3,6,2 // SIM selection performed. GPIO 6 is used as selection pin // and the 2nd external SIM is currently activated OK AT+KSIMSEL=4 +KSIMSEL: 4,0,1 // 1st external SIM is absent and 2nd external SIM is present OK |

## +BOOTDWLCFG Command: Boot Configuration for Firmware Download

### Description

This command configures the USB enumeration time out and USB link time out that are used in detecting a firmware download request.

The USB link time out refers to the time out for correct “AT” start frame to start the firmware download procedure.

The flow diagram below shows the sequence in detecting a firmware download request, which always happens when the module boots or reboots.

Firmware Download Request Detection Sequence

Basically, when the module boots, it polls the USB and UART channels in parallel for firmware download conditions.

For the USB channel, the module polls for the following conditions individually:

- USB reset

- USB enumeration

- USB link establishment (“AT” start frame)

For the UART channel, the module polls for a USB link establishment (“AT” start frame) only.

If time out happens, the firmware download detection sequence breaks, and the module either boots normally or it reboots for another retry depending on the AT parameter <DWL_RETRY>.

Time out values are listed in the following table.

Time Out Values

| Time Out | Default Value | Configurable with +BOOTDWLCFG? (Possible Values) |
| --- | --- | --- |
| UART link time out | 150 milliseconds | No |
| USB reset time out | 400 milliseconds | No |
| USB enumeration time out | 3 seconds | Yes (3s, 30s, 60s, 90s) |
| USB link time out | 1 second | Yes (1s, 30s, 60s, 90s) |

If the primary boot image (PSI) is corrupted during firmware download, another set of time out values, extended timeout values, is used. The extended time out values are listed in the following table.

Extended Time Out Values

| Time Out | Value | Configurable |
| --- | --- | --- |
| UART link time out | 30 seconds | No |
| USB reset time out | 3.5 seconds | No |
| USB enumeration time out | 5 seconds | No |
| USB link time out | 30 seconds | No |

If the PSI is corrupted, the module will neither start normal booting nor reboot itself after time out. A successful firmware download is required to recover the module. Moreover, an external hardware reset will be required to start the firmware download again when the time out happens.

However, a PSI image is not usually corrupted because due to its small size (around 60kbytes in one flash block) and upgrade can be completed quickly in writing to the first NAND flash block.

### Syntax

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ BOOTDWLCFG=? | Response +BOOTDWLCFG: (list of supported <USBHS_ENUM_TOUT>s),(list of supported <USBHS_LINK_TOUT>s),(list of supported <DWL_RETRY>s),(list of supported <SYS_REBOOT>s) OK |
| Read command Syntax AT+ BOOTDWLCFG? | Response +BOOTDWLCFG: <USBHS_ENUM_TOUT>,<USBHS_LINK_TOUT>,<DWL_RETRY> OK |
| Write command Syntax AT+ BOOTDWLCFG= [<USBHS_ENUM_TOUT> [,<USBHS_LINK_TOUT> [,<DWL_RETRY> [,<SYS_REBOOT>]]] | Response OK Parameters <USBHS_ENUM_TOUT> USB enumeration time out value 0 3s 1 30s 2 60s 3 90s <USBHS_LINK_TOUT> USB link establishment time out value 0 1s 1 30s 2 60s 3 90s <DWL_RETRY> Desired firmware download retry count when firmware download conditions are not met (i.e. the download program didn’t start) 0 No retry 1 – 10 Number of retries <SYS_REBOOT> System reboot options after executing this command 0 Do not reboot 1 Reboot immediately without network deregistration |
| Notes | - USB time out happens when the USB cable is connected (VBUS level > 0.8V) and USB RESET happens within a 400ms time out. - <USBHS_ENUM_TOUT> and <USBHS_LINK_TOUT> are automatically reset to their default values, 0, in the following conditions: - Cold boot or hardware reset. - Download program received the reset command from the host to reboot the module. - Successfully booted in the module firmware, which means time out values are reset when time out happens in a previous boot. Basically, the conditions above are terminate conditions that time out values are effective only once. - If <DWL_RETRY> is enabled (non-zero value), and firmware download conditions are not met (i.e. download program didn’t start), the module reboots itself with the input parameters <USBHS_ENUM_TOUT> and <USBHS_LINK_TOUT> for the next “TRY” of USB enumeration and USB link establishment. One of the following conditions stops this firmware download retry loop: - Cold boot or hardware reset. - Retry count exhausted (if not configured to be 255/continually). - Download program successfully started. - Any failures related to firmware download, that includes the following will have the module reboot itself with <USBHS_ENUM_TOUT>=3 and <USBHS_LINK_TOUT>=3, regardless of the setting <DWL_RETRY>: - 10 seconds inactivity time out in download program. - Boot failures due to corrupted firmware images, either detected by the primary boot image (PSI) or secondary boot image (SLB). - Any exceptional failures in download program or boot-up images. Five (5) extra seconds of delay happens before the reboot for the second and third conditions. A successful firmware download is required to recover the module. |
| Examples | AT+BOOTDWLCFG=? +BOOTDWLCFG: (0-3),(0-3),(0-10),(0-1) OK //default values after boot-up AT+BOOTDWLCFG? +BOOTDWLCFG: 0,0,0 OK <USB enumeration time-out = 90s, USB link time-out = 90s> AT+BOOTDWLCFG=3,3,0,0 OK AT+BOOTDWLCFG? +BOOTDWLCFG: 3,3 OK AT+BOOTDWLCFG=3,3 or AT+CFUN=1,1 OK //module reboots for Firmware Download <USB enumeration time-out = 3s, USB link time-out = 30s, reboot automatically> AT+BOOTDWLCFG=0,1 OK //module reboots for Firmware Download |

## +CALA Command: Set Alarm

For HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CALA=? | Response +CALA: ("yy/MM/dd,hh:mm:ss"),(list of supported <n>s) OK |
| Read command Syntax AT+CALA? | Response [+CALA: <time>,<n>] OK |
| Write command Syntax AT+CALA= <time>[,<n>] | Response OK or +CME ERROR: <err> Parameters <time> String type valuewith format "yy/MM/dd,hh:mm:ss", where characters indicate year (last two digits), month, day, hour, minutes and seconds <n> Alarm index |
| Unsolicited Notification | Response +CALV: <value> Parameter <value> Alarm state 1 Alarm is enabled |
| Reference Sierra Wireless Proprietary | Notes - When an alarm is timed out, the unsolicited result code is returned. - Only one alarm can be set at a time; <n> must always be 1. - The alarm will wake the module up even if it is already in the off state (e.g., turned off by AT+CPOF or AT+CFUN=0). The module will then boot up normally, and no unsolicited result code +CALV: 1 is returned. - This command can be used without a SIM. - The year “yy” of <time> must be set to 2004 or later. - <time> can only be set to later than the current time on the internal clock. |
| Examples | AT+CCLK=”16/08/26,15:00:00+0” // Set the date and time OK AT+CALA=? // Test command +CALA: ("yy/mm/dd,hh:mm:ss"),(1) OK AT+CALA? // Read command OK AT+CALA=”16/08/26,15:00:35” // Set an alarm for the date and time OK +CALV: 1 // An URC is indicated when the alarm is expired. |

## +CALD Command: Delete Alarm

For HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CALD=? | Response OK |
| Write command Syntax AT+CALD=<n> | Response OK or +CME ERROR: <err> Parameter <n> Alarm index |
| Reference Sierra Wireless Proprietary | Notes - This command can be used without a SIM. - This write command is only effective when the alarm has already been set by AT+CALA. |
| Examples | AT+CALD=? // Test command OK AT+CALD=1 // Delete the alarm OK |

## +KCCINFO Command: Camped Cell Information

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KCCINFO=? | Response +KCCINFO: (list of supported <mode>s) OK |
| Read command Syntax AT+KCCINFO? | Response +KCCINFO: <mode>,<CI>,<RAC>,<TAC> OK |
| Write command Syntax AT+KCCINFO= <mode> | Response OK or +CME ERROR: <err> Parameters <mode> 0 Camped cell parameters change event notification is disabled 1 Camped cell parameters change event notification is enabled <CI> 4-byte location area code in hexadecimal format (e.g. “00C3” equals 195 in decimal) <RAC> 1-byte routing area code in hexadecimal format. FF will be displayed if routing area identity information is invalid. <TAC> 2-byte tracking area code in hexadecimal format (e.g. “00C3” equals 195 in decimal). FFFF will be displayed if tracking area identity information is invalid. |
| Unsolicited Notification | Response +KCCINFOI: <CI>,<RAC>,<TAC> |
| Reference Sierra Wireless Proprietary | Notes - This command is used to enable or disable the unsolicited response which informs about any change in camped cell parameters. - This command works with a SIM card. - <mode> is automatically stored in persistent memory. - Settings take effect immediately. |
| Examples | AT+KCCINFO=1 // Set mode to 1 OK AT+KCCINFO=? // Test command +KCCINFO: (0-1) OK AT+COPS=0 // Attach to network OK +KCCINFOI: "00006773","01","FFFF” // URC display after attached to network AT+KCCINFO? // Read command +KCCINFO: 1,"00006773","01","FFFF" OK |

## +CALM Command: Alert Sound Mode

For HL7648 and HL7688 only.

| HL7648 and HL7688 |  |
| --- | --- |
| Test command Syntax AT+CALM=? | Response +CALM: (list of supported <mode>s) OK |
| Read command Syntax AT+CALM? | Response +CALM: <mode> OK |
| Write command Syntax AT+CALM= <mode> | Response OK Parameter <mode> 0 Normal mode 1 Silent mode (all sounds from the MT are prevented) |
| Reference [27.007] § 8.20 | Examples AT+CALM? +CALM: 0 OK AT+CALM=1 OK AT+CALM=? +CALM: (0-1) OK |

## +CRSL Command: Ringer Sound Level

For HL7648 and HL7688 only.

| HL7648 and HL7688 |  |
| --- | --- |
| Test command Syntax AT+CRSL=? | Response +CRSL: (list of supported <level>s) OK |
| Read command Syntax AT+CRSL? | Response +CRSL: <level> OK |
| Write command Syntax AT+CRSL= <level> | Response OK Parameter <level> Integer type value with manufacturer specific range (smallest value represents the lowest sound level). Possible values = 0 (default), 1, 2, 3. |
| Reference [27.007] § 8.21 | Examples AT+CRSL? +CRSL: 0 OK AT+CRSL=1 OK AT+CRSL=? +CRSL: (0-3) OK |

## +WESHDOWN Command: Emergency Shutdown

For HL7650 only.

| HL7650 |  |
| --- | --- |
| Test command Syntax AT+WESHDOWN=? | Response +WESHDOWN: (list of supported <mode>s), (list of supported <gpio_index>es) OK |
| Read command Syntax AT+WESHDOWN? | Response +WESHDOWN: <mode>[,<gpio_index>] OK |
| Write command Syntax AT+WESHDOWN=<mode> [,<gpio_index>] | Response OK or +CME ERROR <err> Parameters <mode> 0 Disable emergency shutdown feature by GPIO 1 Enable emergency shutdown feature by GPIO 2 Trigger emergency shutdown Due to the quick shutdown of the module, the OK response sent by AT+WESHDOWN=2 might not be received by the application. <gpio_index> 1 – 8 Defines which GPIO will be used as input to trigger the emergency shutdown on the falling edge. Default value = 4 |
| Reference Sierra Wireless Proprietary Command | Notes - <gpio_index> is only used when <mode> = 1. - Parameters are not saved in non-volatile memory. They must be configured each time the module boots up. - GPIOs may already be used by +KSIMDET, +KSYNC, +KTEMPMON, +KGSMAD, +KSIMSLOT, etc. - Only GPIO 4 is available for use in the HL7650. Since this GPIO is also used to detect the insertion/removal of SIM2, this feature is disabled when emergency shutdown is activated. - This command can be used without a SIM. |
| Examples | AT+WESHDOWN=? +WESHDOWN: (0-2),(1-8) OK AT+WESHDOWN? +WESHDOWN: 0 // Emergency shutdown by GPIO is not active OK AT+WESHDOWN=1,4 // Activate emergency shutdown on GPIO4 OK AT+WESHDOWN? +WESHDOWN: 1,4 // A falling edge on GPIO4 will shut the module down OK AT+WESHDOWN=2 OK // Module shuts down |

## +KMCLASS Command: Change GPRS and EGPRS Multislot Class

For HL7618, HL7650, HL7688, HL7690 and HL7692 only.

| HL7618, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KMCLASS=? | Response +KMCLASS: (list of supported <mclass>es) OK |
| Read command Syntax AT+KMCLASS? | Get Multislot Class Response +KMCLASS: <mclass> OK |
| Write command Syntax AT+KMCLASS= <mclass> | Set Multislot Class for GPRS and EGPRS Response OK Parameter <mclass> Multislot class Multislot Class; Maximum Number of Slots. Rx; Tx; Sum. 1; 1; 1; 2. 2; 2; 1; 3. 3; 2; 2; 3. 4; 3; 1; 4. 5; 2; 2; 4. 6; 3; 2; 4. 7; 3; 3; 4. 8; 4; 1; 5. 9; 3; 2; 5. 10; 4; 2; 5. 11; 4; 3; 5. 12 (default); 4; 4; 5. 30; 5; 1; 6. 31; 5; 2; 6. 32; 5; 3; 6. 33; 5; 4; 6. |
| Reference Sierra Wireless Proprietary | Notes - This AT command works with a SIM card inserted in the modem. - <mclass> changes take effect immediately and is automatically stored in non-volatile memory. |

# Network Service Related Commands

## +CAOC Command: Advice of Charge

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CAOC=? | Response +CAOC: (list of supported <mode>s) OK |
| Read command Syntax AT+CAOC? | Response +CAOC: <mode> OK |
| Write command Syntax AT+CAOC= [<mode>] | Response +CAOC: <ccm> OK or +CME ERROR: <err> Parameters <mode> 0 Query CCM value 1 Deactivate unsolicited notification (+CCCM) 2 Activate unsolicited notification <ccm> String type; three bytes of the current call meter value in hexadecimal format |
| Unsolicited Notification | Response +CCCM: <ccm> |

## +CUSD Command: Unstructured Supplementary Service Data

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CUSD=? | Response +CUSD: (list of supported <n>s) OK |
| Read command Syntax AT+CUSD? | Response +CUSD: <n> OK |
| Write command Syntax AT+CUSD=[<n> [,<str>[,<dcs>]]] | Response OK or +CME ERROR: <err> Parameters <n> Enables or disables the presentation of an unsolicited result code 0 Disable the result code presentation to the TE (default value if no parameter) 1 Enable the result code presentation to the TE 2 Cancel session (not applicable to read command response) <str> String type USSD-string (when <str> parameter is not given, network is not interrogated) <dcs> Cell Broadcast Data Coding Scheme in integer format (default value: 0) <m> 0 No further user action required (network initiated USSD-Notify, or no further information needed after mobile initiated operation) 1 Further user action required (network initiated USSD-Request, or further information needed after mobile initiated operation) 2 USSD terminated by network 3 Other local client has responded 4 Operation not supported 5 Network time out |
| Unsolicited Notification | Response +CUSD: <m>[,<str>,<dcs>] |
| Notes | <n> is saved in non-volatile memory per AT port over module reboot. |

## +CLCK Command: Facility Lock

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CLCK=? | Response +CLCK: (list of supported <fac>s) OK or +CME ERROR: <err> |
| Write command Syntax AT+CLCK=<fac>,<mode> [,<passwd> [,<class>]] | Response If <mode> = 2 and command is successful OK +CLCK: <status>[,<class1>[<CR>,<LF> +CLCK: <status>,class2…]] or +CME ERROR: <err> Parameters <fac> Values reserved by the present document: "PS" PH-SIM (lock Phone to SIM/UICC card installed in the currently selected card slot) (MT asks for the password when other than current SIM/UICC card is inserted; MT may remember certain previously used cards thus not requiring password when they are inserted) "SC" SIM (lock SIM/UICC card) (SIM/UICC asks password in MT power-up and when this lock command issued) "AO" BAOC (Barr All Outgoing Calls) "OI" BOIC (Barr Outgoing International Calls) "OX" BOIC-exHC (Barr Outgoing International Calls except to Home Country) "AI" BAIC (Barr All Incoming Calls) "IR" BIC-Roam (Barr Incoming Calls when Roaming outside the home country) "AB" All Barring services (applicable only for mode>=0) "AG" All outgoing barring services (applicable only for <mode>=0) "AC" All incoming barring services (applicable only for <mode>=0) "FD" SIM card or active application in the UICC (GSM or USIM) fixed dialing memory feature (if PIN2 authentication has not been done during the current session, PIN2 is required as <passwd>) "PN" Network Personalization "PU" Network subset Personalization "PP" Service Provider Personalization "PC" Corporate Personalization <mode> 0 Unlock 1 Lock 2 Query status <status> 0 Not active 1 Active <passwd> String type; shall be the same as password specified for the facility from the ME user interface or with command Change Password +CPWD <classx> Sum of integers each representing a class of information (default value=7) 2 Data (refers to all bearer services; with <mode>=2 this may refer only to some bearer service if TA does not support values 16, 32, 64 and 128) 4 Fax (facsimile services) 8 Short message service 16 Data circuit sync 32 Data circuit async 64 Dedicated packet access 128 Dedicated PAD access |

## +CNUM Command: Subscriber Number

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CNUM=? | Response OK |
| Execute command Syntax AT+CNUM | Response +CNUM: [<alpha1>],<number1>,<type1>[,<speed>,<service>[,<itc>]][<CR><LF> +CNUM: [<alpha2>],<number2>,<type2>[,<speed>,<service>[,<itc>]][...]] OK or +CME ERROR: <err> Parameters <alphax> Optional alphanumeric string associated with <numberx>; used character set should be the one selected with command +CSCS <numberx> String type phone number of format specified by <typex> <typex> Type of address octet in integer format <speed> As defined in 27.007 sub clause 6.7, corresponding to +CBST setting <service> Service related to the phone number 0 Asynchronous modem 1 Synchronous modem 2 PAD Access (asynchronous) 3 Packet Access (synchronous) 5 Fax <itc> Information transfer capability 0 3.1kHz 1 UDI |
| Note | MSISDN information should be stored in the SIM card before using +CNUM. |

## +COLP Command: Connected Line Identification Presentation

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+COLP=? | Response +COLP: (list of supported <n>s) OK |
| Read command Syntax AT+COLP? | Response +COLP: <n>,<m> OK |
| Execute command Syntax AT+COLP=[<n>] | Response OK or +CME ERROR: <err> Parameters <n> 0 Disable result code presentation status to the TE 1 Enable result code presentation status to the TE <m> 0 COLP not provisioned 1 COLP provisioned 2 Unknown (e.g. no network, etc.) |
| Notes | - If the connected line identity of the called party is enabled, (and called subscriber allows it), the intermediate result code +COLP: <number>, <type> [,<subaddr>, <satype> [, <alpha>]] is returned from TA to TE. - <n> is saved in non-volatile memory per AT port over module reboot. |

## +COPN Command: Read Operator Name

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+COPN=? | Response OK |
| Execute command Syntax AT+COPN | Response +COPN: <numeric1>,<alpha1>[<CR><LF> +COPN: <numeric2>,<alpha2> [...]] OK or +CME ERROR: <err> Parameters <numeric> String type; operator in numeric format (see +COPS) <alpha> String type; operator in long alphanumeric format (see +COPS) |
| Notes | If the matching PLMN name is not found then the numeric PLMN ID (MCCMNC) will be displayed. |

## +COPS Command: Operator Selection

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+COPS=? | Response +COPS: [list of supported (<stat>, long alphanumeric <oper>, short alphanumeric <oper>, numeric <oper>[,< AcT>,<plmn_list>)s][,,(list of supported <mode>s),(list of supported <format>s)] OK or +CME ERROR: <err> |
| Read command Syntax AT+COPS? | Response +COPS: <mode>[,<format>,<oper>[,<AcT>]] OK or +CME ERROR: <err> |
| Write command Syntax AT+COPS= [<mode> [,<format> [,<oper> [,< AcT>]]]] | Response OK or +CME ERROR: <err> Parameters <mode> 0 Automatic; in this case other fields are ignored and registration is done automatically by ME 1 Manual (other parameters like format and operator need to be passed) 2 Deregister from network 3 Sets <format> value. In this case <format> becomes a mandatory input 4 Manual/automatic; if manual selection fails then automatic mode is entered <format> 0 Long alphanumeric; if network name is not available it displays a combination of MCC and MNC in string format 1 Short alphanumeric 2 Numeric <oper> String type given in format <format>; this field may be up to 16-character long for long alphanumeric format, up to 8 characters for short alphanumeric format and 5 characters long for numeric format (MCC/MNC codes) <stat> 0 Unknown networks 1 Network available 2 Current (registered) 3 Forbidden network <AcT> 0 GSM 2 UMTS 7 LTE <plmn_list> 0 PLMN is present on the EHPLMN list 1 PLMN is present on the user-controlled PLMN list 2 PLMN is present on the operator-controlled PLMN list Note that this parameter only supports R7 Protocol Stack onwards. |
| Notes | - This command forces an attempt to select and register the GSM, UMTS network. - Set command sets automatic network selection or selects network and a certain access technology AcT. - Read command returns current network. - Test command returns available networks and lists of supported <mode>s and <format>s. - This command is abortable. The port shall be freed for issuing another command. No network abort shall be triggered. - <mode>=0,1,2,4 and <oper> are saved in non-volatile memory over module reboot. - <format> is saved in non-volatile memory per AT port over module reboot. - +CME ERROR: 22 is returned if AT+COPS=0 is sent while data transmission is ongoing. |

## +CPOL Command: Preferred PLMN List

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CPOL=? | Response +CPOL: (list of supported <index>es),(list of supported <format>s) OK or +CME ERROR: <err> |
| Read command Syntax AT+CPOL? | Response +CPOL: <index1>,<format>,<oper1>[,<GSM_AcT1>,<GSM_Compact_AcT1>, <UTRAN_AcT1>,<EUTRAN_AcT>][<CR><LF> +CPOL: <index2>,<format>,<oper2>[,<GSM_AcT2>,<GSM_Compact_AcT2>, <UTRAN_AcT2>,<EUTRAN_AcT>][...]] OK or +CME ERROR: <err> |
| Write command Syntax AT+CPOL= [<index>] [,<format> [,<oper> [,<GSM_AcT>, <GSM_Compact_AcT>,<UTRAN_ AcT>,<EUTRAN_ AcT>]]] | Response OK or +CME ERROR: <err> Parameters <index> Integer type; order number of operator in the SIM/USIM preferred operator list <format> 0 Long format alphanumeric <oper> 1 Short format alphanumeric <oper> 2 Numeric <oper> <opern> String type; <format> indicates if the format is alphanumeric or numeric <GSM_AcT> 0 GSM access technology not selected 1 GSM access technology selected <GSM_Comp_AcT> 0 GSM compact access technology not selected 1 GSM compact access technology selected <UTRA_AcT> 0 UTRA access technology not selected 1 UTRA access technology selected <EUTRA_AcT> 0 EUTRA access technology not selected 1 EUTRA access technology selected |
| Notes | - The read command can have “n” RAT values. - If matching PLMN name is not found, then numeric PLMN ID (MCCMNC) will be displayed. - <format> is saved in non-volatile memory over module reboot. |

## +CPWD Command: Change Password

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CPWD=? | Response +CPWD: list of supported (<fac>,<pwdlength>)s OK |
| Write command Syntax AT+CPWD= <fac>,<oldpwd>, <newpwd> | Response OK or +CME ERROR: <err> Parameters <fac> "PS" PH-SIM (lock Phone to SIM/UICC card installed in the currently selected card slot) (MT asks for the password when other than current SIM/UICC card is inserted; MT may remember certain previously used cards thus not requiring password when they are inserted) "SC" SIM (lock SIM/UICC card) (SIM/UICC asks password in MT power-up and when this lock command issued) "AO" BAOC (Barr All Outgoing Calls) "OI" BOIC (Barr Outgoing International Calls) "OX" BOIC-exHC (Barr Outgoing International Calls except to Home Country) "AI" BAIC (Barr All Incoming Calls) "IR" BIC-Roam (Barr Incoming Calls when Roaming outside the home country) "FD" SIM card or active application in the UICC (GSM or USIM) fixed dialing memory feature (if PIN2 authentication has not been done during the current session, PIN2 is required as <passwd>) "PN" Network Personalization "PU" Network subset Personalization "PP" Service Provider Personalization "PC" Corporate Personalization <oldpwd> String type containing the old password <newpwd> String type containing the new password <pwdlength> Length of password |

## +CREG Command: Network Registration

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CREG=? | Response +CREG: (list of supported <n>s) OK |
| Read command Syntax AT+CREG? | Response +CREG: <n>,<stat>[,<lac>,<ci>[,<AcT>]] OK |
| Write command Syntax AT+CREG=[<n>] | Response OK or +CME ERROR: <err> Parameters <n> 0 Disable network registration unsolicited result code 1 Enable network registration unsolicited result code +CREG: <stat> 2 Enable network registration and location information unsolicited result code +CREG: <stat>[,<lac>,<ci>[,<Act>]] <stat> 0 Not registered, ME is not currently searching a new operator to register to 1 Registered, home network 2 Not registered, but ME is currently searching a new operator to register to 3 Registration denied 4 Unknown 5 Registered, roaming <lac> String type; two-byte location area code in hexadecimal format (e.g. "00C3" equals 195 in decimal) <ci> String type; four-byte UTRAN/E-UTRAN cell ID in hexadecimal format <AcT> 0 GSM 2 UTRAN 3 GSM with EGPRS 4 UTRAN with HSDPA 5 UTRAN with HSUPA 6 UTRAN with HSDPA and HSUPA 7 E-UTRAN |
| Unsolicited Notification | Response When <n>=1 and there is a change in the ME network registration status code: +CREG: <stat> When <n>=2 and there is a change in the network cell: +CREG: <stat>[,<lac>,<ci>[,<Act>]] |
| Notes | <n> is saved in non-volatile memory per AT port over module reboot. |

## +CSSN Command: Supplementary Service Notification

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CSSN=? | Response +CSSN: (list of supported <n>s), (list of supported <m>s) OK |
| Read command Syntax AT+CSSN? | Response +CSSN: <n>,<m> OK |
| Write command Syntax AT+CSSN=[<n> [,<m>]] | Response OK or +CME ERROR: <err> Parameters <n> 0 Disable +CSSI result code presentation status to the TE 1 Enable +CSSI result code presentation status to the TE <m> 0 Disable +CSSU result code presentation status to the TE 1 Enable +CSSU result code presentation status to the TE |
| Unsolicited Notification | Response +CSSI: <code1>[,<index>] +CSSU: <code2>[<index> [,<number>,<type>]] Parameters <code1> 0 Unconditional call forwarding is active 1 Some of the conditional call forwarding are active 2 Call has been forwarded 3 Call is waiting 4 This is a CUG call (also <index> present) 5 Outgoing calls are barred 6 Incoming calls are barred 7 CLIR suppression rejected 8 Call has been deflected <index> 0 – 9 Index 10 No index (prefer to take from subscriber data) <code2> 0 This is a forwarded call (MT call setup) 1 This is a CUG call (<index> present) (MT call setup) 6 Forward check SS message received (can be received whenever) 8 Call has been connected with the other remote party in explicit call transfer operation (during an MT call setup) 9 This is a deflected call (MT call setup) 10 Additional incoming call forwarded <number> String type phone of format specified by <type> <type> Type of address octet in Integer format |
| Notes | <n> and <m> are saved in non-volatile memory per AT port over module reboot. |

## +CPLS Command: Select Preferred PLMN List

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CPLS=? | Response +CPLS: (list of supported <cpls_list>s) OK |
| Read command Syntax AT+CPLS? | Response +CPLS: <cpls_list> OK |
| Write command Syntax AT+CPLS= [<cpls_list>] | Response OK or +CME ERROR: <err> Parameter <cpls_list> 0 User controlled PLMN selector with access technology EFPLMNwAcT, but iff not found in the SIM/UICC, then the PLMN preferred list is EFPLMNsel 1 Operator controlled PLMN selector with access technology EFOPLMNwAcT 2 HPLMN selector with access technology EFHPLMNwAcT |

## +CEREG Command: EPS Network Registration Status

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CEREG=? | Response +CEREG: (list of supported <n>s) OK |
| Read command Syntax AT+CEREG? | Response +CEREG: <n>,<stat>[,<tac>,<ci>[,<AcT>]] OK |
| Execute command Syntax AT+CEREG= [<n>] | Response OK or +CME ERROR: <err> Parameters <n> 0 Disable network registration unsolicited result code 1 Enable network registration unsolicited result code +CEREG: <stat> 2 Enable network registration unsolicited result code +CEREG: <stat> [,<tac>,<ci>[,<AcT>]] <stat> 0 Not registered, MT is not currently searching an operator to register to 1 Registered on the home network 2 Not registered, but MT is currently trying to attach or searching for an operator to register to 3 Registration denied 4 Unknown 5 Registered, roaming 8 Attached for emergency bearer services only (note that this is only available when <AcT> = 2,4,5,6 <tac> String type; two-byte tracking area code in hexadecimal format (e.g. “00C3” is equals to 195 in decimal) <ci> String type; four-byte UTRAN/E-UTRAN cell ID in hexadecimal format <AcT> 2 UTRAN 4 UTRAN with HSDPA 5 UTRAN with HSUPA 6 UTRAN with HSDPA and HSUPA 7 E-UTRAN |
| Notes | <n> is saved in non-volatile memory per AT port over module reboot. |

## +CEMODE Command: UE Modes of Operation for EPS

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CEMODE=? | Response +CEMODE: (list of supported <mode>s) OK |
| Read command Syntax AT+CEMODE? | Response +CEMODE: <mode> OK |
| Write command Syntax AT+CEMODE= [<mode>] | Response OK or +CME ERROR: <err> Parameter <mode> Indicates mode of operation 0 PS mode 2 of operation (default value for HL7618 and HL7618RD) 1 CS/PS mode 1 of operation (default value for HL7650, HL7688 and HL7692) 2 CS/PS mode 2 of operation (default value for HL7648 and HL7690) 3 PS mode 1 of operation |
| Notes | <mode> is saved in non-volatile memory over module reboot. |

## +KAAT Command: GPRS Automatic Attach

For HL7618, HL7618RD, HL7648, HL7650, HL7688 and HL7690 only.

| HL7618, HL7618RD, HL7648, HL7650, HL7688 and HL7690 |  |
| --- | --- |
| Test command Syntax AT+KAAT=? | Response +KAAT: (list of supported <attach mode>s) OK |
| Read command Syntax AT+KAAT? | Get current mode Response +KAAT: <attach mode> OK |
| Write command Syntax AT+KAAT= <attach mode> | Set attach mode Response OK Parameter <attach mode> 0 No GPRS automatic attach at switch on 1 GPRS automatic attach at switch on |
| Reference Sierra Wireless Proprietary | Notes - The write command is used to select the GPRS attach mode at ME switch on. - This AT command works with a SIM card. - <attach mode> is automatically stored in non-volatile memory. |
| Example | <start up with no SIM card> AT+KAAT? +CME ERROR: 10 <Insert SIM card and reset> <default mode is GPRS automatic attach at switch on> AT+KAAT? +KAAT: 1 OK AT+CGATT? +CGATT: 1 OK <set no GPRS automatic attach at switch on> AT+KAAT=0 OK AT+CGATT? +CGATT: 1 OK <reset> AT+CGATT? +CGATT: 0 OK AT+KAAT? AT+KAAT: 0 OK AT+CGATT=1 OK AT+CGATT? +CGATT: 1 OK <reset> AT+KAAT? +KAAT: 0 OK AT+CGATT? +CGATT: 0 OK |

# Phone Book Management

## +CPBF Command: Find Phonebook Entries

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CPBF=? | Response +CPBF: [<nlength>],[<tlength>],[<glength>],[<slength>],[<elength>] OK or +CME ERROR: <err> |
| Write command Syntax AT+CPBF= <findtext> | Response [+CPBF:<index1>,<number>,<type>,<text>[,<hidden>][,<group>][,<adnumber>] [,<adtype>][,<secondtext>][,<email>]] OK or +CME ERROR: <err> Parameters <index1>, <index2> , <index> Integer type values in the range of location numbers of phonebook memory <number> String type phone number of format <type> <type> Type of address octet in integer format <text> String type field of maximum length <tlength>; character set as specified by command +CSCS <group> String type field of maximum length <glength>; character set as specified by command +CSCS <adnumber> String type phone number of format <adtype> <adtype> Type of address octet in integer format <secondtext> String type field of maximum length <slength>; character set as specified by command +CSCS <email> String type field of maximum length <elength>; character set as specified by command +CSCS <nlength> Integer type value indicating the maximum length of field <number> <tlength> Integer type value indicating the maximum length of field <text> <glength> Integer type value indicating the maximum length of field <group> <slength> Integer type value indicating the maximum length of field <secondtext> <elength> Integer type value indicating the maximum length of field <email> <hidden> Indicates if the entry is hidden or not 0 Phonebook entry not hidden 1 Phonebook entry hidden |
| Notes | Execution command returns phonebook entries (from the current phonebook memory storage selected with +CPBS). |

## +CPBR Command: Read Current Phonebook Entries

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CPBR=? | Response +CPBR: (list of supported <index>es),[<nlength>],[<tlength>],[<glength>],[<alength>], [<slength>],[<elength>] OK |
| Write command Syntax AT+CPBR= <index1> [,<index2>] | Response [+CPBR: <index1>,<number>,<type>,<text>[,<hidden>][,<group>][,<adnumber>] [,<adtype>][,<secondtext>][,<email>]][[...] OK or +CME ERROR: <err> Parameters <index1>, <index2>, <index> Integer type values in the range of location numbers of phonebook memory <number> String type phone number of format <type> <type> Type of address octet in integer format <text> String type field of maximum length <tlength> <hidden> Indicates if the entry is hidden or not – only available if a UICC with an active USIM application is present 0 Phonebook entry not hidden 1 Phonebook entry hidden <group> String type field of maximum length <glength> <adnumber> String type phone number of format <adtype> <adtype> Type of address octet in integer format <secondtext> String type field of maximum length <slength> <email> String type field of maximum length <elength> <nlength> Integer type value indicating the maximum length of field <number> <tlength> Integer type value indicating the maximum length of field <text> <glength> Integer type value indicating the maximum length of field <group> <alength> Integer type value indicating the maximum length of field <adnumber> <slength> Integer type value indicating the maximum length of field <secondtext> <elength> Integer type value indicating the maximum length of field <email> |
| Notes | - Optional parameters <nlength>, <tlength>, <glength>, <alength>, <slength>, <elength> are only applicable for 3G UICC. - Execution command returns phonebook entries in location number range <index1>...<index2> from the current phonebook memory storage selected with +CPBS. |

## +CPBS Command: Select Phonebook Memory Storage

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CPBS=? | Response +CPBS: (list of supported <storage>s) OK |
| Read command Syntax AT+CPBS? | Response +CPBS: <storage>[,<used>,<total>] OK or +CME ERROR:<err> |
| Write command Syntax AT+CPBS= <storage> [,<password>] | Response OK or +CME ERROR:<err> Parameters <storage> "FD" SIM/USIM fixdialing phonebook "LD" SIM/UICC last dialing phonebook (LD phonebook can't be deleted) "ON" SIM (or MT) own numbers (MSISDNs) list (reading of this storage may be available through +CNUM also) "SM" SIM/UICC phonebook (default) "BL" Blacklist phonebook (delete only) "EN" SIM emergency-call-codes phonebook (read only) "AP" Selected application phonebook "BN" SIM barred-dialing-number (EF_BDN) phonebook (only valid with PIN2) "SN" SIM service-dialing-number (EF_SDN) phonebook (read only) <password> String type value representing the PIN2-code required when selecting PIN2 code locked <storage>s above <used> Integer type value indicating the number of used locations in the selected memory <total> Integer type value indicating the total number of locations in the selected memory |
| Notes | Set command selects phonebook memory storage <storage>, which is used by other phonebook commands. |

## +CPBW Command: Write Phonebook Entry

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CPBW=? | Response +CPBW: (list of supported <index>es),[<nlength>],(list of supported <type>s),[<tlength>], [<glength>],[<alength>],[<slength>],[<elength>] OK |
| Read command Syntax AT+CPBW? | Response +CPBW: <written_index> OK or +CPBW:-1 OK |
| Write command Syntax AT+CPBW= [<index>] [,<number> [,<type>[,<text> [,<group> [,<adnumber> [,<adtype> [,<secondtext> [,<email> [,<hidden>]]]]]]]]] | Response +CPBW: <written_index> OK or +CME ERROR: <err> Parameters <index> Integer type values in the range of location numbers of phonebook memory <number> String type phone number of format <type> <type> Type of address octet in integer format; default value is 145 when dialing string includes international access code character "+"; otherwise, default value is 129 <text> String type field of maximum length <tlength> <hidden> Indicates if the entry is hidden or not – only available if a UICC with an active USIM application is present 0 Phonebook entry not hidden 1 Phonebook entry hidden <group> String type field of maximum length <glength> <adnumber> String type phone number of format <adtype> <adtype> Type of address octet in integer format <secondtext> String type field of maximum length <slength> <email> String type field of maximum length <elength> <nlength> Integer type value indicating the maximum length of field <number> <tlength> Integer type value indicating the maximum length of field <text> <glength> Integer type value indicating the maximum length of field <group> <alength> Integer type value indicating the maximum length of field <adnumber> <slength> Integer type value indicating the maximum length of field <secondtext> <elength> Integer type value indicating the maximum length of field <email> |
| Notes | - Optional parameters <nlength>, <tlength>, <glength>, <alength>, <slength>, <elength> are only applicable for 3G UICC. - Execution command writes phonebook entry in location number <index> in the current phonebook memory storage selected with +CPBS. |

## +PBREADY URC: Phonebook Ready

+PBREADY URC will be displayed when the phone book is ready for read and write operation on boot-up or upon insertion of a valid SIM card.

# SMS Commands

For other information regarding HL7618 and HL7618RD SMS commands, refer to section 1.5 SMS Commands.

## Parameters Definition

The following parameters are used in the subsequent clauses which describe all commands. The formats of integer and string types referenced here are defined in V.25ter.

The default values are for command parameters, not for result code parameters.

### Message Storage Parameters

<index>	Integer type; value in the range of location numbers supported by the associated memory

<mem1>	String type; memory from which messages are read and/or deleted (by commands 			+CMGL, +CMGR and +CMGD); defined values are as follows:

"BM"		Broadcast message storage

"ME"		ME message storage

"MT"		Any of the storages associated with ME

"SM"		(U)SIM message storage; default value

"TA"		TA message storage

"SR"		Status report storage

<mem2>	String type; memory to which writing and sending operations are made (commands Send 		Message from Storage +CMSS and Write Message to Memory +CMGW); refer <mem1> 		for defined values. Default value is “SM”.

<mem3>	String type; preferred memory to which received SMs are to be stored (unless forwarded 		directly to TE; refer command New Message Indications +CNMI); refer <mem1> for 			defined values; received CBMs are always stored in "BM" (or some manufacturer specific 		storage) unless directly forwarded to TE; received status reports are always stored in 			"SR" (or some manufacturer specific storage) unless directly forwarded to TE. Default 			value is “SM”.

<stat>		Status of message in memory. Integer type in PDU mode, or string type in text mode. 			Available values are as follows:

0	"REC UNREAD"		Received unread message (i.e. new message)

1	"REC READ" 		Received read message

2	"STO UNSENT" 		Stored unsent message (only applicable to SMs)

3	"STO SENT" 		Stored sent message (only applicable to SMs)

4	"ALL" 				All messages (only applicable to +CMGL command)

<total1>	Integer type; total number of message locations in <mem1>

<total2>	Integer type; total number of message locations in <mem2>

<total3>	Integer type; total number of message locations in <mem3>

<used1>	Integer type; number of messages currently in <mem1>

<used2>	Integer type; number of messages currently in <mem2>

<used3>	Integer type; number of messages currently in <mem3>

### Message Data Parameters

<ackpdu>	RP-User-Data element of RP-ACK PDU; format is same as for <pdu> in case of SMS, 			but without SC address field and parameter shall be bounded by double quote characters 		like a normal string type parameter.

<alpha>	String type alphanumeric representation of <da> or <oa> corresponding to the entry 			found in MT phonebook; implementation of this feature is manufacturer specific; used 			character set should be the one selected with command +CSCS.

<cdata>	Command data in text mode responses; ME/TA converts each 8-bit octet into two	IRA 			character long hexadecimal number (e.g. octet with integer value 42 is presented to TE 			as two characters 2A (IRA 50 and 65)).

<ct>		Command type in integer format (default value = 0).

<da>	Address value in string format. BCD numbers (or GSM 7-bit default alphabet characters) are converted to characters of the currently selected TE character set (refer to command +CSCS). Type of address is given by <toda>.

<data>	In the case of user data in text mode responses; format:

if <dcs> indicates that GSM 7-bit default alphabet is used and <fo> indicates that user data header indication is not set

  - if TE character set other than "HEX" (refer to command +CSCS): ME/TA converts GSM alphabet into current TE character set

  - if TE character set is "HEX": ME/TA converts each 7-bit character of GSM 7-bit default alphabet into two IRA character long hexadecimal number (e.g. character  (GSM 7-bit default alphabet 23) is presented as 17 (IRA 49 and 55))

if <dcs> indicates that 8-bit or UCS2 data coding scheme is used, or <fo> indicates that user data header indication is set: ME/TA converts each 8-bit octet into two IRA character long hexadecimal number (e.g. octet with integer value 42 is presented to TE as two characters 2A (IRA 50 and 65))

In the case of CBS: CBM Content of Message in text mode responses; format:

if <dcs> indicates that GSM 7-bit default alphabet is used

  - if TE character set other than "HEX" (refer to command +CSCS); ME/TA converts GSM alphabet into current TE character set

  - if TE character set is "HEX"; ME/TA converts each 7-bit character of the GSM 7-bit default alphabet into two IRA character long hexadecimal number

if <dcs> indicates that 8-bit or UCS2 data coding scheme is used; ME/TA converts each 8-bit octet into two IRA character long hexadecimal number

<length>	Integer type vlayue indicating the length of the actual TP data unit in octets in PDU mode. 		This is 140 characters long according to 8-bit GSM coding scheme.

In text mode, the maximum length of an SMS depends on the used coding scheme (160 			characters if 7-bit).

<mid>		CBM Message Identifier in integer format

<mn>		TP-Message-Number in integer format

<mr>		Message reference in integer format

<oa>		Origiantion address address value field in string format; BCD numbers (or GSM 7-bit			default alphabet characters) are converted to characters of the 	currently selected TE 			character set (refer to command +CSCS); type of address given by <tooa>

<page>	CBM Page Parameter bits 4-7 in integer format

<pages>	CBM Page Parameter bits 0-3 in integer format

<pdu>	GSM 04.11 SC address followed by GSM 03.40 TPDU in hexadecimal format

In the case of CBS, TPDU in hexadecimal format

<pid>		Protocol identifier in integer format. Default value is 0

<ra>		Recipient address address value in string format; BCD numbers (or GSM 7-bit default 			alphabet characters) are converted to characters of the currently selected TE character 			set (refer to command +CSCS); type of address is given by <tora>

<sca>		String value enclosed in quotes indicating the service center address. Note that BCD 			numbers are converted to characters; type of address is given by <tosca>

<scts>	Service centre time stamp in time-string format (refer to <dt>)

<sn>		CBM Serial Number in integer format

<st>		Status in integer format

<toda>	Type of address octet in integer format. Default value is 145 if the first character of <da> 			is “+”; otherwise, default value is 129

<tooa>	Originating address type of address octet in integer format (refer to <toda> for the default 		value)

<tora>	Recipient address type of address octet in integer format (refer to <toda> for the default 			value)

<tosca>	SC address type of address octet in integer format (refer to <toda> for the default value)

<vp>		Depending on SMS-SUBMIT <fo> setting: TP-Validity-Period either in integer format 			(default value = 167) or in time-string format (refer to <dt>)

<vp>		Validity period in either integer format (default value = 167) or in time-string format 			depending on <fo> settings

<dcs>		SMS Data Coding Scheme (default value = 0), or Cell Broadcast Data Coding Scheme in 		integer format

<dt>		Discharge time in time-string format “yy/MM/dd,hh:mm:ss+zz” where the characters 			indicate year, month, day, hour, minutes, seconds and time zone.

For example, May 6, 1994, 10:10 pm GMT+2 hours is equals to “94/05/06,22:10:00+08”

<fo>		First octet of SMS-DELIVER, SMS-SUBMIT (default value = 17), SMS-STATUS-				REPORT, or SMS-COMMAND (default value = 2) in integer format depending on 				command or result code

## +CMGD Command: Delete Message

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CMGD=? | Response +CMGD: (list of supported <index>es)[,(list of supported <delflag>s)] OK |
| Write command Syntax AT+CMGD= <index> [,<delflag>] | Response OK or +CMS ERROR: <err> Parameters <delflag> Integer indicating multiple message deletion request 0 (or omitted) Delete the message specified in <index> 1 Delete all read messages from preferred message storage, leaving unread messages and stored mobile originated messages (whether sent or not) untouched 2 Delete all read messages from preferred message storage and sent mobile originated messages, leaving unread messages and unsent mobile originated messages untouched 3 Delete all read messages from preferred message storage, sent and unsent mobile originated messages leaving unread messages untouched 4 Delete all messages from preferred message storage including unread messages |
| Notes | Execution command deletes message from preferred message storage <mem1>, location <index>. If <delflag> is present and not set to 0 then the ME shall ignore <index> and follow the rules for <delflag> shown above. |

## +CMGF Command: Set Message Format

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CMGF=? | Response +CMGF: (list of supported <mode>s) OK |
| Read command Syntax AT+CMGF? | Response +CMGF: <mode> OK |
| Execute command Syntax AT+CMGF= [<mode>] | Response OK or +CMS ERROR: err> Parameter <mode> 0 PDU mode (default when implemented) 1 Text mode |
| Notes | <mode> is saved in non-volatile memory per AT port over module reboot. |

## +CMGL Command: List Messages

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CMGL=? | Response +CMGL: (list of supported <stat>s) OK |
| Execute command Syntax AT+CMGL [=<stat>] | Response If in text mode, command is successful and SMS-SUBMITs and/or SMS-DELIVERs: +CMGL: <index>,<stat>, <oa/da>,[<alpha>], [<scts>][,<tooa/toda>,<length>] <CR><LF><data>[<CR><LF> +CMGL: <index>,<stat>, <da/oa>,[<alpha>], [<scts>][,<tooa/toda>, <length>] <CR><LF><data> [...]] If in text mode, command is successful and SMS-STATUS-REPORTs: +CMGL: <index>, <stat>,<fo>, <mr>, [<ra>], [<tora>], <scts>, <d-t>,<st>[<CR><LF> +CMGL: <index>, <stat>, <fo>, <mr>,[<ra>], [<tora>],<scts>,<d_t>,<st>[...]] If in text mode, command is successful and SMS-COMMANDs: +CMGL: <index>,<stat>,<fo>,<ct> [<CR><LF> +CMGL: <index>,<stat>, <fo>,<ct>[...]] If in text mode, command is successful and CBM storage: +CMGL : <index>,<stat>,<sn>, <mid>, <page>,<pages> <CR><LF><data>[<CR><LF> +CMGL : <index>,<stat>,<sn>, <mid>,<page>,<pages><CR><LF><data>[...]] If in PDU mode and command is successful: +CMGR: <stat>,[<alpha>],<length><CR><LF><pdu> or +CMS ERROR: <err> Parameters For parameter information and values, refer to section 8.1 Parameters Definition. |

## +CMGR Command: Read Message

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CMGR=? | Response OK |
| Write command Syntax AT+CMGR= <index> | Response If text mode (+CMGF=1), command is successful, and SMS-DELIVER: +CMGR: <stat>,<oa>,[<alpha>],<scts>[,<tooa>,<fo>,<pid>,<dcs>,<sca>,<tosca>, <length>]<CR><LF><data> if text mode (+CMGF=1), command is successful, and SMS-SUBMIT: +CMGR: <stat>,<da>,[<alpha>][,<toda>,<fo>,<pid>,<dcs>,[<vp>],<sca>,<tosca>, <length>]<CR><LF><data> if text mode (+CMGF=1), command is successful, and SMS-STATUS-REPORT: +CMGR: <stat>,<fo>,<mr>,[<ra>], [<tora>],<scts>,<d_t>,<st> if text mode (+CMGF=1), command is successful, and SMS-COMMAND: +CMGR: <stat>, <fo>,<ct>[, <pid>,[<mn>],[<da>],[<toda>],<length><CR><LF><cdata>] if text mode (+CMGF=1), command is successful, and CBM storage: +CMGR: <stat>,<sn>, <mid>,<dcs>,<page>,<pages><CR><LF><data> if PDU mode (+CMGF=0) and command is successful: +CMGR: <stat>,[<alpha>],<length><CR><LF><pdu> or +CMS ERROR: <err> Parameters For parameter information and values, refer to section 8.1 Parameters Definition. |

## +CMGS Command: Send Message

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CMGS=? | Response OK |
| Write command Syntax If text mode (+CMGF=1): AT+CMGS=<da> [,<toda>]<CR> text is entered <ctrl-Z/ESC> If PDU mode (+CMGF=0): AT+CMGS= <length><CR> PDU is given <ctrl-Z/ESC> | Response If text mode (+CMGF=1) and sending is successful: [+CMGS: <mr>[,<scts>]] OK if PDU mode (+CMGF=0) and sending is successful: [+CMGS: <mr>] OK or +CMS ERROR: <err> Parameters For parameter information and values, refer to section 8.1 Parameters Definition. |
| Notes | - The TA shall send a four-character sequence: <CR><LF><greater_than><space> (IRA 13, 10, 62, 32) after command line is terminated with <CR>; after that PDU can be given from TE to ME/TA. - The PDU shall be hexadecimal format (similarly as specified for <pdu>) and given in one line; ME/TA converts this coding into the actual octets of PDU. - When the length octet of the SMSC address (given in the PDU) equals zero, the SMSC address set with command Service Centre Address +CSCA is used; in this case the SMSC Type-of-Address octet shall not be present in the PDU, i.e. TPDU starts right after SMSC length octet. - Sending can be cancelled by giving <ESC> character. - <ctrl-Z> must be used to indicate the ending of PDU. - +CMGS: <mr>[,<scts>] is not available in +CMGS intermediate response as SMS is sent over IMS using 3GPP2 SMS PDU format and protocol. |

## +CMGW Command: Write Message to Memory

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CMGW=? | Response OK |
| Write command Syntax If text mode (+CMGF=1): AT+CMGW[= <oa/da> [,<tooa/toda> [,<stat>]]]<CR> text is entered <ctrl-Z/ESC> If PDU mode (+CMGF=0): AT+CMGW= <length>[,<stat>]<CR> PDU is given <ctrl-Z/ESC> | Response +CMGW: <index> OK or +CMS ERROR: <err> Parameters For parameter information and values, refer to section 8.1 Parameters Definition. |
| Notes | - Execution command stores a message to memory storage <mem2>, and memory location <index> of the stored message is returned. - By default, message status will be set to 'stored unsent', but parameter <stat> also allows other status values to be given. (ME/TA manufacturer may choose to use different default <stat> values for different message types.) - Entering of PDU is done similarly as specified in command +CMGS. |

## +CMSS Command: Send Message from Storage

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CMSS=? | Response OK |
| Write command Syntax AT+CMSS= <index>[,<da> [,<toda>]] | Response If text mode (+CMGF=1) and sending issuccessful: +CMSS: <mr>[,<scts>] If PDU mode (+CMGF=0) and sending is successful: +CMSS: <mr> OK or +CMS ERROR: <err> Parameters For parameter information and values, refer to section 8.1 Parameters Definition. |
| Notes | - Execution command sends message with location value <index> from message storage <mem2> to the network (SMS-SUBMIT or SMS-COMMAND). If new recipient address <da> is given for SMS-SUBMIT, it shall be used instead of the one stored with the message. - Reference value <mr> is returned to the TE on successful message delivery. Optionally (when +CSMS <service> value is 1 and network supports the feature), <scts> is returned in text mode. |

## +CNMI Command: New Message Indication

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CNMI=? | Response +CNMI: (list of supported <mode>s), (list of supported <mt>s), (list of supported <bm>s), (list of supported <ds>es), (list of supported <bfr>s) OK |
| Read command Syntax AT+CNMI? | Response +CNMI: <mode>,<mt>,<bm>,<ds>,<bfr> OK |
| Write command Syntax +CNMI=[<mode> [,<mt>[,<bm> [,<ds>[,<bfr>]]]]] | Response OK or +CMS ERROR: <err> Parameters <mode> 0 Buffer unsolicited result codes in the TA. If TA result code buffer is full, indications can be buffered in some other place or the oldest indications may be discarded and replaced with the new received indications. 1 Discard indication and reject new received message unsolicited result codes when TA-TE link is reserved. Otherwise forward them directly to the TE. 2 Buffer unsolicited result codes in the TA when TA-TE link is reserved (e.g. in on-line data mode) and flush them to the TE after reservation. Otherwise forward them directly to the TE. <mt> 0 No indications are routed to the TE. 1 Result code is sent when ME does not have any other display device other than the AT interface 2 Acknowledgement command must be sent when +CSMS <service> = 1 and ME does not have any other display device other than the AT interface 3 Acknowledgement command must be sent when +CSMS <service> = 1 <bm> 0 No CBM indications are routed to the TE. 1 If CBM is stored into ME/TA, indication of the memory location is routed to the TE using unsolicited result code: +CBMI: <mem>,<index> 2 New CBMs are routed directly to the TE using unsolicited result code: +CBM: <length><CR><LF><pdu> (PDU mode enabled); or +CBM: <sn>,<mid>,<dcs>,<page>,<pages><CR><LF><data> (text mode enabled) 3 Class 3 CBMs are routed directly to TE using unsolicited result codes defined in <bm>=2. If CBM storage is supported, messages of other classes result in indication as defined in <bm>=1 <ds> 0 No SMS-STATUS-REPORTs are routed to the TE. 1 SMS-STATUS-REPORTs are routed to the TE using unsolicited result code: +CDS: <length><CR><LF><pdu> (PDU mode enabled) or +CDS: <fo>,<mr>,[<ra>],[<tora>],<scts>,<dt>, <st> (text mode enabled) 2 If SMS-STATUS-REPORT is stored in ME/TA, indication of the memory location is routed to the TE using unsolicited result code: +CDSI: <mem>,<index> <bfr> 0 TA buffer of unsolicited result codes defined within this command is flushed to the TE when <mode> = 1 – 3 is entered 1 TA buffer of unsolicited result codes defined within this command is cleared when <mode> = 1 – 3 is entered |
| Notes | <mode>, <mt>, <bm> and <ds> are saved in non-volatile memory over module reboot; URC is available on the port that executes the command. |
| Examples | AT+CNMI=1 // Write command OK AT+CNMI=? // Test command +CNMI: (0-2),(0-3),(0-3),(0-2),(0-1) OK AT+CNMI? // Read command +CNMI: 1,0,0,0,0 OK |

## +CSCB Command: Select Cell Broadcast Message Type

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CSCB=? | Response +CSCB: (list of supported <mode>s) OK |
| Read command Syntax AT+CSCB? | Response +CSCB: <mode>,<mids>,<dcss> OK |
| Write command Syntax AT+CSCB= [<mode> [,<mids>]] | Response OK or +CMS ERROR: <err> Parameters <mode> 0 Accepts messages that are defined in <mids> and <dcss> 1 Does not accept messages that are defined in <mids> and <dcss> <mids> String type; combinations of CBM message IDs (e.g. "0,1,5,320-478,922"). Default value is an empty string. The number of ranges in <mids> parameter string is limited to 6. Note that intervals are not allowed. <dcss> String type; all different possible combinations of CBM data coding schemes. Default value is an empty string. |

## +CSCA Command: Service Center Address

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CSCA=? | Response OK |
| Read command Syntax AT+CSCA? | Response +CSCA: <sca>,<tosca> OK |
| Write command Syntax AT+CSCA=<sca> [,<tosca>] | Response OK or +CMS ERROR: <err> Parameters For parameter information and values, refer to section 8.1 Parameters Definition. |

## +CSMP Command: Set Text Mode Parameters

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CSMP=? | Response OK |
| Read command Syntax AT+CSMP? | Response +CSMP: <fo>,<vp>,<pid>,<dcs> OK |
| Write command Syntax AT+CSMP=[<fo> [,<vp>[,<pid> [,<dcs>]]]] | Response OK Parameters For parameter information and values, refer to section 8.1 Parameters Definition. |

## +CSMS Command: Select Message Service

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CSMS=? | Response +CSMS: (list of supported <service>s) OK |
| Read command Syntax AT+CSMS? | Response +CSMS: <service>,<mt>,<mo>,<bm> OK |
| Write command Syntax AT+CSMS= <service> | Response +CSMS: <mt>,<mo>,<bm> OK or +CMS ERROR: <err> Parameters <service> 0 3GPP TS 23.040 and 3GPP TS 23.041 1 3GPP TS 23.040 and 3GPP TS 23.041 (the requirement of setting <service> =1 is mentioned in the corresponding command description) <mt> Message terminated messages 0 Type not supported 1 Type supported <mo> Message originated messages 0 Type not supported 1 Type supported <bm> Broadcast type messages 0 Type not supported 1 Type supported |

## +CPMS Command: Preferred Message Storage

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CPMS=? | Response +CPMS: (list of supported <mem1>s), (list of supported <mem2>s), (list of supported <mem3>s) OK |
| Read command Syntax AT+CPMS? | Response +CPMS: <mem1>,<used1>,<total1>,<mem2>,<used2>,<total2>,<mem3>,<used3>,<total3> OK or +CMS ERROR: <err> |
| Write command Syntax AT+CPMS= <mem1> [,<mem2> [,<mem3>]] | Response +CPMS: <used1>,<total1>,<used2>,<total2>,<used3>,<total3> OK or +CMS ERROR: <err> Parameters For parameter information and values, refer to section 8.1 Parameters Definition. |
| Notes | <mem1>, <mem2> and <mem3> are saved in non-volatile memory over module reboot. |

## +CSDH Command: Show Text Mode Parameters

For HL7618, HL7618RD, HL7648, HL7688, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7648, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CSDH=? | Response +CSDH: (list of supported <show>s) OK |
| Read command Syntax AT+CSDH? | Response +CSDH: <show> OK |
| Write command Syntax AT+CSDH= [<show>] | Response OK or +CME ERROR: <err> Parameter <show> 0 Do not show header values defined in commands +CSCA and +CSMP (<sca>, <tosca>, <fo>, <vp>, <pid> and <dcs>) nor <length>, <toda> or <tooa> in +CMGL, +CMGR result codes for SMS-DELIVERs and SMS-SUBMITs in text mode; for SMS- COMMANDs in +CMGR result code, do not show <pid>, <mn>, <da>, <toda>, <length> or <cdata> 1 Show values in result codes |

## +XCMGS3GPP2 Command: Send 3GPP2 SMS Message

For HL7618, HL7618RD, HL7650, HL7688, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ XCMGS3GPP2=? | Response OK |
| Write command Syntax AT+ XCMGS3GPP2= <length> <message_type><CR> PDU is given <ctrl-Z/ESC> | Response If sending is successful: OK If sending fails: +CMS ERROR: <err> Parameters <length> Indicates the number of total octets coded in the PDU to be given <message_type> 0 Invalid 1 Point to Point 2 Acknowledge 3 Broadcast |
| Notes | - Entered text should be formatted as follows: PDU should be in hexadecimal format and given in one continuous line; the ME/TA converts this coding into the actual octets of PDU. Sending can be called by giving the <ESC> character during input. <ctrl-Z> must be used to indicate the ending of PDU. - AT+CMGF has no impact on this command. - This command returns error when SMS over IP network option is not set. |

## +XCMT3GGP2 Command: Enable or Disable the 3GPP2 MT SMS URC

For HL7618, HL7618RD, HL7650, HL7688, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+XCMT3GPP2=? | Response +XCMT3GPP2: (list of supported <n>s) OK |
| Write command Syntax AT+XCMT3GPP2=<n> | Response OK or +CMS ERROR: <err> Parameters <length> Indicates the number of total octets coded in the PDU to be given <n> Type of 3GPP2 message 0 Disable URC 1 Enable URC |
| Unsolicited Notification | Response +XCMT3GPP2: <length><CR><LF><PDU data> Parameters <length> Indicates the number of total octets coded in the PDU to be given <PDU data> PDU data in 3GPP2 format |
| Notes | If XCMT3GPP2 URC is enabled, then unsolicited result codes are buffered in the TA when the TA-TE link is reserved (e.g. in online data mode) and flushed to the TE after reservation. Otherwise, they are forwarded directly to the TE. |

# Audio Commands

## +KPCMCFG Command: Configure PCM Digital Audio

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KPCMCFG=? | Response For the HL7618, HL7618RD without VoLTE, HL7688, HL7690 and HL7695: +KPCMCFG: (list of supported <Mode>s), (list of supported <SamplingCtrl>s), (list of <BitClk>s) OK For the HL7618RD with VoLTE, HL7648 and HL7650: +KPCMCFG: (list of supported <Mode>s), (list of supported <SamplingCtrl>s), (list of <BitClk>s),(list of <SampleRate>s) |
| Read command Syntax AT+KPCMCFG? | Response For the HL7618, HL7618RD without VoLTE, HL7688, HL7690 and HL7695: +KPCMCFG: <Mode>,<SamplingCtrl>,<BitClk> OK For the HL7618RD with VoLTE, HL7648 and HL7650: +KPCMCFG: <Mode>,<SamplingCtrl>,<BitClk>,<SampleRate> OK |
| Write command Syntax For the HL7618, HL7618RD without VoLTE, HL7688, HL7690 and HL7695: AT+KPCMCFG =<Mode> [,<SamplingCtrl> [,<BitClk>]] For the HL7618RD with VoLTE, HL7648 and HL7650: AT+KPCMCFG =<Mode> [,<SamplingCtrl> [,<BitClk> [,<SampleRate> ]]] | Response OK Parameters <Mode> PCM mode 0 Master 1 Slave <SamplingCtrl> Sampling clock edge control 0 Falling edge 1 Rising edge <BitClk> PCM bit clock 0 256 kHz 1 384 kHz 2 512 kHz <SampleRate> PCM sample rate 0 8kS/s 1 16kS/s This parameter is only available on the HL7648 and HL7650 |
| Reference Sierra Wireless Proprietary | Notes - Settings will take effect immediately; no reset is required. - Parameters are saved and kept after reset. - This command can be used without a SIM. - The sampling rate is fixed at 8 kS/s for the HL7618, HL7618RD without VoLTE, HL7688, HL7690 and HL7692. - Only 16-but linear PCM mode is supported. A-law and µ-law compression modes are not supported. - Only long frame sync is supported. - In slave mode, the acceptable PCM clock is also determined by <BitClk>. |
| Examples | // Example using an HL7618 module: AT+KPCMCFG? //Shows the current configuration +KPCMCFG: 0,1,2 //Master mode, rising edge and PCM clock is 512 kHz OK AT+KPCMCFG=1,0 //Switch to slave mode with falling edge latched. As parameter <BitClk> is omitted, the //old <BitClk> value will be used in the new configuration. OK AT+KPCMCFG? +KPCMCFG: 1,0,2 //Slave mode, falling edge and PCM clock is 512 kHz OK AT+KPCMCFG=0,1 //Turn back to master mode and rising edge latched OK AT+KPCMCFG? +KPCMCFG: 0,1,2 OK // Example using an HL7648 module: AT+KPCMCFG? //Shows the current configuration +KPCMCFG: 0,1,2,0 //Master mode, rising edge, PCM clock is 512 kHz and PCM //sample rate is 8kS/s OK AT+KPCMCFG=1,0 //Turn to slave mode and falling edge latched. As //parameter <BitClk> and <SampleRate> are omitted, old //<BitClk> and <SampleRate> values will be used in the //newconfiguration. OK AT+KPCMCFG? +KPCMCFG: 1,0,2,0 //Slave mode, falling edge, PCM clock is 512 kHz and PCM //sample rate is 8kS/s OK AT+KPCMCFG=0,1 //Turn back to master mode and rising edge latched. OK AT+KPCMCFG? +KPCMCFG: 0,1,2,0 OK |

## +WMAUDIOLOOP Command: Audio Test

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ WMAUDIOLOOP=? | Response +WMAUDIOLOOP: (list of supported <ENABLE>s),(list of supported <TXORGAN>s), (list of supported <RXORGAN>s) OK |
| Read command Syntax AT+ WMAUDIOLOOP? | Response +WMAUDIOLOOP: <ENABLE>[,<TXORGAN>,<RXORGAN>] OK Note that parameters <TXORGAN> and <RXORGAN> are only available if <ENABLE>=1. |
| Write command Syntax AT+ WMAUDIOLOOP=<ENABLE>, <TXORGAN>, <RXORGAN> | Response OK Error Case +CME ERROR: 3 (when a non-supported <TXORGAN> or <RXORGAN> is used) Parameters <ENABLE> 0 Stop the audio loop test 1 Execute the audio loop <TXORGAN> Audio input used as reference for the audio loop 0 PCM in 1 Reserved <RXORGAN> Audio output used to loop the audio input 0 PCM out 1 Reserved |
| Reference Sierra Wireless Proprietary | Notes Audio loop activation involves some restructions on the use of other AT commands: - It must not be enabled when: - Communications is active - A tone is under generation - It must be disabled (if active) before opening up communications. - Tone generation and sidetone modifications are not possible when the audio loop is active. |
| Examples | AT+WMAUDIOLOOP=? +WMAUDIOLOOP: (0-1),(0-1),(0-1) OK AT+WMAUDIOLOOP? +WMAUDIOLOOP: 0 OK AT+WMAUDIOLOOP=1,0,0 OK //Started audio loop AT+WMAUDIOLOOP? +WMAUDIOLOOP: 1,0,0 OK AT+WMAUDIOLOOP=0,0,0 OK //Stopped audio loop |

## +CLVL Command: Loudspeaker Volume Level

For HL7618RD, HL7648, HL7650, HL7688 and HL7692 only.

| HL7618RD, HL7648, HL7650, HL7688 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CLVL=? | Response +CLVL: (list of supported <level>s) OK |
| Read command Syntax AT+CLVL? | Response +CLVL: <level> OK |
| Write command Syntax AT+CLVL= <level> | Response OK Parameter <level> 1 – 10 Loudspeaker level (smallest value represents the lowest sound level) |
| Reference [27.007] § 8.23 | Notes - The value of <level> is not saved; it will return to its nominal value after the module is reset. - This command is supported on the HL7618RD when using a VoLTE-capable software. |
| Examples | AT+CLVL=? +CLVL: (1-10) OK AT+CLVL? +CLVL: 8 OK AT+CLVL=1 //Turn to the lowest volume level OK AT+CLVL=10 //Turn to the loudest volume level OK |

## +KECHO Command: Echo Cancellation

For HL7618RD, HL7648, HL7650, HL7688 and HL7692 only.

| HL7618RD, HL7648, HL7650, HL7688 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KECHO=? | Response +KECHO: (list of supported <mode>s),(list of supposted <param>s) OK |
| Read command Syntax AT+KECHO? | Response +KECHO: <status>,<param_1>,…,<param_n> OK |
| Write command Syntax AT+KECHO= <mode> [,<param_1>,…, <param_n>] | Response OK Parameter <mode> 0 Deactivate echo cancellation 1 Activate echo cancellation <status> Echo cancellation status 0 Deactivated 1 Activated <param_n> - NLMSTaps_band_x stands for number of LMS (Least Mean Squares) filter taps in frequency band x. - Parameters should be bounded by the follow constrain. - NLMSTaps_band_0 + 2*(NLMSTaps_band_1+...+ NLMSTaps_band_5) < 2000 #; Name; Range; Default Handset Profile. 1; <NLMSTaps_band_0>; 2-1096; 100. 2; <NLMSTaps_band_1>; 1-548; 100. 3; <NLMSTaps_band_2>; 1-548; 100. 4; <NLMSTaps_band_3>; 1-994; 2. 5; <NLMSTaps_band_4>; 1-994; 2. 6; <NLMSTaps_band_5>; 1-994; 2. 7; <NLMS_Block_Length>; 1, 2, 4, 5, 8; 1. |
| Reference Sierra Wireless Proprietary | Notes - Settings will take effect immediately. - Parameters are saved and kept after reset. - This command can be used without a SIM card. - +VIP can affect the value of this command’s parameters. - This command is supported on the HL7618RD when using a VoLTE-capable software. |
| Examples | AT+KECHO? //Shows the current configuration +KECHO: 1,100,100,100,1,1,1,2 OK AT+KECHO=0 //Turn off the echo cancellation OK AT+KECHO? +KECHO: 0,100,100,100,1,1,1,2 //Echo cancellation is deactivated OK AT+KECHO=1,150,100,100,1,1,1,2 //Activate echo cancellation again and modify //param_0 to 150 +KECHO: 1,150,100,100,1,1,1,2 //The algorithm is activated again with new //parameters OK AT+CFUN=1,1 OK AT+KECHO? +KECHO: 1,150,100,100,1,1,1,2 //Parameters are retained after reset OK |

## +KNOISE Command: Echo Suppression

For HL7618RD, HL7648, HL7650, HL7688 and HL7692 only.

| HL7618RD, HL7648, HL7650, HL7688 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KNOISE=? | Response +KNOISE: (list of supported <rx_mode>s), (list of supported <tx_mode>s), (list of supported <rx_param_1>s),…,(list of supported <rx_param_5>s), (list of supported <tx_param_1>s),…,(list of supported <tx_param_5>s) OK |
| Read command Syntax AT+KNOISE? | Response +KNOISE: <rx_status>,<tx_status>,<rx_param_1>,...,<rx_param_5>, <tx_param_1>,…,<tx_param_5> OK |
| Write command Syntax AT+KNOISE= <rx_mode>, <tx_mode> [,<rx_param_1>,…,<rx_param_5>,<tx_param_1,…, <tx_param_5>] | Response OK Parameters <rx_mode> Receive mode 0 Deactivate downlink noise suppression 1 Activate downlink noise suppression <tx_mode> Transmit mode 0 Deactivate uplink noise suppression 1 Activate uplink noise suppression <rx_status> Receive noise suppression status 0 Deactivated 1 Activated <tx_status> Transmit noise suppression status 0 Deactivated 1 Activated <rx_param_1> 0-65535 Minimum attenuation Default handset profile value = 12000 <rx_param_2> 0-65535 Over-estimation factor for band 0 Default handset profile value = 10000 <rx_param_3> 0-65535 Over-estimation factor for all other bands Default handset profile value = 10000 <rx_param_4> 0-65535 Exponent factor of the NR Default handset profile value = 1000 <rx_param_5> 0-65535 Over-estimation factor for all other bands Default handset profile value = 16384 <tx_param_1> 0-65535 Minimum attenuation Default handset profilevalue = 6000 <tx_param_2> 0-65535 Over-estimation factor for band 0 Default handset profile value = 8000 <tx_param_3> 0-65535 Over-estimation factor for all other bands Default handset profile value = 8000 <tx_param_4> 0-65535 Exponent factor of the NR Default handset profile value = 1000 <tx_param_5> 0-65535 Over-estimation factor for all other bands Default handset profile value = 19660 |
| Reference Sierra Wireless Proprietary | Notes - Settings will take effect immediately. - Parameters are saved and kept after reset. - This command can be used without a SIM card. - This command is supported on the HL7618RD when using a VoLTE-capable software. |
| Examples | AT+KNOISE=? +KNOISE: (0-1),(0-1),(0-65535),(0-65535),(0-65535),(0-65535),(0-65535),(0-65535), (0-65535),(0-65535),(0-65535),(0-65535) OK AT+KNOISE? //Shows the current configuration +KNOISE: 1,1,6000,8000,8000,1000,19660, 6000,8000,8000,1000,19660 OK AT+KNOISE=0,0 //Disable uplink and downlink noise suppression OK AT+KNOISE=1,1,6500,8000,8000,1000,19660, 6800,8000,8000,1000,19660 //Enable uplink and downlink noise suppression with new parameters OK AT+CFUN=1,1 OK AT+KNOISE? +KNOISE: 1,1,6500,8000,8000,1000,19660, 6800,8000,8000,1000,19660 //Parameters are retained after reset OK |

## +KPC Command: Peak Compressor

For HL7618RD, HL7648, HL7650, HL7688 and HL7692 only.

| HL7618RD, HL7648, HL7650, HL7688 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KPC=? | Response +KPC: (list of supported <rx_mode>s), (list of supported <tx_mode>s) OK |
| Read command Syntax AT+KPC? | Response +KPC: <rx_mode>,<tx_mode> OK |
| Write command Syntax AT+KPC= <rx_mode>, <tx_mode> | Response OK Parameters <rx_mode> 0 Disable 1 Enable (handset profile default) <tx_mode> 0 Disable 1 Enable (handset profile default) |
| Reference Sierra Wireless Proprietary | Notes - Settings will take effect immediately. - Parameters are saved and kept after reset. - This command can be used without a SIM card. - +VIP can affect the value of this command’s parameters. - This command is supported on the HL7618RD when using a VoLTE-capable software. |
| Examples | AT+VIP? //Check the current audio profile +VIP: 0 OK AT+KPC=? +KPC: (0-1),(0-1) OK AT+KPC? //Shows the current value +KPC: 0,0 OK AT+KPC=1,0 //Activate the rx peak compressor OK AT+KPC? +KPC: 1,0 OK AT+VIP=1 //Switch to headset profile OK AT+KPC? +KPC: 0,0 //Peak compressor status is different in different audio profiles OK |

## +KST Command: Side Tone

For HL7618RD, HL7648, HL7650, HL7688 and HL7692 only.

| HL7618RD, HL7648, HL7650, HL7688 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KST=? | Response +KST: (list of supported <level>s) OK |
| Read command Syntax AT+KST? | Response +KST: <level> OK |
| Write command Syntax AT+KST=<level> | Response OK Parameter <level> 0 – 16 Side tone value (side tone gain from -14 dB to +18 dB in steps of 2) 20 Disable sidetone |
| Reference Sierra Wireless Proprietary | Notes - Settings will take effect immediately. - Parameters are saved and kept after reset. - This command can be used without a SIM card. - This command is supported on the HL7618RD when using a VoLTE-capable software. |
| Examples | AT+KST=? +KST: (0-16, 20) OK AT+KST? //Shows the current value +KST: 8 OK AT+KST=0 //Set side tone gain to -14dB OK AT+KST=20 //Disable side tone OK AT+CFUN=1,1 OK AT+KST? +KST: 20 //Parameters are retained after reset OK |

## +KVGR Command: Receive Gain Selection

For HL7618RD, HL7648, HL7650, HL7688 and HL7692 only.

| HL7618RD, HL7648, HL7650, HL7688 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KVGR=? | Response +KVGR: (list of supported <n>s) OK |
| Read command Syntax AT+KVGR? | Response +KVGR: <n> OK |
| Write command Syntax AT+KVGR=<n> | Response OK Parameter <n> -21 to 6 Digital gain of the downlink path in dB. Default value = 0 |
| Reference Sierra Wireless Proprietary | Notes - The value of <n> is not saved; it will return to its nominal value after the module is reset. - <n> can be specified with or without quotes. - Gain can be changed either during a connection or outside of a connection. - This command is supported on the HL7618RD when using a VoLTE-capable software. |
| Examples | AT+KVGR=“-21” //Receive gain is set to 21dB less than the nominal gain OK AT+KVGR=“-22” //Input is out of range ERROR AT+KVGR=“6” //Receive gain is set to 6dB more than the nominal gain OK AT+KVGR=“7” //Input is out of range ERROR AT+VGR=87 //Receive gain is set to -20.5dB less than the nominal gain by //+VGR OK AT+KVGR? //+KVGR response truncates the decimal part of the actual gain +KVGR: -20 OK |

## +KVGT Command: Transmit Gain Selection

For HL7618RD, HL7648, HL7650, HL7688 and HL7692 only.

| HL7618RD, HL7648, HL7650, HL7688 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KVGT=? | Response +KVGT: (list of supported <n>s) OK |
| Read command Syntax AT+KVGT? | Response +KVGT: <n> OK |
| Write command Syntax AT+KVGT=<n> | Response OK Parameter <n> -21 to 6 Digital gain of the uplink path in dB. Default value = 0 |
| Reference Sierra Wireless Proprietary | Notes - The value of <n> is not saved; it will return to its nominal value after the module is reset. - <n> can be specified with or without quotes. - Gain can be changed either during a connection or outside of a connection. - This command is supported on the HL7618RD when using a VoLTE-capable software. |
| Examples | AT+KVGT=“-21” //Transmit gain is set to 21dB less than the nominal gain OK AT+KVGT=“-22” //Input is out of range ERROR AT+KVGT=“6” //Transmit gain is set to 6dB more than the nominal gain OK AT+KVGT=“7” //Input is out of range ERROR AT+VGT=87 //Transmit gain is set to -20.5dB less than the nominal gain by //+VGT OK AT+KVGT? //+KVGT response truncates the decimal part of the actual gain +KVGT: -20 OK |

## +VGR Command: Receive Gain Selection

For HL7618RD, HL7648, HL7650, HL7688 and HL7692 only.

| HL7618RD, HL7648, HL7650, HL7688 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+VGR=? | Response +VGR: (list of supported <n>s) OK |
| Read command Syntax AT+VGR? | Response +VGR: <n> OK |
| Write command Syntax AT+VGR=<n> | Response OK Parameter <n> 86 ≤ n ≤ 140 < 128 (128 – n)/2 dB less than the normal gain (up to -21 dB) 128 Nominal gain > 128 (n – 128)/2 dB more than the nominal gain (up to 6 dB) |
| Reference [27.007] § C.2.5 | Notes - The value of <n> is not saved; it will return to its nominal value after the module is reset. - Gain can be changed either during a connection or outside of a connection. - This command returns an error when the requested value is out of range (-21 dB to 6 dB). - This command is supported on the HL7618RD when using a VoLTE-capable software. |
| Examples | AT+VGR=86 //Receive gain is set to 21 dB less than the nominal gain OK AT+VGR=85 //Input is out of range ERROR AT+VGR=140 //Receive gain is set to 6 dB more than the nominal gain OK AT+VGR=141 //Input is out of range ERROR |

## +VGT Command: Transmit Gain Selection

For HL7618RD, HL7648, HL7650, HL7688 and HL7692 only.

| HL7618RD, HL7648, HL7650, HL7688 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+VGT=? | Response +VGT: (list of supported <n>s) OK |
| Read command Syntax AT+VGT? | Response +VGT: <n> OK |
| Write command Syntax AT+VGT=<n> | Response OK Parameter <n> 86 ≤ n ≤ 140 < 128 (128 – n)/2 dB less than the normal gain (up to -21 dB) 128 Nominal gain > 128 (n – 128)/2 dB more than the nominal gain (up to 6 dB) |
| Reference [27.007] § C.2.5 | Notes - The value of <n> is not saved; it will return to its nominal value after the module is reset. - Gain can be changed either during a connection or outside of a connection. - This command returns an error when the requested value is out of range (-21 dB to 6 dB). - This command is supported on the HL7618RD when using a VoLTE-capable software. |
| Examples | AT+VGT=86 //Transmit gain is set to 21 dB less than the nominal gain OK AT+VGT=85 //Input is out of range ERROR AT+VGT=140 //Transmit gain is set to 6 dB more than the nominal gain OK AT+VGT=141 //Input is out of range ERROR |

## +VIP Command: Initialize Voice Parameters

For HL7618RD, HL7648, HL7650, HL7688 and HL7692 only.

| HL7618RD, HL7648, HL7650, HL7688 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+VIP=? | Response +VIP: (list of <profile>s),(list of supported <persistence>s) OK |
| Read command Syntax AT+VIP? | Response +VIP: <profile>,<persistence> OK |
| Write command Syntax AT+VIP=<profile> [,<persistence>] | Response OK Parameters <profile> Audio profile 0 Handset 1 Headset 2 Handsfree + back speaker 5 TTY 23 Basic (no specific transducer defined) <persistence> Persistence of <profile> after a call is disconnected 0 <profile> will be reset to 0 1 <profile> will not be reset to 0 |
| Reference [27.007] § C.2.6 | Notes - This command can be used without a SIM card. - <profile> takes effect in the next call. - When <persistence>=0, <profile> is automatically returned to its default value after a call is disconnected. - Parameters are not saved in non-volatile memory. - This command is supported on the HL7618RD when using a VoLTE-capable software. |
| Examples | AT+VIP? //Shows the current configuration +VIP: 0,0 OK AT+VIP=1 //Turn to headset profile OK AT+VGT=140 //Transmit gain of headset profile is changed to 140 OK AT+VIP=0 //Turn to handset profile OK AT+VGT? //Transmit gain of handset profile is still 128 +VGT: 128 OK |

## +CODECINFO Command: Display Audio Codec Information

For HL7650, HL7688 and HL7692 only.

| HL7650, HL7688 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CODECINFO=? | Response +CODECINFO: (list of supported <MODE>s) OK |
| Read command Syntax AT+CODECINFO? | Response +CODECINFO: <MODE> OK |
| Write command Syntax AT+CODECINFO=<MODE> | Response OK Parameter <MODE> 0 Disable codec info unsolicited message 1 Enable codec info unsolicited message |
| Reference Sierra Wireless Proprietary | Notes - <MODE> is stored in non-volatile memory immediately when a valid write command is entered, and retained after reset. - <MODE> is effective without a reset. - This command can be used without a SIM card. - If <MODE> = 1, +CODECINFO: x unsolicited message will be displayed in the format below: +CODECINFO: 0 GSM_FR +CODECINFO: 1 GSM_HR +CODECINFO: 2 GSM_EFR +CODECINFO: 3 FR_AMR +CODECINFO: 4 HR_AMR +CODECINFO: 5 UMTS_AMR +CODECINFO: 6 UMTS_AMR2 +CODECINFO: 10 UMTS_AMR_WB |
| Examples | AT+CODECINFO=? // Read available options +CODECINFO: (0-1) OK AT+CODECINFO=1 OK AT+CODECINFO? // Read current setting +CODECINFO: 1 OK RING // An incoming call +CODECINFO: 10 // UMTS_AMR_WB is chosen |

## +KSRAP Command: Save or Restore Audio Parameters

For HL7618RD, HL7648, HL7650, HL7688 and HL7692 only.

| HL7618RD, HL7648, HL7650, HL7688 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KSRAP=? | Response +KSRAP: (list of supported <level>s) OK |
| Write command Syntax AT+KSRAP= <level> | Response OK Parameter <level> 2 Restore audio parameters in non-volatile memory to their default values |
| Reference Sierra Wireless Proprietary | Notes - This command changes the values in the non-volatile memory immediately; settings take effect in the next speech call. - This command is supported on the HL7618RD when using a VoLTE-capable software. |
| Example | at+ksrap ERROR at+ksrap? ERROR at+ksrap=? +KSRAP: (2) OK at+kecho? // Current kecho NVM parameters +KECHO: 0,100,100,100,2,2,2,1 OK at+kecho=1,103,102,101,4,3,2,1 // Change kecho NVM parameters by at+kecho +KECHO: 1,103,102,101,4,3,2,1 OK at+cfun=1,1 OK at+kecho? // kecho NVM parameters are modified +KECHO: 1,103,102,101,4,3,2,1 OK at+ksrap=2 // By ksrap=2, default kecho parameters in NVM are restored OK at+kecho? +KECHO: 0,100,100,100,2,2,2,1 OK |

## +WVR Command: Voice Codec Selection

For HL7650, HL7688 and HL7692 only.

| HL7650, HL7688 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+WVR=? | Response +WVR: (list of supported <aud_coding_type_2G>s),(list of supported <aud_coding_type_3G>s) OK |
| Read command Syntax AT+WVR? | Response +WVR: <aud_coding_type_2G>,<aud_coding_type_3G> OK |
| Write command Syntax AT+WVR= [<aud_coding_ type_2G>] [,<aud_coding_ type_3G>] | Response OK Parameters <aud_coding_type_2G> Supported 2G types (not supported) 5 FR, EFR, HR, AMR-FR, AMR-HR <aud_coding_type_3G> Supported 3G types 3 UMTS AMR v2 4 UMTS AMR v2, UMTS, AMR-WB |
| Reference Sierra Wireless Proprietary | Notes - This command allows the configuration of supported 3G voice codecs of the device; although the final codec decision is actually made by the network. No call would be established and no sound would be heard if the list of supported codecs set in the device does not match the network’s. (Note that 2G voice codecs are not supported in the HL7588.) - <aud_coding_type_2G> has no effect in the HL7588 as it is not supported. - Parameters are stored in non-volatile memory immediately when a valid write command is entered. - This command can be used without a SIM card. |
| Example | AT+WVR=? // Read the available options +WVR: (5),(3-4) OK AT+WVR=,3 // Set 3G codec as UMTS AMR v2, 2G codec is skipped as only 3G // codecs available OK AT+WVR? // Read the current setting +WVR: 5,3 OK |

## +VTD Command: Tone Duration

For HL7650, HL7688 and HL7692 only.

| HL7650, HL7688 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+VTD=? | Response +VTD: (list of supported <n>s) OK |
| Read command Syntax AT+VTD? | Response +VTD: <n> OK |
| Write command Syntax AT+VTD=<n> | Response OK Parameter <n> 0 Default setting (default duration of the tone is 7/10 second) 1 – 100 Duration of the tone in 1/10 seconds |
| Reference [27.007] § C.2.12 | Notes The network shall ensure that the minimum length of tone and the minimum gap between two subsequent tones (according to ETR 206) is achieved. (In ETR 206 the minimum duration of a DTMF tone is 70ms ± 5ms, the minimum gap between DTMF tones is 65ms). There is no defined maximum length to the tone; however, the operator may choose to put a pre-defined time limit on the duration of tones sent to line (refer to [23.014]). That means that with n<6, DTMF will be generated with a duration given by the network. |

## +VTS Command: DTMF and Tone Generation

For HL7618RD, HL7648, HL7650, HL7688 and HL7692 only.

| HL7618RD, HL7648, HL7650, HL7688 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+VTS=? | Response +VTS: (list of supported <DTMF>s) OK |
| Write command Syntax AT+VTS= “<DTMF>1, <DTMF>2, …, <DTMF>n” or AT+VTS= “{<DTMF>1, <DURATION>1}, {<DTMF>2, <DURATION>2},… {<DTMF>n, <DURATION>n}” | Response OK Parameters <DTMF> A single ASCII character in the set 0 – 9, #, *, A – D. This is interpreted as a single ACSII character whose duration is set by the +VTD command. DTMF tones can only be issued during a voice call. <DURATION> This is interpreted as a DTMF tone of different duration from that mandated by the +VTD command. In GSM, this only operates in voice mode. Values are in 1/10 second multiples. |
| Reference [27.007] § C.2.11 | Notes - The network shall ensure that the minimum length of tone and the minimum gap between two subsequent tones (according to ETR 206) is achieved. (In ETR 206 the minimum duration of a DTMF tone is 70ms ± 5ms, the minimum gap between DTMF tones is 65ms). There is no defined maximum length to the tone, however, the operator may choose to put a pre-defined time limit on the duration of tones sent to line (refer to [23.014]). That means that with n<6, DTMF will be generated with a duration given by the network. - Input character string without quotes in the write command is supported for compatibility purposes. - This command is supported on the HL7618RD when using a VoLTE-capable software. |

# Packet Domain Commands

For additional details about PDP context use for the HL7618 and HL7618RD, refer to section 1.4 PDP Context Usage.

## +CGATT Command: PS Attach or Detach

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGATT=? | Response +CGATT: (list of supported <state>s) OK |
| Read command Syntax AT+CGATT? | Response +CGATT: <state> OK |
| Write command Syntax AT+CGATT= [<state>] | Response OK or ERROR Parameter <state> State of PS attachment 0 Detached 1 Attached |

## +CGACT Command: Activate or Deactivate PDP Context

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGACT=? | Response +CGACT: (list of supported <state>s) OK |
| Read command Syntax AT+CGACT? | Response +CGACT: <cid>, <state> [<CR><LF>+CGACT: <cid>,<state> [...]] OK |
| Write command Syntax AT+CGACT= [<state> [,<cid> [,<cid> [,...]]]] | Response OK or ERROR Parameters <state> State of PDP context activation 0 Deactivated 1 Activated <cid> Numeric parameter which specifies a particular PDP context definition. |
| Notes | Up to three (3) PDP contexts can be active at once. |

## +CGANS Command: PDP Context Activation Manual Response

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGANS=? | Response +CGANS: (list of supported <response>s), (list of supported <L2P>s) OK |
| Write command Syntax AT+CGANS= [<response>, [<L2P> ,[<cid>]]] | Response OK or +CME ERROR: <err> Parameters <response> 0 Reject the request (default value if omitted) 1 Accept and request that the PDP context be activated <L2P> String parameter indicating the layer 2 protocol to be used (see +CGDATA) <cid> Numeric parameter that specifies a particular PDP context definition (see +CGDCONT and +CGDSCONT). Parameter <response> allows the TE to accept or reject the request. |
| Notes | - Commands following the +CGANS command in the AT command line shall not be processed by the MT. - If the <L2P> parameter value is unacceptable to the MT, the MT shall return an ERROR or +CME ERROR response. Otherwise, the MT issues the intermediate result code CONNECT and enters V.250 online data state. If no <cid> is given or if there is no matching context definition, the MT will attempt to activate the context using the values for PDP type and PDP address provided by the network, together with any other relevant information known to the MT. The other context parameters will be set to their default values. - If the activation is successful, data transfer may proceed. Note that this is not the same as if the MT issues a +CGDATA (or +CGACT) command after receiving a +CRING unsolicited result code. +CGDATA (or +CGACT) does not command the MT to acknowledge the network request but rather to make a new request for context activation. The network request would be ignored. |

## +CGCMOD Command: Modify PDP Context

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGCMOD=? | Response +CGCMOD: (list of <cid>s addociated with active contexts) OK |
| Write command Syntax AT+CGCMOD= [<cid>[,<cid> [,...]]] | Response OK or +CME ERROR: <err> Parameter <cid> Numeric parameter which specifies a particular PDP context definition (see +CGDCONT and +CGDSCONT) |

## +CGTFT Command: Traffic Flow Template

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGTFT=? | Response +CGTFT: <PDP_type>, (list of supported <packet filter identifier>s) , (list of supported <evaluation precedence index>s), (list of supported <source address and subnet mask>s), (list of supported <protocol number (ipv4) / next header (ipv6)>s), (list of supported <destination port range>s), (list of supported <source port range>s), (list of supported <ipsec security parameter index (spi)>s), (list of supported <type of service (tos) (ipv4) and mask / traffic class (ipv6) and mask>s), (list of supported <flow label (ipv6)>s), (list of supported <direction>s) [<CR><LF>+CGTFT: <PDP_type>, (list of supported <packet filter identifier>s), (list of supported <evaluation precedence index>s), (list of supported <source address and subnet mask>s), (list of supported <protocol number (ipv4) / next header (ipv6)>s), (list of supported <destination port range>s),(list of supported <source port range>s), (list of supported <ipsec security parameter index (spi)>s), (list of supported <type of service (tos) (ipv4) and mask / traffic class (ipv6) and mask>s), (list of supported <flow label (ipv6)>s), (list of supported <direction>s)[...]] |
| Read command Syntax AT+CGTFT? | Response +CGTFT: <cid>, <packet filter identifier>,<evaluation precedence index>, <source address and subnet mask>, <protocol number (ipv4) / next header (ipv6)>, <destination port range>, <source port range>, <ipsec security parameter index (spi)>, <type of service (tos) (ipv4) and mask / traffic class (ipv6) and mask>, <flow label (ipv6)>, <direction> [<CR><LF>+CGTFT: <cid>, <packet filter identifier>, <evaluation precedence index>, <source address and subnet mask>, <protocol number (ipv4) / next header (ipv6)>, <destination port range>, <source port range>, <ipsec security parameter index (spi)>, <type of service (tos) (ipv4) and mask / traffic class (ipv6) and mask>, <flow label (ipv6)>, <direction> [...]] |
| Write command Syntax AT+CGTFT= [<cid>,[<packet filter identifier>, <evaluation precedence index> [,<source address and subnet mask> [,<protocol number (ipv4) / next header (ipv6)> [,<destination port range> [,<source port range> [,<ipsec security parameter index (spi)> [,<type of service (tos) (ipv4) and mask / traffic class (ipv6) and mask> [,<flow label (ipv6)>, <direction> ]]]]]]]]]] | Response OK or ERROR Parameter <cid> Numeric parameter which specifies a particular PDP context definition (see +CGDCONT and +CGDSCONT) <packet filter identifier> Numeric parameter with value range from 1 to 16 <evaluation precedence index> Numeric parameter with value range from 0 to 255 <source address and subnet mask> String tpe given as a dot-separated numeric (0 – 255) parameter of the form "a1.a2.a3.a4.m1.m2.m3.m4" for IPv4 or "a1.a2.a3.a4.a5.a6.a7.a8.a9.a10.a11.a12.a13. a14.a15.a16.m1.m2.m3.m4.m5.m6.m7.m8. m9.m10.m11.m12.m13.m14.m15.m16" for IPv6 <protocol number (ipv4) / next header (ipv6)> Numeric parameter with value range from 0 to 255 <destination port range> String type given as a dot-separated numeric (0 – 65535) parameter on the form ‘f.t.’ <source port range> String type given as a dot-separated numeric (0 – 65535) parameter on the form ‘f.t.’ <ipsec security parameter index (spi)> Numeric value in hecadecimal format with value range from 00000000 to FFFFFFFF <type of service (tos) (ipv4) and mask / traffic class (ipv6) and mask> String type given as a dot-separated numeric (0 – 255) parameter on the form ‘t.m.’ <flow label (ipv6)> Numeric value in hecadecimal format with value range from 00000 to FFFFF. Valid for IPv6 only <direction> Specifies the transmission direction in which the packet filter shall be applied 1 Uplink 2 Downlink 3 Birectional (up and downlink ; default if omitted) |
| Notes | - Some of the listed attributes above may coexist in a Packet Filter while others mutually exclude each other. For the list of possible combinations, refer to 3GPP TS 23.060. - +CGTFT=<cid> causes all of the packet filters in the TFT for context number <cid> to become undefined. |

## +CGCLASS Command: GPRS Mobile Station Class

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGCLASS=? | Response +CGCLASS: (list of supported <class>es) OK |
| Read command Syntax AT+CGCLASS? | Response +CGCLASS: <class> OK |
| Write command Syntax AT+CGCLASS= [<class>] | Response OK or ERROR Parameters <class> Mode of operation “A” Class A “B” Class B “CG” Class C in GPRS mode “CC” Class C in circuit switched mode |
| Notes | <class> is saved in non-volatile memory over module reboot. |

## +CGDCONT Command: Define PDP Context

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGDCONT=? | Response +CGDCONT: (range of supported <cid>s), <PDP_type>,,,(list of supported <d_comp>s), (list of supported <h_comp>s),(list ofsupported <IPv4Addr Alloc>s),(list of supported <emergency_indication>s), (list of supported <PCSCF_discovery>s),(list of supported <IM_CN_Signalling_Flag_Ind>s) [<CR><LF>+CGDCONT: (range of supported <cid>s),<PDP_type>,,,(list of supported <d_comp>s),(list of supported <h_comp>s),(list ofsupported <IPv4AddrAlloc>s),(list of supported <emergency_indication>s),(list of supported <PCSCF_discovery>s),(list of supported <IM_CN_Signalling_Flag_Ind>s) [...]] OK |
| Read command Syntax AT+CGDCONT? | Response [+CGDCONT: <cid>, <PDP_type>, <APN>,<PDP_addr>, <d_comp>, <h_comp> [,<IPv4AddrAlloc>[,<emergency_indication>[,<PCSCF_discovery> [,<IM_CN_Signalling_Flag_Ind>]]]]] [<CR><LF>+CGDCONT: <cid>, <PDP_type>, <APN>,<PDP_addr>, <d_comp>, <h_comp>[,<IPv4AddrAlloc>[,<emergency_indication>[,<PCSCF_discovery> [,<IM_CN_Signalling_Flag_Ind>]]]]] [...]] OK |
| Write command Syntax AT+CGDCONT= [<cid> [,<PDP_type> [,<APN> [,<PDP_addr> [,<d_comp> [,<h_comp> [,<IPv4AddrAlloc>[,<emergency_ indication> [,<PCSCF_ discovery> [,<IM_CN_ Signalling_Flag_ Ind>]]]]]]]]]] | Response OK or ERROR Parameters <cid> PDP Context Identifier. A numeric parameter which specifies a particular PDP context definition. The parameter is local to the TE-MT interface and is used in other PDP context-related commands. The range of the permitted values (minimum value = 1) is returned by the test command. <PDP_type> Packet Data Protocol type "IP" Internet Protocol "IPV6" Internet Protocol, version 6 "IPV4V6" Virtual <PDP_type>introduced to handle dual IP stack UE capability <APN> Access Point Name String parameter which is a logical name that is used to select the GGSN or the external packet data network. If the value is null or omitted, then the subscription value will be requested. <PDP_address> String parameter that identifies the MT in the address space applicable to the PDP. If the value is null or omitted then a value may be provided by the TE during the PDP startup procedure or, failing that, a dynamic address will be requested. The read command will continue to return the null string even if an address has been allocated during the PDP startup procedure. The allocated address may be read using the command +CGPADDR command. Note that IPv6 address obtained on LTE will be prefixed with a constant 8 byte address "FE.80.00.00.00.00.00.00" if the network has not provided any. <d_comp> PDP data compression (applicable for SNDCP only) 0 Off (default if value if omitted) 1 On (manufacturer preferred compression) 2 V.42 bis <h_comp> PDP header compression 0 Off (default if value if omitted) 1 On (manufacturer preferred compression) 2 RFC1144 (applicable for SNDCP only) 3 RFC2507 4 RFC3095 (applicable for PDCP only) <IPv4AddrAlloc> Numeric parameter that controls how MT/TA requests to get IPv4 address information 0 IPv4 address allocated through NAS signalling 1 IPv4 address allocated through DHCP <emergency_indication> Indicates whether the PDP contect is for emergency bearer services or not 0 PDP context is not for emergency bearer services 1 PDP context is for emergency bearer services <P-CSCF_discovery> Numeric parameter that influences how the MT/TA requests get the P-CSCF address 0 Preference of P-CSCF address discovery not influences by +CGDCONT 1 Preference of P-CSCF address discovery through NAS signalling <IM_CN_Signalling_Flag_Ind> Numeric parameter used to indicate whether the PDP context is for IM CN subsystem related signaling only or not 0 UE indicates that the PDP context is not for IM CN subsystem-related signaling only 1 UE indicates that the PDP context is for IM CN subsystem-related signaling only |
| Notes | - If the command is used only with the one parameter <cid>, it means that the corresponding PDP context becomes undefined. - The APN Control List (ACL) will only be checked if a USIM is inserted. Before performing context definition, it will check if the ACL-service is enabled and activated. If yes, all APNs from ACL of EF-ACL of the USIM will be read out and compared with the requested APN. - If the requested APN is listed in the ACL, the context definition will be performed. - If the requested APN is empty ("") and ACL contains "network provided APN", the context definition will also be requested. - If the APN is not listed in the ACL the command returns error. - If the ACL-service is not enabled or not activated in the USIM or a GSM-SIM is inserted the context definition will be performed without any checks. - Parameters are saved in non-volatile memory over module reboot. |

## +CGDSCONT Command: Define Secondary PDP Context

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGDSCONT=? | Response +CGDSCONT: (range of <cid>s),(list of <cid>s for defined primary contexts), <PDP_type>,,,(list of supported <d_comp>s),(list of supported <h_comp>s),(list of supported <IM_CN_Signalling_Flag_Ind>s) [<CR><LF>+CGDSCONT: (range of <cid>s),(list of <cid>s for defined primary contexts), <PDP_type>,,,(list of supported <d_comp>s),(list of supported <h_comp>s),(list of supported <IM_CN_Signalling_Flag_Ind>s) [...]] OK |
| Read command Syntax AT+CGDSCONT? | Response [+CGDSCONT: <cid>, <p_cid>, <d_comp>, <h_comp> [,<IM_CN_Signalling_Flag_Ind>]] [<CR><LF>+CGDSCONT: <cid>, <p_cid>, <d_comp>,<h_comp> [,<IM_CN_Signalling_Flag_Ind>]] [...]]] OK |
| Write command Syntax AT+CGDSCONT= [<cid>,<p_cid> [,<d_comp> [,<h_comp> [,<IM_CN_ Signalling_Flag_ Ind>]]]] | Response OK or ERROR Parameter <cid> PDP Context Identifier. A numeric parameter that specifies a particular PDP context definition. The parameter is local to the TE-MT interface and is used in other PDP context-related commands. The range of the permitted values (minimum value = 1) is returned by the test command. <p_cid> Primary PDP Context Identifier. Numeric parameter that specifies a particular PDP context definition which has been specified by +CGDCONT. The parameter is local to the TE-MT interface. The list of permitted values is returned by the test command. <d_comp> PDP data compression (applicable for SNDCP only) 0 Off (default value if omitted) 1 On (manufacturer preferred compression) 2 V.42 bis <h_comp> PDP header compression 0 Off (default value if omitted) 1 On (manufacturer preferred compression) 2 RFC1144 (applicable for SNDCP only) 3 RFC2507 4 RFC3095 (applicable for PDCP only) <IM_CN_Signalling_Flag_Ind> Numeric parameter used to indicate whether the PDP context is for IM CN subsystem related signaling only or not 0 UE indicates that the PDP context is not for IM CN subsystem-related signaling only 1 UE indicates that the PDP context is for IM CN subsystem-related signaling only |

## +CGDATA Command: Enter Data State

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGDATA=? | Response +CGDATA: (list of supported <L2P>s) OK |
| Write command Syntax AT+CGDATA = [<L2P> [,<cid> [,<cid> [,...]]]] | Response CONNECT (followed by data transfer) or CME ERROR: <err> Parameters <L2P> String parameter that indicates the layer 2 protocol to be used between the TE and MT PPP Point-to-point protocol for a PDP such as IP M-OPT-PPP MS supports manufacturing specific protocol M-HEX MS supports manufacturing specific protocol M-RAW_IP MS supports manufacturing specific protocol <cid> Numeric parameter which specifies a particular PDP context definition (see +CGDCONT and +CGDSCONT) |

## +CGED Command: GPRS Cell Environment

For HL7618, HL7618RD, HL7650, HL7688, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGED=? | Response +CGED: (list of supported <mode>s) OK |
| Read command Syntax AT+CGED? | Response +CGED: <mode> OK |
| Write command Syntax AT+CGED= [<mode>] | Response If UMTS is not supported: +CGED: Service-Cell: <MCC>,<MNC>,<LAC>,<CI>,<BSIC>,<AcT> Equivalent PLMNs: <MCC>,<MNC> <MCC>,<MNC> <arfcn>,<RxLevServ>,<RfChannels>,<Arfcn_ded>,<RxLevFull>,<RxLevSub>, <RxQualFull>,<RxQualSub>,GSM-<ciphering>, GPRS Ciphering Algorithm: GEA<gprs_ciphering>,<ms_txpwr>,<rx_acc_min>,<cbq>, <cba>,<c2_valid>,<cr_offset>,<tmp_offset>,<penalty_t>,<c1>,<c2>,<ch_type>, <ch_mode>,<txpwr>,<dtx_used>,<dtr_used>,<t3212>,<acc>,<t_adv>,<bs_pa_mfrms>,<dsc>,<rll>,<amr_acs>, <amr_cod_ul>,<amr_cod_dl>,<amr_c_i>, BEP GMSK: <mean_bep_gmsk>,<cv_bep_gmsk>, BEP 8PSK: <mean_bep_8psk>,<cv_bep_8psk>, Neighbour Cell <n>: <MCC>,<MNC>,<LAC>,<CI>,<BSIC>,<arfcn>,<RxLev><C1_nc>,<C2_nc> Neighbour cell content may be repeated up to 6 times. GPRS Parameters: <GPRS_sup>,<RAC>,<Split_Pg_Cycle>,<NCO>,<NOM>,<T3192>, <Acc_Burst_type>, <DRX_Timer_Max>,<PBCCH>,<Ext_Measure_Order> <PSI1_r_per>,<si13_location>,<packet_psi_status>,<packet_si_status>, <ext_upl_tbf_supported>,<ccn_active>,<pfc_feat_supported> Coding Scheme: dl_sc: <dl_sc>,ul_sc: <ul_sc> <Count_LR>,<Count_HR>,<C_R_Hyst>,<C31>,<C32>,<Prior_Acc_Thr> OK If UMTS is supported: +CGED: RAT:<rat>,URR:<rrc_state>,DC:<urrcdc_state>, BP:<urrcbp_state>, M:<urrcm_state>, ERR:<as_error_code>, RC:<release_cause>,OOS:<out_of_service>, BLER:<meas_bler>,TSIR:<target_sir>,MSIR:<meas_sir>, DPUC:<dlpc_power_up_commands_count>, DPDC:<dlpc_power_down_commands_count>, UPUC:<ulpc_power_up_commands_count>, UPDC:<ulpc_power_down_commands_count>, CMOD: <compressed_mode> TPCA:<tx_ul_pwr_ctrl_alg>, DCL:<drx_cycle_length>, UPCS: <ul_pwr_ctrl_step_size>,BTRG:<bler_target>,NHSC:<num_hsscch_codes> HSC:<hierarchical_cell_structure>,HMD:<high_mobility_detected>, LM:<limited_mode>,RJCZ: <urrc_con_rej_cause> CMAX:<UMAC data CQI max value>, CMEAN:<UMAC data CQI mean value>, CMIN:<UMAC data CQI min value>, AFTI:<AMR frame type id>, ATYP:<AMR type> CellId:<cell_identity>, DLF:<dl_frequency>, ULF:<ul_frequency>, C:<ciphering>, D:<ps_data_transfered>,PSM:< power_saving_mode>,Cell:<celltype=AS>, SC:<scrambling_code>, RSCP:<rscp>, ECN0:<ecn0> Cell:<celltype=VAS>, SC:<scrambling_code>, RSCP:<rscp>, ECN0:<ecn0>, DLF:<dl_ frequency> Cell:<celltype=M>, SC:<scrambling_code>, RSCP:<rscp>, ECN0:<ecn0> Cell:<celltype=D>, SC:<scrambling_code>, RSCP:<rscp>, ECN0:<ecn0> Cell:<celltype=G>, B:<gsm_band>,Arfcn:<arfcn>, RxLev:<rxLev>, Bsic:<bsic>, RV: <ranking_value> Cell:<celltype=U>, SC:<scrambling_code>, RSCP:<rscp>, ECN0:<ecn0>, DLF:<dl_ frequency>, RV:<ranking_value> Cell:<celltype=NU>, SC:<scrambling_code>, RSCP:<rscp>, ECN0:<ecn0>, DLF:<dl_ frequency>, RS:<ranking_status> Cell:<celltype=NG>, B:<gsm band>, Arfcn:<arfcn>, RxLev:<rxLev>, Bsic:<bsic>, RS: <ranking_status> RR measurement evaluation: MeasId :<meas_id>, EventId :<event_id>, <par 3>,<par 4>, <par 5>, <par 6>,..., <par N>, MeasId :<meas_id>, EventId :,<par 3>,<par 4>,<par 5>,<par 6>,...,<par M>,etc... MM: Process:CO, MMs:<mm_state>,MMSs:<mm_service_state>,MSC:<ms_class>, T:<active_timer_bitmap> Process:CS, MMs:<mm_state>,MMSs:<mm_service_state>,LUS:<location_update_status>, T:<active_timer_bitmap>,L:<limited_service> Process:PS, MMs:<mm_state>,MMSs:<mm_service_state>, LUS:<location_update_status>,T:<active_timer_bitmap>,L:<limited_service>, GS:<gprs_supported>,R:<ready_state> Cell change counters: CRT:<cell_reselecetion_total>,IRCR:<ir_cell_reselecetion>,AIRCR:<attempted_ir_cell_reselecetion>,IRHO:<ir_handover>, AIRHO:<attempted_ir_handover> Equivalent PLMNs: MCC:<mobile_country_code>, MNC:<mobile_network_code> Serving PLMN: MCC:<mobile_country_code>,MNC:<mobile_network_code>, LAC:<location_area_code>,RAC:<routing_area_code> The maximum total number of cells is 24. or CME ERROR: <err> Parameters <mode> 0 One shot dump 1 Periodic refreshed dump 2 Stop periodic dump <MCC> 0 – 999 3-digit mobile country code <MNC> 0 – 99 2-digit mobile network code <LAC> 0h – FFFFh 2-octet location area code <CI> 0h – FFFFh 2-octet cell identity <BSIC> 0h – 3Fh 6-bit base station identify code <AcT> 0 GSM 1 GPRS 2 EGPRS 3 EGPRS_PCR 4 EGPRS_EPCR 5 UMTS (unused) 6 DTM 7 EGPRS_DTM 8 Undefined <arfcn> 0 – 1023 Absolute radio frequency channel number <RfChannels> Number of frequencies in MA 0 N.A. 0x01 Single RF <Arfcn_ded> Single ARFCN of dedicated channel of first ARFCN of MA <RxLevFull> 0h – 3Fh Received signal strength on serving cell, measured on all slots <RxLevSub> 0h – 3Fh Received signal strength on serving cell, measured on a subset of slots <RxQualFull> 0 – 7 Received signal quality on serving cell, measured on all slots <RxQualSub> 0 – 7 Received signal quality on serving cell, measured on a subset of slots <ms_txpwr> 0 – 31 Maximum TX power level an MS may use when accessing the system until otherwise commanded <rx_acc_min> 0 – 63 RXLEV-ACCESS-MIN <cbq> 0 – 1 CELL_BAR_QUALIFY <cba> 0 – 1 CELL_BAR_ACCESS <cs_valid> True if all parameter for calculation of c2 are available <cr_offset> 0 – 63 6-bit CELL_RESELECT_OFFSET <tmp_offset> 0 – 7 (mapped to 0 – 70) TEMPORARY_OFFSET <penalty_t> 0 – 31 Penalty time <c1> Value of c1 <c2> Value of c2 <ch_type> Channel type of the current connection 0 INVALID_CHN_TYPE 1 TCH_F 2 TCH_F 3 SDCCH_4 4 SDCCH_8 5 TCH_H_H 6 TCH_F_M <ch_mode> Channel mode of the current connection. Value = 0 – 255 (mapped to an internal value as detailed below) 0 MODE_SIG_ONLY 1 MODE_SPEECH_F 2 MODE_SPEECH_H 3 MODE_DATA_96_F 4 MODE_DATA_48_F 5 MODE_DATA_48_H 6 MODE_DATA_24_F 7 MODE_DATA_24_H 8 MODE_SPEECH_F_V2 9 MODE_SPEECH_F_V3 10 MODE_SPEECH_H_V2 11 MODE_SPEECH_H_V3 12 MODE_DATA_144_F <txpwr> 0 – 31 5-bit transmit power level of the current connection <dtx_used> 0 – 1 DTX used <dtr_used> 0 – 1 DTX used <t3212> 0 – 255 8-bit T3212 timeout value field coded as the binary representation of the timeout value for periodic updating in decihours <acc> 0 – 65535 Access control class (RACH Control Parameters) <t_adv> FFh Timing Advance (not used) <bs_pa_mfrms> 0 – 7 (mapped to 2 – 9) BS_PA_MFRMS (multiframes period for transmission of PAGING REQUEST) <amr_acs> AMR active codec <amr_cod_dl> AMR codec used in DL <amr_cod_ul> AMR codec used in UL <amr_ci_i> AMR C/I in dB/2 <mean_bep_8psk> 0 – 31 MEAN_BEP_8PSK <cv_bep_8psk> 0 – 7 CV_BEP_8PSK <mean_bep_gmsk> 0 – 31 MEAN_BEP_GMSK <cv_bep_gmsk> 0 – 7 CV_BEP_GMSK GPRS Parameters: <GPRS_sup> 0 – 255 GPRS supported (in serving cell) <RAC> 0 – 1 Routing Area Code <Split_Pg_Cycle> 0 – 1 SPGC_CCH_SUP split pg_cycle on ccch by network <NCO> 0 – 3 NETWORK_CONTROL_ORDER (GPRS_Cell_Options) <NOM> 0 – 3 NETWORK OPERATION MODE (GPRS_Cell_Options) <T3192> 0 – 7 (mapped to 0 – 1500msec) Wait for release time of the TBF after reception of the final block 0 500 msec 1 1000 msec 2 1500 msec 3 0 msec 4 80 msec 5 120 msec 7 200 msec <Acc_Burst_type> 0 8-bit access burst 1 11-bit access burst <DRX_Timer_Max> 0 – 7 DRX_TIMER_MAX <PBCCH> PBCCH present <Ext_Measure_Order> 0 – 3 EXT_MEASUREMENT_ORDER <PSI1_r_per> 0 – 15 (mapped to 1 – 16) PSI1_REPEAT_PERIOD <si14_location> “BCCH_NORM” “BCCH_EXT” “NO_BCCH_TYPE” <packet_psi_status> 0 – 1 <packet_si_status> 0 – 1 <ext_upl_tbf_supported> 0 – 1 <ccn_active> 0 – 1 <pfc_feat_supported> 0 – 1 <dl_sc>, <ul_sc> Current modulation and coding scheme of downlink <dl_sc> or uplink <ul_sc> NB_CS_1 NB_CS_2 NB_CS_3 NB_CS_4 NB_MCS_1 NB_MCS_2 NB_MCS_3 NB_MCS_4 NB_MCS_5 NB_MCS_6 NB_MCS_7 NB_MCS_8 NB_MCS_9 NB_MCS_5_7 NB_MCS_6_9 AB_8 AB_11 AB_11_E <Count_LR> 0 – 63 PSI_COUNT_LR <Count_HR> 0 – 15 (mapped to 1 – 16) PSI_COUNT_HR <C_R_Hyst> 0 – 7 CELL-RESELECT-HYSTERESIS <C1> Integer value of c1 <C2> Integer value of c2 <C31> Integer value of c31 <C32> Integer value of c32 <Prior_Acc_Thr> 0 – 7 Prioriry_ACCESS_THR <rrc_state> "CD" CELL_DCH "CF" CELL_FACH "CP" CELL_PCH "UP" URA_PCH "ID" IDLE "ST" START <urrcdc_state> Indicated by three hex digits (octet1, 2: event, 3: state) <urrcbp_state> Indicated by four hex digits (1, 2: event, 3, 4: state) <urrcm_state> Indicated by three hex digits (1: event, 2: state, 3: number of sent measurements) <as_error_code> Indication about error in UAS; integer value with range from 0 – 99 <release_cause> Integer value with range from 0 – 99 <out_of_service> 0 – 1 <meas_bler> Block error rate. Range of values = 1.0 x 10-6 to 9.9 x 10-1 The value ‘-‘ is indicated if the parameter is not available, or for all cells except DCH. The internal received value is divided by 223 before displayed. <target_sir> Target SIR. Range of value = -10 to 20 (3 digits are always displayed); the value ‘-‘ is displayed if the parameter is not available, or for all cells except DCH. The internal received value is divided by 224 before displayed. <meas_sir> Integer displayed in hexadecimal format with range from -10 to 20; the value ‘-‘ is displayed if the parameter is not available, or for all cells except DCH. The internal received value is divided by 224 before displayed. <hierarchical_cell_structure> 0 – 1 <high_mobility_detected> 0 – 1 <limited_mode> 0 – 1 <dlpc_power_up_commands_count> L1 related data counter <dlpc_power_down_commands_count> L1 related data counter <ulpc_power_up_commands_count> L1 related data counter <ulpc_power_down_commands_count> L1 related data counter <compressed_mode> Flag indicating if Compressed Mode is Active or not <tx_ul_pwr_ctrl_alg> Tx Uplink Power Control Algorithm <drx_cycle_length> DRX Cycle Length value 2k <ciphering> Indicates whether GSM Ciphering may be ON or OFF <ps_data_transfered> 0 – 1 <power_saving_mode> 0 – 1 <cell_type> "AS" Active set "VAS" Virtual active set "M" Monitored cells "D" Detected cells "G" GSM cells << U >> UMTS cells << NU >> Non-ranked UMTS cells "NG" Non-ranked GSM cells <scrambling_code> Integer value with range from 0 – 511 <rscp> Received Signal Code Power with range from 0 – 91; 255 for invalid/default value <ecno> Energy per chip/noice with range from 0 – 24; 255 for invalid/default value <gsm_band> "D" 1800 MHz "P" 1900 MHz "G" 900 MHz <arfcn> Absolute radio frequency channel number with range from 0 – 1023 <ranking_value> Integer value with range from 0 – 999 <ranking_status> Integer value with range from 0 – 9 Measurement Parameters: <meas_id> One hex digit with range from 0 – FH <event_id> Two hex digits with range from 1AH – 3DH <par 3,4,5,...,M,...,N> Integer value with range from 0 – 99 <mm_state> Integer value with range from 0 – 99 <mm_service_state> Integer value with range from 0 – 99 <ms_class> MS GPRS-class (previously stored in ATC either at reception of message MN_GCLASS_IND or sending the message MN_GCLASS_REQ). Possible values are: class A class B class CG: class C in GPRS only mode classCC: class C in circuit switched only mode (lowest class) <active_timer_bitmap> Four hex coded digits <location_update_status> Integer value with range from 0 – 9 <limited_service> 0 – 1 <gprs_supported> 0 – 1 <ready_state> 0 – 1 <cell_reselecetion_total> Integer value with range from 0 – 999 <ir_cell_reseelection_counter> Integer value with range from 0 – 999 <attempted_ir_cell_reselection> Integer value with range from 0 – 999 <ir_handover> Integer value with range from 0 – 999 <attempted_ir_handover> Integer value with range from 0 – 999 <routing_area_code> Integer value with range from 0 – 255 |
| Notes | This command returns a dump of the cell environment, either as a one-shot dump or as a periodic refreshed dump (5 seconds each), dependent on the command parameter <mode>. |

## +CGEREP Command: Packet Domain Event Reporting

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGEREP=? | Response +CGEREP: (list of supported <mode>s),(list of supported <bfr>s) OK |
| Read command Syntax AT+CGEREP? | Response +CGEREP: <mode>, <bfr> OK or ERROR |
| Write command Syntax AT+CGEREP= [<mode>[,<bfr>]] | Response OK or ERROR Parameters <mode> 0 Buffer unsolicited result codes in the MT; if MT result code buffer is full, the oldest ones can be discarded. No codes are forwarded to the TE. 1 Discard unsolicited result codes when MT-TE link is reserved (e.g. in on-line data mode); otherwise forward them directly to the TE 2 Buffer unsolicited result codes in the MT when MT-TE link is reserved (e.g. in on-line data mode) and flush them to the TE when MT-TE link becomes available; otherwise forward them directly to the TE <bfr> 0 MT buffer of unsolicited result codes defined within this command is cleared when <mode> 1 or 2 is entered 1 MT buffer of unsolicited result codes defined within this command is flushed to the TE when <mode> 1 or 2 is entered (OK response shall be given before flushing the codes) |
| Unsolicited Notification | Response +CGEV: NW DETACH The network has forces a PS detach +CGEV: NW CLASS <class> The network has forced a change of MT class +CGEV: ME CLASS <class> The mobile termination has forced a change of MT class +CGEV: ME PDN ACT <cid>[,<reason>] The mobile termination has activated a context +CGEV: NW ACT <p_cid>, <cid>, <event_type> The network has activated a context +CGEV: ME ACT <p_cid>, <cid>, <event_type> The network has responded to an ME initiated context activation +CGEV: NW PDN DEACT <cid> The network has deactivated a context +CGEV: ME PDN DEACT <cid> The mobile termination has deactivated a context +CGEV: NW DEACT <p_cid>, <cid>, <event_type> The network has deactivated a context +CGEV: ME DEACT <p_cid>, <cid>, <event_type> The network has responded to an ME initiated context deactivation request +CGEV: NW MODIFY <cid>, <change_reason>, <event_type> The network has modified a context +CGEV: ME MODIFY <cid>, <change_reason>, <event_type> The mobile termination has modified a context Parameters <reason> 0 IPv4 only allowed 1 IPv6 only allowed 2 Single address bearers only allowed 3 Single address bearers only allowed and MT initiated context activation for a second address type bearer was not successful <event_type> 0 Informational event 1 Information request, acknowledgement required <change_reason> 0 TFT only changed 1 QoS only changed 2 Both TFT and QoS changed |
| Notes | <mode> is saved in non-volatile memory over module reboot; URC is available on the port that executes the command. |

## +CGAUTO Command: Automatic Response

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGAUTO=? | Response +CGAUTO: (list of supported <n>s) OK |
| Read command Syntax AT+CGAUTO? | Response +CGAUTO: <n> OK |
| Read command Syntax AT+CGAUTO= [<n>] | Response OK or +CME ERROR: <err> Parameter <n> 0 Turn off automatic response for packet domain only 1 Turn on automatic response for packet domain only 2 Modem compatibility mode, packet domain only 3 Modem compatibility mode, packet domain and circuit switched calls 4 Turn on automatic negative response for packet domain only |
| Notes | - When the +CGAUTO=1 command is received, the MT shall attempt to perform a PS attach if it is not already attached. - <n> is saved in non-volatile memory over module reboot. |

## +CGPADDR Command: Show PDP Address

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGPADDR=? | Response +CGPADDR: (list of supported <cid>s) OK |
| Write command Syntax AT+CGPADDR= [<cid> ,<cid> [,...]]] | Response +CGPADDR: <cid>[,<PDP_addr_1>[,<PDP_addr_2>]] [<CR><LF> +CGPADDR: <cid>[,<PDP_addr_1>[,<PDP_addr_2>]]][...]] OK Parameters <cid> a numeric parameter which specifies a particular PDP context definition (see the +CGDCONT and +CGDSCONT commands). If no <cid> is specified, the addresses for all defined contexts are returned. <PDP_addr_1>, <PDP_addr_2> String that identifies the MT in the address space applicable to the PDP. The address may be static or dynamic. For a static address, it will be the one set by the +CGDCONT and +CGDSCONT commands when the context was defined. For a dynamic address, it will be the one assigned during the last PDP context activation that used the context definition referred to by <cid>. Both <PDP_addr_1> and <PDP_addr_2> are omitted if none are available. Both <PDP_addr_1> and <PDP_addr_2> are included when both Ipv4 and Ipv6 addresses are assigned, with <PDP_addr_1> containing the IPv4 address and <PDP_addr_2> containing the IPv6 address. The string is given as dot-separated numeric (0 – 255) parameter of the form: a1.a2.a3.a4 for IPv4 and a1.a2.a3.a4.a5.a6.a7.a8.a9.a10.a11.a12.a13.a14.a15.a16 for IPv6. |

## +CGQMIN Command: Quality of Service Profile (Minimum)

For HL7618, HL7618RD, HL7650, HL7688, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGQMIN=? | Response +CGQMIN: <PDP_type>, (list of supported <precedence>s), (list of supported <delay>s), (list of supported <reliability>s), (list of supported <peak>s), (list of supported <mean>s) OK |
| Read command Syntax AT+CGQMIN? | Response +CGQMIN: <cid>, <precedence>, <delay>, <reliability>, <peak>, <mean> OK |
| Write command Syntax AT+CGQMIN= [<cid> [,<precedence> [,<delay> [,<reliability> [,<peak> [,<mean>]]]]]] | Response OK or ERROR Parameters <cid> Numeric parameter that specifies a particular PDP context definition. Refer to the defined values under the +CGDCONT command. <precedence> Numeric parameter for the precedence class <delay> Numeric parameter for the delay class <reliability> Numeric parameter for the reliability class <peak> Numeric parameter for the peak throughput class <mean> Numeric parameter for the mean throughput class |
| Notes | If a value is omitted for a particular class then the value is considered to be unspecified. |

## +CGEQMIN Command: 3G Quality of Service Profile (Minimum)

For HL7618, HL7618RD, HL7650, HL7688, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGEQMIN=? | Response +CGEQMIN: <PDP_type>, (list_of supported <Traffic_class>es) ,(list of supported <Maximum_bitrate_UL>s) , (list of supported <Maximum_bitrate_DL>s), (list of supported <Guaranteed_bitrate_UL>s), (list of supported <Guaranteed_bitrate_DL>s) ,(list of supported <Delivery_order>s) ,(list of supported <Maximum_SDU_size>s) ,(list of supported <SDU_error_ratio>s) , (list of supported <Residual_bit_error_ratio>s) ,(list of supported <Delivery_of_erroneous_SDUs>s) , (list of supported <Transfer_delay>s) ,(list of supported <Traffic_handling_priority>s) [,(list of supported <Source_statistics_descriptor>s) ,(list of supported <Signalling_indication>s)] [<CR><LF> +CGEQMIN: <PDP_type>, (list of supported <Traffic_class>es) ,(list of supported <Maximum_bitrate_UL>s), (list of supported <Maximum_bitrate_DL>s) ,(list of supported <Guaranteed_bitrate_UL>s), (list of supported <Guaranteed_bitrate_DL>s), (list of supported <Delivery_order>s) ,(list of supported <Maximum_SDU_size>s) ,(list of supported <SDU_error_ratio>s),(list of supported <Residual_bit_error_ratio>s) ,(list of supported <Delivery_of_erroneous_SDUs>s) ,(list of supported <Transfer_delay>s) ,(list of supported <Traffic_handling_priority>s) [,(list of supported <Source_statistics_descriptor>s) ,(list of supported <Signalling_indication>s)][...]] ERROR |
| Read command Syntax AT+CGEQMIN? | Response +CGEQMIN: <cid>, <Traffic_class> ,<Maximum_bitrate_UL>, <Maximum_bitrate_DL>, <Guaranteed_bitrate_UL>,<Guaranteed_bitrate_DL>, <Delivery_order>, <Maximum_SDU_size>,<SDU_error_ratio>,<Residual_bit_error_ ratio>, <Delivery_of_erroneous_SDUs>,<Transfer_delay>,<Traffic_handling_ priority> [,<Source_statistics_descriptor>,<Signalling_indication>] [<CR><LF> +CGEQMIN: <cid>,<Traffic_class>,<Maximum_bitrate_UL>, <Maximum_bitrate_DL> ,<Guaranteed_bitrate_UL>,<Guaranteed_bitrate_DL>, <Delivery_order>,<Maximum_SDU_size>,<SDU_error_ratio>, <Residual_bit_error_ ratio>,<Delivery_of_erroneous_SDUs>,<Transfer_delay>, <Traffic_handling_ priority>[,<Source_statistics_descriptor> ,<Signalling_indication>][...]] Error |
| Write command Syntax AT+CGEQMIN= [<cid>[,<Traffic_ class> [,<Maximum_ bitrate_UL> [,<Maximum_ bitrate_DL> [,<Guaranteed_ bitrate_UL> [,<Guaranteed_ bitrate_DL> [,<Delivery_ order> [,<Maximum_ SDU_size> [,<SDU_error_ ratio>[,<Residual_bit_error_ratio> [,<Delivery_of_ erroneous_ SDUs> [,<Transfer_ delay>[,<Traffic_handling_ priority> [,<Source_ statistics_ descriptor>, <Signalling_ indication> ]]]]]]]]]]]]]]] | Response OK or ERROR Parameter <cid> Numeric parameter which specifies a particular PDP context definition (see +CGDCONT and +CGDSCONT commands). <Traffic_class> UMTS bearer service application type 0 Conversational 1 Streaming 2 Interactive 3 Background <Maximum_bitrate_UL> Numeric parameter that indicates the maximum number of kbits/s delivered to UMTS (up-link traffic) at a SAP. <Maximum_bitrate_DL> Numeric parameter that indicates the maximum number of kbits/s delivered by UMTS (down-link traffic) at a SAP. <Guaranteed_bitrate_UL> Numeric parameter that indicates the guaranteed number of kbits/s delivered to UMTS (up-link traffic) at a SAP (provided that there is data to deliver). <Guaranteed_bitrate_DL> Numeric parameter that indicates the guaranteed number of kbits/s delivered by UMTS (down-link traffic) at a SAP (provided that there is data to deliver). <Delivery_order> Numeric parameter that indicates whether the UMTS bearer shall provide in-sequence SDU delivery or not 0 No 1 Yes <Maximum_SDU_size> Numeric parameter that indicates the maximum allowed SDU size in octets <SDU_error_ratio> String parameter that indicates the target value for the fraction of SDUs lost or detected as erroneous. SDU error ratio is defined only for conforming traffic. The value is specified as 'mEe'. <Residual_bit_error_ratio> String parameter that indicates the target value for the undetected bit error ratio in the delivered SDUs. If no error detection is requested, Residual bit error ratio indicates the bit error ratio in the delivered SDUs. The value is specified as 'mEe'. <Delivery_of_erroneous_SDUs> Numeric parameter that indicates whether SDUs detected as erroneous shall be delivered or not 0 No 1 Yes 2 No detect <Transfer_delay> Numeric parameter that indicates the targeted time between request to transfer an SDU at one SAP to its delivery at the other SAP, in milliseconds <Traffic_handling_priority> Numeric parameter that specifies the relative importance for handling of all SDUs belonging to the UMTS bearer compared to the SDUs of other bearers <Source_Statistics_Descriptor> Supported in R7 P S a numeric parameter that specifies characteristics of the source of the submitted SDUs for a PDP context. This parameter should be provided if the Traffic class is specified as conversational or streaming 0 Characteristics of SDUs is unknown 1 Charactersitics of SDUs correspond to a speech source <Signalling_Indication> Supported in R7 P S a numeric parameter used to indicate content of submitted SDUs for a PDP context. This parameter should be provided if the Traffic class is specified as interactive 0 PDP context is not optimized 1 PDP context is optimized <PDP_type> Refer to +CGDCONT and +CGDSCONT commands. |
| Notes | If a value is omitted for a particular class then the value is considered to be unspecified. |

## +CGQREQ Command: Request Quality of Service Profile

For HL7618, HL7618RD, HL7650, HL7688, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGQREQ=? | Response +CGQREQ: <PDP_type>, (list of supported <precedence>s), (list of supported <delay>s), (list of supported <reliability>s),(list of supported <peak>s), (list of supported <mean>s) OK |
| Read command Syntax AT+CGQREQ? | Response +CGQREQ: <cid>, <precedence>, <delay>, <reliability>, <peak>, <mean> OK |
| Write command Syntax AT+CGQREQ = [<cid> [,<precedence> [,<delay> [,<reliability> [,<peak> [,<mean>]]]]]] | Response OK or ERROR Parameters <cid> Numeric parameter that specifies a particular PDP context definition. <precedence> Numeric parameter that specifies the precedence class <delay> Numeric parameter that specifies the delay class <reliability> Numeric parameter that specifies the reliability class <peak> Numeric parameter that specifies the peak throughput class <mean> Numeric parameter that specifies the mean throughput class. |
| Notes | - This command allows the TE to specify a Quality of Service Profile that is used when the MT sends an Activate PDP Context Request message to the network. - If a value is omitted for a particular class then the value is considered to be unspecified. |

## +CGEQREQ Command: 3G Request Quality of Service Profile

For HL7618, HL7618RD, HL7650, HL7688, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGEQREQ=? | Response +CGEQREQ: <PDP_type>, (list_of supported <Traffic_class>es) ,(list of supported <Maximum_bitrate_UL>s) ,(list of supported <Maximum_bitrate_DL>s), (list of supported <Guaranteed_bitrate_UL>s), (list of supported <Guaranteed_bitrate_DL>s) ,(list of supported <Delivery_order>s) ,(list of supported <Maximum_SDU_size>s) ,(list of supported <SDU_error_ratio>s) , (list of supported <Residual_bit_error_ratio>s) ,(list of supported <Delivery_of_erroneous_SDUs>s) ,(list of supported <Transfer_delay>s) ,(list of supported <Traffic_handling_priority>s) [,(list of supported <Source_statistics_descriptor>s) ,(list of supported <Signalling_indication>s)] [<CR><LF>+CGEQREQ: <PDP_type>, (list of supported <Traffic_class>es) ,(list of supported <Maximum_bitrate_UL>s), (list of supported <Maximum_bitrate_DL>s) ,(list of supported <Guaranteed_bitrate_UL>s), (list of supported <Guaranteed_bitrate_DL>s), (list of supported <Delivery_order>s) ,(list of supported <Maximum_SDU_size>s) ,(list of supported <SDU_error_ratio>s) ,(list of supported <Residual_bit_error_ratio>s) ,(list of supported <Delivery_of_erroneous_SDUs>s) ,(list of supported <Transfer_delay>s) ,(list of supported <Traffic_handling_priority>s) [,(list of supported <Source_statistics_descriptor>s),(list of supported <Signalling_indication>s)][...]] |
| Read command Syntax AT+CGEQREQ? | Response +CGEQREQ: <cid>,<Traffic_class>,<Maximum_bitrate_UL>,<Maximum_bitrate_DL>, <Guaranteed_bitrate_UL>,<Guaranteed_bitrate_DL>,<Delivery_order>, <Maximum_SDU_size>,<SDU_error_ratio>,<Residual_bit_error_ratio>, <Delivery_of_erroneous_SDUs>,<Transfer_delay>,<Traffic_handling_priority> [,<Source_statistics_descriptor> ,<Signalling_indication>] [<CR><LF>+CGEQREQ: <cid>,<Traffic_class>,<Maximum_bitrate_UL>, <Maximum_bitrate_DL>,<Guaranteed_bitrate_UL>,<Guaranteed_bitrate_DL>, <Delivery_order>,<Maximum_SDU_size>,<SDU_error_ratio>, <Residual_bit_error_ ratio>,<Delivery_of_erroneous_SDUs>,<Transfer_delay>, <Traffic_handling_ priority>[,<Source_statistics_descriptor>,<Signalling_indication>] [...]] |
| Write command Syntax AT+CGEQREQ= [<cid>[,<Traffic_ class> [,<Maximum_ bitrate _UL> [,<Maximum_ bitrate_DL> [,<Guaranteed_ bitrate_UL> [,<Guaranteed_ bitrate_DL> [,<Delivery_ order> [,<Maximum_ SDU_size> [,<SDU_error_ ratio>[,<Residual_bit_error_ratio> [,<Delivery_of_ erroneous_ SDUs> [,<Transfer_ delay>[,<Traffic_ handling_ priority> [,<Source_ statistics_ descriptor>, <Signalling_ indication> ]]]]]]]]]]]]]]] | Response OK or ERROR Parameters <cid> Numeric parameter which specifies a particular PDP context definition (see +CGDCONT and +CGDSCONT commands) <Traffic_class> UMTS bearer service application type 0 Conversational 1 Streaming 2 Interactive 3 Background <Maximum_bitrate_UL> Numeric parameter that indicates the maximum number of kbits/s delivered to UMTS (up-link traffic) at a SAP. <Maximum_bitrate_DL> Numeric parameter that indicates the maximum number of kbits/s delivered by UMTS (down-link traffic) at a SAP. <Guaranteed_bitrate_UL> Numeric parameter that indicates the guaranteed number of kbits/s delivered to UMTS (up-link traffic) at a SAP (provided that there is data to deliver). <Guaranteed_bitrate_DL> Numeric parameter that indicates the guaranteed number of kbits/s delivered by UMTS (down-link traffic) at a SAP (provided that there is data to deliver). <Delivery_order> Numeric parameter that indicates whether the UMTS bearer shall provide in-sequence SDU delivery or not 0 No 1 Yes <Maximum_SDU_size> Numeric parameter that indicates the maximum allowed SDU size in octets <SDU_error_ratio> String parameter that indicates the target value for the fraction of SDUs lost or detected as erroneous. SDU error ratio is defined only for conforming traffic. The value is specified as 'mEe'. <Residual_bit_error_ratio> String parameter that indicates the target value for the undetected bit error ratio in the delivered SDUs. If no error detection is requested, Residual bit error ratio indicates the bit error ratio in the delivered SDUs. The value is specified as 'mEe'. <Delivery_of_erroneous_SDUs> Numeric parameter that indicates whether SDUs detected as erroneous shall be delivered or not 0 No 1 Yes 2 No detect <Transfer_delay> Numeric parameter that indicates the targeted time between request to transfer an SDU at one SAP to its delivery at the other SAP, in milliseconds <Traffic_handling_priority> Numeric parameter that specifies the relative importance for handling of all SDUs belonging to the UMTS bearer compared to the SDUs of other bearers <Source_Statistics_Descriptor> Supported in R7 P S a numeric parameter that specifies characteristics of the source of the submitted SDUs for a PDP context. This parameter should be provided if the Traffic class is specified as conversational or streaming 0 Characteristics of SDUs is unknown 1 Charactersitics of SDUs correspond to a speech source <Signalling_Indication> Supported in R7 P S a numeric parameter used to indicate content of submitted SDUs for a PDP context. This parameter should be provided if the Traffic class is specified as interactive 0 PDP context is not optimized 1 PDP context is optimized <PDP_type> Refer to +CGDCONT and +CGDSCONT commands. |
| Notes | If a value is omitted for a particular class then the value is considered to be unspecified. |

## +CGEQNEG Command: 3G Negotiated Quality of Service Profile

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGEQNEG=? | Response +CGEQNEG: (list of <cid>s associated with active contexts) |
| Write command Syntax AT+CGEQNEG= [<cid>[,<cid> [,...]]] | Response +CGEQNEG: <cid>,<Traffic class>,<Maximum bitrate UL>, <Maximum bitrate DL>,<Guaranteed bitrate UL>,<Guaranteed bitrate DL>, <Delivery order>,<Maximum SDU size>,<SDU error ratio>,<Residual bit error ratio>, <Delivery of erroneous SDUs>,<Transfer delay>,<Traffic handling priority>,<Source statistics descriptor>,<Signaling indication> [<CR><LF>+CGEQNEG: <cid>,<Traffic class>,<Maximum bitrate UL>, <Maximum bitrate DL>,<Guaranteed bitrate UL>,<Guaranteed bitrate DL>, <Delivery order>,<Maximum SDU size>,<SDU error ratio>,<Residual bit error ratio>, <Delivery of erroneous SDUs>,<Transfer delay>,<Traffic handling priority>,<Source statistics descriptor>,<Signaling indication>[....]] Parameters <cid> numeric parameter which specifies a particular PDP context definition (see +CGDCONT and +CGDSCONT commands) <Traffic_class> UMTS bearer service application type 0 Conversational 1 Streaming 2 Interactive 3 Background <Maximum_bitrate_UL> Numeric parameter that indicates the maximum number of kbits/s delivered to UMTS (up-link traffic) at a SAP. <Maximum_bitrate_DL> Numeric parameter that indicates the maximum number of kbits/s delivered by UMTS (down-link traffic) at a SAP. <Guaranteed_bitrate_UL> Numeric parameter that indicates the guaranteed number of kbits/s delivered to UMTS (up-link traffic) at a SAP (provided that there is data to deliver). <Guaranteed_bitrate_DL> Numeric parameter that indicates the guaranteed number of kbits/s delivered by UMTS (down-link traffic) at a SAP (provided that there is data to deliver). <Delivery_order> Numeric parameter that indicates whether the UMTS bearer shall provide in-sequence SDU delivery or not 0 No 1 Yes <Maximum_SDU_size> Numeric parameter that indicates the maximum allowed SDU size in octets <SDU_error_ratio> String parameter that indicates the target value for the fraction of SDUs lost or detected as erroneous. SDU error ratio is defined only for conforming traffic. The value is specified as 'mEe'. <Residual_bit_error_ratio> String parameter that indicates the target value for the undetected bit error ratio in the delivered SDUs. If no error detection is requested, Residual bit error ratio indicates the bit error ratio in the delivered SDUs. The value is specified as 'mEe'. <Delivery_of_erroneous_SDUs> Numeric parameter that indicates whether SDUs detected as erroneous shall be delivered or not 0 No 1 Yes 2 No detect <Transfer_delay> Numeric parameter that indicates the targeted time between request to transfer an SDU at one SAP to its delivery at the other SAP, in milliseconds <Traffic_handling_priority> Numeric parameter that specifies the relative importance for handling of all SDUs belonging to the UMTS bearer compared to the SDUs of other bearers <Source statistics descriptor> Numeric parameter that specifies the characteristics of the source of submitted SDUs <Signaling indication> Numeric parameter that indicates the signalling nature of the submitted SDUs. This parameter is in addition to the other QoS attributes and does not override them; it is only defined for the interactive traffic class. If signalling indication is set to 'Yes', the UE should set the traffic handling priority to '1'. |
| Notes | - If a value is omitted for a particular class then the value is considered to be unspecified. - Parameter details can be referenced from 3GPP specifications TS24.008 section 10.5.6.5 and TS23.107 section 6.4.3.1. |
| Examples | AT+CGDCONT? +CGDCONT: 1,"IP","smartone","10.149.7.167",0,0,0,0,0,0 +CGDCONT: 3,"IP","internet","121.203.230.208",0,0,0,0,0,0 OK AT+CGEQNEG=? // Test command +CGEQNEG: (1,3) OK AT+CGEQNEG=3 // Write command for cid = 3 +CGEQNEG: 3,4,0,0,0,0,0,0,"0E0","0E0",3,0,0,0,0 OK |

## +CGREG Command: GPRS Network Registration Status

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGREG=? | Response +CGREG: (list of supported <n>s) OK |
| Read command Syntax AT+CGREG? | Response +CGREG: <n>,<stat>[,<lac>,<ci>[,<AcT>,<rac>]] OK |
| Write command Syntax AT+CGREG= [<n>] | Response OK or +CME ERROR: <err> Parameters <n> 0 Disable network registration unsolicited result code 1 Enable network registration unsolicited result code +CGREG: <stat> 2 Enable network registration and location information unsolicited result code +CGREG: <stat>[,<lac>,<ci>[,<AcT>,<rac>]] <stat> 0 Not registered, home network 1 Registered, home network 2 Not registered, but ME is currently searching for a new operator to register to 3 Registration denied 4 Unknown 5 Registered, roaming 8 Attached for emergency bearer services only (only applicable when <AcT>=2, 4, 5, 6) <lac> String type; two-byte location area code in hexadecimal format. “FFFF” indicates that the location area code is invalid and that the <rac> value should also be ignored. <ci> String type; two-byte cell ID in hexadecimal format for GSM; or four-byte UTRAN/E-UTRAN cell ID in hexadecimal format <Act> 0 GSM 1 GSM Compact 2 UTRAN 3 GSM with EGPRS 4 UTRAN with HSDPA 5 UTRAN with HSUPA 6 UTRAN with HSDPA and HSUPA 7 E-UTRAN <rac> String type; one-byte routing area code in hexadecimal format. “00” indicates that the routing area code is invalid. It has the same meaning as “FF” specified in the +KCCINFO command. |
| Unsolicited Notification | Response +CGREG: <stat> +CGREG: <stat>[,<lac>,<ci>[,<AcT>,<rac>]] |
| Reference Sierra Wireless Proprietary | Notes <n> is saved in non-volatile memory per AT port over module reboot |
| Examples | AT+CGREG? // Read command +CGREG: 0,0 OK AT+CGREG=? // Test command +CGREG: (0-2) OK AT+CGREG=2 // Set mode to 2 OK AT+COPS=0 OK +CGREG: 1,"008C","6771",0,"01" // URC displayed after attaching to network |

## +CGSMS Command: Select Service for MO SMS Messages

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGSMS=? | Response +CGSMS: (list of currently available <service>s) OK |
| Read command Syntax AT+CGSMS? | Response +CGSMS: <service> OK |
| Write command Syntax AT+CGSMS= [<service>] | Response OK or ERROR Parameter <service> Indicates the service or service preference to be used 0 Packet Domain 1 Circuit switched 2 Packet Domain preferred (use circuit switched if GPRS is not available) 3 Circuit switched preferred (use packet domain if circuit switched is not available) |
| Note | +CGSMS is ignored for sending SMS over IMS as used in the Verizon network. |

## +CRLP Command: Select Radio Link Protocol

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CRLP=? | Response +CRLP: (list of supported <iws>es),(list of supported <mws>es),(list of supported <T1>s), (list of supported <N2>s) OK |
| Read command Syntax AT+CRLP? | Response +CRLP: <iws>,<mws>,<T1>,<N2> OK |
| Write command Syntax AT+CRLP=[<iws>[,<mws>[,<T1> [,<N2>]]]] | Response OK or +CME ERROR: <err> Parameters <iws> IWF to MS window size <mws> MS to IWF window size <T1> Acknowledgement timer (in units of 10 ms) <N2> Retransmission attempts |

## +XDNS Command: Dynamic DNS Request

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+XDNS=? | Response +XDNS: (list of supported <cid>s),(list of supported <mode>s) OK |
| Read command Syntax AT+XDNS? | Response +XDNS: <cid>, <primary DNS>, <secondary DNS> [+XDNS: <cid>, <primary DNS>, <secondary DNS> [...]] OK |
| Write command Syntax AT+XDNS=<cid>, <mode> | Response OK or +CME ERROR: <err> Parameters <cid> Context ID <mode> 0 Disable dynamic DNS request 1 Enable dynamic DNS request (IPv4) 2 Enable dynamic DNS request (IPv6) 3 Enable dynamic DNS request (IPv4v6) <primary DNS>, <secondary DNS> Strings representing the DNS addresses and given as dot-separated numeric (0 – 255) parameters in the form of: a1.a2.a3.a4 for IPv4, a1.a2.a3.a4.a5.a6.a7.a8.a9.a10.a11.a12.a13.a14.a15.a16 for IPv6, and a1.a2.a3.a4.a5.a6.a7.a8.a9.a10.a11.a12.a13.a14.a15.a16.a17.a18.a19.a20 for IPv4v6 (a1 to a4 here represents IPV4 and a5 to a20 represents IPv6) The DNS address is by default "0.0.0.0" which is not a valid address. Note that IPv6 address obtained on LTE will be prefixed with a constant 8 byte address "FE.80.00.00.00.00.00.00" if the network has not provided any. |

## +CGPIAF Command: Printing IP Address Format

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CGPIAF=? | Response +CGPIAF: (list of supported <IPv6_AddressFormat>s),(list of supported <IPv6_SubnetNotation>s),(list of supported <IPv6_LeadingZeros>s),(list of supported <IPv6_CompressZeros>s) |
| Read command Syntax AT+CGPIAF? | Response +CGPIAF: <IPv6_AddressFormat>,<IPv6_SubnetNotation>,<IPv6_LeadingZeros>, <IPv6_CompressZeros> OK |
| Write command Syntax AT+CGPIAF= [<IPv6_Address Format>[,<IPv6_SubnetNotation> [,<IPv6_Leading Zeros>[,<IPv6_ CompressZeros>]]]] | Response OK or +CME ERROR: <err> Parameters <IPv6_AddressFormat> 0 Use IPv4-like dot notation. IP address and subnetwork mask (if applicable) are dot-separated. 1 Use IPv6-like colon notation. IP address and subnetwork mask (if applicable and when given explicitly) are separated by a space. <IPv6_SubnetNotation> Specifies the subnet notation for remote address and subnet mask. This parameter setting does not apply if <IPv6_AddressFormat> = 0. 0 Both IP address and subnet mask are stated explicitly, and separated by a space 1 The printout format uses a slash (/) subnet-prefix Classless Inter-Domain Routing (CIDR) notation <IPv6_LeadingZeros> Specifies whether leading zeros are omitted or not. This parameter setting does not apply if <IPv6_AddressFormat> = 0. 0 Leading zeros are omitted 1 Leading zeros are inclued <IPv6_CompressZeros> Specifies whether 1-n instances of 16-bit zero values are replaced by “::”.This parameter setting does not apply if <IPv6_AddressFormat> = 0. 0 No zero compression 1 Use zero compression If the address is unspecified (all bytes are zeros), “::” will be displayed. |
| Notes | Parameters are saved in non-volatile memory per AT port over module reboot. |

## +WPPP Command: PDP Context Authentication Configuration

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+WPPP=? | Response +WPPP: (list of supported <Auth>s),[(list of supported <cid>s)] OK |
| Read command Syntax AT+WPPP? | Response +WPPP: <Auth>,[<cid>],[<username>],[<password>] OK |
| Write command Syntax AT+WPPP= <Auth>,[<cid>], [<username>], [<password>] | Response OK or +CME ERROR <err> Parameters <Auth> Type of authentication supported 0 None 1 PAP 2 CHAP <cid> 1 – 20 PDP context identifier used in +CGDCONT. If omitted, the configuration is set for all PDP contexts. <username> Login for the APN. String type, up to 30 characters <password> Password for the APN. String type, up to 30 characters |
| Notes | - +WPPP is available when SIM has been inserted and the pin code is entered. - Parameters are stored in non-volatile memory. |
| Examples | AT+WPPP=? +WPP: (0-2),(1-20) OK AT+WPPP=1,1,“myusername”,“mypassword” OK AT+WPPP? +WPPP: 1,1,“myusername”,“mypassword” OK |

# SIM Application Toolkit Commands

## +STKPRO Command: Display List of Supported Proactive Commands

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+STKPRO=? | Response +STKPRO: (01,05,16,17,18,19,20,21,32,33,34,35,36,37,38,40,52,53,64) OK |
| Unsolicited Notification | Response +STKPRO: <proactive_cmd>... Details of which are as follows: - +STKPRO: 01, <type> - +STKPRO: 05, <event_list> - +STKPRO: 16, <number>, <subaddr>, <type>, <alpha_1>, <icon_id1>, <alpha_2>,<icon_id2> - +STKPRO: 17, <ss_data>, <alpha>, <icon_id>, <ref_number> - +STKPRO: 18, <dcs>, <hex_string>, <alpha>, <icon_id>, <ref_number> - +STKPRO: 19, <alpha>, <icon_id>, <ref_number> - +STKPRO: 20, <alpha>, <icon_id>, <dtmf_string> - +STKPRO: 21, <URL>, <alpha>, <icon_id> - +STKPRO: 32, <tone>, <unit>, <interval>, <alpha>, <icon_id> - +STKPRO: 33, <type>, <dcs>, <hex_string>, <icon_id> - +STKPRO: 34, <type>, <dcs>, <hex_string>, <icon_id> - +STKPRO: 35, <type>, <dcs>, <hex_string>, <max_rsp_len>, <min_rsp_len>, <defaulttext>, <icon_id> - +STKPRO: 36, <type>, <alpha>, <item_id>, <total_items>, <item_text>, <next_action>, <default_item>, <icon_id>, <icon_id_list_element> - +STKPRO: 37, <type>, <alpha>, <item_id>, <total_items>, <item_text>, <next_action>, <icon_id>, <icon_id_list_element> - +STKPRO: 38, <type> - +STKPRO: 40, <dcs>, <hex_string>, <icon_id> - +STKPRO: 52, <type>, <alpha>, <icon_id> - +STKPRO: 53, <language> - +STKPRO: 64, <cmd_qualifier>, <alpha_id>, <icon_refrence>, <dialing_number>, <reconnect_interval>, <reconnect_unit>, <idle_interval>, <idle_unit>, <bearer_type>, <bearer_parameter>, <buffer_size>, <login_dcs>, <login_text>, <password_dcs>, <password_text>, <transaport_level>, <transport_port>, <sub_address>, <destination_address_type>, <destination_address> Parameters <alpha>, <alpha_1>, <alpha_2>, <item_text>, <default text> Text string <dsc> Data coding scheme <default_item> Default items (s. item_id) <event_list> 04 User activity event 05 Idle screen available event 07 Language selection 08 Browser termination event <hex_string> String containing data in hexadecimal format <icon_id>, <icon_id1>, <icon_id2>, <icon_id_list_element> List containing icon IDs. For example, <icon_id1>, <icon_id2> <interval> Time duration in number of units <item_id> Item identifier (identifier of item chosen, refer to GSM 11.14) <language> 2-byte string indicating the language <max rsp len> Maximum response length <min rsp len> Minimum response length <next_action> Next action <number> Called party number <proactive_cmd> 01 Refresh 05 Set up event list 16 Set up call 17 Send SS 18 Send USSD 19 Send SMS 20 Send DTMF 21 Launch browser 32 Play tone 33 Display text 34 Get inkey 35 Get input 36 Select item 37 Set up menu 38 Language setting 39 Timer management 40 Set up idle mode text 52 Run AT command info 53 Language notification 64 Open channel 129 End of the proactive session <ref_number> Reference number <subaddr> Called party subaddress <ss_data> Data string <type> Integer as command qualifier; possible value “4” means language <tone> 01 Dial tone 02 Call subscriber busy 03 Congestion 04 Radio path acknowledge 05 Radio path not available 06 Error/special information 07 Call waiting tone 08 Ringing tone 10 General beep 11 Positive acknowledgement tone 12 Negative acknowledgement or error tone <total items> Total items <unit> 0 Minutes 1 Seconds 2 Tenth of a second <URL> URL to be loaded <reconnect_interval> 1 – 255 Duration for reconnect tries. The interval specifies the time interval of the duration in multiples of the time unit used. The value “0” indicated a non-existing duration object. <reconnect_unit> Used with <reconnect_interval> 0 Minutes 1 Seconds 2 Tenth of a second <idle_interval> 1 – 255 Defines the duration when an idle connection is released automatically. If not present, the terminal will never release a connection automatically. A value of “0” indicates a non-existing duration object. <idle_unit> Used with <idle_interval> 0 Minutes 1 Seconds 2 Tenth of a second <bearer_type> 1 Circuit switched 2 Packet switched 3 Default 255 Invalid <bearer_parameter> Hex string that gived detailed information about the bearer type <buffer_size> Buffer the terminal shall allocate for channel data. The terminal may allocate less or more than this. <login_dcs> Data coding scheme of the text string. Text strings may be coded in 7-bit, 8-bit or UCS2 (16-bit) for user authentication data if requested by the bearer connection. <login_text> Specfies user authentication data is requested by the bearer connection. Coding based on <login_dcs>. <password_dcs> Data coding scheme of the text string. Text strings may be coded in 7-bit, 8-bit or UCS2 (16-bit) for user authentication data if requested by the bearer connection. <password_text> Specifies user authentication data if requested by the bearer connection. Coding based on <password_dcs>. <transport_level> Transport layer protocol of the UICC/terminal connection 1 UDP 2 TCP 255 Invalid; no transport protocol specified <transport_port> Integer that specifies the transport port <sub_address> Called party subaddress (for CS bearers only) <dsc> Data coding scheme <destination_address_type> 33 IPv4 IP address 87 IPv6 IP address 255 Invalid; unknown address type <destination_address> Hex string that specifies the destination point of the connection |

## +STKTR Command: Enter Response

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+STKTR=? | Response +STKTR: (01,05,16,17,18,19,20,21,32,33,34,35,36,37,38,39,40,52,53,64) OK |
| Write command Syntax AT+STKTR=1,0 | Response OK or +CME ERROR: <err> |
| Execute command Syntax AT+STKTR= <proactive_cmd> [,<result>, <add_result> [,<last_cmd>] [,<dcs>] [,<hexstring>]] | Response Response depends on the proactive command - +STKTR: 01, <result>, [<add_result>] - +STKTR: 05, <result> - +STKTR: 16, <result>, [<add_result>] - +STKTR: 17, <result>, <add_result> - +STKTR: 18, <result>, <add_result> - +STKTR: 19, <result>, <add_result> - +STKTR: 20, <result>, [<add_result>] - +STKTR: 21, <result> - +STKTR: 32, <result>, <add_result> - +STKTR: 33, <result>, <add_result> - +STKTR: 34, <result>, <add_result>,0,<dcs>,<hex_string> - +STKTR: 35, <result>, <add_result>,0,<dcs>,<hex_string> - +STKTR: 36, <result>, <add_result>,0,<dcs>,<hex_string> The “0” stands for the parameter <last_cmd> which is obsolete but not yet removed. - +STKTR: 37, <result>, <add_result> - +STKTR: 38, <language as integer, e.g.28261> - +STKTR: 40, <result>, <add_result> - +STKTR: 52, <result>, <add_result> - +STKTR: 53, <result>, <add_result> For general results (<result>) 32, 33, 38, 52, 53, 55, 56, 57 and 58, it is mandatory for the ME to provide a specific cause value as additional information. For others, additional information will be ignored. - +STKTR: 64, <result>[,<add_result>,<last_cmd>,<buffer_size>, <open_channel_id>,<link_status>,<channel_status_state>, <bearer_description_type>,<bearer_description_params>, <address_type>,<address>] Parameters <add_result> Additional result <dcs> Data coding scheme <hex_string> String in hexadecimal format <last_cmd> Last command <proactive_cmd> Decimal code that indicates the proactive command (refer to +STKPRO) <result> 0 Command performed succcessfuly 1 Command performed with partial comprehension 2 Command performed with missing information 3 Refresh performed with additional EFS read 4 Command performed successfully, but requested icon could not be displayed 5 Command performed but modified by call control by SIM 6 Command performed successfully, limited service 7 Command performed with modification 16 Proactive SIM session terminated by the user 17 Backward move in the proactive SIM session requested by the user 18 No response from user 19 Help information required by the user 20 USSD or SS transaction terminated by the user 32 ME currently unable to process command 33 Network currently unable to process the command 34 User did not accept call set-up request 35 User cleared down call before connection or network release 36 Action in contradiction with the current timer state 37 Interaction with call control by SIM, temporary problem 38 Launch browser generic error code 48 Command beyond ME's capabilities 49 Command type not understood by ME 50 Command data not understood by ME 51 Command number not known by ME 52 SS return error 53 SMS RP ERROR 54 Error, required values are missing 55 USSD return error 56 Multiple card command error (if class "a" is supported) 57 Interaction with call control by SIM or MO, short message control by SIM 58 Bearer independent protocol error (if class "e" is supported) <buffer size> Size of the allocated buffer <open_channel_id> 1 – 7 Channel ID 0 Invalid <link_status> Specifies whether link is established or packet data service is activated 1 Enabled 0 Disabled <channel_status_state> Link state 00 No further information can be given <bearer_description_type> Bearer type which can be used to decode the bearer description value 01 Circuit switched UTA_SIM_TK_BEARER 02 Packet switched UTA_SIM_TK_BEARER (GPRS) 03 Terminal default UTA_SIM_TK_BEARER 255 Invalid bearer value; indicates an unknown bearer type which is not supported by the interface version <bearer_description_params> Hexadecimal string; gives detailed information dependent on the bearer type <address_type> Type of address 33 IPv4 IP address 87 IPv6 IP address <address> Address data dependent on bearer type. IPv4 address representation shall follow the format x.x.x.x where 0<x≤255. IPv6 address representation shall follow the format x.x.x.x.x.x.x.x.x.x.x.x.x.x.x.x where 0<x≤255 |

## +STKENV Command: Send a SIM APPL TK Envelope Command

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+STKENV=? | Response +STKENV: OK |
| Write command Syntax AT+STKENV= <envelope_ cmd>, <optional_ENV_ data> | Response OK or +CME ERROR: <err> Parameters <cause> 00 User termination 01 Error termination <envelope_cmd> Code 211 (hex: D3) Menu selection (needs) Code 214 (hex: D6) Event download (note that only one event can be included in the <event_list>) <item_id> Item identification <help_requested> 1 Help is requested 0 Help is not requested <language> Currently used language in the DTE (refer to +STKPROF) <call_id> Call ID <call_direction> 0 MT call 1 MO call <optional_ENV_data> D3 <item_identifier> (for code 211) D6 <event_list> (for code 214) |

## +STKPROF Command: Terminal Profile Data

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+STKPROF=? | Response OK |
| Read command Syntax AT+STKPROF? | Response +STKPROF: <length>,<data> OK |
| Write command Syntax AT+STKPROF= <length>,<data> | Response OK or +CME ERROR: <err> Parameters <length> Integer type; length of characters sent to TE in <data>. When set to “0”, forces a reset to the default terminal profile stored in the ME <data> Terminal profile data in hexadecimal format |

## +STKCC Notification: SIM – APPL – TK Call Control

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Unsolicited Notification | Response +STKCC: <cc_command>... Details of which are as follows: - +STKCC: 1,<res_val>,<alpha>,<number> - +STKCC: 2,<res_val>,<alpha>,<ss_code> - +STKCC: 3,<res_val>,<alpha>,<ussd_code> - +STKCC: 4,<res_val>,<alpha>,<ton_npi>,<sc_addr>,<ton_npi>,<dest_addr> Parameters <cc_command> 1 Set up call 2 Send SS 3 Send USSD 4 Send SM <res_val> Call control result value <alpha> Text string <number> Called party number <ton_npi> Type of number and numbering plan <sc_addr> Service centre address <dest_addr> Destination address |

## +STKCNF Notification: SIM – APPL – TK Proactive Session Status

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Unsolicited Notification | Response +STKCNF: <proactive_cmd>,<result>,<add_result>,<sw1> Parameters <proactive_cmd> Decimal code that indicates the command that was finished (refer to +STKPRO) <result> General result code <add_result> Additional result code <sw1> 0 Command to SIM was suppressed because of multiple terminal response or wrong client. For other responses, refer to GSM 11.11 |

## *PSSTKI Command: SIM Toolkit Configuration

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT*PSSTKI=? | Response *PSSTKI: (list of supported <mode>s) OK |
| Read command Syntax AT*PSSTKI? | Response *PSSTKI: <mode> OK |
| Write command Syntax AT*PSSTKI= <mode> | Response OK Parameter <mode> 0 No unsolicited result code will be sent to the TE; the TE will not send proactive commands to the module. 1 Manual mode. Any unsolicited result codes will be sent to the TE. The TE had to acknowledge with a +STKPRO notification. 2 Auto acknowledge mode. The module answers to STK without the TE. Any unsolicited result codes will be sent to the TE. 3 Auto acknowledge mode without sending unsolicited result codes to the TE. |
| Reference Sierra Wireless Proprietary | Notes - This command cannot be used without a SIM. - <mode> is saved even after the module reboots. - If <mode>=0, the module will automatically restart before the new mode takes effect. - <mode>=2 and <mode>=3 are only possible for a subset of STK proactive commands with user interaction: - Where basic Yes/No responses are expected: - SEND SMS - SEND SS - SEND USSD - SET UP CALL - Where MMI action is need and Yes/No responses are expected when done (for the display part): - SET UP IDLE MODE TEXT - DISPLAY TEXT - PLAY TONE - REFRESH |
| Examples | <SIM card with STK application is inserted> AT*PSSTKI? // read current setting *PSSTKI: 0 OK AT*PSSTKI=? // check supported setting *PSSTKI: (0-3) OK At*psstki=1 // set STK manual mode OK +STKPRO: 33,0,4,"4D6F62696C65204F4B",0 at+stktr=33,0 OK At*psstki=0 // deactivate STK OK +SIM: 1 // module resets +KSUP: 0 +PBREADY <Example: Manual Mode - proactive command SET UP MENU> At*psstki=1 // activate STK manual mode OK // SET UP MENU +STKPRO: 37,0,"GemXplore CASE",1,5,"User interaction",33,0,0 +STKPRO: 37,0,"GemXplore CASE",2,5,"Mobile interaction",33,0,0 +STKPRO: 37,0,"GemXplore CASE",3,5,"Network interaction",33,0,0 +STKPRO: 37,0,"GemXplore CASE",4,5,"Card interaction",33,0,0 +STKPRO: 37,0,"GemXplore CASE",128,5,"Common STK features",33,0,0 at+stktr=37,0 // Terminal Response for SET UP MENU successful OK +STKCNF: 37,0,255,145 // [ACK] SET UP MENU successful, session on-going at+stkenv=211,2,0 // Select menu item #2 +STKCNF: 129, 0, 255, 144 // [ACK] session end OK <Example: Manual Mode - proactive command SELECT ITEM> +STKPRO: 36,0,"Choose an item :",1,5,"Play tone",0,0,0,0 +STKPRO: 36,0,"Choose an item :",2,5,"Provide local info",0,0,0,0 +STKPRO: 36,0,"Choose an item :",3,5,"Refresh",0,0,0,0 +STKPRO: 36,0,"Choose an item :",4,5,"Timer management",0,0,0,0 +STKPRO: 36,0,"Choose an item :",5,5,"Launch browser",0,0,0,0 at+stktr=36,0,0,0,0,"03" // Terminal Response SELECT ITEM #3 OK +STKCNF: 36,0,255,145 // [ACK] SELECT ITEM successful +STKPRO: 36,0,"Choose an item :",1,2,"Init and file change",0,0,0,0 +STKPRO: 36,0,"Choose an item :",2,2,"Reset",0,0,0,0 at+stktr=36,0,0,0,0,"02" // Terminal Response SELECT ITEM #2 OK +STKCNF: 36,0,255,145 // [ACK] SELECT ITEM successful <Example: Manual Mode - proactive command REFRESH> +STKPRO: 01,4,,0,,0 // proactive command: REFRESH - SIM reset at+stktr=01,0 // Terminal Response for REFRESH OK +SIM: 0 // SIM reset +STKCNF: 144, 0 // [ACK] Reset completed +SIM: 1 +STKPRO: 33,0,4,"4D6F62696C65204F4B",0 +PBREADY <Example: Automatic Mode - proactive command REFRESH> at*psstki=2 // set STK automatic mode OK //Proactive command REFRESH is received +STKPRO: 01,4,,0,, 0 // proactive command: REFRESH - SIM reset +SIM: 0 // SIM reset +STKCNF: 144, 0 // [ACK] Reset completed +SIM: 1 +STKPRO: 33,0,4,"4D6F62696C65204F4B",0 +PBREADY <Example: Silent Mode - proactive command REFRESH> At*psstki=3 // set STK silent mode OK +SIM: 0 // SIM reset +SIM: 1 +PBREADY <SIM card is not inserted> at+cpin? +CME ERROR: 10 AT*PSSTKI? // read current setting +CME ERROR: 10 AT*PSSTKI=? // check supported setting +CME ERROR: 10 AT*PSSTKI=1 // deactivate STK +CME ERROR: 10 |

# Protocol Specific Commands

## Preliminary Comments

Sierra Wireless has developed a set of proprietary AT Commands to simplify data exchanges with different protocols:

TCP

UDP

FTP

HTTP

HTTPS

## IP Address Format in AT Commands

Unless specified elsewhere, the following format is used for IP address field in AT commands described in this chapter when using the HL76xx embedded module:

IPv4 address: Consists of dot-separated decimal (0 – 255) parameters of the form a1.a2.a3.a4

IPv6 address: Consists of colon-separated hexadecimal (0 – ffff) parameters of the form a1:a2:a3:a4:a5:a6:a7:a8 with abbreviations

## Session ID

Protocol specific AT commands share the same range of session IDs. A session ID <session_id> is a unique number and ranges from 1 to 32.

## Connection of PDP Contexts

A PDP connection will be started when a session becomes active (e.g. +KTCPCNX) and will only be stopped if all sessions are closed or all sessions request to stop the connection. In case of session errors, the PDP connection deactivation behavior can be configured by +KIPOPT with <option_id>=3. The default setting after the module boot-up is that a PDP connection is requested to stop only when a session is closed by an Internet AT command (e.g. +KTCPCLOSE).

## Buffer Length of AT Commands

In AT command mode, the maximum length of an AT command is 1023 characters; any AT command input longer than this limit will produce an error response. If the maximum length of a parameter is not specified in this manual, it may vary but still bound by this limit.

In AT data mode, the terminal receive buffer size is limited to 32000 bytes; the terminal driver will stop the receive flow at 16000 bytes if hardware handshaking is used.

## Parameter Format of AT Commands

Double quotation marks are optional in the parameter input of protocol specific AT commands.

If the AT command does not meet the following conditions, the AT parser will regard it as an error and will not go to the corresponding AT command handler. It will immediately return +CME ERROR: 3. This means that it will not process any action further or return any specific error code.

If double quotation marks are used to enclose parameters, double quotation marks must appear at both the head and tail of the parameter.

The total number of parameter input (including empty parameters) in the AT commands must be within the minimum and maximum required number of parameters.

## Connection Configuration

### +KCNXCFG Command: GPRS Connection Configuration

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KCNXCFG=? | Response +KCNXCFG: (list of possible <cnx conf>s),"GPRS",(range of possible length of <apn>),(range of possible length of <login>),(range of possible length of <password>), <af>,<ip>,<dns1>,<dns2>,<ipv6>,<dns1v6>,<dns2v6> OK |
| Read command Syntax AT+KCNXCFG? | Response +KCNXCFG: <cnx cnf>, "GPRS", <apn>,<login>,<password>,<af>,<ip>,<dns1>, <dns2>[,<ip_v6>,<dns1_v6>,<dns2_v6>],<state> [...]> OK |
| Write command Syntax AT+KCNXCFG= <cnx cnf>, "GPRS",<apn> [,[<login>] [,[<password>] [,<af> [,[<ip>] [,[<dns1>] [,<dns2>]]]] [,[<ip_v6>] [,[<dns1_v6>] [,<dns2_v6>]]]]]] | Response OK Parameters <cnx cnf> 1 – 5 (PDP context configuration) a numeric parameter which specifies a particular PDP context configuration <apn> (Access Point Name) a string parameter (max size 63 bytes), logical name used to select the GGSN or the external packet data network. <login> string type (max size 24 bytes), indicates the user name of the cnx <password> string type (max size 24 bytes), indicates the password of the cnx <af> Address family used for the connection IPV4 IPv4 only IPV6 IPv6 only IPV4V6 IPv4 and IPv6 <ip> String type. If the mobile is supposed to work with a dynamic address, the value should be “0.0.0.0” or an empty string. <dns1>, <dns2> String type. If the mobile is supposed to work with dynamic DNS addresses, the value should be “0.0.0.0” or an empty string. <ip_v6> IPV6 String type. If the mobile is supposed to work with a dynamic address, the value should be “::” or an empty string. <dns1_v6>, <dns2_v6> IPV6 String type. If the mobile is supposed to work with dynamic DNS addresses, the value should be “::” or an empty string. <state> Connection state 0 Disconnected 1 Connecting 2 Connected 3 Idle, down counting for disconnection 4 Disconnecting |
| Reference Sierra Wireless Proprietary | Notes - This AT command is used to configure the bearer to be used for the future IP services. - By default, the IP and DNS address are dynamic (those values would be affected by the network during the PDP connection). - This connection will be used by the module to access to the IP services described in subsequent chapters. The AT+KCNXCFG command is only defined to set the current parameters. The defined connection will be automatically opened when needed by the IP services (e.g. UDP service). - The use of IPV4 and/or IPV6 addresses is configured by PDP context configuration. - <cnx cfg> values 1 to 5 correspond to PDP context ID 1 to 5 respectively, e.g. <cnx cfg>=3 corresponds to CID=3 in +CGDCONT/+CGACT. - When the connection is up, the read command returns the actual values used by the connection interface. - If reuse of existing activated PDP context is required, <apn> can be set as an empty string or as the existing APN string returned by +CGDCONT read command. |

### +KCNXTIMER Command: Connection Timer Configuration

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KCNXTIMER =? | Response +KCNXTIMER: (list of supported <cnx cnf>s),(list of supported <tim1>s),(list of supported <nbtrial>s),(list of supported <tim2>s) ,(list of supported <idletime>s) OK |
| Read command Syntax AT+KCNXTIMER? | Response +KCNXTIMER: <cnx cnf>,<tim1>,<nbtrial>,<tim2>,<idletime> […] OK |
| Write command Syntax AT+KCNXTIMER=<cnx cnf>[, [<tim1>][, [<nbrtrial>] [,<tim2>] [,<idletime>]]]] | Response OK Parameters <cnx cnf> 1 – 5 (PDP context configuration) a numeric parameter which specifies a particular PDP context configuration <tim1> 1 – 120 s (30 s by default) If the module fails to activate the PDP context, a timer of <tim1> will be started. When this timer expires, it will try to activate the PDP context again. <nbtrial> Attempt times from1 – 4 (2 by default). The module will try to activate the PDP context for a maximum of <nbtrial> times. <tim2> 0 – 300s 0 Deactivated (connection will not close by itself) 60 Default value For client sockets, module will try to connect to the server within <tim2>s; if <tim2> expires, it will give up the connection. <idletime> 0 – 1800 s (30 s by default) When all sessions are closed, the idle timer starts with the idle time. When this timer expires, it will try to deactivate the PDP context. Before the timer expires, connecting any session will stop this timer and the PDP context is reused. |
| Reference Sierra Wireless Proprietary | Notes This command will only have impact on TCP, UDP, FTP, HTTP and HTTP specific commands (+KTCPCNX, +KTCPSTART, +KUDPCFG, +KFTPCFG, etc.) |

### +KCNXPROFILE Command: Current Profile Connection Configuration

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ KCNXPROFILE =? | Response +KCNXPROFILE: (list of possible <cnx cnf>s) OK |
| Read command Syntax AT+ KCNXPROFILE? | Response +KCNXPROFILE: <cnx cnf> OK |
| Write command Syntax AT+ KCNXPROFILE= <cnx cnf> | Response OK Parameters <cnx cnf> 1 – 5 PDP context configuration – a numeric parameter which specifies a particular PDP context configuration |
| Reference Sierra Wireless Proprietary | Notes This command sets the default PDP context configuration ID for +KTCPCFG, +KUDPCFG, +KFTPCFG, +KHTTPCFG and +KHTTPSCFG, if <cnx cnf> parameter is not given in these commands. |

### +KCGPADDR Command: Display PDP Address

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KCGPADDR =? | Response +KCGPADDR: (list of possible <cnx_cnf>s) OK |
| Write command Syntax For all <cnx_cnf>s: AT+KCGPADDR For specific <cnx_cnf>s: AT+KCGPADDR=<cnx_cnf> | Response +KCGPADDR: <cnx cnf>, <PDP_addr_1> [[+KCGPADDR: <cnx cnf>, <PDP_addr_2>] …] OK Parameters <cnx cnf> 1 – 5 PDP context configuration – a numeric parameter which specifies a particular PDP context configuration <PDP_addr> A string that identifies the MT in the address space applicable to the PDP |
| Reference Sierra Wireless Proprietary | Notes - This AT command can be used after +KTCPCNX, +KUDPCFG, etc. to display the local IP address of the module - For IPv6, more than one PDP addresses corresponding to the interface may be displayed. |

### +KCNX_IND Notification: Connection Status Notification

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Unsolicited Notification | Response +KCNX_IND: <cnx cnf>,<status>,<af> (for <status> = 0, 1) +KCNX_IND: <cnx cnf>,<status>,<attempt>,<nbtrial>,<tim1> (for <status> = 2) +KCNX_IND: <cnx cnf>,<status> (for <status> = 3,6) +KCNX_IND: <cnx cnf>,<status>,<attempt> (for <status> = 4) +KCNX_IND: <cnx cnf>,<status>,<idletime> (for <status> = 5) Parameters <cnx cnf> 1 – 5 (PDP context configuration) a numeric parameter which specifies a particular PDP context configuration <status> PDP connection status 0 Disconnected due to network 1 Connected 2 Failed to connect, <tim1> timer is started if <attempt> is less than <nbtrail> 3 Closed 4 Connecting 5 Idle time down counting started for disconnection 6 Idle time down counting canceled <af> 0 IPV4 1 IPV6 <tim1> Refer to +KCNXTIMER <attempt> Current attempt of bringing up of PDP connection <nbtrial> Refer to +KCNXTIMER <idletime> Refer to +KCNXTIMER |
| Reference Sierra Wireless Proprietary |  |

### +KCNXUP Command: Bring the PDP Connection Up

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KCNXUP=? | Response +KCNXUP: (list of possible <cnx_cnf>s) OK |
| Write command Syntax AT+KCNXUP= <cnx_cnf> | Response OK Parameter <cnx cnf> 1 – 5 PDP context configuration – a numeric parameter which specifies a particular PDP context configuration |
| Reference Sierra Wireless Proprietary | Notes - This command activates the PDP context and reserves the activated PDP connection (i.e. keeps the PDP connection up even after the last session is closed). - If this command is not used, the PDP context will be brought down after the last session is closed unless +KCNXDOWN is used. |

### +KCNXDOWN Command: Bring the PDP Connection Down

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KCNXDOWN=? | Response +KCNXDOWN: (list of possible <cnx_cnf>s),(list of possible <mode>s) OK |
| Write command Syntax AT+KCNXDOWN=<cnx_cnf> [,<mode>] | Response OK Parameters <cnx cnf> 1 – 5 PDP context configuration – a numeric parameter which specifies a particular PDP context configuration <mode> 0 Cancels the reservation of the activated PDP connection previously configured by +KCNXUP 1 Similar to 0, but deactivates the PDP connection even if the active session exists |
| Reference Sierra Wireless Proprietary |  |

## Common Configuration

### +KPATTERN Command: Custom End of Data Pattern

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KPATTERN =? | Response OK |
| Read command Syntax AT+KPATTERN? | Response +KPATTERN: <EOF pattern> OK |
| Write command Syntax AT+KPATTERN =<EOF pattern> | Response OK or +CME ERROR <err> Parameter <EOF pattern> String type (max size 128 bytes). This is a pattern used to notify the end of data (or file) during data or file transfer. This string doesn’t have to be human-readable (not printable characters are allowed). |
| Reference Sierra Wireless Proprietary | Notes - The default value of the pattern is: "--EOF--Pattern--". - It is the responsibility of the user to select an appropriate pattern according to the data transferred (i.e. numeric pattern for text files and Readable string for binary files). - The <EOF pattern> pattern is detected within 100ms or higher timeout and without following data. The timeout value is equal to <wait_time> of +KIPOPT. - The received data is stored with buffer size <send size v4> or <send size v6> so that the <EOF pattern> with size larger than it is not detected. The user application should ensure that the value of <send size v4> or <send size v6> is larger than the size of <EOF pattern>. |

### +KURCCFG Command: Enable or Disable the URC from Protocol Commands

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KURCCFG=? | Response +KURCCFG: (list of supported <protoopt>s),(list of supported <noti_act>s),(list of supported <indi_act>s) OK |
| Read command Syntax AT+KURCCFG? | Response +KURCCFG: list of supported (<protoopt>,<noti_act>,<indi_act>) OK |
| Write command Syntax AT+KURCCFG= <protoopt>, <noti_act> [,<indi_act>] | Response OK Parameters <protoopt> Protocol option to enable/disable URC "TCPC" TCP client session "TCPS" TCP server session "UDPC" UDP client session "UDPS" UDP server session "FTP" FTP client session "HTTP" HTTP client session "HTTPS" HTTPS client session "TCP" Both TCP client and TCP server sessions "UDP" Both UDP client and UDP server sessions <noti_act> 1 Enable URC (like +KTCP_NOTIF, +KFTP_ERROR, etc.) 0 Disable URC <indi_act> 1 Enable URC (like +KTCP_SRVREQ, +KTCP_IND, +KTCP_DATA, +KUDP_DATA, +KUDP_RCV, +KFTP_IND, etc.) 0 Disable URC |
| Examples | To disable URC: AT+KURCCFG=“TCP”,0 OK Test and read command: AT+KURCCFG=? +KURCCFG: ("TCPC","TCPS","UDPC","UDPS","FTP","HTTP","HTTPS","TCP","UDP"), (0,-1), (0-1) OK AT+KURCCFG? +KURCCFG: "TCPC",1,1 +KURCCFG: "TCPS",1,1 +KURCCFG: "UDPC",1,1 +KURCCFG: "UDPS",1,1 +KURCCFG: "FTP",1,1 +KURCCFG: "HTTP",1,1 +KURCCFG: "HTTPS",1,1 OK |
| Reference Sierra Wireless Proprietary | Notes - Enabling or disabling +KTCP_NOTIF unsolicited messages is only useful when in polling mode with +KTCPSTAT. - If set to "disable", URCs are discarded and not stored. - Can be used in 07.10 multiplexer. |

### +KIPOPT Command: General Options Configuration

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KIPOPT=? | Response +KIPOPT: 0,<UDP>,(1-100),(8-1472),(8-1452) +KIPOPT: 0,<TCP-based>,(0-100),(0,8-1460),(0,8-1440) +KIPOPT: 1,(0-1) +KIPOPT: 2,(0-255) +KIPOPT: 3,(0-1),(0-1) +KIPOPT: 4,(0-2) OK |
| Read command Syntax AT+KIPOPT? | Response +KIPOPT: 0,<proto>,<wait time>,<send size v4>,<send size v6>] [...] +KIPOPT: 1,<http_chunked> +KIPOPT: 2,<http_max_redirect> +KIPOPT: 3,<stop_on_error>, <stop_on_peer> +KIPOPT: 4,<ssl_ver> OK |
| Write command Syntax If <option_id>=0: AT+KIPOPT= <option_id>, <proto>,<wait time> [,<send size v4> [,<send size v6>]] If <option_id>=1: AT+KIPOPT= <option_id>, <http_chunked> If <option_id>=2: AT+KIPOPT= <option_id>, <http_max_ redirect> If <option_id>=3: AT+KIPOPT= <option_id>, <stop_on_error>, <stop_on_peer> If<option_id>=4: AT+KIPOPT= <option_id>, <ssl_ver> | Response OK or +CME ERROR<err> Parameters <option_id> Option ID 0 Wait time, send size threshold configuration 1 HTTP chunked transfer encoding 2 HTTP maximum redirection 3 PDP connection deactivated behavior 4 SSL version for use in +KHTTPS <proto> Protocol, string type “TCPC” TCP client session “TCPS” TCP server session “UDPC” UDP client session “UDPS” UDP server session “FTP” FTP client session “HTTP” HTTP client session “HTTPS” HTTP server session “TCP” Both client and server TCP sessions “UDP” Both client and server UDP sessions <wait time> Timeout for configuring the packet segmentation on the IP network side; it specifies the timeout after which the buffered data will be sent to the peer irrespective of data packet size. Value is in 100 ms units. Range: For UDP: 1 – 100, default value = 2 For TCP: 0 – 100, default value = 1. Note that value = 0 has the same effect as having value = 1 due to the limitation from +KPATTERN detection timing <send size v4> Data packet size for IPv4 sessions. This parameter specifies the data packet size that needs to be sent to the peer. Range: For UDP: 8 – 1472, default value = 1020 For TCP: 0, 8 – 1460, default value = 0 (disabled) <send size v6> Data packet size for IPv6 sessions. This parameter specifies the data packet size that needs to be sent to the peer. Range: For UDP: 8 – 1452, default value = 1020 For TCP: 0, 8 – 1440, default value = 0 (disabled). Note that value = 0 uses a wait time of 100 ms. <http_chunked> “Chunked” transfer encoding for HTTP POST 0 Data sent with HTTP POST are not encoded 1 Data sent with HTTP POST are automatically encoded using “chenked” transfer encoding <http_max_redirect> Maximum redirection allowed for HTTP GET. Range: 8 – 255; default value = 0 <stop_on_error> PDP connection deactivation behavior when a session is closed due to any error 0 Do not request to stop the connection 1 Request to stop the connection <stop_on_peer> PDP connection deactivation behavior when a session is closed by a peer/server 0 Do not request to stop the connection 1 Request to stop the connection <ssl_ver> SSL version for use in +KHTTPS 0 TLS version 1.1 1 TLS version 1.0 2 TLS version 1.2 |
| Reference Sierra Wireless Proprietary | Notes - "chunked" transfer encoding for HTTP POST is applicable and effective only for HTTP version 1.1. - The default setting of <option_id>=3 is (<stop_on_error>=0, <stop_on_peer>=0) after module boot-up; this means that a PDP connection is requested to stop only when a session is closed by an Internet AT command (e.g. +KTCPCLOSE). - Thresholds <send size v4> and <send size v6> control the maximum size of data received from the AT terminal to be buffered within timeout <wait time>. When the threshold is reached or after timeout, the buffered data are sent to the socket layer for transmission. For UDP: data are sent as a UDP packet For TCP based protocol: data are copied to socket first-in-first-out buffer for transmission but packet segmentation is not guaranteed to be <send size> - For TCP based protocol, when <send size v4> and <send size v6> are disabled (=0), threshold = 4000 is used internally. - The maximum transmission unit (MTU) is 1500 bytes. - After starting a connection or running SSL Certificate write commands, <ssl_ver> is fixed and cannot be changed until module reboot. - <send size v4> and <send size v6> impacts the detection of <EOF pattern>. Refer to the notes of +KAPTTERN for more information. |

## SSL Configuration

### +KSSLCRYPTO Command: Cipher Suite Configuration

For HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ KSSLCRYPTO=? | Response +KSSLCRYPTO: <profile_id>,<mkey_Algo>,<auth_algo>,<enc_algo>,<mac_algo>, <tls_ver>,<auth>,<tls_ver>,<auth> OK |
| Read command Syntax AT+ KSSLCRYPTO? | Response + KSSLCRYPTO: <profile_id>,<mkey_algo>,<auth_algo>,<enc_algo>,<mac_algo>, <tls_ver>,<auth> […]] |
| Write command Syntax AT+ KSSLCRYPTO= <profile_id>, <mkey_Algo>, <auth_algo>, <enc_algo>, <mac_algo>, <tls_ver>,<auth> | Response OK Parameters <profile_id> Index of a set of parameters for configuring one SSL profile <mkey_algo> Key exchange algorithm selection 1 RSA key exchange <auth_algo> Authentication algorithm selection 1 RSA authentication <enc_algo> Encryption algorithm selection 4 RC4 64 AES 128 128 AES 256 8192 AES128GCM <mac_algo> Message authentication code algorithm selection 1 MD5 2 SHA1 64 AEAD <tls_ver> Cipher suite version selection. 1 TLS 1.0 4 TLS 1.2 <auth> Authentication 0 No authentication 1 Authenticate server 2 Provide client certificate to server 3 Authenticate server and provide client certificate to server |
| Reference Sierra Wireless Proprietary |  |

### +KSSLCFG Command: SSL Configuration

For HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KSSLCFG=? | Response +KSSLCFG:<option id>,<option> OK |
| Read command Syntax AT+KSSLCFG? | Response +KSSLCFG:0,<TLS Version> +KSSLCFG:2,<Session Mode> OK |
| Write command Syntax AT+KSSLCFG =<option id>, <option> | Response If <option_id> = 0: AT+KSSLCFG=<option_id>,<TLS Version> OK If <option_id> = 1: AT+KSSLCFG=<option_id>,<Random Seed> OK If <option_id> = 2: AT+KSSLCFG=<option_id>,<Session Mode> OK Parameters <option id> 0 Specify a TLS version to be used for hand shake 1 Setup random seed 2 Specify session mode <TLS Version> 0 Highest possible 1 TLS 1.0 3 TLS 1.2 <Random Seed> String to be added into the entropy of the random number generator <Session Mode> 0 Automatic 1 Always start a new session (not supported) |

## TCP Specific Commands

### +KTCPCFG Command: TCP Connection Configuration

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |  |  |
| --- | --- | --- | --- |
| Test command Syntax AT+KTCPCFG=? |  | Response +KTCPCFG: (list of possible <cnx_cnf>s),(list of possible <mode>s), <remote-name/ip>,(list of possible <tcp_port >s),(list of possible <source_port>s),(list of possible <data_mode>s),(list of possible <URC-ENDTCP-enable>s),(list of possible <af>s),<cipher_index> OK |  |
| Read command Syntax AT+KTCPCFG? |  | Response +KTCPCFG: <session_id>,<status>,<cnx cnf>,<mode>[,<serverID>], <tcp remote address>,<tcp_port>[,<source_port>],<data_mode>, <URC-ENDTCP-enable>,<af>,<cipher_index> […]] |  |
| Write command Syntax AT+KTCPCFG= [<cnx cnf>], <mode>, [<tcp remote address>], <tcp_port>[[, [<source_port>] [,[<data_mode>], [<URC-ENDTCP-enable>]]],<af>] [,<cipher_suite>] |  | Response +KTCPCFG: <session_id> OK Parameters <cnx cnf> Index of a set of parameters for configuring one TCP session (see +KCNXCFG) <session_id> TCP session index <mode> 0 Client 1 Server 2 Child (generated by server sockets) 3 Secure client <tcp remote address> IP address string or explicit name of the remote server. For server configuration, this parameter is left blank <tcp_port> TCP port number; numeric parameter with range 1 – 65535. This parameter is the listening port for a server configuration. <status> Connection state of the selected socket 0 Disconnected 1 Connected <serverID> Server session ID index. Only for sockets in CHILD mode <source_port> Numeric parameter (0-65535). Specifies the local TCP port number. This parameter is left blank for a server configuration. <data_mode> 0 Do not display <data> in URC (default setting) 1 Display <data> in URC <URC-ENDTCP-enable> 0 Do not display URC "+KTCP_ACK" (default setting) 1 Display URC "+KTCP_ACK" <af> Address family used for the connection. 0 IPV4 1 IPV6 <cipher_index> Cipher suite profile index to use for a secured socket; defined by +KSSLCRYPTO |  |
| Reference Sierra Wireless Proprietary | Notes - If the socket is defined as a <CLIENT> socket, <tcp_port> and <tcp remote address> define the port and the IP address of the remote server we want to connect. - Maximum <session_id> is 32. - For child session, the property <data_mode> will be kept the same as the server socket’s setting. - See section 18.5.6 Use Cases for AT+KTCPACKINFO and <URC-ENDTCP-enable> Option. - This command can be used before setting up +KCNXCFG configuration. Note however that the latter is required to start the connection properly. - The connection timeout for TCP socket is about 9 seconds with 3 retransmissions with 3 seconds delay. |

### +KTCPCNX Command: Start TCP Connection

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KTCPCNX=? | Response +KTCPCNX: (list of possible <session_id>s) OK |
| Write command Syntax AT+KTCPCNX= <session_id> | Response OK or +CME ERROR: <err> +KTCP_NOTIF: <session_id>, <tcp_notif> Parameters <session_id> TCP session index <tcp_notif> Integer type. Indicates the cause of the TCP connection failure 0 Network error 1 No more sockets available; max. number already reached 2 Memory problem 3 DNS error 4 TCP disconnection by the server or remote client 5 TCP connection error 6 Generic error 7 Fail to accept client request’s 8 Data sending is OK but KTCPSND was waiting more or less characters 9 Bad session ID 10 Session is already running 11 All sessions are used 12 Socket connection timeout error 13 SSL connection error 14 SSL initialization error |
| Reference Sierra Wireless Proprietary | Notes This command is used for connecting to a remote server or listening to a bound port, depending on the selected mode of <session_id.> |

### +KTCPRCV Command: Receive Data through a TCP Connection

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KTCPRCV=? | Response +KTCPRCV: (list of possible <session_id>s),(list of possible <ndata>s) OK |
| Write command Syntax AT+KTCPRCV= <session_id>, <ndata> | Response CONNECT …<EOF pattern> OK or +KTCP_NOTIF: <session_id>,<tcp_notif> Parameters <session_id> TCP session index <ndata> Number of bytes the device wants to receive (max value 4294967295) <tcp_notif> See command AT+KTCPCNX |
| Reference Sierra Wireless Proprietary | Notes - This function is used to receive <ndata> data bytes through a previously opened TCP socket. - <ndata> indicates the max data number that the terminal wishes to receive. If the TCP socket contains more data than <ndata> bytes then only <ndata> bytes will be received. If the TCP socket contains less data than <ndata> bytes then only TCP socket’s data will be received. - <EOF pattern> would be added at the end of data automatically. - When <ndata> (max value) bytes or only available data in the TCP socket have been received, the module returns to command state and returns OK. - It is highly recommended to configure the module for hardware flow control using AT&K3 before using this command. - Refer to AT&D for the behavior of DTR drop. |

### +KTCPSND Command: Send Data through a TCP Connection

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KTCPSND=? | Response +KTCPSND: (list of possible <session_id>s),(list of possible <ndata>s) OK |
| Write command Syntax AT+KTCPSND= <session_id>, <ndata> | Response CONNECT OK or NO CARRIER +CME ERROR: <err> +KTCP_NOTIF: <session_id>,<tcp_notif> Parameters <session_id> TCP session index <ndata> Number of bytes (max value 4294967295) <tcp_notif> See command AT+KTCPCNX |
| Reference Sierra Wireless Proprietary | Notes - All the data will be sent out ignoring <ndata>. If data sent is not equal to <ndata> then +KTCP_NOTIF will be displayed. - <ndata> is the data size without <EOF pattern>. - It is highly recommended to configure the module for hardware flow control using AT&K3 before using this command. - Refer to AT&D for the behavior of DTR drop. - The data session is automatically ended when <ndata> data bytes are sent/received, and the module returns to command state and returns OK. - The data session can also be ended by <EOF pattern>, +++ or DTR. Refer to section 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table for the behavior of +++, DTR or ATO. |

### +KTCPCLOSE Command: Close Current TCP Operation

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KTCPCLOSE=? | Response +KTCPCLOSE: (list of possible <session_id>s), (list of possible <closing_type>s) OK |
| Write command Syntax AT+KTCPCLOSE=<session_id> [,<closing_type>] | Response OK or +CME ERROR: <err> NO CARRIER +KTCP_NOTIF: <session_id>, <tcp_notif> Parameters <session_id> TCP session index <closing_type> 0 Abort. Fast closing of the TCP connection (not supported). 1 The TCP connection is properly closed, which means that data sent to the module by AT+KTCPSND will be sent to the TCP server and acknowledged before the socket is closed. <tcp_notif> See command AT+KTCPCNX |
| Reference Sierra Wireless Proprietary | Notes - This function first closes the TCP socket and if there is no other session running then the PDP context is released. - AT+KTCPDEL=<session_id> can be used to delete the socket configuration after it’s been closed. |

### +KTCPDEL Command: Delete a Configured TCP Session

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KTCPDEL=? | Response +KTCPDEL: (list of possible <session_id>s) OK |
| Write command Syntax AT+KTCPDEL= <session_id> | Response OK or +CME ERROR: <err> Parameter <session_id> TCP session index |
| Reference Sierra Wireless Proprietary | Notes The session must be closed (using +KTCPCLOSE) before using this command. |

### +KTCP_SRVREQ Notification: Incoming Client Connection Request

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Unsolicited Notification | Response +KTCP_SRVREQ: <session_id>,<subsession_id>,<client_ip>,<client_port> Parameters <session_id> TCP session index <subsession_id> Newly created TCP session index <client_ip> IP address string of the incoming socket <client_port> Numeric parameter (0-65535); port of the incoming client |
| Examples | //Configure the module to TCP servers AT+KCNXCFG=0,"GPRS","szsjmc.gd"; +KTCPCFG=0,1,,179 +KTCPCFG: 1 OK AT+KCNXCFG=0,"GPRS","szsjmc.gd"; +KTCPCFG=0,1,,180 +KTCPCFG: 2 OK //Start the TCP servers AT+KTCPCNX=1 //listen on port 179 OK AT+KTCPCNX=2 //listen on port 180 OK //Show the TCP servers’ IP address AT+KCGPADDR +KCGPADDR: 0,"192.168.1.49" OK //Incoming connection request from remote client, shows ip address and port of remote //client +KTCP_SRVREQ: 1,3,"192.168.0.32",4614 //incoming a connection request from “192.168.0.32” via //listening port 179, the remote //port is 4614 +KTCP_SRVREQ: 2,4,"10.10.10.110",4665 //incoming a connection request from “10.10.10.110” via //listening port 180, the remote //port is 4665 +KTCP_SRVREQ: 2,5,"10.10.10.110",4668 //incoming a connection request from the same ip via the same //listening port, the //remote port is 4668 +KTCP_SRVREQ: 1,6,"192.168.1.117",1739 //incoming a connection request from “192.168.1.117” via //listening port 179, the remote //port is 1739 +KTCP_NOTIF: 4,4 //the connection of sub session id 4 (on listening port 180) is //closed. +KTCP_SRVREQ: 2,4,"10.10.10.8",4672 //incoming a connection request from “10.10.10.8” via listening //port 180, the remote //port is 4672 |
| Reference Sierra Wireless Proprietary | Notes - This notification is sent when a client requests a connection to the server. The connection is automatically accepted. - The created session is driven as any other TCP session with its own session ID. Use +KTCPSND, +KTCPRCV, +KTCPCLOSE, etc. to provide the service associated to this TCP server. - The TCP server corresponding to the session ID is still able to receive connection requests from other clients. These requests are notified with +KTCP_SRVREQ. - The client IP address and port can also be checked using AT+KTCPCFG? after the client is connected to the TCP server. |

### +KTCP_DATA Notification: Incoming Data through a TCP Connection

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Unsolicited Notification | Response +KTCP_DATA: <session_id>,<ndata available>[,<data>] Parameters <session_id> TCP session index <ndata available> For <data_mode> = 0, maximum number of bytes to be read in the TCP receive buffer; for <data_mode> = 1, maximum number of bytes to be read in <data> <data> Data in octet. The length of data is specified by <ndata_available> |
| Reference Sierra Wireless Proprietary | Notes - As soon as the connection is established, the module can receive data through the TCP socket. This notification is sent when data are available in the receive buffer. - This notification is sent for each TCP packet received. - When <data_mode> is set to 1, <ndata_available> will range from 1 to 1500 in the URC. If the user application sends over 1500 bytes of data to the module, the module will display those data with several URCs. - See section 18.6.3 Use Cases for KTCP_DATA and KUDP_DATA. |

### +KTCP_IND Notification: TCP Status

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Unsolicited Notification | Response +KTCP_IND: <session_id>,<status> Parameters <session_id> TCP session index <status> TCP session status. 1 session is set up and ready for operation |
| Reference Sierra Wireless Proprietary |  |

### +KTCPSTAT Command: Get TCP Socket Status

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KTCPSTAT=? | Response OK |
| Read command Syntax AT+KTCPSTAT? | Response OK |
| Write command Syntax For all TCP session IDs: AT+KTCPSTAT or AT+KTCPSTAT= <session_id> | Response +KTCPSTAT: <session_id>,<status>,<tcp_notif>,<rem_data>,<rcv_data> […] OK or +KTCPSTAT: <status>,<tcp_notif>,<rem_data>,<rcv_data> OK Parameters <session_id> TCP session index <status> TCP socket state 0 Socket not defined, use +KTCPCFG to create a TCP socket 1 Socket is only defined but not used 2 Socket is opening and connecting to the server, cannot be used 3 Connection is up, socket can be used to send/receive data 4 Connection is closing, it cannot be used, wait for status 5 5 Socket is closed <tcp_notif> -1 if socket/connection is OK, <tcp_notif> if an error has happened <rem_data> Remaining bytes in the socket buffer, waiting to be sent <rcv_data> Received bytes, can be read with +KTCPRCV command |
| Reference Sierra Wireless Proprietary | Notes - The socket buffer’s size for sending is 17520 bytes. - This command returns +CME ERROR: 910 (Bad Session ID) for undefined <session_id>s. |

### +KTCPSTART Command: Start a TCP Connection in Direct Data Flow

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KTCPSTART=? | Response OK |
| Read command Syntax AT+KTCPSTART? | Response OK |
| Write command Syntax AT+KTCPSTART=<session_id> | Response CONNECT OK or +CME ERROR: an error occurs, syntax error +KTCP_NOTIF: <session_id>,<tcp_notif> : an error occurs Parameters <session_id> TCP session index <tcp_notif> See command AT+KTCPCNX |
| Reference Sierra Wireless Proprietary | Notes - This function is used to send and receive data bytes through a TCP socket. - It is highly recommended to configure the module for hardware flow control using AT&K3 before using this command. - Refer to AT&D for the behavior of DTR drop. - Only one KTCPSTART session can be used. - Can be used in 07.10 multiplexer. - If the session is successfully connected by +KTCPCNX, this command does not restart the connection and the module directly enters direct data flow. - The data session can also be ended by <EOF pattern>, +++ or DTR. Refer to section 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table for the behavior of +++, DTR or ATO. |

### +KTCP_ACK Notification: Status Report for Latest TCP Data

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Unsolicited Notification | Response +KTCP_ACK: <session_id>,<result> <CR><LF> Parameters <session_id> TCP session index <result> 0 Data sent failure: not all data has been received by remote side 1 Data sent success: all the data has already been received by the remote side |
| Reference Sierra Wireless Proprietary | Notes - This URC is enabled or disabled by parameter <URC-ENDTCP-enable> of command +KTCPCFG. The URC is disabled by default. - See section 18.5.6 Use Cases for AT+KTCPACKINFO and <URC-ENDTCP-enable> Option. |

### +KTCPACKINFO Command: Poll ACK Status for the Latest Data

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ KTCPACKINFO =? | Response OK |
| Read command Syntax AT+ KTCPACKINFO? | Response OK |
| Write command Syntax For all TCP session IDs with <URC-ENDTCP-enable>=1: AT+ KTCPACKINFO or AT+ KTCPACKINFO= <session_id> | Response +KTCPACKINFO: <session_id>,<result> […] OK or +KTCPACKINFO: <session_id>,<result> OK or +CME ERROR: <err> Parameters <session_id> TCP session index <result> 0 Data sent failure: not all data has been received by remote side. 1 Data sent success: all the data has already been received by the remote side; or no data transfer has happened yet 2 The status is unknown yet |
| Reference Sierra Wireless Proprietary | Notes - The command will return ERROR if <URC-ENDTCP-enable> of command +KTCPCFG is 0. - After the TCP session is connected and before any data transfer, +KTCPACKINFO returns 1. |

## UDP Specific Commands

### +KUDPCFG Command: UDP Connection Configuration

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KUDPCFG=? | Response +KUDPCFG: (list of possible <cnx cnf>s),(list of possible <mode>s),(list of possible <port>s),(list of possible <data_mode>s),<remote-name/ip>,(list of possible <udp_port>s),(list of possible <af>s) OK |
| Read command Syntax AT+KUDPCFG? | Response +KUDPCFG: <session_id>,<cnx cnf>,<mode>,<port>,<data_mode>,<udp remote address>,<udp_port>,<af> […] OK |
| Write command Syntax AT+KUDPCFG= [<cnx cnf>], <mode>[,[<port>] [,<data_mode>], [<udp remote address>] , <udp_port>,<af>] | Response +KUDPCFG: <session_id> OK or +CME ERROR: <err> +KUDP_NOTIF: <session_id>, <udp_notif> Parameters <session_id> UDP session index <mode> 0 Client 1 Server <port> 0 – 65535 Port (0 = random) <cnx cnf> 1 – 5 (PDP context configuration) a numeric parameter which specifies a particular PDP context configuration (see section 12.7.1 +KCNXCFG Command: GPRS Connection Configuration). <udp_notif> Integer type. Indicates the cause of the UDP connection failure. 0 Network error 1 No more sockets available; max number already reached 2 Memory problem 3 DNS error 5 UDP connection error (Host unreachable) 6 Generic error 8 Data sending is OK but KUDPSND was waiting more or less characters 9 Bad session ID 10 Session is already running 11 All sessions are used <data_mode> 0 Do not display <data> in URC (Default setting) 1 Display <data> in URC <udp remote address> IP address string or explicit name of the remote host, Default is empty (given by +KUDPSND). <udp_port> 0 – 65535 UDP peer port; given by +KUDPSND <af> Address family used for the connection. 0 IPV4 1 IPV6 |
| Reference Sierra Wireless Proprietary | Notes - For UDP socket in server mode, it is bound to a defined port number, incoming connection are notified by +KUDP_DATA. If remote address and port are given, they are saved for use in +KUDPSND. - Maximum <session_id> is 32. - +KCNXCFG configuration should be set up in order to start the connection properly. |

### +KUDPRCV Command: Receive Data through a UDP Connection

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KUDPRCV=? | Response +KUDPRCV: (list of possible <session_id>s),(list of possible <ndata>s) OK |
| Write command Syntax AT+KUDPRCV= <session_id>, <ndata> | Response CONNECT …<EOF pattern> OK +KUDP_RCV: <udp remote address>,<udp remote port>,<ndata available> or NO CARRIER +CME ERROR: <err> +KUDP_NOTIF: <session_id>, <udp_notif> +KUDP_DATA_MISSED: <session_id>, <ndata missed> Parameters <session_id> UDP session index <ndata> Number of bytes the device wants to receive (max value 4294967295) <udp remote address> IP address string of the remote host <udp remote port> 0 – 65535 Remote UDP port <ndata available> Number of bytes to be read in first received packet <udp_notif> See command AT+KUDPCFG <ndata missed> Number of bytes left in the UDP socket |
| Reference Sierra Wireless Proprietary | Notes - This function is used to receive <ndata> data bytes through a previously opened UDP socket. - <ndata> indicates the max data number that the terminal wishes to receive. If the UDP socket contains more data than <ndata> bytes, then only <ndata> bytes will be received and more data can be read by running this command again. - <EOF pattern> would be added at the end of data automatically. - When <ndata> (max value) bytes or only available data in the UDP socket have been received, the module returns to command mode. - It is highly recommended to configure the module for hardware flow control using AT&K3 before using this command. - Refer to AT&D for the behavior of DTR drop. |

### +KUDPSND Command: Send Data through a UDP Connection

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KUDPSND=? | Response +KUDPSND: (list of possible <session_id>s),<remote-name/ip>,(list of possible <udp_port>s),(list of possible <ndata>s) OK |
| Write command Syntax AT+KUDPSND= <session_id>, <udp remote address>, <udp_port>, <ndata> | Response CONNECT OK or NO CARRIER +CME ERROR: <err> +KUDP_NOTIF: <session_id>,<udp_notif> Parameters <session_id> UDP session index <udp remote address> IP address string or explicit name of the remote host <udp_port> 1 – 65535 UDP peer port <ndata> Number of bytes (max value 4294967295) <udp_notif> See command AT+KUDPCFG |
| Reference Sierra Wireless Proprietary | Notes - All data will be sent out ignoring <ndata>. If data sent is not equal to <ndata> then +KUDP_NOTIF will be displayed. - <ndata> is the data size without <EOF pattern>. - It is highly recommended to configure the module for hardware flow control using AT&K3 before using this command. - Refer to AT&D for the behavior of DTR drop. - The maximum transmission unit (MTU) is 1500 Bytes. - The <udp remote address> and <udp_port> are saved internally such that they can be omitted in subsequent calls of +KUDPSND. - The packet segmentation is controlled by +KIPOPT with <option_id>=0, and the maximum UDP packet size is limited by <send size v4> (1472 bytes) or <send size v6> (1452 bytes). Default value for both parameters is 1020 bytes. - The data session is automatically ended when <ndata> data bytes are sent/received, and the module returns to command state and returns OK. - The data session can also be ended by <EOF pattern>, +++ or DTR. Refer to section 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table for the behavior of +++, DTR or ATO. |

### +KUDPCLOSE Command: Close Current UDP Operation

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KUDPCLOSE=? | Response +KUDPCLOSE: (list of possible <session_id>s),(list of possible <keep_cfg>s) OK |
| Write command Syntax AT+KUDPCLOSE=<session_id> [,<keep_cfg>] | Response OK or +KUDP_NOTIF: <session_id>, <udp_notif> Parameters <session_id> UDP session index <udp_notif> See command AT+KUDPCFG <keep_cfg> Specifies whether to delete the session configuration after closing it or not 0 Delete the session configuration 1 Keep the session configuration |
| Reference Sierra Wireless Proprietary | Notes - This function closes the UDP session. If there is no other session running, the PDP context will be released. - This function will delete the session configuration if <keep_cfg> = 0. |

### +KUDPDEL Command: Delete a Configured UDP Session

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KUDPDEL=? | Response +KUDPDEL: (list of possible <session_id>s) OK |
| Write command Syntax AT+KUDPDEL= <session_id> | Response OK or +CME ERROR: <err> Parameters <session_id> UDP session index |
| Reference Sierra Wireless Proprietary | Notes The session must be closed (using +KUDPCLOSE) before using this command. |

### +KUDP_IND Notification: UDP Status

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Unsolicited Notification | Response +KUDP_IND: <session_id>,<status> Parameters <session_id> UDP session index <status> UDP session status. 1 Session is set up and ready for operation |
| Reference Sierra Wireless Proprietary |  |

### +KUDP_DATA Notification: Incoming Data through a UDP Connection

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Unsolicited Notification | Response +KUDP_DATA: <session_id>,<ndata available>[,<udp remote address>,<udp remote port>,<data>] Parameters <session_id> UDP session index <ndata available> Number of bytes to be read <udp remote address> IP address string of the remote host <udp remote port> 0 – 65535 Remote UDP port <data> Data in octet. The length of data is specified by <ndata_available>. |
| Reference Sierra Wireless Proprietary | Notes - As soon as the UDP socket is created, the module can receive data through this socket. This notification is sent when data are available in the receive buffer. - This notification will be sent one time. When <data_mode> was set to 0 (do not display data in URC), the controlling software must read the buffer with +KUDPRCV in order to activate the notification again. - When <data_mode> was set to 1, <ndata_available> will range from 1 – 1500 in the URC. If the user application sends over 1500 bytes of data to the module, the module will display those data with several URCs. It is possible for other applications (e.g. from Windows) to send more than 1472 bytes UDP packets to the module but the packet will be segmented and reassembled by the network stack. - When <data_mode> is set to 1, URC +KUDP_RCV will not be displayed after +KUDP_DATA. - When <data_mode> was set to 1, the fields <udp remote address> and <udp remote port> will be displayed in URC +KUDP_DATA. When <data_mode> was set to 0, they will be displayed in URC +KUDP_RCV. - See section 18.6.3 Use Cases for KTCP_DATA and KUDP_DATA. |

## FTP Client Specific Commands

### +KFTPCFG Command: FTP Configuration

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KFTPCFG=? | Response +KFTPCFG: (list of possible <cnx cnf>s),<server-name/ip>,(range of possible length of <login>),(range of possible length of <password>),(list of possible <port_number>s),(list of possible <mode>s),(list of possible <start>s),(list of possible <af>s) OK |
| Read command Syntax AT+KFTPCFG? | Response +KFTPCFG: <session_id>,<cnx cnf>,<server_name>,<login>,<password>, <port_number>,<mode>,<started>,<af> |
| Write command Syntax AT+KFTPCFG= [<cnx cnf>], <server_name> [,<login> [,<password> [,<port_number> [,<mode>] [,<start>] [,<af>]]]] | Response +KFTPCFG:<session_id> OK or +KFTP_ERROR: <session_id>,<ftp cause> Parameters <cnx cnf> 1 – 5 (PDP context configuration) a numeric parameter which specifies a particular PDP context configuration <session_id> FTP session index <server_name> IP address string of the ftp server or domain name of the server <login> String type, indicates the user name to be used during the FTP connection <password> String type, indicates the password to be used during the FTP connection <port_number> 1 – 65535 Indicates the remote command port (21 by default) <mode> Indicates the initiator of the FTP connection 0 Active. The server is initiator of the FTP data connection 1 Passive. The client is initiator of the FTP data connection in order to avoid the proxy filtrate. The passive data transfer process “listens” on the data port for a connection from the active transfer process in order to open the data connection <start> Specifies whether to start the FTP connection immediately. 0 Start the FTP connection later by +KFTPCNX 1 Start the FTP connection immediately <started> Specifies whether to the FTP connection is started 0 FTP connection is not started yet 1 FTP connection is started <af> Address family used for the connection. 0 IPV4 1 IPV6 <ftp_cause> Integer type that indicates the cause of the FTP connection failure. 0 The sending or the retrieving was impossible due to request timeout 1 Impossible to connect to the server due to DNS resolution failure 2 Impossible to download a file due to connection troubles 3 Download was impossible due to connection timeout 4 No network available 5 Flash access trouble 6 Flash memory full 7 Network error XXX Three-digit reply code from the FTP server. See section 18.2.5 FTP Reply Codes |
| Reference Sierra Wireless Proprietary | Notes - The write command sets the server name, login, password, port number and mode for FTP operations. - This command (with <start> = 0) can be used before setting up +KCNXCFG. Note however that the latter is required to start the connection properly. - The result of the FTP connection is notified using unsolicited response. |
| Example | AT+KFTPCFG=1,"ftp.connect.com","username","password",21,0 |

### +KFTPCNX Command: Start FTP Connection

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KFTPCNX=? | Response +KFTPCNX: (list of possible <session_id>s) OK |
| Write command Syntax AT+KFTPCNX= <session_id> | Response OK or NO CARRIER +CME ERROR: <err> +KFTP_ERROR: <session_id>,<ftp cause> Parameters <session_id> FTP session index <ftp_cause> Integer type that indicates the cause of the FTP connection failure. 0 Sending or the retrieving was impossible due to request timeout 1 Impossible to connect to the server due to DNS resolution failure 2 Impossible to download a file due to connection troubles 3 Download was impossible due to connection timeout 4 No network available 5 Flash access trouble 6 Flash memory full 7 Network error XXX Three-digit reply code from the FTP server. See section 18.2.5 FTP Reply Codes |
| Reference Sierra Wireless Proprietary | Notes - This command is used to start the FTP connection created by +KFTPCFG with <start>=0. - +KFTPRCV, +KFTPSND, +KFTPDEL automatically starts the connection if it has not been started using AT+KFTPCNX. - The result of the FTP connection is notified using unsolicited response. |

### +KFTPRCV Command: Receive FTP Files

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KFTPRCV=? | Response +KFTPRCV: (list of possible <session_id>s),<local_uri>,<server_path>,<file_name>, (list of possible <type_of_file>s),(list of possible <offset>s) OK |
| Write command Syntax AT+KFTPRCV= <session_id>, [<local_uri>,] [<server_path>,] <file_name> [,<type_of_file> [,<offset>]] | Response CONNECT <EOF_pattern> OK or +CME ERROR<err> NO CARRIER +KFTP_ERROR: <session_id>,<ftp cause> Parameters <session_id> FTP session index <local_uri> This argument must be empty. It is reserved for compatibility of command syntax. <server_path> String type. Indicates the path of the file to be downloaded. An empty string or no string indicates the downloading is done from the path given by the FTP server <file_name> string type. Indicates the name of the file to download <type_of_file> Numeric type. Indicates the type of file (ASCII or binary) to transfer 0 Binary (default value) 1 ASCII <offset> 0 – 4294967295 Integer type indicating the offset to “resume transfer”. See section 18.7.2 "FTP Resume" Use Case. When downloading file and transmitting to serial link, module will use the <offset> value and “resume transfer” from this position. <EOF_pattern> End of file notification. See +KPATTERN for value <ftp_cause> Integer type that indicates the cause of the FTP connection failure 0 Sending or the retrieving was impossible due to request timeout 1 Impossible to connect to the server due to DNS resolution failure 2 Impossible to download a file due to connection troubles. 3 Download was impossible due to connection timeout 4 No network available 5 Flash access trouble 6 Flash memory full 7 Network error XXX Three-digit reply code from the FTP server. See section 18.2.5 FTP Reply Codes |
| Reference Sierra Wireless Proprietary | Notes - An FTP connection must have been achieved using AT+KFTPCFG before using this command. - The user will receive the entire data stream after sending the +KFTPRCV command. - The user can abort the download by sending the “End of Data pattern” from the host. In this case, the module will end the transfer by transmitting the EOF followed by NO CARRIER. - Download can also be aborted (disconnected) by +++ or DTR as specified in section 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table. - If AT&C1 is set, DCD will be ON after CONNECT and DCD will be OFF after download is done. - “Resume transfer” feature should be supported by the FTP server to be used. - See section 18.7.2 "FTP Resume" Use Case. - If the FTP server does not support the resume feature, the module will output +KFTP_ERROR. The <ftp_cause> will be in the sets {500, 501, 502, 421, 530}. See section 18.2.5 FTP Reply Codes for error codes. |

### +KFTPSND Command: Send FTP Files

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KFTPSND=? | Response +KFTPSND: (list of possible <session_id>s),<local_uri>,<server_path>,<file_name>, (list of possible <type of file>s),(list of possible <append>s) OK |
| Write command Syntax AT+KFTPSND= <session_id>, [<local_uri>,] [<server_path>,] <file_name> [,<type of file>] [,<append>] | Response CONNECT OK or +CME ERROR <err> NO CARRIER +KFTP_ERROR: <session_id>,<ftp cause> Parameters <session_id> FTP session index <local_uri> This argument must be empty. It is reserved for compatibility of command syntax. <server_path> String type. Indicates the path of the file to be uploaded. An empty string or no string indicates the uploading is done from the path given by the FTP server <file_name> String type. Indicates the name of the file to upload <type of file> Numeric type. Indicates the type of file (ASCII or binary) to transfer 0 Binary 1 ASCII <append> Numeric type. Indicates using "append" or not when uploading. 0 Do not use "append". If the file already exists then the file will be overridden 1 Use "append". If the file already exists then the data will be appended at the end of the file; otherwise the file will be created <EOF pattern> End of file notification. See +KPATTERN for values <ftp_cause> Integer type that indicates the cause of the FTP connection failure. 0 Sending or the retrieving was impossible due to request timeout 1 Impossible to connect to the server due to DNS resolution failure 2 Impossible to download a file due to connection troubles. 3 Download was impossible due to connection timeout 4 No network available 5 Flash access trouble 6 Flash memory full 7 Network error XXX Three-digit reply codes from the FTP server. See section 18.2.5 FTP Reply Codes |
| Reference Sierra Wireless Proprietary | Notes - An FTP connection must have been achieved using AT+KFTPCFG before using this command. - After sending the +KFTPSND command, the host must send the entire data stream of the file. - Upload can also be ended (disconnected) by +++ or DTR as specified in 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table. - ATO is not available for this command. - If AT&C1 is set, DCD will be ON after CONNECT, and it will be OFF after the upload done. |

### +KFTPDEL Command: Delete FTP Files

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KFTPDEL=? | Response +KFTPDEL: (list of possible <session_id>s),<server_path>,<file_name>,(list of possible <type>s) OK |
| Write command Syntax AT+KFTPDEL= <session_id>, [<server_path>,] <file_name> [,<type>] | Response OK or +CME ERROR <err> NO CARRIER +KFTP_ERROR: <session_id>,<ftp cause> Parameters <session_id> FTP session index <server_path> String type. Indicates the path of the file to be deleted. An empty string or no string indicates the deleting is done from the path given by the <server_name> parameter <file_name> String type. Indicates the name of the file to delete <type> Numeric type. Indicates the type of file (ASCII or binary) to transfer 0 Binary 1 ASCII <ftp_cause> Integer type that indicates the cause of the FTP connection failure 0 Sending or the retrieving was impossible due to request timeout 1 Impossible to connect to the server due to DNS resolution failure 2 Impossible to delete a file due to connection troubles 3 Deleting was impossible due to connection timeout 4 No network available XXX Three-digit reply codes from the FTP server. See section 18.2.5 FTP Reply Codes |
| Reference Sierra Wireless Proprietary | Notes - An FTP connection must have been achieved using AT+KFTPCFG before using this command. - The result of the delete operation is notified using unsolicited response. |

### +KFTP_IND Notification: FTP Status

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Unsolicited Notification | Response +KFTP_IND: <session_id>,<status>[,<data_len>] Parameters <session_id> FTP session index <status> FTP session status 1 Session is set up and ready for operation 2 The last FTP command is executed successfully <data_len> Byte length of data downloaded/uploaded to/from the terminal (+KFTPRCV/+KFTPSND) |
| Reference Sierra Wireless Proprietary |  |

### +KFTPCLOSE Command: Close Current FTP Connection

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KFTPCLOSE=? | Response +KFTPCLOSE: (list of possible <session_id>s),(list of possible <keep_cfg>s) OK |
| Write command Syntax AT+KFTPCLOSE=<session_id> [,<keep_cfg>] | Response OK Parameters <session_id> FTP session index <keep_cfg> Specifies whether to delete the session configuration after closing it or not 0 Delete the session configuration 1 Keep the session configuration |
| Reference Sierra Wireless Proprietary | Notes This command will close the connection to the FTP server. |

### +KFTPCFGDEL Command: Delete a Configured FTP Session

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ KFTPCFGDEL=? | Response +KFTPCFGDEL: (list of possible <session_id>s) OK |
| Write command Syntax AT+ KFTPCFGDEL= <session_id> | Response OK or +CME ERROR: <err> Parameter <session_id> FTP session index |
| Reference Sierra Wireless Proprietary | Notes The session must be closed (using +KFTPCLOSE) before using this command. |

## HTTP Client Specific Commands

All commands in this sub-section are for the HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 only.

### +KHTTPCFG Command: HTTP Connection Configuration

| HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KHTTPCFG =? | Response +KHTTPCFG: (list of possible <cnx_cnf>s),<server-name/ip>,(list of possible <http_port>s),(list of possible <http_version>s),(range of possible length of <login>), (range of possible length of <password>),(list of possible <started>s), (list of possible <af>s),<cipher_index> OK |
| Read command Syntax AT+KHTTPCFG? | Response +KHTTPCFG: <session_id>,<cnx cnf>,<http_server>,<http_port>,<http_version>, <login>,<password>,<started>,<af>,<cipher_index> OK |
| Write command Syntax AT+KHTTPCFG= [<cnx cnf>], <http_server> [,<http_port> [,<http_version> [,<login> [,<password>] [,<start>] [,<af>]]] [,<cipher_index> ]] | Response +KHTTPCFG: <session_id> OK or +CME ERROR: <err> Parameters <cnx cnf> 1 – 5 (PDP context configuration) a numeric parameter which specifies a particular PDP context configuration (see KCNXCFG) <session_id> HTTP session index <http_server> IP address string or explicit name of the remote server <http_port> Numeric parameter (1-65535), 80 by default <http_version> 0 HTTP 1.1(by default) 1 HTTP 1.0 2 HTTP 1.1 over TLS (HTTPS) 3 HTTP 1.0 over TLS (HTTPS) <login> String type, indicates the user name to be used during the HTTP connection <password> String type, indicates the password to be used during the HTTP connection <start> Specifies whether to start the HTTP connection immediately or not 0 Start the HTTP connection later using +KTTPCNX 1 Start the HTTP connection immediately <started> Specifies whether the HTTP connection has been started 0 The HTTP connection has not been started yet 1 The HTTP connection has already been started <af> Address family used for the connection. Default is IPV4. 0 IPV4 1 IPV6 <cipher_index> Cipher suite profile index to use for a secured socket; defined by +KSSLCRYPTO |
| Reference Sierra Wireless Proprietary | Notes - <http_port> and <http_server> define the port and the IP address of the remote server one wants to connect. - The connection timeout for TCP socket is about 9 seconds with 3 retransmissions with 3 seconds delay. - This command can be used before setting up +KCNXCFG configuration. Note however that the latter is required to start the connection properly. - For <af>=1 (IPV6), server address (<http_server>) in IP address string format can be optionally quoted with square brackets “[]”, e.g. [FEDC:BA98:7654:3210:FEDC:BA98:7654:3210] |

### +KHTTPCNX Command: Start the HTTP Connection

| HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KHTTPCNX=? | Response +KHTTPCNX: (list of possible <session_id>s) OK |
| Write command Syntax AT+KHTTPCNX=<session_id> | Response OK or +CME ERROR: <err> +KHTTP_ERROR: <session_id>, <http_notif> Parameters <session_id> HTTP session index <http_notif> Integer type. Indicates the cause of the HTTP connection failure 4 DNS error 5 HTTP connection error due to internal trouble 6 HTTP connection timeout 9 Triple plus (+++) error (switch to command mode) 10 HTTP has no data 11 HTTP has partial data |
| Reference Sierra Wireless Proprietary | Notes - This command is used to start the HTTP connection created by +KHTTPCFG with <start>=0. - +KHTTPGET, +KHTTPHEAD, +KHTTPPOST automatically starts the connection if it has not been started before using AT+KHTTPCNX. |

### +KHTTPHEADER Command: Set the HTTP Request Header

| HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ KHTTPHEADER =? | Response +KHTTPHEADER: (list of possible <session_id>s),<local_uri> OK |
| Read command Syntax AT+ KHTTPHEADER? | Response +KHTTPHEADER: <session_id>,<count> […] |
| Write command Syntax AT+ KHTTPHEADER= <session_id> [,<local_uri>] | Response OK or +CME ERROR: <err> Parameters <session_id> HTTP session index <local_uri> This argument must be empty. It is reserved for compatibility of command syntax. <count> Count of HTTP headers |
| Reference Sierra Wireless Proprietary | Notes - The data session is automatically ended when <ndata> data bytes are sent/received, and the module returns to command state and returns OK. - The data session can also be ended by <EOF pattern>, +++ or DTR. Refer to section 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table for the behavior of +++, DTR or ATO. |

### +KHTTPGET Command: Get HTTP Server Information

| HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KHTTPGET =? | Response +KHTTPGET: (list of possible <session_id>s),<request_uri >, (list of possible <show_resp>s) OK |
| Write command Syntax AT+KHTTPGET= <session_id>, <request_uri> [,<show_resp>] | Response CONNECT …<EOF pattern> OK or NO CARRIER +CME ERROR: <err> +KHTTP_ERROR: <session_id>, <http_notif> Parameters <session_id> HTTP session index <request_uri> string type, indicates the information url to get during the HTTP connection <http_notif> Integer type. Indicates the cause of the HTTP connection failure 4 DNS error 5 HTTP connection error due to internal trouble 6 HTTP connection timeout 9 Triple plus (+++) error (switch to command mode) 10 HTTP has no data 11 HTTP has partial data <show_resp> Whether to show HTTP response and HTTP headers 0 Do not show response and headers 1 Show response and headers (default) |
| Reference Sierra Wireless Proprietary | Notes - <session_id> is always 0. - The user can abort the download by sending the “End of Data pattern” from the host. In this case, the module will end the transfer by transmitting the EOF followed by NO CARRIER. - Download can also be aborted (disconnected) by +++ or DTR as specified in 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table. - HTTP does not support DTR1. |

### +KHTTPHEAD Command: Get HTTP Headers

| HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KHTTPHEAD=? | Response +KHTTPHEAD: (list of possible <session_id>s),<request_uri> OK |
| Write command Syntax AT+KHTTPHEAD=<session_id>, <request_uri> | Response CONNECT …<EOF pattern> OK or NO CARRIER +CME ERROR: <err> +KHTTP_ERROR: <session_id>, <http_notif> Parameters <session_id> HTTP session index <request_uri> String type, indicates the information URL to get during HTTP connection <http_notif> Integer type. Indicates the cause of the HTTP connection failure 4 DNS error 5 HTTP connection error due to internal trouble 6 HTTP connection timeout 9 Triple plus (+++) error (switch to command mode) 10 HTTP has no data 11 HTTP has partial data |
| Reference Sierra Wireless Proprietary | Notes - HTTP does not support DTR1 - This method is identical to GET except that the server MUST NOT return a message-body in the response. The meta-information contained in the HTTP headers in response to a HEAD request SHOULD be identical to the information sent in response to a GET request |

### +KHTTPPOST Command: Perform HTTP Post

| HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KHTTPPOST=? | Response +KHTTPPOST: (list of possible <session_id>s),<local_uri>,<request_uri>,(list of possible <show_resp>s) OK |
| Write command Syntax AT+KHTTPPOST= <session_id>, <local_uri>, <request_uri> [,<show_resp>] | Response CONNECT …<EOF pattern> OK or NO CARRIER +CME ERROR: <err> +KHTTP_ERROR: <session_id>, <http_notif> Parameters <session_id> HTTP session index <local_uri> This argument must be empty. It is reserved for compatibility of command syntax. <request_uri> String type, the request data of the HTTP connection <http_notif> Refer to +KHTTPGET <show_resp> Whether to show HTTP response and HTTP headers 0 Do not show HTTP response and headers 1 Show HTTP response and headers (default) |
| Reference Sierra Wireless Proprietary | Notes - <session_id> is always 0. - It is highly recommended to configure the module for hardware flow control using AT&K3 before using this command. - Upload can also be ended (disconnected) by +++ or DTR as specified in 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table. - ATO is not available for this command. |

### +KHTTPCLOSE Command: Close an HTTP Connection

| HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ KHTTPCLOSE=? | Response +KHTTPCLOSE: (list of possible <session_id>s), (list of possible <keep_cfg>s) OK |
| Write command Syntax AT+ KHTTPCLOSE= <session_id> [,<keep_cfg>] | Response OK or +CME ERROR: <err> Parameters <session_id> HTTP session index <keep_cfg> Specifies whether to delete the session configuration after closing it 0 Delete the session configuration 1 Keep the session configuration |
| Reference Sierra Wireless Proprietary |  |

### +KHTTPDEL Command: Delete a Configured HTTP Session

| HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KHTTPDEL =? | Response +KHTTPDEL: (list of possible <session_id>s) OK |
| Write command Syntax AT+KHTTPDEL=<session_id> | Response OK or +CME ERROR: <err> Parameter <session_id> HTTP session index |
| Reference Sierra Wireless Proprietary | Notes The HTTP session must be closed (using +KHTTPCLOSE) before using this command. |

### +KHTTP_IND Notification: HTTP Status

| HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Unsolicited Notification | Response +KHTTP_IND: <session_id>,<status>[,<data_len>,<st_code>,<st_reason>] Parameters <session_id> HTTP session index <status> HTTP session status 1 Session is set up and ready for operation 3 The last HTTP command is executed successfully <data_len> Byte length of data downloaded/uploaded to/from the terminal (using +KHTTPHEAD, +KHTTPGET, or +KHTTPPOST) <st_code> HTTP response status code <st_reason> HTTP response status reason string |
| Reference Sierra Wireless Proprietary |  |

## HTTPS Client Specific Commands

All commands in this sub-section are for the HL7618RD, HL7648, HL7650, HL7690 and HL7692 only.

### +KHTTPSCFG Command: HTTPS Connection Configuration

| HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KHTTPSCFG=? | Response +KHTTPSCFG: (list of possible <cnx_cnf>s),<server-name/ip>,(list of possible <https_port>s),(list of possible <http_version>s),(list of possible <cipher_suite>s) ,(list of possible <sec_level>s) ,(range of possible length of <login>),(range of possible length of <password>),(list of possible <started>s), (list of possible <af>s) OK |
| Read command Syntax AT+KHTTPSCFG? | Response +KHTTPSCFG: <session_id>,<cnx cnf>,<http_server>,<https_port>, <http_version>, <cipher suite>,<sec_level>,<login>,<password>,<started>,<af> OK |
| Write command Syntax AT+KHTTPSCFG=[<cnx cnf>], <http_server> [,<https_port> [,<http_version> [,<cipher_suite> [,<sec_level> [,<login> [,<password>] [,<start>] [,<af>]]]]]] | Response +KHTTPSCFG: <session_id> OK or +CME ERROR: <err> Parameters <cnx cnf> 1 – 5 (PDP context configuration) a numeric parameter which specifies a particular PDP context configuration (see +KCNXCFG). <session_id> HTTPS session index <http_server> IP address string or explicit name of the remote server <https_port> Numeric parameter (1-65535), 443 by default. <http_version> 0 HTTP 1.1 1 HTTP 1.0 <cipher_suite> 0 TLS_RSA_CHOOSE_BY_SERVER 1 TLS_RSA_WITH_RC4_128_MD5 2 TLS_RSA_WITH_RC4_128_SHA 3 TLS_RSA_WITH_DES_CBC_SHA (not supported) 4 TLS_RSA_WITH_3DES_EDE_CBC_SHA (not supported) 5 TLS_RSA_EXPORT1024_WITH_DES_CBC_SHA (not supported) 6 TLS_RSA_WITH_AES_128_CBC_SHA 7 TLS_RSA_WITH_AES_256_CBC_SHA 8 TLS_RSA_WITH_AES_128_GCM_SHA256 <sec_level> 1 No authentication 2 Manage server authentication (renegotiation of client certificate is not supported) 3 Manage server and client authentication if requested by remote server (renegotiation of client certificate is not supported) <login> String type, indicates the user name to be used during the HTTPS connection. <password> String type, indicates the password to be used during the HTTPS connection. <start> Specifies whether to start the HTTPS connection immediately or not 0 Start the HTTPS connection later using +KTTPSCNX 1 Start the HTTPS connection immediately <started> Specifies whether the HTTPS connection has been started 0 The HTTPS connection has not been started yet 1 The HTTPS connection has already been started <af> Address family used for the connection 0 IPV4 1 IPV6 |
| Reference Sierra Wireless Proprietary | Notes - <https_port> and <http_server> define the port and the IP address of the remote server one wants to connect. - The connection timeout for TCP socket is about 9 seconds with 3 retransmissions with 3 seconds delay. - For <sec_level>:2 and 3, certificates or private key must be loaded from internal storage. See SSL Certificate Management for more information. - Any certificates referenced in HTTPS feature should be DER encoded. - Any private key referenced in HTTPS feature should be DER- PKCS#8 encoded. - This command can be used before setting up +KCNXCFG configuration. Note however that the latter is required to start the connection properly. - For <af>=1 (IPV6), server address (<http_server>) in IP address string format can be optionally quoted with square brackets “[]”, e.g. [FEDC:BA98:7654:3210:FEDC:BA98:7654:3210] - SSL version is TLS 1.1 by default; refer to <ssl_ver> of +KIPOPT for configuration. |

### +KHTTPSCNX Command: Start HTTPS Connection

| HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KHTTPSCNX=? | Response +KHTTPSCNX: (list of possible <session_id>s) OK |
| Write command Syntax AT+KHTTPSCNX=<session_id> | Response OK or +CME ERROR: <err> +KHTTPS_ERROR: <session_id>, <http_notif> Parameters <session_id> HTTPS session index <http_notif> Integer type. Indicates the cause of the HTTPS connection failure 4 DNS error 5 HTTPS connection error due to internal trouble 6 HTTPS connection timeout 9 Triple plus (+++) error (switch to command mode) 10 HTTPS got no data 11 HTTPS got partial data |
| Reference Sierra Wireless Proprietary | Notes - This command is used to start the HTTPS connection created by +KHTTPSCFG with <start>=0. - +KHTTPSGET, +KHTTPSHEAD, +KHTTPSPOST automatically starts the connection if it has not been started using AT+KHTTPSCNX. |

### +KHTTPSHEADER Command: Set the HTTPS Request Header

| HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ KHTTPSHEADER=? | Response +KHTTPSHEADER: (list of possible <session_id>s), <local_uri> OK |
| Read command Syntax AT+ KHTTPSHEADER? | Response +KHTTPSHEADER: <session_id>,<count> […] |
| Write command Syntax AT+ KHTTPSHEADER=<session_id> [,<local_uri>] | Response OK or +CME ERROR: <err> Parameters <session_id> HTTPS session index <local_uri> This argument must be empty. It is reserved for compatibility of command syntax. <count> HTTPS header count |
| Reference Sierra Wireless Proprietary | Notes - The data session is automatically ended when <ndata> data bytes are sent/received, and the module returns to command state and returns OK. - The data session can also be ended by <EOF pattern>, +++ or DTR. Refer to section 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table for the behavior of +++, DTR or ATO. |

### +KHTTPSGET Command: Get Information from HTTPS Server

| HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KHTTPSGET=? | Response +KHTTPSGET: (list of possible <session_id>s),<request_uri>,(list of possible <show_resp>s) OK |
| Write command Syntax AT+KHTTPSGET=<session_id>, <request_uri> [,<show_resp>] | Response CONNECT …<EOF pattern> OK or NO CARRIER +CME ERROR: <err> +KHTTPS_ERROR: <session_id>, <http_notif> Parameters <session_id> HTTPS session index <request_uri> String type, indicates the information URL to get during HTTPS connection <http_notif> Integer type. Indicates the cause of the HTTPS connection failure 4 DNS error 5 HTTP connection error due to internal trouble 6 HTTP connection timeout 7 Flash access trouble 8 Flash memory full 9 Triple plus (+++) error (switch to command mode) 10 HTTP has no data 11 HTTP has partial data 12 SSL connection error 13 SSL initialization error <show_resp> Defines whether HTTPS response and HTTPS headers are shown 0 Do not show HTTPS response and headers 1 Show HTTPS response and headers |
| Reference Sierra Wireless Proprietary | Notes - The user can abort the download by sending the “End of Data pattern” from the host. In this case, the module will end the transfer by transmitting the EOF followed by NO CARRIER. - Download can also be aborted (disconnected) by +++ or DTR as specified in 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table. |

### +KHTTPSHEAD Command: Retrieve HTTP Headers

| HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ KHTTPSHEAD=? | Response +KHTTPSHEAD: (list of possible <session_id>s),<request_uri> OK |
| Write command Syntax AT+ KHTTPSHEAD= <session_id>, <request_uri> | Response CONNECT …<EOF pattern> OK or NO CARRIER +CME ERROR: <err> +KHTTPS_ERROR: <session_id>, <http_notif> Parameters <session_id> HTTPS session index <request_uri> String type, indicates the information URL to get during HTTPS connection |
| Reference Sierra Wireless Proprietary | Notes - HTTPS does not support DTR1 - This method is identical to GET except that the server MUST NOT return a message-body in the response. The meta-information contained in the HTTP headers in response to a HEAD request SHOULD be identical to the information sent in response to a GET request. |

### +KHTTPSPOST Command: Send Data to HTTPS Server

| HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ KHTTPSPOST=? | Response +KHTTPSPOST: (list of possible <session_id>s),<local_uri>,<request_uri>,(list of possible <show_resp>s) OK |
| Write command Syntax AT+ KHTTPSPOST= <session_id>, <local_uri>, <request_uri> [,<show_resp>] | Response CONNECT …<EOF pattern> OK or NO CARRIER +CME ERROR: <err> +KHTTPS_ERROR: <session_id>, <http_notif> Parameters <session_id> HTTPS session index <local_uri> This argument must be empty. It is reserved for compatibility of command syntax. <request_uri> String type, indicates the request data of the HTTPS connection <http_notif> Refer to +KHTTPSGET <show_resp> Defines whether HTTPS response and HTTPS headers are shown 0 Do not show HTTPS response and headers 1 Show HTTPS response and headers |
| Reference Sierra Wireless Proprietary | Notes - It is highly recommended to configure the module for hardware flow control using AT&K3 before using this command. - Upload can also be ended (disconnected) by +++ or DTR as specified in 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table. - ATO is not available for this command. |

### +KHTTPSCLOSE Command: Close an HTTPS Connection

| HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ KHTTPSCLOSE =? | Response +KHTTPSCLOSE: (list of possible <session_id>s), (list of possible <keep_cfg>s) OK |
| Write command Syntax AT+ KHTTPSCLOSE= <session_id> [,<keep_cfg>] | Response OK or +CME ERROR: <err> Parameters <session_id> HTTPS session index <keep_cfg> Specified whether to delete the session configuration after closing it 0 Delete the session configuration 1 Keep the session configuration |
| Reference Sierra Wireless Proprietary |  |

### +KHTTPSDEL Command: Close an HTTPS Connection

| HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+KHTTPSDEL=? | Response +KHTTPSDEL: (list of possible <session_id>s) OK |
| Write command Syntax AT+KHTTPSDEL=<session_id> | Response OK or +CME ERROR: <err> Parameter <session_id> HTTPS session index |
| Reference Sierra Wireless Proprietary | Notes The session must be closed (using +KHTTPSCLOSE) before using this command. |

### +KHTTPS_IND Notification: HTTPS Status

| HL7618RD, HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Unsolicited Notification | Response +KHTTPS_IND: <session_id>,<status>[,<data_len>] Parameters <session_id> HTTPS session index <status> HTTPS session status 1 Session is set up and ready for operation 2 The last HTTPS command is executed successfully <data_len> Byte length of data downloaded/uploaded to/from the terminal (using +KHTTPSHEAD, +KHTTPSGET, or +KHTTPSPOST) |
| Reference Sierra Wireless Proprietary |  |

## SSL Certificate Manager

All commands in this sub-section are for the HL7648, HL7650, HL7690 and HL7692 only.

### +KCERTSTORE Command: Store Root CA and Local Certificates to Internal Storage

| HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ KCERTSTORE=? | Response +KCERTSTORE: (list of possible <data_type>s),(range of possible lengths of <NbData>), (list of possible <index>es) OK |
| Read command Syntax AT+ KCERTSTORE? | Response +KCERTSTORE [root_cert,<index>,<NbData><CR><LF> <File_data><CR><LF>] [local_cert,<index>,<NbData><CR><LF> <File_data> <CR><LF>] [...] OK or +CME ERROR: <err> |
| Write command Syntax AT+ KCERTSTORE= <data_type> [,<NbData> [,<index>]] | Response CONNECT OK or +CME ERROR: <err> Parameters <data_type> 0 Root certificate 1 Local certificate <NbData> Number of bytes to read/write. Value range: 1-3000. <index> Stored root/local certificate index. If a root/local certificate is already stored at the index, it will be overloaded. 0 by default. Value range: 0 If <data_type> = 0 0 – 2 If <data_type> = 1 <File_data> File data in bytes |
| Reference Sierra Wireless Proprietary | Notes - The <index> parameter is the link between a local certificate and a private key (refer to +KPRIVKSTORE and +KCERTDELETE for more information). - The data session is automatically ended when <ndata> data bytes are sent/received, and the module returns to command state and returns OK. - The data session can also be ended by <EOF pattern>, +++ or DTR. Refer to section 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table for the behavior of +++ or DTR. - ATO is not available for this command. |

### +KPRIVKSTORE Command: Store Private Key Associated to a Local Certificate

| HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ KPRIVKSTORE =? | Response +KPRIVKSTORE: (list of possible <index>s),( range of possible lengths of <NbData>) OK |
| Read command Syntax AT+ KPRIVKSTORE? | Response +KPRIVKSTORE private_key,<index>,<NbData><CR><LF> <File_data> <CR><LF> OK or +CME ERROR: <err> |
| Write command Syntax AT+ KPRIVKSTORE= <index> [,<NbData>] | Response CONNECT OK or +CME ERROR: <err> Parameters <index> Index of the stored local certificate associated to this private key. Value range: 0 – 2 <NbData> Number of bytes to read/write (mandatory for both reading and writing). Value range: 1-3000. <File_data> File data in bytes |
| Reference Sierra Wireless Proprietary | Notes - The data session is automatically ended when <ndata> data bytes are sent/received, and the module returns to command state and returns OK. - The data session can also be ended by <EOF pattern>, +++ or DTR. Refer to section 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table for the behavior of +++ or DTR. - ATO is not available for this command. |

### +KCERTDELETE Command: Delete Local Certificate from the Index

| HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ KCERTDELETE =? | Response +KCERTDELETE: (list of possible <data_type>s),(list of possible <index>s) OK |
| Read command Syntax AT+ KCERTDELETE? | Response +KCERTDELETE: OK or +CME ERROR: <err> |
| Write command Syntax AT+ KCERTDELETE=<data_type> [,<index>] | Response OK or +CME ERROR: <err> Parameters <data_type> 0 Root certificate 1 Local certificate <index> Stored local certificate index. Default value = 0. Value range: 0 If <data_type> = 0 0 – 2 If <data_type> = 1 |
| Reference Sierra Wireless Proprietary |  |

### +KPRIVKDELETE Command: Delete Private Key from the Index

| HL7648, HL7650, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ KPRIVKDELETE =? | Response +KPRIVKDELETE: (list of possible <index>es) OK |
| Write command Syntax AT+ KPRIVKDELETE=<index> | Response OK or +CME ERROR: <err> Parameter <index> Stored private key index. Value range: 0 – 2 |
| Reference Sierra Wireless Proprietary |  |

# AVMS Commands

## +WDSC Command: Device Services Configuration

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+WDSC=? | Response +WDSC: (0-2), (list of supported <State>s) +WDSC: 3, (list of supported <State>s) +WDSC: 4, (list of supported <Timer_n>s) OK |
| Read command Syntax AT+WDSC? | Response +WDSC: 0,<State> +WDSC: 1,<State> +WDSC: 2,<State> +WDSC: 3,<State> +WDSC: 4,<Timer_1>[[,<Timer_2>]…[,<Timer_n]] OK |
| Write command Syntax For <Mode>= 0, 1, 2 or 3 AT+WDSC= <Mode>,<State> For <Mode>= 4 AT+WDSC= <Mode>, <timer_1> [[,<Timer_2>]… [,<Timer_n>]] | Response OK or +CME ERROR <err> Parameters <Mode> Integer type 0 User agreement for connection When this mode is activated and when a notification SMS is received by the embedded module, an indication (see +WDSI indication for more information) is returned by the embedded module to request for an agreement before connecting to the AirPrime Management Services server 1 User agreement for package download When this mode is activated, an indication (see +WDSI indication for more information) is returned by the embedded module to request for an agreement before downloading any package 2 User agreement for package install When this mode is activated, an indication (see +WDSI indication for more information) is returned by the embedded module to request for an agreement before installing any package 3 Polling mode The embedded module will initiate a connection to the Device Services server according to the defined timer 4 Retry mode If an error occurs during a connection to the Device Services server (GPRS establishment failed, etc.), the embedded module will initiate a new connection according to the defined timers. This mechanism is persistent to the reset. <State> Integer type; mode status For <Mode> = 0, 1 or 2 0 Disabled (default value) 1 Enabled For <Mode> = 3 Value in range 0 – 525600 (units = min) 0 The polling mode is deactivated <Timer_1> Timer between the first failed connection and the next attempt. Value in range 0 – 20160 (units = min). 0 The retry mode is deactivated 15 Default value <Timer_n> Timer between the nth failed attempt connection and the (n+1)th connection (n<=8). Value in range 1 – 20160 (units = min) Default values: <Timer_2> = 60 <Timer_3> = 240 <Timer_4> = 960 <Timer_5> = 2880 <Timer_6> = 10080 <Timer_7> = 10080 |
| Notes | - This command is available when the embedded module has finished the Device Services initialization (see +WDSI) and when the AVMS services are in prohibited state (see +WDSG). - Parameters <State> and <Timer_1> to <Timer_n> are stored in non-volatile memory. The &F command has no impact on these values. - The network registration is considered as “failed” when all connections configured by the retry mode have failed. This registration is forbidden while the APN is not set by the +WDSS command. |
| Examples | AT+WDSC=? +WDSC:(0-2),(0-1) +WDSC:3,(0-525600) +WDSC:4,(0-20160),(1-20160),(1-20160),(1-20160),(1-20160),(1-20160),(1-20160), (1-20160) OK AT+WDSC? // All modes are deactivated except retry mode which is used with default timers +WDSC: 0,0 +WDSC: 1,0 +WDSC: 2,0 +WDSC: 3,0 +WDSC: 4,15,60,240,960,2880,10080,10080 OK AT+WDSC=0,1 OK AT+WDSC? +WDSC: 0,1 +WDSC: 1,0 +WDSC: 2,0 +WDSC: 3,0 +WDSC: 4,15,60,240,960,2880,10080,10080 OK |

## +WDSD Command: Device Services Local Download

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+WDSD=? | Response +WDSD: (list of supported <Size>s) OK |
| Write command Syntax AT+WDSD= <Size> | Response <NACK> // User sends data OK or +CME ERROR <err> Parameter <Size> 1 – 24643584 Package size in bytes |
| Examples | AT+WDSD=? +WDSD: (1-24643584) OK AT+WDSD=1024 //download a 1kBytes package <NACK> //the device is ready to receive data //Send data OK //All data are well received by the module +WDSI: 3 //A package is ready to install (see +WDSI and +WDSR commands) |
| Reference Sierra Wireless Proprietary Command | Notes - This command is available when the embedded module has finished its initialization. - The response to the AT+WDSD=<Size> command is the <NACK> character when the device is ready to receive data using the 1K-Xmodem protocol - The flow control of the TE must be set to ‘Hardware’ - This command will automatically activate the user agreement for install (see +WDSC command description). - No reset is made during the package download. - A timeout will happen (and a +CME ERROR: 3 is returned) if no data is sent to the device in 5 minutes. |

## +WDSE Command: Device Services Error

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Execute command Syntax AT+WDSE | Response [+WDSE:<HTTP_Status>] OK or +CME ERROR <err> Parameter <HTTP_Status> Integer type – last HTTP response received by the module 100 Continue 101 Switching Protocols 200 OK 201 Created 202 Accepted 203 Non-Authoritative Information 204 No Content 205 Reset Content 206 Partial content 300 Multiple Choices 301 Moved Permanently 302 Found 303 See Other 304 Not Modified 305 Use Proxy 307 Temporary Redirect 400 Bad Request 401 Unauthorized 402 Payment Required 403 Forbidden 404 Not Found 405 Method Not Allowed 406 Not Acceptable 407 Proxy Authentication Required 408 Request time-out 409 Conflict 410 Gone 411 Length Required 412 Precondition Failed 413 Request Entity too large 414 Request URI too large 415 Unsupported Media type 416 Request range unsatisfiable 417 Expectation failed 500 Internal server error 501 Not implemented 502 Bad Gateway 503 Service unavailable 504 Gateway time-out 505 HTTP version not supported If no session was made with the server, AT+WDSE only returns OK, without +WDSE: <HTTP_Status> intermediary response. |
| Notes | This command is available when the embedded module has finished the Device Services initialization (see +WDSI) and when AVMS services is activated (see +WDSG). |
| Examples | AT+WDSS=1,1 //A session was made with the server OK AT+WDSE +WDSE: 200 //The last HTTP response received is “OK” OK |

## +WDSF Command: Device Services Fallback

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+WDSF=? | Response +WDSF: (list of supported <Mode>s) OK |
| Read command Syntax AT+WDSF? | Response +WDSF: 1,<FallbackInfo> +WDSF: 2,<EraseInfo> OK |
| Write command Syntax AT+WDSF= <Mode> | Response OK or +CME ERROR <err> Parameters <Mode> Integer type 1 Downgrade to a previous installation 2 Delete the downloaded package which contains the reverse patch <FallbackInfo> Integer type – Indicates the presence of the previous package 0 Previous package is not present 1 Previous package is present <EraseInfo> Integer type – Indicate if a package can be deleted. Be careful, erasing the package will disable the possibility to make any recovery or manual fallback 0 The package cannot be deleted 1 The package can be deleted |
| Notes | This command is available when the embedded module has finished the Device Services initialization (see +WDSI). |
| Examples | AT+WDSF? //a reverse package is present, deletion impossible +WDSF: 1,1 +WDSF: 2,0 OK AT+WDSF=1 //downgrade to the previous installation OK +WDSI: 17,1 //downgrade the package successfully done, displayed only if //+WDSI indication is activated |

## +WDSG Command: Device Services General Status

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+WDSG=? | Response OK |
| Execute command Syntax AT+WDSG | Response +WDSG: <Indication>,<State> [+WDSG: <Indication>,<State>[…]] OK or +CME ERROR <err> Parameters <Indication> Integer type 0 Device services activation state 1 Session and package indication <State> Status of indication For <Indication>=0 0 Device services are prohibited. Devices services will never be activated. 1 Device services are deactivated. Connection parameters to a device services have to be provisioned. 2 Device services must be provisioned. NAP parameters must be provisioned. 3 Device services are activated. If a device has never been activated (first use of device services on this device), <State> is set to 1. The connection parameters are automatically provisioned, no action is needed from the user. For <Indication>=1 0 No session or package 1 A session is under treatment 2 A package is available on the server. 3 A package was downloaded and ready to install When a package was installed or a recovery was made, <State> is set to 0. |
| Notes | This command is available when the embedded module has finished the Device Services initialization (see +WDSI). |
| Examples | AT+WDSG=? OK AT+WDSG +WDSG: 0,3 //Device services are activated, +WDSG: 1,0 //No session to the server, no patch to download or to install OK |

## +WDSI Command: Device Services Indication

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+WDSI=? | Response +WDSI: (list of supported <Level>s) OK |
| Read command Syntax AT+WDSI? | Response [+WDSI: <Level>] OK |
| Write command Syntax AT+WDSI= <Level> | Response OK or +CME ERROR <err> Parameters <Level> Indication level, bit field (default value = 0) Bit set to 0 Indication deactivated Bit set to 1 Indication activated 0 No indication 1 Activate the initialization end indication (<Event> = 0) 2 Activate the server request for a user agreement indication (<Event>= 1, 2 and 3) 4 Activate the authentication indications (<Event> = 4 and 5) 8 Activate the session start indication (<Event> = 6, 7 and 8) 16 Activate the package download indications (<Event> = 9,10 and 11) 32 Activate the certified downloaded package indication (<Event> = 12 and13) 64 Activate the update indications (<Event> = 14,15 and 16) 128 Activate the fallback indication (<Event> = 17) 256 Activate download progress indication (<Event> = 18) 512 Reserved 1024 Reserved 2048 Activate provisioning indication (<Event> = 21) 4096 Reserved <Event> 0 Device services are initialized and can be used. Devices services are initialized when the SIM PIN code is entered and a dedicated NAP is configured (see +WDSS command) 1 The Device Services server requests the device to make a connection. The device requests a user agreement to allow the embedded module to make the connection. The response can be sent using +WDSR command and this indication can be returned by the device if the user has activated the user agreement for connection (see +WDSC command for more information) 2 The Device Services server requests the device to make a package download. The device requests a user agreement to allow the embedded module to make the download. The response can be sent using +WDSR command and this indication can be returned by the device if the user has activated the user agreement for download (see +WDSC command for more information). 3 The device has downloaded a package. The device requests a user agreement to install the downloaded package. The response can be sent using +WDSR command and this indication can be returned by the device if the user has activated the user agreement for install (see +WDSC command for more information). 4 The embedded module starts sending data to the server 5 Authentication with the server failed 6 Authentication has succeeded, a session with the server started 7 Session with the server failed 8 Session with the server is finished 9 A package is available on the server and can be downloaded by the embedded module. A <Data> parameter is returned indicating the package size in kB 10 A package was successfully downloaded and stored in flash 11 An issue happens during the package download. If the download has not started (+WDSI: 9 indication was not returned), this indication indicates that there is not enough space in the device to download the update package. If the download has started (+WDSI: 9 indication was returned), a flash problem implies that the package has not been saved in the device 12 Downloaded package is certified to be sent by the AirPrime Management Services server 13 Downloaded package is not certified to be sent by the AirPrime Management Services server 14 Update will be launched 15 OTA update client has finished unsuccessfully 16 OTA update client has finished successfully 17 A fallback mechanism was launched 18 Download progress. This event is returned without <Data> parameter to indicate that a download starts. During the download, a percentage progress is indicated in <Data> parameter 19 Reserved 20 Reserved 21 A provision was made by the AirPrime Management Services server 22 Reserved <Data> Specific data for some <Event> For<Event>=9, <Data> indicates the package size in bytes, which will be downloaded For<Event>=17, <Data> indicates if the fallback was asked by the user or applied because a recovery was necessary 0 Automatic recovery (a recovery mechanism was made) 1 Fallback asked by the user (see +WDSF for more information) For<Event>=18, <Data> indicates the download progress in percentage For<Event>=21, <Data> indicates the provisioned parameters 0 Reserved 1 Reserved 2 Reserved 3 Reserved 4 Reserved 5 Reserved 6 Reserved 7 Reserved 8 Reserved 9 Device Service polling mode (see +WDSC command for more information) 10 Reserved 11 Reserved 12 Reserved 13 Reserved |
| Unsolicited Notification | Response +WDSI: <Event>[,<Data>] |
| Notes | - This command is available when the embedded module has finished its initialization. - To receive +WDSI indications, Device Services should be activated (see +WDSG for more information). - <Level> is stored in non-volatile memory. The default value can be restored using AT&F. - When the AVMS status is updated, the +WDSI unsolicited response will be displayed according to the AVMS status change at the same time. If there is a power loss when the AVMS status is updating but it was updated successfully, the +WDSI unsolicited response may be lost. |
| Examples | AT+WDSI=? +WDSI: (0-2047) OK AT+WDSI? +WDSI: 0 // All indications are deactivated OK AT+WDSI=207 OK +WDSI: 1 // The devices services server requests a connection to the // embedded module AT+WDSR=1 // Accept the connection OK +WDSI: 4 // The embedded module will send the first data to the // AirPrime Management Services server +WDSI: 6 // The authentication succeeded +WDSI: 8 // The session with the server is over +WDSI: 9,1000 // A package will be downloaded, the size is 1kbytes +WDSI: 18,“1%” // 1% was downloaded +WDSI: 18,“100%” // The whole package was downloaded +WDSI: 10 // The whole package was stored in flash |

## +WDSR Command: Device Services Reply

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+WDSR=? | Response +WDSR: (list of supported <Reply>s),(list of supported <Timer>s) OK |
| Write command Syntax AT+WDSR= <Reply> [,<Timer>] | Response OK or +CME ERROR <err> Parameters <Reply> Reply to user agreement request 0 Delay or refuse the connection to the server 1 Accept the connection to the server 2 Delay or refuse the download 3 Accept the download 4 Accept the install 5 Delay the install <timer> 0 – 1440 Timer (in minutes) until a new user agreement request is returned by the module. This parameter is only available for <Reply>=0, 2 or 5. Default value = 30. Value 0 indicates that the application refuses the user agreement (impossible when <Reply>=5). |
| Notes | - This command is available when the embedded module has finished the Device Services initialization (see +WDSI) and when AVMS services are activated (see +WDSG) - It is impossible to refuse an install request (AT+WDSR=5,0); this will return +CME ERROR: 3. - After an install delay if the embedded module is powered down until after the delay, it is not powered on and the new user agreement request should be returned at the newt start up. |
| Examples | AT+WDSR=? +WDSR: (0-5),(0-1440) OK +WDSI: 1 //The device Services server requests the device to make a connection // to the server. The user is requested to allow the connection. AT+WDSR=1 OK +WDSI: 3 //A user agreement is requested to install a package AT+WDSR=5,10 //A delay of 10 minutes is requested OK +WDSI: 3 //10 minutes later, a new user agreement is requested to install a //package AT+WDSR=4 //The install is requested OK |

## +WDSS Command: Device Services Session

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+WDSS=? | Response +WDSS: 0,(Max length for <Apn>),(Max length for <User>),(Max length for <Pwd>),(list of supported <Cid>s) [+WDSS: 1,(list of supported <Action>s for this <Mode>)] OK |
| Read command Syntax AT+WDSS? | Response [+WDSS: 0,<Apn>[,<User>],<Cid>] [+WDSS: 1,<Action>] OK |
| Write command Syntax For <Mode>=0: AT+WDSS= <Mode>[,<Apn> [,<User> [,<Pwd>[,<Cid>] ]]] For <Mode>=1 AT+WDSS= <Mode>, <Action> | Response OK or +CME ERROR <err> Parameters <Mode> Integer type 0 PDP context configuration for Device Services 1 User initiated connection to the Device services server <Apn> Access Point Name for Devices Services. String type up to 50 characters. For empty string, see <Cid> <User> Login for the APN. String type, up to 30 characters <Pwd> Password for the APN. String type, up to 30 characters <Cid> 1 – 5 Context ID used for AVMS PDP activation For connection to the server: If the PDP of <Cid> has already been activated: - when <Apn> is set as an empty string, AVMS connection will directly reuse the PDP of that <Cid>, or - when <Apn> is set as a non-empty string, it will check if <Apn> matches with +CGDCONT settings to reuse the connection Otherwise, it will activate with APN <Apn>. <Action> For <Mode>=1 only 0 Release the current connection to the Device Services Server 1 Establish a connection to the Device Services Server |
| Notes | - This command is available when the embedded module has finished the Device Services initialization (see +WDSI). - <Apn>, <User>, <Pwd> and <Cid> parameters are automatically stored in non-volatile memory. AT&F has no effect on these parameters. - AT+WDSS? command only returns OK if no APN is defined. - When a request is sent to the embedded module to resume a non-existent or unsuspended session, +CME ERROR: 3 is returned. - When a request is sent to the embedded module to release a non-existent session, +CME ERROR: 3 is returned. - Depending on +WDSM configuration, when no dedicated NAP is defined using +WDSS command and a session is asked (by AT command or notify by SMS), the embedded module will use a NAP defined by +CGDCONT command to activate the dedicated PDP context. This NAP will be recorded to configure the NAP Device Services and it will be used to activate the dedicated PDP context for the next sessions. - When the PDP context cannot be activated because of bad AirVantage Management Services NAP configuration, the embedded module will use a NAP defined by +CGDCONT command to activate the dedicated PDP context (but the initial NAP configuration is not erased). - Activation is done if the embedded module is registered on the network. If the embedded module is not registered when the command is performed, activation will be done at the next network registration (even if the embedded module resets). - No GPRS connection to the AirVantage Management Services server is possible when a registration is not completed. - If reuse of existing activated PDP context is required for all internet connections, set the <Cid> accordingly. For example, in LTE, if the internet connection uses PDP of cid1, then <Cid> should be 1. - AT+WDSS=0 will remove all stored information (<Apn>, <User>, <Pwd> and <Cid>). <Apn> will become unprovisioned, but not an empty string. |
| Examples | AT+WDSS? OK //No APN defined AT+WDSS=? +WDSS: 0, 50,30,30,(1-5) +WDSS: 1,(0-1) OK AT+WDSS=0,“Sierra Wireless”,,,5 //Define the APN for the Device Services; // contect ID = 5 OK AT+WDSS? +WDSS: 0,“Sierra Wireless”,,5 +WDSS: 1,0 OK AT+WDSS=1,1 //Initiation of a connection to the Device Services server OK AT+WDSS=1,0 //Release connection to the Device Services server OK // Example for LTE with only one PDP context is allowed at+cgdcont? +CGDCONT: 1,"IP","broadband","10.191.8.184",0,0,0,0,0,0 OK at+wdss=0,”broadband”,,,1 OK at+wdss? +WDSS: 0,"broadband",,1 +WDSS: 1,0 at+cgact? +CGACT: 1,1 OK at+wdss=1,1 OK +WDSI: 4 +WDSI: 8 at+cgdcont? +CGDCONT: 1,"IP","broadband","10.191.8.184",0,0,0,0,0,0 OK //Example for reusing activated PDP at+cgdcont? +CGDCONT: 1,"IP","broadband","10.191.8.184",0,0,0,0,0,0 OK at+wdss=0 //Clear all setting OK at+wdss? OK at+wdss=0,,,,1 //Define empty string APN OK at+wdss? +WDSS: 0,"",,1 +WDSS: 1,0 OK at+wdss=1,1 //Reuse activated PDP of cid 1 for connection OK +WDSI: 4 +WDSI: 8 |

## +WDSM Command: Manage Device Services

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+WDSM=? | Response +WDSM: (list of supported <Mode>s),(list of supported <State>s) OK |
| Read command Syntax AT+WDSM? | Response +WDSM: 0,<State> +WDSM: 1,<State> OK |
| Write command Syntax AT+WDSM= <Mode>,<State> | Response OK or +CME ERROR <err> Parameters <Mode> APN backup 0 If AVMS APN (filled with +WDSS command) is incorrect, the module will use the APN defined by +CGDCONT command. 1 If AVMS APN has not been filled with +WDSS command, the module will use the APN defined by +CGDCONT command. Each APN will be used until successful session activation. If an AVMS session succeeds, the corresponding APN is copied in the +WDSS command and remains after the AVMS session ends. <State> Status of <Mode> 0 Disable (default value) 1 Enable (not supported) |
| Reference Sierra Wireless Proprietary | Notes <State> is automatically stored in non-volatile memory. AT&F command has no impact on these values. |
| Examples | AT+WDSM=? +WDSM: (0-1),(0) OK AT+WDSM? +WDSM: 0,0 +WDSM: 1,0 OK // all modes are activated AT+WDSM=0,0 OK AT+WDSM? +WDSM: 0,0 +WDSM: 1,0 OK |

# Test Commands

The following commands are used for testing purposes.

## +WMTXPOWER Command: Test RF Tx

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ WMTXPOWER=? | Response +WMTXPOWER: (list of supported <ENABLE>s),(list of supported 2G <BAND>s),(list of supported 2G <CHANNEL>s),(list of supported 2G <POWER_LEVEL>s),(list of supported <MULTISLOT>s) +WMTXPOWER: (list of supported <ENABLE>s),(list of supported 3G <BAND>s),(list of supported 3G <CHANNEL>s),(list of supported 3G <POWER_LEVEL>s) +WMTXPOWER: (list of supported <ENABLE>s),(list of supported 4G <BAND>s),(list of supported 4G <CHANNEL>s), (list of supported 4G <POWER_LEVEL>s),(list of supported <BANDWIDTH>s) OK |
| Read command Syntax AT+ WMTXPOWER? | Response For GSM: +WMTXPOWER: <ENABLE>[,<BAND>,<CHANNEL>,<POWER_LEVEL>, <MULTISLOT>] OK For UMTS: +WMTXPOWER: <ENABLE>[,<BAND>,<CHANNEL>,<POWER_LEVEL>] OK For LTE: +WMTXPOWER: <ENABLE>[,<BAND>,<CHANNEL>,<POWER_LEVEL>, <BANDWIDTH>] OK Note that parameters <BAND>, <CHANNEL>, <POWER_LEVEL>, <MULTISLOT> and <BANDWIDTH> are only available if <ENABLE>=1. |
| Write command Syntax AT+ WMTXPOWER= <ENABLE>, [,<BAND>, <CHANNEL>, <POWER_ LEVEL>, [<MULTISLOT>][,<BANDWIDTH>]] | Response OK Parameters <ENABLE> 0 Stop the burst emission 1 Start the burst emission <BAND> Tx burst band emission. This is a mandatory parameter if <ENABLE>=1, but is not allowed if <ENABLE>=0. For GSM: 900 GSM900 band 1800 DCS band For UMTS: 1 Band I (2100 band) 2 Band II (1900 band) 5 Band V (850 band) 8 Band VIII (700 band) For LTE: 2 PCS 3 DCS 4 AWS 5 CLR 8 E-GSM 12 Lower SMH blocks A/B/C 13 Upper SMH block C 17 Lower SMH blocks B/C 20 EU Digital Dividend 28 APT <CHANNEL> Tx burst channel emission. This is a mandatory parameter if <ENABLE>=1, but is not allowed if <ENABLE>=0. For GSM: If <BAND>=900 0 – 124, 975 – 1023 If <BAND>=1800 512 – 885 For UMTS: If <BAND>=1 9612 – 9888 If <BAND>=2 9262 – 9538, 12, 37, 62, 87, 112, 137, 162, 187, 212, 237, 262, 287 If <BAND>=5 4132 – 4233, 782, 787, 807, 812, 837, 862 If <BAND>=8 2712 – 2863 For LTE: If <BAND>=2 18600 – 19199 If <BAND>=3 19200 – 19949 If <BAND>=4 19950 – 20399 If <BAND>=5 20400 – 20649 If <BAND>=8 21450 – 21799 If <BAND>=12 23010 – 23179 If <BAND>=13 23180 – 23279 If <BAND>=17 23730 – 23849 If <BAND>=20 24150 – 24449 If <BAND>=28 27210 – 27659 <POWER_LEVEL> Tx burst power. This is a mandatory parameter if <ENABLE>=1, but is not allowed if <ENABLE>=0. Range: 5 (33 dBm) to 19 (5 dBm) for GSM900 bands 0 (30 dBm) to 15 (0 dBm) for GSM1800 bands 0 (0 dBm) to 384 (24 dBm) for all UMTS bands 0 (0 dBm) to 368 (23 dBm) for all LTE bands <MULTISLOT> Defines the slot used in Tx burst emissions. This parameter is not allowed if <ENABLE>=0 and is only applicable with GSM bands. 0 Emit on one-time slot (GSM) 1 Emit on two-time slots (GPRS compliant) 2 Emit on three-time slots 3 Emit on four-time slots <BANDWIDTH> Defines the bandwidth of Tx burst emissions. This parameter is not allowed if <ENABLE>=0 and is only applicable with LTE bands. 0 1.4MHz 1 3 MHz 2 5 MHz 3 10 MHz 4 15 MHz 5 20 MHz |
| Reference Sierra Wireless Proprietary | Notes - Burst must be sent on all TDMA frames. - Only one burst can be emitted at a time. - This AT command is not available if AT+WMRXPOWER is enabled. - The module must be restarted after using this command. |
| Examples | // Using an HL7690 module: at+wmtxpower=? +WMTXPOWER: (0-1),(3,8,20),(19200-19949,21450-21799,24150-24449),(0-368),(0-5) OK at+wmtxpower=1,3,19575,0,3 // Tx burst is emitted at Uarfcn 19575 OK at+wmtxpower=0 OK at+wmtxpower=1,8,21799,0,3 // Tx burst is emitted at Uarfcn 21799 OK at+wmtxpower=0 OK // Using an HL7692 module: at+wmtxpower=? +WMTXPOWER: (0-1),(900,1800),(0-124,975-1023,512-885),(5-19,0-15),(0-3) +WMTXPOWER: (0-1),(3,8,20),(19200-19949,21450-21799,24150-24449),(0-368),(0-5) OK at+wmtxpower=1,900,0,19,0 // A Tx burst is emitted at Earfcn 0 OK at+wmtxpower=1,1800,512,15,0 // A Tx burst is emitted at Earfcn 512 OK // Using an HL7648 module: at+wmtxpower=? +WMTXPOWER: (0-1),(2,4,12),(18600-19199,19950-20399,23010-23179),(0-368),(0-5) OK at+wmtxpower=1,2,18600,0,0 // A Tx burst is emitted at Earfcn 18600 OK at+wmtxpower=0 OK // Using an HL7650 module: at+wmtxpower=? +WMTXPOWER: (0,1),(2,5),(9262-9538,12,37,62,87,112,137,162,187,212,237,262,287, 4132-4233,782,787,807,812,837,862),(0-384) +WMTXPOWER: (0-1),(2,4,5,13,17),(18600-19199,19950-20399,20400-20649,23180-23279,23730-23849),(0-368),(0-5) +WMTXPOWER: (0-1),(2,4,12,17),(18600-19199,19950-20399,23010-23179,23730-23849),(0-368),(0-5) OK at+wmtxpower=1,2,9262,0 // A Tx burst is emitted at Uarfcn 9262 OK at+wmtxpower=0 OK at+wmtxpower=1,2,18600,0,0 // A Tx burst is emitted at Earfcn 18600 OK at+wmtxpower=0 OK |

## +WMRXPOWER Command: Test RF Rx

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+ WMRXPOWER=? | Response +WMRXPOWER: (list of supported <ENABLE>s),(list of supported 2G <BAND>s),(list of supported 2G <CHANNEL>s) +WMRXPOWER: (list of supported <ENABLE>s),(list of supported 3G <BAND>s), (list of supported 3G <CHANNEL>s) +WMRXPOWER: (list of supported <ENABLE>s),(list of supported 4G <BAND>s), (list of supported 4G <CHANNEL>s) OK |
| Read command Syntax AT+ WMRXPOWER? | Response +WMRXPOWER: <ENABLE>[,<BAND>,<CHANNEL>,[<EXP_POWER>]] OK Note that parameters <BAND>, <CHANNEL> and <EXP_POWER> are only available if <ENABLE>=1. <EXP_POWER> is only applicable for GSM. |
| Write command Syntax AT+ WMRXPOWER= <ENABLE> [,<BAND>, <CHANNEL>, [<EXP_POWER>]] | Response For GSM: +WMRXPOWER: <POWER1> OK For UMTS and LTE: +WMRXPOWER: <POWER1>,<POWER2> OK Parameters <ENABLE> 0 Stop the Rx measurement 1 Start the Rx measurement <BAND> Rx band to read. This is a mandatory parameter if <ENABLE>=1, but is not allowed if <ENABLE>=0. For GSM: 900 GSM900 band 1800 DCS band For UMTS: 1 Band I (2100 band) 2 Band II (1900 band) 5 Band V (850 band) 8 Band VIII (700 band) For LTE: 2 PCS 3 DCS 4 AWS 5 CLR 8 E-GSM 12 Lower SMH blocks A/B/C 13 Upper SMH block C 17 Lower SMH blocks B/C 20 EU Digital Dividend 28 APT <CHANNEL> Rx channel to read. This is a mandatory parameter if <ENABLE>=1, but is not allowed if <ENABLE>=0. For GSM: If <BAND>=900 0 – 124, 975 – 1023 If <BAND>=1800 512 – 885 For UMTS: If <BAND>=1 10562 – 10838 If <BAND>=2 9662 – 9938 If <BAND>=5 4357 – 4458 If <BAND>=8 2937 – 3088 For LTE: If <BAND>=2 600 – 1199 If <BAND>=3 1200 – 1949 If <BAND>=4 1950 – 2399 If <BAND>=5 2400 – 2649 If <BAND>=8 3450 – 3799 If <BAND>=12 5010 – 5179 If <BAND>=13 5180 – 5279 If <BAND>=17 5730 – 5849 If <BAND>=20 6150 – 6449 If <BAND>=28 9210 – 9659 <EXP_POWER> Expected power in dBm. This is a mandatory parameter if <ENABLE>=1, but is not allowed if <ENABLE>=0. Note that this parameter is only applicable for GSM. <POWER1> Received power at primary antenna in dBm <POWER2> Received power at secondary antenna in dBm |
| Reference Sierra Wireless Proprietary | Examples // Using an HL7690 module: at+wmrxpower=? +WMRXPOWER: (0-1),(3,8,20),(1200-1949,3450-3799,6150-6449) OK at+wmrxpower=1,3,1575 // Read Uarfcn 1575 +WMRXPOWER: -80.0,-101.2 // Rx power -80.0 dBm at primary antenna // Rx power -101.2 dBm at diversity antenna OK at+wmrxpower=1,8,3625 // Read Uarfcn 3625 +WMRXPOWER: -88.8,-98.2 // Rx power -88.8 dBm at primary antenna // Rx power -98.2 dBm at diversity antenna OK // Using a HL7692 module: at+wmrxpower=? +WMRXPOWER: (0-1),(900,1800),(0-124,975-1023,512-885) +WMRXPOWER: (0-1),(3,8,20),(1200-1949,3450-3799,6150-6449) OK at+wmrxpower=1,900,0,-45 // Read Earfcn 0 +WMRXPOWER: -46.8 // Rx power -46.8 dBm at primary antenna OK at+wmrxpower=1,1800,512,-45 // Read Earfcn 512 +WMRXPOWER: -46.4 // Rx power -46.4 dBm at primary antenna OK // Using an HL7648 module: at+wmrxpower=? +WMRXPOWER: (0-1),(2,4,12),(600-1199,1950-2399,5010-5179) OK at+wmrxpower=1,4,1950 // Read Earfcn 1950 +WMRXPOWER: -95.0,-108.8 // Rx power -95.0 dBm at primary antenna // Rx power -108.8 dBm at secondary antenna OK // Using an HL7650 module: at+wmrxpower=? +WMRXPOWER: (0,1),(2,5),(9662-9938,4357-4458) +WMRXPOWER: (0-1),(2,4,5,13,17),(600-1199,1950-2399,2400-2649,5180-5279,5730-5849) +WMRXPOWER: (0-1),(2,4,12,17),(600-1199,1950-2399,5010-5179,5730-5849) OK at+wmrxpower=1,2,9662 // Read Uarfcn 9662 +WMRXPOWER: -97.9,-103.8 // Rx power -97.9 dBm at primary antenna // Rx power -103.8 dBm at secondary antenna OK at+wmrxpower=1,2,600 // Read Earfcn 600 +WMRXPOWER: -95.0,-108.8 // Rx power -95.0 dBm at primary antenna // Rx power -108.8 dBm at secondary antenna OK |

# NV Related Commands

## Auto Generation of NV Backup Files

There are 3 NV partitions in flash used by the firmware:

Static Calibrated NV partition

Static Fixed NV partition

Dynamic NV partition

NV backup is per partition based, with one NV backup file per partition. These are labelled with <file id>=0, 1, 2 in the NV log and by firmware design.

The firmware automatically generates NV backup files from existing NV data at ~8 seconds after boot if one of the following conditions are met:

NV backup of a partition does not exist, or it has been corrupted unexpectedly.

NV backup files exist, but the firmware version has changed while IMEI has not changed, in comparison to the records in the backup file.

NV backup files exist, but the firmware version has changed and a valid IMEI has been updated, in comparison to the records in the backup file.

An automatic backup file generation is notified with +NVBU_IND with <status>=0 on all AT ports.

## Auto Recovery from Backup NV Files

NV recovery is automatically done if an NV corruption is detected during NV initialization at boot.

The firmware automatically recovers NV data from available NV backups when one or more NV items are corrupted. This is notified with +NVBU_IND with <status>=3 on all AT ports.

Manual NV data restores all data from backup file to the original NV partition.

The firmware will try to recover corrupted or missing NV data items instead of all NV data items (partial restore) if possible; otherwise, the firmware restores all NV data items (full restore).

If the firmware crashes with 10 consecutive loops and a full restore has not been performed before, the firmware performs a full restore of all NV data items. Only consecutive crashes that happened within 8 seconds after the module boots is counted for this reset loop detection.

## +NVBU Command: NV Backup Status and Control

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+NVBU=? | Response +NVBU: (0-2) OK |
| Read command Syntax AT+NVBU? | Returns list of NV backup with the format: +NVBU: <file id>,<backup date>,<backup firmware version> Response [+NVBU: 0,<backup date>,<backup firmware version>] [+NVBU: 1,<backup date>,<backup firmware version>] [+NVBU: 2,<backup date>,<backup firmware version>] OK Parameters <file id> Backup file ID corresponding to an NV partition in flash <backup date> NV backup generation date <backup firmware version> Firmware version used to generate the NV backup |
| Write command Syntax For <mode> = 0 or 1 AT+NVBU= <mode> [,<parti_id>] For <mode> = 2 AT+NVBU= <mode>[,<clear>] | Response For <mode>=0 or 1 OK For <mode>=2 and <clear>=0 <log data 0> [<log data 1>] … [<log data n>] OK For <mode>=2 and <clear>=1 OK Parameters <mode> 0 Generate backup of all NV data to NV backup partition 1 Restore all NV data from the NVM backup partition 2 List logs of NV backup operations <log data> NV backup operations log data <parti_id> 0 Static Calibrated NV 1 Static Fixed NV partition 2 Dynamic NV partition 3 All NV partitions <clear log> 0 Read log 1 Clear log |
| Reference Sierra Wireless Proprietary | Notes - Status of operations for <mode>=0 and <mode>=1 is notified by +NVBU_IND unsolicited notifications with <status>=0 and <status>=1 respectively on the AT port that executed the write command. - Execution of the write command with <mode>=1 is followed by a modem reboot automatically; NVs are restored to their default values on booting. - The number of lines of <log data> ranges from 1 to 2142 lines. - This command can be used without a SIM. - <mode>=2 is for retrieving log for R&D analysis and not fully documented, generally: - USER=0 for operations triggered by the firmware - USER=1 for manual operations |
| Example | # automatic backup files generation after FW upgrade, notified by +NVBU_IND +NVBU_IND: 0,0,"2015/11/16 04:23:33","BHL7618.3.0.154401.201511132200.x7120_2" +NVBU_IND: 0,1,"2015/11/16 04:23:33","BHL7618.3.0.154401.201511132200.x7120_2" +NVBU_IND: 0,2,"2015/11/16 04:23:33","BHL7618.3.0.154401.201511132200.x7120_2" # manual generation of backup files from existing NV partitions AT+NVBU=0,3 OK +NVBU_IND: 0,0,"2015/11/16 04:23:39","BHL7618.3.0.154401.201511132200.x7120_2" +NVBU_IND: 0,1,"2015/11/16 04:23:39","BHL7618.3.0.154401.201511132200.x7120_2" +NVBU_IND: 0,2,"2015/11/16 04:23:39","BHL7618.3.0.154401.201511132200.x7120_2" # manual restore of backup files to original NV partitions AT+NVBU=1,3 OK +NVBU_IND: 1,0,"2015/11/16 04:23:39","BHL7618.3.0.154401.201511132200.x7120_2" +NVBU_IND: 1,1,"2015/11/16 04:23:39","BHL7618.3.0.154401.201511132200.x7120_2" +NVBU_IND: 1,2,"2015/11/16 04:23:39","BHL7618.3.0.154401.201511132200.x7120_2" <module reboots automatically> # to retrieve the list of NV related operations done by the Firmware at+nvbu=2 [2015/11/16 04:02:49] BULO: MDM- BHL7618.3.0.154401.201511132200.x7120_2 [2015/11/16 04:02:49] BUFL: GENERATE USER=0 FILE=3 LAS=0,0,0 [2015/11/16 04:02:49] BUFM: ENCODE F=0 REF=0 CNT=15/15 41 [2015/11/16 04:02:49] BUFM: ENCODE F=1 REF=0 CNT=16/16 31 [2015/11/16 04:02:49] BUFM: ENCODE F=2 REF=42 CNT=41/41 57 [2015/11/16 04:23:39] BUFL: GENERATE USER=1 FILE=3 LAS=0,0,0 [2015/11/16 04:23:39] BUFM: ENCODE F=0 REF=0 CNT=15/15 41 [2015/11/16 04:23:39] BUFM: ENCODE F=1 REF=0 CNT=16/16 31 [2015/11/16 04:23:39] BUFM: ENCODE F=2 REF=42 CNT=41/41 57 [2015/11/16 04:23:43] BUFL: RESTORE USER=1 FILE=3 LAS=0,0,0 [2015/11/16 04:23:43] BUFM: DECODE-2 F=0 REF=1 CNT=15/15 15,41 [2015/11/16 04:23:43] BUFM: DECODE-2 F=1 REF=1 CNT=16/16 16,31 [2015/11/16 04:23:43] BUFM: DECODE-2 F=2 REF=43 CNT=41/41 41,57 OK |

## +NVBU_IND Notification: NV Backup Status Notification

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Unsolicited Notification | Response +NVBU_IND: <status>,<file id>,… For <status>=0 +NVBU_IND: <status>,<file id>,<backup date>,<backup firmware version> For <status>=1 +NVBU_IND: <status>,<file id>,<backup date used for restore>,<backup firmware version used for restore> For <status>=2 +NVBU_IND: <status>,<file id>,<backup date used for restore>,<backup firmware version used for restore>,<num NV> <NV ID 1>[<NV ID 2>...[<NV ID 16><CR><LF>]] … Parameters <status> NV backup status 0 Indicates completion of NV backup generation 1 Indicates completion of NV backup restore 2 Indicates that backup data were restored when the NV corruption was detected during NV initialization <backup date> NV backup generation date <backup firmware version> Firmware version used to generate the NV backup <backup date used for restore> Generation date of the NV backup that was used for the NV restore <backup firmware version used for restore> Firmware version used to generate the NV backup that was used for the NV restore <num NV> Total number of NV items restored <NV ID> List of NV item IDs with data restored, expressed in hexadecimal number delimited by spaces, and delimited by <CR><LF> every 16 numbers. |
| Reference Sierra Wireless Proprietary | Notes The list of <NV ID> is expressed in 16 hexadecimal numbers per line. |
| Examples | - # recovery in calibrated NV partition after Firmware boot - # note that the data is also logged by NV log (i.e. AT+NVBU=2) - +NVBU_IND: 2,0,"2015/11/16 04:23:39","BHL7618.3.0.154401.201511132200.x7120_2",15 - 10034900 10034901 10034401 10034402 10034902 10035400 10035401 10035402 10035403 10035500 10035501 10035502 10050000 10310000 10370000 |

# Board Support Commands

## +WCARRIER Command: Show Carrier Name

| HL7618, HL7618RD, HL7648, HL7650, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+WCARRIER=? | Response OK |
| Action command Syntax AT+WCARRIER | Response +WCARRIER: <Carrier Name> OK Parameter <Carrier Name> Carrier string (maximum of 8 characters, without quotes) |
| Notes | The carrier name is written in non-volatile memory during the factory customization process. |
| Example | at+wcarrier +WCARRIER: VZW // when using a module that’s on the Verizon network OK at+wcarrier +WCARRIER: AT&T // when using a module that’s on the AT&T network OK |

# M2M Service Optimization Commands

All commands in this section are for the HL7650 only.

## +MSOSTATUS Command: Operating Status

| HL7650 |  |
| --- | --- |
| Test command Syntax AT+MSOSTATUS=? | Response OK |
| Read command Syntax AT+MSOSTATUS? | Response +MSOSTATUS: "SC","All","ACCEPT","","" +MSOSTATUS: "SC","LTE","ACCEPT","","" +MSOSTATUS: "SC","USSD","ACCEPT","","" +MSOSTATUS: "SC","Reset","ACCEPT","","" +MSOSTATUS: "SC","SMS","ACCEPT","","" +MSOSTATUS: "SC","PDP","RETRY","","" +MSOSTATUS: "RULE","RETRY","TRUE","" +MSOSTATUS: "COND","NETEVT","TRUE","PDP",">=",5,"COUNT:0,5,0,0,0" +MSOSTATUS: <status> OK |
| Write command Syntax AT+MSOSTATUS=<status> | Response OK Parameters <status> 0 MSO disabled 1 MSO enabled |
| Notes | The MSO operating status is stored in non-volatile memory. |
| Examples | AT+MSOSTATUS=0 // Disables the MSO module OK AT+MSOSTATUS=1 // Enables the MSO module OK |

## +MSORTCSTATUS Command: Display Trust RTC Status

| HL7650 |  |
| --- | --- |
| Test command Syntax AT+ MSORTCSTATUS=? | Response OK |
| Read command Syntax AT+ MSORTCSTATUS? | Response +MSORTCSTATUS: <status> OK |
| Write command Syntax AT+ MSORTCSTATUS=<status> | Response OK Parameter <status> 0 MSO gets local time using RTC, and NITZ time zone if available 1 MSO gets local time from the RTC |
| Notes | - Local time is required in MSO for ToD rules and rules with fixed window network events. If the local time is not available, then all ToD rules and rules with fixed window network events are ignored. - The MSO RTC status is stored in non-volatile memory. |
| Examples | AT+MSORTCSTATUS=1 // MSO uses the RTC for local time OK AT+MSORTCSTATUS=0 // MSO uses NITZ for local time (if available) OK |

## +MSOPOLICY Command: Update MSO Policies

| HL7650 |  |
| --- | --- |
| Test command Syntax AT+MSOPOLICY=? | Response OK |
| Read command Syntax AT+MSOPOLICY? | Response +MSOPOLICY: <policy data> OK |
| Write command Syntax AT+MSOPOLICY=<mode> | Response <policy data><CTRL-Z> OK Parameter <mode> 1 Update MSO policy |
| Notes | - Previously written policies to the device are erased during this operation. Policies are generated using the MSO Policy Editor Tool (MPET). Additionally, note that MSO is disabled while the policy is being updated. An MSO policy update will enable the MSO module status after the operation completes successfully; else, MSO will clear the policies on the device and disable the MSO module status. The device requires a reset after a new MSO policy is written to it. - The maximum policy size is based on the maximum sizes of all the SCs, rules and schedules. - The MSO policy status is stored in non-volatile memory. |
| Example | AT+MSOPOLICY=1 // MSO updates the policy and starts executing b4f1b8df0002010017020003010a010a1401141e010103010a030a1401141e010201f301000100012a1100010000010401c002000000000001000105000200030005000600080009001600160022002201010000010401c00100000000000100010300020003000600060008000902010001010401c00100000000000100010300070007001000100016001603010002010401c00000000000000100010400080008001a001b001d00230026002604010001010401c000000000000001000101001c001c05010002010401c000000000000001000201002700270601000402010900000401c002000000000001000101000b000b0701000602010900000401c001000000000001000102000b000b000e000e0801000402010a00000401c002000000000001000102000c000c000d000d0901000602010a00000401c001000000000001000102000c000c000d000d0a01000402010b00000401c002000000000001000101000f000f0b01000602010b00000401c001000000000001000101000f000f0c010007010401c00400000000000100010a00080008000a000a00150016001c001e002600260029002a002f002f0032003200450045005100510d010007010401c00400000000000100010200110011001500150e02010401c000000000000001000101002200220f01010001040190020000000000010002010011001110010101010401900100000000000100020100110011<CTRL-Z> OK AT+MSOPOLICY? // MSO returns the current policy data +MSOPOLICY: b4f1b8df0002010017020003010a010a1401141e010103010a030a1401141e010201f301000100012a1100010000010401c002000000000001000105000200030005000600080009001600160022002201010000010401c00100000000000100010300020003000600060008000902010001010401c00100000000000100010300070007001000100016001603010002010401c00000000000000100010400080008001a001b001d00230026002604010001010401c000000000000001000101001c001c05010002010401c000000000000001000201002700270601000402010900000401c002000000000001000101000b000b0701000602010900000401c001000000000001000102000b000b000e000e0801000402010a00000401c002000000000001000102000c000c000d000d0901000602010a00000401c001000000000001000102000c000c000d000d0a01000402010b00000401c002000000000001000101000f000f0b01000602010b00000401c001000000000001000101000f000f0c010007010401c00400000000000100010a00080008000a000a00150016001c001e002600260029002a002f002f0032003200450045005100510d010007010401c00400000000000100010200110011001500150e02010401c000000000000001000101002200220f01010001040190020000000000010002010011001110010101010401900100000000000100020100110011 OK |

## +MSORETRYINFO Command: Read Retry Information

| HL7650 |  |
| --- | --- |
| Test command Syntax AT+ MSORETRYINFO=? | Response OK |
| Read command Syntax AT+ MSORETRYINFO? | Response + MSORETRYINFO: <rule>,<cid>,<obj>,<time>,<count>,<error> <rule>,<cid>,<obj>,<time>,<count>,<error> … OK |
| Write command Syntax AT+ MSORETRYINFO=<mode>,<rule> [,<cid>] | Response OK Parameters <mode> 0 Resets given retry schedule <obj> 0 CREG (all) 1 GPRS Attach (all) 2 PDP Activation 3 CREG (manual) 4 CREG (auto) 5 GPRS Attach (manual) 6 GPRS Attach (auto) 7 MO-SMS 8 LTE Attach 9 Socket Connection 10 DNS Query |
| Notes | - This command shows all the rules with objects that currently blocked due to a retry schedule. Additionally, this command can also be used to reset any outstanding MSO retry schedules. - The MSO retry schedule states are stored in non-volatile memory. |
| Examples | AT+MSORETRYINFO? // MSO displays all active retry schedules 0,1,2,1,29,17 OK AT+MSORETRYINFO=0,0,1 // MSO uses resets given the retry schedule OK |

## +MSOMONITOR Command: Monitoring Status Control

| HL7650 |  |
| --- | --- |
| Test command Syntax AT+ MSOMONITOR=? | Response OK |
| Read command Syntax AT+ MSOMONITOR? | Response +MSOMONITOR: <mode>,<value>,<period> OK |
| Write command Syntax AT+ MSOMONITOR= <mode> [,<value>] [,<period>] | Response OK Paremeters <mode> 0 MSO monitoring disabled 1 MSO monitoring enabled <period> 0 Minutes 1 Hours 2 Days |
| Notes | - The MSO monitoring period is the time period which upon expiry all monitored values are reset to zero. - The MSO monitoring state is stored in non-volatile memory. |
| Examples | AT+MSOMONITOR=1,1,1 // MSO updates the current monitoring configuration OK AT+MSOMONITOR? +MSOMONITOR: 1,1,1 // MSO displays the current monitoring configuration OK |

## +MSOMONITORVALUE Command: Read Monitored Data

| HL7650 |  |
| --- | --- |
| Test command Syntax AT+MSO MONITORVALUE=? | Response OK |
| Write command Syntax AT+MSO MONITORVALUE=<id>[,<cid>] [,<sc>] | Response +MONITORVALUE: <value> OK Parameters <id> 0 CREG request count 1 CREG success count 2 CREG error count 3 CREG blocked count 4 CREG duration (seconds) 5 GPRS attach request count 6 GPRS attach success count 7 GPRS attach error count 8 GPRS attach blocked count 9 GPRS attach duration (seconds) 10 PDP request count 11 PDP success count 12 PDP error count 13 PDP blocked count 14 PDP duration (seconds) 15 PDP status 16 MO-SMS request count 17 MO-SMS success count 18 MO-SMS error count 19 MO-SMS blocked count 20 MO-SMS bytes sent 21 Device reset count 22 Device reset time 23 Socket send request count 24 Socket send block count 25 Socket send bytes 26 Socket receive bytes 27 LTE attach request count 28 LTE attach success count 29 LTE attach error count 30 LTE attach block count 31 LTE attach duration 32 Socket connect request count 33 Socket connect success count 34 Socket connect error count 35 Socket connect block count 36 DNS query request count 37 DNS query success count 38 DNS query error count 39 DNS query block count <cid> PDP activation ID (only applicable if the monitored value is a PDP value) <sc> Socket data assigned user-defined service class ID (only applicable if the monitored value is a socket type) |
| Note | The MSO monitored values are stored in non-volatile memory. |
| Examples | AT+MSOMONITORVALUE=0 +MONITORVALUE: 1 // MSO retrieves the current CREG request count OK AT+MSOMONITORVALUE=11,1 +MONITORVALUE: 3 // MSO retrieves the current PDP activation // success count for context ID 1 OK AT+MSOMONITORVALUE=25,16 +MONITORVALUE: 150 // MSO retrieves the current number of UL data // bytes assigned to user defined service class 16 OK |

## +MSOEVTLOGSTATUS Command: Event Log Status

| HL7650 |  |
| --- | --- |
| Test command Syntax AT+MSOEVTLOGSTATUS=? | Response OK |
| Read command Syntax AT+MSOEVTLOGSTATUS? | Response +MSOEVTLOGSTATUS: <cmd> OK |
| Write command Syntax AT+MSOEVTLOGSTATUS=<cmd> | Response OK Parameter <cmd> 0 Disable MSO event logging 1 Enable MSO event logging (no overwrite when full) 2 Enable MSO event logging (overwrite buffer) |
| Note | The MSO event log state is stored in non-volatile memory. |
| Examples | AT+MSOEVTLOGSTATUS? +MSOEVTLOGSTATUS: 1 // MSO displays the current event logging // configuration OK AT+MSOEVTLOGSTATUS=1 // MSO updates the current event logging // configuration OK |

## +MSOEVTLOGPUSH Command: Event Log Push

| HL7650 |  |
| --- | --- |
| Test command Syntax AT+MSOEVTLOGPUSH=? | Response OK |
| Read command Syntax AT+MSOEVTLOGPUSH? | Response +MSOEVTLOGPUSH: <cmd> OK |
| Write command Syntax AT+MSOEVTLOGPUSH=<cmd> | Response OK or +CME ERROR: 3 Parameter <cmd> 0 Disable MSO event log push to console 1 Enable MSO event log push to console |
| Note | The MSO event log push to console state is stored in non-volatile memory. |
| Examples | AT+MSOEVTLOGPUSH? +MSOEVTLOGPUSH: 1 // MSO displays the current event log push to // console configuration OK AT+MSOEVTLOGPUSH=1 // MSO updates the current event log push to // console configuration OK |

## +MSOEVTLOG Command: Read Event Log

| HL7650 |  |
| --- | --- |
| Test command Syntax AT+MSOEVTLOG=? | Response OK |
| Read command Syntax AT+MSOEVTLOG? | Response +MSOEVTLOG: <data> OK |
| Note | This command retrieves up to 100 of the oldest MSO event log records since the last event log record read. |
| Example | AT+MSOEVTLOG? +MSOEVTLOG: Up5LfwAAAAr////+AAAAAQAAAAEAAAAAAAAAAAAAAAAAAAAAAAAAAHdtX21zb19zb2NrQ3JlYXRlAAAAAAAAAAAAAAAAAAAA Up5LgAAAAAr////+AAAAAQAAAAEAAAAAAAAAAAAAAAAAAAAAAAAAAHdtX21zb19zb2NrQ3JlYXRlAAAAAAAAAAAAAAAAAAAA Up5LhgAAAAYAAAABAAAAAAAAAAQAAAAAAAAAAAAAAAAAAAAAAAAAAG1zb19wZHBfY3R4X2FjdGl2X3JzcAAAAAAAAAAAAAAA Up3bBwAAABtSndsHUp3bBwAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAG1zb190aW1lX3N5bmNfdXBkYXRlAAAAAAAAAAAAAAAA Up3bBwAAABEAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAG1zb19ncm91cF9hZGRyX3Jlc3RvcmUAAAAAAAAAAAAA Up3bBwAAABEAAABAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAG1zb19ncm91cF9hZGRyX3Jlc3RvcmUAAAAAAAAAAAAA Up3bBwAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAG1zb19zY19yZXN0b3JlAAAAAAAAAAAAAAAAAAAAAAAA Up3bBwAAAAAAAAADAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAG1zb19zY19yZXN0b3JlAAAAAAAAAAAAAAAAAAAAAAAA Up3bBwAAAAAAAAAFAAAAAAAAAHgAAAAAAAAAAAAAAAAAAAAAAAAAAG1zb19zY19yZXN0b3JlAAAAAAAAAAAAAAAAAAAAAAAA Up3bBwAAAAAAAAAGAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAG1zb19zY19yZXN0b3JlAAAAAAAAAAAAAAAAAAAAAAAA Up3bBwAAAAEAAAAAAAAAAAAAAAEAAAAAAAAAAQAAAAAAAAABAAAAAG1zb19uYXJfcmVzdG9yZQAAAAAAAAAAAAAAAAAAAAAA Up3bBwAAAAQAAAABAAAACwAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAG1zb19kcGlfcHJvY2Vzc19zb2NrZXRfdG9fc2MAAAAA Up3bBwAAAAQAAAAAAAAACwAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAG1zb19kcGlfcHJvY2Vzc19zb2NrZXRfdG9fc2MAAAAA OK // MSO displays up to the latest 100 base64 encoded records to the console. The results // above can be saved to a file which can then be parsed using the MSO event log // decoding python application to output a text version of the event logs. |

# Appendix

## Result Codes and Unsolicited Messages

| Verbose Result Code | Numeric | Type | Description |
| --- | --- | --- | --- |
| +CCCM: <ccm> | like verbose | Unsolicited |  |
| +CME ERROR: <err> | like verbose | Final |  |
| +CMS ERROR: <err> | like verbose | Final or unsolicited |  |
| +CMTI | like verbose | Unsolicited |  |
| +CBM | like verbose | Unsolicited |  |
| +CDS | like verbose | Unsolicited |  |
| +COLP: <number>,<type>[,<subaddr>, <satype>[,<alpha>]] | like verbose | Intermediate |  |
| +CR: <type> | like verbose | Intermediate |  |
| +CREG: <stat>[,<lac>,<ci>] | like verbose | Unsolicited |  |
| +CRING: <type> | like verbose | Unsolicited |  |
| +CSSI: <code1>[,<index>] | like verbose | Intermediate |  |
| +CSSU: <code2>[,<index>[,<number>,<type> [,<subaddr>,<satype>]]] | like verbose | Unsolicited |  |
| +CUSD: <m>[,<str>,<dcs>] | like verbose | Unsolicited |  |
| BUSY | 6 | Final |  |
| CONNECT | 1 | Intermediate | connection has been established |
| CONNECT <text> | manufacturer specific | Intermediate | like CONNECT but manufacturer specific <text> gives additional information (e.g. connection data rate) |
| ERROR | 4 | Final | command not accepted |
| NO ANSWER | 7 | Final | connection completion timeout |
| NO CARRIER | 3 | Final | connection terminated |
| NO DIALTONE | 5 | Final | no dial tone detected |
| OK | 0 | Final | acknowledges execution of a command line |
| RING | 2 | Unsolicited | incoming call signal from network |

## Error Codes

### CME Error Codes

| <err> Code | Meaning |
| --- | --- |
| 0 | Phone failure |
| 1 | No connection to phone |
| 2 | Phone-adapter link reserved |
| 3 | Operation not allowed |
| 4 | Operation not supported |
| 5 | PH-SIM PIN required |
| 6 | PH-FSIM PIN required |
| 7 | PH-FSIM PUK required |
| 10 | SIM not inserted |
| 11 | SIM PIN required |
| 12 | SIM PUK required |
| 13 | SIM failure |
| 14 | SIM busy |
| 15 | SIM wrong |
| 16 | Incorrect password |
| 17 | SIM PIN2 required |
| 18 | SIM PUK2 required |
| 20 | Memory full |
| 21 | Invalid index |
| 22 | Not found |
| 23 | Memory failure |
| 24 | Text string too long |
| 25 | Invalid characters in text string |
| 26 | Dial string too long |
| 27 | Invalid characters in dial string |
| 30 | No network service |
| 31 | Network timeout |
| 32 | Network not allowed - emergency call only |
| 40 | Network personalization PIN required |
| 41 | Network personalization PUK required |
| 42 | Network subset personalization PIN required |
| 43 | Network subset personalization PUK required |
| 44 | Service provider personalization PIN required |
| 45 | Service provider personalization PUK required |
| 46 | Corporate personalization PIN required |
| 47 | Corporate personalization PUK required |
| 48 | Hidden key required |
| 49 | EAP method not supported |
| 50 | Incorrect parameters |
| 99 | Resource limitation |
| 100 | Unknown |
| 103 | Illegal MS |
| 106 | Illega lME |
| 107 | GPRS services not allowed |
| 111 | PLMN not allowed |
| 112 | Location area not allowed |
| 113 | Roaming not allowed in this location area |
| 132 | Service option not supported |
| 133 | Requested service option not subscribed |
| 134 | Service option temporarily out of order |
| 148 | Unspecified GPRS error |
| 149 | PDP authentication failure |
| 150 | Invalid mobile class |
| 201 | Alternate SIM conflict |
| 500 | CTS Handover on Progress |
| 501 | Cellular Protocol Stack Out of service state |
| 502 | CTS Unspecified Error |
| 650 | General AVMS error |
| 651 | Communication error |
| 652 | Session in progress |
| 654 | RDMS services are in “deactivated” state |
| 655 | RDMS services are in “prohibited” state |
| 656 | RDMS services are in “to be provisioned” state; no available NAP |
| 800 | SIM Security unspecified error |
| 902 | No more sockets available; the maximum number has been reached |
| 903 | Memory problem |
| 904 | DNS error |
| 905 | TCP disconnection by the server |
| 906 | TCP/UDP connection error |
| 907 | Generic error |
| 908 | Fail to accept client request's |
| 909 | Data send by KTCPSND/KUDPSND are incoherent |
| 910 | Bad session ID |
| 911 | Session is already running |
| 912 | No more sessions can be used (maximum session is 32) |
| 913 | Socket connection timer timeout |
| 914 | Control socket connection timer timeout |
| 915 | A parameter is not expected |
| 916 | A parameter has an invalid range of values |
| 917 | A parameter is missing |
| 918 | Feature is not supported |
| 919 | Feature is not available |
| 920 | Protocol is not supported |
| 921 | Error due to invalid state of bearer connection |
| 922 | Error due to invalid state of session |
| 923 | Error due to invalid state of terminate port data mode |
| 924 | Error due to session busy, retry later |
| 925 | Failed to decode HTTP header's name, missing ':' |
| 926 | Failed to decode HTTP header's value, missing 'cr/lf' |
| 927 | HTTP header's name is an empty string |
| 928 | HTTP header's value is an empty string |
| 929 | Format of input data is invalid |
| 930 | Content of input data is invalid or not supported |
| 931 | The length of a parameter is invalid |
| 932 | The format of a parameter is invalid |

### CEER Error Codes

| <cause> | <description> |
| --- | --- |
| 0 | No cause information available |
| 1 | Unassigned (unallocated) number |
| 3 | No route destination |
| 6 | Channel unacceptable |
| 8 | Operator determined barring |
| 16 | Normal call clearing |
| 17 | User busy |
| 18 | No user responding |
| 19 | User alerting, no answer |
| 21 | Call rejected |
| 22 | Number changed |
| 26 | Non-selected user clearing |
| 27 | Destination out of order |
| 28 | Invalid number format (incomplete number) |
| 29 | Facility rejected |
| 30 | Response to STATUS ENQUIIRY |
| 31 | Normal, unspecified |
| 34 | No circuit / channel available |
| 38 | Network out of order |
| 41 | Temporary failure |
| 42 | Switching equipment congestion |
| 43 | Access information discarded |
| 44 | Requested circuit / channel not available |
| 47 | Resources unavailable, unspecified |
| 49 | Quality of service unavailable |
| 50 | Requested facility not subscribed |
| 55 | Incoming calls barred with in the CUG |
| 57 | Bearer capability not authorized |
| 58 | Bearer capability not presently available |
| 63 | Service or option not available, unspecified |
| 65 | Bearer service not implemented |
| 68 | ACM equal to or greater than AC Mmax |
| 69 | Requested facility not implemented |
| 70 | Only restricted digital information bearer capability is available |
| 79 | Service or option not implemented, unspecified |
| 81 | Invalid transaction identifier value |
| 87 | User not member of CUG |
| 88 | Incompatible destination |
| 91 | Invalid transit network selection |
| 95 | Semantically incorrect message |
| 96 | Invalid mandatory information |
| 97 | Message type non-existent or not implemented |
| 98 | Message type not compatible with protocol state |
| 99 | Information element non-existent or not implemented |
| 100 | Conditional IE error |
| 101 | Message not compatible with protocol state |
| 102 | Recovery on timer expiry |
| 103 | Illegal MS |
| 106 | Illegal ME |
| 107 | GPRS service not allowed |
| 111 | Protocol error, unspecified |
| 112 | Location area not allowed |
| 113 | Roaming not allowed in this location area |
| 124 | MBMS bearer capabilities insufficient for the service |
| 125 | LLC or SNDCP failure |
| 126 | Insufficient resources |
| 127 | Missing or unknown APN |
| 128 | Unknown PDP address or PDP type |
| 129 | User authentication failed |
| 130 | Activation rejected by GGSN |
| 131 | Activation reject, unspecified |
| 132 | Service not supported |
| 133 | Requested service option not subscribed |
| 134 | Service option temporarily out of order |
| 135 | NSAPI already used |
| 136 | Regular PDP context deactivation |
| 137 | QoS not accepted |
| 138 | Network failure |
| 139 | Reactivation requested |
| 140 | Feature not supported |
| 141 | Semantic error in the TFT operation |
| 142 | Syntactical error in the TFT operation |
| 143 | Unknown PDP context |
| 144 | Semantic errors in packet filter(s) |
| 145 | Syntactical errors in packet filter(s) |
| 146 | PDP context without TFT already activated |
| 148 | Unspecified GPRS error |
| 149 | PDP authentification error |
| 152 | Single address bearers only allowed |
| 153 | ESM information only allowed |
| 154 | ESM information not received |
| 155 | PDN connection does not exist |
| 156 | Collision with network initiated request |
| 159 | Unsupported QCI value |
| 160 | Bearer handling not supported |
| 165 | Maximum number of EPS bearers reached |
| 166 | Requested APN not supported in current RAT and PLMN combination |
| 181 | Invalid PTI value |
| 182 | APN restriction value incompatible with active EPS bearer context |
| 183 | PTI already in use |
| 184 | EPS QoS not accepted |
| 185 | Invalid EPS bearer identity |
| 186 | PTI mismatch |
| 187 | Last PDN disconnection not allowed |
| 188 | PDN type IPv4 only allowed |
| 189 | PDN type IPv6 only allowed |
| 212 | APN restriction |
| 256 | Internal unspecified |
| 257 | Out of memory |
| 258 | Invalid parameters |
| 259 | Data call active |
| 260 | Speech call active |
| 262 | Missing ACM information |
| 263 | Temporary forbidden |
| 264 | Called party is blacklisted |
| 265 | Blacklist is full |
| 266 | No service |
| 267 | Limited service |
| 268 | Client conflict |
| 269 | Dual Service call active |
| 271 | Unknown SIM error |
| 274 | Active client is gone |
| 277 | SIM status failure |
| 278 | Rejected by call control |
| 279 | FDN failed |
| 280 | BDN failed |
| 283 | CCBS possible |
| 284 | Invalid alternate service line |
| 285 | LND overview |
| 287 | MM network failure unspecified |
| 288 | MM no service |
| 289 | MM access class barred |
| 290 | MM RR no resource |
| 291 | MM ME busy |
| 292 | MM unspecified |
| 301 | MMI not registered |
| 303 | Rejected by user |
| 304 | Rejected due to time out |
| 306 | Disconnected due to SIM TK call setup |
| 307 | Pending SIM TK call setup |
| 310 | SIM reset |
| 340 | MM sapi3 release |
| 341 | MM lower layer failure |
| 342 | MM authentification failure |
| 343 | MM PS reject |
| 344 | MM service rejected |
| 345 | MM abort by network |
| 346 | MM timeout |
| 347 | MM detach |
| 348 | MM RR connection release |
| 349 | MM not registered |
| 350 | MM reestablishment failure |
| 351 | Failure due to handover |
| 352 | Link establishment failure |
| 353 | Random access failure |
| 354 | Radio link aborted |
| 355 | Lower layer failure in Layer 1 |
| 356 | Immediate assignment reject |
| 357 | Failure due to paging |
| 358 | Abnormal release unspecified |
| 359 | Abnormal release channel unacceptable |
| 360 | Abnormal release timer expired |
| 361 | Abnormal release no act on radio path |
| 362 | Preemptive release |
| 363 | UTRAN configuration unknown |
| 364 | Handover impossible |
| 365 | Channel mode unacceptable |
| 366 | Frequency not implemented |
| 367 | Originator leaving call group area |
| 368 | Lower layer failure from network |
| 369 | Call already cleared |
| 370 | Semantically incorrect message |
| 371 | Invalid mandatory info |
| 372 | Message type non-existing |
| 373 | Message type incompatible in state |
| 374 | Conditional information element error |
| 375 | No cell allocation available |
| 376 | Protocol error unspecified |
| 377 | Normal event |
| 378 | Unspecified |
| 379 | Preemptive release |
| 380 | Congestion |
| 381 | RE establishment reject |
| 382 | Directed sig conn establishment |
| 383 | User inactivity |
| 384 | Lower layer failure downlink |
| 385 | Lower layer failure uplink |
| 386 | Cell barred due to authentication failure |
| 387 | Signalling connection release |
| 388 | CS connection release triggered by MM |
| 389 | RRC connection establishment failure |
| 390 | RRC connection establishment re-ject with redirection |
| 391 | Resource conflict |
| 392 | Layer 2 sequence error |
| 393 | Layer 2 T200 exp N200 plus 1 times |
| 394 | Layer 2 unsolicited DM resp MFES |
| 395 | Layer 2 contention resolution |
| 396 | Layer 2 normal cause |
| 397 | RR connection release due to BAND change (2G) |
| 400 | MM RR connection error while release |
| 500 | User disconnected |
| 510 | Remote user / NW disconnected for call status rather than call proceeding |
| 511 | Remote user / NW disconnected for call status is call proceeding |
| 512 | Request rejected, BCM violation |

#### SS Error Codes

##### SS Network Error Cause

According to 3GPP 24.008, section 4.5 except for internal errors 0 and 255.

| <cause> | <description> |
| --- | --- |
| 0 | MN_MS_INT_SS_ERROR - This is used when the SS operation was unsuccessful due to an MS internal reason |
| 255 | MN_MS_INT_SS_TIME_OUT - This is used when the SS operation was unsuccessful due to a missing answer from the network |
| 1 | MN_UNKNOWN_SUBSCRIBER |
| 9 | MN_ILLEGAL_SUBSCRIBER |
| 10 | MN_BEARER_SERVICE_NOT_ |
| 11 | MN_TELESERVICE_NOT_PROVISIONED |
| 12 | MN_ILLEGAL_EQUIPMENT |
| 13 | SYNONYM MN_CALL_BARRED |
| 16 | MN_ILLEGAL_SS_OPERATION |
| 17 | MN_SS_ERROR_STATUS |
| 18 | MN_SS_NOT_AVAILABLE |
| 19 | MN_SS_SUBSCRIPTION_VIOLATION |
| 20 | MN_SS_INCOMPATIBILITY |
| 21 | MN_FACILITY_NOT_SUPPORTED |
| 27 | MN_ABSENT_SUBSCRIBER |
| 29 | MN_SHORT_TERM_DENIAL |
| 30 | MN_LONG_TERM_DENIAL |
| 34 | MN_SYSTEM_FAILURE |
| 35 | MN_DATA_MISSING |
| 36 | MN_UNEXPECTED_DATA_VALUE |
| 37 | MN_PW_REGISTRATION_FAILURE |
| 38 | MN_NEGATIVE_PW_CHECK |
| 43 | MN_NUMBER_OF_PW_ATTEMPTS_VIOLATION |
| 54 | MN_POS_METHOD_FAILURE |
| 71 | MN_UNKNOWN_ALPHABET |
| 72 | MN_USSD_BUSY |
| 121 | MN_REJECTED_BY_USER |
| 122 | MN_REJECTED_BY_NETWORK |
| 123 | MN_DEFLECTION_TO_SERVER_SUBSCRIBED |
| 124 | MN_SPECIAL_SERVICE_CODE |
| 125 | MN_INVALID_DEFLECTED_NUMBER |
| 126 | MN_MAX_NUMBER_OF_MPTY_PARTICIPANTS_EXCEEDED |
| 127 | MN_RESOURCES_NOT_AVAILABLE |

##### SS Network GSM Cause

According to 3GPP 24.008, section 10.5.4.11 “Cause”. All values greater than 255 are internal values.

| <cause> | <description> |
| --- | --- |
| 1 | MN_UNASSIGNED_NUMBER |
| 3 | MN_NO_ROUTE |
| 6 | MN_CHANNEL_UNACCEPTABLE |
| 8 | MN_OPERATOR_BARRING |
| 16 | MN_NORMAL_CALL_CLEARING |
| 17 | MN_USER_BUSY |
| 18 | MN_NO_USER_RESPONDING |
| 19 | MN_USER_ALERTING_NO_ANSWER |
| 21 | MN_CALL_REJECTED |
| 22 | MN_NUMBER_CHANGED |
| 26 | MN_NON_SELECTED_USER_CLEARING |
| 27 | MN_DESTINATION_OUT_OF_ORDER |
| 28 | MN_INVALID_NUMBER_FORMAT |
| 29 | MN_FACILITY_REJECTED |
| 30 | MN_RESPONSE_TO_STATUS_ENQUIRY |
| 31 | MN_NORMAL_UNSPECIFIED |
| 34 | MN_NO_CIRCUIT_AVAILABLE |
| 38 | MN_NETWORK_OUT_OF_ORDER |
| 41 | MN_TEMPORARY_FAILURE |
| 42 | MN_SWITCH_CONGESTION |
| 43 | MN_ACCESS_INFORMATION_DISCARDED |
| 44 | MN_REQUESTED_CIRCUIT_NOT_AVAILABLE |
| 47 | MN_RESOURCES_UNAVAILABLE |
| 49 | MN_QUALITY_UNAVAILABLE |
| 50 | MN_FACILITY_NOT_SUBSCRIBED |
| 55 | MN_INCOMING_CALLS_BARRED_IN_CUG |
| 57 | MN_BEARER_CAPABILITY_NOT_ALLOWED |
| 58 | MN_BEARER_CAPABILITY_NOT_AVAILABLE |
| 63 | MN_SERVICE_NOT_AVAILABLE |
| 65 | MN_BEARER_SERVICE_NOT_IMPLEMENTED |
| 68 | MN_ACM_GREATER_OR_EQUAL_TO_ACMMAX |
| 69 | MN_FACILITY_NOT_IMPLEMENTED |
| 70 | MN_ONLY_RESTRICTED_DIGITAL |
| 79 | MN_SERVICE_NOT_IMPLEMENTED |
| 81 | MN_INVALID_TI |
| 87 | MN_USER_NOT_IN_CUG |
| 88 | MN_INCOMPATIBLE_DESTINATION |
| 91 | MN_INVALID_TRANSIT_NETWORK |
| 95 | MN_SEMANTICS_INCORRECT |
| 96 | MN_INVALID_MANATORY_INFORMATION |
| 97 | MN_UNKNOWN_MESSAGE_TYPE_1 |
| 98 | MN_UNEXPECTED_MESSAGE_TYPE |
| 99 | MN_UNEXPECTED_IE |
| 100 | MN_CONDITIONAL_IE_ERROR |
| 101 | MN_UNKNOWN_MESSAGE_TYPE_2 |
| 102 | MN_RECOVERY_ON_TIMER_EXPIRY |
| 111 | MN_PROTOCOL_ERROR |
| 127 | MN_INTERWORKING |
| 256 | MN_VOID_CAUSE |
| 257 | MN_OUT_OF_MEMORY |
| 258 | MN_INVALID_PARAM |
| 259 | MN_DATA_CALL_ACTIVE |
| 260 | MN_SPEECH_CALL_ACTIVE |
| 261 | MN_DTMF_REJECTED_DURING_MO_CALL_SETUP |
| 262 | MN_MOC_SETUP_REJ_DUE_TO_MISSING_ACM_INFO_IND |
| 263 | MN_TEMPORARY_FORBIDDEN_CALL_ATTEMPT |
| 264 | MN_CALLED_PARTY_IS_BLACKLISTED |
| 265 | MN_BLACKLIST_IS_FULL_NO_AUTO_CALL_ATTEMPTS |
| 266 | MN_TEMPORARY_FORBIDDEN_CALL_ATTEMPT_NO_SERVICE |
| 267 | MN_TEMPORARY_FORBIDDEN_CALL_ATTEMPT_LIMITED_SERVICE |
| 268 | MN_CLIENT_TEMPORARY_BARRED |
| 269 | MN_DUAL_SERVICE_CALL_ACTIVE |
| 270 | MN_ATC_FCLASS_NOT_SPEECH |
| 271 | MN_SI_UNKNOWN_ERROR |
| 272 | MN_DTMF_REJECTED_DUE_TO_CALL_HELD_OR_MPARTY |
| 273 | MN_CLIENT_NOT_REGISTRATED |
| 274 | MN_ACTIVE_CLIENT_GONE |
| 275 | MN_DTMF_REJECTED_DUE_TO_DATA_TRANSMISSION |
| 276 | MN_NO_APPROPRIATE_DTMF_ENTRY |
| 277 | MN_SIM_STATUS_FAILURE |
| 278 | MN_REJ_BY_CALL_CONTROL |
| 279 | MN_FDN_FAILED |
| 280 | MN_BDN_FAILED |
| 281 | MN_ONLY_ERROR |
| 282 | MN_NOT_IN_SPEECH_CALL |
| 283 | MN_CCBS_POSSIBLE |
| 283 | MN_INVALID_ALS_LINE |
| 284 | MN_INVALID_ALS_LINE |
| 285 | MN_LND_OVERFLOW |
| 286 | MN_DTMF_REJECTED_NO_TCH_AVAILABLE |
| 287 | MN_NW_FAILURE_UNSPECIFIED |
| 288 | MN_MS_NO_SERVICE |
| 289 | MN_MS_ACCESS_CLASS_BARRED |
| 290 | MN_MS_NO_RESOURCE |
| 291 | MN_MS_SERVICE_BUSY |
| 292 | MN_MS_FAILURE_UNSPECIFIED |
| 293 | MN_DTMF_REJECTED_DUE_TO_SUP_TIMER_EXPIRY |
| 300 | MN_SIMTK_SETUP_MODE_NOT_SUPPORTED |
| 301 | MN_MMI_NOT_REGISTERED |
| 302 | MN_SIMTK_SETUP_REJ_DUE_TO_MISSING_ACM_INFO_IND |
| 303 | MN_SIMTK_SETUP_REJECTED_BY_THE_USER |
| 304 | MN_SIMTK_SETUP_REJECTED_TIME_OUT |
| 305 | MN_SIMTK_CALL_CONNECTED |
| 306 | MN_DISCONNECT_DUE_TO_SIMTK_SETUP |
| 307 | MN_SIMTK_SETUP_REJ_DUE_TO_PENDING_SIMTK_SETUP |
| 308 | MN_SIMTK_CALL_CONNECTED_NO_ICON_DISPLAY |
| 309 | MN_PENDING_SIMTK_SETUP |
| 310 | MN_SIMTK_SIM_RESET |
| 311 | MN_SIMTK_DTMF_TRANSMITTED |
| 312 | MN_SIMTK_DTMF_TRANSMITTED_NO_ICON_DISPLAY |
| 313 | MN_USER_DID_NOT_ACCEPT_CALL_SETUP |
| 314 | MN_PROACTIVE_SIM_APPL_TERMINATED_BY_USER |
| 315 | MN_SIMTK_ME_UNABLE_SCREEN_BUSY |
| 316 | MN_SIMTK_ME_UNABLE_NO_SPECIFIC_CAUSE |
| 317 | MN_SIMTK_UNSPECIFIED |
| 318 | MN_SETUP_SS_ERR |
| 319 | MN_SIMTK_NET_UNABLE_NO_SPECIFIC_CAUSE |
| 320 | MN_SIMTK_USSD_TRANSACTION_TERMINATED_BY_USER |
| 330 | MN_PHONEBOOK_NOT_AVAILABLE |
| 331 | MN_ATC_NO_MATCHING_PHONEBOOK_ENTRY |
| 332 | MN_ATC_INVALID_DIALED_NUMBER |
| 333 | MN_ATC_SETUP_TEMPORARY_BLOCKED |
| 334 | MN_ATC_NO_PERMISSION |
| 335 | MN_ATC_INVALID_CALL_SETTINGS |
| 336 | MN_ATC_BLOCKING_CALL_PRESENT |

##### SS Network Reject Cause

According to 3GPP 24.080, 3.6.7, table 3.13.

Tag 128 MN_GENERAL_PROBLEM with Causes

From 3GPP 24.008, table 3.14.

| <cause> | <description> |
| --- | --- |
| 0 | MN_UNRECOGNIZED_COMPONENT |
| 1 | MN_MISTYPED_COMPONENT |
| 2 | MN_BADLY_STRUCTURED_COMPONENT |

Tag 129 MN_INVOKE_PROBLEM with Causes

From 3GPP 24.008, table 3.15.

| <cause> | <description> |
| --- | --- |
| 0 | MN_DUPLICATE_INVOKE_ID |
| 1 | MN_UNRECOGNIZED_OPERATION |
| 2 | MN_MISTYPED_PRO_PARAMETER |
| 3 | MN_RESOURCE_LIMITATION |
| 4 | MN_INITIATING_RELEASE |
| 5 | MN_UNRECOGNIZED_LINKED_ID |
| 6 | MN_LINKED_RESPONSE_UNEXPECTED |
| 7 | MN_UNEXPECTED_LINKED_OPERATION |
| 130 | MN_RETURN_RESULT_PROBLEM |

Tag 130 MN_RETURN_RESULT_PROBLEM with Causes

From 3GPP 24.008, table 3.16.

| <cause> | <description> |
| --- | --- |
| 0 | MN_UNRECOGNIZED_INVOKE_ID |
| 1 | MN_RETURN_RESULT_UNEXPECTED |
| 2 | MN_MISTYPED_RES_PARAMETER |

Tag 131 MN_RETURN_ERROR_PROBLEM with Causes

| <cause> | <description> |
| --- | --- |
| 0 | MN_UNRECOGNIZED_ERROR_INVOKE_ID |
| 1 | MN_RETURN_ERROR_UNEXPECTED |
| 2 | MN_UNRECOGNIZED_ERROR |
| 3 | MN_UNEXPECTED_ERROR |
| 4 | MN_MISTYPED_ERROR_PARAMETER |

### CMS Error Codes

| <err> Code | Meaning |
| --- | --- |
| 1 | Unassigned (unallocated) number |
| 8 | Operator determined barring |
| 10 | Call barred |
| 21 | Short message transfer rejected |
| 27 | Destination out of service |
| 28 | Unidentified subscriber |
| 29 | Facility rejected |
| 30 | Unknown subscriber |
| 38 | Network out of order |
| 41 | Temporary failure |
| 42 | Congestion |
| 47 | Resources unavailable, unspecified |
| 50 | Requested facility not subscribed |
| 69 | Requested facility not implemented |
| 81 | Invalid short message transfer reference value |
| 95 | Invalid message, unspecified |
| 96 | Invalid mandatory information |
| 97 | Message type non-existent or not implemented |
| 98 | Message not compatible with short message protocol state |
| 99 | Information element non-existent or not implemented |
| 111 | Protocol error, unspecified |
| 127 | Interworking, unspecified |
| 128 | Telematic interworking not supported |
| 129 | Short message Type 0 not supported |
| 130 | Cannot replace short message |
| 143 | Unspecified TP-PID error |
| 144 | Data coding scheme (alphabet) not supported |
| 145 | Message class not supported |
| 159 | Unspecified TP-DCS error |
| 160 | Command cannot be executed |
| 161 | Command unsupported |
| 175 | Unspecified TP-Command error |
| 176 | TPDU not supported |
| 192 | SC busy |
| 193 | No SC subscription |
| 194 | SC system failure |
| 195 | Invalid SME address |
| 196 | Destination SME barred |
| 197 | SM Rejected-Duplicate SM |
| 198 | TP-VPF not supported |
| 199 | TP-VP not supported |
| 208 | D0 SIM SMS storage full |
| 209 | No SMS storage capability in SIM |
| 210 | Error in MS |
| 211 | Memory Capacity Exceeded |
| 212 | SIM Application Toolkit Busy |
| 213 | SIM data download error |
| 255 | Unspecified error cause |
| 300 | ME failure |
| 301 | SMS service of ME reserved |
| 302 | Operation not allowed |
| 303 | Operation not supported |
| 304 | Invalid PDU mode parameter |
| 305 | Invalid text mode parameter |
| 310 | SIM not inserted |
| 311 | SIM PIN required |
| 312 | PH-SIM PIN required |
| 313 | SIM failure |
| 314 | SIM busy |
| 315 | SIM wrong |
| 316 | SIM PUK required |
| 317 | SIM PIN2 required |
| 318 | SIM PUK2 required |
| 320 | Memory failure |
| 321 | Invalid memory index |
| 322 | Memory full |
| 330 | SMSC address unknown |
| 331 | no network service |
| 332 | Network timeout |
| 340 | NO +CNMA ACK EXPECTED |
| 500 | Unknown error |
| 606 | ME Busy – CM server request already pending |

### GPRS Error Codes

| <err> Code | Meaning |
| --- | --- |
| Errors related to a failure to Perform an Attach |  |
| 103 | Illegal MS |
| 106 | Illegal ME |
| 107 | GPRS services not allowed |
| 111 | PLMN not allowed |
| 112 | Location area not allowed |
| 113 | Roaming not allowed in this location area |
| Errors related to a failure to Activate a Context |  |
| 132 | Service option not supported |
| 133 | Requested service option not subscribed |
| 134 | Service option temporarily out of order |
| 149 | PDP authentication failure |
| Other GPRS Errors |  |
| 148 | Unspecified GPRS error |
| 150 | Invalid mobile class |

Other values in the range 101 - 150 are reserved for use by GPRS.

### FTP Reply Codes

| FTP Reply Code | Meaning |
| --- | --- |
| 110 | Restart marker reply |
| 120 | Service ready in nnn minutes |
| 125 | Data connection already open: transfer starting |
| 150 | File status okay; about to open data connection |
| 200 | Command okay |
| 202 | Command not implemented, superfluous at this site |
| 211 | System status or system help reply |
| 212 | Directory status |
| 213 | File status |
| 214 | Help message |
| 215 | NAME system type |
| 220 | Service ready for new user |
| 221 | Service closing control connection. Logged out if appropriate. Unassigned (unallocated) number |
| 225 | Data connection open; no transfer in progress |
| 226 | Closing data connection. Requested file action successful (for example, file transfer or file abort) |
| 227 | Entering Passive Mode (h1, h2, h3, h4, p1, p2) |
| 22 | User logged in, proceed |
| 250 | Requested file action okay, completed |
| 257 | "PATHNAME" created |
| 331 | User name okay, need password |
| 332 | Need account for login |
| 350 | Requested file action pending further information |
| 421 | Service not available, closing control connection. This may be a reply to any command if the service knows it must shut down |
| 425 | Can't open data connection |
| 426 | Connection closed; transfer aborted |
| 450 | Requested file action not taken. File unavailable (e.g., file busy) |
| 451 | Requested action aborted: local error in processing |
| 452 | Requested action not taken. Insufficient storage space in system |
| 500 | Syntax error, command unrecognized. This may include errors such as command line too long |
| 501 | Syntax error in parameters or arguments |
| 502 | Command not implemented |
| 503 | Bad sequence of commands |
| 504 | Command not implemented for that parameter |
| 530 | Not logged in |
| 532 | Need account for storing files |
| 550 | Requested action not taken. File unavailable (e.g., file not found, no access) |
| 551 | Requested action aborted: page type unknown |
| 552 | Requested file action aborted. Exceeded storage allocation (for current directory or dataset) |
| 553 | Requested action not taken. File name not allowed |

### AVMS Error Codes

| <err> Code | Meaning |
| --- | --- |
| 3 | Parameter is out of range; Device Services is not in a good state |
| 24 | Parameters <APN>, <User> or <Pwd> are too long |
| 650 | General error |
| 651 | Communication error |
| 652 | Session in progress |
| 654 | AVMS services are in DEACTIVATED state (see +WDSG) |
| 655 | AVMS services are in PROHIBITED state (see +WDSG) |
| 656 | AVMS services are in TO BE PROVISIONED state (see +WDSG) |

### Error Case Examples

AT commands return specific error codes if parameter verification fails. The following tables enumerate some examples to demonstrate specific error cases.

Generic Error Case Examples

| Error Codes | Corresponding Examples |
| --- | --- |
| +CME ERROR: 3 when execute command is not supported | AT+SWITRC AT+COREDUMP AT+LOGLV AT+NVBU AT+KGSN AT+HWREV |
| +CME ERROR: 3 when action command is not supported | AT+KBND AT*PSRDBS AT+KSRAT AT+KSREP AT+WMANTSEL AT+KGPIOCFG AT+KGPIO AT+WMIOTEST AT+KTEMPMON AT+WPPP AT+KADC AT+KCELL AT+KCCINFO AT+KSLEEP AT+KRIC AT&C AT+OMADMST AT+GST AT+HBHV AT+KSIMDET AT+KSIMSEL AT*PSSTKI AT+KPCMCFG AT+WMAUDIOLOOP AT+VIP AT+VGT AT+KVGT AT+VGR AT+KVGR AT+CLVL AT+KECHO AT+KNOISE AT+KST AT+KPC AT+CALM AT+CRSL AT+KSRAP AT+CODECINFO AT+WIMEI AT+WMUSBVCC AT+KUSBCOMP AT+BOOTDWLCFG AT+KLTEMUTE AT+KSYNC AT+KLTEPARAM AT+KAAT AT+CMEC AT+KGSMAD AT+CALA AT+CALD |

Non-Generic Error Case Examples

| Error Codes | Corresponding Examples |
| --- | --- |
| +CME ERROR: 3 instead of ERROR is returned for some cases in some SREG or BASIC AT commands. This is for backward compatibility reason. | ATB100 +CME ERROR: 3 ATB=? ERROR ATB? ERROR ATB=0 ERROR ATB +CME ERROR: 3 |
| +CME ERROR: 16 (incorrect password) instead of +CME ERROR: 18 (PUK2 required) is returned when PIN2 is blocked in AT+CPIN2 write command; while +CME ERROR: 12 (PUK required) is returned when PIN is blocked in AT+CPIN write command. | AT+CPIN2="9876" +CME ERROR: 16 AT+CPIN2="9876" +CME ERROR: 16 AT+CPIN2="9876" +CME ERROR: 16 AT+CPIN2="22222222","5678" OK AT+CPIN="5678" +CME ERROR: 16 AT+CPIN="5678" +CME ERROR: 16 AT+CPIN="5678" +CME ERROR: 12 AT+CPIN? +CPIN: SIM PUK OK AT+CPIN="11111111","1234" OK |
| +CME ERROR: 18 (SIM PUK2 REQUIRED) instead of +CME ERROR: 50 (INCORRECT PARAMETERS) is returned for +CPIN2 when PUK2 is required while only PIN2 was inputted, +CME ERROR: 12 (SIM PUK REQUIRED) instead of +CME ERROR: 50 (INCORRECT PARAMETERS) is returned for +CPIN when PUK is required while only PIN was inputted, | AT+CPIN2="9876" +CME ERROR: 16 AT+CPIN2="9876" +CME ERROR: 16 AT+CPIN2="9876" +CME ERROR: 16 AT+CPIN2="9876" // Incorrect PIN2 – 4th time +CME ERROR: 18 AT+CPIN2="22222222","5678" OK AT+CPIN="5678" +CME ERROR: 16 AT+CPIN="5678" +CME ERROR: 16 AT+CPIN="5678" +CME ERROR: 12 AT+CPIN="5678" // Incorrect PIN1 – 4th time +CME ERROR: 12 AT+CPIN? +CPIN: SIM PUK OK AT+CPIN="11111111","1234" OK |

Internet Error Case Examples

| Error Codes | Corresponding Examples |
| --- | --- |
| +CME ERROR: 907 Generic error/Unsupported read command | AT+KHTTPHEAD? AT+KHTTPGET? AT+KHTTPREAD? AT+KHTTPPOST? AT+KHTTPCLOSE? AT+KHTTPSGET? AT+KHTTPSHEAD? AT+KHTTPSPOST? AT+KHTTPSCLOSE? AT+KFTPCNX? AT+KFTPCLOSE? AT+KFTPCFGDEL? AT+KFTPRCV? AT+KFTPSND? AT+KFTPDEL? AT+KUDPDEL? AT+KUDPCLOSE? AT+KUDPRCV? AT+KUDPSND? AT+KTCPSND? AT+KTCPRCV? AT+KTCPCNX? |
| +CME ERROR: 907 Generic error/Unsupported read command | AT+KTCPCLOSE? AT+KTCPDEL? |
| +CME ERROR: 912 No more sessions can be used | Create a UDP client session repeatedly until 32 sessions are created: AT+KUDPCFG=1,0,1033,,"10.10.10.10" Then try to create a TCP server session (33rd session) AT+KTCPCFG=1,1,,80 |
| +CME ERROR: 915 A parameter is not expected | AT+KHTTPHEADER=1,0 AT+KHTTPHEADER=1,"file" |
|  | AT+KHTTPPOST=1,0,"/" AT+KHTTPPOST=1,"file","/" |
|  | AT+KHTTPSPOST=1,0,"/" AT+KHTTPSPOST=1,1,"/" AT+KHTTPSPOST=1,"file","/" |
|  | AT+KHTTPSHEADER=1,0 AT+KHTTPSHEADER=1,1 AT+KHTTPSHEADER=1,"file" |
|  | AT+KFTPRCV=1,0,,"/sample.txt" AT+KFTPRCV=1,1,,"/sample.txt" AT+KFTPRCV=1,"file",,"/sample.txt" |
| +CME ERROR: 916 A parameter has an invalid range of values | AT+KHTTPGET=0,"/" AT+KHTTPGET=1,"/",2 |
|  | AT+KHTTPHEADER=0 |
|  | AT+KHTTPHEAD=0,"/" |
|  | AT+KHTTPCLOSE=0 AT+KHTTPCLOSE=1,-1 |
|  | AT+KHTTPPOST=0,,"/" AT+KHTTPPOST=1,,"/",2 |
|  | AT+KHTTPCFG=0,"www.example.com" AT+KHTTPCFG=1,"www.example.com",65536 AT+KHTTPCFG=1,"www.example.com",,,,,2 AT+KHTTPSCFG=0,"www.kernel.org" AT+KHTTPSCFG=-1,"www.kernel.org" AT+KHTTPSCFG=1,"www.kernel.org",65536 AT+KHTTPSCFG=1,"www.kernel.org",-1 AT+KHTTPSCFG=1,"www.kernel.org",,2 AT+KHTTPSCFG=1,"www.kernel.org",,,8 AT+KHTTPSCFG=1,"www.kernel.org",,,-1 AT+KHTTPSCFG=1,"www.kernel.org",,,,4 AT+KHTTPSCFG=1,"www.kernel.org",,,,,,,2 AT+KHTTPSCFG=1,"www.kernel.org",,,,,,,-1 |
|  | AT+KHTTPSGET=1,"/",2 AT+KHTTPSGET=1,"/",-1 |
|  | AT+KHTTPSHEAD=0,"/" AT+KHTTPSHEAD=-1,"/" |
|  | AT+KHTTPSPOST=0,,"/" AT+KHTTPSPOST=-1,,"/" AT+KHTTPSPOST=1,,"/",2 AT+KHTTPSPOST=1,,"/",-1 |
|  | AT+KHTTPSHEADER=0 AT+KHTTPSHEADER=-1 |
|  | AT+KHTTPSCLOSE=0 AT+KHTTPSCLOSE=-1 AT+KHTTPSCLOSE=1,2 AT+KHTTPSCLOSE=1,-1 |
| +CME ERROR: 916 A parameter has an invalid range of values | AT+KFTPCFG=0,"ftp.kernel.org" AT+KFTPCFG=1,"ftp.kernel.org",,,65536 AT+KFTPCFG=1,"ftp.kernel.org",,,-1 AT+KFTPCFG=1,"ftp.kernel.org",,,,2 AT+KFTPCFG=1,"ftp.kernel.org",,,,-1 AT+KFTPCFG=1,"ftp.kernel.org",,,,,2 AT+KFTPCFG=1,"ftp.kernel.org",,,,,10 AT+KFTPCFG=1,"ftp.kernel.org",,,,,,-1 AT+KFTPCFG=1,"ftp.kernel.org",,,,,,? |
|  | AT+KFTPCNX=0 AT+KFTPCNX=99 AT+KFTPCNX=-1 |
|  | AT+KFTPCLOSE=0 AT+KFTPCLOSE=1,2 AT+KFTPCLOSE=1,-1 |
|  | AT+KFTPCFGDEL=0 AT+KFTPCFGDEL=-1 |
|  | AT+KFTPRCV=0,,,"/sample.txt" AT+KFTPRCV=-1,,,"/sample.txt" AT+KFTPRCV=1,,,"/sample.txt",2 AT+KFTPRCV=1,,,"/sample.txt",-1 |
|  | AT+KFTPSND=0,,,"/sample.txt" AT+KFTPSND=-1,,,"/sample.txt" AT+KFTPSND=1,,,"/sample.txt",2 AT+KFTPSND=1,,,"/sample.txt",,-1 |
|  | AT+KFTPDEL=0,,"/sample.txt" AT+KFTPDEL=-1,,"/sample.txt" AT+KFTPDEL=1,,"/sample.txt",2 AT+KFTPDEL=1,,"/sample.txt",-1 |
|  | AT+KTCPSND=1,0 |
|  | AT+KTCPRCV=1,0 |
|  | AT+KUDPSND=1,"116.66.221.43",5043,0 |
|  | AT+KUDPRCV=1,0 |
| +CME ERROR: 917 A parameter is missing | AT+KHTTPGET=,"/" AT+KHTTPGET=1, AT+KHTTPGET=, |
|  | AT+KHTTPHEADER=, |
|  | AT+KHTTPHEAD=,"/" AT+KHTTPHEAD=1, AT+KHTTPHEAD=, |
|  | AT+KHTTPCLOSE=, |
|  | AT+KHTTPPOST=,,"/" AT+KHTTPPOST=1,, |
|  | AT+KHTTPCFG=1, AT+KHTTPCFG=, |
|  | AT+KHTTPSCFG=1, AT+KHTTPSCFG=1,www.kernel.org AT+KHTTPSCFG=, |
|  | AT+KHTTPSGET=,"/" AT+KHTTPSGET=1, AT+KHTTPSGET=, |
|  | AT+KHTTPSHEAD=,"/" AT+KHTTPSHEAD=1, AT+KHTTPSHEAD=, |
| +CME ERROR: 917 A parameter is missing | AT+KHTTPSPOST=,,"/" AT+KHTTPSPOST=1,, |
|  | AT+KHTTPSHEADER=, |
|  | AT+KHTTPSCLOSE=, |
|  | AT+KFTPCFG=1, AT+KFTPCFG= |
|  | AT+KFTPCLOSE=, |
|  | AT+KFTPRCV=1,,, |
|  | AT+KFTPSND=1,,, |
|  | AT+KFTPDEL=1,, AT+KFTPDEL=,, |
| +CME ERROR: 918 Feature is not supported | AT+KHTTPSCFG=1,"www.kernel.org",,,1 AT+KHTTPSCFG=1,"www.kernel.org",,,3 |
| +CME ERROR: 919 Feature is not available | AT+KTCPACKINFO=1 |
| +CME ERROR: 932 The format of a parameter is invalid | AT+KHTTPGET=a,"/" |
|  | AT+KHTTPHEADER=a |
|  | AT+KHTTPHEAD=a,"/" |
|  | AT+KHTTPCLOSE=a AT+KHTTPCLOSE=1,? |
|  | AT+KHTTPPOST=a,,"/" AT+KHTTPPOST=1,,"/",? |
|  | AT+KHTTPCFG=a,"www.example.com" AT+KHTTPCFG=1,"www.example.com",,? AT+KHTTPCFG=1,"www.example.com",a AT+KHTTPCFG=1,"www.example.com",,,,,? |
|  | AT+KHTTPSCFG=a,"www.kernel.org" AT+KHTTPSCFG=1,"www.kernel.org",a AT+KHTTPSCFG=1,"www.kernel.org",,? AT+KHTTPSCFG=1,"www.kernel.org",,,,? |
|  | AT+KHTTPSGET=a,"/" AT+KHTTPSGET=1,"/",? |
|  | AT+KHTTPSHEAD=a,"/" |
|  | AT+KHTTPSPOST=a,,"/" AT+KHTTPSPOST=1,,"/",? |
|  | AT+KHTTPSHEADER=a |
|  | AT+KHTTPSCLOSE=a AT+KHTTPSCLOSE=1,? |
|  | AT+KFTPCFG=a,"ftp.kernel.org" AT+KFTPCFG=1,"ftp.kernel.org",,,,,? AT+KFTPCFG=1,"ftp.kernel.org",,,,,,? |
|  | AT+KFTPCNX=a AT+KFTPCNX=# |
|  | AT+KFTPCLOSE=b AT+KFTPCLOSE=1,? |
|  | AT+KFTPCFGDEL=C AT+KFTPCFGDEL=# |
| +CME ERROR: 932 The format of a parameter is invalid | AT+KFTPRCV=D,,,"/sample.txt" AT+KFTPRCV=#,,,"/sample.txt" AT+KFTPRCV=1,,,"/sample.txt",? |
|  | AT+KFTPSND=E,,,"/sample.txt" AT+KFTPSND=#,,,"/sample.txt" AT+KFTPSND=1,,,"/sample.txt",? AT+KFTPSND=1,,,"/sample.txt",,? |
|  | AT+KFTPDEL=f,,"/sample.txt" AT+KFTPDEL=#,,"/sample.txt" AT+KFTPDEL=1,,"/sample.txt",? |
|  | AT+KCGPADDR=a |

## Commands without Pin Code Requirement

Most AT commands are rejected (i.e. an error is returned to the DTE) if the valid PIN code has not been entered.

Commands which can be sent without the PIN code include:

ATD (emergency calls)

AT+CPIN

ATI

AT+CGMI, AT+GMI

AT+CGMM, AT+GMM

AT+CGMR, AT+GMR

AT+CGSN, AT+GSN

AT+CPAS

AT+CIND

AT+CMEE

AT+IPR

ATE, ATV, ATS, ATZ

AT&F

AT+CBST

This list may be modified in case of special needs from the customer. Contact Sierra Wireless to request modifications. Note that some commands may also require the PIN2 code.

## GSM 27.010 Multiplexing Protocol

| Main Options | BASIC | YES |
| --- | --- | --- |
|  | ADVANCED | YES |
|  | advanced WITH ERROR RECOVERY | NO |
| Frames | SABM | YES |
|  | UA | YES |
|  | DM | YES |
|  | DISC | YES |
|  | I (ERM) | NO |
|  | RR (ERM) | NO |
|  | RNR (ERM) | NO |
|  | REJ (ERM) | NO |
|  | UI | YES |
|  | UIH | YES |
| Multiplexer Controls | DLC parameters negotiation (PN) (optional) | YES |
|  | Power Saving Control (PSC) | NO |
|  | Multiplexer Close Down (CLD) | YES |
|  | Test Command (Test) | YES |
|  | Flow control On Command (Fcon) | YES |
|  | Flow control Off Command (Fcoff) | YES |
|  | Modem Status Command (MSC) | YES |
|  | Non Supported Command response (NSC) | YES |
|  | Remote Port Negotiation (RPN). (optional) | NO |
|  | Remote Line Status command (RLS).(optional) | YES |
|  | Service Negotiation Command (SNC) | NO |
| Convergence Layers | Type 1 - Unstructured Octet Stream | YES |
|  | Type 2 - Unstructured Octet Stream with flow control, break signal handling and transmission of v24 signal states | YES |
|  | Type 3 – Uninterruptible Framed Data | NO |
|  | Type 4 - Interruptible Framed Data | NO |
| CMUX Parameters | Link speed | 9600, 19200, 38400, 57600, 115200 |
|  | Maximum frame size | 1540 |
|  | Acknowledgment timer | 100 |
|  | Maximum number of retransmissions | 100 |
|  | Response timer for control channel | 30 |
|  | Wake up response timer | 10 seconds |
| Others | Wake up procedure (see [RE2] sub clause 5.4.7) | YES |
|  | Priority management | YES |
|  | DLCI number limitation | 8 |

## TCP Commands Examples

### Client Mode

| AT&K3 | Hardware flow control activation |
| --- | --- |
| OK |  |
| AT+KCNXCFG=1,"GPRS","APN","log","password","0.0.0.0","0.0.0.0","0.0.0.0" | Set GPRS parameters (APN, login, password) |
| OK |  |
| AT+KTCPCFG=1,0,"www.google.com",80 | Set IP address and port number |
| +KTCPCFG: 1 | Returns session ID 1 |
| OK |  |
| AT+KTCPCNX=1 | Initiate the connection |
| OK |  |
| AT+KTCPSND=1,18 CONNECT …Data send… | Send data with the EOF string at the end "GET / HTTP / 1.0 |
| OK | --EOF--Pattern" |
| +KTCP_DATA: 1,1380 |  |
| AT+KTCPRCV=1, 1380 |  |
| CONNECT |  |
| HTTP/1.0 200 OK |  |
| Cache-Control: private, max-age=0 | Read data |
| … a lot of data… |  |
| --EOF--Pattern-- |  |
| OK |  |
| +KTCP_DATA: 1,1380 | +KTCP_DATA notification |
| AT+KTCPRCV=1,1380 | Read received data |
| CONNECT |  |
| er{padding-bottom:7px !important}#gbar,#guser{font- |  |
| … a lot of data… |  |
| --EOF--Pattern-- |  |
| OK |  |
| +KTCP_DATA: 1,1380 |  |
| AT+KTCPCLOSE=1,1 | Close session 1 |
| OK |  |
| AT+KTCPDEL=1 | Delete session 1 |
| OK |  |
| AT+KTCPCFG? | No session is available |
| OK |  |

### Server Mode

In this simple example, we emulate a daytime server. This server listens to port 13 and for each connection it returns the date.

| AT&K3 | Hardware flow control activation |
| --- | --- |
| OK |  |
| AT+KCNXCFG=1,"GPRS","APN","log","password","0.0.0.0","0.0.0.0","0.0.0.0" | Set GPRS parameters (APN, login, password) |
| OK |  |
| AT+KTCPCFG=1,1,,13 | Set TCP listener and port number |
| +KTCPCFG: 1 | Returns session ID 1 |
| OK |  |
| AT+KTCPCNX=1 | Initiate the server |
| OK |  |
| AT+KCGPADDR +KCGPADDR: 0,"10.35.125.89" | Get the IP address to initiate a connection request with a client |
| OK |  |
| +KTCP_SRVREQ: 1,2 | A client requests a connection (session ID 2) |
| AT+KTCPSND=2,15 |  |
| CONNECT |  |
| …Date and time… | Data is sent to the client read |
| OK |  |
| +KTCP_SRVREQ: 1,3 | Another client requests a connection (session ID 3); child mode for session 3 |
| +KTCP_NOTIF: 2, 4 | Client (session 2) closes the connection |
| AT+KTCPSND=3,15 |  |
| CONNECT |  |
| …Date and time… | Data is sent to the client |
| OK |  |
| AT+KTCPCLOSE=3,1 OK | Close client session 3 and then session 3 is deleted automatically (child mode for session 3) |
| AT+KTCPCLOSE=1,1 | Close server session 1 |
| OK |  |
| AT+KTCPDEL=1 | Delete session 1 |
| OK |  |

### Polling for the Status of a Socket

| AT&K3 | Hardware flow control activation |
| --- | --- |
| OK |  |
| AT+KCNXCFG=1,"GPRS","APN","log","password","0.0.0.0","0.0.0.0","0.0.0.0" | Set GPRS parameters (APN, login, password) |
| OK |  |
| AT+KTCPCFG=1,0,"www.google.com",80 | Set TCP Server address and port number |
| +KTCPCFG: 1 | Returns session ID 1 |
| OK |  |
| AT+KURCCFG="TCP",0 | Disable TCP unsolicited messages |
| OK |  |
| AT+KTCPCNX=1 | Initiate connection, use session 1 |
| OK |  |
| AT+KTCPSTAT=1 | Poll the connection status |
| +KTCPSTAT: 3,-1,0,0 | Connection is UP |
| OK |  |
| AT+KTCPSND=1,3000 CONNECT | Send data on socket 1 for 3000 bytes or less. |
| <…Data send…> | Data can be sent after CONNECT |
| OK | Send the EOF string to finish. The EOF parttern should be defined using the +KPATTERN command. |
| AT+KTCPSTAT=1 | Poll the connection status |
| +KTCPSTAT : 3,-1,1234,0 OK | Connection is up, with 1234 unsent bytes |
| AT+KTCPSTAT=1 | Poll the connection status |
| +KTCPSTAT : 3,-1,100,0 OK | Connection is up, with 100 unsent bytes |
| AT+KTCPSTAT=1 | Poll the connection status |
| +KTCPSTAT : 3,-1,0,0 | Connection is up, all bytes have been sent |
| OK |  |
| AT+KTCPSTAT=1 | Poll the connection status |
| +KTCPSTAT : 3,-1,0,320 OK | Connection is up with 320 bytes available for reading |
| AT+KTCPRCV=1,320 | Read 320 bytes on socket 1 |
| CONNECT |  |
| <… a lot of data…> | Data are sent after CONNECT |
| --EOF--Pattern-- |  |
| OK |  |
| AT+KTCPCLOSE=1,1 | Close session 1 |
| OK |  |
| AT+KTCPDEL=1 | Delete session 1 |
| OK |  |

### End to End TCP Connection

| AT&K3 | Hardware flow control activation |
| --- | --- |
| OK |  |
| AT+KCNXCFG=1,"GPRS","APN","log","password","0.0.0.0","0.0.0.0","0.0.0.0" | Set GPRS parameters (APN, login, password) |
| OK |  |
| AT+KTCPCFG=1,0,"www.google.com",80 | Set the TCP server address and port number |
| +KTCPCFG: 1 | Returns session ID 1 |
| OK |  |
| AT+KTCPSTART=1 | Initiate connection, use session 1 |
| CONNECT …Data sent……Data received……Data sent… | Message CONNECT: connection to server is established, data can be sent |
| …Data sent……Data received……Data sent… |  |
| +++ | Use +++ to enter command mode |
| OK |  |
| ATO1 CONNECT | Use ATO<session_id> to switch back to data mode |
| …Data sent……Data received……Data sent… |  |
| …Data sent……Data received……Data sent… |  |
| OK |  |
|  | Toggle DTR (if using AT&D1 or AT&D2 configuration) to enter command mode |
| AT+KTCPCLOSE=1,1 | Close the session |
| OK |  |
| AT+KTCPDEL=1 | Delete the configured session |
| OK |  |

### Error Case for End to End TCP Connection

| AT+KTCPSTART=1 | Try to initiate connection |
| --- | --- |
| NO CARRIER +KTCP_NOTIF: 1,<tcp_notif> | Connection failed, see the value of <tcp_notif> |
| AT+KTCPSTART=1 | Initiate connection |
| CONNECT |  |
| …Data sent……Data received……Data sent… | Exchange some data |
| …Data sent……Data received……Data sent… |  |
| NO CARRIER |  |
| +KTCP_NOTIF: 1,<tcp_notif> | An error occurs during connection (network lost, server closed, etc.) |

### Use Cases for AT+KTCPACKINFO and <URC-ENDTCP-enable> Option

This section describes the behavior of AT+KTCPACKINFO when the <URC-ENDTCP> option is used with AT+KTCPCFG.

#### <URC-ENDTCP-enable> is Disabled (default setting)

| AT+KCNXCFG=1,"GPRS","CMNET" |  |
| --- | --- |
| OK |  |
| AT+KTCPCFG=1,0,"202.170.131.76",2000 |  |
| +KTCPCFG: 1 |  |
| OK |  |
| AT+KTCPCFG? |  |
| +KTCPCFG: 1,0,0,0,,"202.170.131.76",2000,,0,0 | <URC-ENDTCP-enable> is disabled |
| OK |  |
| AT+KTCPCNX=1 | Connect to TCP server |
| OK |  |
| AT+KTCPSND=1,10 | Use command to send 10 bytes |
| CONNECT |  |
| 0123456789--EOF--Pattern-- | Write to serial |
| OK |  |
|  | The URC “+KTCP_ACK” is not displayed |
| AT+KTCPACKINFO=1 |  |
| +CME ERROR: operation not allowed | Error is returned because <URC-ENDTCP-enable> is disabled |

#### <URC-ENDTCP-enable> is Enabled

| AT+KCNXCFG=1,"GPRS","CMNET" |  |
| --- | --- |
| OK |  |
| AT+KTCPCFG=1,0,"202.170.131.76",2000,,,1 +KTCPCFG: 1 | Set <URC-ENDTCP-enable> to 1, enable URC “+KTCP_ACK” |
| OK |  |
| AT+KTCPCFG? |  |
| +KTCPCFG: 1,0,0,0,,"202.170.131.76",2000,,0,1 | <URC-ENDTCP-enable> is enabled |
| OK |  |
| AT+KTCPCNX=1 | Connect to TCP server |
| OK |  |
| AT+KTCPSND=1,10 | Receive 10 bytes |
| CONNECT | Connect to TCO server |
| 0123456789--EOF--Pattern-- | Write to serial |
| OK |  |
| +KTCP_ACK: 1, 1 | After a short time, URC “+KTCP_ACK” states that the latest TCP data has arrived on the remote side |
| AT+KTCPACKINFO=1 +KTCPACKINFO: 1, 1 | Poll the status of the latest TCP data |
| OK |  |
| AT+KTCPSND=1,1000 | Send 1000 bytes |
| CONNECT |  |
| <1000bytes and --EOF--Pattern | Write to serial |
| OK |  |
|  | URC “+KTCP_ACK” not got yet |
| AT+KTCPACKINFO=1 | Poll the status of the latest TCP data |
| +KTCPACKINFO: 1, 2 | The status of the latest TCP data is unknown |
| OK |  |
|  | Since the “OK” of the latest “+KTCPSND”, 64 seconds has elapsed |
| +KTCP_ACK: 1, 0 | URC “+KTCP_ACK” indicates that data has not arrived on the remote side yet. The network may not be good. |
| AT+KTCPACKINFO=1 | Poll the status of the latest TCP data |
| +KTCPACKINFO: 1, 0 OK | The status of the latest TCP data is “failure”: not all data has been received by the remote side |

## UDP Commands Examples

### Client Mode

| AT&K3 | Hardware flow control activation |
| --- | --- |
| OK |  |
| AT+KCNXCFG=1,"GPRS","APN","log","password",,, OK | Set GPRS parameters (APN, login, password) |
| AT+KUDPCFG=1,0 +KUDPCFG: 1 OK | Create a new UDP socket (returned session 1) with the parameters associated to the connection profile ID number 0 |
| AT+KUDPSND= 1,"82.234.17.52",32,18 |  |
| CONNECT | Send UDP data after “CONNECT” |
| <…Data sent…> |  |
| --EOF--Pattern-- |  |
| OK |  |
| +KUDP_DATA: 1,35 | Received notification that indicates the presence of 35 bytes in the socket |
| AT+KUDPRCV=1, 35 | Try to read 35 bytes from session 1 |
| CONNECT |  |
| This is a simple UDP Protocol test |  |
| --EOF--Pattern-- |  |
| OK |  |
| +KUDP_RCV: "82.234.17.52",32 |  |
| +KUDP_DATA: 1,35 | Received notification that indicates the presence of 35 bytes in the socket |
| AT+KUDPRCV=1, 18 CONNECT | Try to read 18 bytes from session 1 |
| This is a simple |  |
| --EOF--Pattern-- |  |
| OK |  |
| +KUDP_DATA_MISSED: 1,17 | There are 17 unread bytes left and missed in the UDP socket |
| AT+KUDPCLOSE=1 OK | Close the UDP session |
| AT+KUDPCFG? | No sessions are available anymore |
| OK |  |

### Server Mode

| AT&K3 | Hardware flow control activation |
| --- | --- |
| OK |  |
| AT+KCNXCFG=1,"GPRS","APN","log","password","0.0.0.0","0.0.0.0","0.0.0.0" | Set GPRS parameters (APN, login, password) |
| OK |  |
| AT+KUDPCFG=1,1,3000 | Set UDP listener (port 3000). Initiate the server. Session ID is 1 |
| +KUDPCFG: 1 |  |
| OK |  |
| AT+KUDPCFG? | Check if the server is initiated |
| +KUDPCFG: 1,0,1,3000 |  |
| OK |  |
| AT+KCGPADDR | Get local IP address |
| +KCGPADDR: 0, "192.168.0.71" |  |
| OK |  |
| +KUDP_DATA: 1,9 | Data comes in from some client |
| AT+KUDPRCV=1,9 | Read received data |
| CONNECT |  |
| DATA TEST--EOF--Pattern-- |  |
| OK |  |
| +KUDP_RCV: "10.10.10.5",1111 | This data was sent from "10.10.10.5" (port:1111) |
| AT+KUDPSND=1,"10.10.10.5”,3100,18 CONNECT <18 bytes data ended with "--EOF--Pattern--"> | Send 18 bytes to a remote server (port:3100) |
| OK |  |
| AT+KUDPCLOSE=1 OK | Close the UDP server. The session is also deleted at the same time |
| AT+KUDPCFG? | No sessions are available anymore |
| OK |  |

### Use Cases for KTCP_DATA and KUDP_DATA

#### KTCP_DATA and KUDP_DATA without Data Auto Retrieval – Client Mode

| AT+KCNXCFG=1,"GPRS","CMNET" |  |
| --- | --- |
| OK |  |
| AT+KTCPCFG=1,0,"202.170.131.76",2000 |  |
| +KTCPCFG: 1 |  |
| OK |  |
| AT+KTCPCNX=1 | Connect to TCP server |
| OK |  |
| +KTCP_DATA: 1,10 | 10 bytes have arrived |
| AT+KTCPRCV=1,10 CONNECT | Receive the 10 bytes that arrived |
| 0123456789--EOF--Pattern-- |  |
| OK |  |
| AT+KUDPCFG=1,0 | Open a UDP socket |
| +KUDPCFG: 2 |  |
| OK |  |
| +KUDP_DATA: 2,8 | 8 bytes have arrived |
| AT+KUDPRCV=2,8 | Read the data |
| CONNECT |  |
| 01234567--EOF--Pattern-- |  |
| OK |  |
| +KUDP_RCV: "202.170.131.76",2001 |  |

#### KTCP_DATA and KUDP_DATA without Data Auto Retrieval – Server Mode

| AT+KTCPCFG=1,1,,13 | Configure a TCP server socket |
| --- | --- |
| +KTCPCFG: 1 |  |
| OK |  |
| AT+KTCPCNX=1 | Open the listen port |
| OK |  |
| AT+KCGPADDR |  |
| +KCGPADDR: 0,“10.35.125.89” |  |
| OK |  |
| +KTCP_SRVREQ: 1,2 | Session 2 is set |
| +KTCP_SRVREQ: 1,3 | Session 3 is set |
| +KTCP_DATA: 2,10 | 10 bytes have arrived at session 2 |
| +KTCP_DATA: 3,8 | 8 bytes have arrived at session 3 |
| AT+KTCPRCV=2,10 CONNECT | Receive the 10 bytes in session 2 |
| 0123456789--EOF--Pattern-- |  |
| OK |  |
| AT+KTCPRCV=3,8 CONNECT | Receive the 8 bytes in session 3 |
| 01234567--EOF--Pattern-- |  |
| OK |  |
| AT+KUDPCFG=1,1,3000 | Open a UDP socket in server mode |
| +KUDPCFG: 4 |  |
| OK |  |
| +KUDP_DATA: 4,8 | 8 bytes have arrived |
| AT+KUDPRCV=4,8 | Receive the 8 bytes |
| CONNECT |  |
| 01234567--EOF--Pattern-- |  |
| OK |  |
| +KUDP_RCV: "202.170.131.76",2001 |  |

#### KTCP_DATA and KUDP_DATA with Data Auto Retrieval – Client Mode

| AT+KCNXCFG=1,"GPRS","CMNET" |  |
| --- | --- |
| OK |  |
| AT+KTCPCFG=0,0,"202.170.131.76",2000,,1 +KTCPCFG: 1 | When <data_mode> = 1, data will be received by the URC “+KTCP_DATA |
| OK |  |
| AT+KTCPCNX=1 | Connect to TCP server |
| OK |  |
| +KTCP_DATA: 1,10,0123456789 | 10 bytes have arrived. The data are presented in the URC directly |
| AT+KUDPCFG=0,0,3000,1 +KUDPCFG: 2 | When <data_mode> = 1, data will be received by the URC “+KUDP_DATA |
| OK |  |
| +KUDP_DATA: 2,8,"202.170.131.76",2001,01234567 | 8 bytes have arrived. The data are presented in the URC directly |

#### KTCP_DATA and KUDP_DATA with Data Auto Retrieval – Server Mode

| AT+KTCPCFG=1,1,,13,1 +KTCPCFG: 1 OK | When <data_mode> = 1, all child connections will display data in URC mode. Data will be received by the URC “+KTCP_DATA:” |
| --- | --- |
| AT+KTCPCNX=1 | Open the listen port |
| OK |  |
| AT+KCGPADDR |  |
| +KCGPADDR: 1,"10.35.125.89" |  |
| OK |  |
| +KTCP_SRVREQ: 1,2 |  |
| +KTCP_SRVREQ: 1,3 |  |
| +KTCP_DATA: 2,10,0123456789 | 10 bytes have arrived in session 2 |
| +KTCP_DATA: 3,8,01234567 | 8 bytes have arrived in session 3 Data are presented in the URC directly |
| AT+KUDPCFG=1,1,3000,1 +KUDPCFG: 4 OK | Open a UDP socket in server mode. Data will be received by the URC “+KUDP_DATA:” |
| +KUDP_DATA: 4,8,"202.170.131.76",2001,01234567 | 8 bytes have arrived. Data are presented in the URC directly |

## FTP Commands Examples

### Client Mode

| AT&K3 | Hardware flow control activation |
| --- | --- |
| OK |  |
| AT+KCNXCFG=1,"GPRS","APN","log","password",,, OK | Set GPRS parameters (APN, login, password) |
| AT+KFTPCFG=1,"ftp.test.fr","userlogin","userpassword",21,0 | Set FTP server address, login, password and port number |
| OK |  |
| AT+KPATTERN="--EOF--Pattern--" | Customize the End Of File pattern |
| OK |  |
| AT+KFTPSND=0,,"Dir","TestFile.txt",0 CONNECT F6E6E656374696F6E20746573742E--EOF--Pattern-- | Send data and store them in “TestFile.txt” from the FTP server. Data are presented with the EOF string. |
| OK |  |
| AT+KFTPRCV=0,,"Dir","Testfile.txt",0 CONNECT | Read the file named "TestFile.txt" from ftp server, data are sent and end by EOF string |
| F6E6E656374696F6E20746573742E--EOF--Pattern-- |  |
| OK |  |
| AT+KFTPDEL=0,"Dir","TestFile.txt" OK | Delete the file called "TestFile.txt" in the FTP server |
| AT+KFTPCLOSE=0 | Close the connection |
| OK |  |

### "FTP Resume" Use Case

#### Resume Feature when Transmitting Data to Serial Link

| AT+KCNXCFG=1,"GPRS","CMNET" |  |
| --- | --- |
| OK |  |
| AT+KFTPCFG=1,"202.170.131.76","administrator","8ik,(OL>",21,0 |  |
| +KFTPCFG: 1 |  |
| OK |  |
| AT+KFTPRCV=1,,,"111111.txt",0 |  |
| CONNECT |  |
| 750aaaaaaaaa….. aaaaa250bbbbbbb--EOF--Pattern-- | Total of 760 data from the serial link |
| +KFTP_ERROR: 1, 421 | The result code indicates that the download met with some problems which may be due to control or data connection lost |
| AT+KFTPRCV=1,,,"111111.txt",0,760 | Try to resume transfer by using the offset 760. |
| bbbbbb………bbbbbbbbend--EOF--Pattern-- | Total data from the serial link should be 240 |
| OK |  |
|  | The complete file “111111.txt” can be obtained by combining the data received from the two separate downloads |

#### Use Case when FTP Server does not Support the Resume Feature

| AT+KCNXCFG=1,"GPRS","CMNET" |  |
| --- | --- |
| OK |  |
| AT+KFTPCFG=1,"202.170.131.76","administrator","8ik, (OL>",21,0 |  |
| +KFTPCFG: 1 |  |
| OK |  |
| AT+KFTPRCV=1,,,"111111.txt",0 |  |
| CONNECT |  |
| 750aaaaaaaaa….. aaaaa250bbbbbbb--EOF--Pattern-- | Total of 760 data from the serial link |
| +KFTP_ERROR: 1,421 | The result code indicates that the download met with some problems which may be due to control or data connection lost |
| AT+KFTPRCV=1,,,"111111.txt",0,760 |  |
| CONNECT |  |
| --EOF--Pattern-- |  |
| +KFTP_ERROR: 1,502 | ERROR 502 means that some commands in the procedure are not supported by the server |

## HTTP Commands Examples

For the HL7618, HL7618RD, HL7648, HL7650, HL7690 and HL7692 only.

| AT&K3 | Hardware flow control activation |
| --- | --- |
| OK |  |
| AT+KCNXCFG=1,"GPRS","APN","log","password","0.0.0.0","0.0.0.0","0.0.0.0" | Set GPRS parameters (APN, login, password) |
| OK |  |
| AT+KCNXTIMER=1,60,2,70 | Set Timers |
| OK |  |
| AT+KHTTPCFG=1,"www.google.com",80,1 +KHTTPCFG: 1 | Set HTTP address, port number and http version |
| OK |  |
| AT+KHTTPHEADER=1 | Set the header of the request |
| CONNECT |  |
| Accept : text/html If-Modified-Since : Saturday, 15-January-2000 14:37:11 GMT | Send HTTP data after “CONNECT”. Data should end with the EOF string. |
| OK |  |
| AT+KHTTPGET=1, "/index.html" | Get the web page |
| CONNECT |  |
| HTTP/1.0 200 OK Cache-Control: private, max-age=0 Date: Tue, 24 Jun 2008 02:11:35 GMT Expires: -1 Content-Type: text/html; charset=ISO-8859-1 Set-Cookie: PREF=ID=ae1c663417e7799e:NW=1:TM=1214273495:LM=1214273495:S=5Uq9kExK4aTEv_cx; expires=Thu, 24-Jun-2010 02:11:35 GMT; path=/; domain=.google.com Server: gws Connection: Close <html><head><meta http-equiv="content-type" … a lot of data… --EOF--Pattern-- | HTTP server response |
| OK |  |
| AT+KHTTPHEAD=1, "/index.html" | Get the headers of the web page |
| CONNECT |  |
| HTTP/1.0 200 OK Cache-Control: private, max-age=0 Date: Tue, 24 Jun 2008 02:11:35 GMT Expires: -1 Content-Type: text/html; charset=ISO-8859-1 Set-Cookie: PREF=ID=ae1c663417e7799e:NW=1:TM=1214273495:LM=1214273495:S=5Uq9kExK4aTEv_cx; expires=Thu, 24-Jun-2010 02:11:35 GMT; path=/; domain=.google.com Server: gws Connection: Close | HTTP server response |
| OK |  |
| AT+KHTTPHEADER=1 | Send data to the HTTP server |
| CONNECT Accept : text/html Context-Length: 64 OK | Length of HTTP 1.0 POST data should be specified by the HTTP header field Context-Length, otherwise the HTTP server may not expect any data to be uploaded and should close the connection. |
| AT+KHTTPPOST=1,, "/get.cgi" |  |
| CONNECT |  |
| <…Data send…> | Send HTTP data after “CONNECT” |
| HTTP/1.0 200 OK Content-Type: text/plain Context-Length: 37 Your data have been accepted. --EOF--Pattern-- | HTTP server response |
| OK |  |

## Switch Data/Command Mode DTR +++ ATO Behavior Table

When the module is in data mode and the connection encounters an error, NO CARRIER terminal response is shown and the module is switched back to command mode.

The following table shows the behavior when trying to switch mode (when connection is running properly):

Case 1:	"+++" is used to switch from data mode to command mode, and the service is suspended.

Case 2:	If AT&D1 is set, "DTR drop" is used to switch from data mode to command mode, but the service is suspended.

Case 3:	If AT&D2 is set, "DTR drop" is used to switch from data mode to command mode, and the service is stopped.

Case 4:	If AT&D0 is set, "DTR drop" has no any impact on the mode switch.

Case 5:	ATO[n] is used to switch from command mode to data mode.

|  | Case1/Case5 +++/ATO[n] | Case2/Case5 DTR1/ATO[n] | Case3/Case5 DTR2/ATO[n] | Case4/Case5 DTR0 |
| --- | --- | --- | --- | --- |
| TCP/UDP: +KTCPSND: Send data +KTCPRCV: Receive data +KUDPSND: Send data +KUDPRCV: Receive data +KTCPSTART: Direct data flow | OK / CONNECT | OK / CONNECT | NO CARRIER / NO CARRIER (disconnect) | NO IMPACT |
| FTP: +KFTPRCV: Download FTP files +KFTPSND: Upload FTP files | OK / NO CARRIER (disconnect) | OK / NO CARRIER (disconnect) | NO CARRIER / NO CARRIER (disconnect) | NO IMPACT |
| HTTP: +KHTTPGET: Get information +KHTTPHEAD: Get head of information +KHTTPPOST: Send data +KHTTPHEADER: Set the HTTP request header | OK / NO CARRIER (disconnect) | OK / NO CARRIER (disconnect) | NO CARRIER / NO CARRIER (disconnect) | NO IMPACT |
| HTTPS: +KHTTPSGET: Get information +KHTTPSHEAD: Get head of information +KHTTPSPOST: Send data +KHTTPSHEADER: Set the HTTP request header | OK / NO CARRIER (disconnect) | OK / NO CARRIER (disconnect) | NO CARRIER / NO CARRIER (disconnect) | NO IMPACT |
| SSL: +KCERTSTORE: Store root CA +KPRIVKSTORE: Store private key | OK / NO CARRIER (abort) | OK / NO CARRIER (abort) | NO CARRIER / NO CARRIER (abort) | NO IMPACT |

Note the following:

When a data session is suspended, it can be resumed by ATO<session_id>.

For IP AT commands that supports <ndata>:

Data sessions completed with <ndata> are not suspended.

Data sessions with unspecified <ndata> (omitted in AT command) are suspended.

Data sessions with <ndata> but incomplete (less than <ndata> are sent/received) are suspended.

For IP AT commands that do not support <ndata> (e.g. +KTCPSTART), data sessions are suspended.

Only one data session can be suspended per <session_id>; the latest suspension clears the last suspension status. For example, the suspension of +KTCPRCV clears the suspension status of +KTCPSND such that ATO resumes +KTCPRCV instead of +KTCPSND.