AirPrime HL78xx

41111821

0.2

December 03, 2017

# AT Commands Interface Guide

Important Notice

Due to the nature of wireless communications, transmission and reception of data can never be guaranteed. Data may be delayed, corrupted (i.e., have errors) or be totally lost. Although significant delays or losses of data are rare when wireless devices such as the Sierra Wireless modem are used in a normal manner with a well-constructed network, the Sierra Wireless modem should not be used in situations where failure to transmit or receive data could result in damage of any kind to the user or any other party, including but not limited to personal injury, death, or loss of property. Sierra Wireless accepts no responsibility for damages of any kind resulting from delays or errors in data transmitted or received using the Sierra Wireless modem, or for failure of the Sierra Wireless modem to transmit or receive such data.

Safety and Hazards

Do not operate the Sierra Wireless modem in areas where cellular modems are not advised without proper device certifications. These areas include environments where cellular radio can interfere such as explosive atmospheres, medical equipment, or any other equipment which may be susceptible to any form of radio interference. The Sierra Wireless modem can transmit signals that could interfere with this equipment. Do not operate the Sierra Wireless modem in any aircraft, whether the aircraft is on the ground or in flight. In aircraft, the Sierra Wireless modem MUST BE POWERED OFF. When operating, the Sierra Wireless modem can transmit signals that could interfere with various onboard systems.

Some airlines may permit the use of cellular phones while the aircraft is on the ground and the door is open. Sierra Wireless modems may be used at this time.

The driver or operator of any vehicle should not operate the Sierra Wireless modem while in control of a vehicle. Doing so will detract from the driver or operator’s control and operation of that vehicle. In some states and provinces, operating such communications devices while in control of a vehicle is an offence.

Limitations of Liability

This manual is provided “as is”.  Sierra Wireless makes no warranties of any kind, either expressed or implied, including any implied warranties of merchantability, fitness for a particular purpose, or noninfringement.  The recipient of the manual shall endorse all risks arising from its use.

The information in this manual is subject to change without notice and does not represent a commitment on the part of Sierra Wireless. SIERRA WIRELESS AND ITS AFFILIATES SPECIFICALLY DISCLAIM LIABILITY FOR ANY AND ALL DIRECT, INDIRECT, SPECIAL, GENERAL, INCIDENTAL, CONSEQUENTIAL, PUNITIVE OR EXEMPLARY DAMAGES INCLUDING, BUT NOT LIMITED TO, LOSS OF PROFITS OR REVENUE OR ANTICIPATED PROFITS OR REVENUE ARISING OUT OF THE USE OR INABILITY TO USE ANY SIERRA WIRELESS PRODUCT, EVEN IF SIERRA WIRELESS AND/OR ITS AFFILIATES HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES OR THEY ARE FORESEEABLE OR FOR CLAIMS BY ANY THIRD PARTY.

Notwithstanding the foregoing, in no event shall Sierra Wireless and/or its affiliates aggregate liability arising under or in connection with the Sierra Wireless product, regardless of the number of events, occurrences, or claims giving rise to liability, be in excess of the price paid by the purchaser for the Sierra Wireless product.

Patents

This product may contain technology developed by or for Sierra Wireless Inc.

This product includes technology licensed from QUALCOMM®.

This product is manufactured or sold by Sierra Wireless Inc. or its affiliates under one or more patents licensed from MMP Portfolio Licensing.

Copyright

© 2017 Sierra Wireless. All rights reserved.

Trademarks

Sierra Wireless®, AirPrime®, AirLink®, AirVantage®, WISMO®, ALEOS® and the Sierra Wireless and Open AT logos are registered trademarks of Sierra Wireless, Inc. or one of its subsidiaries.

Watcher® is a registered trademark of NETGEAR, Inc., used under license.

Windows® and Windows Vista® are registered trademarks of Microsoft Corporation.

Macintosh® and Mac OS X® are registered trademarks of Apple Inc., registered in the U.S. and other countries.

QUALCOMM® is a registered trademark of QUALCOMM Incorporated.  Used under license.

Other trademarks are the property of their respective owners.

Contact Information

| Sales information and technical support, including warranty and returns | Web: sierrawireless.com/company/contact-us/ Global toll-free number: 1-877-687-7795 6:00 am to 6:00 pm PST |
| --- | --- |
| Corporate and product information | Web: sierrawireless.com |

Document History

| Version | Date | Updates |
| --- | --- | --- |
| 0.1 | August 03, 2017 | Creation |
| 0.2 | December 03, 2017 | CGEREP Default value set to “1” |

Contents

1.	Introduction	12

1.1.	Reference Configuration	12

1.2.	AT Command Principles	12

1.2.1.	Parameters	13

1.2.2.	Answers and Responses	13

1.2.3.	Multiple AT Commands on the Same Command Line	13

1.2.4.	AT Commands on Separate Lines	13

1.3.	Unsolicited Result Codes (URCs)	14

1.4.	Document Modification	14

1.5.	Abbreviations	14

2.	V25ter AT Commands	19

2.1.	+++ Command: Switch from Data Mode to Command Mode	19

2.2.	O Command: Switch from Command Mode to Data Mode	19

2.3.	E Command: Enable Echo Command	20

2.4.	S4 Command: Set Response Formatting Character	20

2.5.	&C Command: Set Data Carrier Detect (DCD) Function Mode	21

2.6.	&D Command: Set Data Terminal Ready (DTR) Function Mode	21

2.7.	&F Command: Restore Factory Settings	22

2.8.	&W Command: Save Stored Profile	22

2.9.	&V Command: Display Current Configuration	23

2.10.	&K Command: Flow Control Option	24

2.11.	&S Command: DSR Option	24

2.12.	IPR Command: Set Fixed Local/DTE Rate	25

3.	General AT Commands	26

3.1.	I Command: Request Identification Information	26

3.2.	Z Command: Reset and Restore User Configuration	29

3.3.	+CGMI Command: Request Manufacturer Identification	29

3.4.	+CGMM Command: Request Model Identification	29

3.5.	+CGMR Command: Request Revision Identification	30

3.6.	+CGSN Command: Request Product Serial Number Identification (IMEI)	31

3.7.	+KGSN Command: Request Product Serial Number and Software Version	31

3.8.	+HWREV Command: Request Hardware Revision	32

3.9.	+CSCS Command: Set TE Character Set	33

3.10.	+CIMI Command: Request International Mobile Subscriber Identity	34

3.11.	+GMI Command: Request Manufacturer Identification	34

3.12.	+GMM Command: Request Model Identification	35

3.13.	+GMR Command: Request Revision Identification	35

3.14.	+GSN Command: Request Product Serial Number (IMEI)	36

3.15.	+CMUX Command: Multiplexing Mode	36

3.16.	+GCAP Command: Request Complete TA Capability List	38

3.17.	+WIMEI Command: IMEI Write and Read	38

3.18.	&R Command: RTS and CTS Option	39

3.19.	\N Command: Data Transmission Mode	39

3.20.	B Command: Data Rate Selection	40

3.21.	S2 Command: Set Character for the Escape Sequence (Data to Command Mode)	40

4.	Call Control Commands	41

4.1.	D Command: Dial Number	41

4.2.	+CEER Command: Extended Error Report	42

5.	Mobile Equipment Control and Status Commands	43

5.1.	+CCLK Command: Real Time Clock	43

5.2.	+CLAC Command: List Available AT Commands	43

5.3.	+CFUN Command: Set Phone Functionality	44

5.4.	+CMER Command: Mobile Equipment Event Reporting	45

5.5.	+CMEE Command: Report Mobile Termination Error	48

5.6.	+CCID Command: Request SIM Card Identification	48

5.7.	+CPIN Command: Enter Pin	49

5.8.	+CPAS Command: Phone Activity Status	50

5.9.	+CSQ Command: Signal Quality	51

5.10.	+KCELL Command: Cell Environment Information	51

5.11.	+KGPIO Command: Hardware IO Control	54

5.12.	+KGPIOCFG Command: GPIO Configuration	55

5.13.	+KADC Command: Analog Digital Converter	57

5.14.	+CSIM Command: Generic SIM Access	58

5.15.	+CCHO Command: Open Logical Channel	58

5.16.	+CCHC Command: Close Logical Channel	59

5.17.	+CGLA Command: Generic UICC Logical Channel Access	60

5.18.	+CRLA Command: Restricted UICC Logical Channel Access	60

5.19.	+CUAD Command: UICC Application Discovery	61

5.20.	+CRSM Command: Restricted SIM Access	62

5.21.	+KTEMPMON Command: Temperature Monitor	64

5.22.	+CTZU Command: Automatic Time Zone Update	65

5.23.	+CTZR Command: Time Zone Reporting	66

5.24.	+KSLEEP Command: Power Management Control for UART	67

5.25.	+CESQ Command: Extended Signal Quality	68

5.26.	+KRIC Command: Ring Indicator Control	70

5.27.	+KBND Command: Current Networks Band Indicator	72

5.28.	+KSRAT Command: Set Radio Access Technology	72

5.29.	+CPOF Command: Power Off	73

5.30.	+KSREP Command: Mobile Start-up Reporting	74

5.31.	+WESHDOWN Command: Emergency Shutdown	75

6.	Network Service Related Commands	77

6.1.	+CLCK Command: Facility Lock	77

6.2.	+CNUM Command: Subscriber Number	78

6.3.	+COPN Command: Read Operator Name	79

6.4.	+COPS Command: Operator Selection	80

6.5.	+CPOL Command: Preferred PLMN List	81

6.6.	+CPWD Command: Change Password	82

6.7.	+CREG Command: Network Registration	83

6.8.	+CPLS Command: Select Preferred PLMN List	84

6.9.	+CEREG Command: EPS Network Registration Status	85

6.10.	+CEMODE Command: UE Modes of Operation for EPS	86

7.	Phone Book Management	87

7.1.	+CPBF Command: Find Phonebook Entries	87

7.2.	+CPBR Command: Read Current Phonebook Entries	88

7.3.	+CPBS Command: Select Phonebook Memory Storage	89

7.4.	+CPBW Command: Write Phonebook Entry	90

8.	SMS Commands	92

8.1.	Parameters Definition	92

8.1.1.	Message Storage Parameters	92

8.1.2.	Message Data Parameters	93

8.2.	+CMGD Command: Delete Message	94

8.3.	+CMGF Command: Set Message Format	95

8.4.	+CMGL Command: List Messages	96

8.5.	+CMGR Command: Read Message	96

8.6.	+CMGS Command: Send Message	97

8.7.	+CMGW Command: Write Message to Memory	98

8.8.	+CMSS Command: Send Message from Storage	99

8.9.	+CNMI Command: New Message Indication	99

8.10.	+CSCA Command: Service Center Address	101

8.11.	+CSMP Command: Set Text Mode Parameters	101

8.12.	+CSMS Command: Select Message Service	102

8.13.	+CPMS Command: Preferred Message Storage	103

8.14.	+CSDH Command: Show Text Mode Parameters	103

9.	Packet Domain Commands	106

9.1.	+CGATT Command: PS Attach or Detach	106

9.2.	+CGACT Command: Activate or Deactivate PDP Context	106

9.3.	+CGCMOD Command: Modify PDP Context	107

9.4.	+CGTFT Command: Traffic Flow Template	108

9.5.	+CGDCONT Command: Define PDP Context	109

9.6.	+CGDSCONT Command: Define Secondary PDP Context	112

9.7.	+CGEREP Command: Packet Domain Event Reporting	113

9.8.	+CGPADDR Command: Show PDP Address	114

9.9.	+CGSMS Command: Select Service for MO SMS Messages	115

9.10.	+XDNS Command: Dynamic DNS Request	116

10.	SIM Application Toolkit Commands	118

11.	Protocol Specific Commands	119

11.1.	Preliminary Comments	119

11.2.	IP Address Format in AT Commands	119

11.3.	Session ID	119

11.4.	Connection of PDP Contexts	119

11.5.	Buffer Length of AT Commands	119

11.6.	Parameter Format of AT Commands	120

11.7.	Connection Configuration	120

11.7.1.	+KCNXCFG Command: GPRS Connection Configuration	120

11.7.2.	+KCNXTIMER Command: Connection Timer Configuration	121

11.7.3.	+KCNXPROFILE Command: Current Profile Connection Configuration	122

11.7.4.	+KCGPADDR Command: Display PDP Address	123

11.7.5.	+KCNX_IND Notification: Connection Status Notification	124

11.7.6.	+KCNXUP Command: Bring the PDP Connection Up	124

11.7.7.	+KCNXDOWN Command: Bring the PDP Connection Down	125

11.8.	Common Configuration	126

11.8.1.	+KPATTERN Command: Custom End of Data Pattern	126

11.8.2.	+KURCCFG Command: Enable or Disable the URC from Protocol Commands	127

11.8.3.	+KIPOPT Command: General Options Configuration	128

11.9.	SSL Configuration	130

11.9.1.	+KSSLCRYPTO Command: Cipher Suite Configuration	130

11.9.2.	+KSSLCFG Command: SSL Configuration	131

11.10.	TCP Specific Commands	133

11.10.1.	+KTCPCFG Command: TCP Connection Configuration	133

11.10.2.	+KTCPCNX Command: Start TCP Connection	134

11.10.3.	+KTCPRCV Command: Receive Data through a TCP Connection	135

11.10.4.	+KTCPSND Command: Send Data through a TCP Connection	136

11.10.5.	+KTCPCLOSE Command: Close Current TCP Operation	137

11.10.6.	+KTCPDEL Command: Delete a Configured TCP Session	137

11.10.7.	+KTCP_SRVREQ Notification: Incoming Client Connection Request	138

11.10.8.	+KTCP_DATA Notification: Incoming Data through a TCP Connection	139

11.10.9.	+KTCP_IND Notification: TCP Status	140

11.10.10.	+KTCPSTAT Command: Get TCP Socket Status	140

11.10.11.	+KTCPSTART Command: Start a TCP Connection in Direct Data Flow	141

11.10.12.	+KTCP_ACK Notification: Status Report for Latest TCP Data	142

11.10.13.	+KTCPACKINFO Command: Poll ACK Status for the Latest Data	142

11.11.	UDP Specific Commands	143

11.11.1.	+KUDPCFG Command: UDP Connection Configuration	143

11.11.2.	+KUDPRCV Command: Receive Data through a UDP Connection	145

11.11.3.	+KUDPSND Command: Send Data through a UDP Connection	146

11.11.4.	+KUDPCLOSE Command: Close Current UDP Operation	147

11.11.5.	+KUDP_IND Notification: UDP Status	148

11.11.6.	+KUDP_DATA Notification: Incoming Data through a UDP Connection	148

11.12.	SSL Certificate Manager	149

11.12.1.	+KCERTSTORE Command: Store Root CA and Local Certificates to Internal Storage	149

11.12.2.	+KPRIVKSTORE Command: Store Private Key Associated to a Local Certificate	150

11.12.3.	+KCERTDELETE Command: Delete Local Certificate from the Index	151

11.12.4.	+KPRIVKDELETE Command: Delete Private Key from the Index	152

12.	AVMS Commands	153

12.1.	+WDSC Command: Device Services Configuration	153

12.2.	+WDSD Command: Device Services Local Download	155

12.3.	+WDSE Command: Device Services Error	156

12.4.	+WDSG Command: Device Services General Status	157

12.5.	+WDSI Command: Device Services Indication	158

12.6.	+WDSR Command: Device Services Reply	161

12.7.	+WDSS Command: Device Services Session	162

13.	Test Commands	166

13.1.	+WMTXPOWER Command: Test RF Tx	166

13.2.	+WMRXPOWER Command: Test RF Rx	169

14.	NV Related Commands	173

14.1.	Auto Generation of NV Backup Files	173

14.2.	Auto Recovery from Backup NV Files	173

14.3.	+NVBU Command: NV Backup Status and Control	174

14.4.	+NVBU_IND Notification: NV Backup Status Notification	176

15.	Appendix	177

15.1.	Result Codes and Unsolicited Messages	177

15.2.	Error Codes	178

15.2.1.	CME Error Codes	178

15.2.2.	CEER Error Codes	180

15.2.2.1.	SS Error Codes	185

15.2.3.	CMS Error Codes	189

15.2.4.	GPRS Error Codes	191

15.2.5.	AVMS Error Codes	192

15.2.6.	Error Case Examples	192

15.3.	Commands without Pin Code Requirement	195

15.4.	TCP Commands Examples	196

15.4.1.	Client Mode	196

15.4.2.	Server Mode	197

15.4.3.	Polling for the Status of a Socket	198

15.4.4.	End to End TCP Connection	199

15.4.5.	Error Case for End to End TCP Connection	200

15.4.6.	Use Cases for AT+KTCPACKINFO and <URC-ENDTCP-enable> Option	200

15.4.6.1.	<URC-ENDTCP-enable> is Disabled (default setting)	200

15.4.6.2.	<URC-ENDTCP-enable> is Enabled	201

15.5.	UDP Commands Examples	202

15.5.1.	Client Mode	202

15.5.2.	Server Mode	203

15.5.3.	Use Cases for KTCP_DATA and KUDP_DATA	204

15.5.3.1.	KTCP_DATA and KUDP_DATA without Data Auto Retrieval – Client Mode	204

15.5.3.2.	KTCP_DATA and KUDP_DATA without Data Auto Retrieval – Server Mode	204

15.5.3.3.	KTCP_DATA and KUDP_DATA with Data Auto Retrieval – Client Mode	205

15.5.3.4.	KTCP_DATA and KUDP_DATA with Data Auto Retrieval – Server Mode	206

15.6.	Switch Data/Command Mode DTR +++ ATO Behavior Table	206

List of Tables

Table 1.	Types of Extended AT Commands	16

Table 2.	Tag 128 MN_GENERAL_PROBLEM with Causes	192

Table 3.	Tag 129 MN_INVOKE_PROBLEM with Causes	193

Table 4.	Tag 130 MN_RETURN_RESULT_PROBLEM with Causes	193

Table 5.	Tag 131 MN_RETURN_ERROR_PROBLEM with Causes	193

Table 6.	Generic Error Case Examples	196

Table 7.	Non-Generic Error Case Examples	197

Table 8.	Internet Error Case Examples	199

# Introduction

This document presents the AT Command Set for the AirPrime HL78xx series of embedded modules. AirPrime HL78xx variants covered in this manual are:

HL7800

## Reference Configuration

Reference Configuration

The User Equipment (UE) consists of the mobile equipment (ME) and the (U)SIM messages may be stored in either, but the present document does not distinguish between messages stored in the (U)SIM or in the ME. The management of message storage in the two parts of the UE is a matter for the UE implementation.

## AT Command Principles

The "AT" or "at" prefix must be set at the beginning of each line. To terminate a command line, a <CR> character must be inserted.

Commands are usually followed by a response that includes ‘<CR><LF><response><CR><LF>’. Throughout this document, only the responses are indicated, the <CR> and <LF> characters are omitted intentionally.

Four kinds of extended AT commands are implemented as listed in the table below.

Types of Extended AT Commands

| Command Type | Syntax | Definition |
| --- | --- | --- |
| Test Command | AT+CXXX=? | The equipment returns the list of parameters and values ranges set with the corresponding Write command or by internal processes |
| Read Command | AT+CXXX? | This command returns the currently set value of parameters |
| Write Command | AT+CXXX=<…> | This command sets user-related parameter values |
| Execution command | AT+CXXX | The execution command reads non-variable parameters affected by internal processes in the equipment |

### Parameters

In this document, the default parameters are underlined and the optional parameters are enclosed in square brackets.

Optional parameters or sub-parameters can be omitted unless they are followed by other parameters. A parameter in the middle of a string can be omitted by replacing it with a comma.

When the parameter is a character string, the string must be enclosed in quotation marks.

All space characters will be ignored when using strings without quotation marks.

### Answers and Responses

There is always an answer sent by the TA to an AT Command line (except the very special case of a TA setup for no answer, see ATQ).

The answer is always terminated by an indication of success or failure. However, regarding the setup of the TA (by AT Commands), the message may be different.

Classical messages	OK or ERROR

Extended Error message (see AT+CMEE)	+CME ERROR: <n>

(See Appendix for the different values for <n>)

Numeric Mode (see ATV)	<n> with: <n> = 0  OK or <n> is an error code

### Multiple AT Commands on the Same Command Line

You may enter several AT commands on the same line. This eliminates the need to type the "AT" or "at" prefix before each command and to wait for the answer for each command. The main advantage is to avoid losing bandwidth on the link between DTE and the Module.

There is no separator between two basic commands but a semi-colon character is necessary between two extended commands (prefix +).The command line buffer accepts a maximum of 391 characters. If this number is exceeded none of the commands will be executed and TA returns ERROR.

If a command is not supported, then the treatment of the line is stopped (i.e. the following ones are not treated) and an error message is returned.

Example:

Command:	ATZE1+CBST=7,0,1;+CBST?

Answer:	+CBST=7,0,1

OK

### AT Commands on Separate Lines

When you enter a series of AT commands on separate lines, it is strongly advised to leave a pause between the preceding and the following command until the final answer (OK or Error message) appears. This avoids sending too many AT commands at a time without waiting for a response for each.

## Unsolicited Result Codes (URCs)

Unsolicited result codes (URCs) are sent simultaneously to all the channels (UART) configured in AT commands mode.

URCs are not sent to channels configured in Data/ Traces modes.

In sleep mode URCs wake up the module and are sent to the AT commands channels.

## Document Modification

The commands described in this document are only to be used for usual AT commands use.

The information provided for the commands are subject to change without notice.

## Abbreviations

| Abbreviation | Definition |
| --- | --- |
| ACM | Accumulated Call Meter |
| ADC | Analog Digital Converter |
| ADN | Abbreviated Dialing Number (Phonebook) |
| AMR | Adaptive Multi-Rate |
| AMR-FR | AMR Full Rate (full rate speech version 3) |
| AMR-HR | AMR Half Rate (half rate speech version 3) |
| AOC | Advice Of Charge |
| APN | Access Point Name |
| ARN | Address Resolution Protocol |
| ARFCN | Absolute Radio Frequency Channel Number |
| ASCII | American Standard Code for Information Interchange |
| AT | ATtention; Hayes Standard AT command Set |
| BCCH | Broadcast Channel |
| BER | Bit Err Rate |
| BM | Broadcast Message Storage |
| CBM | Cell Broadcast Message |
| CB | Cell Broadcast |
| CCK | Corporate Control Key |
| CCM | Current Call Meter |
| CHV | Card Holder Verification |
| CHAP | Challenge handshake Authentication Protocol |
| CI | Cell Identifier |
| CLI | Client Line Identification |
| CNL | Cooperative Network List |
| CODEC | Coder Decoder |
| COLP | Connected Line Identification Presentation |
| CPHS | Common PCN Handset Specification |
| CPU | Central Processing Unit |
| CSD | Circuit Switched Data |
| CSP | Customer Service Profile |
| CTM | Cellular Text telephone Modem |
| CTS | Clear To Send signal |
| CUG | Closed User Group |
| DAC | Digital to Analog Converter |
| DCS | Digital Cellular System |
| DCE | Data Circuit Equipment |
| DCD | Data Carrier Detect |
| DLC | Data Link Connection |
| DLCI | Data Link Connection Identifier |
| DM | Device Management |
| DNS | Domain Name System |
| DSR | Data Set Ready |
| DTE | Date Terminal Equipment |
| DTMF | Dual Tone Multi-Frequency |
| DTR | Data Terminal Ready |
| ECC | Emergency Call Codes |
| ECM | Error Correction Mode |
| ECT | Explicit Call Transfer |
| EDGE | Enhanced Data rates for GSM Evolution |
| EEPROM | Electrically Erasable Programming Only Memory |
| EF | Elementary Files |
| EFR | Enhanced Full Rate (full rate speech version 2) |
| EGPRS | Enhanced GPRS |
| ENS | Enhanced Network Selection |
| E-ONS | Enhanced Operator Name Service |
| ERMES | European Radio Messaging System |
| ETSI | European Telecommunications Standards Institute |
| FD | FIFO depth |
| FDN | Fixed Dialing Number (Phonebook) |
| FR | Full Rate (full rate speech version 1) |
| GERAN | GSM EDGE Radio Access Network |
| GPIO | General Purpose Input Output |
| GPRS | General Packet Radio Service |
| GSM | Global System for Mobile communication |
| HDLC | High-level Data Link Control |
| HFR | High Frequency Regeneration |
| HLR | Home Location Register |
| HR | Half Rate (half rate speech version 1) |
| ID | IDentifier |
| IETF | Internet Engineering Task Force |
| IMEI | International Mobile Equipment Identity |
| IMSI | International Mobile Subscriber Identity |
| IN/OUT/IN_OUT | In, out or In Out |
| I/O | Input/Output |
| IP | Internet Protocol |
| LAC | Local Area Code |
| LED | Light Emitting Diode |
| LND | Last Number Dialed |
| LP | Language Preferred |
| LPI | Lines Per Inch |
| M | Mandatory |
| MCC | Mobile Country Code |
| ME | Mobile Equipment |
| MMI | Man Machine Interface |
| MNC | Mobile Network Code |
| MNP | Microcom Networking Protocol |
| MO | Mobile Originated |
| MOC | Mobile Originated Call (outgoing call) |
| MS | Mobile Station |
| MSB | Most Significant Bit |
| MSISDN | Mobile Station International ISDN Number |
| MT | Mobile Terminal |
| MTC | Mobile Terminated Call (incoming call) |
| N.A. | Not applicable |
| NCK | Network Control Key |
| NITZ | Network Information and Time Zone |
| NSCK | Network Subset Control Key |
| NTC | Negative Temperature Coefficient |
| N.U. | Not used |
| O | Optional |
| OA | Outgoing Access |
| OPL | Operator PLMN List |
| OS | Operating System |
| OTA | Over the Air |
| PAD | Portable Application Description |
| PAP | Password Authentication Protocol |
| PC | Personal Computer |
| PCCP | PC character set Code Page |
| PCK | Personalization Control Key |
| PCL | Power Control Level |
| PCM | Protection Circuit Module |
| PCN | Personal Communication Network |
| PCS 1900 | Personal Communication Service |
| PDP | Packet Data Protocol |
| PDU | Protocol Description Unit |
| PIN | Personal Identification Number |
| PLMN | Public Land Mobile Networks |
| PNN | PLMN Network Name |
| PPP | Point-to-Point Protocol/Peer to Peer |
| PSTN | Public Switched Telephone Network |
| PTS | Product Technical Specification |
| PUCT | Price per Unit and Currency Table |
| PUK | PIN Unlock Key |
| PWM | Pulse Width Modulation |
| QoS | Quality of Service |
| RAM | Random Access Memory |
| RDMS | Remote Device Management Services |
| RI | Ring Indicator |
| RIL | Radio Interface Layer |
| RLP | Radio Link Protocol |
| RSSI | Received Signal Strength Indication |
| RTS | Ready To Send signal |
| RX | Reception |
| SAP | Service Access Point |
| SC | Service Center |
| SDU | Service Data Unit |
| SIM | Subscriber Information Module |
| SMSR | Short Message Status Report |
| SMS | Short Message Service |
| SS | Supplementary Services |
| SPCK | Service Provider Control Key |
| SPN | Service Provider Name |
| STK | SIM ToolKit |
| SVN | Software Version Number |
| TA | Terminal Adaptor |
| TBF | Temporary Block Flow |
| TE | Terminal Equipment |
| TTY | TeleTYpe |
| TON/NPI | Type Of Number/Numbering Plan Identification |
| TX | Transmission |
| UART | Universal Asynchronous Receiver Transmitter |
| UCS2 | Universal Character Set 2 Character table (2-byte coding) |
| UDUB | User Determined User Busy |
| UIH | Unnumbered Information with Header check |
| USB | Universal Serial Bus |
| USSD | Unstructured Supplementary Service Data |

# V25ter AT Commands

## +++ Command: Switch from Data Mode to Command Mode

| HL7800 |  |
| --- | --- |
| Execute command Syntax +++ | Response OK |
| Reference V.25Ter | Notes - This command is only available during data mode. The +++ character sequence suspends the data flow over the AT interface and switches to command mode. This allows entering AT commands while maintaining the data connection to the remote device. - To return to data mode, use the ATO[n] command. - Line needs one second silence before and one second after (do not end with terminating character). - The "+" character may be changed with the ATS2 command (see following chapters). - The +++ characters are not transmitted in the data flow. |

## O Command: Switch from Command Mode to Data Mode

| HL7800 |  |
| --- | --- |
| Test command Syntax ATO[<n>] | Response TA returns to data mode from command mode: CONNECT <text> If connection is not successfully resumed: NO CARRIER Parameter <n> 0 Switch from command mode to data mode 1 – 200 Session ID |
| Reference V.25Ter | Notes ATO is the alternative command to the +++ escape sequence described in section 2.1. When a data call has been established and TA is in command mode, ATO causes the TA to resume the data connection and return to data mode. |

## E Command: Enable Echo Command

| HL7800 |  |
| --- | --- |
| Execute command Syntax ATE[<value>] | Response OK or +CME ERROR: <err> Parameter <value> 0 Echo OFF 1 Echo ON |
| Notes | - This setting determines whether or not the TA echoes characters received from the TE in the command state. - <value> is set for all AT ports. |

## S4 Command: Set Response Formatting Character

| HL7800 |  |
| --- | --- |
| Read command Syntax ATS4? | Response <n> OK |
| Write command Syntax ATS4=<n> | Response OK Parameter <n> 10 Response formatting character <LF>: line feed |
| Notes | This parameter determines the character recognized by TA to terminate answer line (10 = <LF> by default); it cannot be changed. |

## &C Command: Set Data Carrier Detect (DCD) Function Mode

| HL7800 |  |
| --- | --- |
| Execute command Syntax AT&C<value> | Response OK Parameter <value> 0 DCD line is always active 1 DCD line is active in the presence of data carrier only |
| Reference V.25Ter | Notes DCD/AT&C is only applicable to the USB AT port; it has no effect on UART1. |

## &D Command: Set Data Terminal Ready (DTR) Function Mode

| HL7800 |  |
| --- | --- |
| Execute command Syntax AT&D<value> | Response OK or +CME ERROR: <err> Parameter <value> 0 TA ignores status on DTR 1 DTR drops from active to inactive. Change to command mode while retaining the connected data call 2 DTR drops from active to inactive. Disconnect data call, change to command mode. Auto-answer is off during DTR inactive state |
| Reference V.25Ter | Notes - This command only applies to data calls. - DTR/AT&D is only applicable to the USB AT port; it has no effect on UART1. |

## &F Command: Restore Factory Settings

| HL7800 |  |
| --- | --- |
| Execute command Syntax AT&F[<value>] | Response OK Parameter <value> 0 or Omitted Restore STORED PROFILE 0 and 1 to factory settings |
| Reference V.25Ter | Notes This command also restores the factory settings to the active profile. |
| Examples | AT&F OK AT&F0 OK AT&F1 ERROR |

## &W Command: Save Stored Profile

| HL7800 |  |
| --- | --- |
| Execute command Syntax AT&W[<value>] | Response OK Parameters <value> 0 or Omitted Save in STORED PROFILE 0 1 Save in STORED PROFILE 1 |
| Reference V.25Ter | Notes - This command saves the current configuration in a non-erasable place. - &R, S05, S06 and S08 have no effect. Their parameters are not saved in non-volatile memory. |
| Examples | AT&W // Save current configuration to Profile 0 OK AT&W0 // Save current configuration to Profile 0 OK AT&W1 // Save current configuration to Profile 1 OK |

## &V Command: Display Current Configuration

| HL7800 |  |
| --- | --- |
| Execute command Syntax AT&V[<value>] | Response ACTIVE PROFILE: <current configuration> STORED PROFILE 0: <user default configuration> STORED PROFILE 1: <manufacturer configuration> OK Parameter <value> 0 Profile number This command indicates the result of certain actions as shown below: Active Profile Stored profile Default Settings ATZ AT&F AT&W |
| Reference Sierra Wireless Proprietary | Notes - At startup, the latest profile stored with AT&W is restored to the Active profile (no restoration if AT&W has not been used). - The configuration is a text string on multiple lines as shown in the example below. This string may vary depending on the manufacturer, the product and the user setup. - Registers S05, S06 and S08 have no effect. They are only implemented for compliance with V.25ter. Their parameters are always read as values 8, 2 and 2 respectively. - &R has no effect and it is not defined in the V.25ter specification. It is only implemented here for compatibility purposes. Its parameter is always read as 1. |
| Example | AT&V ACTIVE PROFILE: E1 Q0 V1 X0 &C1 &D1 &R:1 &S0 &K0 +IPR:115200 +FCLASS0 S00:0 S01:0 S04:10 S05:8 S06:2 S07:255 S08:2 STORED PROFILE 0: E1 Q0 V1 X0 &C1 &D1 &R:1 &S0 &K3 +IPR:115200 +FCLASS0 S00:0 S01:0 S04:10 S05:8 S06:2 S07:255 S08:2 STORED PROFILE 1: E1 Q0 V1 X0 &C1 &D1 &R:1 &S0 &K3 +IPR:115200 +FCLASS0 S00:0 S01:0 S04:10 S05:8 S06:2 S07:255 S08:2 OK |

## &K Command: Flow Control Option

| HL7800 |  |
| --- | --- |
| Execute command Syntax AT&K[<mode>] | Response OK Parameter <mode> 0 or omitted Disable all flow control 3 Enable bi-directional hardware flow control |
| Reference V.25ter | Notes - Use AT&V0 to display the current flow control setting. - Sierra Wireless recommends the use of hardware flow control. - AT&K3 hardware flow control is only effective for UART1 and +KSLEEP=2 (UART always ON); it has no effect on the USB AT port. - This command is not supported in MUX mode; flow control is not supported in the DLC channel. |

## &S Command: DSR Option

| HL7800 |  |
| --- | --- |
| Write command Syntax AT&S [<override>] | Response OK Parameter <override> 0 or omitted DSR signal is always ON (0 is the default value) 1 DSR signal is always OFF |
| Reference V.25ter | Notes This is a dummy command and has no effect on the DSR signal. |

## +IPR Command: Set Fixed Local/DTE Rate

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+IPR=? | Response With Autobaud: +IPR: (list of supported auto detectable <baud_rate>s)[,(list of fixed only <baud_rate>s)] OK Without Autobaud: +IPR: ()[,(list of fixed only <baud_rate>s)] OK |
| Read command Syntax AT+IPR? | Response +IPR: <baud_rate> OK |
| Write command Syntax AT+IPR= <baud_rate> | Response OK or +CME ERROR: <err> Parameter <baud_rate> 115200 (default value) 0 (autobaud), 300, 1200, 2400, 4800, 9600, 19200, 38400, 57600, 230400, 460800, 500000, 750000, 921600, 1843200, 3000000, 3250000, 3686400, 4000000, 6000000 |
| Notes | - Not all listed rates may be available as they depend on the target. - The full range of data rate values may be reduced depending on hardware or other criteria. - <baud_rate> is saved in non-volatile memory per AT port over module reboot when AT&W is entered. It is not automatically saved after a +IPR write command. - After autobaud is activated on a UART port, the next AT command can be entered at a different speed. +IPR is then set to the speed of this command. Autobaud is then automatically deactivated. - When autobaud is activated on a USB COM port, any speed provided by the USB driver is accepted. AT+IPR? responds with +IPR: 0 regardless of USB speed used. |

# General AT Commands

## I Command: Request Identification Information

| HL7800 |  |
| --- | --- |
| Execute command Syntax ATI[<value>] | Response If <value> = 0 or omitted: <model> OK If <value> = 1: <short version name> OK If <value> = 3: <version name> OK If <value> = 4: <fuse state> OK If <value> = 9: <version name> <model> <short version name> <chipset> <fuse state> <build date & time> <source rev> OK If <value> = 10: Modem-Firmware: <version name> <model> <short version name> <chipset> <fuse state> <build date & time> <source rev> Primary-Boot: <version name> <build date & time> <source rev> Secondary-Boot: <version name> <build date & time> <source rev> Update-Agent: <version name> <build date & time> <source rev> 4G-Firmware: <4G FW version name> 3G-Firmware: <3G FW version name> OK Parameters <model> Model identifier <version name> Firmware version string HL7618, HL7618RD, HL7688, HL7690 and HL7692 follow the format: AHL75xx_TEST.0.0.141506 <…> (test firmware) AHL75xx.1.0.141506.<…> (official firmware) HL7648 and HL7650 follow the format: <prefix>.<major no.>.<minor no.>.<baseline no.>.<date-time>.<integration> of size: <variable, up to 32 characters>.<2digits>.<2digits >.<6digits >.<12digits>.<2digits> Note that test firmware will have <major no.>.<minor no.> = 00.00. For example: SWIMCB71XX-G.00.00.163500.201609231719.01 (test firmware) SWIMCB71XX-G.01.00.163500.201609231719.01 (official firmware) <short version name> Firmware version string in short format (without date and time) For example: HL75xx_TEST.0.0 (test firmware) HL75xx.1.0 (official firmware) <4G FW version name> 4G Firmware version string <3G FW version name> 3G Firmware version string <chipset> Chipset name <build date & time> Firmware build time in format YYYY-MM-DD HH:MM:SS <source rev> Source code revision in version control <fuse state> Fuse state information FUSED Fused module with secure boot NON-FUSED Non-fused module |
| Reference V.25ter | Notes - ATI3 is identical to AT+GMR and AT+CGMR. - ATI0 and ATI are identical to AT+GMM and AT+CGMM. |
| Examples | ATI HL7618 //When using an HL7618 module OK ATI0 HL7618 //When using an HL7618 module OK # For fused modules ATI4 FUSED OK # For non-fused modules ATI4 NON-FUSED OK # Examples on a test firmware for HL7648/HL7650 ATI1 SWIMCB71XX-G.00.00.163500 OK ATI3 SWIMCB71XX-G.00.00.163500.201609261356.01 OK # Example of a test firmware with TEST as the version name ATI3 BHL7618_TEST.0.0.154401.201511132200.x7120_2 OK # Examples on official firmware ATI1 HL7618.3.0 //When using an HL7618 module OK ATI3 BHL7618.3.0.154401.201511132200.x7120_2 OK |

## Z Command: Reset and Restore User Configuration

| HL7800 |  |
| --- | --- |
| Execute command Syntax ATZ[<value>] | Response OK or +CME ERROR: <err> Parameter <value> 0 Reset and restore user configuration with profile 0 1 Reset and restore user configuration with profile 1 |

## +CGMI Command: Request Manufacturer Identification

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CGMI=? | Response OK |
| Execute command Syntax AT+CGMI | Response (manufacturer identification text) OK |
| Reference [27.007] § 5.1 | Note This command is identical to AT+GMI. |
| Example | AT+CGMI Sierra Wireless OK |

## +CGMM Command: Request Model Identification

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CGMM=? | Response OK |
| Execute command Syntax AT+CGMM | Response <mode> OK Parameter <model> Model identifier |
| Reference [27.007] § 5.2 | Note This command is identical to AT+GMM, ATI and ATI0. |
| Example | AT+CGMM HL7800 //When using an HL7800 module OK |

## +CGMR Command: Request Revision Identification

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CGMR=? | Response OK |
| Execute command Syntax AT+CGMR | Response (model revision identification text) OK |
| Reference [27.007] § 5.3 | Note This command is identical to ATI3 and AT+GMR. |
| Examples | AT+CGMR ? // test HL7800 firmware OK AT+CGMR ? // test HL7800 firmware OK |

## +CGSN Command: Request Product Serial Number Identification (IMEI)

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CGSN=? | Response OK |
| Execute command Syntax AT+CGSN | Response <IMEI> (identification text for determination of the individual ME) OK |
| Reference V.25ter | Notes - This command is identical to AT+GSN. - This command can work with or without a SIM. - See also AT+KGSN. |

## +KGSN Command: Request Product Serial Number and Software Version

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KGSN=? | Response +KGSN: (list of supported <number type>s) OK |
| Write command Syntax AT+KGSN= <number type> | Response If <number type> = 0: +KGSN: <IMEI> OK If <number type> = 1: +KGSN: <IMEISV> OK If <number type> = 2: +KGSN: <IMEISV_STR> OK If <number type> = 3: +KGSN: <FSN> OK If <number type> = 4: +KGSN: <FSN-BB> OK Parameters <IMEI> 15 digits IMEI (8 digits for TAC + 6 digits for SNR + 1 check digit) <IMEISV> 16 digits IMEISV (8 digits for TAC + 6 digits for SNR + 2 SVN digits) <IMEISV_STR> Formatted string; <15 digits>-<Check digit> SV: <Software version> <FSN> 14 digits Serial Number <FSN-BB> 16 digits Serial Number + BB |
| Reference Sierra Wireless Proprietary | Notes This command has been developed to provide the IMEI SV and Serial Number through an AT command and it can work without a SIM. |
| Examples | AT+KGSN=0 +KGSN: 351578000023006 OK AT+KGSN=1 +KGSN: 3515780000230001 OK AT+KGSN=2 +KGSN: 35157800002300-6 SV:01 OK AT+KGSN=3 +KGSN: 0123456789ABCD OK AT+KGSN=4 +KGSN: 0123456789ABCD01 OK |

## +HWREV Command: Request Hardware Revision

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+HWREV=? | Response OK |
| Read command Syntax AT+HWREV? | Response Hardware revision: X.Y OK Parameter X.Y These are the HH numbers in FSN (returned by TTYWWDNNNNPPHH-BB) |
| Reference Sierra Wireless Proprietary | Notes This command works with or without a SIM. |
| Example | Assuming FSN=TTYWWDNNNNPP01-BB AT+HWREV? Hardware revision: 0.1 OK |

## +CSCS Command: Set TE Character Set

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CSCS=? | Response +CSCS: (list of supported <vail>s) OK |
| Read command Syntax AT+CSCS? | Response +CSCS: <vail> OK or +CME ERROR: <err> |
| Write command Syntax AT+CSCS= [<vail>] | Response OK or +CME ERROR: <err> Parameter <vail> "GSM" GSM default alphabet (3GPP TS 23.038) "HEX" Character strings only consist of hexadecimal numbers from 00 to FF. For example, “032FE6” equals three 8-bit characters with decimal values 3, 47 and 230. No converstions to the original MT character set shall be done "IRA" International reference alphabet (ITU-T T.50) "UCS2" 16-bit universal multiple-octet coded character set (ISO/IEC 10646) |
| Notes | <vail> is saved in non-volatile memory per AT port over module reboot. |

## +CIMI Command: Request International Mobile Subscriber Identity

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CIMI=? | Response OK |
| Execute command Syntax AT+CIMI | Response <IMSI> OK or +CME ERROR: <err> Parameter <IMSI> International Mobile Subscriber Identity |

## +GMI Command: Request Manufacturer Identification

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+GMI=? | Response OK |
| Execute command Syntax AT+GMI | Response (manufacturer identification text) OK |
| Reference [27.007] § 5.1 | Note This command is identical to AT+CGMI. |
| Example | AT+GMI Sierra Wireless OK |

## +GMM Command: Request Model Identification

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+GMM=? | Response OK |
| Execute command Syntax AT+GMM | Response <model> OK Parameter <mode> Model identifier |
| Reference [27.007] § 5.2 | Note This command is identical to AT+CGMM, ATI and ATI0. |
| Example | AT+GMM HL7800 //When using an HL7800 module OK |

## +GMR Command: Request Revision Identification

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+GMR=? | Response OK |
| Execute command Syntax AT+GMR | Response (model revision identification text) OK |
| Reference [27.007] § 5.3 | Note This command is identical to ATI3 and AT+CGMR. |
| Examples | AT+CGMR ? OK AT+CGMR ? OK |

## +GSN Command: Request Product Serial Number (IMEI)

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+GSN=? | Response OK |
| Execute command Syntax AT+GSN | Response <IMEI> (identification text for determination of the individual ME) OK |
| Reference V.25ter | Notes - This command is identical to AT+CGSN. - This command can work with or without a SIM. - See also AT+KGSN. |

## +CMUX Command: Multiplexing Mode

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CMUX=? | Response +CMUX: (list of supported <mode>s),(list of supported <subset>s),(list of supported <port_speed>s),(list of supported <N1>s),(list of supported <T1>s),(list of supported <N2>s),(list of supported <T2>s),(list of supported <T3>s),(list of supported <k>s) OK |
| Read command Syntax AT+CMUX? | Response +CMUX: <mode>,<subset>,<port_speed>,<N1>,<T1>,<N2>,<T2>,<T3>,<k> OK or +CME ERROR: <err> OK |
| Write command Syntax AT+CMUX= <mode> [,<subset> [,<port_speed> [,<N1>[,<T1> [,<N2>[,<T2> [,<T3>[,<k>]]]]]]]] | Response OK or +CME ERROR: <err> OK Parameters <mode> Multiplexer transparency mechanism 0 Basic option 1 Advanced option (not supported) <subset> 0 UIH frames used only 1 UI frames used only (not supported) 2 I frames used only (not supported) <port_speed> Transmission rate 1 9 600 bit/s 2 19 200 bit/s 3 38 400 bit/s 4 57 600 bit/s 5 115 200 bit/s 6 230 400 bit/s 7 1 Mbit/s <N1> 1 – 1509 Maximum frame size. Default value = 31 (64 if Advanced option is used) <T1> 1 – 255 Acknowledgement time in units of ten milliseconds. Default value = 10 (100 ms) <N2> 0 – 5 Maximum number of re-transmissions. Default value = 3 <T2> 2 – 255 Response time for the multiplexer control channel in units of ten milliseconds. Default value = 30 (300 ms). Note that <T2> must be longer than <T1>. <T3> 1 – 255 Wake up response timer in seconds. Currently not supported; in case of read command, 0 is returned. <k> 1 – 7 Window size for advanced operation with error recovery options. Currently not supported; in case of read command, 0 is returned. |
| Notes | - This command enables the multiplexing protocol control channel as defined in GSM07.10. - The AT command sets parameters for the Control Channel. If parameters are left out the default values are used. If no autobauding is supported, a customer related interface speed is pre selected. The final response code OK or CME ERROR: <err> is returned using the old interface speed; the parameters become active only after sending OK. - The "+++" escape sequence is not supported in the DLC port in CMUX mode. Alternatively, DTR can be used to switch from data mode to command mode, or use another DLC port to send AT commands. - The module handles the frame data step by step in CMUX mode. If there are any wrong data in the frame, e.g. wrong CRC, nothing will be returned to the terminal, and the module will wait for a valid frame data. - If the AT+CFUN command is entered with <rst>=1, all open CMUX channels will be closed and the module will reset. - There is no activity timeout to return to AT mode after entering MUX mode. - MUX DLC ports are not persistent over power cycles. After a power cycle, DLC ports need to be re-established. - When an established MT call is hanged up from the caller side, NO CARRIER will only be sent to the port on which the call was established (i.e. the port on which ATD/ATA was sent). |

## +GCAP Command: Request Complete TA Capability List

| HL7800 |  |
| --- | --- |
| Execute command Syntax AT+GCAP | Response +GCAP: list of <name>s OK |
| Example | The response code is CLTE1, CLTE3 or CLTE4 depending on the configured LTE category. |

## +WIMEI Command: IMEI Write and Read

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+WIMEI=? | Response OK |
| Read command Syntax AT+WIMEI? | Response +WIMEI: <IMEI> OK |
| Write command Syntax AT+WIMEI= <IMEI> | Response +WIMEI: <IMEI> OK Parameter <IMEI> 14 or 15-digit IMEI as defined in GSM 23.003 |
| Notes | - The default IMEI is 012345678901237. - The write command can only be used once for IMEI programming. - The IMEI to be written must be different from the default IMEI. - If a 14-digit IMEI is entered, the 15th checksum digit is automatically calculated. - The NV backup of the static calibrated NV partition which stores the IMEI is automatically updated after successfully executing the write command (i.e. backup is updated when OK is returned). |
| Examples | at+wimei? +WIMEI: 012345478901237 // Default IMEI OK at+wimei=354610060035829 // Enter 15-digit IMEI OK at+wimei? +WIMEI: 354610060035829 OK at+wimei=35461006003582 // Enter 14digit IMEI OK at+wimei? +WIMEI: 354610060035829 OK |

## &R Command: RTS and CTS Option

| HL7800 |  |
| --- | --- |
| Write command Syntax AT&R <option> | Response OK Parameter <option> 1 In sync mode, CTS is always ON (RTS transitions are ignored). In async mode, CTS will only drop if required by the flow control |
| Notes | This command has no effect and is not defined in the V.25ter specification; it was only implemented for compatibility reasons. Parameters are ignored and are not saved in non-volatile memory. |

## \N Command: Data Transmission Mode

| HL7800 |  |
| --- | --- |
| Execute command Syntax AT\N<x> | Response OK Parameter <x> 0 Transparent mode 4, 6 RLP mode (non-transparent) |
| Notes | This command has no effect and is not defined in the V.25ter specification; it was only implemented for compatibility reasons. Parameters are ignored and are not saved in non-volatile memory. |

## B Command: Data Rate Selection

| HL7800 |  |
| --- | --- |
| Execute command Syntax ATB<rate> | Response OK Parameter <rate> 0 – 99 Data rate |
| Reference V.25Ter | Notes - This command has no effect and was only implemented for compliance with V.25ter. Parameters are ignored and are not saved in non-volatile memory. |

## S2 Command: Set Character for the Escape Sequence (Data to Command Mode)

| HL7800 |  |
| --- | --- |
| Read command Syntax ATS2? | Response <n> OK |
| Write command Syntax ATS2=<n> | Response OK Parameter <n> Only 43 ("+") is supported |
| Reference V.25Ter | Notes - This command has no effect and was only implemented for compliance with V.25ter. Parameters are ignored and are not saved in non-volatile memory - <n> will not be shown by the AT&V command. |

# Call Control Commands

## D Command: Dial Number

| HL7800 |  |
| --- | --- |
| Test command Syntax ATD=? | Response 1 2 3 4 5 6 7 8 9 0 * # + A B C D P T W , @ ! OK |
| Read command Syntax ATD? | Response 1 2 3 4 5 6 7 8 9 0 * # + A B C D P T W , @ ! OK |
| Execute command Syntax ATD[<n>] | Response OK If successfully connected CONNECT Connection has been established RING The DCE has detected an incoming call signal from the network NO CARRIER The connection cannot be established BUSY Engaged (busy) signal detected NO ANSWER If no hang up is detected after a fixed network timeout CONNECT <data rate> Same as CONNECT but includes the data rate RING CTM The MS has detected an incoming CTM call signal from the network; this code is proprietary CONNECT FAX Same as CONNECT but includes the indication related to a fax call Parameter <n> String of dialing digits and optionally V.25ter modifiers (dialing digits): 0-9, * , #, +, A, B, C, D, P, T, W, ,, @, ! (maximum length: 20 digits) |
| Reference V.25Ter | Notes - This command may generally be aborted when receiving an ATH command during execution. - Resposne “OK” may arrive just after the ATD command or after the call is actually active (see AT+COLP). - <n> is ignored when it is set to ",", "T", "!", "W" or "@" - When an established MT call is hanged up from the caller side, NO CARRIER will only be sent to the port on which the call was established (i.e. the port on which ATD was sent). |
| Examples | ATD*99***3# CONNECT ~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~~ÿ}#À!}!}!} }4}"}&} } } } }%}&R}8}0D}'}"}(}"ná~ NO CARRIER ATD=? 1 2 3 4 5 6 7 8 9 0 * # + A B C D P T W , @ ! OK |

|  | - - |
| --- | --- |

## +CEER Command: Extended Error Report

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CEER=? | Response OK |
| Write command Syntax AT+CEER | Response +CEER: <category>[,<cause>,<descriptions>] OK Parameter <category> "No report available" "CC setup error" "CC modification error" "CC release" "SM attach error" "SM detach" "SM activation error" "SM deactivation" "SS network error cause" "SS network reject cause" "SS network GSM cause" <cause> Digit representing the error cause sent internally or by the network. Refer to 18.2.2 CEER Error Codes for more information. <description> Verbose string containing the textual representation of <cause>. Refer to 18.2.2 CEER Error Codes for more information. |

# Mobile Equipment Control and Status Commands

## +CCLK Command: Real Time Clock

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CCLK=? | Response OK |
| Read command Syntax AT+CCLK? | Response +CCLK: <time> or +CME ERROR: <err> |
| Write command Syntax AT+CCLK= <time> | Response OK or +CME ERROR: <err> Parameter <time> String type value; format is "yy/MM/dd,hh:mm:ss+/-TZ", where characters indicate year (last two digits), month, day, hour, minutes, seconds and time zone (optional). |
| Notes | Year must be 2004 or later. |

## +CLAC Command: List Available AT Commands

| HL7800 |  |
| --- | --- |
| Execute command Syntax AT+CLAC | Response <AT command 1> [<CR><LF><AT command 2>[..]] OK or +CME ERROR: <err> Parameter <AT command> AT command (including the prefix “AT”) |
| Notes | This command provides the AT Command list available for the user. |

## +CFUN Command: Set Phone Functionality

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CFUN=? | Response +CFUN: (list of supported <fun>s), (list of supported <rst>s) OK or +CME ERROR: <err> |
| Read command Syntax AT+CFUN? | Response +CFUN: <power_mode>,<STK_mode> or +CME ERROR: <err> |
| Write command Syntax AT+CFUN=<fun> [,<rst>] | Response OK or +CME ERROR: <err> Parameters <fun> 0 Switch off MS 1 Full functionality 4 Disable both phone’s transmit and receive RF circuits; airplane mode 5 Fast detach 6 Enable SIMTK and fetching of proactive commands 7 Disable SIMTK and enable fetching of proactive commands 8 Disable fetching of proactive commands 15 Perform hidden reset (reset MS without resetting SIM) 16 Simulate reset (reset MS including SIM) 27 Perform SIM reset and restore previous PIN validation state (reset SIM without resetting MS) 31 Disable single stack with an option to power off/power on single (U)SIM card 32 Disable all stacks with an option to power off/power on single (U)SIM card 33 Enable single stack with an option to reset (U)SIM card 34 Enable all stacks with an option to reset (U)SIM cards 35 Power off/power on single (U)SIM card 36 Power off/power on all (U)SIM cards 37 Reset single stack 38 Reset all stacks Note that when <fun> = 0, 15 or 16, the OK response may be missed due to race conditions, as MT may switch off by the time the OK response is triggered. <rst> Reset value If <fun> = 1 or 4: 0 Do not reset MT before resetting it to <fun> power level 1 Reset MT before setting it to <fun> power level If <fun> = 31, 32, 35 or 36: 0 SIM is switched OFF 1 SIM is switched ON If <fun> = 33 or 34: 0 SIM reset not needed 1 SIM reset needed If <fun> = 27 0 Hidden SIM reset (MS is not informed of SIM reset) 1 Normal SIM reset (MS is informed of SIM reset) <power_mode> 1 MS is switched ON 2 Invalid mode 4 Airplane mode <STK_mode> 0 Inactive state 6 Enable the SIM-toolkit interface and fetching of proactive commands by SIM-APPL from the SIM card 7 Disable the SIM-toolkit interface and enable fetching of proactive commands by SIM-APPL from the SIM card 8 Disable fetching of proactive commands by SIM-APPLU from the SIM card |

## +CMER Command: Mobile Equipment Event Reporting

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CMER=? | Response +CMER: (list of supported <mode>s),(list of supported <keyp>s),(list of supported <disp>s),(list of supported <ind>s),(list of supported <bfr>s) OK |
| Read command Syntax AT+CMER? | Response +CMER: <mode>,<keyp>,<disp>,<ind>,<bfr> OK |
| Write command Syntax AT+CMER= [<mode>[,<keyp>[,<disp>[,<ind> [,<bfr>]]]]] | Response OK or +CME ERROR: <err> Parameters <mode> 0 Buffer unsolicited result codes in the TA; if TA result code buffer is full, codes can be buffered in some other place or the oldest ones can be discarded 1 Discard unsolicited result codes when the TA-TE link is reserved (e.g. in on-line data mode); otherwise forward them directly to the TE 2 Buffer unsolicited result codes in the TA when the TA-TE link is reserved (e.g. in online data mode) and flush them to the TE after reservation; otherwise forward them directly to the TE <keyp> 0 No keypad event reporting <disp> 0 No display event reporting <ind> 0 No indicator event reporting 1 Indicator event reporting using result code +CIEV: <ind>,<value>. <ind> indicates the indicator order number (as specified for +CIND) and <value> is the new value of the indicator. Only indicator events which are not caused by +CIND shall be indicated by the TA to the TE. <bfr> 0 TA buffer of unsolicited result codes defined within this command is cleared when <mode>=1 or 2 is entered. 1 TA buffer of unsolicited result codes defined within this command is flushed to the TE when <mode>=1 or 2 is entered (OK response shall be given before flushing the codes) |
| Unsolicited Notification | Responses - +CIEV: 1,(0-5) indicates the battery charging level - +CIEV: 2,(0-5) indicates the received signal level - +CIEV: 3,(0-1) indicates the network service status - +CIEV: 4,(0-1) indicates the message status - +CIEV: 5,(0-1) indicates the active call status - +CIEV: 6,(0-1) indicates the roaming status - +CIEV: 7,(0-1) indicates the sms full status Refer to +CIND for more information regarding indicator control. |
| Reference Sierra Wireless Proprietary | Notes This command can be used without a SIM. |
| Example | at+cmer=? +CMER: (1-2),0,0,(0-1),(0-1) OK at+cmer=2,,,1 OK # mode =2 : enable indication if AT link is available # ind = 1 : enable indicator event report (+CIND) at+cmer? +CMER: 2,0,0,1,0 OK # +CMER setting can be preserved after boot at+cfun=1,1 OK at+cmer? +CMER: 2,0,0,1,0 OK # roaming status = 0 update on registration status change +CIEV: 6,0 +PBREADY # enable +CMER <mode> = 0 buffering at+cmer=0 OK at+cfun=4 OK at+cfun=1 OK # wait for registration, one +CIEV: 6 should be buffered, some +CGEV buffered at+creg? +CREG: 0,1 OK # buffered +CIEV is flushed with <bfr>=1 and <mode>=2 at+cmer=2,,,,1 OK +CIEV: 6,0 at+cmer=0 OK at+cfun=4 OK at+cfun=1 OK # wait for registration, one +CIEV: 6 should be buffered, some +CGEV buffered at+creg? +CREG: 0,1 OK # buffered +CIEV is cleared with <bfr>=1 and <mode>=2 at+cmer=2,,,,0 OK |

## +CMEE Command: Report Mobile Termination Error

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CMEE=? | Response +CMEE: (list of supported <n>s) OK |
| Read command Syntax AT+CMEE? | Response +CMEE: <n> OK |
| Write command Syntax AT+CMEE=[<n>] | Response OK Parameter <n> 0 Disable +CME ERROR: <err> result code and use ERROR instead 1 +CME ERROR: <err> result code and use numeric <err> values 2 +CME ERROR: <err> result code and use verbose <err> values |
| Notes | <n> is saved in non-volatile memory per AT port over module reboot. |

## +CCID Command: Request SIM Card Identification

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CCID=? | Response OK |
| Read command Syntax AT+CCID? | Response +CCID: <ICCID> OK or +CME ERROR: <err> |

## +CPIN Command: Enter Pin

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CPIN=? | Response OK |
| Read command Syntax AT+CPIN? | Response +CPIN: <code> OK or +CME ERROR: <err> |
| Write command Syntax AT+CPIN=<pin> [,<newpin>] | Response OK or +CME ERROR: <err> Parameters <code> Values when queried using the read command READY MT is not pending for any password SIM PIN MT is waiting for SIM PIN to be given SIM PUK MT is waiting for SIM PUK to be given SIM PIN2 MT is waiting SIM PIN2 to be given (this <code> is recommended to be returned only when the last executed command resulted in PIN2 authentication failure (i.e. +CME ERROR: 17); if PIN2 is not entered right after the failure, it is recommended that MT does not block its operation) SIM PUK2 MT is waiting SIM PUK2 to be given (this <code> is recommended to be returned only when the last executed command resulted in PUK2 authentication failure (i.e. +CME ERROR: 18); if PUK2 and new PIN2 are not entered right after the failure, it is recommended that ME does not block its operation). PH-NET PIN MT is waiting for the network personalization password to be given PH-NET PUK MT is waiting network personalization unblocking password to be given PH-NETSUB PIN MT is waiting network subset personalization password to be given PH-NETSUB PUK MT is waiting network subset personalization unblocking password to be given PH-SP PIN MT is waiting service provider personalization password to be given PH-SP PUK MT is waiting service provider personalization unblocking password to be given PH-CORP PIN MT is waiting corporate personalization password to be given PH-CORP PUK MT is waiting corporate personalization unblocking password to be given <pin>, <newpin> String type values |

## +CPAS Command: Phone Activity Status

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CPAS=? | Response +CPAS: (list of supported <pas>es) OK or +CME ERROR: <err> |
| Execute command Syntax AT+CPAS | Response +CPAS: <pas> OK or +CME ERROR: <err> Parameter <pas> 0 Ready (ME allows commands from TA/TE) 1 Unavailable (ME does not allow commands from TA/TE) 2 Unknown (ME is not guaranteed to respond to instructions) 3 Ringing (ME is ready for commands from TA/TE, but the ringer is active) 4 Call in progress (ME is ready for commands from TA/TE, but a call is in progress) 5 Asleep (ME is unable to process commands from TA/TE because it is in a low function-ality state) |

## +CSQ Command: Signal Quality

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CSQ=? | Response +CSQ: (list of supported <rssi>s),(list of supported <ber>s) OK |
| Execute command Syntax AT+CSQ | Response +CSQ: <rssi>,<ber> or +CME ERROR: <err> Parameters <rssi> Received signal strength indication 0 -113 dBm or less 1 – 30 -111 to -53 dBm 31 -51 dBm or greater 99 Not known or not detectable <ber> Integer type; channel bit error rate (in percent) 0 – 7 As RXQUAL values in the table in 3GPP TS 45.008 [20] subclause 8.2.4 99 Not known or not detectable |
| Notes | - For LTE, <rssi> is scaled from the current radio signal strength (RSRP) value of the serving cell. RSRP is defined according to 3GPP TS 36.133 section 9.1.4, from -140 dBm to -44 dBm with 1 dB resolution. - For LTE, <ber> is scaled to 0 – 7 from RSRQ signal quality 34 – 0. RSRQ is defined according to specification 3GPP 36.133 section 9.1.7, from -19.5 dBm to -3 dBm with 0.5 dB resolution. |

## +KCELL Command: Cell Environment Information

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KCELL=? | Response +KCELL: (list of supported <revision>s) OK |
| Read command Syntax AT+KCELL? | Response OK |
| Write command Syntax AT+KCELL= <revision> | Response For GSM cells: +KCELL: <nbGSMcells>[,<cell_typei>,<ARFCNi>,<BSICi>,<PLMNi>,<LACi>, <GSM_CIi>,<RSSIi>,<GSM_TA>][,<cell_typei>,<ARFCNi>,<BSICi>,<PLMNi>, <LACi>,<CIi>,<RSSIi>][…]] For UMTS cells: +KCELL: <nbUMTScells>[,<cell_typek>,<dl_UARFCNk>,<PLMNk>,<LACk>, <UMTS_CIk>,<scrambling_codek>,<rscpk>,<ecnok>[,<pathlossk>]][…]] OK For LTE cells: +KCELL: <nbLTEcells>[,<cell_type>,<PLMN>,<LTE_CI>,<PhyCellInd>, <trackingAreaCode>,<RSRPResult>,<RSRQResult>,<LTE_TA>][<cell_type>,[[Earfcn>, [<PhyCellID>,[<RSRPResult>,[<RSRQResult>]]]]][…]] OK Parameters <revision> Reserved for future development (only 0 for the moment) <nbGSMcells> 0 i 7 Number of base stations available <cell_type> 0 GSM serving cell 1 GSM neighbor cell 2 UMTS serving cell 3 UMTS neighbor cell 4 UMTS detected cell 5 LTE serving cell 6 LTE neighbor cell <ARFCN> 0 – 1023 Absolute Radio Frequency Channel Number in decimal format <BSIC> 0 – 63 Base Station Identity Code in 6 bits decimal format <PLMN> PLMN identifiers (3 bytes) in hexadecimal format, made of MCC (Mobile Country Code), and MNC (Mobile Network Code) <LAC> Location Area in hexadecimal format, 4 digits <GSM_CI> Cell ID, 4 hexadecimal digits, e.g. ABCD <RSSI> 0 – 63 Received signal level of the BCCH carrier. The indicated value is an offset which should be added to –110 dBm to get a value in dBm. See the formula specified in TS 05.08 Radio Subsystem Link Control <GSM_TA> 0 – 63 Timing advance; only available for serving cell 255 Not available (there is no active CS/PS connection) <nbUMTScells> 0 k 25 Number of UMTS base stations available <dl_UARFCN> DL UARFCN of serving cell in decimal format. The range can be found at 3GPP TS 25.101 <UMTS_CI> Cell ID, 8 hexadecimal digits, 32 bits <scrambling code> 0 – 511 Downlink scrambling code in decimal format <rscp> 0 – 91 Received Signal Code Power. The power level in one chip 255 Invalid/default value <ecno> 0 – 24 Ratio of energy per modulating bit to the noise spectral density. This is the cell quality and is equal to RSCP/RSSI Energy per chip/noise 255 Invalid/default value <pathloss> 46 dB to 158 dB Path loss in decimal format 255 Not available <nbLTEcells> 0 k 33 Number of LTE base stations available <LTE_CI> Cell Identity in 8 hexadecimal digits with length = 28 bits. (Ref: 3GPP TS 36.331, 6.3.4, CellIdentity IE) <PhyCellInd> 0 – 503 Physical Cell ID (Ref: 3GPP TS 36.331, 6.3.4, PhysCellId IE) <TrackingAreacode> Tracking Area Code (Ref: 3GPP TS 36.331, 6.3.4, Tracking AreaCode IE) Integer type with length = 16 bits <RSRPResult> 0 – 97 Reference Signal Received Power (Ref: 3GPP TS 36.331, 6.3.5, RSRP-Range IE) <RSRQResult> 0 – 34 Reference Signal Received Quality (Ref: 3GPP TS 36.331, 6.3.5, RSRQ-Range IE) <LTE_TA> 0 – 1282 Timing advance (as per [3GPP 36.321]) <Earfcn> 0 – 0xFFFF The carrier frequency of the neighbor cell designated by the EUTRA Absolute Radio Frequency Channel Number (EARFCN) (Ref: 3GPP TS 36.101, 5.7.3) <PhyCellInd> 0 – 503 Physical Cell ID (Ref: 3GPP TS 36.331, 6.3.4, PhysCellId IE) |
| Reference Sierra Wireless Proprietary | Notes - This command provides information related to the network environment and can be used, for example, for localization calculation. - This command can only be used with a SIM. The cell information can only be retrieved when the UE stays in attached mode. |

## +KGPIO Command: Hardware IO Control

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KGPIO=? | Response +KGPIO: (list of supported <IO>s),(list of supported <cde>s) OK |
| Read command Syntax AT+KGPIO? | Response OK |
| Write command Syntax AT+KGPIO=<IO>,<cde> | Response If <cde> = 2: +KGPIO: <IO>, <current_value> OK Else OK Parameters <IO> 1 – 8, 10, 11, 13 – 15 Selected IO <cde> 0 Reset the selected IO 1 Set the selected IO 2 Request the current value of the IO <current_value> 0 GPIO is Low 1 GPIO is High |
| Reference Sierra Wireless Proprietary | Notes - The current configuration is saved in non-volatile memory over module reboot. - Check the configuration of +KGPIOCFG when +CME ERROR: 3 is issued. - GPIO 3 is used for SIM detection by default; it cannot be reconfigured. - The test command AT+KGPIO=? returns a dynamic list of supported GPIOs. GPIOs assigned to a specific purpose are not listed. - This command can be used without a SIM. |
| Examples | Make GPIO1 output high/low level AT+KGPIOCFG=1,0,2 // Configure GPIO1 as output mode; <pull mode> must // be "no pull" OK AT+KGPIO=1,1 // Set GPIO1 OK AT+KGPIO=1,0 // Reset GPIO1 OK Define input/output mode for GPIO1 AT+KGPIOCFG=1,1,0 // Configure GPIO1 as input mode;<pull mode> is "pull // down" OK AT+KGPIO=1,2 // Request the current value of GPIO1 +KGPIO: 1,1 // Value is HIGH for GPIO1 OK at+kgpio=? +KGPIO: (1,2,4,5,6,7,8,10,11,13,14,15),(0-2) OK at+kgpio=9,1 // Set GPIO9, and it should return ERROR +CME ERROR: 3 |

## +KGPIOCFG Command: GPIO Configuration

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KGPIOCFG= ? | Response +KGPIOCFG: (list of supported <n>s),(list of supported <dir>s), (list of supported <pull mode>s) OK |
| Read command Syntax AT+KGPIOCFG? | Response +KGPIOCFG: <n>,<dir>,<pull mode>[<CR><LF> +KGPIOCFG: <n>,<dir>,<pull mode> [...]] OK |
| Write command Syntax AT+KGPIOCFG =<n>,<dir>,<pull mode> | Response OK Parameters <n> 1 – 8, 10, 11, 13 –15 GPIO number <dir> Direction 0 Output 1 Input <pull mode> 0 Pull down. Internal pull down resistor available. Only used in input mode 1 Pull up. Internal pull up resistor available. Only used in input mode 2 No pull. Internal pull up/down resistor NOT available. Only used in output mode |
| Reference Sierra Wireless Proprietary | Notes - This command provides configuration for +KGPIO command. - The current configuration is saved in non-volatile memory before a reset. - By default, GPIO 3 is used by SIM detection; it cannot be reconfigured. - Pull down/up mode provides a stable input level. - Commands AT+KGPIOCFG=? and AT+KGPIOCFG? return a dynamic list of supported GPIOs. GPIOs assigned to a specific purpose are not listed. - This command can be used without a SIM. |
| Examples | at+kgpiocfg=1,0,0 // When setting GPIO1 as Output, with incorrect <pull mode> ERROR at+kgpiocfg=1,0,1 // When setting GPIO1 as Output, with incorrect <pull mode> ERROR at+kgpiocfg=1,0,2 // When setting GPIO1 as Output, with correct <pull mode> OK at+kgpiocfg=1,1,0 // When setting GPIO1 as Input, with pull down OK at+kgpiocfg=1,1,1 // When setting GPIO1 as Input, with pull up OK at+kgpiocfg=1,1,2 // When setting GPIO1 as Input, with incorrect <pull mode> ERROR at+kgpiocfg=? +KGPIOCFG: (1,2,4,5,6,7,8,10,11,13,14,15),(0-1),(0-2) OK at+kgpiocfg? // GPIO 9 is not available for use +KGPIOCFG: 1,0,2 +KGPIOCFG: 2,0,2 +KGPIOCFG: 4,0,2 +KGPIOCFG: 5,0,2 +KGPIOCFG: 6,0,2 +KGPIOCFG: 7,0,2 +KGPIOCFG: 8,0,2 +KGPIOCFG: 10,0,2 +KGPIOCFG: 11,0,2 +KGPIOCFG: 13,0,2 +KGPIOCFG: 14,0,2 +KGPIOCFG: 15,0,2 OK at+kgpiocfg=9,1,0 // When setting GPIO9, it returns ERROR +CME ERROR: 3 |

## +KADC Command: Analog Digital Converter

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KADC=? | Response +KADC: (list of supported <Meas id>s),(list of supported <Meas time>s) OK |
| Read command Syntax AT+KADC= <Meas id>, <Meas time> | Response +KADC: <Meas result>,<Meas id>,<Meas time>[,<Temperature>] Parameters <Meas id> Measurement ID 0 VBATT – “VBATT” voltage 1 VCOIN – “BAT_RTC” backup battery voltage 2 THERM – Connected to RT400 (the thermistor on board which is located close to the 26MHz VCTCXO) 3 Reserved 4 ADC0 5 Reserved 6 Reserved 7 ADC1 <Meas time> Measurement time 1 During TX 2 Far from TX 3 No constraint <Meas result> Measurement result is in µV <Temperature> Temperature in degrees Celsius |
| Reference Sierra Wireless Proprietary | Notes - 10 bits converter. - ADC0 is only supported on the HL7800. Check the module variant using AT+CGMM. - VBATT does not support no constraint measurement time. - This command can be used without a SIM. - Available range for voltage input are as follows: <Meas id>; Range (V). VBATT; 3.2 - 4.5. VCOIN; 0 - 1.8. THERM; 0 - 1.2. ADC0; 0 - 1.2. ADC1; 0 - 1.2. |

## +CSIM Command: Generic SIM Access

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CSIM=? | Response OK |
| Write command Syntax AT+CSIM= <length>, <command> | Response +CSIM: <length>,<response> OK or +CME ERROR: <err> Parameters <length> Integer type; length of the characters that are sent to TE in <command> or <response> <command> Command passed on by MT to the SIM in hexadecimal format <response> Response to the command passed on by the SIM to the MT in hexadecimal format |

## +CCHO Command: Open Logical Channel

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CCHO=? | Response OK |
| Write command Syntax AT+CCHO= <dfname> | Response <session_id> OK or +CME ERROR: <err> Parameters <dfname> DF name coded on 1 to 16 bytes that references to all selectable application in the UICC <session_id> Session ID to be used in order to target a specific application on the smart card using logical channels mechanism (string without double quotes that represents a decimal value). |
| Notes | The +CCHO execute command gives the <session_id> when it receives SIM application response status words as shown below: - ‘90’ ‘00’ – normal ending of the command - ‘91’ ‘XX’ – normal ending of the command with extra information from the proactive UICC containing a command for the terminal.length ‘XX’ of the response data - ‘92’ ‘XX’ – normal ending of the command with extra information concerning an ongoing data transfer session |

## +CCHC Command: Close Logical Channel

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CCHC=? | Response OK |
| Write command Syntax AT+CCHC= <session_id> | Response OK or +CME ERROR: <err> Parameter <session_id> Session ID to be used in order to target a specific application on the smart card using logical channels mechanism (string without double quotes that represents a decimal value). |

## +CGLA Command: Generic UICC Logical Channel Access

| HL7800 |  |
| --- | --- |
| Write command Syntax AT+CGLA= <sessionid>, <length>, <command> | Response +CGLA: <length>,<response> OK or +CME ERROR: <err> Parameters <sessionid> Integer type; used as the identifier of the session to be used in order to send the APDU commands to the UICC. It is mandatory in order to send commands to the UICC when targeting applications on the smart card using a logical channel other than the default channel (channel "0"). <length> Integer type; length of the characters that are sent to TE in <command> or <response> (two times the actual length of the command or response). <command> Command passed on by the MT to the UICC in the format as described in 3GPP TS 31.101 in hexadecimal format (refer to +CSCS). <response> Response to the command passed on by the UICC to the MT in the format as described in 3GPP TS 31.101 in hexadecimal format (refer to +CSCS). |

## +CRLA Command: Restricted UICC Logical Channel Access

| HL7800 |  |
| --- | --- |
| Write command Syntax AT+CRLA= <sessionid>, <command> [,<file id>[,<P1>, <P2>,<P3> [,<data> [,<pathid>]]]]> | Response +CRLA: <sw1>,<sw2>[,<response>] OK or +CME ERROR: <err> Parameters <sessionid> Integer typewhich identifies the session to be used in order to send the APDU commands to the UICC. It is mandatory in order to send commands to the UICC when targeting applications on the smart card using a logical channel other than the default channel (channel "0"). <command> 176 READ BINARY 178 READ RECORD 192 GET RESPONSE 214 UPDATE BINARY 220 UPDATE RECORD 242 STATUS 219 SET DATA All other values are reserved <fileid> Integer type that identifies the elementary datafile on SIM. Mandatory for every <command> except STATUS. <P1>, <P2>, <P3> Integer type; parameters passed on by the MT to the UICC. These parameters are mandatory for every command, except GET RESPONSE and STATUS. <data> Information which shall be written to the SIM in hexadecimal format <pathid> String type containing the path of an elementary file on the UICC in hexadecimal format . <sw1>, <sw2> Integer type; information from the UICC about the execution of the actual command. These parameters are delivered to the TE in both cases, on successful or failed execution of the command <response> Response of a successful completion of the command previously issued in hexadecimal format. STATUS and GET RESPONSE returns data, which gives information about the current elementary datafield. This information includes the type of file and its size (refer to 3GPP TS 31.101). After READ BINARY, READ RECORD or RETRIEVE DATA command the requested data will be returned. |
| Notes | By using this command instead of generic UICC access command, +CGLA, the TE application has an easier but more limited access to the UICC database. |

## +CUAD Command: UICC Application Discovery

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CUAD=? | Response OK |
| Execute command Syntax AT+CUAD | Response <response> OK or +CME ERROR: <err> Parameter <response> Content of the EFDIR. String type in hexadecimal format. |

## +CRSM Command: Restricted SIM Access

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CRSM=? | Response OK |
| Write command Syntax AT+CRSM= <command> [,<fileid>[,<P1>, <P2>,<P3> [,<data> [,<pathid>]]]] | Response +CRSM: <sw1>,<sw2>[,<response>] OK or +CME ERROR: <err> Parameters <command> 176 READ BINARY 178 READ RECORD 192 GET RESPONSE 214 UPDATE BINARY 220 UPDATE RECORD 242 STATUS <fileid> Integer type; this is the identifier of an elementary data file on the SIM. Mandatory for every command except STATUS. 28423 IMSI file (6F07) 28473 ACM file (6F39) 28481 PUKT file (6F41) 28482 SMS file (6F42) <P1>, <P2>, <P3> Integer type defining the request. These parameters are mandatory for every command, except GET RESPONE and STATUS. The values are described in GSM 51.011 <data> Information which shall be written to the SIM (hexadecimal character format; refer +CSCS) <sw1>, <sw2> Integer type containing SIM information 0x90 0x00 Normal entry of the command 0x9F 0xXX Length XX of the response data 0x92 0x0X Update successful but after using an internal retry routine X times 0x92 0x40 Memory problem 0x94 0x00 No EF selected 0x94 0x02 Out of range (invalid address) 0x94 0x04 File ID not found; pattern not found 0x94 0x08 File is inconsistent with the command 0x98 0x02 No CHV initialized 0x98 0x04 Access cond. Not fullfiled / unsuccessful CHV verify / authentication failed 0x98 0x08 In contradiction with CHV status 0x98 0x10 In contradiction with invalidation status 0x98 0x40 Unsucc. CHV-verif. Or UNBLOCK CHF / CHV blocked /UNBL.blocked 0x98 0x50 Increase can not be performed. Maximum value reached 0x61 0xXX SW2 indicates the number of response bytes still available. Use Get Response to access this data. 0x62 0xXX Warning - state unchanged 0x62 0x00 Warning - no information provided 0x62 0x81 Warning - part of returned data may be corrupt 0x62 0x82 Warning - end of file/record reached (bad cmd) 0x62 0x83 Warning - selected file invalidated 0x62 0x84 Warning - bad file control information format 0x63 0xXX Warning - state unchanged 0x63 0x00 Warning - no information provided 0x63 0x81 Warning - file filled up with last write 0x63 0xCx Warning - counter value is x 0x64 0xXX Error - state unchanged 0x65 0xXX Error - state changed 0x65 0x00 Error - no information provided 0x65 0x81 Error - memory failure 66 xx Security Error 0x66 0xXX Security Error 0x67 0xXX Incorrect parameter P3 0x68 0xXX Check Error - CLA function not supported 0x68 0x00 Check Error - no information provided 0x68 0x81 Check Error - logical channel not supported 0x68 0x82 Check Error - secure messaging not supported 0x69 0xXX Check Error - command not allowed 0x69 0x00 Check Error - no information provided 0x69 0x81 Check Error - command incompatible with file structure 0x69 0x82 Check Error - security status not satisfied 0x69 0x83 Check Error - authentication method blocked 0x69 0x84 Check Error - referenced data invalidated 0x69 0x85 Check Error - conditions of use not satisfied 0x69 0x86 Check Error - command not allowed (no current EF) 0x69 0x87 Check Error - expected SM data objects missing 0x69 0x88 Check Error - SM data objects incorrect 0x6A 0xXX Check Error - wrong parameters 0x6A 0x00 Check Error - no information provided 0x6A 0x80 Check Error - incorrect parameters in data field 0x6A 0x81 Check Error - function not supported 0x6A 0x82 Check Error - file not found 0x6A 0x83 Check Error - record not found 0x6A 0x84 Check Error - not enough memory space in the file 0x6A 0x85 Check Error - Lc vailable on with TLV structure 0x6A 0x86 Check Error - vailable on parameters P1-P2 0x6A 0x87 Check Error - Lc vailable on with P1-P2 0x6A 0x88 Check Error - referenced data not found 0x6B 0xXX Incorrect parameter P1 or P2 0x6C 0xXX Check Error - wrong length - xx is the correct length 0x6D 0xXX Unknown instruction code given in the command 0x6E 0xXX Wrong instruction class given in the command 0x6F 0xXX Technical problem with no diagnostic given <response> Response of successful completion of the command previously issued in hexadecimal character format; refer to +CSCS. STATUS and GET RESPONSE returns data, which gives information about the current elementary datafield. This information includes the type of file and its size (refer to GSM 51.011 [28]). After READ BINARY or READ RECORD commands, the requested data will be returned. <response> is not returned after a successful UPDATE BINARY or UPDATE RECORD command. <pathid> String type that contains the path of an elementary file on the SIM/USIM in hexadecimal format as defined in ETSI TS 102 221 (e.g. "7F205F70" in SIM and USIM case). |

## +KTEMPMON Command: Temperature Monitor

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KTEMPMON=? | Response +KTEMPMON: (list of supported <mod>s),(list of supported <temperature>s),(list of supported <urcMode>s),(list of supported <action>s),(list of supported <hystTime>s),(list of supported <repGPIO>s) OK |
| Read command Syntax AT+KTEMPMON? | Response +KTEMPMON: <mod>,<temperature>,<urcMode>,<action>,<hystTime>,<repGPIO> OK |
| Write command Syntax AT+KTEMPMON=<mod>, [<temperature> [,<urcMode> [,<action> [,<hystTime> [,<repGPIO>]]]]] | Response +KTEMPMON: <level>,<value> OK Parameters <mod> 0 Disable the module’s internal temperature monitor 1 Enable the module’s internal temperature monitor <temperature> Temperature limit before the module acts as defined by <action>. Default value: 0 <urcMode> 0 Disables the presentation of the temperature monitor URC 1 Enables the presentation of the temperature monitor URC <action> 0 No action 1 Automatic shut-down when the temperature is beyond <temperature> 2 The output pin <repGPIO> is tied HIGH when <temperature> is reached; when the temperature is normal the output pin <repGPIO> is tied LOW. Note that if this parameter is required, it is mandatory to set the <repGPIO> parameter. <hyst_time> 0 – 255 Hysteresis time in seconds. Action will only happen if <temperature> is maintained for at least as long as this period. This parameter is mandatory if <action> is not zero. Default value: 30. <repGPIO> 1 – 8, 10, 11, 13 – 15 Defines which GPIO is used as output pin. This parameter is mandatory only if <action>=2 is required. Default value: 6. |
| Notes | - When the module’s internal temperature reaches either operating or extreme levels; the unsolicited message is in the format: +KTEMPMEAS: <level>,<value> where: <level> is the threshold level: -2 Extreme temperature lower bound (-40°C) -1 Operating temperature lower bound (-20°C) 0 Normal temperature 1 Operating temperature upper bound (+55°C) 2 Extreme temperature upper bound (+85°C) <value> is the actual temperature expressed in degrees Celsius - Due to temperature measurement uncertainty there is a tolerance of ± 2°C. - Check available GPIOs with +KGPIOCFG when using this command. |

## +CTZU Command: Automatic Time Zone Update

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CTZU=? | Response +CTZU: (list of supported <onoff>s) OK |
| Read command Syntax AT+CTZU? | Response +CTZU: <onoff> OK |
| Write command Syntax AT+CTZU =<onoff> | Response OK or +CME ERROR: <err> Parameter <onoff> 0 Disable automatic time zone update via NITZ 1 Enable automatic time zone update via NITZ |
| Notes | - <onoff> is saved in non-volatile memory over module reboot. - CTZU (onoff=1) is enabled by default for proper Verizon Administration (SIM provision, OMADM, etc.) |

## +CTZR Command: Time Zone Reporting

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CTZR=? | Response +CTZR: (list of supported <onoff>s) OK |
| Read command Syntax AT+CTZR? | Response +CTZR: <onoff> OK |
| Write command Syntax AT+CTZR =<onoff> | Response OK or +CME ERROR: <err> Parameter <onoff> 0 Disable time zone change event reporting 1 Enable time zone change event reporting |
| Unsolicited Notification | Response +CTZV: <tz>,<time> XNITZINFO: <timzone_variance>,<time> +CTZDST: <dst> Parameters <tz> Integer value indicating the time zone <time> String type value in format “YY/MM/dd,hh:mm:ss” wherein the characters indicate year, month, date, hour, minutes and seconds. <dst> Daylight sabings time value 0 Disable time zone change event reporting and URC +XNITZINFO, +CTZDST 1 Enable time zone change event reporting and URC +XNITZINFO, +CTZDST <timzone_variance> String of format “GMT+HH:MM” or “GMT-HH:MM” (for example, GMT+5:30) |
| Reference [27.007] §8.41 | Notes - The Time Zone reporting is not affected by the Automatic Time Zone setting command +CTZU - If the reporting is enabled the MT returns the unsolicited result code +CTZV: <tz> whenever the time zone is changed - <onoff> is saved in non-volatile memory per AT port over module reboot |

## +KSLEEP Command: Power Management Control for UART

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KSLEEP=? | Response +KSLEEP: (list of supported <mngt>s) OK |
| Read command Syntax AT+KSLEEP? | Response +KSLEEP: <mngt> OK |
| Write command Syntax AT+KSLEEP= <mngt> | Response OK Parameters <mngt> 0 The UART doesn’t go to sleep mode as long as the DTR is active (low level). The DTR has to be active to send AT commands. 1 The UART decides by itself (internal timing) when it will go to sleep mode, and it will be woken up with a character 2 The UART never goes in sleep mode regardless of the DTR state |
| Reference Sierra Wireless Proprietary | Notes - The current configuration is kept in non-volatile memory over module reboot. - This command only controls UART power management, and does not affect the USB AT command port. - This command can be used without a SIM. - When AT+KSLEEP=1 and the module is in sleep mode, the user needs to input a character to wake the module up. After which, AT commands can be input normally. |
| Example | AT+KSLEEP=? +KSLEEP: (0-2) OK AT+KSLEEP? +KSLEEP: 2 OK AT+KSLEEP=0 // Change settings to mode 0 OK AT+KSLEEP? +KSLEEP: 0 OK AT+KSLEEP=2 // Change settings to mode 2 OK AT+KSLEEP? +KSLEEP: 2 OK |

## +CESQ Command: Extended Signal Quality

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CESQ=? | Response +CESQ: (list of supported <rxlev>s),(list of supported <ber>s),(list of supported <rscp>s), (list of supported <ecno>s),(list of supported <rsrq>s),(list of supported <rsrp>s) OK |
| Execute command Syntax AT+CESQ | Response +CESQ: <rxlev>,<ber>,<rscp>,<ecno>,<rsrq>,<rsrp> OK Parameters <rxlev> Integer type; received signal strength level (see 3GPP TS 45.008 [20] subclause 8.1.4) 0 rssi < -110 dBm 1 -110 dBm ≤ rssi < -109 dBm 2 -109 dBm ≤ rssi < -108 dBm … 61 -50 dBm ≤ rssi < -49 dBm 62 -49 dBm ≤ rssi < -48 dBm 63 -48 dBm ≤ rssi 99 not known or not detectable <ber> Integer type; channel bit error rate (in percent) 0 – 7 As RXQUAL values in the table in 3GPP TS 45.008 [20] subclause 8.2.4 99 Not known or not detectable <rscp> Integer type; received signal code power (see 3GPP TS 25.133 [95] subclause 9.1.1.3 and 3GPP TS 25.123 [96] subclause 9.1.1.1.3) 0 rscp < -120 dBm 1 -120 dBm ≤ rscp < -119 dBm 2 -119 dBm ≤ rscp < -118 dBm … 94 -27 dBm ≤ rscp < -26 dBm 95 -26 dBm ≤ rscp < -25 dBm 96 -25 dBm ≤ rscp 255 Not known or not detectable <ecno> Integer type; ratio of the received energy per PN chip to the total received power spectral density (see 3GPP TS 25.133 [95] subclause) 0 Ec/Io < -24 dB 1 -24 dB ≤ Ec/Io < -23.5 dB 2 -23.5 dB ≤ Ec/Io < -23 dB … 47 -1 dB ≤ Ec/Io < -0.5 dB 48 -0.5 dB ≤ Ec/Io < 0 dB 49 0 dB ≤ Ec/Io 255 Not known or not detectable <rsrq> Integer type; reference signal received quality (see 3GPP TS 36.133 [96] subclause 9.1.7) 0 rsrq < -19.5 dB 1 -19.5 dB ≤ rsrq < -19 dB 2 -19 dB ≤ rsrq < -18.5 dB … 32 -4 dB ≤ rsrq < -3.5 dB 33 -3.5 dB ≤ rsrq < -3 dB 34 -3 dB ≤ rsrq 255 Not known or not detectable <rsrp> Integer type; reference signal received power (see 3GPP TS 36.133 [96] subclause 9.1.4) 0 rsrp < -140 dBm 1 -140 dBm ≤ rsrp < -139 dBm 2 -139 dBm ≤ rsrp < -138 dBm … 95 -46 dBm ≤ rsrp < -45 dBm 96 -45 dBm ≤ rsrp < -44 dBm 97 -44 dBm ≤ rsrp 255 Not known or not detectable |
| Notes | - If the current serving cell is not a GERAN cell, <rxlev> and <ber> are set to value 99. - If the current serving cell is not a UTRA FDD or UTRA TDD cell, <rscp> is set to 255. - If the current serving cell is not a UTRA FDD cell, <ecno> is set to 255. - If the current serving cell is not an E-UTRA cell, <rsrq> and <rsrp> are set to 255. |

|  | - - - |
| --- | --- |

## +KRIC Command: Ring Indicator Control

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KRIC=? | Response +KRIC: (list of supported <masks>es),(list of supported <shape>s) OK |
| Read command Syntax AT+KRIC? | Response +WEXTCLK: <masks>,<shape> OK |
| Write command Syntax AT+KRIC= <masks> [,<shape>] | Response OK Parameters <masks> Use of RI signal 0x00 RI is not used 0x01 RI is activated on incoming calls (+CRING, RING) 0x02 RI is activated on SMS (+CMT, +CMTI) 0x04 RI is activated on SMS-CB (+CBM, +CBMI) 0x08 RI is activated on USSD (+CUSD) 0x10 RI is activated on network state (+CIEV) <shape> Signal shape (only available for incoming calls) 0 Repeat pulses. The total length of the pulse is equivalent to the transfer of the RING or CRING notification 1 Always active. The signal is set to be active during the whole incoming call notification |
| Reference Sierra Wireless Proprietary | Notes - The current configuration is kept in non-volatile memory after a reset. - For SMS and other unsolicited messages, only one pulse is set regardless of <shape>. - The width of the pulse is 1s. For repeated pulse on incoming calls, pulse width is 1s, and then rest for 4 seconds, and then repeated. - This command should not be used during an incoming call, SMS, SMSCB, USSD, etc. - This command can be used without a SIM. - If <shape> is omitted, the previously saved value will be used. |
| Examples | AT+KRIC=? +KRIC: (0-31),(0-1) OK AT+KRIC? +KRIC: 15,0 OK AT+KRIC=1,1 //RI is always activated on incoming calls OK AT+KRIC? +KRIC: 1,1 OK AT+KRIC=2 //RI is activated on SMS OK AT+KRIC? +KRIC: 2,1 OK |

## +KBND Command: Current Networks Band Indicator

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KBND=? | Response +KBND: (list of supported <bnd>s) OK |
| Read command Syntax AT+KBND? | Response +KBND: <bnd> OK Parameter <bnd> Band in hexadecimal format 0x00000000 Not available 0x00000002 GSM 900 MHz (HL7692 only) 0x00000004 DCS 1800 MHz (HL7692 only) 0x00000010 UMTS Band I (2100 MHz) (HL7650 only) 0x00000020 UMTS Band II (1900 MHz) (HL7688 only) 0x00000040 UMTS Band V (850 MHz) (HL7650 and HL7688) 0x00000100 UMTS Band VIII (900 MHz) (HL7650 only) 0x00000800 LTE Band 2 (1900 MHz) (HL7648 and HL7688) 0x00001000 LTE Band 3 (1800 MHz) (HL7650, HL7690 and HL7692) 0x00002000 LTE Band 4 (1700 MHz) (HL7618, HL7618RD, HL7648 and HL7688) 0x00004000 LTE Band 5 (850 MHz) (HL7650 and HL7688) 0x00010000 LTE Band 13 (700 MHz) (HL7618, HL7618RD and HL7688) 0x00020000 LTE Band 17 (700MHz) (HL7688 only) 0x00040000 LTE Band 28 (700 MHz) (HL7650 only) 0x00800000 LTE Band 8 (900MHz) (HL7650, HL7690 and HL7692) 0x01000000 LTE Band 20 (800MHz) (HL7690 and HL7692) 0x02000000 LTE Band 12 (700 MHz) (HL7648 only) |
| Reference Sierra Wireless Proprietary | Notes - This command returns the GSM, UMTS or LTE band that the module is currently using. - This command cannot be used without a SIM. |

## +KSRAT Command: Set Radio Access Technology

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KSRAT=? | Response +KSRAT: (list of supported <mode>s) OK |
| Read command Syntax AT+KSRAT? | Get current band Response +KSRAT: <mode> OK |
| Write command Syntax AT+KSRAT= <mode> | Set current mode Response OK Parameter <mode> 5 LTE only 6 Search for LTE first then UMTS 7 Search for LTE first then UMTS 8 Search for GSM first then LTE 9 Search for LTE first then GSM |
| Reference Sierra Wireless Proprietary | Notes - This command can be used without a SIM. - <mode> is automatically stored in persistent memory. - Settings take effect immediately. |

## +CPOF Command: Power Off

| HL7800 |  |
| --- | --- |
| Execute command Syntax AT+CPOF | Response OK |
| Notes | - This command powers the module off. - “OK” is immediately returned after the power off sequence |

## +KSREP Command: Mobile Start-up Reporting

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KSREP=? | Response +KSREP: (list of supported <mode>s) OK |
| Read command Syntax AT+KSREP? | Response +KSREP: <mode>,<stat>,<PB ready> OK |
| Write command Syntax AT+KSREP= <mode> | Response OK or +CME ERROR: <err> Parameters <mode> Unsolicited result code mode 0 Disable the start-up URC 1 Enable the start-up URC <stat> Module status 0 The module is ready to receive commands for the TE. No access code is required 1 The module is waiting for an access code. Use AT+CPIN? to determine the code 2 The SIM card is not present 3 The module is in “SIM lock” state 4 Unrecoverable error 5 Unknown state <PB ready> Phone book status 0 Phone book is not ready 1 Phone book is ready for read and write |
| Unsolicited Notification | Response +KSUP: <stat> |
| Reference Sierra Wireless Proprietary | Notes - URC +KSUP: <stat> will only be displayed once after reboot if <mode>=1. - If <mode>=0, +PBREADY and +SIM URC notifications will not be sent at the start-up process. However, they will still be sent afterwards during normal module operation. - This command can be used without a SIM. - <mode> is saved in non-volatile memory. |
| Example | // SIM Card is inserted AT+KSREP? +KSREP: 1,0,1 // <mode>=1. The module and phone book are ready OK AT+KSREP=? +KSREP: (0-1) OK AT+KSREP=0 // Set mode to 0 OK AT+KSREP? +KSREP: 0,0,1 // Mode is changed to 0 and save to non-volatile memory OK // Reboot the module AT+KSREP? +KSREP: 0,0,1 // Mode=0 which is restored from non-volatile memory OK // SIM card is not inserted // Reboot the module +SIM: 0 // URC after reboot +KSUP: 2 // Start-up report shows that the SIM is not present AT+KSREP? +KSREP: 1,2,0 // SIM is not present and the phone book is not ready OK +SIM: 1 // Insert SIM card +PBREADY // Phone Book is ready AT+KSREP? +KSREP: 1,0,1 // Start-up reporting is enabled. Both module and phone book // are ready OK |

## +WESHDOWN Command: Emergency Shutdown

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+WESHDOWN=? | Response +WESHDOWN: (list of supported <mode>s), (list of supported <gpio_index>es) OK |
| Read command Syntax AT+WESHDOWN? | Response +WESHDOWN: <mode>[,<gpio_index>] OK |
| Write command Syntax AT+WESHDOWN=<mode> [,<gpio_index>] | Response OK or +CME ERROR <err> Parameters <mode> 0 Disable emergency shutdown feature by GPIO 1 Enable emergency shutdown feature by GPIO 2 Trigger emergency shutdown Due to the quick shutdown of the module, the OK response sent by AT+WESHDOWN=2 might not be received by the application. <gpio_index> 1 – 8 Defines which GPIO will be used as input to trigger the emergency shutdown on the falling edge. Default value = 4 |
| Reference Sierra Wireless Proprietary Command | Notes - <gpio_index> is only used when <mode> = 1. - Parameters are not saved in non-volatile memory. They must be configured each time the module boots up. - GPIOs may already be used by +KSIMDET, +KSYNC, +KTEMPMON, +KGSMAD, +KSIMSLOT, etc. - Only GPIO 4 is available for use in the HL7650. Since this GPIO is also used to detect the insertion/removal of SIM2, this feature is disabled when emergency shutdown is activated. - This command can be used without a SIM. |
| Examples | AT+WESHDOWN=? +WESHDOWN: (0-2),(1-8) OK AT+WESHDOWN? +WESHDOWN: 0 // Emergency shutdown by GPIO is not active OK AT+WESHDOWN=1,4 // Activate emergency shutdown on GPIO4 OK AT+WESHDOWN? +WESHDOWN: 1,4 // A falling edge on GPIO4 will shut the module down OK AT+WESHDOWN=2 OK // Module shuts down |

# Network Service Related Commands

|  | - - - |
| --- | --- |

## +CLCK Command: Facility Lock

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CLCK=? | Response +CLCK: (list of supported <fac>s) OK or +CME ERROR: <err> |
| Write command Syntax AT+CLCK=<fac>,<mode> [,<passwd> [,<class>]] | Response If <mode> = 2 and command is successful OK +CLCK: <status>[,<class1>[<CR>,<LF> +CLCK: <status>,class2…]] or +CME ERROR: <err> Parameters <fac> Values reserved by the present document: "PS" PH-SIM (lock Phone to SIM/UICC card installed in the currently selected card slot) (MT asks for the password when other than current SIM/UICC card is inserted; MT may remember certain previously used cards thus not requiring password when they are inserted) "SC" SIM (lock SIM/UICC card) (SIM/UICC asks password in MT power-up and when this lock command issued) "AO" BAOC (Barr All Outgoing Calls) "OI" BOIC (Barr Outgoing International Calls) "OX" BOIC-exHC (Barr Outgoing International Calls except to Home Country) "AI" BAIC (Barr All Incoming Calls) "IR" BIC-Roam (Barr Incoming Calls when Roaming outside the home country) "AB" All Barring services (applicable only for mode>=0) "AG" All outgoing barring services (applicable only for <mode>=0) "AC" All incoming barring services (applicable only for <mode>=0) "FD" SIM card or active application in the UICC (GSM or USIM) fixed dialing memory feature (if PIN2 authentication has not been done during the current session, PIN2 is required as <passwd>) "PN" Network Personalization "PU" Network subset Personalization "PP" Service Provider Personalization "PC" Corporate Personalization <mode> 0 Unlock 1 Lock 2 Query status <status> 0 Not active 1 Active <passwd> String type; shall be the same as password specified for the facility from the ME user interface or with command Change Password +CPWD <classx> Sum of integers each representing a class of information (default value = 7) 2 Data (refers to all bearer services; with <mode>=2 this may refer only to some bearer service if TA does not support values 16, 32, 64 and 128) 4 Fax (facsimile services) 8 Short message service 16 Data circuit sync 32 Data circuit async 64 Dedicated packet access 128 Dedicated PAD access |

## +CNUM Command: Subscriber Number

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CNUM=? | Response OK |
| Execute command Syntax AT+CNUM | Response +CNUM: [<alpha1>],<number1>,<type1>[,<speed>,<service>[,<itc>]][<CR><LF> +CNUM: [<alpha2>],<number2>,<type2>[,<speed>,<service>[,<itc>]][...]] OK or +CME ERROR: <err> Parameters <alphax> Optional alphanumeric string associated with <numberx>; used character set should be the one selected with command +CSCS <numberx> String type phone number of format specified by <typex> <typex> Type of address octet in integer format <speed> As defined in 27.007 sub clause 6.7, corresponding to +CBST setting <service> Service related to the phone number 0 Asynchronous modem 1 Synchronous modem 2 PAD Access (asynchronous) 3 Packet Access (synchronous) 5 Fax <itc> Information transfer capability 0 3.1kHz 1 UDI |
| Note | MSISDN information should be stored in the SIM card before using +CNUM. |

## +COPN Command: Read Operator Name

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+COPN=? | Response OK |
| Execute command Syntax AT+COPN | Response +COPN: <numeric1>,<alpha1>[<CR><LF> +COPN: <numeric2>,<alpha2> [...]] OK or +CME ERROR: <err> Parameters <numeric> String type; operator in numeric format (see +COPS) <alpha> String type; operator in long alphanumeric format (see +COPS) |
| Notes | If the matching PLMN name is not found then the numeric PLMN ID (MCCMNC) will be displayed. |

## +COPS Command: Operator Selection

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+COPS=? | Response +COPS: [list of supported (<stat>, long alphanumeric <oper>, short alphanumeric <oper>, numeric <oper>[,< AcT>,<plmn_list>)s][,,(list of supported <mode>s),(list of supported <format>s)] OK or +CME ERROR: <err> |
| Read command Syntax AT+COPS? | Response +COPS: <mode>[,<format>,<oper>[,<AcT>]] OK or +CME ERROR: <err> |
| Write command Syntax AT+COPS= [<mode> [,<format> [,<oper> [,< AcT>]]]] | Response OK or +CME ERROR: <err> Parameters <mode> 0 Automatic; in this case other fields are ignored and registration is done automatically by ME 1 Manual (other parameters like format and operator need to be passed) 2 Deregister from network 3 Sets <format> value. In this case <format> becomes a mandatory input 4 Manual/automatic; if manual selection fails then automatic mode is entered <format> 0 Long alphanumeric; if network name is not available it displays a combination of MCC and MNC in string format 1 Short alphanumeric 2 Numeric <oper> String type given in format <format>; this field may be up to 16 character long for long alphanumeric format, up to 8 characters for short alphanumeric format and 5 characters long for numeric format (MCC/MNC codes) <stat> 0 Unknown networks 1 Network available 2 Current (registered) 3 Forbidden network <AcT> 0 GSM 2 UMTS 7 LTE <plmn_list> 0 PLMN is present on the EHPLMN list 1 PLMN is present on the user-controlled PLMN list 2 PLMN is present on the operator-controlled PLMN list Note that this parameter only supports R7 Protocol Stack onwards. |
| Notes | - This command forces an attempt to select and register the GSM, UMTS network. - Set command sets automatic network selection or selects network and a certain access technology AcT. - Read command returns current network. - Test command returns available networks and lists of supported <mode>s and <format>s. - This command is abortable. The port shall be freed for issuing another command. No network abort shall be triggered. - <mode>=0,1,2,4 and <oper> are saved in non-volatile memory over module reboot. - <format> is saved in non-volatile memory per AT port over module reboot. - +CME ERROR: 22 is returned if AT+COPS=0 is sent while data transmission is ongoing. |

## +CPOL Command: Preferred PLMN List

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CPOL=? | Response +CPOL: (list of supported <index>es),(list of supported <format>s) OK or +CME ERROR: <err> |
| Read command Syntax AT+CPOL? | Response +CPOL: <index1>,<format>,<oper1>[,<GSM_AcT1>,<GSM_Compact_AcT1>, <UTRAN_AcT1>,<EUTRAN_AcT>][<CR><LF> +CPOL: <index2>,<format>,<oper2>[,<GSM_AcT2>,<GSM_Compact_AcT2>, <UTRAN_AcT2>,<EUTRAN_AcT>][...]] OK or +CME ERROR: <err> |
| Write command Syntax AT+CPOL= [<index>] [,<format> [,<oper> [,<GSM_AcT>, <GSM_Compact_AcT>,<UTRAN_ AcT>,<EUTRAN_ AcT>]]] | Response OK or +CME ERROR: <err> Parameters <index> Integer type; order number of operator in the SIM/USIM preferred operator list <format> 0 Long format alphanumeric <oper> 1 Short format alphanumeric <oper> 2 Numeric <oper> <opern> String type; <format> indicates if the format is alphanumeric or numeric <GSM_AcT> 0 GSM access technology not selected 1 GSM access technology selected <GSM_Comp_AcT> 0 GSM compact access technology not selected 1 GSM compact access technology selected <UTRA_AcT> 0 UTRA access technology not selected 1 UTRA access technology selected <EUTRA_AcT> 0 EUTRA access technology not selected 1 EUTRA access technology selected |
| Notes | - The read command can have “n” RAT values. - If matching PLMN name is not found, then numeric PLMN ID (MCCMNC) will be displayed. - <format> is saved in non-volatile memory over module reboot. |

## +CPWD Command: Change Password

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CPWD=? | Response +CPWD: list of supported (<fac>,<pwdlength>)s OK |
| Write command Syntax AT+CPWD= <fac>,<oldpwd>, <newpwd> | Response OK or +CME ERROR: <err> Parameters <fac> "PS" PH-SIM (lock Phone to SIM/UICC card installed in the currently selected card slot) (MT asks for the password when other than current SIM/UICC card is inserted; MT may remember certain previously used cards thus not requiring password when they are inserted) "SC" SIM (lock SIM/UICC card) (SIM/UICC asks password in MT power-up and when this lock command issued) "AO" BAOC (Barr All Outgoing Calls) "OI" BOIC (Barr Outgoing International Calls) "OX" BOIC-exHC (Barr Outgoing International Calls except to Home Country) "AI" BAIC (Barr All Incoming Calls) "IR" BIC-Roam (Barr Incoming Calls when Roaming outside the home country) "FD" SIM card or active application in the UICC (GSM or USIM) fixed dialing memory feature (if PIN2 authentication has not been done during the current session, PIN2 is required as <passwd>) "PN" Network Personalization "PU" Network subset Personalization "PP" Service Provider Personalization "PC" Corporate Personalization <oldpwd> String type containing the old password <newpwd> String type containing the new password <pwdlength> Length of password |

## +CREG Command: Network Registration

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CREG=? | Response +CREG: (list of supported <n>s) OK |
| Read command Syntax AT+CREG? | Response +CREG: <n>,<stat>[,<lac>,<ci>[,<AcT>]] OK |
| Write command Syntax AT+CREG=[<n>] | Response OK or +CME ERROR: <err> Parameters <n> 0 Disable network registration unsolicited result code 1 Enable network registration unsolicited result code +CREG: <stat> 2 Enable network registration and location information unsolicited result code +CREG: <stat>[,<lac>,<ci>[,<Act>]] <stat> 0 Not registered, ME is not currently searching a new operator to register to 1 Registered, home network 2 Not registered, but ME is currently searching a new operator to register to 3 Registration denied 4 Unknown 5 Registered, roaming <lac> String type; two byte location area code in hexadecimal format (e.g. "00C3" equals 195 in decimal) <ci> String type; four byte UTRAN/E-UTRAN cell ID in hexadecimal format <AcT> 0 GSM 2 UTRAN 3 GSM with EGPRS 4 UTRAN with HSDPA 5 UTRAN with HSUPA 6 UTRAN with HSDPA and HSUPA 7 E-UTRAN |
| Unsolicited Notification | Response When <n>=1 and there is a change in the ME network registration status code: +CREG: <stat> When <n>=2 and there is a change in the network cell: +CREG: <stat>[,<lac>,<ci>[,<Act>]] |
| Notes | <n> is saved in non-volatile memory per AT port over module reboot. |

## +CPLS Command: Select Preferred PLMN List

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CPLS=? | Response +CPLS: (list of supported <cpls_list>s) OK |
| Read command Syntax AT+CPLS? | Response +CPLS: <cpls_list> OK |
| Write command Syntax AT+CPLS= [<cpls_list>] | Response OK or +CME ERROR: <err> Parameter <cpls_list> 0 User controlled PLMN selector with access technology EFPLMNwAcT, but iff not found in the SIM/UICC, then the PLMN preferred list is EFPLMNsel 1 Operator controlled PLMN selector with access technology EFOPLMNwAcT 2 HPLMN selector with access technology EFHPLMNwAcT |

## +CEREG Command: EPS Network Registration Status

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CEREG=? | Response +CEREG: (list of supported <n>s) OK |
| Read command Syntax AT+CEREG? | Response +CEREG: <n>,<stat>[,<tac>,<ci>[,<AcT>]] OK |
| Execute command Syntax AT+CEREG= [<n>] | Response OK or +CME ERROR: <err> Parameters <n> 0 Disable network registration unsolicited result code 1 Enable network registration unsolicited result code +CEREG: <stat> 2 Enable network registration unsolicited result code +CEREG: <stat> [,<tac>,<ci>[,<AcT>]] <stat> 0 Not registered, MT is not currently searching an operator to register to 1 Registered on the home network 2 Not registered, but MT is currently trying to attach or searching for an operator to register to 3 Registration denied 4 Unknown 5 Registered, roaming 8 Attached for emergency bearer services only (note that this is only available when <AcT> = 2,4,5,6 <tac> String type; two-byte tracking area code in hexadecimal format (e.g. “00C3” is equals to 195 in decimal) <ci> String type; four-byte UTRAN/E-UTRAN cell ID in hexadecimal format <AcT> 2 UTRAN 4 UTRAN with HSDPA 5 UTRAN with HSUPA 6 UTRAN with HSDPA and HSUPA 7 E-UTRAN |
| Notes | <n> is saved in non-volatile memory per AT port over module reboot. |

## +CEMODE Command: UE Modes of Operation for EPS

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CEMODE=? | Response +CEMODE: (list of supported <mode>s) OK |
| Read command Syntax AT+CEMODE? | Response +CEMODE: <mode> OK |
| Write command Syntax AT+CEMODE= [<mode>] | Response OK or +CME ERROR: <err> Parameter <mode> Indicates mode of operation 0 PS mode 2 of operation (default value for HL7618 and HL7618RD) 1 CS/PS mode 1 of operation (default value for HL7650, HL7688 and HL7692) 2 CS/PS mode 2 of operation (default value for HL7648 and HL7690) 3 PS mode 1 of operation |
| Notes | <mode> is saved in non-volatile memory over module reboot. |

# Phone Book Management

## +CPBF Command: Find Phonebook Entries

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CPBF=? | Response +CPBF: [<nlength>],[<tlength>],[<glength>],[<slength>],[<elength>] OK or +CME ERROR: <err> |
| Write command Syntax AT+CPBF= <findtext> | Response [+CPBF:<index1>,<number>,<type>,<text>[,<hidden>][,<group>][,<adnumber>] [,<adtype>][,<secondtext>][,<email>]] OK or +CME ERROR: <err> Parameters <index1>, <index2> , <index> Integer type values in the range of location numbers of phonebook memory <number> String type phone number of format <type> <type> Type of address octet in integer format <text> String type field of maximum length <tlength>; character set as specified by command +CSCS <group> String type field of maximum length <glength>; character set as specified by command +CSCS <adnumber> String type phone number of format <adtype> <adtype> Type of address octet in integer format <secondtext> String type field of maximum length <slength>; character set as specified by command +CSCS <email> String type field of maximum length <elength>; character set as specified by command +CSCS <nlength> Integer type value indicating the maximum length of field <number> <tlength> Integer type value indicating the maximum length of field <text> <glength> Integer type value indicating the maximum length of field <group> <slength> Integer type value indicating the maximum length of field <secondtext> <elength> Integer type value indicating the maximum length of field <email> <hidden> Indicates if the entry is hidden or not 0 Phonebook entry not hidden 1 Phonebook entry hidden |
| Notes | Execution command returns phonebook entries (from the current phonebook memory storage selected with +CPBS). |

## +CPBR Command: Read Current Phonebook Entries

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CPBR=? | Response +CPBR: (list of supported <index>es),[<nlength>],[<tlength>],[<glength>],[<alength>], [<slength>],[<elength>] OK |
| Write command Syntax AT+CPBR= <index1> [,<index2>] | Response [+CPBR: <index1>,<number>,<type>,<text>[,<hidden>][,<group>][,<adnumber>] [,<adtype>][,<secondtext>][,<email>]][[...] OK or +CME ERROR: <err> Parameters <index1>, <index2>, <index> Integer type values in the range of location numbers of phonebook memory <number> String type phone number of format <type> <type> Type of address octet in integer format <text> String type field of maximum length <tlength> <hidden> Indicates if the entry is hidden or not – only available if a UICC with an active USIM application is present 0 Phonebook entry not hidden 1 Phonebook entry hidden <group> String type field of maximum length <glength> <adnumber> String type phone number of format <adtype> <adtype> Type of address octet in integer format <secondtext> String type field of maximum length <slength> <email> String type field of maximum length <elength> <nlength> Integer type value indicating the maximum length of field <number> <tlength> Integer type value indicating the maximum length of field <text> <glength> Integer type value indicating the maximum length of field <group> <alength> Integer type value indicating the maximum length of field <adnumber> <slength> Integer type value indicating the maximum length of field <secondtext> <elength> Integer type value indicating the maximum length of field <email> |
| Notes | - Optional parameters <nlength>, <tlength>, <glength>, <alength>, <slength>, <elength> are only applicable for 3G UICC. - Execution command returns phonebook entries in location number range <index1>...<index2> from the current phonebook memory storage selected with +CPBS. |

## +CPBS Command: Select Phonebook Memory Storage

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CPBS=? | Response +CPBS: (list of supported <storage>s) OK |
| Read command Syntax AT+CPBS? | Response +CPBS: <storage>[,<used>,<total>] OK or +CME ERROR:<err> |
| Write command Syntax AT+CPBS= <storage> [,<password>] | Response OK or +CME ERROR:<err> Parameters <storage> "FD" SIM/USIM fixdialing phonebook "LD" SIM/UICC last dialing phonebook (LD phonebook can't be deleted) "ON" SIM (or MT) own numbers (MSISDNs) list (reading of this storage may be available through +CNUM also) "SM" SIM/UICC phonebook (default) "BL" Blacklist phonebook (delete only) "EN" SIM emergency-call-codes phonebook (read only) "AP" Selected application phonebook "BN" SIM barred-dialing-number (EF_BDN) phonebook (only valid with PIN2) "SN" SIM service-dialing-number (EF_SDN) phonebook (read only) <password> String type value representing the PIN2-code required when selecting PIN2 code locked <storage>s above <used> Integer type value indicating the number of used locations in the selected memory <total> Integer type value indicating the total number of locations in the selected memory |
| Notes | Set command selects phonebook memory storage <storage>, which is used by other phonebook commands. |

## +CPBW Command: Write Phonebook Entry

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CPBW=? | Response +CPBW: (list of supported <index>es),[<nlength>],(list of supported <type>s),[<tlength>], [<glength>],[<alength>],[<slength>],[<elength>] OK |
| Read command Syntax AT+CPBW? | Response +CPBW: <written_index> OK or +CPBW:-1 OK |
| Write command Syntax AT+CPBW= [<index>] [,<number> [,<type>[,<text> [,<group> [,<adnumber> [,<adtype> [,<secondtext> [,<email> [,<hidden>]]]]]]]]] | Response +CPBW: <written_index> OK or +CME ERROR: <err> Parameters <index> Integer type values in the range of location numbers of phonebook memory <number> String type phone number of format <type> <type> Type of address octet in integer format; default value is 145 when dialing string includes international access code character "+"; otherwise, default value is 129 <text> String type field of maximum length <tlength> <hidden> Indicates if the entry is hidden or not – only available if a UICC with an active USIM application is present 0 Phonebook entry not hidden 1 Phonebook entry hidden <group> String type field of maximum length <glength> <adnumber> String type phone number of format <adtype> <adtype> Type of address octet in integer format <secondtext> String type field of maximum length <slength> <email> String type field of maximum length <elength> <nlength> Integer type value indicating the maximum length of field <number> <tlength> Integer type value indicating the maximum length of field <text> <glength> Integer type value indicating the maximum length of field <group> <alength> Integer type value indicating the maximum length of field <adnumber> <slength> Integer type value indicating the maximum length of field <secondtext> <elength> Integer type value indicating the maximum length of field <email> |
| Notes | - Optional parameters <nlength>, <tlength>, <glength>, <alength>, <slength>, <elength> are only applicable for 3G UICC. - Execution command writes phonebook entry in location number <index> in the current phonebook memory storage selected with +CPBS. |

# SMS Commands

For other information regarding HL78xx SMS commands, refer to section 1.5 SMS Commands.

## Parameters Definition

The following parameters are used in the subsequent clauses which describe all commands. The formats of integer and string types referenced here are defined in V.25ter.

The default values are for command parameters, not for result code parameters.

### Message Storage Parameters

<index>	Integer type; value in the range of location numbers supported by the associated memory

<mem1>	String type; memory from which messages are read and.or deleted (by commands 			+CMGL, +CMGR and +CMGD); defined values are as follows:

"BM"		Broadcast message storage

"ME"		ME message storage

"MT"		Any of the storages associated with ME

"SM"		(U)SIM message storage ; default value

"TA"		TA message storage

"SR"		Status report storage

<mem2>	String type; memory to which writing and sending operations are made (commands Send 		Message from Storage +CMSS and Write Message to Memory +CMGW); refer <mem1> 		for defined values. Default value is “SM”.

<mem3>	String type; preferred memory to which received SMs are to be stored (unless forwarded 		directly to TE; refer command New Message Indications +CNMI); refer <mem1> for 			defined values; received CBMs are always stored in "BM" (or some manufacturer specific 		storage) unless directly forwarded to TE; received status reports are always stored in 			"SR" (or some manufacturer specific storage) unless directly forwarded to TE. Default 			value is “SM”.

<stat>		Status of message in memory. Integer type in PDU mode, or string type in text mode. 			Available values are as follows:

0	"REC UNREAD"		Received unread message (i.e. new message)

1	"REC READ" 		Received read message

2	"STO UNSENT" 		Stored unsent message (only applicable to SMs)

3	"STO SENT" 		Stored sent message (only applicable to SMs)

4	"ALL" 				All messages (only applicable to +CMGL command)

<total1>	Integer type; total number of message locations in <mem1>

<total2>	Integer type; total number of message locations in <mem2>

<total3>	Integer type; total number of message locations in <mem3>

<used1>	Integer type; number of messages currently in <mem1>

<used2>	Integer type; number of messages currently in <mem2>

<used3>	Integer type; number of messages currently in <mem3>

### Message Data Parameters

<ackpdu>	RP-User-Data element of RP-ACK PDU; format is same as for <pdu> in case of SMS, 			but without SC address field and parameter shall be bounded by double quote characters 		like a normal string type parameter.

<alpha>	String type alphanumeric representation of <da> or <oa> corresponding to the entry 			found in MT phonebook; implementation of this feature is manufacturer specific; used 			character set should be the one selected with command +CSCS.

<cdata>	Command data in text mode responses; ME/TA converts each 8-bit octet into two	IRA 			character long hexadecimal number (e.g. octet with integer value 42 is presented to TE 			as two characters 2A (IRA 50 and 65)).

<ct>		Command type in integer format (default value = 0).

<da>	Address value in string format. BCD numbers (or GSM 7-bit default alphabet characters) are converted to characters of the currently selected TE character set (refer to command +CSCS). Type of address is given by <toda>.

<data>	In the case of user data in text mode responses; format:

if <dcs> indicates that GSM 7-bit default alphabet is used and <fo> indicates that user data header indication is not set

  - if TE character set other than "HEX" (refer to command +CSCS): ME/TA converts GSM alphabet into current TE character set

  - if TE character set is "HEX": ME/TA converts each 7-bit character of GSM 7-bit default alphabet into two IRA character long hexadecimal number (e.g. character  (GSM 7-bit default alphabet 23) is presented as 17 (IRA 49 and 55))

if <dcs> indicates that 8-bit or UCS2 data coding scheme is used, or <fo> indicates that user data header indication is set: ME/TA converts each 8-bit octet into two IRA character long hexadecimal number (e.g. octet with integer value 42 is presented to TE as two characters 2A (IRA 50 and 65))

In the case of CBS: CBM Content of Message in text mode responses; format:

if <dcs> indicates that GSM 7-bit default alphabet is used

  - if TE character set other than "HEX" (refer to command +CSCS); ME/TA converts GSM alphabet into current TE character set

  - if TE character set is "HEX"; ME/TA converts each 7-bit character of the GSM 7-bit default alphabet into two IRA character long hexadecimal number

if <dcs> indicates that 8-bit or UCS2 data coding scheme is used; ME/TA converts each 8-bit octet into two IRA character long hexadecimal number

<length>	Integer type vlayue indicating the length of the actual TP data unit in octets in PDU mode. 		This is 140 characters long according to 8-bit GSM coding scheme.

In text mode, the maximum length of an SMS depends on the used coding scheme (160 			characters if 7-bit).

<mid>		CBM Message Identifier in integer format

<mn>		TP-Message-Number in integer format

<mr>		Message reference in integer format

<oa>		Origiantion address address value field in string format; BCD numbers (or GSM 7-bit			default alphabet characters) are converted to characters of the 	currently selected TE 			character set (refer to command +CSCS); type of address given by <tooa>

<page>	CBM Page Parameter bits 4-7 in integer format

<pages>	CBM Page Parameter bits 0-3 in integer format

<pdu>	GSM 04.11 SC address followed by GSM 03.40 TPDU in hexadecimal format

In the case of CBS, TPDU in hexadecimal format

<pid>		Protocol identifier in integer format. Default value is 0

<ra>		Recipient address address value in string format; BCD numbers (or GSM 7-bit default 			alphabet characters) are converted to characters of the currently selected TE character 			set (refer to command +CSCS); type of address is given by <tora>

<sca>		String value enclosed in quotes indicating the service center address. Note that BCD 			numbers are converted to characters; type of address is given by <tosca>

<scts>	Service centre time stamp in time-string format (refer to <dt>)

<sn>		CBM Serial Number in integer format

<st>		Status in integer format

<toda>	Type of address octet in integer format. Default value is 145 if the first character of <da> 			is “+”; otherwise, default value is 129

<tooa>	Originating address type of address octet in integer format (refer to <toda> for the default 		value)

<tora>	Recipient address type of address octet in integer format (refer to <toda> for the default 			value)

<tosca>	SC address type of address octet in integer format (refer to <toda> for the default value)

<vp>		Depending on SMS-SUBMIT <fo> setting: TP-Validity-Period either in integer format 			(default value = 167) or in time-string format (refer to <dt>)

<vp>		Validity period in either integer format (default value = 167) or in time-string format 			depending on <fo> settings

<dcs>		SMS Data Coding Scheme (default value = 0), or Cell Broadcast Data Coding Scheme in 		integer format

<dt>		Discharge time in time-string format “yy/MM/dd,hh:mm:ss+zz” where the characters 			indicate year, month, day, hour, minutes, seconds and time zone.

For example, May 6, 1994, 10:10 pm GMT+2 hours is equals to “94/05/06,22:10:00+08”

<fo>		First octet of SMS-DELIVER, SMS-SUBMIT (default value = 17), SMS-STATUS-				REPORT, or SMS-COMMAND (default value = 2) in integer format depending on 				command or result code

## +CMGD Command: Delete Message

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CMGD=? | Response +CMGD: (list of supported <index>es)[,(list of supported <delflag>s)] OK |
| Write command Syntax AT+CMGD= <index> [,<delflag>] | Response OK or +CMS ERROR: <err> Parameters <delflag> Integer indicating multiple message deletion request 0 (or omitted) Delete the message specified in <index> 1 Delete all read messages from preferred message storage, leaving unread messages and stored mobile originated messages (whether sent or not) untouched 2 Delete all read messages from preferred message storage and sent mobile originated messages, leaving unread messages and unsent mobile originated messages untouched 3 Delete all read messages from preferred message storage, sent and unsent mobile originated messages leaving unread messages untouched 4 Delete all messages from preferred message storage including unread messages |
| Notes | Execution command deletes message from preferred message storage <mem1>, location <index>. If <delflag> is present and not set to 0 then the ME shall ignore <index> and follow the rules for <delflag> shown above. |

## +CMGF Command: Set Message Format

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CMGF=? | Response +CMGF: (list of supported <mode>s) OK |
| Read command Syntax AT+CMGF? | Response +CMGF: <mode> OK |
| Execute command Syntax AT+CMGF= [<mode>] | Response OK or +CMS ERROR: err> Parameter <mode> 0 PDU mode (default when implemented) 1 Text mode |
| Notes | <mode> is saved in non-volatile memory per AT port over module reboot. |

## +CMGL Command: List Messages

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CMGL=? | Response +CMGL: (list of supported <stat>s) OK |
| Execute command Syntax AT+CMGL [=<stat>] | Response If in text mode, command is successful and SMS-SUBMITs and/or SMS-DELIVERs: +CMGL: <index>,<stat>, <oa/da>,[<alpha>], [<scts>][,<tooa/toda>,<length>] <CR><LF><data>[<CR><LF> +CMGL: <index>,<stat>, <da/oa>,[<alpha>], [<scts>][,<tooa/toda>, <length>] <CR><LF><data> [...]] If in text mode, command is successful and SMS-STATUS-REPORTs: +CMGL: <index>, <stat>,<fo>, <mr>, [<ra>], [<tora>], <scts>, <d-t>,<st>[<CR><LF> +CMGL: <index>, <stat>, <fo>, <mr>,[<ra>], [<tora>],<scts>,<d_t>,<st>[...]] If in text mode, command is successful and SMS-COMMANDs: +CMGL: <index>,<stat>,<fo>,<ct> [<CR><LF> +CMGL: <index>,<stat>, <fo>,<ct>[...]] If in text mode, command is successful and CBM storage: +CMGL : <index>,<stat>,<sn>, <mid>, <page>,<pages> <CR><LF><data>[<CR><LF> +CMGL : <index>,<stat>,<sn>, <mid>,<page>,<pages><CR><LF><data>[...]] If in PDU mode and command is successful: +CMGR: <stat>,[<alpha>],<length><CR><LF><pdu> or +CMS ERROR: <err> Parameters For parameter information and values, refer to section 8.1 Parameters Definition. |

## +CMGR Command: Read Message

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CMGR=? | Response OK |
| Write command Syntax AT+CMGR= <index> | Response If text mode (+CMGF=1), command is successful, and SMS-DELIVER: +CMGR: <stat>,<oa>,[<alpha>],<scts>[,<tooa>,<fo>,<pid>,<dcs>,<sca>,<tosca>, <length>]<CR><LF><data> if text mode (+CMGF=1), command is successful, and SMS-SUBMIT: +CMGR: <stat>,<da>,[<alpha>][,<toda>,<fo>,<pid>,<dcs>,[<vp>],<sca>,<tosca>, <length>]<CR><LF><data> if text mode (+CMGF=1), command is successful, and SMS-STATUS-REPORT: +CMGR: <stat>,<fo>,<mr>,[<ra>], [<tora>],<scts>,<d_t>,<st> if text mode (+CMGF=1), command is successful, and SMS-COMMAND: +CMGR: <stat>, <fo>,<ct>[, <pid>,[<mn>],[<da>],[<toda>],<length><CR><LF><cdata>] if text mode (+CMGF=1), command is successful, and CBM storage: +CMGR: <stat>,<sn>, <mid>,<dcs>,<page>,<pages><CR><LF><data> if PDU mode (+CMGF=0) and command is successful: +CMGR: <stat>,[<alpha>],<length><CR><LF><pdu> or +CMS ERROR: <err> Parameters For parameter information and values, refer to section 8.1 Parameters Definition. |

## +CMGS Command: Send Message

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CMGS=? | Response OK |
| Write command Syntax If text mode (+CMGF=1): AT+CMGS=<da> [,<toda>]<CR> text is entered <ctrl-Z/ESC> If PDU mode (+CMGF=0): AT+CMGS= <length><CR> PDU is given <ctrl-Z/ESC> | Response If text mode (+CMGF=1) and sending is successful: [+CMGS: <mr>[,<scts>]] OK if PDU mode (+CMGF=0) and sending is successful: [+CMGS: <mr>] OK or +CMS ERROR: <err> Parameters For parameter information and values, refer to section 8.1 Parameters Definition. |
| Notes | - The TA shall send a four character sequence <CR><LF><greater_than><space> (IRA 13, 10, 62, 32) after command line is terminated with <CR>; after that PDU can be given from TE to ME/TA. - The PDU shall be hexadecimal format (similarly as specified for <pdu>) and given in one line; ME/TA converts this coding into the actual octets of PDU. - When the length octet of the SMSC address (given in the PDU) equals zero, the SMSC address set with command Service Centre Address +CSCA is used; in this case the SMSC Type-of-Address octet shall not be present in the PDU, i.e. TPDU starts right after SMSC length octet. - Sending can be cancelled by giving <ESC> character. - <ctrl-Z> must be used to indicate the ending of PDU. - +CMGS: <mr>[,<scts>] is not available in +CMGS intermediate response as SMS is sent over IMS using 3GPP2 SMS PDU format and protocol. |

## +CMGW Command: Write Message to Memory

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CMGW=? | Response OK |
| Write command Syntax If text mode (+CMGF=1): AT+CMGW[= <oa/da> [,<tooa/toda> [,<stat>]]]<CR> text is entered <ctrl-Z/ESC> If PDU mode (+CMGF=0): AT+CMGW= <length>[,<stat>]<CR> PDU is given <ctrl-Z/ESC> | Response +CMGW: <index> OK or +CMS ERROR: <err> Parameters For parameter information and values, refer to section 8.1 Parameters Definition. |
| Notes | - Execution command stores a message to memory storage <mem2>, and memory location <index> of the stored message is returned. - By default, message status will be set to 'stored unsent', but parameter <stat> also allows other status values to be given. (ME/TA manufacturer may choose to use different default <stat> values for different message types.) - Entering of PDU is done similarly as specified in command +CMGS. |

## +CMSS Command: Send Message from Storage

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CMSS=? | Response OK |
| Write command Syntax AT+CMSS= <index>[,<da> [,<toda>]] | Response If text mode (+CMGF=1) and sending issuccessful: +CMSS: <mr>[,<scts>] If PDU mode (+CMGF=0) and sending is successful: +CMSS: <mr> OK or +CMS ERROR: <err> Parameters For parameter information and values, refer to section 8.1 Parameters Definition. |
| Notes | - Execution command sends message with location value <index> from message storage <mem2> to the network (SMS-SUBMIT or SMS-COMMAND). If new recipient address <da> is given for SMS-SUBMIT, it shall be used instead of the one stored with the message. - Reference value <mr> is returned to the TE on successful message delivery. Optionally (when +CSMS <service> value is 1 and network supports the feature), <scts> is returned in text mode. |

## +CNMI Command: New Message Indication

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CNMI=? | Response +CNMI: (list of supported <mode>s), (list of supported <mt>s), (list of supported <bm>s), (list of supported <ds>es), (list of supported <bfr>s) OK |
| Read command Syntax AT+CNMI? | Response +CNMI: <mode>,<mt>,<bm>,<ds>,<bfr> OK |
| Write command Syntax +CNMI=[<mode> [,<mt>[,<bm> [,<ds>[,<bfr>]]]]] | Response OK or +CMS ERROR: <err> Parameters <mode> 0 Buffer unsolicited result codes in the TA. If TA result code buffer is full, indications can be buffered in some other place or the oldest indications may be discarded and replaced with the new received indications. 1 Discard indication and reject new received message unsolicited result codes when TA-TE link is reserved. Otherwise forward them directly to the TE. 2 Buffer unsolicited result codes in the TA when TA-TE link is reserved (e.g. in on-line data mode) and flush them to the TE after reservation. Otherwise forward them directly to the TE. <mt> 0 No indications are routed to the TE. 1 Result code is sent when ME does not have any other display device other than the AT interface 2 Acknowledgement command must be sent when +CSMS <service> = 1 and ME does not have any other display device other than the AT interface 3 Acknowledgement command must be sent when +CSMS <service> = 1 <bm> 0 No CBM indications are routed to the TE. 1 If CBM is stored into ME/TA, indication of the memory location is routed to the TE using unsolicited result code: +CBMI: <mem>,<index> 2 New CBMs are routed directly to the TE using unsolicited result code: +CBM: <length><CR><LF><pdu> (PDU mode enabled); or +CBM: <sn>,<mid>, <dcs>,<page>,<pages><CR><LF><data> (text mode enabled) 3 Class 3 CBMs are routed directly to TE using unsolicited result codes defined in <bm>=2. If CBM storage is supported, messages of other classes result in indication as defined in <bm>=1 <ds> 0 No SMS-STATUS-REPORTs are routed to the TE. 1 SMS-STATUS-REPORTs are routed to the TE using unsolicited result code: +CDS: <length><CR><LF><pdu> (PDU mode enabled) or +CDS: <fo>,<mr>,[<ra>],[<tora>],<scts>,<dt>, <st> (text mode enabled) 2 If SMS-STATUS-REPORT is stored in ME/TA, indication of the memory location is routed to the TE using unsolicited result code: +CDSI: <mem>,<index> <bfr> 0 TA buffer of unsolicited result codes defined within this command is flushed to the TE when <mode> = 1 – 3 is entered 1 TA buffer of unsolicited result codes defined within this command is cleared when <mode> = 1 – 3 is entered |
| Notes | <mode>, <mt>, <bm> and <ds> are saved in non-volatile memory over module reboot; URC is available on the port that executes the command. |
| Examples | AT+CNMI=1 // Write command OK AT+CNMI=? // Test command +CNMI: (0-2),(0-3),(0-3),(0-2),(0-1) OK AT+CNMI? // Read command +CNMI: 1,0,0,0,0 OK |

## +CSCA Command: Service Center Address

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CSCA=? | Response OK |
| Read command Syntax AT+CSCA? | Response +CSCA: <sca>,<tosca> OK |
| Write command Syntax AT+CSCA=<sca> [,<tosca>] | Response OK or +CMS ERROR: <err> Parameters For parameter information and values, refer to section 8.1 Parameters Definition. |

## +CSMP Command: Set Text Mode Parameters

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CSMP=? | Response OK |
| Read command Syntax AT+CSMP? | Response +CSMP: <fo>,<vp>,<pid>,<dcs> OK |
| Write command Syntax AT+CSMP=[<fo> [,<vp>[,<pid> [,<dcs>]]]] | Response OK Parameters For parameter information and values, refer to section 8.1 Parameters Definition. |

## +CSMS Command: Select Message Service

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CSMS=? | Response +CSMS: (list of supported <service>s) OK |
| Read command Syntax AT+CSMS? | Response +CSMS: <service>,<mt>,<mo>,<bm> OK |
| Write command Syntax AT+CSMS= <service> | Response +CSMS: <mt>,<mo>,<bm> OK or +CMS ERROR: <err> Parameters <service> 0 3GPP TS 23.040 and 3GPP TS 23.041 1 3GPP TS 23.040 and 3GPP TS 23.041 (the requirement of setting <service> =1 is mentioned in the corresponding command description) <mt> Message terminated messages 0 Type not supported 1 Type supported <mo> Message originated messages 0 Type not supported 1 Type supported <bm> Broadcast type messages 0 Type not supported 1 Type supported |

## +CPMS Command: Preferred Message Storage

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CPMS=? | Response +CPMS: (list of supported <mem1>s), (list of supported <mem2>s), (list of supported <mem3>s) OK |
| Read command Syntax AT+CPMS? | Response +CPMS: <mem1>,<used1>,<total1>,<mem2>,<used2>,<total2>,<mem3>,<used3>,<total3> OK or +CMS ERROR: <err> |
| Write command Syntax AT+CPMS= <mem1> [,<mem2> [,<mem3>]] | Response +CPMS: <used1>,<total1>,<used2>,<total2>,<used3>,<total3> OK or +CMS ERROR: <err> Parameters For parameter information and values, refer to section 8.1 Parameters Definition. |
| Notes | <mem1>, <mem2> and <mem3> are saved in non-volatile memory over module reboot. |

## +CSDH Command: Show Text Mode Parameters

For HL7618, HL7618RD, HL7648, HL7688, HL7690 and HL7692 only.

| HL7618, HL7618RD, HL7648, HL7688, HL7690 and HL7692 |  |
| --- | --- |
| Test command Syntax AT+CSDH=? | Response +CSDH: (list of supported <show>s) OK |
| Read command Syntax AT+CSDH? | Response +CSDH: <show> OK |
| Write command Syntax AT+CSDH= [<show>] | Response OK or +CME ERROR: <err> Parameter <show> 0 Do not show header values defined in commands +CSCA and +CSMP (<sca>, <tosca>, <fo>, <vp>, <pid> and <dcs>) nor <length>, <toda> or <tooa> in +CMGL, +CMGR result codes for SMS-DELIVERs and SMS-SUBMITs in text mode; for SMS-COMMANDs in +CMGR result code, do not show <pid>, <mn>, <da>, <toda>, <length> or <cdata> 1 Show values in result codes |

# Packet Domain Commands

For additional details about PDP context use for the HL7618 and HL7618RD, refer to section 1.4 PDP Context Usage.

## +CGATT Command: PS Attach or Detach

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CGATT=? | Response +CGATT: (list of supported <state>s) OK |
| Read command Syntax AT+CGATT? | Response +CGATT: <state> OK |
| Write command Syntax AT+CGATT= [<state>] | Response OK or ERROR Parameter <state> State of PS attachment 0 Detached 1 Attached |

## +CGACT Command: Activate or Deactivate PDP Context

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CGACT=? | Response +CGACT: (list of supported <state>s) OK |
| Read command Syntax AT+CGACT? | Response +CGACT: <cid>, <state> [<CR><LF>+CGACT: <cid>,<state> [...]] OK |
| Write command Syntax AT+CGACT= [<state> [,<cid> [,<cid> [,...]]]] | Response OK or ERROR Parameters <state> State of PDP context activation 0 Deactivated 1 Activated <cid> Numeric parameter which specifies a particular PDP context definition. |
| Notes | Up to three (3) PDP contexts can be active at once. |

## +CGCMOD Command: Modify PDP Context

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CGCMOD=? | Response +CGCMOD: (list of <cid>s addociated with active contexts) OK |
| Write command Syntax AT+CGCMOD= [<cid>[,<cid> [,...]]] | Response OK or +CME ERROR: <err> Parameter <cid> Numeric parameter which specifies a particular PDP context definition (see +CGDCONT and +CGDSCONT) |

## +CGTFT Command: Traffic Flow Template

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CGTFT=? | Response +CGTFT: <PDP_type>, (list of supported <packet filter identifier>s) , (list of supported <evaluation precedence index>s), (list of supported <source address and subnet mask>s), (list of supported <protocol number (ipv4) / next header (ipv6)>s), (list of supported <destination port range>s), (list of supported <source port range>s), (list of supported <ipsec security parameter index (spi)>s), (list of supported <type of service (tos) (ipv4) and mask / traffic class (ipv6) and mask>s), (list of supported <flow label (ipv6)>s), (list of supported <direction>s) [<CR><LF>+CGTFT: <PDP_type>, (list of supported <packet filter identifier>s), (list of supported <evaluation precedence index>s), (list of supported <source address and subnet mask>s), (list of supported <protocol number (ipv4) / next header (ipv6)>s), (list of supported <destination port range>s),(list of supported <source port range>s), (list of supported <ipsec security parameter index (spi)>s), (list of supported <type of service (tos) (ipv4) and mask / traffic class (ipv6) and mask>s), (list of supported <flow label (ipv6)>s), (list of supported <direction>s)[...]] |
| Read command Syntax AT+CGTFT? | Response +CGTFT: <cid>, <packet filter identifier>,<evaluation precedence index>, <source address and subnet mask>, <protocol number (ipv4) / next header (ipv6)>, <destination port range>, <source port range>, <ipsec security parameter index (spi)>, <type of service (tos) (ipv4) and mask / traffic class (ipv6) and mask>, <flow label (ipv6)>, <direction> [<CR><LF>+CGTFT: <cid>, <packet filter identifier>, <evaluation precedence index>, <source address and subnet mask>, <protocol number (ipv4) / next header (ipv6)>, <destination port range>, <source port range>, <ipsec security parameter index (spi)>, <type of service (tos) (ipv4) and mask / traffic class (ipv6) and mask>, <flow label (ipv6)>, <direction> [...]] |
| Write command Syntax AT+CGTFT= [<cid>,[<packet filter identifier>, <evaluation precedence index> [,<source address and subnet mask> [,<protocol number (ipv4) / next header (ipv6)> [,<destination port range> [,<source port range> [,<ipsec security parameter index (spi)> [,<type of service (tos) (ipv4) and mask / traffic class (ipv6) and mask> [,<flow label (ipv6)>, <direction> ]]]]]]]]]] | Response OK or ERROR Parameter <cid> Numeric parameter which specifies a particular PDP context definition (see +CGDCONT and +CGDSCONT) <packet filter identifier> Numeric parameter with value range from 1 to 16 <evaluation precedence index> Numeric parameter with value range from 0 to 255 <source address and subnet mask> String tpe given as a dot-separated numeric (0 – 255) parameter of the form "a1.a2.a3.a4.m1.m2.m3.m4" for IPv4 or "a1.a2.a3.a4.a5.a6.a7.a8.a9.a10.a11.a12.a13. a14.a15.a16.m1.m2.m3.m4.m5.m6.m7.m8. m9.m10.m11.m12.m13.m14.m15.m16" for IPv6 <protocol number (ipv4) / next header (ipv6)> Numeric parameter with value range from 0 to 255 <destination port range> String type given as a dot-separated numeric (0 – 65535) parameter on the form ‘f.t.’ <source port range> String type given as a dot-separated numeric (0 – 65535) parameter on the form ‘f.t.’ <ipsec security parameter index (spi)> Numeric value in hecadecimal format with value range from 00000000 to FFFFFFFF <type of service (tos) (ipv4) and mask / traffic class (ipv6) and mask> String type given as a dot-separated numeric (0 – 255) parameter on the form ‘t.m.’ <flow label (ipv6)> Numeric value in hecadecimal format with value range from 00000 to FFFFF. Valid for IPv6 only <direction> Specifies the transmission direction in which the packet filter shall be applied 1 Uplink 2 Downlink 3 Birectional (up and downlink ; default if omitted) |
| Notes | - Some of the listed attributes above may coexist in a Packet Filter while others mutually exclude each other. For the list of possible combinations, refer to 3GPP TS 23.060. - +CGTFT=<cid> causes all of the packet filters in the TFT for context number <cid> to become undefined. |

## +CGDCONT Command: Define PDP Context

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CGDCONT=? | Response +CGDCONT: (range of supported <cid>s), <PDP_type>,,,(list of supported <d_comp>s), (list of supported <h_comp>s),(list ofsupported <IPv4Addr Alloc>s),(list of supported <emergency_indication>s), (list of supported <PCSCF_discovery>s),(list of supported <IM_CN_Signalling_Flag_Ind>s) [<CR><LF>+CGDCONT: (range of supported <cid>s),<PDP_type>,,,(list of supported <d_comp>s),(list of supported <h_comp>s),(list ofsupported <IPv4AddrAlloc>s),(list of supported <emergency_indication>s),(list of supported <PCSCF_discovery>s),(list of supported <IM_CN_Signalling_Flag_Ind>s) [...]] OK |
| Read command Syntax AT+CGDCONT? | Response [+CGDCONT: <cid>, <PDP_type>, <APN>,<PDP_addr>, <d_comp>, <h_comp> [,<IPv4AddrAlloc>[,<emergency_indication>[,<PCSCF_discovery> [,<IM_CN_Signalling_Flag_Ind>]]]]] [<CR><LF>+CGDCONT: <cid>, <PDP_type>, <APN>,<PDP_addr>, <d_comp>, <h_comp>[,<IPv4AddrAlloc>[,<emergency_indication>[,<PCSCF_discovery> [,<IM_CN_Signalling_Flag_Ind>]]]]] [...]] OK |
| Write command Syntax AT+CGDCONT= [<cid> [,<PDP_type> [,<APN> [,<PDP_addr> [,<d_comp> [,<h_comp> [,<IPv4AddrAlloc>[,<emergency_ indication> [,<PCSCF_ discovery> [,<IM_CN_ Signalling_Flag_ Ind>]]]]]]]]]] | Response OK or ERROR Parameters <cid> PDP Context Identifier. A numeric parameter which specifies a particular PDP context definition. The parameter is local to the TE-MT interface and is used in other PDP context-related commands. The range of the permitted values (minimum value = 1) is returned by the test command. <PDP_type> Packet Data Protocol type "IP" Internet Protocol "IPV6" Internet Protocol, version 6 "IPV4V6" Virtual <PDP_type>introduced to handle dual IP stack UE capability <APN> Access Point Name String parameter which is a logical name that is used to select the GGSN or the external packet data network. If the value is null or omitted, then the subscription value will be requested. <PDP_address> String parameter that identifies the MT in the address space applicable to the PDP. If the value is null or omitted then a value may be provided by the TE during the PDP startup procedure or, failing that, a dynamic address will be requested. The read command will continue to return the null string even if an address has been allocated during the PDP startup procedure. The allocated address may be read using the command +CGPADDR command. Note that IPv6 address obtained on LTE will be prefixed with a constant 8 byte address "FE.80.00.00.00.00.00.00" if the network has not provided any. <d_comp> PDP data compression (applicable for SNDCP only) 0 Off (default if value if omitted) 1 On (manufacturer preferred compression) 2 V.42 bis <h_comp> PDP header compression 0 Off (default if value if omitted) 1 On (manufacturer preferred compression) 2 RFC1144 (applicable for SNDCP only) 3 RFC2507 4 RFC3095 (applicable for PDCP only) <IPv4AddrAlloc> Numeric parameter that controls how MT/TA requests to get IPv4 address information 0 IPv4 address allocated through NAS signalling 1 IPv4 address allocated through DHCP <emergency_indication> Indicates whether the PDP contect is for emergency bearer services or not 0 PDP context is not for emergency bearer services 1 PDP context is for emergency bearer services <P-CSCF_discovery> Numeric parameter that influences how the MT/TA requests get the P-CSCF address 0 Preference of P-CSCF address discovery not influences by +CGDCONT 1 Preference of P-CSCF address discovery through NAS signalling <IM_CN_Signalling_Flag_Ind> Numeric parameter used to indicate whether the PDP context is for IM CN subsystem related signaling only or not 0 UE indicates that the PDP context is not for IM CN subsystem-related signaling only 1 UE indicates that the PDP context is for IM CN subsystem-related signaling only |
| Notes | - If the command is used only with the one parameter <cid>, it means that the corresponding PDP context becomes undefined. - The APN Control List (ACL) will only be checked if a USIM is inserted. Before performing context definition it will check if the ACL-service is enabled and activated. If yes, all APNs from ACL of EF-ACL of the USIM will be read out and compared with the requested APN. - If the requested APN is listed in the ACL, the context definition will be performed. - If the requested APN is empty ("") and ACL contains "network provided APN", the context definition will also be requested. - If the APN is not listed in the ACL the command returns error. - If the ACL-service is not enabled or not activated in the USIM or a GSM-SIM is inserted the context definition will be performed without any checks. - Parameters are saved in non-volatile memory over module reboot. |

## +CGDSCONT Command: Define Secondary PDP Context

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CGDSCONT=? | Response +CGDSCONT: (range of <cid>s),(list of <cid>s for defined primary contexts), <PDP_type>,,,(list of supported <d_comp>s),(list of supported <h_comp>s),(list of supported <IM_CN_Signalling_Flag_Ind>s) [<CR><LF>+CGDSCONT: (range of <cid>s),(list of <cid>s for defined primary contexts), <PDP_type>,,,(list of supported <d_comp>s),(list of supported <h_comp>s),(list of supported <IM_CN_Signalling_Flag_Ind>s) [...]] OK |
| Read command Syntax AT+CGDSCONT? | Response [+CGDSCONT: <cid>, <p_cid>, <d_comp>, <h_comp> [,<IM_CN_Signalling_Flag_Ind>]] [<CR><LF>+CGDSCONT: <cid>, <p_cid>, <d_comp>,<h_comp> [,<IM_CN_Signalling_Flag_Ind>]] [...]]] OK |
| Write command Syntax AT+CGDSCONT= [<cid>,<p_cid> [,<d_comp> [,<h_comp> [,<IM_CN_ Signalling_Flag_ Ind>]]]] | Response OK or ERROR Parameter <cid> PDP Context Identifier. A numeric parameter that specifies a particular PDP context definition. The parameter is local to the TE-MT interface and is used in other PDP context-related commands. The range of the permitted values (minimum value = 1) is returned by the test command. <p_cid> Primary PDP Context Identifier. Numeric parameter that specifies a particular PDP context definition which has been specified by +CGDCONT. The parameter is local to the TE-MT interface. The list of permitted values is returned by the test command. <d_comp> PDP data compression (applicable for SNDCP only) 0 Off (default value if omitted) 1 On (manufacturer preferred compression) 2 V.42 bis <h_comp> PDP header compression 0 Off (default value if omitted) 1 On (manufacturer preferred compression) 2 RFC1144 (applicable for SNDCP only) 3 RFC2507 4 RFC3095 (applicable for PDCP only) <IM_CN_Signalling_Flag_Ind> Numeric parameter used to indicate whether the PDP context is for IM CN subsystem related signaling only or not 0 UE indicates that the PDP context is not for IM CN subsystem-related signaling only 1 UE indicates that the PDP context is for IM CN subsystem-related signaling only |

## +CGEREP Command: Packet Domain Event Reporting

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CGEREP=? | Response +CGEREP: (list of supported <mode>s),(list of supported <bfr>s) OK |
| Read command Syntax AT+CGEREP? | Response +CGEREP: <mode>, <bfr> OK or ERROR |
| Write command Syntax AT+CGEREP= [<mode>[,<bfr>]] | Response OK or ERROR Parameters <mode> 0 Buffer unsolicited result codes in the MT; if MT result code buffer is full, the oldest ones can be discarded. No codes are forwarded to the TE. 1 Discard unsolicited result codes when MT-TE link is reserved (e.g. in on-line data mode); otherwise forward them directly to the TE (1 is the default value) 2 Buffer unsolicited result codes in the MT when MT-TE link is reserved (e.g. in on-line data mode) and flush them to the TE when MT-TE link becomes available; otherwise forward them directly to the TE <bfr> 0 MT buffer of unsolicited result codes defined within this command is cleared when <mode> 1 or 2 is entered 1 MT buffer of unsolicited result codes defined within this command is flushed to the TE when <mode> 1 or 2 is entered (OK response shall be given before flushing the codes) |
| Unsolicited Notification | Response +CGEV: NW DETACH The network has forces a PS detach +CGEV: NW CLASS <class> The network has forced a change of MT class +CGEV: ME CLASS <class> The mobile termination has forced a change of MT class +CGEV: ME PDN ACT <cid>[,<reason>] The mobile termination has activated a context +CGEV: NW ACT <p_cid>, <cid>, <event_type> The network has activated a context +CGEV: ME ACT <p_cid>, <cid>, <event_type> The network has responded to an ME initiated context activation +CGEV: NW PDN DEACT <cid> The network has deactivated a context +CGEV: ME PDN DEACT <cid> The mobile termination has deactivated a context +CGEV: NW DEACT <p_cid>, <cid>, <event_type> The network has deactivated a context +CGEV: ME DEACT <p_cid>, <cid>, <event_type> The network has responded to an ME initiated context deactivation request +CGEV: NW MODIFY <cid>, <change_reason>, <event_type> The network has modified a context +CGEV: ME MODIFY <cid>, <change_reason>, <event_type> The mobile termination has modified a context Parameters <reason> 0 IPv4 only allowed 1 IPv6 only allowed 2 Single address bearers only allowed 3 Single address bearers only allowed and MT initiated context activation for a second address type bearer was not successful <event_type> 0 Informational event 1 Information request, acknowledgement required <change_reason> 0 TFT only changed 1 QoS only changed 2 Both TFT and QoS changed |
| Notes | <mode> is saved in non-volatile memory over module reboot; URC is available on the port that executes the command. |

## +CGPADDR Command: Show PDP Address

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CGPADDR=? | Response +CGPADDR: (list of supported <cid>s) OK |
| Write command Syntax AT+CGPADDR= [<cid> ,<cid> [,...]]] | Response +CGPADDR: <cid>[,<PDP_addr_1>[,<PDP_addr_2>]] [<CR><LF> +CGPADDR: <cid>[,<PDP_addr_1>[,<PDP_addr_2>]]][...]] OK Parameters <cid> a numeric parameter which specifies a particular PDP context definition (see the +CGDCONT and +CGDSCONT commands). If no <cid> is specified, the addresses for all defined contexts are returned. <PDP_addr_1>, <PDP_addr_2> String that identifies the MT in the address space applicable to the PDP. The address may be static or dynamic. For a static address, it will be the one set by the +CGDCONT and +CGDSCONT commands when the context was defined. For a dynamic address it will be the one assigned during the last PDP context activation that used the context definition referred to by <cid>. Both <PDP_addr_1> and <PDP_addr_2> are omitted if none are available. Both <PDP_addr_1> and <PDP_addr_2> are included when both Ipv4 and Ipv6 addresses are assigned, with <PDP_addr_1> containing the IPv4 address and <PDP_addr_2> containing the IPv6 address. The string is given as dot-separated numeric (0 – 255) parameter of the form: a1.a2.a3.a4 for IPv4 and a1.a2.a3.a4.a5.a6.a7.a8.a9.a10.a11.a12.a13.a14.a15.a16 for IPv6. |

## +CGSMS Command: Select Service for MO SMS Messages

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+CGSMS=? | Response +CGSMS: (list of currently available <service>s) OK |
| Read command Syntax AT+CGSMS? | Response +CGSMS: <service> OK |
| Write command Syntax AT+CGSMS= [<service>] | Response OK or ERROR Parameter <service> Indicates the service or service preference to be used 0 Packet Domain 1 Circuit switched 2 Packet Domain preferred (use circuit switched if GPRS is not available) 3 Circuit switched preferred (use packet domain if circuit switched is not available) |
| Note | +CGSMS is ignored for sending SMS over IMS as used in the Verizon network. |

## +XDNS Command: Dynamic DNS Request

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+XDNS=? | Response +XDNS: (list of supported <cid>s),(list of supported <mode>s) OK |
| Read command Syntax AT+XDNS? | Response +XDNS: <cid>, <primary DNS>, <secondary DNS> [+XDNS: <cid>, <primary DNS>, <secondary DNS> [...]] OK |
| Write command Syntax AT+XDNS=<cid>, <mode> | Response OK or +CME ERROR: <err> Parameters <cid> Context ID <mode> 0 Disable dynamic DNS request 1 Enable dynamic DNS request (IPv4) 2 Enable dynamic DNS request (IPv6) 3 Enable dynamic DNS request (IPv4v6) <primary DNS>, <secondary DNS> Strings representing the DNS addresses and given as dot-separated numeric (0 – 255) parameters in the form of: a1.a2.a3.a4 for IPv4, a1.a2.a3.a4.a5.a6.a7.a8.a9.a10.a11.a12.a13.a14.a15.a16 for IPv6, and a1.a2.a3.a4.a5.a6.a7.a8.a9.a10.a11.a12.a13.a14.a15.a16.a17.a18.a19.a20 for IPv4v6 (a1 to a4 here represents IPV4 and a5 to a20 represents IPv6) The DNS address is by default "0.0.0.0" which is not a valid address. Note that IPv6 address obtained on LTE will be prefixed with a constant 8 byte address "FE.80.00.00.00.00.00.00" if the network has not provided any. |

# SIM Application Toolkit Commands

# Protocol Specific Commands

## Preliminary Comments

Sierra Wireless has developed a set of proprietary AT Commands to simplify data exchanges with different protocols:

TCP

UDP

FTP

HTTP

HTTPS

## IP Address Format in AT Commands

Unless specified elsewhere, the following format is used for IP address field in AT commands described in this chapter when using the HL78xx embedded module:

IPv4 address: Consists of dot-separated decimal (0 – 255) parameters of the form a1.a2.a3.a4

IPv6 address: Consists of colon-separated hexadecimal (0 – ffff) parameters of the form a1:a2:a3:a4:a5:a6:a7:a8 with abbreviations

## Session ID

Protocol specific AT commands share the same range of session IDs. A session ID <session_id> is a unique number and ranges from 1 to 32.

## Connection of PDP Contexts

A PDP connection will be started when a session becomes active (e.g. +KTCPCNX) and will only be stopped if all sessions are closed or all sessions request to stop the connection. In case of session errors, the PDP connection deactivation behavior can be configured by +KIPOPT with <option_id>=3. The default setting after the module boot-up is that a PDP connection is requested to stop only when a session is closed by an Internet AT command (e.g. +KTCPCLOSE).

## Buffer Length of AT Commands

In AT command mode, the maximum length of an AT command is 1023 characters; any AT command input longer than this limit will produce an error response. If the maximum length of a parameter is not specified in this manual, it may vary but still bound by this limit.

In AT data mode, the terminal receive buffer size is limited to 32000 bytes; the terminal driver will stop the receive flow at 16000 bytes if hardware handshaking is used.

## Parameter Format of AT Commands

Double quotation marks are optional in the parameter input of protocol specific AT commands.

If the AT command does not meet the following conditions, the AT parser will regard it as an error and will not go to the corresponding AT command handler. It will immediately return +CME ERROR: 3. This means that it will not process any action further or return any specific error code.

If double quotation marks are used to enclose parameters, double quotation marks must appear at both the head and tail of the parameter.

The total number of parameter input (including empty parameters) in the AT commands must be within the minimum and maximum required number of parameters.

## Connection Configuration

### +KCNXCFG Command: GPRS Connection Configuration

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KCNXCFG=? | Response +KCNXCFG: (list of possible <cnx conf>s),"GPRS",(range of possible length of <apn>),(range of possible length of <login>),(range of possible length of <password>), <af>,<ip>,<dns1>,<dns2>,<ipv6>,<dns1v6>,<dns2v6> OK |
| Read command Syntax AT+KCNXCFG? | Response +KCNXCFG: <cnx cnf>, "GPRS", <apn>,<login>,<password>,<af>,<ip>,<dns1>, <dns2>[,<ip_v6>,<dns1_v6>,<dns2_v6>],<state> [...]> OK |
| Write command Syntax AT+KCNXCFG= <cnx cnf>, "GPRS",<apn> [,[<login>] [,[<password>] [,<af> [,[<ip>] [,[<dns1>] [,<dns2>]]]] [,[<ip_v6>] [,[<dns1_v6>] [,<dns2_v6>]]]]]] | Response OK Parameters <cnx cnf> 1 – 5 (PDP context configuration) a numeric parameter which specifies a particular PDP context configuration <apn> (Access Point Name) a string parameter (max size 63 bytes), logical name used to select the GGSN or the external packet data network. <login> string type (max size 24 bytes), indicates the user name of the cnx <password> string type (max size 24 bytes), indicates the password of the cnx <af> Address family used for the connection IPV4 IPv4 only IPV6 IPv6 only IPV4V6 IPv4 and IPv6 <ip> String type. If the mobile is supposed to work with a dynamic address, the value should be “0.0.0.0” or an empty string. <dns1>, <dns2> String type. If the mobile is supposed to work with dynamic DNS addresses, the value should be “0.0.0.0” or an empty string. <ip_v6> IPV6 String type. If the mobile is supposed to work with a dynamic address, the value should be “::” or an empty string. <dns1_v6>, <dns2_v6> IPV6 String type. If the mobile is supposed to work with dynamic DNS addresses, the value should be “::” or an empty string. <state> Connection state 0 Disconnected 1 Connecting 2 Connected 3 Idle, down counting for disconnection 4 Disconnecting |
| Reference Sierra Wireless Proprietary | Notes - This AT command is used to configure the bearer to be used for the future IP services. - By default, the IP and DNS address are dynamic (those values would be affected by the network during the PDP connection). - This connection will be used by the module to access to the IP services described in subsequent chapters. The AT+KCNXCFG command is only defined to set the current parameters. The defined connection will be automatically opened when needed by the IP services (e.g. UDP service). - The use of IPV4 and/or IPV6 addresses is configured by PDP context configuration. - <cnx cfg> values 1 to 5 correspond to PDP context ID 1 to 5 respectively, e.g. <cnx cfg>=3 corresponds to CID=3 in +CGDCONT/+CGACT. - When the connection is up, the read command returns the actual values used by the connection interface. - If reuse of existing activated PDP context is required, <apn> can be set as an empty string or as the existing APN string returned by +CGDCONT read command. |

### +KCNXTIMER Command: Connection Timer Configuration

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KCNXTIMER =? | Response +KCNXTIMER: (list of supported <cnx cnf>s),(list of supported <tim1>s),(list of supported <nbtrial>s),(list of supported <tim2>s) ,(list of supported <idletime>s) OK |
| Read command Syntax AT+KCNXTIMER? | Response +KCNXTIMER: <cnx cnf>,<tim1>,<nbtrial>,<tim2>,<idletime> […] OK |
| Write command Syntax AT+KCNXTIMER=<cnx cnf>[, [<tim1>][, [<nbrtrial>] [,<tim2>] [,<idletime>]]]] | Response OK Parameters <cnx cnf> 1 – 5 (PDP context configuration) a numeric parameter which specifies a particular PDP context configuration <tim1> 1 – 120 s (30 s by default) If the module fails to activate the PDP context, a timer of <tim1> will be started. When this timer expires, it will try to activate the PDP context again. <nbtrial> Attempt times from1 – 4 (2 by default). The module will try to activate the PDP context for a maximum of <nbtrial> times. <tim2> 0 – 300s (60 s by default) 0 Deactivated (connection will not close by itself) For client sockets, module will try to connect to the server within <tim2>s; if <tim2> expires, it will give up the connection. <idletime> 0 – 1800 s (30 s by default) When all sessions are closed, the idle timer starts with the idle time. When this timer expires, it will try to deactivate the PDP context. Before the timer expires, connecting any session will stop this timer and the PDP context is reused. |
| Reference Sierra Wireless Proprietary | Notes This command will only have impact on TCP, UDP, FTP, HTTP and HTTP specific commands (+KTCPCNX, +KTCPSTART, +KUDPCFG, +KFTPCFG, etc.) |

### +KCNXPROFILE Command: Current Profile Connection Configuration

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+ KCNXPROFILE =? | Response +KCNXPROFILE: (list of possible <cnx cnf>s) OK |
| Read command Syntax AT+ KCNXPROFILE? | Response +KCNXPROFILE: <cnx cnf> OK |
| Write command Syntax AT+ KCNXPROFILE= <cnx cnf> | Response OK Parameters <cnx cnf> 1 – 5 PDP context configuration – a numeric parameter which specifies a particular PDP context configuration |
| Reference Sierra Wireless Proprietary | Notes This command sets the default PDP context configuration ID for +KTCPCFG, +KUDPCFG, +KFTPCFG, +KHTTPCFG and +KHTTPSCFG, if <cnx cnf> parameter is not given in these commands. |

### +KCGPADDR Command: Display PDP Address

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KCGPADDR =? | Response +KCGPADDR: (list of possible <cnx_cnf>s) OK |
| Write command Syntax For all <cnx_cnf>s: AT+KCGPADDR For specific <cnx_cnf>s: AT+KCGPADDR=<cnx_cnf> | Response +KCGPADDR: <cnx cnf>, <PDP_addr_1> [[+KCGPADDR: <cnx cnf>, <PDP_addr_2>] …] OK Parameters <cnx cnf> 1 – 5 PDP context configuration – a numeric parameter which specifies a particular PDP context configuration <PDP_addr> A string that identifies the MT in the address space applicable to the PDP |
| Reference Sierra Wireless Proprietary | Notes - This AT command can be used after +KTCPCNX, +KUDPCFG, etc. to display the local IP address of the module - For IPv6, more than one PDP addresses corresponding to the interface may be displayed. |

### +KCNX_IND Notification: Connection Status Notification

| HL7800 |  |
| --- | --- |
| Unsolicited Notification | Response +KCNX_IND: <cnx cnf>,<status>,<af> (for <status> = 0, 1) +KCNX_IND: <cnx cnf>,<status>,<attempt>,<nbtrial>,<tim1> (for <status> = 2) +KCNX_IND: <cnx cnf>,<status> (for <status> = 3,6) +KCNX_IND: <cnx cnf>,<status>,<attempt> (for <status> = 4) +KCNX_IND: <cnx cnf>,<status>,<idletime> (for <status> = 5) Parameters <cnx cnf> 1 – 5 (PDP context configuration) a numeric parameter which specifies a particular PDP context configuration <status> PDP connection status 0 Disconnected due to network 1 Connected 2 Failed to connect, <tim1> timer is started if <attempt> is less than <nbtrail> 3 Closed 4 Connecting 5 Idle time down counting started for disconnection 6 Idle time down counting canceled <af> 0 IPV4 1 IPV6 <tim1> Refer to +KCNXTIMER <attempt> Current attempt of bringing up of PDP connection <nbtrial> Refer to +KCNXTIMER <idletime> Refer to +KCNXTIMER |
| Reference Sierra Wireless Proprietary |  |

### +KCNXUP Command: Bring the PDP Connection Up

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KCNXUP=? | Response +KCNXUP: (list of possible <cnx_cnf>s) OK |
| Write command Syntax AT+KCNXUP= <cnx_cnf> | Response OK Parameter <cnx cnf> 1 – 5 PDP context configuration – a numeric parameter which specifies a particular PDP context configuration |
| Reference Sierra Wireless Proprietary | Notes - This command activates the PDP context and reserves the activated PDP connection (i.e. keeps the PDP connection up even after the last session is closed). - If this command is not used, the PDP context will be brought down after the last session is closed unless +KCNXDOWN is used. |

### +KCNXDOWN Command: Bring the PDP Connection Down

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KCNXDOWN=? | Response +KCNXDOWN: (list of possible <cnx_cnf>s),(list of possible <mode>s) OK |
| Write command Syntax AT+KCNXDOWN=<cnx_cnf> [,<mode>] | Response OK Parameters <cnx cnf> 1 – 5 PDP context configuration – a numeric parameter which specifies a particular PDP context configuration <mode> 0 Cancels the reservation of the activated PDP connection previously configured by +KCNXUP 1 Similar to 0, but deactivates the PDP connection even if the active session exists |
| Reference Sierra Wireless Proprietary |  |

## Common Configuration

### +KPATTERN Command: Custom End of Data Pattern

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KPATTERN =? | Response OK |
| Read command Syntax AT+KPATTERN? | Response +KPATTERN: <EOF pattern> OK |
| Write command Syntax AT+KPATTERN =<EOF pattern> | Response OK or +CME ERROR <err> Parameter <EOF pattern> String type (max size 128 bytes). This is a pattern used to notify the end of data (or file) during data or file transfer. This string doesn’t have to be human-readable (not printable characters are allowed). |
| Reference Sierra Wireless Proprietary | Notes - The default value of the pattern is: "--EOF--Pattern--". - It is the responsibility of the user to select an appropriate pattern according to the data transferred (i.e. numeric pattern for text files and Readable string for binary files). - The <EOF pattern> pattern is detected within 100ms or higher timeout and without following data. The timeout value is equal to <wait_time> of +KIPOPT. - The received data is stored with buffer size <send size v4> or <send size v6> so that the <EOF pattern> with size larger than it is not detected. The user application should ensure that the value of <send size v4> or <send size v6> is larger than the size of <EOF pattern>. |

### +KURCCFG Command: Enable or Disable the URC from Protocol Commands

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KURCCFG=? | Response +KURCCFG: (list of supported <protoopt>s),(list of supported <noti_act>s),(list of supported <indi_act>s) OK |
| Read command Syntax AT+KURCCFG? | Response +KURCCFG: list of supported (<protoopt>,<noti_act>,<indi_act>) OK |
| Write command Syntax AT+KURCCFG= <protoopt>, <noti_act> [,<indi_act>] | Response OK Parameters <protoopt> Protocol option to enable/disable URC "TCPC" TCP client session "TCPS" TCP server session "UDPC" UDP client session "UDPS" UDP server session "FTP" FTP client session "HTTP" HTTP client session "HTTPS" HTTPS client session "TCP" Both TCP client and TCP server sessions "UDP" Both UDP client and UDP server sessions <noti_act> 1 Enable URC (like +KTCP_NOTIF, +KFTP_ERROR, etc.) 0 Disable URC <indi_act> 1 Enable URC (like +KTCP_SRVREQ, +KTCP_IND, +KTCP_DATA, +KUDP_DATA, +KUDP_RCV, +KFTP_IND, etc.) 0 Disable URC |
| Examples | To disable URC: AT+KURCCFG=“TCP”,0 OK Test and read command: AT+KURCCFG=? +KURCCFG: ("TCPC","TCPS","UDPC","UDPS","FTP","HTTP","HTTPS","TCP","UDP"), (0,-1),(0-1) OK AT+KURCCFG? +KURCCFG: "TCPC",1,1 +KURCCFG: "TCPS",1,1 +KURCCFG: "UDPC",1,1 +KURCCFG: "UDPS",1,1 +KURCCFG: "FTP",1,1 +KURCCFG: "HTTP",1,1 +KURCCFG: "HTTPS",1,1 OK |
| Reference Sierra Wireless Proprietary | Notes - Enabling or disabling +KTCP_NOTIF unsolicited messages is only useful when in polling mode with +KTCPSTAT. - If set to "disable", URCs are discarded and not stored. - Can be used in 07.10 multiplexer. |

### +KIPOPT Command: General Options Configuration

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KIPOPT=? | Response +KIPOPT: 0,<UDP>,(1-100),(8-1472),(8-1452) +KIPOPT: 0,<TCP-based>,(0-100),(0,8-1460),(0,8-1440) +KIPOPT: 1,(0-1) +KIPOPT: 2,(0-255) +KIPOPT: 3,(0-1),(0-1) +KIPOPT: 4,(0-2) OK |
| Read command Syntax AT+KIPOPT? | Response +KIPOPT: 0,<proto>,<wait time>,<send size v4>,<send size v6>] [...] +KIPOPT: 1,<http_chunked> +KIPOPT: 2,<http_max_redirect> +KIPOPT: 3,<stop_on_error>, <stop_on_peer> +KIPOPT: 4,<ssl_ver> OK |
| Write command Syntax If <option_id>=0 AT+KIPOPT= <option_id>, <proto>,<wait time> [,<send size v4> [,<send size v6>]] If <option_id>=1 AT+KIPOPT= <option_id>, <http_chunked> If <option_id>=2 AT+KIPOPT= <option_id>, <http_max_ redirect> If <option_id>=3 AT+KIPOPT= <option_id>, <stop_on_error>, <stop_on_peer> If<option_id>=4 AT+KIPOPT= <option_id>, <ssl_ver> | Response OK or +CME ERROR<err> Parameters <option_id> Option ID 0 Wait time, send size threshold configuration 1 HTTP chunked transfer encoding 2 HTTP maximum redirection 3 PDP connection deactivated behavior 4 SSL version for use in +KHTTPS <proto> Protocol, string type “TCPC” TCP client session “TCPS” TCP server session “UDPC” UDP client session “UDPS” UDP server session “FTP” FTP client session “HTTP” HTTP client session “HTTPS” HTTP server session “TCP” Both client and server TCP sessions “UDP” Both client and server UDP sessions <wait time> Timeout for configuring the packet segmentation on the IP network side; it specifies the timeout after which the buffered data will be sent to the peer irrespective of data packet size. Value is in 100 ms units. Range: For UDP: 1 – 100, default value = 2 For TCP: 0 – 100, default value = 1. Note that value = 0 has the same effect as having value = 1 due to the limitation from +KPATTERN detection timing <send size v4> Data packet size for IPv4 sessions. This parameter specifies the data packet size that needs to be sent to the peer. Range: For UDP: 8 – 1472, default value = 1020 For TCP: 0, 8 – 1460, default value = 0 (disabled) <send size v6> Data packet size for IPv6 sessions. This parameter specifies the data packet size that needs to be sent to the peer. Range: For UDP: 8 – 1452, default value = 1020 For TCP: 0, 8 – 1440, default value = 0 (disabled). Note that value = 0 uses a wait time of 100 ms. <http_chunked> “Chunked” transfer encoding for HTTP POST 0 Data sent with HTTP POST are not encoded 1 Data sent with HTTP POST are automatically encoded using “chenked” transfer encoding <http_max_redirect> Maximum redirection allowed for HTTP GET. Range: 8 – 255; default value = 0 <stop_on_error> PDP connection deactivation behavior when a session is closed due to any error 0 Do not request to stop the connection 1 Request to stop the connection <stop_on_peer> PDP connection deactivation behavior when a session is closed by a peer/server 0 Do not request to stop the connection 1 Request to stop the connection <ssl_ver> SSL version for use in +KHTTPS 0 TLS version 1.1 1 TLS version 1.0 2 TLS version 1.2 |
| Reference Sierra Wireless Proprietary | Notes - "chunked" transfer encoding for HTTP POST is applicable and effective only for HTTP version 1.1. - The default setting of <option_id>=3 is (<stop_on_error>=0, <stop_on_peer>=0) after module boot-up; this means that a PDP connection is requested to stop only when a session is closed by an Internet AT command (e.g. +KTCPCLOSE). - Thresholds <send size v4> and <send size v6> control the maximum size of data received from the AT terminal to be buffered within timeout <wait time>. When the threshold is reached or after timeout, the buffered data are sent to the socket layer for transmission. For UDP: data are sent as a UDP packet For TCP based protocol: data are copied to socket first-in-first-out buffer for transmission but packet segmentation is not guaranteed to be <send size> - For TCP based protocol, when <send size v4> and <send size v6> are disabled (=0), threshold = 4000 is used internally. - The maximum transmission unit (MTU) is 1500 bytes. - After starting a connection or running SSL Certificate write commands, <ssl_ver> is fixed and cannot be changed until module reboot. - <send size v4> and <send size v6> impacts the detection of <EOF pattern>. Refer to the notes of +KAPTTERN for more information. |

## SSL Configuration

### +KSSLCRYPTO Command: Cipher Suite Configuration

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+ KSSLCRYPTO=? | Response +KSSLCRYPTO: <profile_id>,<mkey_Algo>,<auth_algo>,<enc_algo>,<mac_algo>, <tls_ver>,<auth>,<tls_ver>,<auth> OK |
| Read command Syntax AT+ KSSLCRYPTO? | Response + KSSLCRYPTO: <profile_id>,<mkey_algo>,<auth_algo>,<enc_algo>,<mac_algo>, <tls_ver>,<auth> […]] |
| Write command Syntax AT+ KSSLCRYPTO= <profile_id>, <mkey_Algo>, <auth_algo>, <enc_algo>, <mac_algo>, <tls_ver>,<auth> | Response OK Parameters <profile_id> Index of a set of parameters for configuring one SSL profile <mkey_algo> Key exchange algorithm selection 1 RSA key exchange <auth_algo> Authentication algorithm selection 1 RSA authentication <enc_algo> Encryption algorithm selection 4 RC4 64 AES 128 128 AES 256 8192 AES128GCM <mac_algo> Message authentication code algorithm selection 1 MD5 2 SHA1 64 AEAD <tls_ver> Cipher suite version selection. 1 TLS 1.0 4 TLS 1.2 <auth> Authentication 0 No authentication 1 Authenticate server 2 Provide client certificate to server 3 Authenticate server and provide client certificate to server |
| Reference Sierra Wireless Proprietary |  |

### +KSSLCFG Command: SSL Configuration

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KSSLCFG=? | Response +KSSLCFG:<option id>,<option> OK |
| Read command Syntax AT+KSSLCFG? | Response +KSSLCFG:0,<TLS Version> +KSSLCFG:2,<Session Mode> OK |
| Write command Syntax AT+KSSLCFG =<option id>, <option> | Response If <option_id> = 0: AT+KSSLCFG=<option_id>,<TLS Version> OK If <option_id> = 1: AT+KSSLCFG=<option_id>,<Random Seed> OK If <option_id> = 2: AT+KSSLCFG=<option_id>,<Session Mode> OK Parameters <option id> 0 Specify a TLS version to be used for hand shake 1 Setup random seed 2 Specify session mode <TLS Version> 0 Highest possible 1 TLS 1.0 3 TLS 1.2 <Random Seed> String to be added into the entropy of the random number generator <Session Mode> 0 Automatic 1 Always start a new session (not supported) |

## TCP Specific Commands

### +KTCPCFG Command: TCP Connection Configuration

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KTCPCFG=? | Response +KTCPCFG: (list of possible <cnx_cnf>s),(list of possible <mode>s), <remote-name/ip>,(list of possible <tcp_port >s),(list of possible <source_port>s),(list of possible <data_mode>s),(list of possible <URC-ENDTCP-enable>s),(list of possible <af>s),<cipher_index> OK |
| Read command Syntax AT+KTCPCFG? | Response +KTCPCFG: <session_id>,<status>,<cnx cnf>,<mode>[,<serverID>], <tcp remote address>,<tcp_port>[,<source_port>],<data_mode>, <URC-ENDTCP-enable>,<af>,<cipher_index> […]] |
| Write command Syntax AT+KTCPCFG= [<cnx cnf>], <mode>, [<tcp remote address>], <tcp_port>[[, [<source_port>] [,[<data_mode>], [<URC-ENDTCP-enable>]]],<af>] [,<cipher_suite>] | Response +KTCPCFG: <session_id> OK Parameters <cnx cnf> Index of a set of parameters for configuring one TCP session (see +KCNXCFG) <session_id> TCP session index <mode> 0 Client 1 Server 2 Child (generated by server sockets) 3 Secure client <tcp remote address> IP address string or explicit name of the remote server. For server configuration, this parameter is left blank <tcp_port> TCP port number; numeric parameter with range 1 – 65535. This parameter is the listening port for a server configuration. <status> Connection state of the selected socket 0 Disconnected 1 Connected <serverID> Server session ID index. Only for sockets in CHILD mode <source_port> Numeric parameter (0-65535). Specifies the local TCP port number. This parameter is left blank for a server configuration. <data_mode> 0 Do not display <data> in URC (default setting) 1 Display <data> in URC <URC-ENDTCP-enable> 0 Do not display URC "+KTCP_ACK" (default setting) 1 Display URC "+KTCP_ACK" <af> Address family used for the connection. 0 IPV4 1 IPV6 <cipher_index> Cipher suite profile index to use for a secured socket; defined by +KSSLCRYPTO |
| Reference Sierra Wireless Proprietary | Notes - If the socket is defined as a <CLIENT> socket, <tcp_port> and <tcp remote address> define the port and the IP address of the remote server we want to connect. - Maximum <session_id> is 32. - For child session, the property <data_mode> will be kept the same as the server socket’s setting. - See section 18.5.6 Use Cases for AT+KTCPACKINFO and <URC-ENDTCP-enable> Option. - This command can be used before setting up +KCNXCFG configuration. Note however that the latter is required to start the connection properly. - The connection timeout for TCP socket is about 9 seconds with 3 retransmissions with 3 seconds delay. |

### +KTCPCNX Command: Start TCP Connection

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KTCPCNX=? | Response +KTCPCNX: (list of possible <session_id>s) OK |
| Write command Syntax AT+KTCPCNX= <session_id> | Response OK or +CME ERROR: <err> +KTCP_NOTIF: <session_id>, <tcp_notif> Parameters <session_id> TCP session index <tcp_notif> Integer type. Indicates the cause of the TCP connection failure 0 Network error 1 No more sockets available; max. number already reached 2 Memory problem 3 DNS error 4 TCP disconnection by the server or remote client 5 TCP connection error 6 Generic error 7 Fail to accept client request’s 8 Data sending is OK but KTCPSND was waiting more or less characters 9 Bad session ID 10 Session is already running 11 All sessions are used 12 Socket connection timeout error 13 SSL connection error 14 SSL initialization error |
| Reference Sierra Wireless Proprietary | Notes This command is used for connecting to a remote server or listening to a bound port, depending on the selected mode of <session_id.> |

### +KTCPRCV Command: Receive Data through a TCP Connection

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KTCPRCV=? | Response +KTCPRCV: (list of possible <session_id>s),(list of possible <ndata>s) OK |
| Write command Syntax AT+KTCPRCV= <session_id>, <ndata> | Response CONNECT …<EOF pattern> OK or +KTCP_NOTIF: <session_id>,<tcp_notif> Parameters <session_id> TCP session index <ndata> Number of bytes the device wants to receive (max value 4294967295) <tcp_notif> See command AT+KTCPCNX |
| Reference Sierra Wireless Proprietary | Notes - This function is used to receive <ndata> data bytes through a previously opened TCP socket. - <ndata> indicates the max data number that the terminal wishes to receive. If the TCP socket contains more data than <ndata> bytes then only <ndata> bytes will be received. If the TCP socket contains less data than <ndata> bytes then only TCP socket’s data will be received. - <EOF pattern> would be added at the end of data automatically. - When <ndata> (max value) bytes or only available data in the TCP socket have been received, the module returns to command state and returns OK. - It is highly recommended to configure the module for hardware flow control using AT&K3 before using this command. - Refer to AT&D for the behavior of DTR drop. |

### +KTCPSND Command: Send Data through a TCP Connection

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KTCPSND=? | Response +KTCPSND: (list of possible <session_id>s),(list of possible <ndata>s) OK |
| Write command Syntax AT+KTCPSND= <session_id>, <ndata> | Response CONNECT OK or NO CARRIER +CME ERROR: <err> +KTCP_NOTIF: <session_id>,<tcp_notif> Parameters <session_id> TCP session index <ndata> Number of bytes (max value 4294967295) <tcp_notif> See command AT+KTCPCNX |
| Reference Sierra Wireless Proprietary | Notes - All the data will be sent out ignoring <ndata>. If data sent is not equal to <ndata> then +KTCP_NOTIF will be displayed. - <ndata> is the data size without <EOF pattern>. - It is highly recommended to configure the module for hardware flow control using AT&K3 before using this command. - Refer to AT&D for the behavior of DTR drop. - The data session is automatically ended when <ndata> data bytes are sent/received, and the module returns to command state and returns OK. - The data session can also be ended by <EOF pattern>, +++ or DTR. Refer to section 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table for the behavior of +++, DTR or ATO. |

### +KTCPCLOSE Command: Close Current TCP Operation

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KTCPCLOSE=? | Response +KTCPCLOSE: (list of possible <session_id>s), (list of possible <closing_type>s) OK |
| Write command Syntax AT+KTCPCLOSE=<session_id> [,<closing_type>] | Response OK or +CME ERROR: <err> NO CARRIER +KTCP_NOTIF: <session_id>, <tcp_notif> Parameters <session_id> TCP session index <closing_type> 0 Abort. Fast closing of the TCP connection (not supported). 1 The TCP connection is properly closed, which means that data sent to the module by AT+KTCPSND will be sent to the TCP server and acknowledged before the socket is closed. <tcp_notif>: See command AT+KTCPCNX |
| Reference Sierra Wireless Proprietary | Notes - This function first closes the TCP socket and if there is no other session running then the PDP context is released. - AT+KTCPDEL=<session_id> can be used to delete the socket configuration after it’s been closed. |

### +KTCPDEL Command: Delete a Configured TCP Session

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KTCPDEL=? | Response +KTCPDEL: (list of possible <session_id>s) OK |
| Write command Syntax AT+KTCPDEL= <session_id> | Response OK or +CME ERROR: <err> Parameter <session_id> TCP session index |
| Reference Sierra Wireless Proprietary | Notes The session must be closed (using +KTCPCLOSE) before using this command. |

### +KTCP_SRVREQ Notification: Incoming Client Connection Request

| HL7800 |  |
| --- | --- |
| Unsolicited Notification | Response +KTCP_SRVREQ: <session_id>,<subsession_id>,<client_ip>,<client_port> Parameters <session_id> TCP session index <subsession_id> Newly created TCP session index <client_ip> IP address string of the incoming socket <client_port> Numeric parameter (0-65535); port of the incoming client |
| Examples | Configure the module to TCP servers AT+KCNXCFG=0,"GPRS","szsjmc.gd"; +KTCPCFG=0,1,,179 +KTCPCFG: 1 OK AT+KCNXCFG=0,"GPRS","szsjmc.gd"; +KTCPCFG=0,1,,180 +KTCPCFG: 2 OK Start the TCP servers AT+KTCPCNX=1 //listen on port 179 OK AT+KTCPCNX=2 //listen on port 180 OK Show the TCP servers’ IP address AT+KCGPADDR +KCGPADDR: 0,"192.168.1.49" OK Incoming connection request from remote client, shows ip address and port of remote client +KTCP_SRVREQ: 1,3,"192.168.0.32",4614 //incoming a connection request from “192.168.0.32” via //listening port 179, the remote port //is 4614 +KTCP_SRVREQ: 2,4,"10.10.10.110",4665 //incoming a connection request from “10.10.10.110” via //listening port 180, the remote port //is 4665 +KTCP_SRVREQ: 2,5,"10.10.10.110",4668 //incoming a connection request from the same ip via the same //listening port, the remote //port is 4668 +KTCP_SRVREQ: 1,6,"192.168.1.117",1739 //incoming a connection request from “192.168.1.117” via //listening port 179, the remote //port is 1739 +KTCP_NOTIF: 4,4 //the connection of sub session id 4 (on listening port 180) is //closed. +KTCP_SRVREQ: 2,4,"10.10.10.8",4672 //incoming a connection request from “10.10.10.8” via listening //port 180, the remote port is //4672 |
| Reference Sierra Wireless Proprietary | Notes - This notification is sent when a client requests a connection to the server. The connection is automatically accepted. - The created session is driven as any other TCP session with its own session ID. Use +KTCPSND, +KTCPRCV, +KTCPCLOSE, etc. to provide the service associated to this TCP server. - The TCP server corresponding to the session ID is still able to receive connection requests from other clients. These requests are notified with +KTCP_SRVREQ. - The client IP address and port can also be checked using AT+KTCPCFG? after the client is connected to the TCP server. |

### +KTCP_DATA Notification: Incoming Data through a TCP Connection

| HL7800 |  |
| --- | --- |
| Unsolicited Notification | Response +KTCP_DATA: <session_id>,<ndata available>[,<data>] Parameters <session_id> TCP session index <ndata available> For <data_mode> = 0, maximum number of bytes to be read in the TCP receive buffer; for <data_mode> = 1, maximum number of bytes to be read in <data> <data> Data in octet. The length of data is specified by <ndata_available> |
| Reference Sierra Wireless Proprietary | Notes - As soon as the connection is established, the module can receive data through the TCP socket. This notification is sent when data are available in the receive buffer. - This notification is sent for each TCP packet received. - When <data_mode> is set to 1, <ndata_available> will range from 1 to 1500 in the URC. If the user application sends over 1500 bytes of data to the module, the module will display those data with several URCs. - See section 18.6.3 Use Cases for KTCP_DATA and KUDP_DATA. |

### +KTCP_IND Notification: TCP Status

| HL7800 |  |
| --- | --- |
| Unsolicited Notification | Response +KTCP_IND: <session_id>,<status> Parameters <session_id> TCP session index <status> TCP session status. 1 session is set up and ready for operation |
| Reference Sierra Wireless Proprietary |  |

### +KTCPSTAT Command: Get TCP Socket Status

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KTCPSTAT=? | Response OK |
| Read command Syntax AT+KTCPSTAT? | Response OK |
| Write command Syntax For all TCP session IDs: AT+KTCPSTAT or AT+KTCPSTAT= <session_id> | Response +KTCPSTAT: <session_id>,<status>,<tcp_notif>,<rem_data>,<rcv_data> […] OK or +KTCPSTAT: <status>,<tcp_notif>,<rem_data>,<rcv_data> OK Parameters <session_id> TCP session index <status> TCP socket state 0 Socket not defined, use +KTCPCFG to create a TCP socket 1 Socket is only defined but not used 2 Socket is opening and connecting to the server, cannot be used 3 Connection is up, socket can be used to send/receive data 4 Connection is closing, it cannot be used, wait for status 5 5 Socket is closed <tcp_notif> -1 if socket/connection is OK, <tcp_notif> if an error has happened <rem_data> Remaining bytes in the socket buffer, waiting to be sent <rcv_data> Received bytes, can be read with +KTCPRCV command |
| Reference Sierra Wireless Proprietary | Notes - The socket buffer’s size for sending is 17520 bytes. - This command returns +CME ERROR: 910 (Bad Session ID) for undefined <session_id>s. |

### +KTCPSTART Command: Start a TCP Connection in Direct Data Flow

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KTCPSTART=? | Response OK |
| Read command Syntax AT+KTCPSTART? | Response OK |
| Write command Syntax AT+KTCPSTART=<session_id> | Response CONNECT OK or +CME ERROR: an error occurs, syntax error +KTCP_NOTIF: <session_id>,<tcp_notif> : an error occurs Parameters <session_id> TCP session index <tcp_notif> See command AT+KTCPCNX |
| Reference Sierra Wireless Proprietary | Notes - This function is used to send and receive data bytes through a TCP socket. - It is highly recommended to configure the module for hardware flow control using AT&K3 before using this command. - Refer to AT&D for the behavior of DTR drop. - Only one KTCPSTART session can be used. - Can be used in 07.10 multiplexer. - If the session is successfully connected by +KTCPCNX, this command does not restart the connection and the module directly enters direct data flow. - The data session can also be ended by <EOF pattern>, +++ or DTR. Refer to section 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table for the behavior of +++, DTR or ATO. |

### +KTCP_ACK Notification: Status Report for Latest TCP Data

| HL7800 |  |
| --- | --- |
| Unsolicited Notification | Response +KTCP_ACK: <session_id>,<result> <CR><LF> Parameters <session_id> TCP session index <result> 0 Data sent failure: not all data has been received by remote side 1 Data sent success: all the data has already been received by the remote side |
| Reference Sierra Wireless Proprietary | Notes - This URC is enabled or disabled by parameter <URC-ENDTCP-enable> of command +KTCPCFG. The URC is disabled by default. - See section 18.5.6 Use Cases for AT+KTCPACKINFO and <URC-ENDTCP-enable> Option. |

### +KTCPACKINFO Command: Poll ACK Status for the Latest Data

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+ KTCPACKINFO =? | Response OK |
| Read command Syntax AT+ KTCPACKINFO? | Response OK |
| Write command Syntax For all TCP session IDs with <URC-ENDTCP-enable>=1: AT+ KTCPACKINFO or AT+ KTCPACKINFO= <session_id> | Response +KTCPACKINFO: <session_id>,<result> […] OK or +KTCPACKINFO: <session_id>,<result> OK or +CME ERROR: <err> Parameters <session_id> TCP session index <result> 0 Data sent failure: not all data has been received by remote side. 1 Data sent success: all the data has already been received by the remote side; or no data transfer has happened yet 2 The status is unknown yet |
| Reference Sierra Wireless Proprietary | Notes - The command will return ERROR if <URC-ENDTCP-enable> of command +KTCPCFG is 0. - After the TCP session is connected and before any data transfer, +KTCPACKINFO returns 1. |

## UDP Specific Commands

### +KUDPCFG Command: UDP Connection Configuration

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KUDPCFG=? | Response +KUDPCFG: (list of possible <cnx cnf>s),(list of possible <mode>s),(list of possible <port>s),(list of possible <data_mode>s),<remote-name/ip>,(list of possible <udp_port>s),(list of possible <af>s) OK |
| Read command Syntax AT+KUDPCFG? | Response +KUDPCFG: <session_id>,<cnx cnf>,<mode>,<port>,<data_mode>,<udp remote address>,<udp_port>,<af> […] OK |
| Write command Syntax AT+KUDPCFG= [<cnx cnf>], <mode>[,[<port>] [,<data_mode>], [<udp remote address>] , <udp_port>,<af>] | Response +KUDPCFG: <session_id> OK or +CME ERROR: <err> +KUDP_NOTIF: <session_id>, <udp_notif> Parameters <session_id> UDP session index <mode> 0 Client 1 Server <port> 0 – 65535 Port (0 = random) <cnx cnf> 1 – 5 (PDP context configuration) a numeric parameter which specifies a particular PDP context configuration (see section 12.7.1 +KCNXCFG Command: GPRS Connection Configuration). <udp_notif> Integer type. Indicates the cause of the UDP connection failure. 0 Network error 1 No more sockets available; max number already reached 2 Memory problem 3 DNS error 5 UDP connection error(Host unreachable) 6 Generic error 8 Data sending is OK but KUDPSND was waiting more or less characters 9 Bad session ID 10 Session is already running 11 All sessions are used <data_mode> 0 Do not display <data> in URC (Default setting) 1 Display <data> in URC <udp remote address> IP address string or explicit name of the remote host, Default is empty (given by +KUDPSND). <udp_port> 0 – 65535 UDP peer port; given by +KUDPSND <af> Address family used for the connection. 0 IPV4 1 IPV6 |
| Reference Sierra Wireless Proprietary | Notes - For UDP socket in server mode, it is bound to a defined port number, incoming connection are notified by +KUDP_DATA. If remote address and port are given, they are saved for use in +KUDPSND. - Maximum <session_id> is 32. - +KCNXCFG configuration should be set up in order to start the connection properly. |

### +KUDPRCV Command: Receive Data through a UDP Connection

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KUDPRCV=? | Response +KUDPRCV: (list of possible <session_id>s),(list of possible <ndata>s) OK |
| Write command Syntax AT+KUDPRCV= <session_id>, <ndata> | Response CONNECT …<EOF pattern> OK +KUDP_RCV: <udp remote address>,<udp remote port>,<ndata available> or NO CARRIER +CME ERROR: <err> +KUDP_NOTIF: <session_id>, <udp_notif> +KUDP_DATA_MISSED: <session_id>, <ndata missed> Parameters <session_id> UDP session index <ndata> Number of bytes the device wants to receive (max value 4294967295) <udp remote address> IP address string of the remote host <udp remote port> 0 – 65535 Remote UDP port <ndata available> Number of bytes to be read in first received packet <udp_notif> See command AT+KUDPCFG <ndata missed> Number of bytes left in the UDP socket |
| Reference Sierra Wireless Proprietary | Notes - This function is used to receive <ndata> data bytes through a previously opened UDP socket. - <ndata> indicates the max data number that the terminal wishes to receive. If the UDP socket contains more data than <ndata> bytes, then only <ndata> bytes will be received and more data can be read by running this command again. - <EOF pattern> would be added at the end of data automatically. - When <ndata> (max value) bytes or only available data in the UDP socket have been received, the module returns to command mode. - It is highly recommended to configure the module for hardware flow control using AT&K3 before using this command. - Refer to AT&D for the behavior of DTR drop. |

### +KUDPSND Command: Send Data through a UDP Connection

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KUDPSND=? | Response +KUDPSND: (list of possible <session_id>s),<remote-name/ip>,(list of possible <udp_port>s),(list of possible <ndata>s) OK |
| Write command Syntax AT+KUDPSND= <session_id>, <udp remote address>, <udp_port>, <ndata> | Response CONNECT OK or NO CARRIER +CME ERROR: <err> +KUDP_NOTIF: <session_id>,<udp_notif> Parameters <session_id> UDP session index <udp remote address> IP address string or explicit name of the remote host <udp_port> 1 – 65535 UDP peer port <ndata> Number of bytes (max value 4294967295) <udp_notif> See command AT+KUDPCFG |
| Reference Sierra Wireless Proprietary | Notes - All data will be sent out ignoring <ndata>. If data sent is not equal to <ndata> then +KUDP_NOTIF will be displayed. - <ndata> is the data size without <EOF pattern>. - It is highly recommended to configure the module for hardware flow control using AT&K3 before using this command. - Refer to AT&D for the behavior of DTR drop. - The maximum transmission unit (MTU) is 1500 Bytes. - The <udp remote address> and <udp_port> are saved internally such that they can be omitted in subsequent calls of +KUDPSND. - The packet segmentation is controlled by +KIPOPT with <option_id>=0, and the maximum UDP packet size is limited by <send size v4> (1472 bytes) or <send size v6> (1452 bytes). Default value for both parameters is 1020 bytes. - The data session is automatically ended when <ndata> data bytes are sent/received, and the module returns to command state and returns OK. - The data session can also be ended by <EOF pattern>, +++ or DTR. Refer to section 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table for the behavior of +++, DTR or ATO. |

### +KUDPCLOSE Command: Close Current UDP Operation

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+KUDPCLOSE=? | Response +KUDPCLOSE: (list of possible <session_id>s),(list of possible <keep_cfg>s) OK |
| Write command Syntax AT+KUDPCLOSE=<session_id> [,<keep_cfg>] | Response OK or +KUDP_NOTIF: <session_id>, <udp_notif> Parameters <session_id> UDP session index <udp_notif> See command AT+KUDPCFG <keep_cfg> Specifies whether to delete the session configuration after closing it or not 0 Delete the session configuration 1 Keep the session configuration |
| Reference Sierra Wireless Proprietary | Notes - This function closes the UDP session. If there is no other session running, the PDP context will be released. - This function will delete the session configuration if <keep_cfg> = 0. |

### +KUDP_IND Notification: UDP Status

| HL7800 |  |
| --- | --- |
| Unsolicited Notification | Response +KUDP_IND: <session_id>,<status> Parameters <session_id> UDP session index <status> UDP session status. 1 Session is set up and ready for operation |
| Reference Sierra Wireless Proprietary |  |

### +KUDP_DATA Notification: Incoming Data through a UDP Connection

| HL7800 |  |
| --- | --- |
| Unsolicited Notification | Response +KUDP_DATA: <session_id>,<ndata available>[,<udp remote address>,<udp remote port>,<data>] Parameters <session_id> UDP session index <ndata available> Number of bytes to be read <udp remote address> IP address string of the remote host <udp remote port> 0 – 65535 Remote UDP port <data> Data in octet. The length of data is specified by <ndata_available>. |
| Reference Sierra Wireless Proprietary | Notes - As soon as the UDP socket is created, the module can receive data through this socket. This notification is sent when data are available in the receive buffer. - This notification will be sent one time. When <data_mode> was set to 0 (do not display data in URC), the controlling software must read the buffer with +KUDPRCV in order to activate the notification again. - When <data_mode> was set to 1, <ndata_available> will range from 1 – 1500 in the URC. If the user application sends over 1500 bytes of data to the module, the module will display those data with several URCs. It is possible for other applications (e.g. from Windows) to send more than 1472 bytes UDP packets to the module but the packet will be segmented and reassembled by the network stack. - When <data_mode> is set to 1, URC +KUDP_RCV will not be displayed after +KUDP_DATA. - When <data_mode> was set to 1, the fields <udp remote address> and <udp remote port> will be displayed in URC +KUDP_DATA. When <data_mode> was set to 0, they will be displayed in URC +KUDP_RCV. - See section 18.6.3 Use Cases for KTCP_DATA and KUDP_DATA. |

## SSL Certificate Manager

### +KCERTSTORE Command: Store Root CA and Local Certificates to Internal Storage

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+ KCERTSTORE=? | Response +KCERTSTORE: (list of possible <data_type>s),(range of possible lengths of <NbData>), (list of possible <index>es) OK |
| Read command Syntax AT+ KCERTSTORE? | Response +KCERTSTORE [root_cert,<index>,<NbData><CR><LF> <File_data><CR><LF>] [local_cert,<index>,<NbData><CR><LF> <File_data> <CR><LF>] [...] OK or +CME ERROR: <err> |
| Write command Syntax AT+ KCERTSTORE= <data_type> [,<NbData> [,<index>]] | Response CONNECT OK or +CME ERROR: <err> Parameters <data_type> 0 Root certificate 1 Local certificate <NbData> Number of bytes to read/write. Value range: 1-3000. <index> Stored root/local certificate index. If a root/local certificate is already stored at the index, it will be overloaded. 0 by default. Value range: 0 If <data_type> = 0 0 – 2 If <data_type> = 1 <File_data> File data in bytes |
| Reference Sierra Wireless Proprietary | Notes - The <index> parameter is the link between a local certificate and a private key (refer to +KPRIVKSTORE and +KCERTDELETE for more information). - The data session is automatically ended when <ndata> data bytes are sent/received, and the module returns to command state and returns OK. - The data session can also be ended by <EOF pattern>, +++ or DTR. Refer to section 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table for the behavior of +++ or DTR. - ATO is not available for this command. |

### +KPRIVKSTORE Command: Store Private Key Associated to a Local Certificate

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+ KPRIVKSTORE =? | Response +KPRIVKSTORE: (list of possible <index>s),( range of possible lengths of <NbData>) OK |
| Read command Syntax AT+ KPRIVKSTORE? | Response +KPRIVKSTORE private_key,<index>,<NbData><CR><LF> <File_data> <CR><LF> OK or +CME ERROR: <err> |
| Write command Syntax AT+ KPRIVKSTORE= <index> [,<NbData>] | Response CONNECT OK or +CME ERROR: <err> Parameters <index> Index of the stored local certificate associated to this private key. Value range: 0 – 2 <NbData> Number of bytes to read/write (mandatory for both reading and writing). Value range: 1-3000. <File_data> File data in bytes |
| Reference Sierra Wireless Proprietary | Notes - The data session is automatically ended when <ndata> data bytes are sent/received, and the module returns to command state and returns OK. - The data session can also be ended by <EOF pattern>, +++ or DTR. Refer to section 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table for the behavior of +++ or DTR. - ATO is not available for this command. |

### +KCERTDELETE Command: Delete Local Certificate from the Index

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+ KCERTDELETE =? | Response +KCERTDELETE: (list of possible <data_type>s),(list of possible <index>s) OK |
| Read command Syntax AT+ KCERTDELETE? | Response +KCERTDELETE: OK or +CME ERROR: <err> |
| Write command Syntax AT+ KCERTDELETE=<data_type> [,<index>] | Response OK or +CME ERROR: <err> Parameters <data_type> 0 Root certificate 1 Local certificate <index> Stored local certificate index. Default value = 0. Value range: 0 If <data_type> = 0 0 – 2 If <data_type> = 1 |
| Reference Sierra Wireless Proprietary |  |

### +KPRIVKDELETE Command: Delete Private Key from the Index

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+ KPRIVKDELETE =? | Response +KPRIVKDELETE: (list of possible <index>es) OK |
| Write command Syntax AT+ KPRIVKDELETE=<index> | Response OK or +CME ERROR: <err> Parameter <index> Stored private key index. Value range: 0 – 2 |
| Reference Sierra Wireless Proprietary |  |

# AVMS Commands

## +WDSC Command: Device Services Configuration

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+WDSC=? | Response +WDSC: (0-2), (list of supported <State>s) +WDSC: 3, (list of supported <State>s) +WDSC: 4, (list of supported <Timer_n>s) OK |
| Read command Syntax AT+WDSC? | Response +WDSC: 0,<State> +WDSC: 1,<State> +WDSC: 2,<State> +WDSC: 3,<State> +WDSC: 4,<Timer_1>[[,<Timer_2>]…[,<Timer_n]] OK |
| Write command Syntax For <Mode>= 0, 1, 2 or 3 AT+WDSC= <Mode>,<State> For <Mode>= 4 AT+WDSC= <Mode>, <timer_1> [[,<Timer_2>]… [,<Timer_n>]] | Response OK or +CME ERROR <err> Parameters <Mode> Integer type 0 User agreement for connection When this mode is activated and when a notification SMS is received by the embedded module, an indication (see +WDSI indication for more information) is returned by the embedded module to request for an agreement before connecting to the AirPrime Management Services server 1 User agreement for package download When this mode is activated, an indication (see +WDSI indication for more information) is returned by the embedded module to request for an agreement before downloading any package 2 User agreement for package install When this mode is activated, an indication (see +WDSI indication for more information) is returned by the embedded module to request for an agreement before installing any package 3 Polling mode The embedded module will initiate a connection to the Device Services server according to the defined timer 4 Retry mode If an error occurs during a connection to the Device Services server (GPRS establishment failed, etc.), the embedded module will initiate a new connection according to the defined timers. This mechanism is persistent to the reset. <State> Integer type; mode status For <Mode> = 0, 1 or 2 0 Disabled (default value) 1 Enabled For <Mode> = 3 Value in range 0 – 525600 (units = min) 0 The polling mode is deactivated <Timer_1> Timer between the first failed connection and the next attempt. Value in range 0 – 20160 (units = min). 0 The retry mode is deactivated 15 Default value <Timer_n> Timer between the nth failed attempt connection and the (n+1)th connection (n<=8). Value in range 1 – 20160 (units = min) Default values: <Timer_2> = 60 <Timer_3> = 240 <Timer_4> = 960 <Timer_5> = 2880 <Timer_6> = 10080 <Timer_7> = 10080 |
| Notes | - This command is available when the embedded module has finished the Device Services initialization (see +WDSI) and when the AVMS services are in prohibited state (see +WDSG). - Parameters <State> and <Timer_1> to <Timer_n> are stored in non-volatile memory. The &F command has no impact on these values. - The network registration is considered as “failed” when all connections configured by the retry mode have failed. This registration is forbidden while the APN is not set by the +WDSS command. |
| Examples | AT+WDSC=? +WDSC:(0-2),(0-1) +WDSC:3,(0-525600) +WDSC:4,(0-20160),(1-20160),(1-20160),(1-20160),(1-20160),(1-20160),(1-20160), (1-20160) OK AT+WDSC? // All modes are deactivated except retry mode which is used with default timers +WDSC: 0,0 +WDSC: 1,0 +WDSC: 2,0 +WDSC: 3,0 +WDSC: 4,15,60,240,960,2880,10080,10080 OK AT+WDSC=0,1 OK AT+WDSC? +WDSC: 0,1 +WDSC: 1,0 +WDSC: 2,0 +WDSC: 3,0 +WDSC: 4,15,60,240,960,2880,10080,10080 OK |

## +WDSD Command: Device Services Local Download

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+WDSD=? | Response +WDSD: (list of supported <Size>s) OK |
| Write command Syntax AT+WDSD= <Size> | Response <NACK> // User sends data OK or +CME ERROR <err> Parameter <Size> 1 – 24643584 Package size in bytes |
| Examples | AT+WDSD=? +WDSD: (1-24643584) OK AT+WDSD=1024 //download a 1kBytes package <NACK> //the device is ready to receive data //Send data OK //All data are well received by the module +WDSI: 3 //A package is ready to install (see +WDSI and +WDSR commands) |
| Reference Sierra Wireless Proprietary Command | Notes - This command is available when the embedded module has finished its initialization. - The response to the AT+WDSD=<Size> command is the <NACK> character when the device is ready to receive data using the 1K-Xmodem protocol - The flow control of the TE has to be set to ‘Hardware’ - This command will automatically activate the user agreement for install (see +WDSC command description). - No reset is made during the package download. - A timeout will happen (and a +CME ERROR: 3 is returned) if no data is sent to the device in 5 minutes. |

## +WDSE Command: Device Services Error

| HL7800 |  |
| --- | --- |
| Execute command Syntax AT+WDSE | Response [+WDSE:<HTTP_Status>] OK or +CME ERROR <err> Parameter <HTTP_Status> Integer type – last HTTP response received by the module 100 Continue 101 Switching Protocols 200 OK 201 Created 202 Accepted 203 Non-Authoritative Information 204 No Content 205 Reset Content 206 Partial content 300 Multiple Choices 301 Moved Permanently 302 Found 303 See Other 304 Not Modified 305 Use Proxy 307 Temporary Redirect 400 Bad Request 401 Unauthorized 402 Payment Required 403 Forbidden 404 Not Found 405 Method Not Allowed 406 Not Acceptable 407 Proxy Authentication Required 408 Request time-out 409 Conflict 410 Gone 411 Length Required 412 Precondition Failed 413 Request Entity too large 414 Request URI too large 415 Unsupported Media type 416 Request range unsatisfiable 417 Expectation failed 500 Internal server error 501 Not implemented 502 Bad Gateway 503 Service unavailable 504 Gateway time-out 505 HTTP version not supported If no session was made with the server, AT+WDSE only returns OK, without +WDSE: <HTTP_Status> intermediary response. |
| Notes | This command is available when the embedded module has finished the Device Services initialization (see +WDSI) and when AVMS services is activated (see +WDSG). |
| Examples | AT+WDSS=1,1 //A session was made with the server OK AT+WDSE +WDSE: 200 //The last HTTP response received is “OK” OK |

## +WDSG Command: Device Services General Status

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+WDSG=? | Response OK |
| Execute command Syntax AT+WDSG | Response +WDSG: <Indication>,<State> [+WDSG: <Indication>,<State>[…]] OK or +CME ERROR <err> Parameters <Indication> Integer type 0 Device services activation state 1 Session and package indication <State> Status of indication For <Indication>=0 0 Device services are prohibited. Devices services will never be activated. 1 Device services are deactivated. Connection parameters to a device services have to be provisioned. 2 Device services have to be provisioned. NAP parameters have to be provisioned. 3 Device services are activated. If a device has never been activated (first use of device services on this device), <State> is set to 1. The connection parameters are automatically provisioned, no action is needed from the user. For <Indication>=1 0 No session or package 1 A session is under treatment 2 A package is available on the server. 3 A package was downloaded and ready to install When a package was installed or a recovery was made, <State> is set to 0. |
| Notes | This command is available when the embedded module has finished the Device Services initialization (see +WDSI). |
| Examples | AT+WDSG=? OK AT+WDSG +WDSG: 0,3 //Device services are activated, +WDSG: 1,0 //No session to the server, no patch to download or to install OK |

## +WDSI Command: Device Services Indication

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+WDSI=? | Response +WDSI: (list of supported <Level>s) OK |
| Read command Syntax AT+WDSI? | Response [+WDSI: <Level>] OK |
| Write command Syntax AT+WDSI= <Level> | Response OK or +CME ERROR <err> Parameters <Level> Indication level, bit field (default value = 0) Bit set to 0 Indication deactivated Bit set to 1 Indication activated 0 No indication 1 Activate the initialization end indication (<Event> = 0) 2 Activate the server request for a user agreement indication (<Event> = 1, 2 and 3) 4 Activate the authentication indications (<Event> = 4 and 5) 8 Activate the session start indication (<Event> = 6, 7 and 8) 16 Activate the package download indications (<Event> = 9,10 and 11) 32 Activate the certified downloaded package indication (<Event> = 12 and13) 64 Activate the update indications (<Event> = 14,15 and 16) 128 Activate the fallback indication (<Event> = 17) 256 Activate download progress indication (<Event> = 18) 512 Reserved 1024 Reserved 2048 Activate provisioning indication (<Event> = 21) 4096 Reserved <Event> 0 Device services are initialized and can be used. Devices services are initialized when the SIM PIN code is entered and a dedicated NAP is configured (see +WDSS command) 1 The Device Services server requests the device to make a connection. The device requests a user agreement to allow the embedded module to make the connection. The response can be sent using +WDSR command and this indication can be returned by the device if the user has activated the user agreement for connection (see +WDSC command for more information) 2 The Device Services server requests the device to make a package download. The device requests a user agreement to allow the embedded module to make the download. The response can be sent using +WDSR command and this indication can be returned by the device if the user has activated the user agreement for download (see +WDSC command for more information). 3 The device has downloaded a package. The device requests a user agreement to install the downloaded package. The response can be sent using +WDSR command and this indication can be returned by the device if the user has activated the user agreement for install (see +WDSC command for more information). 4 The embedded module starts sending data to the server 5 Authentication with the server failed 6 Authentication has succeeded, a session with the server started 7 Session with the server failed 8 Session with the server is finished 9 A package is available on the server and can be downloaded by the embedded module. A <Data> parameter is returned indicating the package size in kB 10 A package was successfully downloaded and stored in flash 11 An issue happens during the package download. If the download has not started (+WDSI: 9 indication was not returned), this indication indicates that there is not enough space in the device to download the update package. If the download has started (+WDSI: 9 indication was returned), a flash problem implies that the package has not been saved in the device 12 Downloaded package is certified to be sent by the AirPrime Management Services server 13 Downloaded package is not certified to be sent by the AirPrime Management Services server 14 Update will be launched 15 OTA update client has finished unsuccessfully 16 OTA update client has finished successfully 17 A fallback mechanism was launched 18 Download progress. This event is returned without <Data> parameter to indicate that a download starts. During the download, a percentage progress is indicated in <Data> parameter 19 Reserved 20 Reserved 21 A provision was made by the AirPrime Management Services server 22 Reserved <Data> Specific data for some <Event> For<Event>=9, <Data> indicates the package size in bytes, which will be downloaded For<Event>=17, <Data> indicates if the fallback was asked by the user or applied because a recovery was necessary 0 Automatic recovery (a recovery mechanism was made) 1 Fallback asked by the user (see +WDSF for more information) For<Event>=18, <Data> indicates the download progress in percentage For<Event>=21, <Data> indicates the provisioned parameters 0 Reserved 1 Reserved 2 Reserved 3 Reserved 4 Reserved 5 Reserved 6 Reserved 7 Reserved 8 Reserved 9 Device Service polling mode (see +WDSC command for more information) 10 Reserved 11 Reserved 12 Reserved 13 Reserved |
| Unsolicited Notification | Response +WDSI: <Event>[,<Data>] |
| Notes | - This command is available when the embedded module has finished its initialization. - To receive +WDSI indications, Device Services should be activated (see +WDSG for more information). - <Level> is stored in non-volatile memory. The default value can be restored using AT&F. - When the AVMS status is updated, the +WDSI unsolicited response will be displayed according to the AVMS status change at the same time. If there is a power loss when the AVMS status is updating but it was updated successfully, the +WDSI unsolicited response may be lost. |
| Examples | AT+WDSI=? +WDSI: (0-2047) OK AT+WDSI? +WDSI: 0 // All indications are deactivated OK AT+WDSI=207 OK +WDSI: 1 // The devices services server request a connection to the // embedded module AT+WDSR=1 // Accept the connection OK +WDSI: 4 // The embedded module will send the first data to the AirPrime // Management Services server +WDSI: 6 // The authentication succeeded +WDSI: 8 // The session with the server is over +WDSI: 9,1000 // A package will be downloaded, the size is 1kbytes +WDSI: 18,“1%” // 1% was downloaded +WDSI: 18,“100%” // The whole package was downloaded +WDSI: 10 // The whole package was stored in flash |

## +WDSR Command: Device Services Reply

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+WDSR=? | Response +WDSR: (list of supported <Reply>s),(list of supported <Timer>s) OK |
| Write command Syntax AT+WDSR= <Reply> [,<Timer>] | Response OK or +CME ERROR <err> Parameters <Reply> Reply to user agreement request 0 Delay or refuse the connection to the server 1 Accept the connection to the server 2 Delay or refuse the download 3 Accept the download 4 Accept the install 5 Delay the install <timer> 0 – 1440 Timer (in minutes) until a new user agreement request is returned by the module. This parameter is only available for <Reply>=0, 2 or 5. Default value = 30. Value 0 indicates that the application refuses the user agreement (impossible when <Reply>=5). |
| Notes | - This command is available when the embedded module has finished the Device Services initialization (see +WDSI) and when AVMS services are activated (see +WDSG) - It is impossible to refuse an install request (AT+WDSR=5,0); this will return +CME ERROR: 3. - After an install delay if the embedded module is powered down until after the delay, it is not powered on and the new user agreement request should be returned at the newt start up. |
| Examples | AT+WDSR=? +WDSR: (0-5),(0-1440) OK +WDSI: 1 //The device Services server requests the device to make a connection // to the server. The user is requested to allow the connection. AT+WDSR=1 OK +WDSI: 3 //A user agreement is requested to install a package AT+WDSR=5,10 //A delay of 10 minutes is requested OK +WDSI: 3 //10 minutes later, a new user agreement is requested to install a //package AT+WDSR=4 //The install is requested OK |

## +WDSS Command: Device Services Session

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+WDSS=? | Response +WDSS: 0,(Max length for <Apn>),(Max length for <User>),(Max length for <Pwd>),(list of supported <Cid>s) [+WDSS: 1,(list of supported <Action>s for this <Mode>)] OK |
| Read command Syntax AT+WDSS? | Response [+WDSS: 0,<Apn>[,<User>],<Cid>] [+WDSS: 1,<Action>] OK |
| Write command Syntax For <Mode>=0: AT+WDSS= <Mode>[,<Apn> [,<User> [,<Pwd>[,<Cid>] ]]] For <Mode>=1 AT+WDSS= <Mode>,<Action> | Response OK or +CME ERROR <err> Parameters <Mode> Integer type 0 PDP context configuration for Device Services 1 User initiated connection to the Device services server <Apn> Access Point Name for Devices Services. String type up to 50 characters. For empty string, see <Cid> <User> Login for the APN. String type, up to 30 characters <Pwd> Password for the APN. String type, up to 30 characters <Cid> 1 – 5 Context ID used for AVMS PDP activation For connection to the server: If the PDP of <Cid> has already been activated: - when <Apn> is set as an empty string, AVMS connection will directly reuse the PDP of that <Cid>, or - when <Apn> is set as a non-empty string, it will check if <Apn> matches with +CGDCONT settings to reuse the connection Otherwise, it will activate with APN <Apn>. <Action> For <Mode>=1 only 0 Release the current connection to the Device Services Server 1 Establish a connection to the Device Services Server |
| Notes | - This command is available when the embedded module has finished the Device Services initialization (see +WDSI). - <Apn>, <User>, <Pwd> and <Cid> parameters are automatically stored in non-volatile memory. AT&F has no effect on these parameters. - AT+WDSS? command only returns OK if no APN is defined. - When a request is sent to the embedded module to resume a non-existent or unsuspended session, +CME ERROR: 3 is returned. - When a request is sent to the embedded module to release a non-existent session, +CME ERROR: 3 is returned. - Depending on +WDSM configuration, when no dedicated NAP is defined using +WDSS command and a session is asked (by AT command or notify by SMS), the embedded module will use a NAP defined by +CGDCONT command to activate the dedicated PDP context. This NAP will be recorded to configure the NAP Device Services and it will be used to activate the dedicated PDP context for the next sessions. - When the PDP context cannot be activated because of bad AirVantage Management Services NAP configuration, the embedded module will use a NAP defined by +CGDCONT command to activate the dedicated PDP context (but the initial NAP configuration is not erased). - Activation is done if the embedded module is registered on the network. If the embedded module is not registered when the command is performed, activation will be done at the next network registration (even if the embedded module resets). - No GPRS connection to the AirVantage Management Services server is possible when a registration is not completed. - If reuse of existing activated PDP context is required for all internet connections, set the <Cid> accordingly. For example, in LTE, if the internet connection uses PDP of cid1, then <Cid> should be 1. - AT+WDSS=0 will remove all stored information (<Apn>, <User>, <Pwd> and <Cid>). <Apn> will become unprovisioned, but not an empty string. |
| Examples | AT+WDSS? OK //No APN defined AT+WDSS=? +WDSS: 0, 50,30,30,(1-5) +WDSS: 1,(0-1) OK AT+WDSS=0,“Sierra Wireless”,,,5 //Define the APN for the Device Services; contect //ID = 5 OK AT+WDSS? +WDSS: 0,“Sierra Wireless”,,5 +WDSS: 1,0 OK AT+WDSS=1,1 //Initiation of a connection to the Device Services server OK AT+WDSS=1,0 //Release connection to the Device Services server OK // Example for LTE with only one PDP context is allowed at+cgdcont? +CGDCONT: 1,"IP","broadband","10.191.8.184",0,0,0,0,0,0 OK at+wdss=0,”broadband”,,,1 OK at+wdss? +WDSS: 0,"broadband",,1 +WDSS: 1,0 at+cgact? +CGACT: 1,1 OK at+wdss=1,1 OK +WDSI: 4 +WDSI: 8 at+cgdcont? +CGDCONT: 1,"IP","broadband","10.191.8.184",0,0,0,0,0,0 OK //Example for reusing activated PDP at+cgdcont? +CGDCONT: 1,"IP","broadband","10.191.8.184",0,0,0,0,0,0 OK at+wdss=0 //Clear all setting OK at+wdss? OK at+wdss=0,,,,1 //Define empty string APN OK at+wdss? +WDSS: 0,"",,1 +WDSS: 1,0 OK at+wdss=1,1 //Reuse activated PDP of cid 1 for connection OK +WDSI: 4 +WDSI: 8 |

# Test Commands

The following commands are used for testing purposes.

## +WMTXPOWER Command: Test RF Tx

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+ WMTXPOWER=? | Response +WMTXPOWER: (list of supported <ENABLE>s),(list of supported 4G <BAND>s), (list of supported 4G <CHANNEL>s), (list of supported 4G <POWER_LEVEL>s),(list of supported <BANDWIDTH>s) OK |
| Read command Syntax AT+ WMTXPOWER? | Response For GSM: +WMTXPOWER: <ENABLE>[,<BAND>,<CHANNEL>,<POWER_LEVEL>, <MULTISLOT>] OK For UMTS: +WMTXPOWER: <ENABLE>[,<BAND>,<CHANNEL>,<POWER_LEVEL>] OK For LTE: +WMTXPOWER: <ENABLE>[,<BAND>,<CHANNEL>,<POWER_LEVEL>, <BANDWIDTH>] OK Note that parameters <BAND>, <CHANNEL>, <POWER_LEVEL>, <MULTISLOT> and <BANDWIDTH> are only available if <ENABLE>=1. |
| Write command Syntax AT+ WMTXPOWER= <ENABLE>, [,<BAND>, <CHANNEL>, <POWER_ LEVEL>, [<MULTISLOT>][,<BANDWIDTH>]] | Response OK Parameters <ENABLE> 0 Stop the burst emission 1 Start the burst emission <BAND> Tx burst band emission. This is a mandatory parameter if <ENABLE>=1, but is not allowed if <ENABLE>=0. For GSM: 900 GSM900 band 1800 DCS band For UMTS: 1 Band I (2100 band) 2 Band II (1900 band) 5 Band V (850 band) 8 Band VIII (700 band) For LTE: 2 PCS 3 DCS 4 AWS 5 CLR 8 E-GSM 12 Lower SMH blocks A/B/C 13 Upper SMH block C 17 Lower SMH blocks B/C 20 EU Digital Dividend 28 APT <CHANNEL> Tx burst channel emission. This is a mandatory parameter if <ENABLE>=1, but is not allowed if <ENABLE>=0. For GSM: If <BAND>=900 0 – 124, 975 – 1023 If <BAND>=1800 512 – 885 For UMTS: If <BAND>=1 9612 – 9888 If <BAND>=2 9262 – 9538, 12, 37, 62, 87, 112, 137, 162, 187, 212, 237, 262, 287 If <BAND>=5 4132 – 4233, 782, 787, 807, 812, 837, 862 If <BAND>=8 2712 – 2863 For LTE: If <BAND>=2 18600 – 19199 If <BAND>=3 19200 – 19949 If <BAND>=4 19950 – 20399 If <BAND>=5 20400 – 20649 If <BAND>=8 21450 – 21799 If <BAND>=12 23010 – 23179 If <BAND>=13 23180 – 23279 If <BAND>=17 23730 – 23849 If <BAND>=20 24150 – 24449 If <BAND>=28 27210 – 27659 <POWER_LEVEL> Tx burst power. This is a mandatory parameter if <ENABLE>=1, but is not allowed if <ENABLE>=0. Range: 5 (33 dBm) to 19 (5 dBm) for GSM900 bands 0 (30 dBm) to 15 (0 dBm) for GSM1800 bands 0 (0 dBm) to 384 (24 dBm) for all UMTS bands 0 (0 dBm) to 368 (23 dBm) for all LTE bands <MULTISLOT> Defines the slot used in Tx burst emissions. This parameter is not allowed if <ENABLE>=0 and is only applicable with GSM bands. 0 Emit on one time slot (GSM) 1 Emit on two time slots (GPRS compliant) 2 Emit on three time slots 3 Emit on four time slots <BANDWIDTH> Defines the bandwidth of Tx burst emissions. This parameter is not allowed if <ENABLE>=0 and is only applicable with LTE bands. 0 1.4MHz 1 3 MHz 2 5 MHz 3 10 MHz 4 15 MHz 5 20 MHz |
| Reference Sierra Wireless Proprietary | Notes - Burst must be sent on all TDMA frames. - Only one burst can be emitted at a time. - This AT command is not available if AT+WMRXPOWER is enabled. - The module must be restarted after using this command. |
| Examples | // Using an HL7690 module: at+wmtxpower=? +WMTXPOWER: (0-1),(3,8,20),(19200-19949,21450-21799,24150-24449),(0-368),(0-5) OK at+wmtxpower=1,3,19575,0,3 // Tx burst is emitted at Uarfcn 19575 OK at+wmtxpower=0 OK at+wmtxpower=1,8,21799,0,3 // Tx burst is emitted at Uarfcn 21799 OK at+wmtxpower=0 OK // Using an HL7692 module: at+wmtxpower=? +WMTXPOWER: (0-1),(900,1800),(0-124,975-1023,512-885),(5-19,0-15),(0-3) +WMTXPOWER: (0-1),(3,8,20),(19200-19949,21450-21799,24150-24449),(0-368),(0-5) OK at+wmtxpower=1,900,0,19,0 // A Tx burst is emitted at Earfcn 0 OK at+wmtxpower=1,1800,512,15,0 // A Tx burst is emitted at Earfcn 512 OK // Using an HL7648 module: at+wmtxpower=? +WMTXPOWER: (0-1),(2,4,12),(18600-19199,19950-20399,23010-23179),(0-368),(0-5) OK at+wmtxpower=1,2,18600,0,0 // A Tx burst is emitted at Earfcn 18600 OK at+wmtxpower=0 OK // Using an HL7650 module: at+wmtxpower=? +WMTXPOWER: (0,1),(2,5),(9262-9538,12,37,62,87,112,137,162,187,212,237,262,287, 4132-4233,782,787,807,812,837,862),(0-384) +WMTXPOWER: (0-1),(2,4,5,13,17),(18600-19199,19950-20399,20400-20649,23180-23279,23730-23849),(0-368),(0-5) +WMTXPOWER: (0-1),(2,4,12,17),(18600-19199,19950-20399,23010-23179,23730-23849),(0-368),(0-5) OK at+wmtxpower=1,2,9262,0 // A Tx burst is emitted at Uarfcn 9262 OK at+wmtxpower=0 OK at+wmtxpower=1,2,18600,0,0 // A Tx burst is emitted at Earfcn 18600 OK at+wmtxpower=0 OK |

## +WMRXPOWER Command: Test RF Rx

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+ WMRXPOWER=? | Response +WMRXPOWER: (list of supported <ENABLE>s),(list of supported 4G <BAND>s), (list of supported 4G <CHANNEL>s) OK |
| Read command Syntax AT+ WMRXPOWER? | Response +WMRXPOWER: <ENABLE>[,<BAND>,<CHANNEL>,[<EXP_POWER>]] OK Note that parameters <BAND>, <CHANNEL> and <EXP_POWER> are only available if <ENABLE>=1. <EXP_POWER> is only applicable for GSM. |
| Write command Syntax AT+ WMRXPOWER= <ENABLE> [,<BAND>, <CHANNEL>, [<EXP_POWER>]] | Response For GSM: +WMRXPOWER: <POWER1> OK For UMTS and LTE: +WMRXPOWER: <POWER1>,<POWER2> OK Parameters <ENABLE> 0 Stop the Rx measurement 1 Start the Rx measurement <BAND> Rx band to read. This is a mandatory parameter if <ENABLE>=1, but is not allowed if <ENABLE>=0. For GSM: 900 GSM900 band 1800 DCS band For UMTS: 1 Band I (2100 band) 2 Band II (1900 band) 5 Band V (850 band) 8 Band VIII (700 band) For LTE: 2 PCS 3 DCS 4 AWS 5 CLR 8 E-GSM 12 Lower SMH blocks A/B/C 13 Upper SMH block C 17 Lower SMH blocks B/C 20 EU Digital Dividend 28 APT <CHANNEL> Rx channel to read. This is a mandatory parameter if <ENABLE>=1, but is not allowed if <ENABLE>=0. For GSM: If <BAND>=900 0 – 124, 975 – 1023 If <BAND>=1800 512 – 885 For UMTS: If <BAND>=1 10562 – 10838 If <BAND>=2 9662 – 9938 If <BAND>=5 4357 – 4458 If <BAND>=8 2937 – 3088 For LTE: If <BAND>=2 600 – 1199 If <BAND>=3 1200 – 1949 If <BAND>=4 1950 – 2399 If <BAND>=5 2400 – 2649 If <BAND>=8 3450 – 3799 If <BAND>=12 5010 – 5179 If <BAND>=13 5180 – 5279 If <BAND>=17 5730 – 5849 If <BAND>=20 6150 – 6449 If <BAND>=28 9210 – 9659 <EXP_POWER> Expected power in dBm. This is a mandatory parameter if <ENABLE>=1, but is not allowed if <ENABLE>=0. Note that this parameter is only applicable for GSM. <POWER1> Received power at primary antenna in dBm <POWER2> Received power at secondary antenna in dBm |
| Reference Sierra Wireless Proprietary | Examples // Using an HL7690 module: at+wmrxpower=? +WMRXPOWER: (0-1),(3,8,20),(1200-1949,3450-3799,6150-6449) OK at+wmrxpower=1,3,1575 // Read Uarfcn 1575 +WMRXPOWER: -80.0,-101.2 // Rx power -80.0 dBm at primary antenna // Rx power -101.2 dBm at diversity antenna OK at+wmrxpower=1,8,3625 // Read Uarfcn 3625 +WMRXPOWER: -88.8,-98.2 // Rx power -88.8 dBm at primary antenna // Rx power -98.2 dBm at diversity antenna OK // Using a HL7692 module: at+wmrxpower=? +WMRXPOWER: (0-1),(900,1800),(0-124,975-1023,512-885) +WMRXPOWER: (0-1),(3,8,20),(1200-1949,3450-3799,6150-6449) OK at+wmrxpower=1,900,0,-45 // Read Earfcn 0 +WMRXPOWER: -46.8 // Rx power -46.8 dBm at primary antenna OK at+wmrxpower=1,1800,512,-45 // Read Earfcn 512 +WMRXPOWER: -46.4 // Rx power -46.4 dBm at primary antenna OK // Using an HL7648 module: at+wmrxpower=? +WMRXPOWER: (0-1),(2,4,12),(600-1199,1950-2399,5010-5179) OK at+wmrxpower=1,4,1950 // Read Earfcn 1950 +WMRXPOWER: -95.0,-108.8 // Rx power -95.0 dBm at primary antenna // Rx power -108.8 dBm at secondary antenna OK // Using an HL7650 module: at+wmrxpower=? +WMRXPOWER: (0,1),(2,5),(9662-9938,4357-4458) +WMRXPOWER: (0-1),(2,4,5,13,17),(600-1199,1950-2399,2400-2649,5180-5279,5730-5849) +WMRXPOWER: (0-1),(2,4,12,17),(600-1199,1950-2399,5010-5179,5730-5849) OK at+wmrxpower=1,2,9662 // Read Uarfcn 9662 +WMRXPOWER: -97.9,-103.8 // Rx power -97.9 dBm at primary antenna // Rx power -103.8 dBm at secondary antenna OK at+wmrxpower=1,2,600 // Read Earfcn 600 +WMRXPOWER: -95.0,-108.8 // Rx power -95.0 dBm at primary antenna // Rx power -108.8 dBm at secondary antenna OK |

# NV Commands

## Auto Generation of NV Backup Files

There are 3 NV partitions in flash used by the firmware:

Static Calibrated NV partition

Static Fixed NV partition

Dynamic NV partition

NV backup is per partition based, with one NV backup file per partition. These are labelled with <file id>=0, 1, 2 in the NV log and by firmware design.

The firmware automatically generates NV backup files from existing NV data at ~8 seconds after boot if one of the following conditions are met:

NV backup of a partition does not exist, or it has been corrupted unexpectedly.

NV backup files exist, but the firmware version has changed while IMEI has not changed, in comparison to the records in the backup file.

NV backup files exist, but the firmware version has changed and a valid IMEI has been updated, in comparison to the records in the backup file.

An automatic backup file generation is notified with +NVBU_IND with <status>=0 on all AT ports.

## Auto Recovery from Backup NV Files

NV recovery is automatically done if an NV corruption is detected during NV initialization at boot.

The firmware automatically recovers NV data from available NV backups when one or more NV items are corrupted. This is notified with +NVBU_IND with <status>=3 on all AT ports.

Manual NV data restores all data from backup file to the original NV partition.

The firmware will try to recover corrupted or missing NV data items instead of all NV data items (partial restore) if possible; otherwise, the firmware restores all NV data items (full restore).

If the firmware crashes with 10 consecutive loops and a full restore has not been performed before, the firmware performs a full restore of all NV data items. Only consecutive crashes that happened within 8 seconds after the module boots is counted for this reset loop detection.

## +NVBU Command: NV Backup Status and Control

| HL7800 |  |
| --- | --- |
| Test command Syntax AT+NVBU=? | Response +NVBU: (0-2) OK |
| Read command Syntax AT+NVBU? | Returns list of NV backup with the format: +NVBU: <file id>,<backup date>,<backup firmware version> Response [+NVBU: 0,<backup date>,<backup firmware version>] [+NVBU: 1,<backup date>,<backup firmware version>] [+NVBU: 2,<backup date>,<backup firmware version>] OK Parameters <file id> Backup file ID corresponding to an NV partition in flash <backup date> NV backup generation date <backup firmware version> Firmware version used to generate the NV backup |
| Write command Syntax For <mode> = 0 or 1 AT+NVBU= <mode> [,<parti_id>] For <mode> = 2 AT+NVBU= <mode>[,<clear>] | Response For <mode>=0 or 1 OK For <mode>=2 and <clear>=0 <log data 0> [<log data 1>] … [<log data n>] OK For <mode>=2 and <clear>=1 OK Parameters <mode> 0 Generate backup of all NV data to NV backup partition 1 Restore all NV data from the NVM backup partition 2 List logs of NV backup operations <log data> NV backup operations log data <parti_id> 0 Static Calibrated NV 1 Static Fixed NV partition 2 Dynamic NV partition 3 All NV partitions <clear log> 0 Read log 1 Clear log |
| Reference Sierra Wireless Proprietary | Notes - Status of operations for <mode>=0 and <mode>=1 is notified by +NVBU_IND unsolicited notifications with <status>=0 and <status>=1 respectively on the AT port that executed the write command. - Execution of the write command with <mode>=1 is followed by a modem reboot automatically; NVs are restored to their default values on booting. - The number of lines of <log data> ranges from 1 to 2142 lines. - This command can be used without a SIM. - <mode>=2 is for retrieving log for R&D analysis and not fully documented, generally: - USER=0 for operations triggered by the firmware - USER=1 for manual operations |
| Example | # automatic backup files generation after FW upgrade, notified by +NVBU_IND +NVBU_IND: 0,0,"2015/11/16 04:23:33","BHL7618.3.0.154401.201511132200.x7120_2" +NVBU_IND: 0,1,"2015/11/16 04:23:33","BHL7618.3.0.154401.201511132200.x7120_2" +NVBU_IND: 0,2,"2015/11/16 04:23:33","BHL7618.3.0.154401.201511132200.x7120_2" # manual generation of backup files from existing NV partitions AT+NVBU=0,3 OK +NVBU_IND: 0,0,"2015/11/16 04:23:39","BHL7618.3.0.154401.201511132200.x7120_2" +NVBU_IND: 0,1,"2015/11/16 04:23:39","BHL7618.3.0.154401.201511132200.x7120_2" +NVBU_IND: 0,2,"2015/11/16 04:23:39","BHL7618.3.0.154401.201511132200.x7120_2" # manual restore of backup files to original NV partitions AT+NVBU=1,3 OK +NVBU_IND: 1,0,"2015/11/16 04:23:39","BHL7618.3.0.154401.201511132200.x7120_2" +NVBU_IND: 1,1,"2015/11/16 04:23:39","BHL7618.3.0.154401.201511132200.x7120_2" +NVBU_IND: 1,2,"2015/11/16 04:23:39","BHL7618.3.0.154401.201511132200.x7120_2" <module reboots automatically> # to retrieve the list of NV related operations done by the Firmware at+nvbu=2 [2015/11/16 04:02:49] BULO: MDM- BHL7618.3.0.154401.201511132200.x7120_2 [2015/11/16 04:02:49] BUFL: GENERATE USER=0 FILE=3 LAS=0,0,0 [2015/11/16 04:02:49] BUFM: ENCODE F=0 REF=0 CNT=15/15 41 [2015/11/16 04:02:49] BUFM: ENCODE F=1 REF=0 CNT=16/16 31 [2015/11/16 04:02:49] BUFM: ENCODE F=2 REF=42 CNT=41/41 57 [2015/11/16 04:23:39] BUFL: GENERATE USER=1 FILE=3 LAS=0,0,0 [2015/11/16 04:23:39] BUFM: ENCODE F=0 REF=0 CNT=15/15 41 [2015/11/16 04:23:39] BUFM: ENCODE F=1 REF=0 CNT=16/16 31 [2015/11/16 04:23:39] BUFM: ENCODE F=2 REF=42 CNT=41/41 57 [2015/11/16 04:23:43] BUFL: RESTORE USER=1 FILE=3 LAS=0,0,0 [2015/11/16 04:23:43] BUFM: DECODE-2 F=0 REF=1 CNT=15/15 15,41 [2015/11/16 04:23:43] BUFM: DECODE-2 F=1 REF=1 CNT=16/16 16,31 [2015/11/16 04:23:43] BUFM: DECODE-2 F=2 REF=43 CNT=41/41 41,57 OK |

## +NVBU_IND Notification: NV Backup Status Notification

| HL7800 |  |
| --- | --- |
| Unsolicited Notification | Response +NVBU_IND: <status>,<file id>,… For <status>=0 +NVBU_IND: <status>,<file id>,<backup date>,<backup firmware version> For <status>=1 +NVBU_IND: <status>,<file id>,<backup date used for restore>,<backup firmware version used for restore> For <status>=2 +NVBU_IND: <status>,<file id>,<backup date used for restore>,<backup firmware version used for restore>,<num NV> <NV ID 1>[<NV ID 2>...[<NV ID 16><CR><LF>]] … Parameters <status> NV backup status 0 Indicates completion of NV backup generation 1 Indicates completion of NV backup restore 2 Indicates that backup data were restored when the NV corruption was detected during NV initialization <backup date> NV backup generation date <backup firmware version> Firmware version used to generate the NV backup <backup date used for restore> Generation date of the NV backup that was used for the NV restore <backup firmware version used for restore> Firmware version used to generate the NV backup that was used for the NV restore <num NV> Total number of NV items restored <NV ID> List of NV item IDs with data restored, expressed in hexadecimal number delimited by spaces, and delimited by <CR><LF> every 16 numbers. |
| Reference Sierra Wireless Proprietary | Notes The list of <NV ID> is expressed in 16 hexadecimal numbers per line. |
| Examples | - # recovery in calibrated NV partition after Firmware boot - # note that the data is also logged by NV log (i.e. AT+NVBU=2) - +NVBU_IND: 2,0,"2015/11/16 04:23:39","BHL7618.3.0.154401.201511132200.x7120_2",15 - 10034900 10034901 10034401 10034402 10034902 10035400 10035401 10035402 10035403 10035500 10035501 10035502 10050000 10310000 10370000 |

# Appendix

## Result Codes and Unsolicited Messages

| Verbose Result Code | Numeric | Type | Description |
| --- | --- | --- | --- |
| +CCCM: <ccm> | like verbose | Unsolicited |  |
| +CME ERROR: <err> | like verbose | Final |  |
| +CMS ERROR: <err> | like verbose | Final or unsolicited |  |
| +CMTI | like verbose | Unsolicited |  |
| +CBM | like verbose | Unsolicited |  |
| +CDS | like verbose | Unsolicited |  |
| +COLP: <number>,<type>[,<subaddr>, <satype>[,<alpha>]] | like verbose | Intermediate |  |
| +CR: <type> | like verbose | Intermediate |  |
| +CREG: <stat>[,<lac>,<ci>] | like verbose | Unsolicited |  |
| +CRING: <type> | like verbose | Unsolicited |  |
| +CSSI: <code1>[,<index>] | like verbose | Intermediate |  |
| +CSSU: <code2>[,<index>[,<number>,<type> [,<subaddr>,<satype>]]] | like verbose | Unsolicited |  |
| +CUSD: <m>[,<str>,<dcs>] | like verbose | Unsolicited |  |
| BUSY | 6 | Final |  |
| CONNECT | 1 | Intermediate | connection has been established |
| CONNECT <text> | manufacturer specific | Intermediate | like CONNECT but manufacturer specific <text> gives additional information (e.g. connection data rate) |
| ERROR | 4 | Final | command not accepted |
| NO ANSWER | 7 | Final | connection completion timeout |
| NO CARRIER | 3 | Final | connection terminated |
| NO DIALTONE | 5 | Final | no dial tone detected |
| OK | 0 | Final | acknowledges execution of a command line |
| RING | 2 | Unsolicited | incoming call signal from network |

## Error Codes

### CME Error Codes

| <err> Code | Meaning |
| --- | --- |
| 0 | Phone failure |
| 1 | No connection to phone |
| 2 | Phone-adapter link reserved |
| 3 | Operation not allowed |
| 4 | Operation not supported |
| 5 | PH-SIM PIN required |
| 6 | PH-FSIM PIN required |
| 7 | PH-FSIM PUK required |
| 10 | SIM not inserted |
| 11 | SIM PIN required |
| 12 | SIM PUK required |
| 13 | SIM failure |
| 14 | SIM busy |
| 15 | SIM wrong |
| 16 | Incorrect password |
| 17 | SIM PIN2 required |
| 18 | SIM PUK2 required |
| 20 | Memory full |
| 21 | Invalid index |
| 22 | Not found |
| 23 | Memory failure |
| 24 | Text string too long |
| 25 | Invalid characters in text string |
| 26 | Dial string too long |
| 27 | Invalid characters in dial string |
| 30 | No network service |
| 31 | Network timeout |
| 32 | Network not allowed - emergency call only |
| 40 | Network personalization PIN required |
| 41 | Network personalization PUK required |
| 42 | Network subset personalization PIN required |
| 43 | Network subset personalization PUK required |
| 44 | Service provider personalization PIN required |
| 45 | Service provider personalization PUK required |
| 46 | Corporate personalization PIN required |
| 47 | Corporate personalization PUK required |
| 48 | Hidden key required |
| 49 | EAP method not supported |
| 50 | Incorrect parameters |
| 99 | Resource limitation |
| 100 | Unknown |
| 103 | Illegal MS |
| 106 | Illega lME |
| 107 | GPRS services not allowed |
| 111 | PLMN not allowed |
| 112 | Location area not allowed |
| 113 | Roaming not allowed in this location area |
| 132 | Service option not supported |
| 133 | Requested service option not subscribed |
| 134 | Service option temporarily out of order |
| 148 | Unspecified GPRS error |
| 149 | PDP authentication failure |
| 150 | Invalid mobile class |
| 201 | Alternate SIM conflict |
| 500 | CTS Handover on Progress |
| 501 | Cellular Protocol Stack Out of service state |
| 502 | CTS Unspecified Error |
| 650 | General AVMS error |
| 651 | Communication error |
| 652 | Session in progress |
| 654 | RDMS services are in “deactivated” state |
| 655 | RDMS services are in “prohibited” state |
| 656 | RDMS services are in “to be provisioned” state; no available NAP |
| 800 | SIM Security unspecified error |
| 902 | No more sockets available; the maximum number has been reached |
| 903 | Memory problem |
| 904 | DNS error |
| 905 | TCP disconnection by the server |
| 906 | TCP/UDP connection error |
| 907 | Generic error |
| 908 | Fail to accept client request's |
| 909 | Data send by KTCPSND/KUDPSND are incoherent |
| 910 | Bad session ID |
| 911 | Session is already running |
| 912 | No more sessions can be used (maximum session is 32) |
| 913 | Socket connection timer timeout |
| 914 | Control socket connection timer timeout |
| 915 | A parameter is not expected |
| 916 | A parameter has an invalid range of values |
| 917 | A parameter is missing |
| 918 | Feature is not supported |
| 919 | Feature is not available |
| 920 | Protocol is not supported |
| 921 | Error due to invalid state of bearer connection |
| 922 | Error due to invalid state of session |
| 923 | Error due to invalid state of terminate port data mode |
| 924 | Error due to session busy, retry later |
| 925 | Failed to decode HTTP header's name, missing ':' |
| 926 | Failed to decode HTTP header's value, missing 'cr/lf' |
| 927 | HTTP header's name is an empty string |
| 928 | HTTP header's value is an empty string |
| 929 | Format of input data is invalid |
| 930 | Content of input data is invalid or not supported |
| 931 | The length of a parameter is invalid |
| 932 | The format of a parameter is invalid |

### CEER Error Codes

| <cause> | <description> |
| --- | --- |
| 0 | No cause information available |
| 1 | Unassigned (unallocated) number |
| 3 | No route destination |
| 6 | Channel unacceptable |
| 8 | Operator determined barring |
| 16 | Normal call clearing |
| 17 | User busy |
| 18 | No user responding |
| 19 | User alerting, no answer |
| 21 | Call rejected |
| 22 | Number changed |
| 26 | Non selected user clearing |
| 27 | Destination out of order |
| 28 | Invalid number format (incomplete number) |
| 29 | Facility rejected |
| 30 | Response to STATUS ENQUIIRY |
| 31 | Normal, unspecified |
| 34 | No circuit / channel available |
| 38 | Network out of order |
| 41 | Temporary failure |
| 42 | Switching equipment congestion |
| 43 | Access information discarded |
| 44 | Requested circuit / channel not available |
| 47 | Resources unavailable, unspecified |
| 49 | Quality of service unavailable |
| 50 | Requested facility not subscribed |
| 55 | Incoming calls barred with in the CUG |
| 57 | Bearer capability not authorized |
| 58 | Bearer capability not presently available |
| 63 | Service or option not available, unspecified |
| 65 | Bearer service not implemented |
| 68 | ACM equal to or greater than AC Mmax |
| 69 | Requested facility not implemented |
| 70 | Only restricted digital information bearer capability is available |
| 79 | Service or option not implemented, unspecified |
| 81 | Invalid transaction identifier value |
| 87 | User not member of CUG |
| 88 | Incompatible destination |
| 91 | Invalid transit network selection |
| 95 | Semantically incorrect message |
| 96 | Invalid mandatory information |
| 97 | Message type non-existent or not implemented |
| 98 | Message type not compatible with protocol state |
| 99 | Information element non-existent or not implemented |
| 100 | Conditional IE error |
| 101 | Message not compatible with protocol state |
| 102 | Recovery on timer expiry |
| 103 | Illegal MS |
| 106 | Illegal ME |
| 107 | GPRS service not allowed |
| 111 | Protocol error, unspecified |
| 112 | Location area not allowed |
| 113 | Roaming not allowed in this location area |
| 124 | MBMS bearer capabilities insufficient for the service |
| 125 | LLC or SNDCP failure |
| 126 | Insufficient resources |
| 127 | Missing or unknown APN |
| 128 | Unknown PDP address or PDP type |
| 129 | User authentication failed |
| 130 | Activation rejected by GGSN |
| 131 | Activation reject,unspecified |
| 132 | Service not supported |
| 133 | Requested service option not subscribed |
| 134 | Service option temporarily out of order |
| 135 | NSAPI already used |
| 136 | Regular PDP context deactivation |
| 137 | QoS not accepted |
| 138 | Network failure |
| 139 | Reactivation requested |
| 140 | Feature not supported |
| 141 | Semantic error in the TFT operation |
| 142 | Syntactical error in the TFT operation |
| 143 | Unknown PDP context |
| 144 | Semantic errors in packet filter(s) |
| 145 | Syntactical errors in packet filter(s) |
| 146 | PDP context without TFT already activated |
| 148 | Unspecified GPRS error |
| 149 | PDP authentification error |
| 152 | Single address bearers only allowed |
| 153 | ESM information only allowed |
| 154 | ESM information not received |
| 155 | PDN connection does not exist |
| 156 | Collision with network initiated request |
| 159 | Unsupported QCI value |
| 160 | Bearer handling not supported |
| 165 | Maximum number of EPS bearers reached |
| 166 | Requested APN not supported in current RAT and PLMN combination |
| 181 | Invalid PTI value |
| 182 | APN restriction value incompatible with active EPS bearer context |
| 183 | PTI already in use |
| 184 | EPS QoS not accepted |
| 185 | Invalid EPS bearer identity |
| 186 | PTI mismatch |
| 187 | Last PDN disconnection not allowed |
| 188 | PDN type IPv4 only allowed |
| 189 | PDN type IPv6 only allowed |
| 212 | APN restriction |
| 256 | Internal unspecified |
| 257 | Out of memory |
| 258 | Invalid parameters |
| 259 | Data call active |
| 260 | Speech call active |
| 262 | Missing ACM information |
| 263 | Temporary forbidden |
| 264 | Called party is blacklisted |
| 265 | Blacklist is full |
| 266 | No service |
| 267 | Limited service |
| 268 | Client conflict |
| 269 | Dual Service call active |
| 271 | Unknown SIM error |
| 274 | Active client is gone |
| 277 | SIM status failure |
| 278 | Rejected by call control |
| 279 | FDN failed |
| 280 | BDN failed |
| 283 | CCBS possible |
| 284 | Invalid alternate service line |
| 285 | LND overview |
| 287 | MM network failure unspecified |
| 288 | MM no service |
| 289 | MM access class barred |
| 290 | MM RR no resource |
| 291 | MM ME busy |
| 292 | MM unspecified |
| 301 | MMI not registered |
| 303 | Rejected by user |
| 304 | Rejected due to time out |
| 306 | Disconnected due to SIM TK call setup |
| 307 | Pending SIM TK call setup |
| 310 | SIM reset |
| 340 | MM sapi3 release |
| 341 | MM lower layer failure |
| 342 | MM authentification failure |
| 343 | MM PS reject |
| 344 | MM service rejected |
| 345 | MM abort by network |
| 346 | MM timeout |
| 347 | MM detach |
| 348 | MM RR connection release |
| 349 | MM not registered |
| 350 | MM reestablishment failure |
| 351 | Failure due to handover |
| 352 | Link establishment failure |
| 353 | Random access failure |
| 354 | Radio link aborted |
| 355 | Lower layer failure in Layer 1 |
| 356 | Immediate assignment reject |
| 357 | Failure due to paging |
| 358 | Abnormal release unspecified |
| 359 | Abnormal release channel unacceptable |
| 360 | Abnormal release timer expired |
| 361 | Abnormal release no act on radio path |
| 362 | Preemptive release |
| 363 | UTRAN configuration unknown |
| 364 | Handover impossible |
| 365 | Channel mode unacceptable |
| 366 | Frequency not implemented |
| 367 | Originator leaving call group area |
| 368 | Lower layer failure from network |
| 369 | Call already cleared |
| 370 | Semantically incorrect message |
| 371 | Invalid mandatory info |
| 372 | Message type non existing |
| 373 | Message type incompatible in state |
| 374 | Conditional information element error |
| 375 | No cell allocation available |
| 376 | Protocol error unspecified |
| 377 | Normal event |
| 378 | Unspecified |
| 379 | Preemptive release |
| 380 | Congestion |
| 381 | RE establishment reject |
| 382 | Directed sig conn establishment |
| 383 | User inactivity |
| 384 | Lower layer failure downlink |
| 385 | Lower layer failure uplink |
| 386 | Cell barred due to authentication failure |
| 387 | Signalling connection release |
| 388 | CS connection release triggered by MM |
| 389 | RRC connection establishment failure |
| 390 | RRC connection establishment re-ject with redirection |
| 391 | Resource conflict |
| 392 | Layer 2 sequence error |
| 393 | Layer 2 T200 exp N200 plus 1 times |
| 394 | Layer 2 unsolicited DM resp MFES |
| 395 | Layer 2 contention resolution |
| 396 | Layer 2 normal cause |
| 397 | RR connection release due to BAND change (2G) |
| 400 | MM RR connection error while release |
| 500 | User disconnected |
| 510 | Remote user / NW disconnected for call status rather than call proceeding |
| 511 | Remote user / NW disconnected for call status is call proceeding |
| 512 | Request rejected, BCM violation |

#### SS Error Codes

##### SS Network Error Cause

According to 3GPP 24.008, section 4.5 except for internal errors 0 and 255.

| <cause> | <description> |
| --- | --- |
| 0 | MN_MS_INT_SS_ERROR - This is used when the SS operation was unsuccessful due to an MS internal reason |
| 255 | MN_MS_INT_SS_TIME_OUT - This is used when the SS operation was unsuccessful due to a missing answer from the network |
| 1 | MN_UNKNOWN_SUBSCRIBER |
| 9 | MN_ILLEGAL_SUBSCRIBER |
| 10 | MN_BEARER_SERVICE_NOT_ |
| 11 | MN_TELESERVICE_NOT_PROVISIONED |
| 12 | MN_ILLEGAL_EQUIPMENT |
| 13 | SYNONYM MN_CALL_BARRED |
| 16 | MN_ILLEGAL_SS_OPERATION |
| 17 | MN_SS_ERROR_STATUS |
| 18 | MN_SS_NOT_AVAILABLE |
| 19 | MN_SS_SUBSCRIPTION_VIOLATION |
| 20 | MN_SS_INCOMPATIBILITY |
| 21 | MN_FACILITY_NOT_SUPPORTED |
| 27 | MN_ABSENT_SUBSCRIBER |
| 29 | MN_SHORT_TERM_DENIAL |
| 30 | MN_LONG_TERM_DENIAL |
| 34 | MN_SYSTEM_FAILURE |
| 35 | MN_DATA_MISSING |
| 36 | MN_UNEXPECTED_DATA_VALUE |
| 37 | MN_PW_REGISTRATION_FAILURE |
| 38 | MN_NEGATIVE_PW_CHECK |
| 43 | MN_NUMBER_OF_PW_ATTEMPTS_VIOLATION |
| 54 | MN_POS_METHOD_FAILURE |
| 71 | MN_UNKNOWN_ALPHABET |
| 72 | MN_USSD_BUSY |
| 121 | MN_REJECTED_BY_USER |
| 122 | MN_REJECTED_BY_NETWORK |
| 123 | MN_DEFLECTION_TO_SERVER_SUBSCRIBED |
| 124 | MN_SPECIAL_SERVICE_CODE |
| 125 | MN_INVALID_DEFLECTED_NUMBER |
| 126 | MN_MAX_NUMBER_OF_MPTY_PARTICIPANTS_EXCEEDED |
| 127 | MN_RESOURCES_NOT_AVAILABLE |

##### SS Network GSM Cause

According to 3GPP 24.008, section 10.5.4.11 “Cause”. All values greater than 255 are internal values.

| <cause> | <description> |
| --- | --- |
| 1 | MN_UNASSIGNED_NUMBER |
| 3 | MN_NO_ROUTE |
| 6 | MN_CHANNEL_UNACCEPTABLE |
| 8 | MN_OPERATOR_BARRING |
| 16 | MN_NORMAL_CALL_CLEARING |
| 17 | MN_USER_BUSY |
| 18 | MN_NO_USER_RESPONDING |
| 19 | MN_USER_ALERTING_NO_ANSWER |
| 21 | MN_CALL_REJECTED |
| 22 | MN_NUMBER_CHANGED |
| 26 | MN_NON_SELECTED_USER_CLEARING |
| 27 | MN_DESTINATION_OUT_OF_ORDER |
| 28 | MN_INVALID_NUMBER_FORMAT |
| 29 | MN_FACILITY_REJECTED |
| 30 | MN_RESPONSE_TO_STATUS_ENQUIRY |
| 31 | MN_NORMAL_UNSPECIFIED |
| 34 | MN_NO_CIRCUIT_AVAILABLE |
| 38 | MN_NETWORK_OUT_OF_ORDER |
| 41 | MN_TEMPORARY_FAILURE |
| 42 | MN_SWITCH_CONGESTION |
| 43 | MN_ACCESS_INFORMATION_DISCARDED |
| 44 | MN_REQUESTED_CIRCUIT_NOT_AVAILABLE |
| 47 | MN_RESOURCES_UNAVAILABLE |
| 49 | MN_QUALITY_UNAVAILABLE |
| 50 | MN_FACILITY_NOT_SUBSCRIBED |
| 55 | MN_INCOMING_CALLS_BARRED_IN_CUG |
| 57 | MN_BEARER_CAPABILITY_NOT_ALLOWED |
| 58 | MN_BEARER_CAPABILITY_NOT_AVAILABLE |
| 63 | MN_SERVICE_NOT_AVAILABLE |
| 65 | MN_BEARER_SERVICE_NOT_IMPLEMENTED |
| 68 | MN_ACM_GREATER_OR_EQUAL_TO_ACMMAX |
| 69 | MN_FACILITY_NOT_IMPLEMENTED |
| 70 | MN_ONLY_RESTRICTED_DIGITAL |
| 79 | MN_SERVICE_NOT_IMPLEMENTED |
| 81 | MN_INVALID_TI |
| 87 | MN_USER_NOT_IN_CUG |
| 88 | MN_INCOMPATIBLE_DESTINATION |
| 91 | MN_INVALID_TRANSIT_NETWORK |
| 95 | MN_SEMANTICS_INCORRECT |
| 96 | MN_INVALID_MANATORY_INFORMATION |
| 97 | MN_UNKNOWN_MESSAGE_TYPE_1 |
| 98 | MN_UNEXPECTED_MESSAGE_TYPE |
| 99 | MN_UNEXPECTED_IE |
| 100 | MN_CONDITIONAL_IE_ERROR |
| 101 | MN_UNKNOWN_MESSAGE_TYPE_2 |
| 102 | MN_RECOVERY_ON_TIMER_EXPIRY |
| 111 | MN_PROTOCOL_ERROR |
| 127 | MN_INTERWORKING |
| 256 | MN_VOID_CAUSE |
| 257 | MN_OUT_OF_MEMORY |
| 258 | MN_INVALID_PARAM |
| 259 | MN_DATA_CALL_ACTIVE |
| 260 | MN_SPEECH_CALL_ACTIVE |
| 261 | MN_DTMF_REJECTED_DURING_MO_CALL_SETUP |
| 262 | MN_MOC_SETUP_REJ_DUE_TO_MISSING_ACM_INFO_IND |
| 263 | MN_TEMPORARY_FORBIDDEN_CALL_ATTEMPT |
| 264 | MN_CALLED_PARTY_IS_BLACKLISTED |
| 265 | MN_BLACKLIST_IS_FULL_NO_AUTO_CALL_ATTEMPTS |
| 266 | MN_TEMPORARY_FORBIDDEN_CALL_ATTEMPT_NO_SERVICE |
| 267 | MN_TEMPORARY_FORBIDDEN_CALL_ATTEMPT_LIMITED_SERVICE |
| 268 | MN_CLIENT_TEMPORARY_BARRED |
| 269 | MN_DUAL_SERVICE_CALL_ACTIVE |
| 270 | MN_ATC_FCLASS_NOT_SPEECH |
| 271 | MN_SI_UNKNOWN_ERROR |
| 272 | MN_DTMF_REJECTED_DUE_TO_CALL_HELD_OR_MPARTY |
| 273 | MN_CLIENT_NOT_REGISTRATED |
| 274 | MN_ACTIVE_CLIENT_GONE |
| 275 | MN_DTMF_REJECTED_DUE_TO_DATA_TRANSMISSION |
| 276 | MN_NO_APPROPRIATE_DTMF_ENTRY |
| 277 | MN_SIM_STATUS_FAILURE |
| 278 | MN_REJ_BY_CALL_CONTROL |
| 279 | MN_FDN_FAILED |
| 280 | MN_BDN_FAILED |
| 281 | MN_ONLY_ERROR |
| 282 | MN_NOT_IN_SPEECH_CALL |
| 283 | MN_CCBS_POSSIBLE |
| 283 | MN_INVALID_ALS_LINE |
| 284 | MN_INVALID_ALS_LINE |
| 285 | MN_LND_OVERFLOW |
| 286 | MN_DTMF_REJECTED_NO_TCH_AVAILABLE |
| 287 | MN_NW_FAILURE_UNSPECIFIED |
| 288 | MN_MS_NO_SERVICE |
| 289 | MN_MS_ACCESS_CLASS_BARRED |
| 290 | MN_MS_NO_RESOURCE |
| 291 | MN_MS_SERVICE_BUSY |
| 292 | MN_MS_FAILURE_UNSPECIFIED |
| 293 | MN_DTMF_REJECTED_DUE_TO_SUP_TIMER_EXPIRY |
| 300 | MN_SIMTK_SETUP_MODE_NOT_SUPPORTED |
| 301 | MN_MMI_NOT_REGISTERED |
| 302 | MN_SIMTK_SETUP_REJ_DUE_TO_MISSING_ACM_INFO_IND |
| 303 | MN_SIMTK_SETUP_REJECTED_BY_THE_USER |
| 304 | MN_SIMTK_SETUP_REJECTED_TIME_OUT |
| 305 | MN_SIMTK_CALL_CONNECTED |
| 306 | MN_DISCONNECT_DUE_TO_SIMTK_SETUP |
| 307 | MN_SIMTK_SETUP_REJ_DUE_TO_PENDING_SIMTK_SETUP |
| 308 | MN_SIMTK_CALL_CONNECTED_NO_ICON_DISPLAY |
| 309 | MN_PENDING_SIMTK_SETUP |
| 310 | MN_SIMTK_SIM_RESET |
| 311 | MN_SIMTK_DTMF_TRANSMITTED |
| 312 | MN_SIMTK_DTMF_TRANSMITTED_NO_ICON_DISPLAY |
| 313 | MN_USER_DID_NOT_ACCEPT_CALL_SETUP |
| 314 | MN_PROACTIVE_SIM_APPL_TERMINATED_BY_USER |
| 315 | MN_SIMTK_ME_UNABLE_SCREEN_BUSY |
| 316 | MN_SIMTK_ME_UNABLE_NO_SPECIFIC_CAUSE |
| 317 | MN_SIMTK_UNSPECIFIED |
| 318 | MN_SETUP_SS_ERR |
| 319 | MN_SIMTK_NET_UNABLE_NO_SPECIFIC_CAUSE |
| 320 | MN_SIMTK_USSD_TRANSACTION_TERMINATED_BY_USER |
| 330 | MN_PHONEBOOK_NOT_AVAILABLE |
| 331 | MN_ATC_NO_MATCHING_PHONEBOOK_ENTRY |
| 332 | MN_ATC_INVALID_DIALED_NUMBER |
| 333 | MN_ATC_SETUP_TEMPORARY_BLOCKED |
| 334 | MN_ATC_NO_PERMISSION |
| 335 | MN_ATC_INVALID_CALL_SETTINGS |
| 336 | MN_ATC_BLOCKING_CALL_PRESENT |

##### SS Network Reject Cause

According to 3GPP 24.080, 3.6.7, table 3.13.

Tag 128 MN_GENERAL_PROBLEM with Causes

From 3GPP 24.008, table 3.14.

| <cause> | <description> |
| --- | --- |
| 0 | MN_UNRECOGNIZED_COMPONENT |
| 1 | MN_MISTYPED_COMPONENT |
| 2 | MN_BADLY_STRUCTURED_COMPONENT |

Tag 129 MN_INVOKE_PROBLEM with Causes

From 3GPP 24.008, table 3.15.

| <cause> | <description> |
| --- | --- |
| 0 | MN_DUPLICATE_INVOKE_ID |
| 1 | MN_UNRECOGNIZED_OPERATION |
| 2 | MN_MISTYPED_PRO_PARAMETER |
| 3 | MN_RESOURCE_LIMITATION |
| 4 | MN_INITIATING_RELEASE |
| 5 | MN_UNRECOGNIZED_LINKED_ID |
| 6 | MN_LINKED_RESPONSE_UNEXPECTED |
| 7 | MN_UNEXPECTED_LINKED_OPERATION |
| 130 | MN_RETURN_RESULT_PROBLEM |

Tag 130 MN_RETURN_RESULT_PROBLEM with Causes

From 3GPP 24.008, table 3.16.

| <cause> | <description> |
| --- | --- |
| 0 | MN_UNRECOGNIZED_INVOKE_ID |
| 1 | MN_RETURN_RESULT_UNEXPECTED |
| 2 | MN_MISTYPED_RES_PARAMETER |

Tag 131 MN_RETURN_ERROR_PROBLEM with Causes

| <cause> | <description> |
| --- | --- |
| 0 | MN_UNRECOGNIZED_ERROR_INVOKE_ID |
| 1 | MN_RETURN_ERROR_UNEXPECTED |
| 2 | MN_UNRECOGNIZED_ERROR |
| 3 | MN_UNEXPECTED_ERROR |
| 4 | MN_MISTYPED_ERROR_PARAMETER |

### CMS Error Codes

| <err> Code | Meaning |
| --- | --- |
| 1 | Unassigned (unallocated) number |
| 8 | Operator determined barring |
| 10 | Call barred |
| 21 | Short message transfer rejected |
| 27 | Destination out of service |
| 28 | Unidentified subscriber |
| 29 | Facility rejected |
| 30 | Unknown subscriber |
| 38 | Network out of order |
| 41 | Temporary failure |
| 42 | Congestion |
| 47 | Resources unavailable, unspecified |
| 50 | Requested facility not subscribed |
| 69 | Requested facility not implemented |
| 81 | Invalid short message transfer reference value |
| 95 | Invalid message, unspecified |
| 96 | Invalid mandatory information |
| 97 | Message type non-existent or not implemented |
| 98 | Message not compatible with short message protocol state |
| 99 | Information element non-existent or not implemented |
| 111 | Protocol error, unspecified |
| 127 | Interworking, unspecified |
| 128 | Telematic interworking not supported |
| 129 | Short message Type 0 not supported |
| 130 | Cannot replace short message |
| 143 | Unspecified TP-PID error |
| 144 | Data coding scheme (alphabet) not supported |
| 145 | Message class not supported |
| 159 | Unspecified TP-DCS error |
| 160 | Command cannot be executed |
| 161 | Command unsupported |
| 175 | Unspecified TP-Command error |
| 176 | TPDU not supported |
| 192 | SC busy |
| 193 | No SC subscription |
| 194 | SC system failure |
| 195 | Invalid SME address |
| 196 | Destination SME barred |
| 197 | SM Rejected-Duplicate SM |
| 198 | TP-VPF not supported |
| 199 | TP-VP not supported |
| 208 | D0 SIM SMS storage full |
| 209 | No SMS storage capability in SIM |
| 210 | Error in MS |
| 211 | Memory Capacity Exceeded |
| 212 | SIM Application Toolkit Busy |
| 213 | SIM data download error |
| 255 | Unspecified error cause |
| 300 | ME failure |
| 301 | SMS service of ME reserved |
| 302 | Operation not allowed |
| 303 | Operation not supported |
| 304 | Invalid PDU mode parameter |
| 305 | Invalid text mode parameter |
| 310 | SIM not inserted |
| 311 | SIM PIN required |
| 312 | PH-SIM PIN required |
| 313 | SIM failure |
| 314 | SIM busy |
| 315 | SIM wrong |
| 316 | SIM PUK required |
| 317 | SIM PIN2 required |
| 318 | SIM PUK2 required |
| 320 | Memory failure |
| 321 | Invalid memory index |
| 322 | Memory full |
| 330 | SMSC address unknown |
| 331 | no network service |
| 332 | Network timeout |
| 340 | NO +CNMA ACK EXPECTED |
| 500 | Unknown error |
| 606 | ME Busy – CM server request already pending |

### GPRS Error Codes

| <err> Code | Meaning |
| --- | --- |
| Errors related to a failure to Perform an Attach |  |
| 103 | Illegal MS |
| 106 | Illegal ME |
| 107 | GPRS services not allowed |
| 111 | PLMN not allowed |
| 112 | Location area not allowed |
| 113 | Roaming not allowed in this location area |
| Errors related to a failure to Activate a Context |  |
| 132 | Service option not supported |
| 133 | Requested service option not subscribed |
| 134 | Service option temporarily out of order |
| 149 | PDP authentication failure |
| Other GPRS Errors |  |
| 148 | Unspecified GPRS error |
| 150 | Invalid mobile class |

Other values in the range 101 - 150 are reserved for use by GPRS.

### AVMS Error Codes

| <err> Code | Meaning |
| --- | --- |
| 3 | Parameter is out of range; Device Services is not in a good state |
| 24 | Parameters <APN>, <User> or <Pwd> are too long |
| 650 | General error |
| 651 | Communication error |
| 652 | Session in progress |
| 654 | AVMS services are in DEACTIVATED state (see +WDSG) |
| 655 | AVMS services are in PROHIBITED state (see +WDSG) |
| 656 | AVMS services are in TO BE PROVISIONED state (see +WDSG) |

### Error Case Examples

AT commands return specific error codes if parameter verification fails. The following tables enumerate some examples to demonstrate specific error cases.

Generic Error Case Examples

| Error Codes | Corresponding Examples |
| --- | --- |
| +CME ERROR: 3 when execute command is not supported | AT+NVBU AT+KGSN AT+HWREV AT+KBND AT+KSRAT AT+KSREP AT+WMANTSEL AT+KGPIOCFG AT+KGPIO AT+WMIOTEST AT+KTEMPMON AT+KADC AT+KCELL AT+KSLEEP AT+KRIC AT&C AT+GST AT+HBHV AT+KSIMSEL AT*PSSTKI AT+KPCMCFG |
| +CME ERROR: 3 when action command is not supported | AT+WMAUDIOLOOP AT+VIP AT+VGT AT+KVGT AT+VGR AT+KVGR AT+CLVL AT+KECHO AT+KNOISE AT+KST AT+KPC AT+CALM AT+CRSL AT+KSRAP AT+CODECINFO AT+WIMEI AT+CMEC |

Non-Generic Error Case Examples

| Error Codes | Corresponding Examples |
| --- | --- |
| +CME ERROR: 3 instead of ERROR is returned for some cases in some SREG or BASIC AT commands. This is for backward compatibility reason. | ATB100 +CME ERROR: 3 ATB=? ERROR ATB? ERROR ATB=0 ERROR ATB +CME ERROR: 3 |
| +CME ERROR: 16 (incorrect password) instead of +CME ERROR: 18 (PUK2 required) is returned when PIN2 is blocked in AT+CPIN2 write command; while +CME ERROR: 12 (PUK required) is returned when PIN is blocked in AT+CPIN write command. | AT+CPIN2="9876" +CME ERROR: 16 AT+CPIN2="9876" +CME ERROR: 16 AT+CPIN2="9876" +CME ERROR: 16 AT+CPIN2="22222222","5678" OK AT+CPIN="5678" +CME ERROR: 16 AT+CPIN="5678" +CME ERROR: 16 AT+CPIN="5678" +CME ERROR: 12 AT+CPIN? +CPIN: SIM PUK OK AT+CPIN="11111111","1234" OK |
| +CME ERROR: 18 (SIM PUK2 REQUIRED) instead of +CME ERROR: 50 (INCORRECT PARAMETERS) is returned for +CPIN2 when PUK2 is required while only PIN2 was inputted, +CME ERROR: 12 (SIM PUK REQUIRED) instead of +CME ERROR: 50 (INCORRECT PARAMETERS) is returned for +CPIN when PUK is required while only PIN was inputted, | AT+CPIN2="9876" +CME ERROR: 16 AT+CPIN2="9876" +CME ERROR: 16 AT+CPIN2="9876" +CME ERROR: 16 AT+CPIN2="9876" // Incorrect PIN2 – 4th time +CME ERROR: 18 AT+CPIN2="22222222","5678" OK AT+CPIN="5678" +CME ERROR: 16 AT+CPIN="5678" +CME ERROR: 16 AT+CPIN="5678" +CME ERROR: 12 AT+CPIN="5678" // Incorrect PIN1 – 4th time +CME ERROR: 12 AT+CPIN? +CPIN: SIM PUK OK AT+CPIN="11111111","1234" OK |

Internet Error Case Examples

| Error Codes | Corresponding Examples |
| --- | --- |
| +CME ERROR: 907 Generic error/Unsupported read command | AT+KUDPDEL? AT+KUDPCLOSE? AT+KUDPRCV? AT+KUDPSND? AT+KTCPSND? AT+KTCPRCV? AT+KTCPCNX? AT+KTCPCLOSE? AT+KTCPDEL? |
| +CME ERROR: 912 No more sessions can be used | Create a UDP client session repeatedly until 32 sessions are created: AT+KUDPCFG=1,0,1033,,"10.10.10.10" Then try to create a TCP server session (33rd session) AT+KTCPCFG=1,1,,80 |
| +CME ERROR: 916 A parameter has an invalid range of values | AT+KTCPSND=1,0 |
|  | AT+KTCPRCV=1,0 |
|  | AT+KUDPSND=1,"116.66.221.43",5043,0 |
|  | AT+KUDPRCV=1,0 |

## Commands without Pin Code Requirement

Most AT commands are rejected (i.e. an error is returned to the DTE) if the valid PIN code has not been entered.

Commands which can be sent without the PIN code include:

ATD (emergency calls)

AT+CPIN

ATI

AT+CGMI, AT+GMI

AT+CGMM, AT+GMM

AT+CGMR, AT+GMR

AT+CGSN, AT+GSN

AT+CPAS

AT+CIND

AT+CMEE

AT+IPR

ATE, ATV, ATS, ATZ

AT&F

AT+CBST

This list may be modified in case of special needs from the customer. Contact Sierra Wireless to request modifications. Note that some commands may also require the PIN2 code.

## TCP Commands Examples

### Client Mode

| AT&K3 | Hardware flow control activation |
| --- | --- |
| OK |  |
| AT+KCNXCFG=1,"GPRS","APN","log","password","0.0.0.0","0.0.0.0","0.0.0.0" | Set GPRS parameters (APN, login, password) |
| OK |  |
| AT+KTCPCFG=1,0,"www.google.com",80 | Set IP address and port number |
| +KTCPCFG: 1 | Returns session ID 1 |
| OK |  |
| AT+KTCPCNX=1 | Initiate the connection |
| OK |  |
| AT+KTCPSND=1,18 CONNECT …Data send… | Send data with the EOF string at the end "GET / HTTP / 1.0 |
| OK | --EOF--Pattern" |
| +KTCP_DATA: 1,1380 |  |
| AT+KTCPRCV=1, 1380 |  |
| CONNECT |  |
| HTTP/1.0 200 OK |  |
| Cache-Control: private, max-age=0 | Read data |
| … a lot of data… |  |
| --EOF--Pattern-- |  |
| OK |  |
| +KTCP_DATA: 1,1380 | +KTCP_DATA notification |
| AT+KTCPRCV=1,1380 | Read received data |
| CONNECT |  |
| er{padding-bottom:7px !important}#gbar,#guser{font- |  |
| … a lot of data… |  |
| --EOF--Pattern-- |  |
| OK |  |
| +KTCP_DATA: 1,1380 |  |
| AT+KTCPCLOSE=1,1 | Close session 1 |
| OK |  |
| AT+KTCPDEL=1 | Delete session 1 |
| OK |  |
| AT+KTCPCFG? | No session is available |
| OK |  |

### Server Mode

In this simple example we emulate a daytime server. This server listens to port 13 and for each connection it returns the date.

| AT&K3 | Hardware flow control activation |
| --- | --- |
| OK |  |
| AT+KCNXCFG=1,"GPRS","APN","log","password","0.0.0.0","0.0.0.0","0.0.0.0" | Set GPRS parameters (APN, login, password) |
| OK |  |
| AT+KTCPCFG=1,1,,13 | Set TCP listener and port number |
| +KTCPCFG: 1 | Returns session ID 1 |
| OK |  |
| AT+KTCPCNX=1 | Initiate the server |
| OK |  |
| AT+KCGPADDR +KCGPADDR: 0,"10.35.125.89" | Get the IP address to initiate a connection request with a client |
| OK |  |
| +KTCP_SRVREQ: 1,2 | A client requests a connection (session ID 2) |
| AT+KTCPSND=2,15 |  |
| CONNECT |  |
| …Date and time… | Data is sent to the client read |
| OK |  |
| +KTCP_SRVREQ: 1,3 | Another client requests a connection (session ID 3); child mode for session 3 |
| +KTCP_NOTIF: 2, 4 | Client (session 2) closes the connection |
| AT+KTCPSND=3,15 |  |
| CONNECT |  |
| …Date and time… | Data is sent to the client |
| OK |  |
| AT+KTCPCLOSE=3,1 OK | Close client session 3 and then session 3 is deleted automatically (child mode for session 3) |
| AT+KTCPCLOSE=1,1 | Close server session 1 |
| OK |  |
| AT+KTCPDEL=1 | Delete session 1 |
| OK |  |

### Polling for the Status of a Socket

| AT&K3 | Hardware flow control activation |
| --- | --- |
| OK |  |
| AT+KCNXCFG=1,"GPRS","APN","log","password","0.0.0.0","0.0.0.0","0.0.0.0" | Set GPRS parameters (APN, login, password) |
| OK |  |
| AT+KTCPCFG=1,0,"www.google.com",80 | Set TCP Server address and port number |
| +KTCPCFG: 1 | Returns session ID 1 |
| OK |  |
| AT+KURCCFG="TCP",0 | Disable TCP unsolicited messages |
| OK |  |
| AT+KTCPCNX=1 | Initiate connection, use session 1 |
| OK |  |
| AT+KTCPSTAT=1 | Poll the connection status |
| +KTCPSTAT: 3,-1,0,0 | Connection is UP |
| OK |  |
| AT+KTCPSND=1,3000 CONNECT | Send data on socket 1 for 3000 bytes or less. |
| <…Data send…> | Data can be sent after CONNECT |
| OK | Send the EOF string to finish. The EOF parttern should be defined using the +KPATTERN command. |
| AT+KTCPSTAT=1 | Poll the connection status |
| +KTCPSTAT : 3,-1,1234,0 OK | Connection is up, with 1234 unsent bytes |
| AT+KTCPSTAT=1 | Poll the connection status |
| +KTCPSTAT : 3,-1,100,0 OK | Connection is up, with 100 unsent bytes |
| AT+KTCPSTAT=1 | Poll the connection status |
| +KTCPSTAT : 3,-1,0,0 | Connection is up, all bytes have been sent |
| OK |  |
| AT+KTCPSTAT=1 | Poll the connection status |
| +KTCPSTAT : 3,-1,0,320 OK | Connection is up with 320 bytes available for reading |
| AT+KTCPRCV=1,320 | Read 320 bytes on socket 1 |
| CONNECT |  |
| <… a lot of data…> | Data are sent after CONNECT |
| --EOF--Pattern-- |  |
| OK |  |
| AT+KTCPCLOSE=1,1 | Close session 1 |
| OK |  |
| AT+KTCPDEL=1 | Delete session 1 |
| OK |  |

### End to End TCP Connection

| AT&K3 | Hardware flow control activation |
| --- | --- |
| OK |  |
| AT+KCNXCFG=1,"GPRS","APN","log","password","0.0.0.0","0.0.0.0","0.0.0.0" | Set GPRS parameters (APN, login, password) |
| OK |  |
| AT+KTCPCFG=1,0,"www.google.com",80 | Set the TCP server address and port number |
| +KTCPCFG: 1 | Returns session ID 1 |
| OK |  |
| AT+KTCPSTART=1 | Initiate connection, use session 1 |
| CONNECT …Data sent……Data received……Data sent… | Message CONNECT: connection to server is established, data can be sent |
| …Data sent……Data received……Data sent… |  |
| +++ | Use +++ to enter command mode |
| OK |  |
| ATO1 CONNECT | Use ATO<session_id> to switch back to data mode |
| …Data sent……Data received……Data sent… |  |
| …Data sent……Data received……Data sent… |  |
| OK |  |
|  | Toggle DTR (if using AT&D1 or AT&D2 configuration) to enter command mode |
| AT+KTCPCLOSE=1,1 | Close the session |
| OK |  |
| AT+KTCPDEL=1 | Delete the configured session |
| OK |  |

### Error Case for End to End TCP Connection

| AT+KTCPSTART=1 | Try to initiate connection |
| --- | --- |
| NO CARRIER +KTCP_NOTIF: 1,<tcp_notif> | Connection failed, see the value of <tcp_notif> |
| AT+KTCPSTART=1 | Initiate connection |
| CONNECT |  |
| …Data sent……Data received……Data sent… | Exchange some data |
| …Data sent……Data received……Data sent… |  |
| NO CARRIER |  |
| +KTCP_NOTIF: 1,<tcp_notif> | An error occurs during connection (network lost, server closed, etc.) |

### Use Cases for AT+KTCPACKINFO and <URC-ENDTCP-enable> Option

This section describes the behavior of AT+KTCPACKINFO when the <URC-ENDTCP> option is used with AT+KTCPCFG.

#### <URC-ENDTCP-enable> is Disabled (default setting)

| AT+KCNXCFG=1,"GPRS","CMNET" |  |
| --- | --- |
| OK |  |
| AT+KTCPCFG=1,0,"202.170.131.76",2000 |  |
| +KTCPCFG: 1 |  |
| OK |  |
| AT+KTCPCFG? |  |
| +KTCPCFG: 1,0,0,0,,"202.170.131.76",2000,,0,0 | <URC-ENDTCP-enable> is disabled |
| OK |  |
| AT+KTCPCNX=1 | Connect to TCP server |
| OK |  |
| AT+KTCPSND=1,10 | Use command to send 10 bytes |
| CONNECT |  |
| 0123456789--EOF--Pattern-- | Write to serial |
| OK |  |
|  | The URC “+KTCP_ACK” is not displayed |
| AT+KTCPACKINFO=1 |  |
| +CME ERROR: operation not allowed | Error is returned because <URC-ENDTCP-enable> is disabled |

#### <URC-ENDTCP-enable> is Enabled

| AT+KCNXCFG=1,"GPRS","CMNET" |  |
| --- | --- |
| OK |  |
| AT+KTCPCFG=1,0,"202.170.131.76",2000,,,1 +KTCPCFG: 1 | Set <URC-ENDTCP-enable> to 1, enable URC “+KTCP_ACK” |
| OK |  |
| AT+KTCPCFG? |  |
| +KTCPCFG: 1,0,0,0,,"202.170.131.76",2000,,0,1 | <URC-ENDTCP-enable> is enabled |
| OK |  |
| AT+KTCPCNX=1 | Connect to TCP server |
| OK |  |
| AT+KTCPSND=1,10 | Receive 10 bytes |
| CONNECT | Connect to TCO server |
| 0123456789--EOF--Pattern-- | Write to serial |
| OK |  |
| +KTCP_ACK: 1, 1 | After a short time, URC “+KTCP_ACK” states that the latest TCP data has arrived on the remote side |
| AT+KTCPACKINFO=1 +KTCPACKINFO: 1, 1 | Poll the status of the latest TCP data |
| OK |  |
| AT+KTCPSND=1,1000 | Send 1000 bytes |
| CONNECT |  |
| <1000bytes and --EOF--Pattern | Write to serial |
| OK |  |
|  | URC “+KTCP_ACK” not got yet |
| AT+KTCPACKINFO=1 | Poll the status of the latest TCP data |
| +KTCPACKINFO: 1, 2 | The status of the latest TCP data is unknown |
| OK |  |
|  | Since the “OK” of the latest “+KTCPSND”, 64 seconds has elapsed |
| +KTCP_ACK: 1, 0 | URC “+KTCP_ACK” indicates that data has not arrived on the remote side yet. The network may not be good. |
| AT+KTCPACKINFO=1 | Poll the status of the latest TCP data |
| +KTCPACKINFO: 1, 0 OK | The status of the latest TCP data is “failure”: not all data has been received by the remote side |

## UDP Commands Examples

### Client Mode

| AT&K3 | Hardware flow control activation |
| --- | --- |
| OK |  |
| AT+KCNXCFG=1,"GPRS","APN","log","password",,, OK | Set GPRS parameters (APN, login, password) |
| AT+KUDPCFG=1,0 +KUDPCFG: 1 OK | Create a new UDP socket (returned session 1) with the parameters associated to the connection profile ID number 0 |
| AT+KUDPSND= 1,"82.234.17.52",32,18 |  |
| CONNECT | Send UDP data after “CONNECT” |
| <…Data sent…> |  |
| --EOF--Pattern-- |  |
| OK |  |
| +KUDP_DATA: 1,35 | Received notification that indicates the presence of 35 bytes in the socket |
| AT+KUDPRCV=1, 35 | Try to read 35 bytes from session 1 |
| CONNECT |  |
| This is a simple UDP Protocol test |  |
| --EOF--Pattern-- |  |
| OK |  |
| +KUDP_RCV: "82.234.17.52",32 |  |
| +KUDP_DATA: 1,35 | Received notification that indicates the presence of 35 bytes in the socket |
| AT+KUDPRCV=1, 18 CONNECT | Try to read 18 bytes from session 1 |
| This is a simple |  |
| --EOF--Pattern-- |  |
| OK |  |
| +KUDP_DATA_MISSED: 1,17 | There are 17 unread bytes left and missed in the UDP socket |
| AT+KUDPCLOSE=1 OK | Close the UDP session |
| AT+KUDPCFG? | No sessions are available anymore |
| OK |  |

### Server Mode

| AT&K3 | Hardware flow control activation |
| --- | --- |
| OK |  |
| AT+KCNXCFG=1,"GPRS","APN","log","password","0.0.0.0","0.0.0.0","0.0.0.0" | Set GPRS parameters (APN, login, password) |
| OK |  |
| AT+KUDPCFG=1,1,3000 | Set UDP listener (port 3000). Initiate the server. Session ID is 1 |
| +KUDPCFG: 1 |  |
| OK |  |
| AT+KUDPCFG? | Check if the server is initiated |
| +KUDPCFG: 1,0,1,3000 |  |
| OK |  |
| AT+KCGPADDR | Get local IP address |
| +KCGPADDR: 0, "192.168.0.71" |  |
| OK |  |
| +KUDP_DATA: 1,9 | Data comes in from some client |
| AT+KUDPRCV=1,9 | Read received data |
| CONNECT |  |
| DATA TEST--EOF--Pattern-- |  |
| OK |  |
| +KUDP_RCV: "10.10.10.5",1111 | This data was sent from "10.10.10.5" (port:1111) |
| AT+KUDPSND=1,"10.10.10.5”,3100,18 CONNECT <18 bytes data ended with "--EOF--Pattern--"> | Send 18 bytes to a remote server (port:3100) |
| OK |  |
| AT+KUDPCLOSE=1 OK | Close the UDP server. The session is also deleted at the same time |
| AT+KUDPCFG? | No sessions are available anymore |
| OK |  |

### Use Cases for KTCP_DATA and KUDP_DATA

#### KTCP_DATA and KUDP_DATA without Data Auto Retrieval – Client Mode

| AT+KCNXCFG=1,"GPRS","CMNET" |  |
| --- | --- |
| OK |  |
| AT+KTCPCFG=1,0,"202.170.131.76",2000 |  |
| +KTCPCFG: 1 |  |
| OK |  |
| AT+KTCPCNX=1 | Connect to TCP server |
| OK |  |
| +KTCP_DATA: 1,10 | 10 bytes have arrived |
| AT+KTCPRCV=1,10 CONNECT | Receive the 10 bytes that arrived |
| 0123456789--EOF--Pattern-- |  |
| OK |  |
| AT+KUDPCFG=1,0 | Open a UDP socket |
| +KUDPCFG: 2 |  |
| OK |  |
| +KUDP_DATA: 2,8 | 8 bytes have arrived |
| AT+KUDPRCV=2,8 | Read the data |
| CONNECT |  |
| 01234567--EOF--Pattern-- |  |
| OK |  |
| +KUDP_RCV: "202.170.131.76",2001 |  |

#### KTCP_DATA and KUDP_DATA without Data Auto Retrieval – Server Mode

| AT+KTCPCFG=1,1,,13 | Configure a TCP server socket |
| --- | --- |
| +KTCPCFG: 1 |  |
| OK |  |
| AT+KTCPCNX=1 | Open the listen port |
| OK |  |
| AT+KCGPADDR |  |
| +KCGPADDR: 0,“10.35.125.89” |  |
| OK |  |
| +KTCP_SRVREQ: 1,2 | Session 2 is set |
| +KTCP_SRVREQ: 1,3 | Session 3 is set |
| +KTCP_DATA: 2,10 | 10 bytes have arrived at session 2 |
| +KTCP_DATA: 3,8 | 8 bytes have arrived at session 3 |
| AT+KTCPRCV=2,10 CONNECT | Receive the 10 bytes in session 2 |
| 0123456789--EOF--Pattern-- |  |
| OK |  |
| AT+KTCPRCV=3,8 CONNECT | Receive the 8 bytes in session 3 |
| 01234567--EOF--Pattern-- |  |
| OK |  |
| AT+KUDPCFG=1,1,3000 | Open a UDP socket in server mode |
| +KUDPCFG: 4 |  |
| OK |  |
| +KUDP_DATA: 4,8 | 8 bytes have arrived |
| AT+KUDPRCV=4,8 | Receive the 8 bytes |
| CONNECT |  |
| 01234567--EOF--Pattern-- |  |
| OK |  |
| +KUDP_RCV: "202.170.131.76",2001 |  |

#### KTCP_DATA and KUDP_DATA with Data Auto Retrieval – Client Mode

| AT+KCNXCFG=1,"GPRS","CMNET" |  |
| --- | --- |
| OK |  |
| AT+KTCPCFG=0,0,"202.170.131.76",2000,,1 +KTCPCFG: 1 | When <data_mode> = 1, data will be received by the URC “+KTCP_DATA |
| OK |  |
| AT+KTCPCNX=1 | Connect to TCP server |
| OK |  |
| +KTCP_DATA: 1,10,0123456789 | 10 bytes have arrived. The data are presented in the URC directly |
| AT+KUDPCFG=0,0,3000,1 +KUDPCFG: 2 | When <data_mode> = 1, data will be received by the URC “+KUDP_DATA |
| OK |  |
| +KUDP_DATA: 2,8,"202.170.131.76",2001,01234567 | 8 bytes have arrived. The data are presented in the URC directly |

#### KTCP_DATA and KUDP_DATA with Data Auto Retrieval – Server Mode

| AT+KTCPCFG=1,1,,13,1 +KTCPCFG: 1 OK | When <data_mode> = 1, all child connections will display data in URC mode. Data will be received by the URC “+KTCP_DATA:” |
| --- | --- |
| AT+KTCPCNX=1 | Open the listen port |
| OK |  |
| AT+KCGPADDR |  |
| +KCGPADDR: 1,"10.35.125.89" |  |
| OK |  |
| +KTCP_SRVREQ: 1,2 |  |
| +KTCP_SRVREQ: 1,3 |  |
| +KTCP_DATA: 2,10,0123456789 | 10 bytes have arrived in session 2 |
| +KTCP_DATA: 3,8,01234567 | 8 bytes have arrived in session 3 Data are presented in the URC directly |
| AT+KUDPCFG=1,1,3000,1 +KUDPCFG: 4 OK | Open a UDP socket in server mode. Data will be received by the URC “+KUDP_DATA:” |
| +KUDP_DATA: 4,8,"202.170.131.76",2001,01234567 | 8 bytes have arrived. Data are presented in the URC directly |

## Switch Data/Command Mode DTR +++ ATO Behavior Table

When the module is in data mode and the connection encounters an error, NO CARRIER terminal response is shown and the module is switched back to command mode.

The following table shows the behavior when trying to switch mode (when connection is running properly):

Case 1:	"+++" is used to switch from data mode to command mode, and the service is suspended.

Case 2:	If AT&D1 is set, "DTR drop" is used to switch from data mode to command mode, but the service is suspended.

Case 3:	If AT&D2 is set, "DTR drop" is used to switch from data mode to command mode, and the service is stopped.

Case 4:	If AT&D0 is set, "DTR drop" has no any impact on the mode switch.

Case 5:	ATO[n] is used to switch from command mode to data mode.

|  | Case1/Case5 +++/ATO[n] | Case2/Case5 DTR1/ATO[n] | Case3/Case5 DTR2/ATO[n] | Case4/Case5 DTR0 |
| --- | --- | --- | --- | --- |
| TCP/UDP: +KTCPSND: Send data +KTCPRCV: Receive data +KUDPSND: Send data +KUDPRCV: Receive data +KTCPSTART: Direct data flow | OK/CONNECT | OK/CONNECT | NO CARRIER/NO CARRIER (disconnect) | NO IMPACT |
| FTP: +KFTPRCV: Download FTP files +KFTPSND: Upload FTP files | OK/NO CARRIER (disconnect) | OK/NO CARRIER (disconnect) | NO CARRIER/NO CARRIER (disconnect) | NO IMPACT |
| HTTP: +KHTTPGET: Get information +KHTTPHEAD: Get head of information +KHTTPPOST: Send data +KHTTPHEADER: Set the HTTP request header | OK/NO CARRIER (disconnect) | OK/NO CARRIER (disconnect) | NO CARRIER/NO CARRIER (disconnect) | NO IMPACT |
| HTTPS: +KHTTPSGET: Get information +KHTTPSHEAD: Get head of information +KHTTPSPOST: Send data +KHTTPSHEADER: Set the HTTP request header | OK/NO CARRIER (disconnect) | OK/NO CARRIER (disconnect) | NO CARRIER/NO CARRIER (disconnect) | NO IMPACT |
| SSL: +KCERTSTORE: Store root CA +KPRIVKSTORE: Store private key | OK/NO CARRIER (abort) | OK/NO CARRIER (abort) | NO CARRIER/NO CARRIER (abort) | NO IMPACT |

Note the following:

When a data session is suspended, it can be resumed by ATO<session_id>.

For IP AT commands that supports <ndata>:

Data sessions completed with <ndata> are not suspended.

Data sessions with unspecified <ndata> (omitted in AT command) are suspended.

Data sessions with <ndata> but incomplete (less than <ndata> are sent/received) are suspended.

For IP AT commands that do not support <ndata> (e.g. +KTCPSTART), data sessions are suspended.

Only one data session can be suspended per <session_id>; the latest suspension clears the last suspension status. For example, the suspension of +KTCPRCV clears the suspension status of +KTCPSND such that ATO resumes +KTCPRCV instead of +KTCPSND.