# Purpose

# The following is a quick guide on how to setup a cmux client on Linux using open source tools. It has been written for Ubuntu, but could be easily ported on any Linux system.

# Linux GSM 0710 tty multiplexor:

You will need this driver to be installed to support CMUX/DLCs, etc.

# 1. Retrieve ubuntu current kernel version

# $ uname -r

# 3.8.13-generic

# 2. Download corresponding vanilla kernel and uncompress it

# $ wget https://www.kernel.org/pub/linux/kernel/v3.0/linux-3.8.13.tar.bz2

# $ tar -xvf linux-3.8.13.tar.bz2

# 3. Create n_gsm directory and retrieve native linux n_gsmtar -xvfn_gsm

# $ mkdir n_gsm

# $ cd n_gsm

# $ cp ../linux-3.8.13/drivers/tty/n_gsm.c .

# 4. Create Makefile (example below): This file is also attached.

# $ cat Makefile

# obj-m := n_gsm.o

# KDIR := /lib/modules/$(shell uname -r)/build

# PWD := $(shell pwd)

# OUTPUTDIR := /lib/modules/$(shell uname -r)/kernel/drivers/tty

# all: clean

# $(MAKE) -C $(KDIR) M=$(PWD) modules

# install: all

# mkdir -p $(OUTPUTDIR)

# cp -f n_gsm.ko $(OUTPUTDIR)

# depmod

# clean:

# rm -rf *.o *~ core .depend .*.cmd *.ko *.mod.c .tmp_versions Module.* modules.order

# 5. Compile n_gsm.c (GSM 0710 tty multiplexor), install and load the module

# $ make

# $ sudo make install

# $ sudo modprobe n_gsm

# CMUX client

# 1. Download cmux client directory and application

# $ cd ..

# $ wget https://github.com/Rtone/cmux/archive/master.zip

# $ unzip master.zip

# $ cd cmux

# 2. Patch cmux.c (here for HL77xx support)

# - Serial Port is /dev/ttyUSB0

# #define SERIAL_PORT    "/dev/ttyUSB0"

# - MTU is 120 bytes

# #define MTU         120

# - Replace specific commands for HL77xx

# if (send_at_command(serial_fd, "AT\r") == -1)

# warnx("AT: bad response");

# if (send_at_command(serial_fd, "AT+GMM\r") == -1)

# warnx("AT+GMM: bad response");

# sprintf(atcommand, "AT+CMUX=0,0,5,%d,5,3,15,5\r", MTU);

# if (send_at_command(serial_fd, atcommand) == -1)

# errx(EXIT_FAILURE, "Cannot enable modem CMUX");

# - Remove the delay to avoid timeout on CMUX activation (for all module support)

# /* use n_gsm line discipline */

# //sleep(2);

# - It is also possible to define the DLC number as well as linux interface (node) name (default is 4 nodes named ttyGSMx x from 1 up to 4)

# - You can use BASENAME_NODES to modify the ttyGSMx to any other name you prefer for your system, eg: ttySerial instead.

# 3. Compile and start cmux client with superuser privilege

# $ make

# $ sudo ./cmux  or sudo ./cmux-master/cmux)​

# At this point you should see four more serial ports : ttyGSM0, ttyGSM1, ttyGSM2 and ttyGSM3 in the /dev list.

# You can now use these ports as regular serial ports and may use one port for AT, and another for PPP.​

​Makefile