[No FH number]

[R01.00]

[December 7th, 2016]

Dummy Commands

# Interface Specification

Important Notice

Due to the nature of wireless communications, transmission and reception of data can never be guaranteed. Data may be delayed, corrupted (i.e., have errors) or be totally lost. Although significant delays or losses of data are rare when wireless devices such as the Sierra Wireless modem are used in a normal manner with a well-constructed network, the Sierra Wireless modem should not be used in situations where failure to transmit or receive data could result in damage of any kind to the user or any other party, including but not limited to personal injury, death, or loss of property. Sierra Wireless accepts no responsibility for damages of any kind resulting from delays or errors in data transmitted or received using the Sierra Wireless modem, or for failure of the Sierra Wireless modem to transmit or receive such data.

Safety and Hazards

Do not operate the Sierra Wireless modem in areas where cellular modems are not advised without proper device certifications. These areas include environments where cellular radio can interfere such as explosive atmospheres, medical equipment, or any other equipment which may be susceptible to any form of radio interference. The Sierra Wireless modem can transmit signals that could interfere with this equipment. Do not operate the Sierra Wireless modem in any aircraft, whether the aircraft is on the ground or in flight. In aircraft, the Sierra Wireless modem MUST BE POWERED OFF. When operating, the Sierra Wireless modem can transmit signals that could interfere with various onboard systems.

Some airlines may permit the use of cellular phones while the aircraft is on the ground and the door is open. Sierra Wireless modems may be used at this time.

The driver or operator of any vehicle should not operate the Sierra Wireless modem while in control of a vehicle. Doing so will detract from the driver or operator’s control and operation of that vehicle. In some states and provinces, operating such communications devices while in control of a vehicle is an offence.

Limitations of Liability

This manual is provided “as is”.  Sierra Wireless makes no warranties of any kind, either expressed or implied, including any implied warranties of merchantability, fitness for a particular purpose, or noninfringement.  The recipient of the manual shall endorse all risks arising from its use.

The information in this manual is subject to change without notice and does not represent a commitment on the part of Sierra Wireless. SIERRA WIRELESS AND ITS AFFILIATES SPECIFICALLY DISCLAIM LIABILITY FOR ANY AND ALL DIRECT, INDIRECT, SPECIAL, GENERAL, INCIDENTAL, CONSEQUENTIAL, PUNITIVE OR EXEMPLARY DAMAGES INCLUDING, BUT NOT LIMITED TO, LOSS OF PROFITS OR REVENUE OR ANTICIPATED PROFITS OR REVENUE ARISING OUT OF THE USE OR INABILITY TO USE ANY SIERRA WIRELESS PRODUCT, EVEN IF SIERRA WIRELESS AND/OR ITS AFFILIATES HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES OR THEY ARE FORESEEABLE OR FOR CLAIMS BY ANY THIRD PARTY.

Notwithstanding the foregoing, in no event shall Sierra Wireless and/or its affiliates aggregate liability arising under or in connection with the Sierra Wireless product, regardless of the number of events, occurrences, or claims giving rise to liability, be in excess of the price paid by the purchaser for the Sierra Wireless product.

Customer understands that Sierra Wireless is not providing cellular or GPS (including A-GPS) services. These services are provided by a third party and should be purchased directly by the Customer.

SPECIFIC DISCLAIMERS OF LIABILITY: CUSTOMER RECOGNIZES AND ACKNOWLEDGES SIERRA WIRELESS IS NOT RESPONSIBLE FOR AND SHALL NOT BE HELD LIABLE FOR ANY DEFECT OR DEFICIENCY OF ANY KIND OF CELLULAR OR GPS (INCLUDING A-GPS) SERVICES.

Patents

This product may contain technology developed by or for Sierra Wireless Inc.

This product includes technology licensed from QUALCOMM®.

This product is manufactured or sold by Sierra Wireless Inc. or its affiliates under one or more patents licensed from InterDigital Group and MMP Portfolio Licensing.

Copyright

© 2013 Sierra Wireless. All rights reserved.

Trademarks

Sierra Wireless®, AirPrime®, AirLink®, AirVantage® and the Sierra Wireless logo are registered trademarks of Sierra Wireless.

Watcher® is a registered trademark of Netgear, Inc., used under license.

Windows® and Windows Vista® are registered trademarks of Microsoft Corporation.

Macintosh® and Mac OS X® are registered trademarks of Apple Inc., registered in the U.S. and other countries.

QUALCOMM® is a registered trademark of QUALCOMM Incorporated.  Used under license.

Other trademarks are the property of their respective owners.

Contact Information

| Sales Desk: | Phone: | 1-604-232-1488 |
| --- | --- | --- |
|  | Hours: | 8:00 AM to 5:00 PM Pacific Time |
|  | E-mail: | sales@sierrawireless.com |
| Post: | Sierra Wireless 13811 Wireless Way Richmond, BC Canada V6V 3A4 |  |
| Technical Support: | support@sierrawireless.com |  |
| RMA Support: | repairs@sierrawireless.com |  |
| Fax: | 1-604-231-1109 |  |
| Web: | www.sierrawireless.com |  |

Consult our website for up-to-date product descriptions, documentation, application notes, firmware upgrades, troubleshooting tips, and press releases: www.sierrawireless.com

Document History

| Version | Date | Updates |
| --- | --- | --- |
| R01.00 | December 7th, 2016 | Creation (pcarre) |

Contents

1.	Group of AT Commands	6

1.1.	S0 Command: Set Number of Rings before Automatic Call Answering	6

1.2.	S1 Command: Ring Count	6

1.3.	S3 Command: Command Line Termination Character	7

1.4.	S4 Command: Set Response Formatting Character	7

1.5.	S5 Command: Write Command Line Editing Character	8

1.6.	S6 Command: Pause before Blind Dialing	8

1.7.	S8 Command: Comma Dial Modifier Time	8

1.8.	S10 Command: Automatic Disconnect Delay	9

1.9.	S11 Command: DTMF Dialing Speed	9

1.10.	B Command: Data Rate Selection	10

1.11.	\N Command: Data Transmission Mode	10

1.12.	L Command: Monitor Speaker Loudness	10

1.13.	M Command: Monitor Speaker Mode	11

1.14.	N Command: Negotiate Handshake Option	11

2.	Development notes	12

2.1.	References	12

2.2.	Applicability	12

# V25ter AT Commands

## S0 Command: Set Number of Rings before Automatic Call Answering

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Read command Syntax ATS0? | Response <n> OK |
| Write command Syntax ATS0=<n> | Response OK Parameters <n> 0 – 255 Number of rings |
| Reference V.25ter | Notes The responses of this command are compliant with the recommendation but this command has no effect |

## S1 Command: Ring Count

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Read command Syntax ATS1? | Response 0 OK |
| Reference | Notes The default value “0” is returned, but it has no meaning. |

## S3 Command: Command Line Termination Character

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Read command Syntax ATS3? | Response <n> OK |
| Write command Syntax ATS3=<n> | Response OK Parameters <n> 13 command line termination character<CR>: carriage return |
| Reference V.25Ter | Notes This parameter determines the character recognized by TA to terminate an incoming command line (13 = <CR> by default); it cannot be changed. |

## S4 Command: Set Response Formatting Character

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Read command Syntax ATS4? | Response <n> OK |
| Write command Syntax ATS4=<n> | Response OK Parameters <n> 10 Response formatting character <LF>: line feed |
| Reference V.25Ter | Notes This parameter determines the character recognized by TA to terminate answer line (10 = <LF> by default); it cannot be changed. |

## S5 Command: Write Command Line Editing Character

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Read command Syntax ATS5? | Response <n> OK |
| Write command Syntax ATS5=<n> | Response OK Parameters <n> 8 Deletion character (backspace) |
| Reference V.25Ter | Notes This parameter determines the character recognized by TA to delete the previous character; it cannot be changed; |

## S6 Command: Pause before Blind Dialing

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Write command Syntax ATS6=<time> | Response OK Parameter <time> 0 – 999 |
| Reference ITU-T V.250 § 6.3.9 | Notes The responses of this command are compliant with the recommendation but this command has no effect |

## S8 Command: Comma Dial Modifier Time

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Read command Syntax ATS8? | Response <time> OK |
| Write command Syntax ATS8=<time> | Response OK Parameter <time> 0 – 255 |
| Reference ITU-T V.250 §6.3.11 | Notes The responses of this command are compliant with the recommendation but this command has no effect |

## S10 Command: Automatic Disconnect Delay

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Read command Syntax ATS10? | Response <time> OK |
| Write command Syntax ATS10=<time> | Response OK Parameter <time> 1 – 254 |
| Reference ITU-T V.250 §6.3.12 | Notes The responses for this command are compliant with the recommendation but this command has no effect |

## S11 Command: DTMF Dialing Speed

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Write command Syntax ATS11=<time> | Response OK Parameter <time> 0 – 999 |
| Reference | Notes The responses for this command are compliant with the recommendation but this command has no effect |

## B Command: Data Rate Selection

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Execute command Syntax ATB<rate> | Response OK Parameters <rate> number from 0-99, but meaningless |
| Reference V.25ter | Notes The responses of this command are compliant with the recommendation but this command has no effect. |

## \N Command: Data Transmission Mode

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Execute command Syntax AT\N<x> | Response OK Parameters <x> 0 transparent mode 4, 6 RLP mode (nontransparent) |
| Reference V.25ter | Notes The responses of this command are compliant with the recommendation but this command has no effect. |

## L Command: Monitor Speaker Loudness

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Write command Syntax ATL [<volume>] | Response OK Parameter <volume> 0 – 9 |
| Reference ITU-T V.250 § 6.3.13 | Notes The responses of this command are compliant with the recommendation but this command has no effect |

## M Command: Monitor Speaker Mode

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Write command Syntax ATM[<mode>] | Response OK Parameter <mode> 0 – 9 |
| Reference ITU-T V.250 § 6.3.14 | Notes The responses of this command are compliant with the recommendation but this command has no effect |

## N Command: Negotiate Handshake Option

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Write command Syntax ATN[<option>] | Response OK Parameter <option> 0 – 9 |
| Reference | Notes The responses for this command are compliant with the recommendation but this command has no effect |

# Development notes

## References

The basis for AT command development is: HL6/HL8 (AirPrime - HL6 and HL8 Series - AT Commands Interface Guide - Rev15.0.docx)

The ATS1 command is based on the historical behavior of Hayes manufacturer (Hayes AT Command set).

## Applicability

The described behavior is applicable on all ports.