[No FH number]

[R01.01]

[April 27th, 2017)]

IP connectivity AT Commands

Interface Specification

Important Notice

Due to the nature of wireless communications, transmission and reception of data can never be guaranteed. Data may be delayed, corrupted (i.e., have errors) or be totally lost. Although significant delays or losses of data are rare when wireless devices such as the Sierra Wireless modem are used in a normal manner with a well-constructed network, the Sierra Wireless modem should not be used in situations where failure to transmit or receive data could result in damage of any kind to the user or any other party, including but not limited to personal injury, death, or loss of property. Sierra Wireless accepts no responsibility for damages of any kind resulting from delays or errors in data transmitted or received using the Sierra Wireless modem, or for failure of the Sierra Wireless modem to transmit or receive such data.

Safety and Hazards

Do not operate the Sierra Wireless modem in areas where cellular modems are not advised without proper device certifications. These areas include environments where cellular radio can interfere such as explosive atmospheres, medical equipment, or any other equipment which may be susceptible to any form of radio interference. The Sierra Wireless modem can transmit signals that could interfere with this equipment. Do not operate the Sierra Wireless modem in any aircraft, whether the aircraft is on the ground or in flight. In aircraft, the Sierra Wireless modem MUST BE POWERED OFF. When operating, the Sierra Wireless modem can transmit signals that could interfere with various onboard systems.

Some airlines may permit the use of cellular phones while the aircraft is on the ground and the door is open. Sierra Wireless modems may be used at this time.

The driver or operator of any vehicle should not operate the Sierra Wireless modem while in control of a vehicle. Doing so will detract from the driver or operator’s control and operation of that vehicle. In some states and provinces, operating such communications devices while in control of a vehicle is an offence.

Limitations of Liability

This manual is provided “as is”.  Sierra Wireless makes no warranties of any kind, either expressed or implied, including any implied warranties of merchantability, fitness for a particular purpose, or noninfringement.  The recipient of the manual shall endorse all risks arising from its use.

The information in this manual is subject to change without notice and does not represent a commitment on the part of Sierra Wireless. SIERRA WIRELESS AND ITS AFFILIATES SPECIFICALLY DISCLAIM LIABILITY FOR ANY AND ALL DIRECT, INDIRECT, SPECIAL, GENERAL, INCIDENTAL, CONSEQUENTIAL, PUNITIVE OR EXEMPLARY DAMAGES INCLUDING, BUT NOT LIMITED TO, LOSS OF PROFITS OR REVENUE OR ANTICIPATED PROFITS OR REVENUE ARISING OUT OF THE USE OR INABILITY TO USE ANY SIERRA WIRELESS PRODUCT, EVEN IF SIERRA WIRELESS AND/OR ITS AFFILIATES HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES OR THEY ARE FORESEEABLE OR FOR CLAIMS BY ANY THIRD PARTY.

Notwithstanding the foregoing, in no event shall Sierra Wireless and/or its affiliates aggregate liability arising under or in connection with the Sierra Wireless product, regardless of the number of events, occurrences, or claims giving rise to liability, be in excess of the price paid by the purchaser for the Sierra Wireless product.

Customer understands that Sierra Wireless is not providing cellular or GPS (including A-GPS) services. These services are provided by a third party and should be purchased directly by the Customer.

SPECIFIC DISCLAIMERS OF LIABILITY: CUSTOMER RECOGNIZES AND ACKNOWLEDGES SIERRA WIRELESS IS NOT RESPONSIBLE FOR AND SHALL NOT BE HELD LIABLE FOR ANY DEFECT OR DEFICIENCY OF ANY KIND OF CELLULAR OR GPS (INCLUDING A-GPS) SERVICES.

Patents

This product may contain technology developed by or for Sierra Wireless Inc.

This product includes technology licensed from QUALCOMM®.

This product is manufactured or sold by Sierra Wireless Inc. or its affiliates under one or more patents licensed from InterDigital Group and MMP Portfolio Licensing.

Copyright

© 2013 Sierra Wireless. All rights reserved.

Trademarks

Sierra Wireless®, AirPrime®, AirLink®, AirVantage® and the Sierra Wireless logo are registered trademarks of Sierra Wireless.

Watcher® is a registered trademark of Netgear, Inc., used under license.

Windows® and Windows Vista® are registered trademarks of Microsoft Corporation.

Macintosh® and Mac OS X® are registered trademarks of Apple Inc., registered in the U.S. and other countries.

QUALCOMM® is a registered trademark of QUALCOMM Incorporated.  Used under license.

Other trademarks are the property of their respective owners.

Contact Information

| Sales Desk: | Phone: | 1-604-232-1488 |
| --- | --- | --- |
|  | Hours: | 8:00 AM to 5:00 PM Pacific Time |
|  | E-mail: | sales@sierrawireless.com |
| Post: | Sierra Wireless 13811 Wireless Way Richmond, BC Canada V6V 3A4 |  |
| Technical Support: | support@sierrawireless.com |  |
| RMA Support: | repairs@sierrawireless.com |  |
| Fax: | 1-604-231-1109 |  |
| Web: | www.sierrawireless.com |  |

Consult our website for up-to-date product descriptions, documentation, application notes, firmware upgrades, troubleshooting tips, and press releases: www.sierrawireless.com

Document History

| Version | Date | Updates |
| --- | --- | --- |
| R01.00 | March 22nd, 2017 | Creation (EDU) |
| R01.01 | April 27th, 2017 | +++ and ATH updates |

Contents

1.	V25ter AT Commands	6

1.1.	+++ Command: Switch from Data Mode to Command Mode	6

1.2.	O Command: Switch from Command Mode to Data Mode	6

1.3.	H Command: Disconnect Existing Connection	7

1.4.	D Command: Mobile Originated Call to Dial a Number	7

2.	Development notes	8

2.1.	References	8

2.2.	Applicability	8

2.3.	Other	8

# V25ter AT Commands

## +++ Command: Switch from Data Mode to Command Mode

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Execute command Syntax +++ | Response OK |
| Reference V.25ter | Notes This command is only available during data mode. The +++ character sequence suspends the data flow over the AT interface and switches to command mode. This allows entering AT commands while maintaining the data connection to the remote device. To return to data mode, use the ATO[n] command. The +++ characters are not transmitted in the data flow. |

## O Command: Switch from Command Mode to Data Mode

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Execute command Syntax ATO[<n>] | Response TA returns to data mode from command mode: CONNECT If connection is not successfully resumed: ERROR Parameter <n> 0 Switch from command mode to data mode |
| Reference V.25ter | Notes ATO is the reverse command to the +++ escape sequence described in previous section. When a data call has been established and TA is in command mode, ATO causes the TA to resume the data connection and return to data mode. ERROR response will be replaced by NO CARRIER in a future firmware version |

## H Command: Disconnect Existing Connection

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Execute command Syntax ATH[<type>] | Response OK Disconnection succeeded, or there is no call to disconnect ERROR <type> is out of range Parameters <type> Type of call affected by ATH request. 0 Same behavior as without parameter. Disconnect ALL calls on the channel the command is requested. |
| Reference V.25Ter | Notes - None |

## D Command: Mobile Originated Call to Dial a Number

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Execute command Syntax ATD[<n>] | Response ERROR The connection cannot be established CONNECT Data connection successfully connected Parameters <n> String of dialing digits and optionally V.25ter modifiers (dialing digits): 0-9, * , #, +, A, |
| Reference V25Ter | Notes - Only data call with ATD*99[<n>] is supported. - ERROR response will be replaced by NO CARRIER in a future firmware version |

# Development notes

## References

This AT command development is based on HL8548.

## Applicability

The described behavior is applicable on all ports.

## Other

None.