[No FH number]

[R01.00]

[December 12th, 2016)]

Identification AT Commands

# Interface Specification

Important Notice

Due to the nature of wireless communications, transmission and reception of data can never be guaranteed. Data may be delayed, corrupted (i.e., have errors) or be totally lost. Although significant delays or losses of data are rare when wireless devices such as the Sierra Wireless modem are used in a normal manner with a well-constructed network, the Sierra Wireless modem should not be used in situations where failure to transmit or receive data could result in damage of any kind to the user or any other party, including but not limited to personal injury, death, or loss of property. Sierra Wireless accepts no responsibility for damages of any kind resulting from delays or errors in data transmitted or received using the Sierra Wireless modem, or for failure of the Sierra Wireless modem to transmit or receive such data.

Safety and Hazards

Do not operate the Sierra Wireless modem in areas where cellular modems are not advised without proper device certifications. These areas include environments where cellular radio can interfere such as explosive atmospheres, medical equipment, or any other equipment which may be susceptible to any form of radio interference. The Sierra Wireless modem can transmit signals that could interfere with this equipment. Do not operate the Sierra Wireless modem in any aircraft, whether the aircraft is on the ground or in flight. In aircraft, the Sierra Wireless modem MUST BE POWERED OFF. When operating, the Sierra Wireless modem can transmit signals that could interfere with various onboard systems.

Some airlines may permit the use of cellular phones while the aircraft is on the ground and the door is open. Sierra Wireless modems may be used at this time.

The driver or operator of any vehicle should not operate the Sierra Wireless modem while in control of a vehicle. Doing so will detract from the driver or operator’s control and operation of that vehicle. In some states and provinces, operating such communications devices while in control of a vehicle is an offence.

Limitations of Liability

This manual is provided “as is”.  Sierra Wireless makes no warranties of any kind, either expressed or implied, including any implied warranties of merchantability, fitness for a particular purpose, or noninfringement.  The recipient of the manual shall endorse all risks arising from its use.

The information in this manual is subject to change without notice and does not represent a commitment on the part of Sierra Wireless. SIERRA WIRELESS AND ITS AFFILIATES SPECIFICALLY DISCLAIM LIABILITY FOR ANY AND ALL DIRECT, INDIRECT, SPECIAL, GENERAL, INCIDENTAL, CONSEQUENTIAL, PUNITIVE OR EXEMPLARY DAMAGES INCLUDING, BUT NOT LIMITED TO, LOSS OF PROFITS OR REVENUE OR ANTICIPATED PROFITS OR REVENUE ARISING OUT OF THE USE OR INABILITY TO USE ANY SIERRA WIRELESS PRODUCT, EVEN IF SIERRA WIRELESS AND/OR ITS AFFILIATES HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES OR THEY ARE FORESEEABLE OR FOR CLAIMS BY ANY THIRD PARTY.

Notwithstanding the foregoing, in no event shall Sierra Wireless and/or its affiliates aggregate liability arising under or in connection with the Sierra Wireless product, regardless of the number of events, occurrences, or claims giving rise to liability, be in excess of the price paid by the purchaser for the Sierra Wireless product.

Customer understands that Sierra Wireless is not providing cellular or GPS (including A-GPS) services. These services are provided by a third party and should be purchased directly by the Customer.

SPECIFIC DISCLAIMERS OF LIABILITY: CUSTOMER RECOGNIZES AND ACKNOWLEDGES SIERRA WIRELESS IS NOT RESPONSIBLE FOR AND SHALL NOT BE HELD LIABLE FOR ANY DEFECT OR DEFICIENCY OF ANY KIND OF CELLULAR OR GPS (INCLUDING A-GPS) SERVICES.

Patents

This product may contain technology developed by or for Sierra Wireless Inc.

This product includes technology licensed from QUALCOMM®.

This product is manufactured or sold by Sierra Wireless Inc. or its affiliates under one or more patents licensed from InterDigital Group and MMP Portfolio Licensing.

Copyright

© 2013 Sierra Wireless. All rights reserved.

Trademarks

Sierra Wireless®, AirPrime®, AirLink®, AirVantage® and the Sierra Wireless logo are registered trademarks of Sierra Wireless.

Watcher® is a registered trademark of Netgear, Inc., used under license.

Windows® and Windows Vista® are registered trademarks of Microsoft Corporation.

Macintosh® and Mac OS X® are registered trademarks of Apple Inc., registered in the U.S. and other countries.

QUALCOMM® is a registered trademark of QUALCOMM Incorporated.  Used under license.

Other trademarks are the property of their respective owners.

Contact Information

| Sales Desk: | Phone: | 1-604-232-1488 |
| --- | --- | --- |
|  | Hours: | 8:00 AM to 5:00 PM Pacific Time |
|  | E-mail: | sales@sierrawireless.com |
| Post: | Sierra Wireless 13811 Wireless Way Richmond, BC Canada V6V 3A4 |  |
| Technical Support: | support@sierrawireless.com |  |
| RMA Support: | repairs@sierrawireless.com |  |
| Fax: | 1-604-231-1109 |  |
| Web: | www.sierrawireless.com |  |

Consult our website for up-to-date product descriptions, documentation, application notes, firmware upgrades, troubleshooting tips, and press releases: www.sierrawireless.com

Document History

| Version | Date | Updates |
| --- | --- | --- |
| R01.00 | December 12th, 2016 | Creation (EDU) |

Contents

1.	General AT Commands	6

1.1.	I Command: Request Identification Information	6

1.2.	+CGMI Command: Request Manufacturer Identification	7

1.3.	+CGMM Command: Request Model Identification	7

1.4.	+ CGMR Command: Request Revision Identification	8

1.5.	+GMI Command: Request Manufacturer Identification	9

1.6.	+GMM Command: Request Model Identification	9

1.7.	+ GMR Command: Request Revision Identification	10

1.8.	+KGSN Command: Request Product Serial Number and Software Version	10

2.	Data AT Commands	12

2.1.	+FMI Command: Manufacturer Identification	12

2.2.	+FMM Command: Model Identification	12

2.3.	+ FMR Command: Revision Identification	13

3.	Development notes	14

3.1.	References	14

3.2.	Applicability	14

3.3.	Other	14

# General AT Commands

## I Command: Request Identification Information

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Execute command Syntax ATI[<value>] | Response If <value> = 0 or omitted: <model> OK If <value> = 3: <version tag> OK If <value> = 9: <version tag> <build date & time> OK Parameters <model> Model identifier <version tag> Version tag (No Revision) : non-tagged version xHL77xxx…x : tagged version string <build date & time> YYYY/MM/DD HH:MM:SS |
| Examples | // Using an HL7749 module ATI HL7749 OK ATI3 AHL77xx_NQ.01.00.14_0.1.1.20161209.x1210 OK ATI9 AHL77xx_NQ.01.00.14_0.1.1.20161209.x1210 2016/12/09 18:04:53 OK |
| Reference V.25ter | Notes - None |

## +CGMI Command: Request Manufacturer Identification

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Test command Syntax AT+CGMI=? | Response OK |
| Execute command Syntax AT+CGMI | Response (manufacturer identification text) OK Example AT+CGMI Sierra Wireless OK |
| Reference [27.007] § 5.1 | Notes - None |

## +CGMM Command: Request Model Identification

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Test command Syntax AT+CGMM=? | Response OK |
| Execute command Syntax AT+CGMM | Response (model identification text) OK Example AT+CGMM HL7749 OK |
| Reference [27.007] § 5.2 | Notes - None |

## + CGMR Command: Request Revision Identification

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Test command Syntax AT+CGMR=? | Response OK |
| Execute command Syntax AT+CGMR | Response (model revision identification text) OK Example AT+CGMR AHL77xx_NQ.01.00.14_0.1.1.20161209.x1210 OK |
| Reference [27.007] § 5.3 | Notes - None |

## +GMI Command: Request Manufacturer Identification

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Test command Syntax AT+GMI=? | Response OK |
| Execute command Syntax AT+GMI | Response (manufacturer identification text) OK Example AT+GMI Sierra Wireless OK |
| Reference [27.007] § 5.1 | Notes - None |

## +GMM Command: Request Model Identification

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Test command Syntax AT+GMM=? | Response OK |
| Execute command Syntax AT+GMM | Response (model identification text) OK Example AT+GMM HL7749 OK |
| Reference [27.007] § 5.2 | Notes - None |

## + GMR Command: Request Revision Identification

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Test command Syntax AT+GMR=? | Response OK |
| Execute command Syntax AT+GMR | Response (model revision identification text) OK Example AT+GMR AHL77xx_NQ.01.00.14_0.1.1.20161209.x1210 OK |
| Reference [27.007] § 5.3 | Notes - None |

## +KGSN Command: Request Product Serial Number and Software Version

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Test command Syntax AT+KGSN=? | Response +KGSN: (list of supported <number type>s) OK |
| Execute command Syntax AT+KGSN= <number type> | Response If <number type> = 0: +KGSN: <IMEI> OK If <number type> = 1: +KGSN: <IMEISV> OK If <number type> = 2: +KGSN: <IMEISV_STR> OK If <number type> = 3: +KGSN: <FSN> OK Parameters <IMEI> 15 digits IMEI (8 digits for TAC + 6 digits for SNR + 1 check digit) <IMEISV> 16 digits IMEISV (8 digits for TAC + 6 digits for SNR + 2 SVN digits) <IMEISV_STR> formatted string: <15 digits>-<Check digit> SV: <Software version> <FSN> 14 digits Serial Number |
| Examples | AT+KGSN=0 +KGSN: 351578000023006 OK AT+KGSN=1 +KGSN: 3515780000230001 OK AT+KGSN=2 +KGSN: 35157800002300-6 SV:01 OK AT+KGSN=3 +KGSN: UC640400011101 OK |
| Reference Sierra Wireless proprietary | Notes - This command has been developed to provide the IMEI SV and Serial Number through an AT Command and it can work with or without SIM card. |

# Data AT Commands

## +FMI Command: Manufacturer Identification

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Test command Syntax AT+FMI=? | Response OK |
| Execute command Syntax AT+FMI | Response (manufacturer identification text) OK Example AT+FMI Sierra Wireless OK |
| Reference EIA/TIA-578-D | Notes - See Manufacturer identification +CGMI |

## +FMM Command: Model Identification

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Test command Syntax AT+FMM=? | Response OK |
| Execute command Syntax AT+FMM | Response (model identification text) OK Example AT+FMM HL7749 OK |
| Reference EIA/TIA-578-D | Notes - See Model identification +CGMM |

## + FMR Command: Revision Identification

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Test command Syntax AT+FMR=? | Response OK |
| Execute command Syntax AT+FMR | Response (model revision identification text) OK Example AT+FMR AHL77xx_NQ.01.00.14_0.1.1.20161209.x1210 OK |
| Reference EIA/TIA-578-D | Notes - See Revision identification +CGMR |

# Development notes

## References

This AT command development is based on HL8548.

## Applicability

The described behavior is applicable on all ports.

## Other

None.