[No FH number]

R01.00

March 28, 2017

+SWITRACE AT commands

# Interface Specification

Important Notice

Due to the nature of wireless communications, transmission and reception of data can never be guaranteed. Data may be delayed, corrupted (i.e., have errors) or be totally lost. Although significant delays or losses of data are rare when wireless devices such as the Sierra Wireless modem are used in a normal manner with a well-constructed network, the Sierra Wireless modem should not be used in situations where failure to transmit or receive data could result in damage of any kind to the user or any other party, including but not limited to personal injury, death, or loss of property. Sierra Wireless accepts no responsibility for damages of any kind resulting from delays or errors in data transmitted or received using the Sierra Wireless modem, or for failure of the Sierra Wireless modem to transmit or receive such data.

Safety and Hazards

Do not operate the Sierra Wireless modem in areas where cellular modems are not advised without proper device certifications. These areas include environments where cellular radio can interfere such as explosive atmospheres, medical equipment, or any other equipment which may be susceptible to any form of radio interference. The Sierra Wireless modem can transmit signals that could interfere with this equipment. Do not operate the Sierra Wireless modem in any aircraft, whether the aircraft is on the ground or in flight. In aircraft, the Sierra Wireless modem MUST BE POWERED OFF. When operating, the Sierra Wireless modem can transmit signals that could interfere with various onboard systems.

Some airlines may permit the use of cellular phones while the aircraft is on the ground and the door is open. Sierra Wireless modems may be used at this time.

The driver or operator of any vehicle should not operate the Sierra Wireless modem while in control of a vehicle. Doing so will detract from the driver or operator’s control and operation of that vehicle. In some states and provinces, operating such communications devices while in control of a vehicle is an offence.

Limitations of Liability

This manual is provided “as is”.  Sierra Wireless makes no warranties of any kind, either expressed or implied, including any implied warranties of merchantability, fitness for a particular purpose, or noninfringement.  The recipient of the manual shall endorse all risks arising from its use.

The information in this manual is subject to change without notice and does not represent a commitment on the part of Sierra Wireless. SIERRA WIRELESS AND ITS AFFILIATES SPECIFICALLY DISCLAIM LIABILITY FOR ANY AND ALL DIRECT, INDIRECT, SPECIAL, GENERAL, INCIDENTAL, CONSEQUENTIAL, PUNITIVE OR EXEMPLARY DAMAGES INCLUDING, BUT NOT LIMITED TO, LOSS OF PROFITS OR REVENUE OR ANTICIPATED PROFITS OR REVENUE ARISING OUT OF THE USE OR INABILITY TO USE ANY SIERRA WIRELESS PRODUCT, EVEN IF SIERRA WIRELESS AND/OR ITS AFFILIATES HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES OR THEY ARE FORESEEABLE OR FOR CLAIMS BY ANY THIRD PARTY.

Notwithstanding the foregoing, in no event shall Sierra Wireless and/or its affiliates aggregate liability arising under or in connection with the Sierra Wireless product, regardless of the number of events, occurrences, or claims giving rise to liability, be in excess of the price paid by the purchaser for the Sierra Wireless product.

Customer understands that Sierra Wireless is not providing cellular or GPS (including A-GPS) services. These services are provided by a third party and should be purchased directly by the Customer.

SPECIFIC DISCLAIMERS OF LIABILITY: CUSTOMER RECOGNIZES AND ACKNOWLEDGES SIERRA WIRELESS IS NOT RESPONSIBLE FOR AND SHALL NOT BE HELD LIABLE FOR ANY DEFECT OR DEFICIENCY OF ANY KIND OF CELLULAR OR GPS (INCLUDING A-GPS) SERVICES.

Patents

This product may contain technology developed by or for Sierra Wireless Inc.

This product includes technology licensed from QUALCOMM®.

This product is manufactured or sold by Sierra Wireless Inc. or its affiliates under one or more patents licensed from InterDigital Group and MMP Portfolio Licensing.

Copyright

© 2013 Sierra Wireless. All rights reserved.

Trademarks

Sierra Wireless®, AirPrime®, AirLink®, AirVantage® and the Sierra Wireless logo are registered trademarks of Sierra Wireless.

Watcher® is a registered trademark of Netgear, Inc., used under license.

Windows® and Windows Vista® are registered trademarks of Microsoft Corporation.

Macintosh® and Mac OS X® are registered trademarks of Apple Inc., registered in the U.S. and other countries.

QUALCOMM® is a registered trademark of QUALCOMM Incorporated.  Used under license.

Other trademarks are the property of their respective owners.

Contact Information

| Sales Desk: | Phone: | 1-604-232-1488 |
| --- | --- | --- |
|  | Hours: | 8:00 AM to 5:00 PM Pacific Time |
|  | E-mail: | sales@sierrawireless.com |
| Post: | Sierra Wireless 13811 Wireless Way Richmond, BC Canada V6V 3A4 |  |
| Technical Support: | support@sierrawireless.com |  |
| RMA Support: | repairs@sierrawireless.com |  |
| Fax: | 1-604-231-1109 |  |
| Web: | www.sierrawireless.com |  |

Consult our website for up-to-date product descriptions, documentation, application notes, firmware upgrades, troubleshooting tips, and press releases: www.sierrawireless.com

Document History

| Version | Date | Updates |
| --- | --- | --- |
| R01.00 | March 28, 2017 | Creation |

Contents

1.	Trace Commands	6

1.1.	+SWITRACEMODE Command: Switch module into specific profile	6

1.2.	+SWITRACECFG Command: Enable and tune specific module traces	7

1.3.	+SWITRACEGET Command: Upload module logs to host	8

2.	Development notes	9

2.1.	References	9

2.2.	Applicability	9

# Trace Commands

## +SWITRACEMODE Command: Switch module into specific profile

INTERNAL USE ONLY

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Test command Syntax AT+SWITRACEMODE=? | Response ERROR |
| Read command Syntax AT+SWITRACEMODE? | Response ERROR |
| Write command Syntax AT+SWITRACEMODE= <profile> | Response OK or ERROR Parameters <profile> Module profile 0 Customer profile. All undocumented services, ports and traces are off RnD R&D profile. All services available. All ports open. All traces to mid-level verbosity BFT BFT profile. RFT RFT profile. CUST Customization profile. CERT Certification profile. FLASH Software flashing and dumping profile. |
| Reference Sierra Wireless Proprietary | Notes - This Sierra Wireless proprietary command is used during the production process and allows performing testing, calibration of the module as well as setting up credentials, latest firmware and writing IMEI, … - This command returns ERROR when the read or test command is input. This is an expected result as to show that this command is strictly confidential and for security reasons it is not possible to read the data. - The FLASH profile allows software flashing/dumping at the next boot only. Subsequent boots will not allow flashing/dumping. |

## +SWITRACECFG Command: Enable and tune specific module traces

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Test command Syntax AT+SWITRACECFG=? | Response +SWITRACECFG: 0 +SWITRACECFG: enable,(list of supported <what>)[,(list of supported <how>)] +SWITRACECFG: disable,(list of supported <what>) +SWITRACECFG: set,(list of supported <what>),(list of supported <level>) +SWITRACECFG: custom,<args>… OK |
| Read command Syntax AT+SWITRACECFG? | Response ERROR |
| Write command Syntax If <subcmd>=0 AT+SWITRACECFG= <subcmd> If <subcmd>=enable AT+SWITRACECFG= <subcmd>,<what>[,<how>] If <subcmd>=set AT+SWITRACECFG= <subcmd>,<what>,<loglevel> If <subcmd>=custom AT+SWITRACECFG= <subcmd>,<args …> | Response OK or ERROR Parameters <subcmd> subcommand 0 Disable all traces enable Enable specific traces. By default, this sets a mid-level logging. disable Disable specific traces. set Tune specific traces. “enable” must be issued prior to this subcommand with the same <what > argument custom Custom trace configuration. <what> traces to enable/disable/set. applogs application logs/loglevels. Application logs are available for download through the SWITRACEGET command modemlogs modem logs/loglevels. Modem logs are available either on the USB serial gadget or through the SWITRACEGET command depending on the “how” argument. <how> Applicable to “modemlogs” only. ext Modem logs will be available through the external USB serial gadget as a continuous stream of data int Modem logs will be recorded on the module and available for download through the SWITRACEGET command <level> level of logging 1..5 for application loglevels. 1 is the lowest loglevel 1..6 for modem loglevels. 1 is the lowest loglevel <args> One or more arguments |
| Reference Sierra Wireless Proprietary | Notes - This Sierra Wireless proprietary command is targeted to the customer but is to be issued on Sierra request only for diagnosis purposes. - This command is cumulative and the settings are persistent, but will only take effect at the next boot. |
| Examples | Enable applications logs AT+SWITRACECFG=enable,applogs Enable Modem logs, save them to module AT+SWITRACECFG=enable,modemlogs,int Reboot of the module is required in order for changes to take effect and get the logs using SWITRACEGET. Reset logging to none (both modem and app) AT+SWITRACECFG=0 Reboot again |

## +SWITRACEGET Command: Upload module logs to host

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Test command Syntax AT+SWITRACEGET=? | Response +SWITRACEGET: (bootlogs,applogs,modemlogs),<ip:port> OK |
| Read command Syntax AT+SWITRACEGET? | Response ERROR |
| Write command Syntax AT+SWITRACEGET= <what>,<ip:port> | Response +SWITRACEGET: <size> OK or ERROR Parameters <what> traces to download. applogs application logs modemlogs modem logs. Only possible when modemlogs are enabled in “ext” mode with the SWITRACECFG command bootlogs boot logs <ip:port> IP address and TCP port of the host. Example: 10.0.0.100:8080. Host must be ready to receive them prior to issuing this command by listening to the given TCP port on the given host address. |
| Examples | Download application and modem logs to host with IP 10.0.0.100 on port 8080: AT+SWITRACEGET=modemlogs,10.0.0.100:8080 AT+SWITRACEGET=applogs,10.0.0.100:8080 |

# Development notes

## References

These commands are specific to HL77xx series.

## Applicability

These commands are available on all input ports.