[No FH number]

R01.00

August 23, 2017

+WDSS AT command

# Interface Specification

Important Notice

Due to the nature of wireless communications, transmission and reception of data can never be guaranteed. Data may be delayed, corrupted (i.e., have errors) or be totally lost. Although significant delays or losses of data are rare when wireless devices such as the Sierra Wireless modem are used in a normal manner with a well-constructed network, the Sierra Wireless modem should not be used in situations where failure to transmit or receive data could result in damage of any kind to the user or any other party, including but not limited to personal injury, death, or loss of property. Sierra Wireless accepts no responsibility for damages of any kind resulting from delays or errors in data transmitted or received using the Sierra Wireless modem, or for failure of the Sierra Wireless modem to transmit or receive such data.

Safety and Hazards

Do not operate the Sierra Wireless modem in areas where cellular modems are not advised without proper device certifications. These areas include environments where cellular radio can interfere such as explosive atmospheres, medical equipment, or any other equipment which may be susceptible to any form of radio interference. The Sierra Wireless modem can transmit signals that could interfere with this equipment. Do not operate the Sierra Wireless modem in any aircraft, whether the aircraft is on the ground or in flight. In aircraft, the Sierra Wireless modem MUST BE POWERED OFF. When operating, the Sierra Wireless modem can transmit signals that could interfere with various onboard systems.

Some airlines may permit the use of cellular phones while the aircraft is on the ground and the door is open. Sierra Wireless modems may be used at this time.

The driver or operator of any vehicle should not operate the Sierra Wireless modem while in control of a vehicle. Doing so will detract from the driver or operator’s control and operation of that vehicle. In some states and provinces, operating such communications devices while in control of a vehicle is an offence.

Limitations of Liability

This manual is provided “as is”.  Sierra Wireless makes no warranties of any kind, either expressed or implied, including any implied warranties of merchantability, fitness for a particular purpose, or noninfringement.  The recipient of the manual shall endorse all risks arising from its use.

The information in this manual is subject to change without notice and does not represent a commitment on the part of Sierra Wireless. SIERRA WIRELESS AND ITS AFFILIATES SPECIFICALLY DISCLAIM LIABILITY FOR ANY AND ALL DIRECT, INDIRECT, SPECIAL, GENERAL, INCIDENTAL, CONSEQUENTIAL, PUNITIVE OR EXEMPLARY DAMAGES INCLUDING, BUT NOT LIMITED TO, LOSS OF PROFITS OR REVENUE OR ANTICIPATED PROFITS OR REVENUE ARISING OUT OF THE USE OR INABILITY TO USE ANY SIERRA WIRELESS PRODUCT, EVEN IF SIERRA WIRELESS AND/OR ITS AFFILIATES HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES OR THEY ARE FORESEEABLE OR FOR CLAIMS BY ANY THIRD PARTY.

Notwithstanding the foregoing, in no event shall Sierra Wireless and/or its affiliates aggregate liability arising under or in connection with the Sierra Wireless product, regardless of the number of events, occurrences, or claims giving rise to liability, be in excess of the price paid by the purchaser for the Sierra Wireless product.

Customer understands that Sierra Wireless is not providing cellular or GPS (including A-GPS) services. These services are provided by a third party and should be purchased directly by the Customer.

SPECIFIC DISCLAIMERS OF LIABILITY: CUSTOMER RECOGNIZES AND ACKNOWLEDGES SIERRA WIRELESS IS NOT RESPONSIBLE FOR AND SHALL NOT BE HELD LIABLE FOR ANY DEFECT OR DEFICIENCY OF ANY KIND OF CELLULAR OR GPS (INCLUDING A-GPS) SERVICES.

Patents

This product may contain technology developed by or for Sierra Wireless Inc.

This product includes technology licensed from QUALCOMM®.

This product is manufactured or sold by Sierra Wireless Inc. or its affiliates under one or more patents licensed from InterDigital Group and MMP Portfolio Licensing.

Copyright

© 2013 Sierra Wireless. All rights reserved.

Trademarks

Sierra Wireless®, AirPrime®, AirLink®, AirVantage® and the Sierra Wireless logo are registered trademarks of Sierra Wireless.

Watcher® is a registered trademark of Netgear, Inc., used under license.

Windows® and Windows Vista® are registered trademarks of Microsoft Corporation.

Macintosh® and Mac OS X® are registered trademarks of Apple Inc., registered in the U.S. and other countries.

QUALCOMM® is a registered trademark of QUALCOMM Incorporated.  Used under license.

Other trademarks are the property of their respective owners.

Contact Information

| Sales Desk: | Phone: | 1-604-232-1488 |
| --- | --- | --- |
|  | Hours: | 8:00 AM to 5:00 PM Pacific Time |
|  | E-mail: | sales@sierrawireless.com |
| Post: | Sierra Wireless 13811 Wireless Way Richmond, BC Canada V6V 3A4 |  |
| Technical Support: | support@sierrawireless.com |  |
| RMA Support: | repairs@sierrawireless.com |  |
| Fax: | 1-604-231-1109 |  |
| Web: | www.sierrawireless.com |  |

Consult our website for up-to-date product descriptions, documentation, application notes, firmware upgrades, troubleshooting tips, and press releases: www.sierrawireless.com

Document History

| Version | Date | Updates |
| --- | --- | --- |
| R01.00 | August 23, 2017 | Creation |

Contents

1.	+WDSS Command: Device Services Session	6

2.	Development notes	8

2.1.	References	8

2.2.	Applicability	8

# +WDSS Command: Device Services Session

| HL7718, HL7748, HL7749 |  |
| --- | --- |
| Test command Syntax AT+WDSS=? | Response +WDSS: 0,(Max length for <Apn>),(Max length for <User>),(Max length for <Pwd>) +WDSS: 1,(list of supported <Action>s for this <Mode>) OK |
| Read command Syntax AT+WDSS? | Response [+WDSS: 0,<Apn>[,<User>]] [+WDSS: 1,1] OK |
| Write command Syntax For <Mode>=0 AT+WDSS= <Mode>,<Apn>[, <User>[,<Pwd>]] For <Mode>=1 AT+WDSS= <Mode>,<Action> | Response OK or ERROR Parameters <Mode> Integer type 0 PDP context configuration for Device Services 1 User Initiated connection to the Device services server <Apn> Access Point Name for Devices Services. String type up to 50 characters <User> Login for the APN. String type, up to 30 characters. <Pwd> Password for the APN. String type, up to 30 characters. <Action> For <Mode>=1 only 1 Establish a connection to the Device Services Server |
| Examples | AT+WDSS=? +WDSS: 0, 50,30,30 +WDSS: 1,1 OK AT+WDSS? OK //No APN defined AT+WDSS=0,”Sierra Wireless”,,”password” ERROR //Illegal null user AT+WDSS=0,”Sierra Wireless”,”user” ERROR //no password provided AT+WDSS=0,”” ERROR //Illegal empty APN AT+WDSS=0,”Sierra Wireless”,”” ERROR //Illegal empty user AT+WDSS=0,”Sierra Wireless”,”user”,”” ERROR //Illegal empty password AT+WDSS=0,”Sierra Wireless” //Define the APN for the Device Services OK //Sierra Wireless AT+WDSS? +WDSS: 0,”Sierra Wireless” +WDSS: 1,1 OK AT+WDSS=1,1 //Initiation of a connection to the Device Services server OK |
| Notes | - <Apn>, <User> and <Pwd> parameters are stored in flash without using AT&W command. AT&F has no effect on these parameters. - If one of <User> or <Pwd> parameters is supplied, the other one has to be supplied too. - Both <User> and <Pwd>, when supplied, must be non-empty strings enclosed in double quotes. - The sizes returned by WDSS=? are to be taken for the strings without the double quotes. - AT+WDSS? command only returns OK if no APN is defined. - AT+WDSS=0,… is provided for legacy compatibility only and has no effect on PDN configuration on LTE networks. - +WDSS=1,1 response is provided for legacy compatibility only. It does not reflect the actual LWM2M client connection status to to Device Services server. - As per LWM2M protocol specification, the connection to the Device Services server is periodically initiated. The AT+WDSS=1,1 command will force immediate connection when issued. |

# Development notes

## References

The basis for AT command development is: AirPrime - HL6 and HL8 Series - AT Commands Interface Guide - Rev15.0 (FileHold Document 4114680).

## Applicability

The +WDSS command is available on all input ports.