Rev 1.0

19th September, 2017

AT+KGPIOCFG

# Interface Specification

Important Notice

Due to the nature of wireless communications, transmission and reception of data can never be guaranteed. Data may be delayed, corrupted (i.e., have errors) or be totally lost. Although significant delays or losses of data are rare when wireless devices such as the Sierra Wireless modem are used in a normal manner with a well-constructed network, the Sierra Wireless modem should not be used in situations where failure to transmit or receive data could result in damage of any kind to the user or any other party, including but not limited to personal injury, death, or loss of property. Sierra Wireless accepts no responsibility for damages of any kind resulting from delays or errors in data transmitted or received using the Sierra Wireless modem, or for failure of the Sierra Wireless modem to transmit or receive such data.

Safety and Hazards

Do not operate the Sierra Wireless modem in areas where cellular modems are not advised without proper device certifications. These areas include environments where cellular radio can interfere such as explosive atmospheres, medical equipment, or any other equipment which may be susceptible to any form of radio interference. The Sierra Wireless modem can transmit signals that could interfere with this equipment. Do not operate the Sierra Wireless modem in any aircraft, whether the aircraft is on the ground or in flight. In aircraft, the Sierra Wireless modem MUST BE POWERED OFF. When operating, the Sierra Wireless modem can transmit signals that could interfere with various onboard systems.

Some airlines may permit the use of cellular phones while the aircraft is on the ground and the door is open. Sierra Wireless modems may be used at this time.

The driver or operator of any vehicle should not operate the Sierra Wireless modem while in control of a vehicle. Doing so will detract from the driver or operator’s control and operation of that vehicle. In some states and provinces, operating such communications devices while in control of a vehicle is an offence.

Limitations of Liability

This manual is provided “as is”.  Sierra Wireless makes no warranties of any kind, either expressed or implied, including any implied warranties of merchantability, fitness for a particular purpose, or noninfringement.  The recipient of the manual shall endorse all risks arising from its use.

The information in this manual is subject to change without notice and does not represent a commitment on the part of Sierra Wireless. SIERRA WIRELESS AND ITS AFFILIATES SPECIFICALLY DISCLAIM LIABILITY FOR ANY AND ALL DIRECT, INDIRECT, SPECIAL, GENERAL, INCIDENTAL, CONSEQUENTIAL, PUNITIVE OR EXEMPLARY DAMAGES INCLUDING, BUT NOT LIMITED TO, LOSS OF PROFITS OR REVENUE OR ANTICIPATED PROFITS OR REVENUE ARISING OUT OF THE USE OR INABILITY TO USE ANY SIERRA WIRELESS PRODUCT, EVEN IF SIERRA WIRELESS AND/OR ITS AFFILIATES HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES OR THEY ARE FORESEEABLE OR FOR CLAIMS BY ANY THIRD PARTY.

Notwithstanding the foregoing, in no event shall Sierra Wireless and/or its affiliates aggregate liability arising under or in connection with the Sierra Wireless product, regardless of the number of events, occurrences, or claims giving rise to liability, be in excess of the price paid by the purchaser for the Sierra Wireless product.

Customer understands that Sierra Wireless is not providing cellular or GPS (including A-GPS) services. These services are provided by a third party and should be purchased directly by the Customer.

SPECIFIC DISCLAIMERS OF LIABILITY: CUSTOMER RECOGNIZES AND ACKNOWLEDGES SIERRA WIRELESS IS NOT RESPONSIBLE FOR AND SHALL NOT BE HELD LIABLE FOR ANY DEFECT OR DEFICIENCY OF ANY KIND OF CELLULAR OR GPS (INCLUDING A-GPS) SERVICES.

Patents

This product may contain technology developed by or for Sierra Wireless Inc.

This product includes technology licensed from QUALCOMM®.

This product is manufactured or sold by Sierra Wireless Inc. or its affiliates under one or more patents licensed from InterDigital Group and MMP Portfolio Licensing.

Copyright

© 2013 Sierra Wireless. All rights reserved.

Trademarks

Sierra Wireless®, AirPrime®, AirLink®, AirVantage® and the Sierra Wireless logo are registered trademarks of Sierra Wireless.

Watcher® is a registered trademark of Netgear, Inc., used under license.

Windows® and Windows Vista® are registered trademarks of Microsoft Corporation.

Macintosh® and Mac OS X® are registered trademarks of Apple Inc., registered in the U.S. and other countries.

QUALCOMM® is a registered trademark of QUALCOMM Incorporated.  Used under license.

Other trademarks are the property of their respective owners.

Contact Information

| Sales Desk: | Phone: | 1-604-232-1488 |
| --- | --- | --- |
|  | Hours: | 8:00 AM to 5:00 PM Pacific Time |
|  | E-mail: | sales@sierrawireless.com |
| Post: | Sierra Wireless 13811 Wireless Way Richmond, BC Canada V6V 3A4 |  |
| Technical Support: | support@sierrawireless.com |  |
| RMA Support: | repairs@sierrawireless.com |  |
| Fax: | 1-604-231-1109 |  |
| Web: | www.sierrawireless.com |  |

Consult our website for up-to-date product descriptions, documentation, application notes, firmware upgrades, troubleshooting tips, and press releases: www.sierrawireless.com

Document History

| Version | Date | Updates | Author |
| --- | --- | --- | --- |
| 1.0 | 12th Sep, 2017 | Creation | Infosys |
| 1.1 | 19th Sep, 2017 | Updated parameters definition in section 2 | Infosys |
| 1.2 | 2nd Jul, 2018 | Updated with GPIO supported on HL78 | yannick |

Contents

1.	Introduction	6

2.	AT command	6

1.	Development notes	8

1.1.	References	8

1.2.	Applicability	8

1.3.	Other	8

# Introduction

This document explains about AT command interface definition and behavior for +KGPIOCFG command usage on HL78xx module.

# AT command

This command provides configuration for +KGPIO command.

| AT+KGPIOCFG : GPIO Configuration |  |
| --- | --- |
| Test command Syntax AT+KGPIOCFG=? | Response +KGPIOCFG: (list of supported <n>s),(list of supported <dir>s), (list of supported <pull mode>s) OK Example AT+KGPIOCFG=? +KGPIOCFG: (1,2,3,4,5,8,10,11,14,15),(0-1),(0-2) OK |
| Read command Syntax AT+KGPIOCFG? | Response +KGPIOCFG: <n>,<dir>,<pull mode>[<CR><LF> +KGPIOCFG: <n>,<dir>,<pull mode> [...]] OK Example AT+KGPIOCFG? +KGPIOCFG: 1,0,2 +KGPIOCFG: 2,0,2 +KGPIOCFG: 3,0,2 +KGPIOCFG: 4,0,2 +KGPIOCFG: 5,0,2 +KGPIOCFG: 8,0,2 +KGPIOCFG: 10,0,2 +KGPIOCFG: 11,0,2 +KGPIOCFG: 14,0,2 +KGPIOCFG: 15,0,2 OK |
| Write command Syntax AT+KGPIOCFG =<n>,<dir>,<pull mode> | Response OK Parameters <n> 1,2,3,4,5,8,10,11,14,15 GPIO number <dir> Direction 0 Output 1 Input <pull mode> 0 - Pull down. Internal pull down resistor available. Only used in input mode. 1 - Pull up. Internal pull up resistor available. Only used in input mode. 2 - No pull. Internal pull up/down resistor NOT available. Only used in output mode. Example AT+KGPIOCFG=1,0,2 OK AT+KGPIOCFG=1,1,1 OK |
| Notes | The current configuration is saved in non-volatile memory before a reset Pull down/up mode provides a stable input level The test command AT+KGPIOCFG=? and AT+KGPIOCFG? return a dynamic list of supported GPIOs. GPIOs assigned to a specific purpose are not listed This command can be used without SIM |

# Development notes

## References

The basis for AT command development is: HL85xx

## Applicability

The legato application will work on telnet with port number 1234 and UART1.

## Other

This command returns OK when the command is executed correctly.

This command returns ERROR when:

- the parameter is out of range (incorrect parameter)

- the provided parameter is NOT valid (invalid parameter).

Output configuration will work only for no pull mode.

Input configuration will work with either pull up or pull down mode.

Limitaion:

Currently implementation is done for GPIO pins 2,6,7,8.

Disable the pull up/down resistor is not possible. The corresponding API service is not implemented.