[No FH number]

01.00

May 22, 2018

IP/SSL AT Commands

# Interface Specification

Important Notice

Due to the nature of wireless communications, transmission and reception of data can never be guaranteed. Data may be delayed, corrupted (i.e., have errors) or be totally lost. Although significant delays or losses of data are rare when wireless devices such as the Sierra Wireless modem are used in a normal manner with a well-constructed network, the Sierra Wireless modem should not be used in situations where failure to transmit or receive data could result in damage of any kind to the user or any other party, including but not limited to personal injury, death, or loss of property. Sierra Wireless accepts no responsibility for damages of any kind resulting from delays or errors in data transmitted or received using the Sierra Wireless modem, or for failure of the Sierra Wireless modem to transmit or receive such data.

Safety and Hazards

Do not operate the Sierra Wireless modem in areas where cellular modems are not advised without proper device certifications. These areas include environments where cellular radio can interfere such as explosive atmospheres, medical equipment, or any other equipment which may be susceptible to any form of radio interference. The Sierra Wireless modem can transmit signals that could interfere with this equipment. Do not operate the Sierra Wireless modem in any aircraft, whether the aircraft is on the ground or in flight. In aircraft, the Sierra Wireless modem MUST BE POWERED OFF. When operating, the Sierra Wireless modem can transmit signals that could interfere with various onboard systems.

Some airlines may permit the use of cellular phones while the aircraft is on the ground and the door is open. Sierra Wireless modems may be used at this time.

The driver or operator of any vehicle should not operate the Sierra Wireless modem while in control of a vehicle. Doing so will detract from the driver or operator’s control and operation of that vehicle. In some states and provinces, operating such communications devices while in control of a vehicle is an offence.

Limitations of Liability

This manual is provided “as is”.  Sierra Wireless makes no warranties of any kind, either expressed or implied, including any implied warranties of merchantability, fitness for a particular purpose, or noninfringement.  The recipient of the manual shall endorse all risks arising from its use.

The information in this manual is subject to change without notice and does not represent a commitment on the part of Sierra Wireless. SIERRA WIRELESS AND ITS AFFILIATES SPECIFICALLY DISCLAIM LIABILITY FOR ANY AND ALL DIRECT, INDIRECT, SPECIAL, GENERAL, INCIDENTAL, CONSEQUENTIAL, PUNITIVE OR EXEMPLARY DAMAGES INCLUDING, BUT NOT LIMITED TO, LOSS OF PROFITS OR REVENUE OR ANTICIPATED PROFITS OR REVENUE ARISING OUT OF THE USE OR INABILITY TO USE ANY SIERRA WIRELESS PRODUCT, EVEN IF SIERRA WIRELESS AND/OR ITS AFFILIATES HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES OR THEY ARE FORESEEABLE OR FOR CLAIMS BY ANY THIRD PARTY.

Notwithstanding the foregoing, in no event shall Sierra Wireless and/or its affiliates aggregate liability arising under or in connection with the Sierra Wireless product, regardless of the number of events, occurrences, or claims giving rise to liability, be in excess of the price paid by the purchaser for the Sierra Wireless product.

Customer understands that Sierra Wireless is not providing cellular or GPS (including A-GPS) services. These services are provided by a third party and should be purchased directly by the Customer.

SPECIFIC DISCLAIMERS OF LIABILITY: CUSTOMER RECOGNIZES AND ACKNOWLEDGES SIERRA WIRELESS IS NOT RESPONSIBLE FOR AND SHALL NOT BE HELD LIABLE FOR ANY DEFECT OR DEFICIENCY OF ANY KIND OF CELLULAR OR GPS (INCLUDING A-GPS) SERVICES.

Patents

This product may contain technology developed by or for Sierra Wireless Inc.

This product includes technology licensed from QUALCOMM®.

This product is manufactured or sold by Sierra Wireless Inc. or its affiliates under one or more patents licensed from InterDigital Group and MMP Portfolio Licensing.

Copyright

© 2013 Sierra Wireless. All rights reserved.

Trademarks

Sierra Wireless®, AirPrime®, AirLink®, AirVantage® and the Sierra Wireless logo are registered trademarks of Sierra Wireless.

Watcher® is a registered trademark of Netgear, Inc., used under license.

Windows® and Windows Vista® are registered trademarks of Microsoft Corporation.

Macintosh® and Mac OS X® are registered trademarks of Apple Inc., registered in the U.S. and other countries.

QUALCOMM® is a registered trademark of QUALCOMM Incorporated.  Used under license.

Other trademarks are the property of their respective owners.

Contact Information

| Sales Desk: | Phone: | 1-604-232-1488 |
| --- | --- | --- |
|  | Hours: | 8:00 AM to 5:00 PM Pacific Time |
|  | E-mail: | sales@sierrawireless.com |
| Post: | Sierra Wireless 13811 Wireless Way Richmond, BC Canada V6V 3A4 |  |
| Technical Support: | support@sierrawireless.com |  |
| RMA Support: | repairs@sierrawireless.com |  |
| Fax: | 1-604-231-1109 |  |
| Web: | www.sierrawireless.com |  |

Consult our website for up-to-date product descriptions, documentation, application notes, firmware upgrades, troubleshooting tips, and press releases: www.sierrawireless.com

Document History

| Version | Date | Updates |
| --- | --- | --- |
| R01.00 | March 12th, 2018 | Creation |
| R01.01 | March 23, 2018 | Appendix section added |
| R01.02 | April 12, 2018 | TCP Server Session appendix added |
| R01.03 | May 4, 2018 | Update TLS crypto suites for KSSLCRYPTO command. |
| R01.04 | May 14, 2018 | Remove weak cipher suites. |
| R01.06 | May 22, 2018 | Remove all non-approved Cipher suites |
| R01.07 | May 22, 2018 | Add AES-128-CCM and AES-256-CCM in KSSLCRYPTO command |

Contents

1.	General	7

1.1.	Preliminary Comments	7

1.2.	IP Address Format in AT Commands	7

1.3.	Session ID	7

1.4.	Connection of PDP Contexts	7

1.5.	Buffer Length of AT Commands	7

1.6.	Parameter Format of AT Commands	8

2.	AT  Commands	9

2.1.	Connection Configuration	9

2.1.1.	+KCNXCFG Command: GPRS Connection Configuration	9

2.1.2.	+KCNXTIMER Command: Connection Timer Configuration	10

2.1.3.	+KCNXPROFILE Command: Current Profile Connection Configuration	11

2.1.4.	+KCGPADDR Command: Display PDP Address	12

2.1.5.	+KCNX_IND Notification: Connection Status Notification	13

2.1.6.	+KCNXUP Command: Bring the PDP Connection Up	13

2.1.7.	+KCNXDOWN Command: Bring the PDP Connection Down	14

2.2.	Common Configuration	15

2.2.1.	+KPATTERN Command: Custom End of Data Pattern	15

2.2.2.	+KURCCFG Command: Enable or Disable the URC from Protocol Commands	16

2.2.3.	+KIPOPT Command: General Options Configuration	17

2.3.	SSL Configuration	19

2.3.1.	+KSSLCRYPTO Command: Cipher Suite Configuration	19

2.3.2.	+KSSLCFG Command: SSL Configuration	20

2.4.	TCP Specific Commands	22

2.4.1.	+KTCPCFG Command: TCP Connection Configuration	22

2.4.2.	+KTCPCNX Command: Start TCP Connection	23

2.4.3.	+KTCPRCV Command: Receive Data through a TCP Connection	24

2.4.4.	+KTCPSND Command: Send Data through a TCP Connection	25

2.4.5.	+KTCPCLOSE Command: Close Current TCP Operation	26

2.4.6.	+KTCPDEL Command: Delete a Configured TCP Session	26

2.4.7.	+KTCP_SRVREQ Notification: Incoming Client Connection Request	27

2.4.8.	+KTCP_DATA Notification: Incoming Data through a TCP Connection	28

2.4.9.	+KTCP_IND Notification: TCP Status	29

2.4.10.	+KTCPSTAT Command: Get TCP Socket Status	29

2.4.11.	+KTCPSTART Command: Start a TCP Connection in Direct Data Flow	30

2.4.12.	+KTCP_ACK Notification: Status Report for Latest TCP Data	31

2.4.13.	+KTCPACKINFO Command: Poll ACK Status for the Latest Data	31

2.5.	UDP Specific Commands	32

2.5.1.	+KUDPCFG Command: UDP Connection Configuration	32

2.5.2.	+KUDPRCV Command: Receive Data through a UDP Connection	34

2.5.3.	+KUDPSND Command: Send Data through a UDP Connection	35

2.5.4.	+KUDPCLOSE Command: Close Current UDP Operation	36

2.5.5.	+KUDPDEL Command: Delete a Configured UDP Session	37

2.5.6.	+KUDP_IND Notification: UDP Status	37

2.5.7.	+KUDP_DATA Notification: Incoming Data through a UDP Connection	37

2.6.	SSL Certificate Manager	38

2.6.1.	+KCERTSTORE Command: Store Root CA and Local Certificates to Internal Storage	38

2.6.2.	+KPRIVKSTORE Command: Store Private Key Associated to a Local Certificate	40

2.6.3.	+KCERTDELETE Command: Delete Local Certificate from the Index	41

2.6.4.	+KPRIVKDELETE Command: Delete Private Key from the Index	41

3.	Appendix	43

3.1.	How to Use TCP Commands	43

3.1.1.	Client Mode	43

3.1.2.	Server Mode	44

3.1.3.	Polling for the Status of a Socket	45

3.1.4.	End to End TCP Connection	46

3.1.5.	Error Case for End to End TCP Connection	47

3.1.6.	Use Cases for AT+KTCPACKINFO and <URC-ENDTCP-enable> Option	47

3.2.	How to Use UDP Specific Commands	49

3.2.1.	Client Mode	49

3.2.2.	Use Cases for KTCP_DATA and KUDP_DATA (with/without data auto retrieval)	50

# General

## Preliminary Comments

Sierra Wireless has developed a set of proprietary AT Commands to simplify data exchanges with different protocols:

TCP

UDP

## IP Address Format in AT Commands

Unless specified elsewhere, the following format is used for IP address field in AT commands described in this chapter when using the HL78xx embedded module:

IPv4 address: Consists of dot-separated decimal (0 – 255) parameters of the form a1.a2.a3.a4

IPv6 address: Consists of colon-separated hexadecimal (0 – ffff) parameters of the form a1:a2:a3:a4:a5:a6:a7:a8 with abbreviations

## Session ID

Protocol specific AT commands share the same range of session IDs. A session ID <session_id> is a unique number and ranges from 1 to 32.

## Connection of PDP Contexts

A PDP connection will be started when a session becomes active (e.g. +KTCPCNX) and will only be stopped if all sessions are closed or all sessions request to stop the connection. In case of session errors, the PDP connection deactivation behavior can be configured by +KIPOPT with <option_id>=3. The default setting after the module boot-up is that a PDP connection is requested to stop only when a session is closed by an Internet AT command (e.g. +KTCPCLOSE).

## Buffer Length of AT Commands

In AT command mode, the maximum length of an AT command is 1023 characters; any AT command input longer than this limit will produce an error response. If the maximum length of a parameter is not specified in this manual, it may vary but still bound by this limit.

In AT data mode, the terminal receive buffer size is limited to 32000 bytes; the terminal driver will stop the receive flow at 16000 bytes if hardware handshaking is used.

## Parameter Format of AT Commands

Double quotation marks are optional in the parameter input of protocol specific AT commands.

If the AT command does not meet the following conditions, the AT parser will regard it as an error and will not go to the corresponding AT command handler. It will immediately return +CME ERROR: 3. This means that it will not process any action further or return any specific error code.

If double quotation marks are used to enclose parameters, double quotation marks must appear at both the head and tail of the parameter.

The total number of parameter input (including empty parameters) in the AT commands must be within the minimum and maximum required number of parameters.

# AT  Commands

## Connection Configuration

### +KCNXCFG Command: GPRS Connection Configuration

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KCNXCFG=? | Response +KCNXCFG: (list of possible <cnx conf>s),"GPRS",(range of possible length of <apn>),(range of possible length of <login>),(range of possible length of <password>), <af>,<ip>,<dns1>,<dns2>,<ip_v6>,<dns1_v6>,<dns2_v6> OK |
| Read command Syntax AT+KCNXCFG? | Response +KCNXCFG: <cnx cnf>, "GPRS", <apn>,<login>,<password>,<af>,<ip>,<dns1>, <dns2>[,<ip_v6>,<dns1_v6>,<dns2_v6>],<state> [...]> OK |
| Write command Syntax AT+KCNXCFG= <cnx cnf>, "GPRS",<apn> [,[<login>] [,[<password>] [,<af> [,[<ip>] [,[<dns1>] [,<dns2>]]]] [,[<ip_v6>] [,[<dns1_v6>] [,<dns2_v6>]]]]]] | Response OK Parameters <cnx cnf> 1 – 5 (PDP context configuration) a numeric parameter which specifies a particular PDP context configuration <apn> (Access Point Name) a string parameter (max size 63 bytes), logical name used to select the GGSN or the external packet data network. <login> string type (max size 24 bytes), indicates the user name of the cnx <password> string type (max size 24 bytes), indicates the password of the cnx <af> Address family used for the connection (up to 3GPP Release 7 compliant) IPV4 IPv4 only IPV6 IPv6 only IPV4V6 IPv4 and IPv6 <ip> String type. If the mobile is supposed to work with a dynamic address, the value should be “0.0.0.0” or an empty string. <dns1>, <dns2> String type. If the mobile is supposed to work with dynamic DNS addresses, the value should be “0.0.0.0” or an empty string. <ip_v6> IPV6 String type. If the mobile is supposed to work with a dynamic address, the value should be “::” or an empty string. <dns1_v6>, <dns2_v6> IPV6 String type. If the mobile is supposed to work with dynamic DNS addresses, the value should be “::” or an empty string. <state> Connection state 0 Disconnected 1 Connecting 2 Connected 3 Idle, down counting for disconnection 4 Disconnecting |
| Reference Sierra Wireless Proprietary | Notes - This AT command is used to configure the bearer to be used for the future IP services. - By default, the IP and DNS address are dynamic (those values would be affected by the network during the PDP connection). - This connection will be used by the module to access to the IP services described in subsequent chapters. The AT+KCNXCFG command is only defined to set the current parameters. The defined connection will be automatically opened when needed by the IP services (e.g. UDP service). - The use of IPV4 and/or IPV6 addresses is configured by PDP context configuration. - <cnx cfg> values 1 to 5 correspond to PDP context ID 1 to 5 respectively, e.g. <cnx cfg>=3 corresponds to CID=3 in +CGDCONT/+CGACT. - When the connection is up, the read command returns the actual values used by the connection interface. - If reuse of existing activated PDP context is required, <apn> can be set as an empty string or as the existing APN string returned by +CGDCONT read command. |

### +KCNXTIMER Command: Connection Timer Configuration

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KCNXTIMER =? | Response +KCNXTIMER: (list of supported <cnx cnf>s),(list of supported <tim1>s),(list of supported <nbtrial>s),(list of supported <tim2>s) ,(list of supported <idletime>s) OK |
| Read command Syntax AT+KCNXTIMER? | Response +KCNXTIMER: <cnx cnf>,<tim1>,<nbtrial>,<tim2>,<idletime> […] OK |
| Write command Syntax AT+KCNXTIMER=<cnx cnf>[, [<tim1>][, [<nbrtrial>] [,<tim2>] [,<idletime>]]]] | Response OK Parameters <cnx cnf> 1 – 5 (PDP context configuration) a numeric parameter which specifies a particular PDP context configuration <tim1> 1 – 120 s (30 s by default) If the module fails to activate the PDP context, a timer of <tim1> will be started. When this timer expires, it will try to activate the PDP context again. <nbtrial> Attempt times from1 – 4 (2 by default). The module will try to activate the PDP context for a maximum of <nbtrial> times. <tim2> 0 – 300s (60 s by default) 0 Deactivated (connection will not close by itself) For client sockets, module will try to connect to the server within <tim2>s; if <tim2> expires, it will give up the connection. <idletime> 0 – 1800 s (30 s by default) When all sessions are closed, the idle timer starts with the idle time. When this timer expires, it will try to deactivate the PDP context. Before the timer expires, connecting any session will stop this timer and the PDP context is reused. |
| Reference Sierra Wireless Proprietary | Notes This command will only have impact on TCP & UDP |

### +KCNXPROFILE Command: Current Profile Connection Configuration

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+ KCNXPROFILE =? | Response +KCNXPROFILE: (list of possible <cnx cnf>s) OK |
| Read command Syntax AT+ KCNXPROFILE? | Response +KCNXPROFILE: <cnx cnf> OK |
| Write command Syntax AT+ KCNXPROFILE= <cnx cnf> | Response OK Parameters <cnx cnf> 1 – 5 PDP context configuration – a numeric parameter which specifies a particular PDP context configuration |
| Reference Sierra Wireless Proprietary | Notes This command sets the default PDP context configuration ID for +KTCPCFG & +KUDPCFG, if <cnx cnf> parameter is not given in these commands. |

### +KCGPADDR Command: Display PDP Address

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KCGPADDR =? | Response +KCGPADDR: (list of possible <cnx_cnf>s) OK |
| Write command Syntax For all <cnx_cnf>s: AT+KCGPADDR For specific <cnx_cnf>s: AT+KCGPADDR=<cnx_cnf> | Response +KCGPADDR: <cnx cnf>, <PDP_addr_1> [[+KCGPADDR: <cnx cnf>, <PDP_addr_2>] …] OK Parameters <cnx cnf> 1 – 5 PDP context configuration – a numeric parameter which specifies a particular PDP context configuration <PDP_addr> A string that identifies the MT in the address space applicable to the PDP |
| Reference Sierra Wireless Proprietary | Notes - This AT command can be used after +KTCPCNX, +KUDPCFG, etc. to display the local IP address of the module - For IPv6, more than one PDP addresses corresponding to the interface may be displayed. |

### +KCNX_IND Notification: Connection Status Notification

| HL78xx |  |
| --- | --- |
| Unsolicited Notification | Response +KCNX_IND: <cnx cnf>,<status>,<af> (for <status> = 0, 1) +KCNX_IND: <cnx cnf>,<status>,<attempt>,<nbtrial>,<tim1> (for <status> = 2) +KCNX_IND: <cnx cnf>,<status> (for <status> = 3,6) +KCNX_IND: <cnx cnf>,<status>,<attempt> (for <status> = 4) +KCNX_IND: <cnx cnf>,<status>,<idletime> (for <status> = 5) Parameters <cnx cnf> 1 – 5 (PDP context configuration) a numeric parameter which specifies a particular PDP context configuration <status> PDP connection status 0 Disconnected due to network 1 Connected 2 Failed to connect, <tim1> timer is started if <attempt> is less than <nbtrail> 3 Closed 4 Connecting 5 Idle time down counting started for disconnection 6 Idle time down counting canceled <af> 0 IPV4 1 IPV6 <tim1> Refer to +KCNXTIMER <attempt> Current attempt of bringing up of PDP connection <nbtrial> Refer to +KCNXTIMER <idletime> Refer to +KCNXTIMER |
| Reference Sierra Wireless Proprietary |  |

### +KCNXUP Command: Bring the PDP Connection Up

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KCNXUP=? | Response +KCNXUP: (list of possible <cnx_cnf>s) OK |
| Write command Syntax AT+KCNXUP= <cnx_cnf> | Response OK Parameter <cnx cnf> 1 – 5 PDP context configuration – a numeric parameter which specifies a particular PDP context configuration |
| Reference Sierra Wireless Proprietary | Notes - This command activates the PDP context and reserves the activated PDP connection (i.e. keeps the PDP connection up even after the last session is closed). - If this command is not used, the PDP context will be brought down after the last session is closed unless +KCNXDOWN is used. |

### +KCNXDOWN Command: Bring the PDP Connection Down

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KCNXDOWN=? | Response +KCNXDOWN: (list of possible <cnx_cnf>s),(list of possible <mode>s) OK |
| Write command Syntax AT+KCNXDOWN=<cnx_cnf> [,<mode>] | Response OK Parameters <cnx cnf> 1 – 5 PDP context configurations – a numeric parameter which specifies a particular PDP context configuration <mode> 0 Cancels the reservation of the activated PDP connection previously configured by +KCNXUP 1 Similar to 0, but deactivates the PDP connection even if the active session exists |
| Reference Sierra Wireless Proprietary |  |

## Common Configuration

### +KPATTERN Command: Custom End of Data Pattern

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KPATTERN =? | Response OK |
| Read command Syntax AT+KPATTERN? | Response +KPATTERN: <EOF pattern> OK |
| Write command Syntax AT+KPATTERN =<EOF pattern> | Response OK or +CME ERROR <err> Parameter <EOF pattern> String type (max size 128 bytes). This is a pattern used to notify the end of data (or file) during data or file transfer. This string doesn’t have to be human-readable (not printable characters are allowed). |
| Reference Sierra Wireless Proprietary | Notes - The default value of the pattern is: "--EOF--Pattern--". - It is the responsibility of the user to select an appropriate pattern according to the data transferred (i.e. numeric pattern for text files and Readable string for binary files). - The <EOF pattern> pattern is detected within 100ms or higher timeout and without following data. The timeout value is equal to <wait_time> of +KIPOPT. - The received data is stored with buffer size <send size v4> or <send size v6> so that the <EOF pattern> with size larger than it is not detected. The user application should ensure that the value of <send size v4> or <send size v6> is larger than the size of <EOF pattern>. |

### +KURCCFG Command: Enable or Disable the URC from Protocol Commands

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KURCCFG=? | Response +KURCCFG: (list of supported <protoopt>s),(list of supported <noti_act>s),(list of supported <indi_act>s) OK |
| Read command Syntax AT+KURCCFG? | Response +KURCCFG: list of supported (<protoopt>,<noti_act>,<indi_act>) OK |
| Write command Syntax AT+KURCCFG= <protoopt>, <noti_act> [,<indi_act>] | Response OK Parameters <protoopt> Protocol option to enable/disable URC "TCPC" TCP client session "TCPS" TCP server session "UDPC" UDP client session "UDPS" UDP server session "TCP" Both TCP client and TCP server sessions "UDP" Both UDP client and UDP server sessions <noti_act> 1 Enable URC (like +KTCP_NOTIF) 0 Disable URC <indi_act> 1 Enable URC (like +KTCP_SRVREQ, +KTCP_IND, +KTCP_DATA, +KUDP_DATA, +KUDP_RCV etc.) 0 Disable URC |
| Examples | To disable URC: AT+KURCCFG=“TCP”,0 OK Test and read command: AT+KURCCFG=? +KURCCFG: ("TCPC","TCPS","UDPC","UDPS","TCP","UDP"), (0,-1),(0-1) OK AT+KURCCFG? +KURCCFG: "TCPC",1,1 +KURCCFG: "TCPS",1,1 +KURCCFG: "UDPC",1,1 +KURCCFG: "UDPS",1,1 OK |
| Reference Sierra Wireless Proprietary | Notes - Enabling or disabling +KTCP_NOTIF unsolicited messages is only useful when in polling mode with +KTCPSTAT. - If set to "disable", URCs are discarded and not stored. - Can be used in 07.10 multiplexer. |

### +KIPOPT Command: General Options Configuration

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KIPOPT=? | Response +KIPOPT: 0,<UDP>,(1-100),(8-1472),(8-1452) +KIPOPT: 0,<TCP-based>,(0-100),(0,8-1460),(0,8-1440) +KIPOPT: 3,(0-1),(0-1) OK |
| Read command Syntax AT+KIPOPT? | Response +KIPOPT: 0,<proto>,<wait time>,<send size v4>,<send size v6>] [...] +KIPOPT: 3,<stop_on_error>, <stop_on_peer> OK |
| Write command Syntax If <option_id>=0 AT+KIPOPT= <option_id>, <proto>,<wait time> [,<send size v4> [,<send size v6>]] If <option_id>=1 AT+KIPOPT= <option_id> If <option_id>=2 AT+KIPOPT= <option_id> If <option_id>=3 AT+KIPOPT= <option_id>, <stop_on_error>, <stop_on_peer> If<option_id>=4 AT+KIPOPT= <option_id>, <ssl_ver> | Response OK or +CME ERROR<err> Parameters <option_id> Option ID 0 Wait time, send size threshold configuration 1 Internal use or compatibility purposes 2 Internal use or compatibility purposes 3 PDP connection deactivated behavior 4 Internal use or compatibility purposes <proto> Protocol, string type “TCPC” TCP client session “TCPS” TCP server session “UDPC” UDP client session “UDPS” UDP server session “TCP” Both client and server TCP sessions “UDP” Both client and server UDP sessions <wait time> Timeout for configuring the packet segmentation on the IP network side; it specifies the timeout after which the buffered data will be sent to the peer irrespective of data packet size. Value is in 100 ms units. Range: For UDP: 1 – 100, default value = 2 For TCP: 0 – 100, default value = 1. Note that value = 0 has the same effect as having value = 1 due to the limitation from +KPATTERN detection timing <send size v4> Data packet size for IPv4 sessions. This parameter specifies the data packet size that needs to be sent to the peer. Range: For UDP: 8 – 1472, default value = 1020 For TCP: 0, 8 – 1460, default value = 0 (disabled) <send size v6> Data packet size for IPv6 sessions. This parameter specifies the data packet size that needs to be sent to the peer. Range: For UDP: 8 – 1452, default value = 1020 For TCP: 0, 8 – 1440, default value = 0 (disabled). Note that value = 0 uses a wait time of 100 ms. <stop_on_error> PDP connection deactivation behavior when a session is closed due to any error 0 Do not request to stop the connection 1 Request to stop the connection <stop_on_peer> PDP connection deactivation behavior when a session is closed by a peer/server 0 Do not request to stop the connection 1 Request to stop the connection |
| Reference Sierra Wireless Proprietary | Notes - The default setting of <option_id>=3 is (<stop_on_error>=0, <stop_on_peer>=0) after module boot-up; this means that a PDP connection is requested to stop only when a session is closed by an Internet AT command (e.g. +KTCPCLOSE). - Thresholds <send size v4> and <send size v6> control the maximum size of data received from the AT terminal to be buffered within timeout <wait time>. When the threshold is reached or after timeout, the buffered data are sent to the socket layer for transmission. For UDP: data are sent as a UDP packet For TCP based protocol: data are copied to socket first-in-first-out buffer for transmission but packet segmentation is not guaranteed to be <send size> - For TCP based protocol, when <send size v4> and <send size v6> are disabled (=0), threshold = 4000 is used internally. - The maximum transmission unit (MTU) is 1500 bytes. - <send size v4> and <send size v6> impacts the detection of <EOF pattern>. Refer to the notes of +KAPTTERN for more information. |

## SSL Configuration

### +KSSLCRYPTO Command: Cipher Suite Configuration

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+ KSSLCRYPTO=? | Response +KSSLCRYPTO: <profile_id>,<mkey_Algo>,<auth_algo>,<enc_algo>,<mac_algo>, <tls_ver>,<auth>,<tls_ver>,<auth> OK |
| Read command Syntax AT+ KSSLCRYPTO? | Response + KSSLCRYPTO: <profile_id>,<mkey_algo>,<auth_algo>,<enc_algo>,<mac_algo>, <tls_ver>,<auth> […]] |
| Write command Syntax AT+ KSSLCRYPTO= <profile_id>, <mkey_Algo>, <auth_algo>, <enc_algo>, <mac_algo>, <tls_ver>,<auth> | Response OK Parameters <profile_id> Index of a set of parameters for configuring one SSL profile <mkey_algo> Key exchange algorithm selection 1 RSA 8 ECDHE <auth_algo> Authentication algorithm selection 1 RSA 2 ECDSA <enc_algo> Encryption algorithm selection 16 AES-128-CCM 32 AES-256-CCM 64 AES-128-CBC 256 AES-128-CCM-8 512 AES-256-CCM-8 8192 AES-128-GCM 16384 AES-256-GCM <mac_algo> Message authentication code algorithm selection 0 NULL 4 SHA256 8 SHA384 <tls_ver> Cipher suite version selection. 1 TLS 1.0 2 TLS 1.1 4 TLS 1.2 <auth> Authentication 0 No authentication 1 Authenticate server 2 Provide client certificate to server 3 Authenticate server and provide client certificate to server |
| Reference Sierra Wireless Proprietary |  |

Here is the list of cipher suites supported on HL780x Airprime modules:

| NIST Name | <mkey_algo> | <auth_algo> | <enc_algo> | <mac_algo> |
| --- | --- | --- | --- | --- |
| TLS-RSA-WITH-AES-128-GCM-SHA256 | RSA | RSA | AES-128-GCM | SHA256 |
| TLS-RSA-WITH-AES-256-GCM-SHA384 | RSA | RSA | AES-256-GCM | SHA384 |
| TLS-RSA-WITH-AES-128-CCM | RSA | RSA | AES-128-CCM | NULL |
| TLS-RSA-WITH-AES-256-CCM | RSA | RSA | AES-256-CCM | NULL |
| TLS-RSA-WITH-AES-128-CCM-8 | RSA | RSA | AES-128-CCM-8 | NULL |
| TLS-RSA-WITH-AES-256-CCM-8 | RSA | RSA | AES-256-CCM-8 | NULL |
| TLS-ECDHE-RSA-WITH-AES-128-CBC-SHA256 | ECDHE | RSA | AES-128-CBC | SHA256 |
| TLS-ECDHE-RSA-WITH-AES-128-GCM-SHA256 | ECDHE | RSA | AES-128-GCM | SHA256 |
| TLS-ECDHE-ECDSA-WITH-AES-128-CBC-SHA256 | ECDHE | ECDSA | AES-128-CBC | SHA256 |
| TLS-ECDHE-ECDSA-WITH-AES-128-GCM-SHA256 | ECDHE | ECDSA | AES-128-GCM | SHA256 |
| TLS-ECDHE-ECDSA-WITH-AES-256-GCM-SHA384 | ECDHE | ECDSA | AES-256-GCM | SHA384 |
| TLS-ECDHE-ECDSA-WITH-AES-128-CCM | ECDHE | ECDSA | AES-128-CCM | NULL |
| TLS-ECDHE-ECDSA-WITH-AES-256-CCM | ECDHE | ECDSA | AES-256-CCM | NULL |
| TLS-ECDHE-ECDSA-WITH-AES-128-CCM-8 | ECDHE | ECDSA | AES-128-CCM-8 | NULL |
| TLS-ECDHE-ECDSA-WITH-AES-256-CCM-8 | ECDHE | ECDSA | AES-256-CCM-8 | NULL |

### +KSSLCFG Command: SSL Configuration

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KSSLCFG=? | Response +KSSLCFG:<option id>,<option> OK |
| Read command Syntax AT+KSSLCFG? | Response +KSSLCFG:0,<TLS Version> +KSSLCFG:2,<Session Mode> OK |
| Write command Syntax AT+KSSLCFG =<option id>, <option> | Response If <option_id> = 0: AT+KSSLCFG=<option_id>,<TLS Version> OK If <option_id> = 1: AT+KSSLCFG=<option_id>,<Random Seed> OK If <option_id> = 2: AT+KSSLCFG=<option_id>,<Session Mode> OK Parameters <option id> 0 Specify a TLS version to be used for hand shake 1 Setup random seed 2 Specify session mode <TLS Version> 0 Highest possible 1 TLS 1.0 3 TLS 1.2 <Random Seed> String to be added into the entropy of the random number generator <Session Mode> 0 Automatic 1 Always start a new session (not supported) |

## TCP Specific Commands

### +KTCPCFG Command: TCP Connection Configuration

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KTCPCFG=? | Response +KTCPCFG: (list of possible <cnx_cnf>s),(list of possible <mode>s), <remote-name/ip>,(list of possible <tcp_port >s),(list of possible <source_port>s),(list of possible <data_mode>s),(list of possible <URC-ENDTCP-enable>s),(list of possible <af>s),<cipher_index> OK |
| Read command Syntax AT+KTCPCFG? | Response +KTCPCFG: <session_id>,<status>,<cnx cnf>,<mode>[,<serverID>], <tcp remote address>,<tcp_port>[,<source_port>],<data_mode>, <URC-ENDTCP-enable>,<af>,<cipher_index> […]] |
| Write command Syntax AT+KTCPCFG= [<cnx cnf>], <mode>, [<tcp remote address>], <tcp_port>[[, [<source_port>] [,[<data_mode>], [<URC-ENDTCP-enable>]]],<af>] [,<cipher_suite>] | Response +KTCPCFG: <session_id> OK Parameters <cnx cnf> Index of a set of parameters for configuring one TCP session (see +KCNXCFG) <session_id> TCP session index <mode> 0 Client 1 Server 2 Child (generated by server sockets) 3 Secure client <tcp remote address> IP address string or explicit name of the remote server. For server configuration, this parameter is left blank <tcp_port> TCP port number; numeric parameter with range 1 – 65535. This parameter is the listening port for a server configuration. <status> Connection state of the selected socket 0 Disconnected 1 Connected <serverID> Server session ID index. Only for sockets in CHILD mode <source_port> Numeric parameter (0-65535). Specifies the local TCP port number. This parameter is left blank for a server configuration. <data_mode> 0 Do not display <data> in URC (default setting) 1 Display <data> in URC <URC-ENDTCP-enable> 0 Do not display URC "+KTCP_ACK" (default setting) 1 Display URC "+KTCP_ACK" <af> Address family used for the connection. 0 IPV4 1 IPV6 <cipher_index> Cipher suite profile index to use for a secured socket; defined by +KSSLCRYPTO |
| Reference Sierra Wireless Proprietary | Notes - If the socket is defined as a <CLIENT> socket, <tcp_port> and <tcp remote address> define the port and the IP address of the remote server we want to connect. - Maximum <session_id> is 32. - For child session, the property <data_mode> will be kept the same as the server socket’s setting. - See section 18.5.6 Use Cases for AT+KTCPACKINFO and <URC-ENDTCP-enable> Option. - This command can be used before setting up +KCNXCFG configuration. Note however that the latter is required to start the connection properly. - The connection timeout for TCP socket is about 9 seconds with 3 retransmissions with 3 seconds delay. |

### +KTCPCNX Command: Start TCP Connection

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KTCPCNX=? | Response +KTCPCNX: (list of possible <session_id>s) OK |
| Write command Syntax AT+KTCPCNX= <session_id> | Response OK or +CME ERROR: <err> +KTCP_NOTIF: <session_id>, <tcp_notif> Parameters <session_id> TCP session index <tcp_notif> Integer type. Indicates the cause of the TCP connection failure 0 Network error 1 No more sockets available; max. number already reached 2 Memory problem 3 DNS error 4 TCP disconnection by the server or remote client 5 TCP connection error 6 Generic error 7 Fail to accept client request’s 8 Data sending is OK but KTCPSND was waiting more or less characters 9 Bad session ID 10 Session is already running 11 All sessions are used 12 Socket connection timeout error 13 SSL connection error 14 SSL initialization error |
| Reference Sierra Wireless Proprietary | Notes This command is used for connecting to a remote server or listening to a bound port, depending on the selected mode of <session_id.> |

### +KTCPRCV Command: Receive Data through a TCP Connection

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KTCPRCV=? | Response +KTCPRCV: (list of possible <session_id>s),(list of possible <ndata>s) OK |
| Write command Syntax AT+KTCPRCV= <session_id>, <ndata> | Response CONNECT …<EOF pattern> OK or +KTCP_NOTIF: <session_id>,<tcp_notif> Parameters <session_id> TCP session index <ndata> Number of bytes the device wants to receive (max value 4294967295) <tcp_notif> See command AT+KTCPCNX |
| Reference Sierra Wireless Proprietary | Notes - This function is used to receive <ndata> data bytes through a previously opened TCP socket. - <ndata> indicates the max data number that the terminal wishes to receive. If the TCP socket contains more data than <ndata> bytes then only <ndata> bytes will be received. If the TCP socket contains less data than <ndata> bytes then only TCP socket’s data will be received. - <EOF pattern> would be added at the end of data automatically. - When <ndata> (max value) bytes or only available data in the TCP socket have been received, the module returns to command state and returns OK. - It is highly recommended to configure the module for hardware flow control using AT&K3 before using this command. - Refer to AT&D for the behavior of DTR drop. |

### +KTCPSND Command: Send Data through a TCP Connection

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KTCPSND=? | Response +KTCPSND: (list of possible <session_id>s),(list of possible <ndata>s) OK |
| Write command Syntax AT+KTCPSND= <session_id>, <ndata> | Response CONNECT OK or NO CARRIER +CME ERROR: <err> +KTCP_NOTIF: <session_id>,<tcp_notif> Parameters <session_id> TCP session index <ndata> Number of bytes (max value 4294967295) <tcp_notif> See command AT+KTCPCNX |
| Reference Sierra Wireless Proprietary | Notes - All the data will be sent out ignoring <ndata>. If data sent is not equal to <ndata> then +KTCP_NOTIF will be displayed. - <ndata> is the data size without <EOF pattern>. - It is highly recommended to configure the module for hardware flow control using AT&K3 before using this command. - Refer to AT&D for the behavior of DTR drop. - The data session is automatically ended when <ndata> data bytes are sent/received, and the module returns to command state and returns OK. - The data session can also be ended by <EOF pattern>, +++ or DTR. Refer to section 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table for the behavior of +++, DTR or ATO. |

### +KTCPCLOSE Command: Close Current TCP Operation

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KTCPCLOSE=? | Response +KTCPCLOSE: (list of possible <session_id>s), (list of possible <closing_type>s) OK |
| Write command Syntax AT+KTCPCLOSE=<session_id> [,<closing_type>] | Response OK or +CME ERROR: <err> NO CARRIER +KTCP_NOTIF: <session_id>, <tcp_notif> Parameters <session_id> TCP session index <closing_type> 0 Abort. Fast closing of the TCP connection (not supported). 1 The TCP connection is properly closed, which means that data sent to the module by AT+KTCPSND will be sent to the TCP server and acknowledged before the socket is closed. <tcp_notif>: See command AT+KTCPCNX |
| Reference Sierra Wireless Proprietary | Notes - This function first closes the TCP socket and if there is no other session running then the PDP context is released. - AT+KTCPDEL=<session_id> can be used to delete the socket configuration after it’s been closed. |

### +KTCPDEL Command: Delete a Configured TCP Session

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KTCPDEL=? | Response +KTCPDEL: (list of possible <session_id>s) OK |
| Write command Syntax AT+KTCPDEL= <session_id> | Response OK or +CME ERROR: <err> Parameter <session_id> TCP session index |
| Reference Sierra Wireless Proprietary | Notes The session must be closed (using +KTCPCLOSE) before using this command. |

### +KTCP_SRVREQ Notification: Incoming Client Connection Request

| HL78xx |  |
| --- | --- |
| Unsolicited Notification | Response +KTCP_SRVREQ: <session_id>,<subsession_id>,<client_ip>,<client_port> Parameters <session_id> TCP session index <subsession_id> Newly created TCP session index <client_ip> IP address string of the incoming socket <client_port> Numeric parameter (0-65535); port of the incoming client |
| Examples | Configure the module to TCP servers AT+KCNXCFG=0,"GPRS","szsjmc.gd"; +KTCPCFG=0,1,,179 +KTCPCFG: 1 OK AT+KCNXCFG=0,"GPRS","szsjmc.gd"; +KTCPCFG=0,1,,180 +KTCPCFG: 2 OK Start the TCP servers AT+KTCPCNX=1 //listen on port 179 OK AT+KTCPCNX=2 //listen on port 180 OK Show the TCP servers’ IP address AT+KCGPADDR +KCGPADDR: 0,"192.168.1.49" OK Incoming connection request from remote client, shows ip address and port of remote client +KTCP_SRVREQ: 1,3,"192.168.0.32",4614 //incoming a connection request from “192.168.0.32” via //listening port 179, the remote port //is 4614 +KTCP_SRVREQ: 2,4,"10.10.10.110",4665 //incoming a connection request from “10.10.10.110” via //listening port 180, the remote port //is 4665 +KTCP_SRVREQ: 2,5,"10.10.10.110",4668 //incoming a connection request from the same ip via the same //listening port, the remote //port is 4668 +KTCP_SRVREQ: 1,6,"192.168.1.117",1739 //incoming a connection request from “192.168.1.117” via //listening port 179, the remote //port is 1739 +KTCP_NOTIF: 4,4 //the connection of sub session id 4 (on listening port 180) is //closed. +KTCP_SRVREQ: 2,4,"10.10.10.8",4672 //incoming a connection request from “10.10.10.8” via listening //port 180, the remote port is //4672 |
| Reference Sierra Wireless Proprietary | Notes - This notification is sent when a client requests a connection to the server. The connection is automatically accepted. - The created session is driven as any other TCP session with its own session ID. Use +KTCPSND, +KTCPRCV, +KTCPCLOSE, etc. to provide the service associated to this TCP server. - The TCP server corresponding to the session ID is still able to receive connection requests from other clients. These requests are notified with +KTCP_SRVREQ. - The client IP address and port can also be checked using AT+KTCPCFG? after the client is connected to the TCP server. |

### +KTCP_DATA Notification: Incoming Data through a TCP Connection

| HL78xx |  |
| --- | --- |
| Unsolicited Notification | Response +KTCP_DATA: <session_id>,<ndata available>[,<data>] Parameters <session_id> TCP session index <ndata available> For <data_mode> = 0, maximum number of bytes to be read in the TCP receive buffer; for <data_mode> = 1, maximum number of bytes to be read in <data> <data> Data in octet. The length of data is specified by <ndata_available> |
| Reference Sierra Wireless Proprietary | Notes - As soon as the connection is established, the module can receive data through the TCP socket. This notification is sent when data are available in the receive buffer. - This notification is sent for each TCP packet received. - When <data_mode> is set to 1, <ndata_available> will range from 1 to 1500 in the URC. If the user application sends over 1500 bytes of data to the module, the module will display those data with several URCs. - See section 18.6.3 Use Cases for KTCP_DATA and KUDP_DATA. |

### +KTCP_IND Notification: TCP Status

| HL78xx |  |
| --- | --- |
| Unsolicited Notification | Response +KTCP_IND: <session_id>,<status> Parameters <session_id> TCP session index <status> TCP session status. 1 session is set up and ready for operation |
| Reference Sierra Wireless Proprietary |  |

### +KTCPSTAT Command: Get TCP Socket Status

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KTCPSTAT=? | Response OK |
| Read command Syntax AT+KTCPSTAT? | Response OK |
| Write command Syntax For all TCP session IDs: AT+KTCPSTAT or AT+KTCPSTAT= <session_id> | Response +KTCPSTAT: <session_id>,<status>,<tcp_notif>,<rem_data>,<rcv_data> […] OK or +KTCPSTAT: <status>,<tcp_notif>,<rem_data>,<rcv_data> OK Parameters <session_id> TCP session index <status> TCP socket state 0 Socket not defined, use +KTCPCFG to create a TCP socket 1 Socket is only defined but not used 2 Socket is opening and connecting to the server, cannot be used 3 Connection is up, socket can be used to send/receive data 4 Connection is closing, it cannot be used, wait for status 5 5 Socket is closed <tcp_notif> -1 if socket/connection is OK, <tcp_notif> if an error has happened <rem_data> Remaining bytes in the socket buffer, waiting to be sent <rcv_data> Received bytes, can be read with +KTCPRCV command |
| Reference Sierra Wireless Proprietary | Notes - The socket buffer’s size for sending is 17520 bytes. - This command returns +CME ERROR: 910 (Bad Session ID) for undefined <session_id>s. |

### +KTCPSTART Command: Start a TCP Connection in Direct Data Flow

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KTCPSTART=? | Response OK |
| Read command Syntax AT+KTCPSTART? | Response OK |
| Write command Syntax AT+KTCPSTART=<session_id> | Response CONNECT OK or +CME ERROR: an error occurs, syntax error +KTCP_NOTIF: <session_id>,<tcp_notif> : an error occurs Parameters <session_id> TCP session index <tcp_notif> See command AT+KTCPCNX |
| Reference Sierra Wireless Proprietary | Notes - This function is used to send and receive data bytes through a TCP socket. - It is highly recommended to configure the module for hardware flow control using AT&K3 before using this command. - Refer to AT&D for the behavior of DTR drop. - Only one KTCPSTART session can be used. - Can be used in 07.10 multiplexer. - If the session is successfully connected by +KTCPCNX, this command does not restart the connection and the module directly enters direct data flow. - The data session can also be ended by <EOF pattern>, +++ or DTR. Refer to section 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table for the behavior of +++, DTR or ATO. |

### +KTCP_ACK Notification: Status Report for Latest TCP Data

| HL78xx |  |
| --- | --- |
| Unsolicited Notification | Response +KTCP_ACK: <session_id>,<result> <CR><LF> Parameters <session_id> TCP session index <result> 0 Data sent failure: not all data has been received by remote side 1 Data sent success: all the data has already been received by the remote side |
| Reference Sierra Wireless Proprietary | Notes - This URC is enabled or disabled by parameter <URC-ENDTCP-enable> of command +KTCPCFG. The URC is disabled by default. - See section 18.5.6 Use Cases for AT+KTCPACKINFO and <URC-ENDTCP-enable> Option. |

### +KTCPACKINFO Command: Poll ACK Status for the Latest Data

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+ KTCPACKINFO =? | Response OK |
| Read command Syntax AT+ KTCPACKINFO? | Response OK |
| Write command Syntax For all TCP session IDs with <URC-ENDTCP-enable>=1: AT+ KTCPACKINFO or AT+ KTCPACKINFO= <session_id> | Response +KTCPACKINFO: <session_id>,<result> […] OK or +KTCPACKINFO: <session_id>,<result> OK or +CME ERROR: <err> Parameters <session_id> TCP session index <result> 0 Data sent failure: not all data has been received by remote side. 1 Data sent success: all the data has already been received by the remote side; or no data transfer has happened yet 2 The status is unknown yet |
| Reference Sierra Wireless Proprietary | Notes - The command will return ERROR if <URC-ENDTCP-enable> of command +KTCPCFG is 0. - After the TCP session is connected and before any data transfer, +KTCPACKINFO returns 1. |

## UDP Specific Commands

### +KUDPCFG Command: UDP Connection Configuration

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KUDPCFG=? | Response +KUDPCFG: (list of possible <cnx cnf>s),(list of possible <mode>s),(list of possible <port>s),(list of possible <data_mode>s),<remote-name/ip>,(list of possible <udp_port>s),(list of possible <af>s) OK |
| Read command Syntax AT+KUDPCFG? | Response +KUDPCFG: <session_id>,<cnx cnf>,<mode>,<port>,<data_mode>,<udp remote address>,<udp_port>,<af> […] OK |
| Write command Syntax AT+KUDPCFG= [<cnx cnf>], <mode>[,[<port>] [,<data_mode>], [<udp remote address>] , <udp_port>,<af>] | Response +KUDPCFG: <session_id> OK or +CME ERROR: <err> +KUDP_NOTIF: <session_id>, <udp_notif> Parameters <session_id> UDP session index <mode> 0 Client 1 Server <port> 0 – 65535 Port (0 = random) <cnx cnf> 1 – 5 (PDP context configuration) a numeric parameter which specifies a particular PDP context configuration (see section 12.7.1 +KCNXCFG Command: GPRS Connection Configuration). <udp_notif> Integer type. Indicates the cause of the UDP connection failure. 0 Network error 1 No more sockets available; max number already reached 2 Memory problem 3 DNS error 5 UDP connection error(Host unreachable) 6 Generic error 8 Data sending is OK but KUDPSND was waiting more or less characters 9 Bad session ID 10 Session is already running 11 All sessions are used <data_mode> 0 Do not display <data> in URC (Default setting) 1 Display <data> in URC <udp remote address> IP address string or explicit name of the remote host, Default is empty (given by +KUDPSND). <udp_port> 0 – 65535 UDP peer port; given by +KUDPSND <af> Address family used for the connection. 0 IPV4 1 IPV6 |
| Reference Sierra Wireless Proprietary | Notes - For UDP socket in server mode, it is bound to a defined port number, incoming connection are notified by +KUDP_DATA. If remote address and port are given, they are saved for use in +KUDPSND. - Maximum <session_id> is 32. - +KCNXCFG configuration should be set up in order to start the connection properly. |

### +KUDPRCV Command: Receive Data through a UDP Connection

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KUDPRCV=? | Response +KUDPRCV: (list of possible <session_id>s),(list of possible <ndata>s) OK |
| Write command Syntax AT+KUDPRCV= <session_id>, <ndata> | Response CONNECT …<EOF pattern> OK +KUDP_RCV: <udp remote address>,<udp remote port>,<ndata available> or NO CARRIER +CME ERROR: <err> +KUDP_NOTIF: <session_id>, <udp_notif> +KUDP_DATA_MISSED: <session_id>, <ndata missed> Parameters <session_id> UDP session index <ndata> Number of bytes the device wants to receive (max value 4294967295) <udp remote address> IP address string of the remote host <udp remote port> 0 – 65535 Remote UDP port <ndata available> Number of bytes to be read in first received packet <udp_notif> See command AT+KUDPCFG <ndata missed> Number of bytes left in the UDP socket |
| Reference Sierra Wireless Proprietary | Notes - This function is used to receive <ndata> data bytes through a previously opened UDP socket. - <ndata> indicates the max data number that the terminal wishes to receive. If the UDP socket contains more data than <ndata> bytes, then only <ndata> bytes will be received and more data can be read by running this command again. - <EOF pattern> would be added at the end of data automatically. - When <ndata> (max value) bytes or only available data in the UDP socket have been received, the module returns to command mode. - It is highly recommended to configure the module for hardware flow control using AT&K3 before using this command. - Refer to AT&D for the behavior of DTR drop. |

### +KUDPSND Command: Send Data through a UDP Connection

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KUDPSND=? | Response +KUDPSND: (list of possible <session_id>s),<remote-name/ip>,(list of possible <udp_port>s),(list of possible <ndata>s) OK |
| Write command Syntax AT+KUDPSND= <session_id>, <udp remote address>, <udp_port>, <ndata> | Response CONNECT OK or NO CARRIER +CME ERROR: <err> +KUDP_NOTIF: <session_id>,<udp_notif> Parameters <session_id> UDP session index <udp remote address> IP address string or explicit name of the remote host <udp_port> 1 – 65535 UDP peer port <ndata> Number of bytes (max value 4294967295) <udp_notif> See command AT+KUDPCFG |
| Reference Sierra Wireless Proprietary | Notes - All data will be sent out ignoring <ndata>. If data sent is not equal to <ndata> then +KUDP_NOTIF will be displayed. - <ndata> is the data size without <EOF pattern>. - It is highly recommended to configure the module for hardware flow control using AT&K3 before using this command. - Refer to AT&D for the behavior of DTR drop. - The maximum transmission unit (MTU) is 1500 Bytes. - The <udp remote address> and <udp_port> are saved internally such that they can be omitted in subsequent calls of +KUDPSND. - The packet segmentation is controlled by +KIPOPT with <option_id>=0, and the maximum UDP packet size is limited by <send size v4> (1472 bytes) or <send size v6> (1452 bytes). Default value for both parameters is 1020 bytes. - The data session is automatically ended when <ndata> data bytes are sent/received, and the module returns to command state and returns OK. - The data session can also be ended by <EOF pattern>, +++ or DTR. Refer to section 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table for the behavior of +++, DTR or ATO. |

### +KUDPCLOSE Command: Close Current UDP Operation

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KUDPCLOSE=? | Response +KUDPCLOSE: (list of possible <session_id>s),(list of possible <keep_cfg>s) OK |
| Write command Syntax AT+KUDPCLOSE=<session_id> [,<keep_cfg>] | Response OK or +KUDP_NOTIF: <session_id>, <udp_notif> Parameters <session_id> UDP session index <udp_notif> See command AT+KUDPCFG <keep_cfg> Specifies whether to delete the session configuration after closing it or not 0 Delete the session configuration 1 Keep the session configuration |
| Reference Sierra Wireless Proprietary | Notes - This function closes the UDP session. If there is no other session running, the PDP context will be released. - This function will delete the session configuration if <keep_cfg> = 0. |

### +KUDPDEL Command: Delete a Configured UDP Session

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+KUDPDEL=? | Response +KUDPDEL: (list of possible <session_id>s) OK |
| Write command Syntax AT+KUDPDEL= <session_id> | Response OK or +CME ERROR: <err> Parameters <session_id> UDP session index |
| Reference Sierra Wireless Proprietary | Notes The session must be closed (using +KUDPCLOSE) before using this command. |

### +KUDP_IND Notification: UDP Status

| HL78xx |  |
| --- | --- |
| Unsolicited Notification | Response +KUDP_IND: <session_id>,<status> Parameters <session_id> UDP session index <status> UDP session status. 1 Session is set up and ready for operation |
| Reference Sierra Wireless Proprietary |  |

### +KUDP_DATA Notification: Incoming Data through a UDP Connection

| HL78xx |  |
| --- | --- |
| Unsolicited Notification | Response +KUDP_DATA: <session_id>,<ndata available>[,<udp remote address>,<udp remote port>,<data>] Parameters <session_id> UDP session index <ndata available> Number of bytes to be read <udp remote address> IP address string of the remote host <udp remote port> 0 – 65535 Remote UDP port <data> Data in octet. The length of data is specified by <ndata_available>. |
| Reference Sierra Wireless Proprietary | Notes - As soon as the UDP socket is created, the module can receive data through this socket. This notification is sent when data are available in the receive buffer. - This notification will be sent one time. When <data_mode> was set to 0 (do not display data in URC), the controlling software must read the buffer with +KUDPRCV in order to activate the notification again. - When <data_mode> was set to 1, <ndata_available> will range from 1 – 1500 in the URC. If the user application sends over 1500 bytes of data to the module, the module will display those data with several URCs. It is possible for other applications (e.g. from Windows) to send more than 1472 bytes UDP packets to the module but the packet will be segmented and reassembled by the network stack. - When <data_mode> is set to 1, URC +KUDP_RCV will not be displayed after +KUDP_DATA. - When <data_mode> was set to 1, the fields <udp remote address> and <udp remote port> will be displayed in URC +KUDP_DATA. When <data_mode> was set to 0, they will be displayed in URC +KUDP_RCV. - See section 18.6.3 Use Cases for KTCP_DATA and KUDP_DATA. |

## SSL Certificate Manager

### +KCERTSTORE Command: Store Root CA and Local Certificates to Internal Storage

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+ KCERTSTORE=? | Response +KCERTSTORE: (list of possible <data_type>s),(range of possible lengths of <NbData>), (list of possible <index>es) OK |
| Read command Syntax AT+ KCERTSTORE? | Response +KCERTSTORE [root_cert,<index>,<NbData><CR><LF> <File_data><CR><LF>] [local_cert,<index>,<NbData><CR><LF> <File_data> <CR><LF>] [...] OK or +CME ERROR: <err> |
| Write command Syntax AT+ KCERTSTORE= <data_type> [,<NbData> [,<index>]] | Response CONNECT OK or +CME ERROR: <err> Parameters <data_type> 0 Root certificate 1 Local certificate <NbData> Number of bytes to read/write. Value range: 1-3000. <index> Stored root/local certificate index. If a root/local certificate is already stored at the index, it will be overloaded. 0 by default. Value range: 0 If <data_type> = 0 0 – 2 If <data_type> = 1 <File_data> File data in bytes |
| Reference Sierra Wireless Proprietary | Notes - The <index> parameter is the link between a local certificate and a private key (refer to +KPRIVKSTORE and +KCERTDELETE for more information). - The data session is automatically ended when <ndata> data bytes are sent/received, and the module returns to command state and returns OK. - The data session can also be ended by <EOF pattern>, +++ or DTR. Refer to section 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table for the behavior of +++ or DTR. - ATO is not available for this command. |

### +KPRIVKSTORE Command: Store Private Key Associated to a Local Certificate

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+ KPRIVKSTORE =? | Response +KPRIVKSTORE: (list of possible <index>s),( range of possible lengths of <NbData>) OK |
| Read command Syntax AT+ KPRIVKSTORE? | Response +KPRIVKSTORE private_key,<index>,<NbData><CR><LF> <File_data> <CR><LF> OK or +CME ERROR: <err> |
| Write command Syntax AT+ KPRIVKSTORE= <index> [,<NbData>] | Response CONNECT OK or +CME ERROR: <err> Parameters <index> Index of the stored local certificate associated to this private key. Value range: 0 – 2 <NbData> Number of bytes to read/write (mandatory for both reading and writing). Value range: 1-3000. <File_data> File data in bytes |
| Reference Sierra Wireless Proprietary | Notes - The data session is automatically ended when <ndata> data bytes are sent/received, and the module returns to command state and returns OK. - The data session can also be ended by <EOF pattern>, +++ or DTR. Refer to section 18.9 Switch Data/Command Mode DTR +++ ATO Behavior Table for the behavior of +++ or DTR. - ATO is not available for this command. |

### +KCERTDELETE Command: Delete Local Certificate from the Index

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+ KCERTDELETE =? | Response +KCERTDELETE: (list of possible <data_type>s),(list of possible <index>s) OK |
| Read command Syntax AT+ KCERTDELETE? | Response +KCERTDELETE: OK or +CME ERROR: <err> |
| Write command Syntax AT+ KCERTDELETE=<data_type> [,<index>] | Response OK or +CME ERROR: <err> Parameters <data_type> 0 Root certificate 1 Local certificate <index> Stored local certificate index. Default value = 0. Value range: 0 If <data_type> = 0 0 – 2 If <data_type> = 1 |
| Reference Sierra Wireless Proprietary |  |

### +KPRIVKDELETE Command: Delete Private Key from the Index

| HL78xx |  |
| --- | --- |
| Test command Syntax AT+ KPRIVKDELETE =? | Response +KPRIVKDELETE: (list of possible <index>es) OK |
| Write command Syntax AT+ KPRIVKDELETE=<index> | Response OK or +CME ERROR: <err> Parameter <index> Stored private key index. Value range: 0 – 2 |
| Reference Sierra Wireless Proprietary |  |

# Appendix

## How to Use TCP Commands

### Client Mode

| AT&K3 | Hardware flow control activation |
| --- | --- |
| OK |  |
| AT+KCNXCFG=1,"GPRS","APN","log","password","0.0.0.0","0.0.0.0","0.0.0.0" | Set GPRS parameters (APN, login, password) |
| OK |  |
| AT+KTCPCFG=1,0,"www.google.com",80 | Set IP address and port number |
| +KTCPCFG: 1 | Returns session ID |
| OK |  |
| AT+KTCPCNX=1 | Initiate the connection |
| OK |  |
| AT+KTCPSND=1,18 | Send data with KPATTERN string at the end. e.g. “GET / HTTP/1.0 |
| CONNECT |  |
| …Data send… |  |
| OK | --EOF--Pattern--" |
| +KTCP_DATA: 1,1380 |  |
| AT+KTCPRCV=1, 1380 |  |
| CONNECT |  |
| HTTP/1.0 200 OK |  |
| Cache-Control: private, max-age=0 | DATA read |
| … a lot of data… |  |
| --EOF--Pattern-- |  |
| OK |  |
| +KTCP_DATA: 1,1380 | +KTCP_DATA notification |
| AT+KTCPRCV=1,1380 |  |
| CONNECT |  |
| er{padding-bottom:7px !important}#gbar,#guser{font- | DATA read |
| … a lot of data… |  |
| --EOF--Pattern-- |  |
| OK |  |
| +KTCP_DATA: 1,1380 |  |
| AT+KTCPCLOSE=1,1 | Close session 1 |
| OK |  |
| AT+KTCPDEL=1 | Delete session 1 |
| OK |  |
| AT+KTCPCFG? | No session is available |
| OK |  |

### Server Mode

In this simple example we emulate a daytime server. This server listens to port 13 and for each connection it returns the date.

| AT&K3 | Hardware flow control activation |
| --- | --- |
| OK |  |
| AT+KCNXCFG=1,"GPRS","APN","log","password","0.0.0.0","0.0.0.0","0.0.0.0" | Set GPRS parameters (APN, login, password) |
| OK |  |
| AT+KTCPCFG=1,1,,13 | Set TCP listener and port number |
| +KTCPCFG: 1 | Returns session 1 |
| OK |  |
| AT+KTCPCNX=1 | Initiate the server |
| OK |  |
| AT+KCGPADDR +KCGPADDR: 0,"10.35.125.89" | Get the IP address to initiate a connection request with a client |
| OK |  |
| +KTCP_SRVREQ: 1,2 | A client requests a connection (subsession 2) |
| AT+KTCPSND=2,15 |  |
| CONNECT |  |
| …Date and time… | Data is sent to the client read (based on subsession 2) |
| OK |  |
| +KTCP_SRVREQ: 1,3 | Another client requests a connection (subsession 3); child mode for session 3 |
| +KTCP_NOTIF: 2, 4 | Client (subsession 2) closes the connection |
| AT+KTCPSND=3,15 |  |
| CONNECT |  |
| …Date and time… OK | Data is sent to the client |
| +KTCP_DATA: 3,6 | Data received from client (subsession 3) |
| AT+KTCPRCV=3,6 | Read data received from client |
| CONNECT |  |
| … Data… --EOF--Pattern-- |  |
| OK |  |
| AT+KTCPCLOSE=3,1 OK | Close client subsession 3 and then subsession 3 is deleted automatically |
| AT+KTCPCLOSE=1,1 | Close server session 1 |
| OK |  |
| AT+KTCPDEL=1 | Delete session 1 |
| OK |  |

### Polling for the Status of a Socket

| AT&K3 | Hardware flow control activation |
| --- | --- |
| OK |  |
| AT+KCNXCFG=1,"GPRS","APN","log","password","0.0.0.0","0.0.0.0","0.0.0.0" | Set GPRS parameters (APN, login, password) |
| OK |  |
| AT+KTCPCFG=1,0,"www.google.com",80 | Set TCP Server address and port number Returns the session ID |
| +KTCPCFG: 1 |  |
| OK |  |
| AT+KURCCFG="TCP",0 | Disable TCP unsolicited messages |
| OK |  |
| AT+KTCPCNX=1 | Initiate the connection, use session 1 |
| OK |  |
| AT+KTCPSTAT=1 | Poll the connection status |
| +KTCPSTAT : 3,-1,0,0 | Connection is UP |
| OK |  |
| AT+KTCPSND=1,3000 | Send data on socket 1, we expect to send 3000 bytes but you can send less. |
| CONNECT | You can send data after CONNECT |
| …Data send… | To finish send the KPATTERN (EOF), you can define this with +KPATTERN command. |
| OK |  |
| AT+KTCPSTAT=1 | Poll the connection status |
| +KTCPSTAT : 3,-1,1234,0 | Connection is UP, there are 1234 bytes not yet sent |
| OK |  |
| AT+KTCPSTAT=1 | Poll the connection status |
| +KTCPSTAT : 3,-1,100,0 | Connection is UP, there are 100 bytes not yet sent |
| OK |  |
| AT+KTCPSTAT=1 | Poll the connection status |
| +KTCPSTAT : 3,-1,0,0 | Connection is UP, all bytes have been sent |
| OK |  |
| AT+KTCPSTAT=1 | Poll the connection status |
| +KTCPSTAT : 3,-1,0,320 | Connection is UP, 320 bytes are available for reading |
| OK |  |
| AT+KTCPRCV=1,320 | Read 320 bytes on socket 1 |
| CONNECT |  |
| … a lot of data… | Data are sent after CONNECT |
| --EOF--Pattern-- | Receive KPATTERN |
| OK |  |
| AT+KTCPCLOSE=1,1 | Close session 1 |
| OK |  |
| AT+KTCPDEL=1 | Delete session 1 |
| OK |  |

### End to End TCP Connection

| AT&K3 | Hardware flow control activation |
| --- | --- |
| OK |  |
| AT+KCNXCFG=1,"GPRS","APN","log","password","0.0.0.0","0.0.0.0","0.0.0.0" | Set GPRS parameters (APN, login, password) |
| OK |  |
| AT+KTCPCFG=1,0,"www.google.com",80 | Set TCP Server address and port number Returns session ID |
| +KTCPCFG: 1 |  |
| OK |  |
| AT+KTCPSTART=1 | Initiate the connection, use session 1 |
| CONNECT | Message CONNECT : connection to server is established, you can send data |
| …Data sent……Data received……Data sent… |  |
| …Data sent……Data received……Data sent… |  |
| +++ | Use +++ to enter in command mode |
| OK |  |
| ATO1 | Use ATO<session_id> to switch back in data mode |
| CONNECT |  |
| …Data sent……Data received……Data sent… |  |
| …Data sent……Data received……Data sent… |  |
| OK | Toggle DTR (if AT&D1 or AT&D2 configuration) to enter in command mode |
| AT+KTCPCLOSE=1,1 | Use KTCPCLOSE to close the session |
| OK |  |
| AT+KTCPDEL=1 | Delete the configured session |
| OK |  |

### Error Case for End to End TCP Connection

| AT+KTCPSTART=1 | Try to initiate the connection |
| --- | --- |
| NO CARRIER | Connection fails, see the value of <tcp_notif> |
| +KTCP_NOTIF: 1,<tcp_notif> |  |
| AT+KTCPSTART=1 | Initiate the connection |
| CONNECT |  |
| …Data sent……Data received……Data sent… | Exchange some data |
| …Data sent……Data received……Data sent… |  |
| NO CARRIER | An error occurs during connection (network lost, server closed, etc.) |
| +KTCP_NOTIF: 1,<tcp_notif> |  |

### Use Cases for AT+KTCPACKINFO and <URC-ENDTCP-enable> Option

This section describes the behavior of AT+KTCPACKINFO when the <URC-ENDTCP > option is used with AT+KTCPCFG.

#### <URC-ENDTCP-enable> is Disabled (default setting)

| AT+KCNXCFG=1,"GPRS","CMNET" |  |
| --- | --- |
| OK |  |
| AT+KTCPCFG=1,0,"202.170.131.76",2000 |  |
| +KTCPCFG: 1 |  |
| OK |  |
| AT+KTCPCFG? |  |
| +KTCPCFG: 1,0,0,0,,"202.170.131.76",2000,,0,0 | <URC-ENDTCP-enable> is disabled |
| OK |  |
| AT+KTCPCNX=1 | Connect to TCP server |
| OK |  |
| AT+KTCPSND=1,10 | Use command to send 10 bytes |
| CONNECT |  |
|  | write to serial: 0123456789--EOF--Pattern-- |
| OK |  |
|  | The URC “+KTCP_ACK” is not displayed |
| AT+KTCPACKINFO=1 |  |
| +CME ERROR: operation not allowed | Since <URC-ENDTCP-enable> is disabled, this returns error |

#### <URC-ENDTCP-enable> is Enabled

| AT+KCNXCFG=1,"GPRS","CMNET" |  |
| --- | --- |
| OK |  |
| AT+KTCPCFG=1,0,"202.170.131.76",2000,,,1 | Set <URC-ENDTCP-enable> to 1, enable URC “+KTCP_ACK” |
| +KTCPCFG: 1 |  |
| OK |  |
| AT+KTCPCFG? |  |
| +KTCPCFG: 1,0,0,0,,"202.170.131.76",2000,,0,1 | <URC-ENDTCP-enable> is enabled |
| OK |  |
| AT+KTCPCNX=1 | Connect to TCP server |
| OK |  |
| AT+KTCPSND=1,10 | Use command to receive those 10 bytes |
| CONNECT |  |
|  | write to serial: 0123456789--EOF--Pattern-- |
| OK |  |
| ….. |  |
| +KTCP_ACK: 1, 1 | Only after a short time, URC “+KTCP_ACK” tells us the latest TCP data arrived remote side |
| AT+KTCPACKINFO=1 | We can use this command to poll the status of the latest TCP data |
| +KTCPACKINFO: 1, 1 |  |
| OK |  |
| AT+KTCPSND=1,1000 | Use command to send 1000 bytes |
| CONNECT |  |
|  | write to serial: 1000bytes and --EOF--Pattern-- |
| OK |  |
| … | URC “+KTCP_ACK” not got yet |
| AT+KTCPACKINFO=1 | After a few seconds, this command can be used to poll the status of the latest TCP data |
| +KTCPACKINFO: 1, 2 | The status of the latest TCP data is unknown |
| OK |  |
| … |  |
| +KTCP_ACK: 1, 0 | Since the “OK” of the latest “+KTCPSND”, 64 seconds elapsed |
|  | URC “+KTCP_ACK” indicates that data has not arrived on remote side yet |
|  | Network may be too bad |
| AT+KTCPACKINFO=1 | We can use this command to poll the status of the latest TCP data |
| +KTCPACKINFO: 1, 0 | The status of the latest TCP data is “failure”: not all data has been received by remote side |
| OK |  |

## How to Use UDP Specific Commands

### Client Mode

| AT&K3 OK | Hardware flow control activation |
| --- | --- |
| AT+KCNXCFG=1,"GPRS","APN","log","password",,, | Set GPRS parameters (APN, login, password) |
| OK |  |
| AT+KUDPCFG=1,0 | Create a new UDP socket (returned session 1) with the parameters associated to the connection profile id number 0 |
| +KUDPCFG: 1 |  |
| OK |  |
| AT+KUDPSND= 1,"82.234.17.52",32,18 |  |
| CONNECT | Send UDP data after “CONNECT” |
| …Data sent… |  |
| --EOF--Pattern-- |  |
| OK |  |
| +KUDP_DATA: 1,35 | Received notification that indicates the presence of 35 bytes in the socket |
| AT+KUDPRCV=1, 35 | Try to read 35 bytes from session 1 |
| CONNECT |  |
| This is a simple UDP Protocol test |  |
| -EOF--Pattern-- |  |
| OK |  |
| +KUDP_RCV: "82.234.17.52",32 |  |
| +KUDP_DATA: 1,35 | Received notification that indicates the presence of 35 bytes in the socket |
| AT+KUDPRCV=1, 16 | Same test but try to read 16 bytes from session 1 |
| CONNECT |  |
| This is a simple |  |
| -EOF--Pattern-- |  |
| OK |  |
| +KUDP_DATA_MISSED: 1,19 | There are 19 unread bytes left and missed in the UDP socket |
| AT+KUDPCLOSE=1 | Definitely close the UDP session and at the same time session is deleted |
| OK |  |
| AT+KUDPCFG? | No sessions are available now |
| OK |  |

### Use Cases for KTCP_DATA and KUDP_DATA (with/without data auto retrieval)

1)	Previous features are kept (ascending compatibility of the AT commands) - Client mode

| AT+KCNXCFG=1,"GPRS","CMNET" |  |
| --- | --- |
| OK |  |
| AT+KTCPCFG=1,0,"202.170.131.76",2000 |  |
| +KTCPCFG: 1 |  |
| OK |  |
| AT+KTCPCNX=1 | Connect to TCP server |
| OK |  |
| +KTCP_DATA: 1,10 | URC tells us that 10 bytes arrived |
| AT+KTCPRCV=1,10 | Use KTCPRCV command to receive those 10 bytes |
| CONNECT |  |
| 0123456789--EOF--Pattern-- |  |
| OK |  |
| AT+KUDPCFG=0,0 | Open a UDP socket |
| +KUDPCFG: 2 |  |
| OK |  |
| +KUDP_DATA: 2,8 | URC tells us that 8 bytes arrived |
| AT+KUDPRCV=2,8 | Use command to receive those 8 bytes |
| CONNECT |  |
| 01234567--EOF--Pattern-- |  |
| OK |  |
| +KUDP_RCV: "202.170.131.76",2001 |  |

2)	New optional feature: URC takes out the data - Client mode

| AT+KCNXCFG=1,"GPRS","CMNET" |  |
| --- | --- |
| OK |  |
| AT+KTCPCFG=0,0,"202.170.131.76",2000,,1 | Extend a parameter for the new feature |
|  | When setting to 1, data will be received by the URC “+KTCP_DATA:” |
| +KTCPCFG: 1 |  |
| OK |  |
| AT+KTCPCNX=1 | Connect to TCP server |
| OK |  |
| +KTCP_DATA: 1,10,0123456789 | 10 bytes arrived. The URC takes them out directly |
| AT+KUDPCFG=0,0,3000,1 | Extend a parameter for the new feature |
|  | When setting to 1, data will be received by the URC “+KUDP_DATA:” |
| +KUDPCFG: 2 |  |
| OK |  |
| +KUDP_DATA: 2,8,"202.170.131.76",2001,01234567 | 8 bytes arrived. The URC takes them out directly |