[No FH number]

[R01.xx]

[Date (December 7th, 2016)]

NV Backup Interface

# Interface Specification

Important Notice

Due to the nature of wireless communications, transmission and reception of data can never be guaranteed. Data may be delayed, corrupted (i.e., have errors) or be totally lost. Although significant delays or losses of data are rare when wireless devices such as the Sierra Wireless modem are used in a normal manner with a well-constructed network, the Sierra Wireless modem should not be used in situations where failure to transmit or receive data could result in damage of any kind to the user or any other party, including but not limited to personal injury, death, or loss of property. Sierra Wireless accepts no responsibility for damages of any kind resulting from delays or errors in data transmitted or received using the Sierra Wireless modem, or for failure of the Sierra Wireless modem to transmit or receive such data.

Safety and Hazards

Do not operate the Sierra Wireless modem in areas where cellular modems are not advised without proper device certifications. These areas include environments where cellular radio can interfere such as explosive atmospheres, medical equipment, or any other equipment which may be susceptible to any form of radio interference. The Sierra Wireless modem can transmit signals that could interfere with this equipment. Do not operate the Sierra Wireless modem in any aircraft, whether the aircraft is on the ground or in flight. In aircraft, the Sierra Wireless modem MUST BE POWERED OFF. When operating, the Sierra Wireless modem can transmit signals that could interfere with various onboard systems.

Some airlines may permit the use of cellular phones while the aircraft is on the ground and the door is open. Sierra Wireless modems may be used at this time.

The driver or operator of any vehicle should not operate the Sierra Wireless modem while in control of a vehicle. Doing so will detract from the driver or operator’s control and operation of that vehicle. In some states and provinces, operating such communications devices while in control of a vehicle is an offence.

Limitations of Liability

This manual is provided “as is”.  Sierra Wireless makes no warranties of any kind, either expressed or implied, including any implied warranties of merchantability, fitness for a particular purpose, or noninfringement.  The recipient of the manual shall endorse all risks arising from its use.

The information in this manual is subject to change without notice and does not represent a commitment on the part of Sierra Wireless. SIERRA WIRELESS AND ITS AFFILIATES SPECIFICALLY DISCLAIM LIABILITY FOR ANY AND ALL DIRECT, INDIRECT, SPECIAL, GENERAL, INCIDENTAL, CONSEQUENTIAL, PUNITIVE OR EXEMPLARY DAMAGES INCLUDING, BUT NOT LIMITED TO, LOSS OF PROFITS OR REVENUE OR ANTICIPATED PROFITS OR REVENUE ARISING OUT OF THE USE OR INABILITY TO USE ANY SIERRA WIRELESS PRODUCT, EVEN IF SIERRA WIRELESS AND/OR ITS AFFILIATES HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES OR THEY ARE FORESEEABLE OR FOR CLAIMS BY ANY THIRD PARTY.

Notwithstanding the foregoing, in no event shall Sierra Wireless and/or its affiliates aggregate liability arising under or in connection with the Sierra Wireless product, regardless of the number of events, occurrences, or claims giving rise to liability, be in excess of the price paid by the purchaser for the Sierra Wireless product.

Customer understands that Sierra Wireless is not providing cellular or GPS (including A-GPS) services. These services are provided by a third party and should be purchased directly by the Customer.

SPECIFIC DISCLAIMERS OF LIABILITY: CUSTOMER RECOGNIZES AND ACKNOWLEDGES SIERRA WIRELESS IS NOT RESPONSIBLE FOR AND SHALL NOT BE HELD LIABLE FOR ANY DEFECT OR DEFICIENCY OF ANY KIND OF CELLULAR OR GPS (INCLUDING A-GPS) SERVICES.

Patents

This product may contain technology developed by or for Sierra Wireless Inc.

This product includes technology licensed from QUALCOMM®.

This product is manufactured or sold by Sierra Wireless Inc. or its affiliates under one or more patents licensed from InterDigital Group and MMP Portfolio Licensing.

Copyright

© 2013 Sierra Wireless. All rights reserved.

Trademarks

Sierra Wireless®, AirPrime®, AirLink®, AirVantage® and the Sierra Wireless logo are registered trademarks of Sierra Wireless.

Watcher® is a registered trademark of Netgear, Inc., used under license.

Windows® and Windows Vista® are registered trademarks of Microsoft Corporation.

Macintosh® and Mac OS X® are registered trademarks of Apple Inc., registered in the U.S. and other countries.

QUALCOMM® is a registered trademark of QUALCOMM Incorporated.  Used under license.

Other trademarks are the property of their respective owners.

Contact Information

| Sales Desk: | Phone: | 1-604-232-1488 |
| --- | --- | --- |
|  | Hours: | 8:00 AM to 5:00 PM Pacific Time |
|  | E-mail: | sales@sierrawireless.com |
| Post: | Sierra Wireless 13811 Wireless Way Richmond, BC Canada V6V 3A4 |  |
| Technical Support: | support@sierrawireless.com |  |
| RMA Support: | repairs@sierrawireless.com |  |
| Fax: | 1-604-231-1109 |  |
| Web: | www.sierrawireless.com |  |

Consult our website for up-to-date product descriptions, documentation, application notes, firmware upgrades, troubleshooting tips, and press releases: www.sierrawireless.com

Document History

| Version | Date | Updates |
| --- | --- | --- |
| R01.00 | December 7th, 2016 | Creation |
| R01.01 | …. | Update description |
| … |  |  |

Contents

1.	NV Commands	6

1.1.	Auto Generation of NV backup Files	6

1.2.	Auto Recovery from Backup NV Files	6

1.3.	+NVBU Command: NV Backup Status and Control	6

2.	Development notes	10

2.1.	References	10

2.2.	Applicability	10

2.3.	Other	10

# NV Commands

## Auto Generation of NV backup Files

There are 3 NV partitions in flash used by the Firmware:

- Static Calibrated partition

- Static config parameters partition

- Dynamic default parameters partition

NV backup feature only backup the Calibrated and static config partition together. The dynamic partition is never backup, although it is reset to the default configuration when a backup recovery is performed.

The firmware automatically generates NV backup files from existing NV data (calibaration & static config parameters only) at ~12 seconds after boot if one of the following conditions are met:

- NV backup does not exist

- NV backup has been corrupted unexpectedly

An automatic backup file generation is notified with +NVBU_IND with <status>=0 on the AT port

## Auto Recovery from Backup NV Files

NV recovery is automatically done if an NV corruption is detected during NV initialization at boot.

The firmware automatically recovers NV data from available NV backup when:

- The calibrated partition is corrupted.

- The static config partition is corrupted

- A file in the dynamic partition is missing

This is notified with +NVBU_IND with <status>=3 on the AT port.

Manual NV data restores all data from the backup file to the original NV partition.

If the modem firmware crashes with 10 consecutive loops and a full restore has not been performed before, the firmware performs a full restore of all NV data items. Only consecutive crashes that happened within 12 seconds after the module boots is counted for this reset loop detection.

## +NVBU Command: NV Backup Status and Control

| HL78XX |  |
| --- | --- |
| Test command Syntax AT+NVBU=? | Response +NVBU: (0-2) OK |
| Read command Syntax AT+NVBU? | Returns list of NV backup with the format: +NVBU: <file id>,<backup date>,<backup firmware version> Response [+NVBU:0,<backup date>,<backup firmware version>] OK Error case ERROR when no backup available Parameters <file id> Backup file ID corresponding to one NV partition in non-volatile memory <backup date> Generation date of the NV backup <backup firmware version> Firmware version used to generate the NV backup |
| Write command Syntax For <mode>=0 or 1: AT+NVBU= <mode> [,<parti_id>] For <mode>=2: AT+NVBU= <mode>[,<clear>] | Response For <mode>=0 or 1: OK For <mode>=2 and <clear>=0: <log data 0> [<log data 1>] … [<log data n>] OK For <mode>=2 and <clear>=1: OK Parameters <mode> 0 Generate backup of both Static Calibrated and static config NV data to NV backup partition 1 Restore all NV data from the NVM backup partition and default dynamic parameters 2 List logs of NV backup operations <log data> NV backup operations log data <parti_id> 0 Static Calibrated and static config NV 1 Idem as 0. For retro compatibility purpose 2 Idem as 0. For retro compatibility purpose 3 Idem as 0. For retro compatibility purpose <clear log> 0 Read log 1 Clear log |
| Unsolicited Notification | Response +NVBU_IND:<status>,<file id>,<errcode> For <status>=0: +NVBU_IND:<status>,<file id>,<errcode>,<backup date>,<backup firmware version> For <status>=1 & 2: +NVBU_IND:<status>,<file id>,<errcode>,<cause>,<backup date used for restore>,<backup firmware version used for restore> Parameters <status> NV backup status 0 NV backup generation 1 NV backup restoration 2 Backup data restored (when NV corruption is detected during NV initialization) <errcode> Error code 0 No error 1 General error 2 Reserved 3 Flash erase error 4 Backup file corrupted 5 Flash read / write error <cause> Root cause 0 User request 1 Modem firmware problem 2 Whole NV partition structure corrupted 3 NV Calibrated partition corrupted 4 NV Static config partition corrupted 5 NV dynamic parameter missing <backup date> NV backup generation date <backup firmware version> Firmware version used to generate the NV backup <backup date used for restore> Generation date of the NV backup that was used for the NV restore <backup firmware version used for restore> Firmware version used to generate the NV backup that was used for the NV restore |
| Examples | # automatic backup files generation, notified by +NVBU_IND +NVBU_IND: 0,0,0,"2015/07/22-04:23:33","AHL77xx_NQ.01.00.27_0.2.2.20170207.x1210" # manual generation of backup files from existing NV partitions AT+NVBU=0,3 OK +NVBU_IND: 0,0,0,"2015/07/22 04:23:39","AHL78xx.1.3.0.1.20170207" # manual restore of backup files to original NV partitions AT+NVBU=1,3 OK +NVBU_IND: 1,0,0,0,"2015/07/22 04:23:39","AHL78xx.1.3.0.1.20170207" <module reboots automatically> |
| Reference Sierra Wireless Proprietary | Notes - Status of operations for <mode>=0 and <mode>=1 is notified by +NVBU_IND unsolicited notifications with <status>=0 and <status>=1 respectively on the AT port executed the write command. - Execution of the write command with <mode>=1 is followed by a modem reboot automatically; NVs are restored to their default values on booting. - The log file is limited to 4ko. - NO SIM card is required for this command. - The backup date and the backup firmware are displayed only when available (i.e. backup not corrupted) - <mode>=2 is for retrieving log for R&D analysis and not fully documented; generally: |