[No FH number]

[R01.00]

November 30, 2017

+KLTEBAND AT Command

# Interface Specification

Important Notice

Due to the nature of wireless communications, transmission and reception of data can never be guaranteed. Data may be delayed, corrupted (i.e., have errors) or be totally lost. Although significant delays or losses of data are rare when wireless devices such as the Sierra Wireless modem are used in a normal manner with a well-constructed network, the Sierra Wireless modem should not be used in situations where failure to transmit or receive data could result in damage of any kind to the user or any other party, including but not limited to personal injury, death, or loss of property. Sierra Wireless accepts no responsibility for damages of any kind resulting from delays or errors in data transmitted or received using the Sierra Wireless modem, or for failure of the Sierra Wireless modem to transmit or receive such data.

Safety and Hazards

Do not operate the Sierra Wireless modem in areas where cellular modems are not advised without proper device certifications. These areas include environments where cellular radio can interfere such as explosive atmospheres, medical equipment, or any other equipment which may be susceptible to any form of radio interference. The Sierra Wireless modem can transmit signals that could interfere with this equipment. Do not operate the Sierra Wireless modem in any aircraft, whether the aircraft is on the ground or in flight. In aircraft, the Sierra Wireless modem MUST BE POWERED OFF. When operating, the Sierra Wireless modem can transmit signals that could interfere with various onboard systems.

Some airlines may permit the use of cellular phones while the aircraft is on the ground and the door is open. Sierra Wireless modems may be used at this time.

The driver or operator of any vehicle should not operate the Sierra Wireless modem while in control of a vehicle. Doing so will detract from the driver or operator’s control and operation of that vehicle. In some states and provinces, operating such communications devices while in control of a vehicle is an offence.

Limitations of Liability

This manual is provided “as is”.  Sierra Wireless makes no warranties of any kind, either expressed or implied, including any implied warranties of merchantability, fitness for a particular purpose, or noninfringement.  The recipient of the manual shall endorse all risks arising from its use.

The information in this manual is subject to change without notice and does not represent a commitment on the part of Sierra Wireless. SIERRA WIRELESS AND ITS AFFILIATES SPECIFICALLY DISCLAIM LIABILITY FOR ANY AND ALL DIRECT, INDIRECT, SPECIAL, GENERAL, INCIDENTAL, CONSEQUENTIAL, PUNITIVE OR EXEMPLARY DAMAGES INCLUDING, BUT NOT LIMITED TO, LOSS OF PROFITS OR REVENUE OR ANTICIPATED PROFITS OR REVENUE ARISING OUT OF THE USE OR INABILITY TO USE ANY SIERRA WIRELESS PRODUCT, EVEN IF SIERRA WIRELESS AND/OR ITS AFFILIATES HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES OR THEY ARE FORESEEABLE OR FOR CLAIMS BY ANY THIRD PARTY.

Notwithstanding the foregoing, in no event shall Sierra Wireless and/or its affiliates aggregate liability arising under or in connection with the Sierra Wireless product, regardless of the number of events, occurrences, or claims giving rise to liability, be in excess of the price paid by the purchaser for the Sierra Wireless product.

Customer understands that Sierra Wireless is not providing cellular or GPS (including A-GPS) services. These services are provided by a third party and should be purchased directly by the Customer.

SPECIFIC DISCLAIMERS OF LIABILITY: CUSTOMER RECOGNIZES AND ACKNOWLEDGES SIERRA WIRELESS IS NOT RESPONSIBLE FOR AND SHALL NOT BE HELD LIABLE FOR ANY DEFECT OR DEFICIENCY OF ANY KIND OF CELLULAR OR GPS (INCLUDING A-GPS) SERVICES.

Patents

This product may contain technology developed by or for Sierra Wireless Inc.

This product includes technology licensed from QUALCOMM®.

This product is manufactured or sold by Sierra Wireless Inc. or its affiliates under one or more patents licensed from InterDigital Group and MMP Portfolio Licensing.

Copyright

© 2013 Sierra Wireless. All rights reserved.

Trademarks

Sierra Wireless®, AirPrime®, AirLink®, AirVantage® and the Sierra Wireless logo are registered trademarks of Sierra Wireless.

Watcher® is a registered trademark of Netgear, Inc., used under license.

Windows® and Windows Vista® are registered trademarks of Microsoft Corporation.

Macintosh® and Mac OS X® are registered trademarks of Apple Inc., registered in the U.S. and other countries.

QUALCOMM® is a registered trademark of QUALCOMM Incorporated.  Used under license.

Other trademarks are the property of their respective owners.

Contact Information

| Sales Desk: | Phone: | 1-604-232-1488 |
| --- | --- | --- |
|  | Hours: | 8:00 AM to 5:00 PM Pacific Time |
|  | E-mail: | sales@sierrawireless.com |
| Post: | Sierra Wireless 13811 Wireless Way Richmond, BC Canada V6V 3A4 |  |
| Technical Support: | support@sierrawireless.com |  |
| RMA Support: | repairs@sierrawireless.com |  |
| Fax: | 1-604-231-1109 |  |
| Web: | www.sierrawireless.com |  |

Consult our website for up-to-date product descriptions, documentation, application notes, firmware upgrades, troubleshooting tips, and press releases: www.sierrawireless.com

Document History

| Version | Date | Updates |
| --- | --- | --- |
| R01.00 | November 29th, 2017 | Creation |

Contents

1.	Group of AT Commands	6

1.1.	AT Command AT+KLTEBAND	6

2.	Development notes	8

2.1.	References	8

2.2.	Applicability	8

2.3.	Other	8

# Group of AT Commands

## AT Command AT+KLTEBAND

| HL7800 |  |
| --- | --- |
| Execute command Syntax AT+KLTEBAND =<bnd>[,bnd2[,bnd3[,bnd4[,bnd5[,bnd6[,bnd7[,bnd8[,bnd9[,bnd10]]]]]]]]] | Response +KLTEBAND: <bnd> OK Parameter <bnd> Band in hexadecimal format 0x00000800 LTE Band 2 (1900 MHz) (HL7800, HL7648 and HL7688) 0x00001000 LTE Band 3 (1800 MHz) (HL7800, HL7650, HL7690 and HL7692) 0x00002000 LTE Band 4 (1700 MHz) (HL7800, HL7618, HL7618RD, HL7648 and HL7688) 0x00004000 LTE Band 5 (850 MHz) (HL7800, HL7650 and HL7688) 0x00008000 LTE Band 1 (2000 MHz) (HL7800) 0x00010000 LTE Band 13 (700 MHz) (HL7800, HL7618, HL7618RD and HL7688) 0x00020000 LTE Band 17 (700MHz) (HL7800, HL7688) 0x00040000 LTE Band 28 (700 MHz) (HL7800, HL7650) 0x00080000 LTE Band 14 (800MHz) (HL7800) 0x00800000 LTE Band 8 (900MHz) (HL7800, HL7650, HL7690 and HL7692) 0x00100000 LTE Band 9 (1800MHz) (HL7800) 0x00200000 LTE Band 10 (1700MHz) (HL7800) 0x01000000 LTE Band 20 (800MHz) (HL7800, HL7690 and HL7692) 0x02000000 LTE Band 12 (700 MHz) (HL7800, HL7648) 0x04000000 LTE Band 18 (800MHz) (HL7800) 0x08000000 LTE Band 19 (800MHz) (HL7800) 0x10000000 LTE Band 25 (1800MHz) (HL7800) 0x20000000 LTE Band 26 (1800MHz) (HL7800) 0x40000000 LTE Band 27 (1800MHz) (HL7800) 0x80000000 LTE Band 66 (1800MHz) (HL7800) <bnd2>, <bnd3>, <bnd4>, <bnd5>,<bnd6>, <bnd7>,<bnd8>, <bnd9>,<bnd10> Same as <bnd> |
| Unsolicited Notification | None |
| Examples | AT+KLTEBAND=80000 // LTE Band 14 selected, no 0x prefix +KLTEBAND: 80000 OK AT+KLTEBAND=1000 // LTE Band 3 selected, no 0x prefix +KLTEBAND: 1000 OK AT+KBND? // Get current network band +KBND: 1000 OK AT+KLTEBAND=0 // Not defined +CME ERROR:3 |
| Reference Sierra Wireless Proprietary | Notes - The available bands to activate depend on the supported ones which are saved in bandbp file in CONFIGFS. It is a read only file. To read them use AT+KBND? command. - This command AT+KLTEBAND=<bnd> sets the active LTE band(s) that the module is currently using. - Each active band is saved in DOP file. - To read the current/active band(s), use AT+KBND? Command. - The prefix 0x has to be removed from the value to set in the command (see example). - This command cannot be used without a SIM. - There are maximum 10 bands to define at same time. |

# Development notes

## References

The AT command development is based on HL76 - AT Commands Interface Guide – Rev7.0).

## Applicability

This command sets the active LTE band(s) that the module is currently using.

The bands in +KLTEBAND are available depending on supported network bands by AirPrime  HL7800.

## Other

None