The AirPrime HL7800 module provides advanced and efficient low power modes in addition to 3GPP advanced power saving features like PSM (Power Saving Mode) and eDRX (extended DRX).

PSM is intended for seldom send data applications (for instance once a day) and does not need to be reachable or reachable with a very long latency (for instance several hours).

eDRX is intended for applications that need to be reachable but tolerate latency in an order of magnitude of 10 seconds or more.

The purpose of this document is to provide more detailed description of the AirPrime HL7800 module Low power modes, while it is configured in PSM or eDRX.

AirPrime HL7800 Low Power Modes - Application Note

Contents

Contents	2

References	2

Definitions	2

3GPP Power saving features	3

PSM	3

eDRX	6

AirPrime HL7800 Low Power modes	10

System description	10

AirPrime HL7800 power modes	11

Low power mode operation	13

Host interface	15

Low Power mode use cases	16

PSM use cases	16

eDRX use cases	18

References

| Ref. # | Document title | Doc. # |
| --- | --- | --- |
| [R-1] | AirPrime - HL78xx - AT Commands Interface Guide | #41111821 |
| [R-2] | AirPrime - HL7800 - Product Technical Specification | #41111094 |

Definitions

| Term | Description |
| --- | --- |
| Idle mode | Idle mode refers to the modem state when it has no dedicated channel allocated and it listens periodically to the paging channel according to its DRX or eDRX period. |
| PSM dormant | PSM dormant refers to the modem state when it is registered to the network and doesn’t listen any more to the paging channel. |
| TAU | Tracking Area Update |
| Connected mode | Modem state when a connection is established with the network for data or signaling exchange. |
| E-UTRAN modes | WB-S1 mode = Cat-M1 NB-S1 mode = NB1 |

3GPP Power saving features

## PSM

### Description

Power Saving Mode (PSM) is a 3GPP release 12 feature. PSM is very similar to a modem-off state, except that the AirPrime HL7800 module is still registered with the network and may have open EPS session and/or PDN connections. When a host processor needs to send data over the network, it can wake-up the module and send data without having to reattach and re-establish IP connection. While the AirPrime HL7800 is configured in PSM mode, it is reachable only when it is in connected mode or for a short period of time after this connected mode (Active timer).

The AirPrime HL7800 module can request PSM mode during attach or tracking area update (TAU) procedures. Two timers are requested within PSM mode:

- T3324 called active timer. The T3324 active timer determines the duration during which the AirPrime HL7800 module remains reachable for mobile terminated transaction on transition from connected to idle mode. Upon the active timer expiration, the AirPrime HL7800 module enters Power Saving Mode dormant state and may enter in a low power state.

- Extended periodic TAU (T3412 timer). The T3412 (Tracking Area Update timer) is used to periodically notify the availability of the AirPrime HL7800 module to the network. The AirPrime module may request a longer value than the default one assigned by the network using the extended periodic TAU value. Thanks to this, the AirPrime HL7800 module may stay in PSM dormant state for a very long period of time

Power Saving Mode – No data (simplified view)

### CPSMS AT command

See Document [R-1] for detailed AT command description

Syntax

AT+CPSMS=[<mode>[,<Requested_Periodic-RAU>[,<Requested_GPRS-READY-timer>[,<Requested_Periodic-TAU>[,<Requested_Active-Time>]]]]]

Description

The set command controls the setting of the UEs power saving mode (PSM) parameters. The command controls whether the UE wants to apply PSM or not, as well as the requested extended periodic TAU value in E-UTRAN and the requested Active Time value. See the unsolicited result codes provided by commands +CEREG for the Active Time value and the extended periodic TAU value that are allocated to the UE by the network in E-UTRAN.

Defined values

<mode>: integer type. Indication to disable or enable the use of PSM in the UE.

0	Disable the use of PSM

1	Enable the use of PSM

<Requested_Periodic-RAU>: Not used on AirPrime HL7800 module

<Requested_GPRS-READY-timer>: Not used on AirPrime HL7800 module

<Requested_Periodic-TAU>:  Requested extended periodic TAU value (T3412).

<Requested_Active-Time>: Requested Active Time value (T3324)

Extended periodic TAU value is coded as shown below:

| Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| Unit |  |  | Timer value |  |  |  |  |

Bits 5 to 1 represent the binary coded timer value and unit is coded as below:

| Bit |  |  |  |
| --- | --- | --- | --- |
| 8 | 7 | 6 |  |
| 0 | 0 | 0 | value is incremented in multiples of 10 minutes |
| 0 | 0 | 1 | value is incremented in multiples of 1 hour |
| 0 | 1 | 0 | value is incremented in multiples of 10 hours |
| 0 | 1 | 1 | value is incremented in multiples of 2 seconds |
| 1 | 0 | 0 | value is incremented in multiples of 30 seconds |
| 1 | 0 | 1 | value is incremented in multiples of 1 minute |
| 1 | 1 | 0 | value is incremented in multiples of 320 hours |
| 1 | 1 | 1 | value indicates that the timer is deactivated |

Active Time value is coded as shown below:

| Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| Unit |  |  | Timer value |  |  |  |  |

Bits 5 to 1 represent the binary coded timer value and unit is coded as below:

| Bit |  |  |  |
| --- | --- | --- | --- |
| 8 | 7 | 6 |  |
| 0 | 0 | 0 | value is incremented in multiples of 2 seconds |
| 0 | 0 | 1 | value is incremented in multiples of 1 minute |
| 0 | 1 | 0 | value is incremented in multiples of decihours |
| 1 | 1 | 1 | value indicates that the timer is deactivated |

### CEREG AT command

See Document [R-1] for detailed AT command description. Only part related to PSM is described here.

Syntax (case <n>=4)

AT+CEREG=<n>

OK

AT+CEREG?

+CEREG: <n>,<stat>[,[<tac>],[<ci>],[<AcT>][,[<cause_type>],[<reject_cause>][,[<Active-Time>],[<Periodic-TAU>]]]]

OK

Description

This command and unsolicited command will return the negotiated value for active timer and extended periodic TAU

When <n>=4 the unsolicited result code will provide the UE with additional information for the Active Time value and optionally the extended periodic TAU value if there is a change of the network cell in E-UTRAN.

If Active Timer value is returned, this means network allows use of PSM.

Defined values

<Active-Time> and <Periodic-TAU> are coded as in CPSMS command.

## eDRX

### Description

The extended Discontinuous Reception (eDRX) is a 3GPP release 13 feature. The eDRX feature allows AirPrime HL7800 module to not monitor every Paging Occasion (PO), but only the POs that belong to the Paging Transmission Window (PTW) as described in the below figure. As in DRX mode, when the AirPrime HL7800 enters in this mode, it is still registered on the network and all open PDNs remain active.

Extended DRX in Idle mode

The eDRX parameter values, i.e. timer values for TeDRX and TPTW are negotiated and configured during ATTACH and RAU procedure. During sleep period, the AirPrime HL7800 module may enter low power mode state to reduce its power consumption.

TeDRX can be set from 5,12 seconds up to 2621.44 seconds (~44 minutes) in Cat M1 and from 20,48 seconds up to 10485,76 seconds (~175 minutes) in NB1. Requested value may be configured by the customer through +CEDRXS command. Negotiated eDRX values (TeDRX and TPTW) may be read through +CEDRXRDP command.

### +CEDRXS AT command

See Document [R-1] for detailed AT command description

Syntax

AT+CEDRXS=[<mode>,[,<AcT-type>[,<Requested_eDRX_value>]]]

Description

The set command controls the setting of the UEs eDRX parameters. The command controls whether the UE wants to apply eDRX or not, as well as the requested eDRX value for each specified type of access technology.

Defined values

<mode>: integer type, indicates to disable or enable the use of eDRX in the UE.

0	Disable the use of eDRX

1	Enable the use of eDRX

2	Enable the use of eDRX and enable the unsolicited result code
+CEDRXP: <AcT-type>[,<Requested_eDRX_value>[,<NW-provided_eDRX_value>[,<Paging_time_window>]]]

3	Disable the use of eDRX and discard all parameters for eDRX or, if available, reset to the manufacturer specific default values.

<AcT-type>: integer type, indicates the type of access technology. This AT-command is used to specify the relationship between the type of access technology and the requested eDRX value.

4	   E-UTRAN (WB-S1 mode) = Cat-M1

5  	E-UTRAN (NB-S1 mode) = NB1

<Requested_eDRX_value>: string type; half a byte in a 4-bit format. eDRX value is coded as described below:

| bit |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 4 | 3 | 2 | 1 | TeDRX | Comment |
| 0 | 0 | 0 | 0 | 5,12 seconds | For Cat-M1 only |
| 0 | 0 | 0 | 1 | 10,24 seconds | For Cat-M1 only |
| 0 | 0 | 1 | 0 | 20,48 seconds | For Cat-M1 AND NB1 |
| 0 | 0 | 1 | 1 | 40,96 seconds | For Cat-M1 AND NB1 |
| 0 | 1 | 0 | 0 | 61,44 seconds | For Cat-M1 only |
| 0 | 1 | 0 | 1 | 81,92 seconds | For Cat-M1 AND NB1 |
| 0 | 1 | 1 | 0 | 102,4 seconds | For Cat-M1 only |
| 0 | 1 | 1 | 1 | 122,88 seconds | For Cat-M1 only |
| 1 | 0 | 0 | 0 | 143,36 seconds | For Cat-M1 only |
| 1 | 0 | 0 | 1 | 163,84 seconds | For Cat-M1 AND NB1 |
| 1 | 0 | 1 | 0 | 327,68 seconds | For Cat-M1 AND NB1 |
| 1 | 0 | 1 | 1 | 655,36 seconds | For Cat-M1 AND NB1 |
| 1 | 1 | 0 | 0 | 1310,72 seconds | For Cat-M1 AND NB1 |
| 1 | 1 | 0 | 1 | 2621,44 seconds | For Cat-M1 AND NB1 |
| 1 | 1 | 1 | 0 | 5242,88 seconds | For NB1 only |
| 1 | 1 | 1 | 1 | 10485,76 seconds | For NB1 only |

### +CEDRXRDP AT command

See Document [R-1] for detailed AT command description

Syntax

AT+CEDRXRDP

+CEDRXRDP: <AcT-type>[,<Requested_eDRX_value>[,<NW-provided_eDRX_value>[,<Paging_time_window>]]]

OK

Description

The execution command returns <AcT-type> and <Requested_eDRX_value>, <NW-provided_eDRX_value> and <Paging_time_window> if eDRX is used for the cell that the MS is currently registered to.

Defined values

<Act-Type> and <Requested_eDRX_value> same as +CEDRXS command

<Paging_time_window>

| The PTW value is derived as follows for <Act-Type> = 4 (Cat-M1): bit |  |  |  |  |
| --- | --- | --- | --- | --- |
| 4 | 3 | 2 | 1 | Paging Time Window length |
| 0 | 0 | 0 | 0 | 1,28 seconds |
| 0 | 0 | 0 | 1 | 2,56 seconds |
| 0 | 0 | 1 | 0 | 3,84 seconds |
| 0 | 0 | 1 | 1 | 5,12 seconds |
| 0 | 1 | 0 | 0 | 6,4 seconds |
| 0 | 1 | 0 | 1 | 7,68 seconds |
| 0 | 1 | 1 | 0 | 8,96 seconds |
| 0 | 1 | 1 | 1 | 10,24 seconds |
| 1 | 0 | 0 | 0 | 11,52 seconds |
| 1 | 0 | 0 | 1 | 12,8 seconds |
| 1 | 0 | 1 | 0 | 14,08 seconds |
| 1 | 0 | 1 | 1 | 15,36 seconds |
| 1 | 1 | 0 | 0 | 16,64 seconds |
| 1 | 1 | 0 | 1 | 17,92 seconds |
| 1 | 1 | 1 | 0 | 19,20 seconds |
| 1 | 1 | 1 | 1 | 20,48 seconds |
| The PTW value is derived as follows for <Act-Type> = 5 (NB1):: bit |  |  |  |  |
| 8 | 7 | 6 | 5 | Paging Time Window length |
| 0 | 0 | 0 | 0 | 2,56 seconds |
| 0 | 0 | 0 | 1 | 5,12 seconds |
| 0 | 0 | 1 | 0 | 7,68 seconds |
| 0 | 0 | 1 | 1 | 10,24 seconds |
| 0 | 1 | 0 | 0 | 12,8 seconds |
| 0 | 1 | 0 | 1 | 15,36 seconds |
| 0 | 1 | 1 | 0 | 17,92 seconds |
| 0 | 1 | 1 | 1 | 20,48 seconds |
| 1 | 0 | 0 | 0 | 23,04 seconds |
| 1 | 0 | 0 | 1 | 25,6 seconds |
| 1 | 0 | 1 | 0 | 28,16 seconds |
| 1 | 0 | 1 | 1 | 30,72 seconds |
| 1 | 1 | 0 | 0 | 33,28 seconds |
| 1 | 1 | 0 | 1 | 35,84 seconds |
| 1 | 1 | 1 | 0 | 38,4 seconds |
| 1 | 1 | 1 | 1 | 40,96 seconds |

### Example

To request an eDRX cycle of 81,92s, which is the more optimized value for the AirPrime HL7800 module

AT+CEDRXS=1,4,5

OK

AT+CEDRXRDP

+CEDRXRDP: 4,5,5,0

OK

In this example, the network accepted the 81,92s eDRX cycle and provided a PTW of 1,28 seconds

Note: (1): While 3GPP specifications allow eDRX sleep times of up to 44 minutes, Sierra has been able to achieve significant eDRX current savings by enforcing a 81,92s cycle time which also allows much lower latency.  The clock drift associated with sleep times >> 81,92s significantly increases the time and current consumption to resynchronize with the network upon eDRX wake.  For this reason, any current savings from cycle times >> 81,92s are negligible.

AirPrime HL7800 Low Power modes

## System description

The figure below presents an overview of the AirPrime HL7800 module’s internal architecture and the different power domains.

AirPrime HL7800 Power Domains

The AirPrime HL7800 module is composed of multiple power domains:

- Low Power Mode (LPM) Power domain: it corresponds to the Always ON domain and comprises the RTC, the 32KHz clock, a RAM retention memory and some I/Os:

  - RESET_IN_N to reset the module

  - WAKE_UP to wake-up the module from low power modes

  - PWR_ON_N to manage the Power ON of the module (will be available in a future release)

- I/O Power domain: This power domain is used to maintain I/Os in some low power modes and to allow module wake-up from UART DTR signal. When this domain is ON, VGPIO output is available.

- Core Power Domain: This power domain corresponds to the main system and implies that LPM and I/O Power domains are ON.

- High Clock Power Domain: This power domain corresponds to the main system clock and implies Core Power domain is ON.

- RF Power Domain: This power domain is ON during RX/TX activity.

For more details on the Hardware architecture of the AirPrime HL7800 module, please refer to document [R-2].

## AirPrime HL7800 power modes

3 low power modes are supported on the AirPrime HL7800 module. This mode can be configured through the AT+KSLEEP command (See document [R-1] for details).

Low power modes may be permanently disabled through +KSLEEP AT command or enabled/disabled through UART DTR signal depending on KSLEEP command configuration. When low power mode is disabled, AirPrime module will stay in running mode.

### +KSLEEP AT command

See Document [R-1] for detailed AT command description.

### Running mode

All power domains are ON, noted that RF power domain is ON only during 4G Rx/Tx activity.

### Sleep mode

In Sleep mode, all power domains, except High Clock and RF power domains are ON.

This mode is entered by the AirPrime HL7800 module if the following conditions are met:

Sleep mode is allowed (KSLEEP and UART DTR),

Modem is in DRX or in extended DRX sleep in Flight mode  (AT+CFUN=0 or 4) or PSM dormant or in no network coverage mode,

WAKE-UP Pin is set to inactive.

RESET_IN_N pin is set to inactive

The available wake up sources for this low power modes are:

WAKE_UP signal

UART DTR signal only if requested mode in KSLEEP is not Hibernate mode.

Modem events such as SMS or IP data reception, network registration state changes

All Unsolicited Result Code activated by the customer application.

Internal processing need such as internal timer expiration, interrupts, …

- Note: In this mode, UART data reception doesn’t trigger a module wake-up, but module may be responsive to AT command sent on the UART if module is awake due to internal processing.

### Lite Hibernate mode

In Lite Hibernate mode, only I/O and LPM power domains are ON and only modem state and data are retained in the LPM RAM. All other volatile data (application data) are lost: for instance, UDP/TCP socket connections, PPP connection, unsolicited command configuration, When the AirPrime HL7800 module exits from this mode, it performs a warm boot, meaning that the Host processor shall act as after a module reset.

This mode is entered by the AirPrime HL7800 module if the following conditions are met:

Lite Hibernate is allowed (KSLEEP and UART DTR),

Modem is in extended DRX sleep, Flight mode (AT+CFUN=0 or 4) or PSM dormant or in no network coverage mode.

WAKE-UP Pin is set to inactive.

RESET_IN_N pin is set to inactive

In this mode, the AirPrime HL7800 module can wake-up from:

WAKE_UP signal

UART DTR signal only if requested mode in KSLEEP is not Hibernate mode.

T3412 timer expiration if PSM mode is configured

SMS or IP data reception if eDRX mode is configured

Modem state change as network coverage lost or recovery.

- Note: In this mode, UART data reception doesn’t trigger a module wake-up, but module may be responsive to AT command sent on the UART if module is awake due to internal processing.

### Hibernate mode

In Hibernate mode, only LPM power domains is ON and only modem state and data are retained in the LPM RAM. As for Lite Hibernate, all other volatile data (application data) are lost: for instance, UDP/TCP socket connections, PPP connection, unsolicited command configuration, When the AirPrime HL7800 module exits from this mode, the Host processor shall act as after a module reset.

This mode is entered by the AirPrime HL7800 module if the following conditions are met:

Hibernate Low Power mode is allowed (KSLEEP and UART DTR),

Modem is in Flight mode (AT+CFUN=0 or 4)  or PSM dormant mode.

Modem is extended DRX sleep mode and UICC card can be deactivated (1).

WAKE-UP Pin is set to inactive.

RESET_IN_N pin is set to inactive

In this mode, the AirPrime HL7800 module can wake-up from:

WAKE_UP signal

T3412 timer expiration if PSM mode is configured

SMS or IP data reception if eDRX mode is configured

Modem state change as network coverage lost.

Note (1): UICC DEACTIVATION mechanism is a release 13 feature: description in ETSI TS 131 102 V13.4.0 clause 5.1.10 and 5.1 11

  - In PSM, UICC deactivation can be done if PIN code is disabled

  - In eDRX, UICC deactivation can be done if PIN code is disabled AND deactivation of UICC is authorized in EFAD.

If UICC deactivation is not allowed, AirPrime HL78000 module selects automatically Lite Hibernate mode.

Support of this feature by the UICC may be retrieved by reading EFad file with AT+CRSM=176,28589,0,0,4 and by checking response: the first bit of Byte 1 and the fourth bit of the Byte 3 shall be set to 1 (for example  +CRSM: 145,15,0 1 000 8 02).

## Low power mode operation

The customer can configure the lowest power mode that may be entered by the AirPrime HL7800 module. The AirPrime HL7800 selects the best power mode according to the KSLEEP configuration and the modem state according to the below table.

| AT+KSLEEP | Possible Modem states | IOs State | Hardware Wake-up Signal source | Application Data |
| --- | --- | --- | --- | --- |
| SLEEP | OFF, DRX, eDRX, PSM, No service | Retained | UART_DTR WAKE-UP | Retained |
| LITE_HIBERNATE | OFF, eDRX, PSM, No service | Retained | UART_DTR WAKE-UP | Not retained |
| HIBERNATE | OFF, eDRX, PSM, No service | Not retained | WAKE-UP | Not retained |

AirPrime HL7800 Power Modes table

The following table describes the recommended mode that shall be used for the different 3GPP low power states:

| 3GPP Low power state | Recommended Low Power mode | Remark |
| --- | --- | --- |
| DRX | SLEEP |  |
| eDRX | HIBERNATE | LITE_HIBERNATE may be used for applications that need I/Os to be retained over eDRX cycle, but this mode has an impact on the power consumption (> 200µA). |
| PSM | HIBERNATE | With this power mode the module will reach a few µA power consumption during PSM dormant state. |

AirPrime HL7800 Power Mode recommendations

| Configured KSLEEP mode | Radio state | Possible power save modes | HW signals to enter power save mode (AND conditions) | HW Wakeup source (OR conditions) | Modem Wakeup source from (lite) Hibernate |
| --- | --- | --- | --- | --- | --- |
| Any | Connected | No power save mode | N/A | N/A | - |
| SLEEP | DRX, eDRX, Flight Mode, PSM, out of coverage | SLEEP | DTR: high level if mngt = 0 WAKEUP: low level | DTR: low level WAKEUP: high level | - |
| LITE HIBERNATE | DRX | SLEEP | DTR: high level if mngt = 0 WAKEUP: low level | DTR: low level WAKEUP: high level | - |
|  | Flight Mode | LITE HIBERNATE |  |  | N/A |
|  | Out of coverage | LITE HIBERNATE |  |  | Network recovery |
|  | eDRX | LITE HIBERNATE |  |  | SMS or IP reception Network lost |
|  | PSM | LITE HIBERNATE |  |  | T3412 (TAU timer) |
| HIBERNATE | DRX | SLEEP | DTR: high level if mngt = 0 WAKEUP: low level | WAKEUP: high level | - |
|  | Flight Mode | LITE HIBERNATE |  |  | N/A |
|  | Out of coverage | LITE HIBERNATE |  |  | Network recovery |
|  | eDRX (UICC activated) | LITE HIBERNATE |  |  | SMS or IP reception Network lost |
|  | eDRX (UICC deactivated) | HIBERNATE |  |  | SMS or IP reception Network lost |
|  | PSM | HIBERNATE |  |  | T3412 (TAU Timer) |

AirPrime HL7800 Power Mode summary table

Host interface

When the AirPrime HL7800 module enters in Hibernate mode, all IOs excepted wake-up and reset pins are not retained.

How to monitor Hibernate mode

As VGPIO is not powered during Hibernate mode, this IO can be used to monitor when the AirPrime HL7800 module enters in Hibernate mode. Any IOs state (except wake-up and reset pins) is not significant if VGPIO is low.

Host wake-up upon SMS or IP packet reception when AirPrime HL7800 module is in Hibernate mode

Host wake-up Pin

If the customer application goes in low power mode while the AirPrime HL7800 module is in Hibernate mode and requires to be waken up only on SMS reception or incoming IP packet reception, it is then recommended to not use UART_RI signal but GPIO2.

UART_RI signal is active low but it cannot maintain high during Hibernate mode. Therefore, this signal may then wake-up the Host each time the AirPrime HL7800 module transitions to Hibernate mode.

It is possible to use GPIO2 to wake-up the host on same events than UART_RI signal. GPIO2 is active high and is kept low during Hibernate or running modes.

To enable GPIO2, +KRIC command shall be used (please refer to Document [R-1] for detailed AT command description).

IP frame routing

As mentioned previously, when the module enters in (Lite) Hibernate mode the application context is not retained. However, the AirPrime HL7800 module proposes a new mode where a TCP socket in server mode or an UDP socket may be restored at cold or warm boot time. Thanks to this option, an incoming TCP connection request or UDP packet received during eDRX mode may be processed and notified to the host. If not configured or if the AirPrime HL7800 module receives another IP frame, the IP frame will wake-up the module, but it will be discarded.

To allow time for IP packet reception and process while Airprime HL7800 module exists from (Lite) Hibernate mode, it is recommended to configure KSLEEP with <delay> parameter set to 10 seconds. This delay parameter prevents the module to go back in (Lite) Hibernate immediately and then to let time to the host processor to wake up and to raise the Wake-up pin or DTR.

Low Power mode use cases

## PSM use cases

The recommended Low Power state while the AirPrime HL7800 module is in PSM mode is the HIBERNATE mode. When configured in PSM mode, the module enters the lowest power consumption mode once the Active Timer expires and wakes up periodically (based on the extended TAU timer) to perform a Tracking Area Update.

In this state, the IOs are not retained and AirPrime HL7800 module can be wake-up only from WAKE_UP pin.

#### AT command configuration

| Description | Command |
| --- | --- |
| PSM configuration: - T3412 = 24 hours - T3324 = 20 seconds | AT+CPSMS=1,,,””,”” OK |
| SLEEP Mode configuration - HIBERNATE Mode - DTR Managed | AT+KSLEEP=0,2 OK |
| RING indicator configuration: - Active on SMS reception - Pulse = 1 second | AT+KRIC=1 OK |

#### Sequence diagram – PSM - Attach + Tracking Area Update Procedure

PSM mode – basic use case

#### Data transmission initiated by the Host

While AirPrime HL7800 module is in PSM mode and HIBERNATE low power mode, host may want to send data over the network. Before sending data, host shall first wake-up the module thanks to WAKE-UP signal. Once the UART_CTS signal is Active, host shall reopen TCP/UDP socket or reinitiate PPP connection over UART. As PDN/PDP context is saved in PSM mode, these procedures are speed up.

## eDRX use cases

To achieve the best power consumption saving while the AirPrime HL7800 module is in eDRX mode, the recommended Low Power state is the HIBERNATE mode. When configured in eDRX mode, the module enters the lowest power consumption mode during eDRX sleep period. In particular, this mode depends on the UICC configuration as explained in previous paragraph:

- If the UICC doesn’t allow UICC deactivation in eDRX mode, the AirPrime HL7800 enters in LITE HIBERNATE mode during eDRX sleep period.

- If the UICC allows UICC deactivation in eDRX mode, the AirPrime HL7800 enters in HIBERNATE mode during eDRX sleep period.

AirPrime HL7800 module wakes-up periodically to listen to the paging as described in the first chapter. This wake-up is a partial wake-up meaning that only the modem subsystem of the module will be awake..

eDRX Calibration procedure

During the few first minutes after having enter in (Lite) Hibernate mode, the AirPrime module wakes up with an incremental period manner to calibrate internal clock and keep synchronize to the network. This calibration starts with a wake-up every 5 seconds, then 10, 20, 40 seconds until reaching the min between eDRX period and 81,92 seconds. This calibration procedure is relaunched after each RRC connection (TAU or Data) or if network synchronization is lost.

For an 81,92s eDRX period or greater, the calibration procedure can take about 10 minutes. The duration of each calibration wake-up is equivalent to the duration of one paging listening.

eDRX mode – Calibration procedure

#### eDRX/Hibernate IP Push use case - AT command configuration

| Description | Command |
| --- | --- |
| eDRX configuration: - eDRX cycle=81,82 seconds | AT+CEDRXS=1,4,5 OK |
| SLEEP Mode configuration - HIBERNATE Mode - DTR Managed - 10 seconds delay | AT+KSLEEP=0,2,10 OK |
| RING indicator configuration: - Active on incoming TCP connection - Pulse = 1 second - GPIO2 | AT+KRIC=32,0,1,2 OK |
| TCP socket configuration: - TCP Server listening | AT+KTCPCFG=1,1,,80,,,,,,1 AT+KTCPCNX=1 |
| Hardware Flow Control: - Active | AT&K3 |

#### eDRX/Hibernate IP Push use case – Flow chart

#### eDRX/Hibernate IP Push use case – Sequence diagram