Firmware High Level Design

for

+KSLEEP AT command

of HL78xx

| Document #: |  |
| --- | --- |
| Legacy #: | 411xxxxxx |
| Revision: | R01.01 |
| Document Type: | Design Document |
| Customer Name: | N/A |
| Author: | OVC |
| Author Document Creation Site: | ILM |
| Document Location: |  |
| Date: | 2018/06/26 |

Document History

| Rev XX.YY | Date YYYY/MM/DD | Updates | Author |
| --- | --- | --- | --- |
| 01.00 | 2018/05/31 | Creation | OVC |
| 01.01 | 2018/06/26 | New Altair API management | OVC |

Template History

| Rev XX.YY | Date YYYY/MM/DD | Updates | Author |
| --- | --- | --- | --- |
| 01.03 | 2013/12/11 | Creation, first approved version | SEPG |
| 01.04 | 2015/12/22 | ACT00648: Addition of §2.3 to list the child requirements. The traceability to product requirement is done via the history revision table. Styles cleaned, remove useless Content Control, add Template History table. | SEPG |

- Contents

1	Introduction	4

1.1	Purpose	4

1.2	Scope	4

1.3	References	4

1.4	Glossary of terms	4

2	System Overview	4

2.1	Environment	4

2.2	Platform	4

2.3	Requirements	4

2.4	Altair Reference Design	5

3	Low power mode management with AT+KSLEEP command.	5

3.1	AT+KSLEEP syntax	5

3.1.1	NVM	7

4	Altair's power management APIs	7

4.1	Sleep level	8

4.1.1	To get current maximum allow sleep level:	8

4.1.2	To set the maximum sleep level – effective immediately:	8

4.2	Sleep enabling/disabling	8

5	Implementation	9

# Introduction

## Purpose

This document contains the High-Level Design for the AT command of the HL78xx_1250 firmware for the low power mode feature.

## Scope

This document defines how Sierra Wireless will manage low power mode regarding Altair’s possibilities

## References

| Ref. # | Document title | Doc. # |
| --- | --- | --- |
| - [R-1] | AT command set for User Equipment | 3GPP TS 27.007 version 14.2.0 Release 14 |

## Glossary of terms

| Term | Definition |
| --- | --- |
| LTE | Long Term Evolution |
| PSM | Power Saving Mode |
| eDRX | Extended Discontinuous Reception |

# System Overview

## Environment

HL78xx is an Essential Airprime module based on the CF3 form-factor.

This module embeds the Altair 1250 chipset.

## Platform

HL78xx product.

## Requirements

HL78xx products must follow as much as possible sleep mode management of other HL series products configured by the +KSLEEP AT command; i.e. 3 modes:

- Sleep mode always forbidden.

- Sleep mode forbidden when DTR is active. When DTR is inactive, sleep mode is allowed. CTS goes inactive when DTR is inactive.

- Standalone sleep mode: module decides by itself when it can go in sleep mode from the UART point of view (nothing to send on UART, nothing received on RX for a while). Sleep mode state is indicated by the CTS going to inactive.

Module should be always using the lowest power mode possible.

## Altair Reference Design

Altair reference design is a module with almost all the modem functionalities implemented and with an application processor:

- Modem firmware is provided by Altair as a binary.

- Application processor firmware is a RTOS (FreeRTOS) based environment.

# Low power mode management with AT+KSLEEP command.

This command configures power saving modes dynamically and save in non-volatile memory the selected configuration.

## AT+KSLEEP syntax

AT+KSLEEP=<mngt>[,<level>] with:

<mngt>:

0: Low power modes permission are driven by a hardware signal (DTR or WAKEUP pin) – if the signal is active (low level for DTR, high level for WAKEUP pin), the module does not enter low power modes.

1: Standalone low power modes – the module decides by itself when it enters in low power modes.

2: Low power modes always disabled.

<level>:

0: SLEEP

1: LITE_HIBERNATE

2: HIBERNATE

See following table for level details.

This parameter is allowed only when <mngt>=0 or 1

Phase 1:

| AT+KSLEEP | Altair Mode to be configured | current floor | Conditions | IOs State | Wake-up source | Wake-up time | Application context |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SLEEP | Deep sleep | Deep Sleep 1,2 mA (floor) | DTR if <mgt> = 0 Modem in idle mode | Retained | DTR SMS/IP data reception Wakeup ? | few ms | retained |
| LITE_HIBERNATE | DH2 | DH2 200 µA (floor) | DTR if <Mgt>=0 Modem in eDRX mode or PSM mode/OFF or no coverage. | Retained | Wake-up PIN SMS reception IP data reception PSM timer Network search timer | ~2 seconds | not retained |
| HIBERNATE | DH0 | DH0 3 µA (floor) | DTR if <mgt> = 0 Modem in PSM mode/OFF or no coverage | Not Retained | Wake-up PIN PSM timer when PSM Network search timer when no coverage | ~2 seconds | not retained |
|  |  | DH1 60 µA (floor) | DTR if <Mgt>=0 Modem in eDRX mode UICC SIM card allows USIM deactivation | Not Retained | Wake-up PIN SMS reception IP data reception | ~2 seconds | not retained |
|  |  | DH2 200 µA (floor) | DTR if <Mgt>=0 Modem in eDRX mode UICC SIM card doesn't allow USIM deactivation | Retained | Wake-up PIN SMS reception IP data reception | ~2 seconds | not retained |

Phase 2:

| AT+KSLEEP | Altair Mode to be configured | current floor | Conditions | IOs State | Wake-up source | Wake-up time | Application context |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SLEEP | Deep sleep | Deep Sleep 1,2 mA (floor) | DTR if <mgt> = 0 Modem in idle mode | Retained | DTR SMS/IP data reception Wakeup ? | few ms | retained |
| LITE_HIBERNATE | DH2 | DH2 65 µA (floor) | DTR if <Mgt>=0 Modem in eDRX mode or PSM mode/OFF or no coverage. Modem in Idle mode | Retained | Wake-up PIN SMS reception IP data reception PSM timer Network search timer DTR | ~2 seconds | retained |
| HIBERNATE | DH0 | DH0 3 µA (floor) | DTR if <mgt> = 0 Modem in PSM mode/OFF or no coverage | Not Retained | Wake-up PIN PSM timer when PSM Network search timer when no coverage | ~2 seconds | not retained |
|  |  | DH1 60 µA (floor) | DTR if <Mgt>=0 Modem in eDRX mode UICC SIM card allows USIM deactivation | Not Retained | Wake-up PIN SMS reception IP data reception | ~2 seconds | not retained |
|  |  | DH2 200 µA (floor) | DTR if <Mgt>=0 Modem in eDRX mode UICC SIM card doesn't allow USIM deactivation | Retained | Wake-up PIN SMS reception IP data reception | ~2 seconds | not retained |

### NVM

Configuration values of +KSLEEP command must be saved in NVM. Last configured values must be set at init.

# Altair's power management APIs

Altair provides 5 types of power save modes:

|  | CPU clock | Retention RAM | 26MHz | RTC | I/O |
| --- | --- | --- | --- | --- | --- |
| DH0 | OFF | OFF | OFF | ON | Not maintained |
| DH1 | OFF | Yes (n*64KB) | OFF | ON | Not maintained |
| DH2 | OFF | Yes (n*64KB) | OFF | ON | Maintained |
| Lite Sleep | Halted | Self-refresh | ON | ON | Maintained |
| Deep Sleep | Halted | Self-refresh | OFF | ON | Maintained |

During the operation time of the modem, the software power manager is designed to choose the optimal sleep level that will minimize the power consumption while minimizing the impact on the system functionality.

The implementation for this is done by using multiple logical clients that are communicating with the sleep manager. Each client is allowing or disallowing sleep at specific time while also “voting” for the max sleep level that is suitable for the client at that specific time. When a client allows a “max sleep level”, it means that as far as that specific client concerns, it allows the system to go to the “voted” sleep level or any other sleep level that are considers as lower sleep level. The sleep manager will take all the votes from all clients and will take the minimum level that was reported.

The sleep levels are determined in the order of (from Maximum to Minimum):
DH0->DH1->DH2->DS->LS

## Sleep level

Altair provides an API to change the RTOS vote during runtime or in configuration file.
The API will set the maximum sleep level and will not allow for the sleep manager to any state bellow the voted state

### To get current maximum allow sleep level:

ePowerSaveMode pm_runtime_get_max_sleep_mode (void)

### To set the maximum sleep level – effective immediately:

int pm_runtime_set_max_sleep_mode(ePowerSaveMode max_pm_mode)

## Sleep enabling/disabling

The smngr is the voting api that can be used from RTOS applications to enable or disable sleep (the level is set globally by using the api from the previous section).

Different Drivers and tasks can register (become smngr clients) and effect the smngr (thus allowing or preventing the system from going to sleep).

We differentiated between 2 types of smngr clients:

The driver/task knows when it wants to prevent and when it allows sleep (we call them sleep manager synchronous dev)

A driver/task can tell only before going to sleep, if it allows or prevents the system from going to sleep (using a callback which it need to implement - we call them sleep manager asynchronous dev)

The user should decide whether to work synchronously (know when he want to prevent sleep and when he allows) or asynchronously (only when we go to sleep)

Subscribe to the SM by either:

int smngr_register_dev_sync(char *dev_name, u32_t *id);

or:

int smngr_register_dev_async(char *dev_name, void *callback, void  *hw, u32_t *id);

For sync device use:

int smngr_dev_busy_set(u32_t id);

int smngr_dev_busy_clr(u32_t id);

To set and clear when you are busy.

# Implementation

At initialization:

swiserver must register as sleep manager client using smngr_register_dev_sync API

sleep mode should be enabled using pm_runtime_set_sleep_mode API

Then depending on <mngt> and <level> parameters and DTR signal state, swiserver should allow or prevent system to go to power save mode (using smngr_dev_busy_set/smngr_dev_busy_clr API) and set maximum power save level (using pm_runtime_get_max_sleep_mode API)

<level> parameter of +KSLEEP, will define the max power saving mode requested to Altair:

| level | Max power saving mode |
| --- | --- |
| 0 | Deep sleep |
| 1 | DH2 |
| 2 | DH0 |

<mngt> parameter plus DTR signal state will define if swiserver should allow/prevent system going to sleep:

| mngt | DTR_State | Going to sleep |
| --- | --- | --- |
| 0 | 0 | Prevent |
| 0 | 1 | Allowed |
| 1 | 0/1 | Allowed |
| 2 | 0/1 | Prevent |