Version

This document may be updated over its lifetime. To ensure you design with the correct version, please check The Source for latest versions.

# Introduction

This document is provided to Sierra Wireless distributors and clients to aid more rapid development of embedded applications using the Sierra Wireless portfolio of cellular solutions. To request a new application note, contact your regional Sierra Wireless Product Marketing Manager.

# Overview

The HL7800-M is designed as a global product, which entails the following at the end of the production line:

No RF band customization – this means that a predefined set of bands is enabled.

No operator-specific requirements are activated; default behavior is activated.

Depending on the final product, it may be necessary to perform the following at the production site:

Configure or customize the RF bands the final product will be operating on

Set the mobile network operator the final product will be operating on (so that operator requirements are taken into account)

In order not to degrade the user experience and avoid sensible impact on power consumption, the RF bands should be carefully configured.

This application note describes how to perform these customization and explains the impact of these configurations. It also details the differences between each operator configuration.

# Glossary

| Term/Initials | Definition |
| --- | --- |
| CIoT | Cellular Internet Of Things |
| CP | Control Plane |
| eDRX | Extended Discontinuous Recepton |
| FOTA | Firmware Over-The-Air |
| MNO | Mobile Network Operator |
| NV | Non Volatile |
| PSM | Power Saving Mode |
| RF | Radio Frequency |
| SKU | Stock Keeping Unit |
| UP | User Plane |
| CID | A numeric parameter which specifies a particular PDP context or EPS bearer context definition (cf. 3GPP 27.007) |

# Customizing RF Bands

The module is configured with some predefined bands enabled by default. RF band customization is done using command AT+KBNDCFG. Refer to document [1] AirPrime HL78xx AT Commands Interface Guide for details about this command.

## Preliminary note and warning

Full band scanning can be very long, depending on the numbers of bands being enabled. Full band scanning would decrease the user experience and increase the power consumption. Even if some mechanisms have been added into the software to avoid full band scanning, this full band scanning may however happen, e.g.:

- During the first time activation, when the device has no previous knowledge of the radio environment and the avaialble cells

- After long out-of-coverage periods

- In roaming scenarios

It is therefore highly recommended to strictly limit the RF bands being enabled to only the ones in use by the targeted network.

## Check Supported Bands

Send AT+KBNDCFG=? to check all bands supported by the module. The module will respond with +KBNDCFG: <RAT>,(list of supported <bnd bitmap>s). The response is given for all supported radio access technologies, with supported bands per <RAT> as a bitmap. Refer to section 5.7 Bitmap Decoding Examples for more details regarding band bitmaps.

Example:

AT+KBNDCFG=?

+KBNDCFG: 0,0000000000000A0E189F

+KBNDCFG: 1,0

+KBNDCFG: 2,0

OK

In the example given above:

Bands 1, 2, 3, 4, 5, 8, 12, 13, 18, 19, 20, 26 and 28 are supported for CAT-M1 (<RAT> = 0)

No supported bands for NB1 (<RAT> = 1)

No supported bands for GSM (<RAT> = 2)

Only CAT-M1 is supported by the HL7800-M. NB1 and GSM will be supported in future products.

## Check Enabled Bands

Send AT+KBNDCFG? to check all enabled bands. The module will respond with +KBNDCFG: <RAT>,(list of configured <bnd bitmap>s). The response is given for all enabled <RAT>s, with supported bands per radio access technology as a bitmap. Refer to section 5.7 Bitmap Decoding Examples for more details regarding band bitmaps.

Example:

AT+KBNDCFG?

+KBNDCFG: 0,00000000000000001004

+KBNDCFG: 1,0

+KBNDCFG: 2,0

OK

In the example given above:

Bands 3 and 13 are enabled for CAT-M1 (<RAT> = 0)

No enabled bands for NB1 (<RAT> = 1)

No enabled bands for GSM (<RAT> = 2)

Enabled bands are a subset of the supported bands for each RAT.

## Set RF Bands on which to Operate

It is necessary to define the RF bands on which the final product will operate. (Prior to defining the RF bands, check the list of bands supported by the module. Refer to section 5.2 Check Supported Bands for details.)

To define RF bands on which to operate, send AT+KBNDCFG=<RAT>,<bnd bitmap>.

Example:

If the final product is supposed to only work on CAT-M1 Band 3, send AT+KBNDCFG=0,00000000000000000004.

ERROR or +CME ERROR will be returned if the entered configuration is not available (e.g if the band bitmap includes a band which is not supported by the module):

AT+KBNDCFG=0,00000000000000000044

+CME ERROR: 3

If the entered configuration is available, OK will be returned along with the current configuration:

AT+KBNDCFG=0,00000000000000000004

+KBNDCFG: 0,00000000000000000004

OK

## Applicability of New Band Configuration

New band configuration set with AT+KBNDCFG=<RAT>,<bnd bitmap> is automatically saved in NV memory.

However,

the new band configuration will only be taken into account after reset.

## Status after Power Cycle and Software Upgrade

### Power Cycle

New band configuration set with AT+KBNDCFG=<RAT>,<bnd bitmap> is applied after the next reset. Configuration is retained after future power cycles.

### Software Upgrade

New band configuration set with AT+KBNDCFG=<RAT>,<bnd bitmap> is retained after software upgrade.

This is applicable for both local upgrade using AT+WDSD and FOTA using AirVantage.

## Bitmap Decoding Examples

Band bitmap displayed and used in AT+KBNDCFG are not easy to read or define. As explained in document [1] AirPrime HL78xx AT Commands Interface Guide, <bnd_bitmap> is the band bitmap in hexadecimal format, without a 0x prefix.

For CAT-M1, each bit within the bitmap represents one LTE band:

Bit 1 gives the status on LTE Band 1

Bit 2 gives the status on LTE Band 2

Bit n gives the status on LTE Band n

A “1” at bit position n indicates that LTE Band n is configured, and a “0” at bit position n indicates that LTE Band n is not configured. Refer to the following table for examples.

Band Bitmap Example

| Bitmap (Hexadecimal Format) | Bitmap (Binary Format) | Bands Configured |
| --- | --- | --- |
| 00000000000000000001 | 000…0000000000000000000000000001 | 1 |
| 00000000000000000004 | 000…0000000000000000000000000100 | 3 |
| 00000000000000001004 | 000…0000000000000001000000000100 | 3, 13 |
| 0000000000000A0E189F | 000…1010000011100001100010011111 | 1, 2, 3, 4, 5, 8, 12, 13, 18, 19, 20, 26, 28 |

## RF Bands configuration required by main operators

In order to ease device configuration related to RF bands, bands support requested by main operators is given in the table below.

| Operator | Cat-M1 Frequency bands availability (Oct. 2018)* | Cat-M1 Frequency bands to come in the future* |
| --- | --- | --- |
| KDDI | 18, 26 | N/A |
| Telstra | 3, 28 | N/A |
| Verizon | 13 | B4 |
| AT&T | 2, 4, 12 | N/A |

* Cat-M1 frequency bands availability on different networks is evolving. It is up to customer to check frequency bands to be enabled in its end-product depending also on international roaming

# Customizing the Operator

MNOs may have specific requirements either on modem behavior (timer value, number of retries, etc.) or on general product configuration (PDN configuration, features configuration, etc.)

In order to comply with these requirements, it is necessary to carefully set the operator on which the product will operate.

Operator customization is done using command AT+KCARRIERCFG. Refer to document [1] AirPrime HL78xx AT Commands Interface Guide for details about this command.

Even if operators are indicated as supported at AT commands level, an operator certification may be needed. Please refer to Customer Release Notes to check relevant certifications.

## Check Supported Operators

Send AT+KCARRIERCFG=? to get the list of supported operators. The module will respond with a range (or several ranges) of integers, which are indexes that correspond to available operators. Corresponding indexes and operator names are defined in document [1] AirPrime HL78xx AT Commands Interface Guide.

Example:

AT+KCARRIERCFG=?

+KCARRIERCFG: (0-13)

OK

The list of supported operators on software release 1.7.12 are as follows:

DEFAULT

VERIZON

CMCC

RJIL

KDDI

AT&T

USCC

DOCOMO

SOFTBANK

LGU+

KT

T-MOBILE

SKT

TELSTRA

This list may evolve in future releases.

## Check Current Operator

Send AT+KCARRIERCFG? to check the current operator. The module will respond with an integer value, which is an index that corresponds to the operator currently set on the module. Corresponding indexes and operator names are defined in document [1] AirPrime HL78xx AT Commands Interface Guide.

Example:

AT+KCARRIERCFG?

+KCARRIERCFG: 0

OK

In the example given above, operator DEFAULT is currently set on the device.

Operator is set to DEFAULT at the end of the Sierra Wireless production line.

## Set the Operator

### Operator Available in the List of Supported Operators

Send AT+KCARRIERCFG=<operator_idx> to set a specific operator. <operator_idx> is the index that matches the operator name as defined in document [1] AirPrime HL78xx AT Commands Interface Guide.

Example:

If the final product is intended to operate on the AT&T network, send AT+KCARRIERCFG=5 to set AT&T as the current operator.

ERROR or +CME ERROR will be returned if the specified index is not available. OK will be returned when the index is available.

### Operator Not Available in the List of Supported Operators

If the operator on which the product is intended to operate is not in the supported operators list, it means that this operator has no (known) specific requirements. Operator should be set to DEFAULT by sending AT+KCARRIERCFG=0.

## Applicability of New Operator Setting

New operator set with AT+KCARRIERCFG=<operator_idx> is automatically saved in NV memory.

However,

the new operator setting will only be taken into account after reset.

## Status after Power Cycle and Software Upgrade

### Power Cycle

The new operator set with AT+KCARRIERCFG=<operator_idx> is applied after the next reset. Configuration is retained after future power cycles.

### Software Upgrade

The new operator set with AT+KCARRIERCFG=<operator_idx> is retained after software upgrade.

This is applicable for both local upgrade using AT+WDSD and FOTA using AirVantage.

## Impact of Setting a Specific Operator

### RF Bands

Setting a specific operator does not modify the enabled RF bands on which the module will operate.

If the operator uses specific bands and the customer would like to restrict the RF bands on which the module will operate, then these RF bands have to be set using AT+KBNDCFG.

Make sure that the RF bands set using AT+KBNDCFG are compatible with the bands used by the operator.

### PDN Configuration

Some operators have requirements related to PDN configuration such as APN to be used, PDN type (IPv4, IPv6), default PDN to be used when attaching to the network, etc.

Setting an operator with AT+KCARRIERCFG will reset the module’s PDN configuration to the default PDN configuration to be used for the specified operator (refer to Table 2: Device Configuration per Operator). All PDN configurations made previously will be deleted and replaced by the operator’s default PDN configuration.

Modifications on PDN configurations pushed over the air, i.e. for custom APNs, will also be lost when setting an operator with AT+KCARRIERCFG.

If the customer wants to define specific PDN configuratons, these should be done after operator customization.

PDN configuration reset only happens if the new operator is different from the current operator.

Setting a new operator example:

AT+KCARRIERCFG?

+KCARRIERCFG: 0					// Current operator is DEFAULT

OK

AT+CGDCONT?						// Check current PDN configuration

+CGDCONT: 1,”IPV4V6”,,,0,0,0,0,0,,0,,,,,,,	// Current PDN configuration

OK

AT+CGDCONT=1,”IP”,”internet”			// Modification of PDN configuration

OK

AT+CGDCONT?

+CGDCONT: 1,”IP”,”internet”,,0,0,,0,,,,,,,,,,		// Current PDN configuration

OK

AT+KCARRIERCFG=6					// Setting new operator: USCC

OK

AT+CFUN=1,1					// Reset for the new configuration to be taken into account

OK

AT+KCARRIERCFG?					// Check current operator

+KCARRIERCFG: 6					// Current operator is USCC

OK

AT+CGDCONT?						// Check current PDN configuration

+CGDCONT: 1,”IPV4V6”,,,0,0,0,0,0,,0,,,,,,,	// PDN configuration has been reset to USCC default 								// PDN configuration

OK

Setting the same operator example:

AT+KCARRIERCFG?

+KCARRIERCFG: 0					// Current operator is DEFAULT

OK

AT+CGDCONT?

+CGDCONT: 1,”IPV4V6”,,,0,0,0,0,0,,0,,,,,,,	// Current PDN configuration

OK

AT+CGDCONT=1,”IP”,”internet”			// Modification of current PDN configuration

OK

AT+CGDCONT?

+CGDCONT: 1,”IP”,”internet”,,0,0,,0,,,,,,,,,,	// Current PDN configuration

OK

AT+KCARRIERCFG=0			// Set same operator DEFAULT

OK

AT+CFUN=1,1				// Reset for the new configuration to be taken into account

OK

AT+KCARRIERCFG?			// Check current operator

+KCARRIERCFG: 0			// Current operator is DEFAULT

OK

AT+CGDCONT?

+CGDCONT: 1,”IP”,”internet”,,0,0,0,0,0,,0,,,,,,,	// PDN configuration has NOT been reset to 									// DEFAULT default PDN configuration

OK

### PSM Configuration

Some operators have requirements related to default PSM configuration such as PSM enabling, T3324 or T3412 values.

Setting an operator with AT+KCARRIERCFG will reset the module’s PSM configuration to the default PSM configuration to be used for the specified operator. All PSM configurations made previously will be deleted and replaced by the operator’s default PSM configuration.

If the customer wants to define specific PSM timers (or to enable/disable PSM), this should be done after operator customization.

PSM configuration reset only happens if the new operator is different from the current operator.

Setting a new operator example:

AT+KCARRIERCFG?

+KCARRIERCFG: 0					// Current operator is DEFAULT

OK

AT+CPSMS?

+CPSMS: 0,”11100000”,”11100000”,”44”,”10”	// Current PSM configuration

OK

AT+CPSMS=1,”11100000”,”11100000”,”00111000”,”00100001”	// Modification of current PSM 										// configuration

OK

AT+CPSMS?

+CPSMS: 1,”11100000”,”11100000”,”00111000”,”00100001”		// Current PSM configuration

OK

AT+KCARRIERCFG=6				// Set new operator USCC

OK

AT+CFUN=1,1					// Reset for the new configuration to be taken into account

OK

AT+KCARRIERCFG?

+KCARRIERCFG: 6				// Current operator is USCC

OK

AT+CPSMS?						// Check current PSM configuration

+CPSMS: 0,”11100000”,”11100000”,”44”,”10”	// PSM configuration has been reset

OK

Setting the same operator example:

AT+KCARRIERCFG?

+KCARRIERCFG: 0					// Current operator is DEFAULT

OK

AT+CPSMS?

+CPSMS: 0,”11100000”,”11100000”,”44”,”10”	// Current PSM configuration

OK

AT+CPSMS=1,”11100000”,”11100000”,”00111000”,”00100001”	// Modification of current PSM 										// configuration

OK

AT+CPSMS?

+CPSMS: 1,”11100000”,”11100000”,”00111000”,”00100001”		// Current PSM configuration

OK

AT+KCARRIERCFG=0				// Set same operator DEFAULT

OK

AT+CFUN=1,1					// Reset for the new configuration to be taken into account

OK

AT+KCARRIERCFG?

+KCARRIERCFG: 0				// Current operator is DEFAULT

OK

AT+CPSMS?

+CPSMS: 1,”11100000”,”11100000”,”00111000”,”00100001”		// PSM configuration has NOT been 									// reset

OK

### eDRX Configuration

Some operators have requirements related to default eDRX configuration such as eDRX enabling or requested eDRX value.

Setting an operator with AT+KCARRIERCFG will reset the module’s eDRX configuration to the default eDRX configuration to be used for the specified operator. All eDRX configurations made previously will be deleted and replaced by the operator’s default eDRX configuration.

If the customer wants to define specific eDRX requested value (or to enable/disable eDRX), this should be done after operator customization.

eDRX configuration reset only happens if the new operator is different from the current operator.

# Operator Configuration

Setting a specific operator impacts both device configuration and modem behavior. It is a mandatory step in order for the module to be correctly configured and to work properly on the operator network.

The differences between each operator configuration are listed in the following sub-sections. Status specified below is given for software release 1.7.12. These will be updated for future software releases if needed.

## Application Level (General Configuration)

Setting an operator impacts the device configuration on the application level.

Even if more than 10 operator values are available, only 5 possible configurations exists on the application level: DEFAULT, VZW, ATT, SOFTBANK and TELSTRA.

Setting the operator to other options (CMCC, RJIL, KDDI, USCC, Docomo, LGU+, KT, T-Mobile or SKT) leads to the same configuration on the application level as setting the operator to DEFAULT.

The configurations described in Table 2: are applicable to software release 1.7.12.

Device Configuration per Operator

| Parameter |  | DEFAULT (and all other operator options) |  | VZW |  | ATT |  | SOFTBANK |  | TELSTRA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AT+KCARRIERCFG ID |  | 0 (2, 3, 4, 6, 7, 9, 10, 11, 12) |  | 1 |  | 5 |  | 8 |  | 13 |
| RF bands enabled |  | No link with operator configuration |  | No link with operator configuration |  | No link with operator configuration |  | No link with operator configuration |  | No link with operator configuration |
| Predefined CIDs |  | 1 |  | 1, 2, 3, 4, 6, 7 |  | 1, 2 |  | 1 |  | 1 |
| Available CIDs for host use1 |  | 1 |  | 3, 4, 6, 7 |  | 1 |  | 1 |  | 1 |
| Reserved CIDs for internal use (cannot be used by the host)2 |  | N/A |  | 1, 2 |  | 2 |  | N/A |  | N/A |
| Default PDN configuration3 |  | AT+CGDCONT? +CGDCONT: 1,"IPV4V6",,,0,0,0,0,0,,0,,,,, OK |  | AT+CGDCONT? +CGDCONT: 1,"IPV4V6",,,0,0,,0,,,,,,,, +CGDCONT: 2,"IPV4V6", "VZWADMIN",,0,0,0,0,0,,0,,,,, +CGDCONT: 3,"IPV4V6",,,0,0,0,0,0,,0,,,,, +CGDCONT: 4,"IPV4V6", "VZWAPP",,0,0,0,0,0,,0,,,,, +CGDCONT: 6,"IPV4V6", "VZWCLASS6",,0,0,0,0,0,,0,,,,, +CGDCONT: 7,"IPV4V6", "VZWIOTTS",,0,0,0,0,0,,0,,,,, OK |  | AT+CGDCONT? +CGDCONT: 1,"IPV4V6", "broadband",,0,0,0,0,0,,0,,,,, +CGDCONT: 2,"IPV4V6", "attm2mglobal",,0,0,0,0,0,,0,,,,, OK |  | AT+CGDCONT? +CGDCONT: 1,"IPV4V6",,,0,0,0,0,0,,0,,,,, OK |  | AT+CGDCONT? +CGDCONT: 1,"IPV4V6","telstra.iot",,0,0,0,0,0,,0,,,,, OK |
| Default PDN4 |  | 1 |  | 3 |  | 1 |  | 1 |  | 1 |
| LWM2M |  | Disabled |  | Enabled |  | Enabled |  | Enabled |  | Disabled |
| LWM2M configuration5 |  | No configuration |  | Verizon bootstrap server coaps://boot.lwm2m.vzwdm.com:5684 |  | ATT bootstrap server coaps://lwprod.iot.attcompute.com:5684 |  | Softbank bootstrap server coaps://lab2-device.iotpf.mb.softbank.jp:5884 |  | No configuration |
| PSM Enabling |  | Disabled |  | Disabled |  | Disabled |  | Disabled |  | Disabled |
| PSM timers values (T3412/T3324) |  | N/A |  | N/A |  | N/A |  | N/A |  | 24h / 1min |
| eDRX Enabling | Disabled | Disabled | Disabled | Disabled | Disabled |
| eDRX requested value | 81,92s | 81,92s | 81,92s | 81,92s | 20,48s |
| MTU IPv4 | 1500 bytes | 1428 bytes | 1500 bytes | 1500 bytes | 1358 bytes |
| MTU IPv6 | 1500 bytes | 1500 bytes | 1500 bytes | 1500 bytes | 1358 bytes |
| CIoT attach without PDN |  | Not supported |  | Not supported |  | Not supported |  | Not supported |  | Not supported |
| CIoT CP EPS optimizations |  | Not supported |  | Not supported |  | Not supported |  | Not supported |  | Not supported |
| CIoT UP EPS optimizations |  | Not supported |  | Not supported |  | Not supported |  | Not supported |  | Not supported |
| CIoT preferred EPS optimizations |  | Not supported |  | Not supported |  | Not supported |  | Not supported |  | Not supported |

1	CIDs that can be used by the host in commands using <cid> (AT+CGDCONT, AT+CGACT, etc.) Other CIDs cannot be used by the host.

2	Using these values in commands using <cid> will lead to ERROR.

3	PDN configuration when operator has been set and after the device is reset.

4	CID that will be used during the ATTACH procedure.

5	LWM2M configuration used if LWM2M is enabled.

## Modem Level

Likewise, setting an operator impacts the modem configuration and behavior by configuring the modem as per operator requirements.

As both the number of available operators and the specific modem behavior/configuration that can be impacted are high, it is not possible to list all modem behavior/configuration combinations.

# Modem behavior/configuration are tested against operator requirements during operator certification. If needed, specific behavior/configuration can be provided upon request.

# Reference Documents

AirPrime HL78xx AT Commands Interface Guide

Reference number: 41111821

# Document History

| Level | Date | History |
| --- | --- | --- |
| 1.0 | August 14, 2018 | Creation |
| 1.1 | September 06, 2018 | Updated: - 5.1 Check Supported Bands - 5.3 Set RF Bands on which to Operate - 5.4 Applicability of New Band Configuration - 5.5.2 Software Upgrade - 6.5.2 Software Upgrade - 6.6.1 RF Bands |
| 1.2 | October 9, 2018 | Added: - 5.1 Preliminary note and warning - 5.8 RF Bands configuration required by main operators Updated: - 6.1 Check Supported Operators - 7.1 Application Level (General Configuration) |

# Legal Notice

Important Notice

Due to the nature of wireless communications, transmission and reception of data can never be guaranteed. Data may be delayed, corrupted (i.e., have errors) or be totally lost. Although significant delays or losses of data are rare when wireless devices such as the Sierra Wireless modem are used in a normal manner with a well-constructed network, the Sierra Wireless modem should not be used in situations where failure to transmit or receive data could result in damage of any kind to the user or any other party, including but not limited to personal injury, death, or loss of property. Sierra Wireless accepts no responsibility for damages of any kind resulting from delays or errors in data transmitted or received using the Sierra Wireless modem, or for failure of the Sierra Wireless modem to transmit or receive such data.

Safety and Hazards

Do not operate the Sierra Wireless modem in areas where cellular modems are not advised without proper device certifications. These areas include environments where cellular radio can interfere such as explosive atmospheres, medical equipment, or any other equipment which may be susceptible to any form of radio interference. The Sierra Wireless modem can transmit signals that could interfere with this equipment. Do not operate the Sierra Wireless modem in any aircraft, whether the aircraft is on the ground or in flight. In aircraft, the Sierra Wireless modem MUST BE POWERED OFF. When operating, the Sierra Wireless modem can transmit signals that could interfere with various onboard systems.

Note:  Some airlines may permit the use of cellular phones while the aircraft is on the ground and the door is open. Sierra Wireless modems may be used at this time.

The driver or operator of any vehicle should not operate the Sierra Wireless modem while in control of a vehicle. Doing so will detract from the driver or operator’s control and operation of that vehicle. In some states and provinces, operating such communications devices while in control of a vehicle is an offence.

Limitations of Liability

This manual is provided “as is”.  Sierra Wireless makes no warranties of any kind, either expressed or implied, including any implied warranties of merchantability, fitness for a particular purpose, or noninfringement.  The recipient of the manual shall endorse all risks arising from its use.

The information in this manual is subject to change without notice and does not represent a commitment on the part of Sierra Wireless. SIERRA WIRELESS AND ITS AFFILIATES SPECIFICALLY DISCLAIM LIABILITY FOR ANY AND ALL DIRECT, INDIRECT, SPECIAL, GENERAL, INCIDENTAL, CONSEQUENTIAL, PUNITIVE OR EXEMPLARY DAMAGES INCLUDING, BUT NOT LIMITED TO, LOSS OF PROFITS OR REVENUE OR ANTICIPATED PROFITS OR REVENUE ARISING OUT OF THE USE OR INABILITY TO USE ANY SIERRA WIRELESS PRODUCT, EVEN IF SIERRA WIRELESS AND/OR ITS AFFILIATES HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES OR THEY ARE FORESEEABLE OR FOR CLAIMS BY ANY THIRD PARTY.

Notwithstanding the foregoing, in no event shall Sierra Wireless and/or its affiliates aggregate liability arising under or in connection with the Sierra Wireless product, regardless of the number of events, occurrences, or claims giving rise to liability, be in excess of the price paid by the purchaser for the Sierra Wireless product.

Patents

This product may contain technology developed by or for Sierra Wireless Inc.

This product includes technology licensed from QUALCOMM®.

This product is manufactured or sold by Sierra Wireless Inc. or its affiliates under one or more patents licensed from MMP Portfolio Licensing.

Copyright

© 2018 Sierra Wireless. All rights reserved.

Trademarks

Sierra Wireless®, AirPrime®, AirLink®, AirVantage®, WISMO®, ALEOS® and the Sierra Wireless and Open AT logos are registered trademarks of Sierra Wireless, Inc. or one of its subsidiaries.

Watcher® is a registered trademark of NETGEAR, Inc., used under license.

Windows® and Windows Vista® are registered trademarks of Microsoft Corporation.

Macintosh® and Mac OS X® are registered trademarks of Apple Inc., registered in the U.S. and other countries.

QUALCOMM® is a registered trademark of QUALCOMM Incorporated.  Used under license.

Other trademarks are the property of their respective owners.