Interface Specification

For 3rd party FOTA AT Commands

| Document #: | n/a |
| --- | --- |
| Legacy #: | n/a |
| Revision: | 1.0 |
| Document Type: | Protocol/Interface Specification |
| Customer Name : | CalAmp |
| Author: | Legato Team |
| Location: | ILM |
| Date: | November 08th, 2018 |

# Table of Contents

Table of Contents	2

1	General	2

1.1	Scope	2

1.2	References	2

1.3	Glossary of terms	3

2	AT Command Documentation Format	3

2.1	Description	3

2.2	Syntax and format guidelines	4

2.3	Device Services Third Party FOTA +TPF	4

# General

## Scope

This document specifies the Third Party FOTA (TPF) related AT Commands to be supported across Sierra Wireless.

A key aim of this document is to ensure consistent syntax and operation for TPF commands across the SWI device portfolio providing customers with a uniform user experience.

This is an internal document intended for Sierra Wireless staff.

## References

| Ref. # | Doc. # |  | Document title |  |
| --- | --- | --- | --- | --- |
| [1] | 4116802 | Interface Specification for AVMS AT Commands |

## Glossary of terms

| Term | Description |
| --- | --- |
| TPF | Third Party FOTA |
| FOTA | Firmware Over The Air |
| HTTP | Hypertext Transfer Protocol |
| HTTPS | Hypertext Transfer Protocol Secure |

# AT Command Documentation Format

This section provides an overview of how the document is architected and provides a template for adding new commands.

## Device Services Third Party FOTA +WDSTPF

Description

This Sierra Wireless proprietary command allows user to manage a FOTA from a third-party server. It can be used to:

- Set the package address (URL).

- Initiate a package download from the specified server.

Syntax

- For <mode>= 0

This mode is used to set the address of the package which will be downloaded.

Action command AT+WDSTPF=<mode>,<addr> OK

- For <mode>=1

This mode is used to start a download from the server.

Action command AT+WDSTPF=<mode> OK

Read command AT+WDSTPF? +WDSTPF: 0,<addr> +WDSTPF: 1,<state> OK

Test command AT+WDSTPF=? +WDSTPF: 0,(maximum length of <addr>) +WDSTPF: 1,(list of supported <state>s) OK

Parameters and Defined Values

| <mode>: |  | Mode of operation |  |
| --- | --- | --- | --- |
|  | 0 | Set the package URL. This address is stored in memory, and it is persistent to reset. |  |
|  |  |  | 1 |  |  | Start the FOTA operation. When this mode is activated, +WDSIthe download starts depending on the user agreements configuration (see the +WDSC configuration in [1]). |  |  |  |  |  |
| <addr>: |  |  |  |  |  | Package address String parameter which format is "<url>[:<port>]" Maximum length: 255 |  |  |  |  |  |
| <url>: |  |  |  |  |  | Package URL String parameter |  |  |  |  |  |
| <port>: |  |  |  |  |  | Port String parameter. The port number is optional, the default setting is 80. Maximum length: 5 |  |  |  |  |  |
| <state>: |  |  |  |  |  | FOTA operation status |  |  |  |  |  |
|  |  |  | 0 |  |  | Not started (default value) |  |  |  |  |  |
|  |  |  | 1 |  |  | Started |  |  |  |  |  |

Notes

The user agreements for download and install are applicable for the third-party FOTA service. These user agreements are controlled by the Device Services Configuration AT command (+WDSC) and Device Services Reply AT command (+WDSR), which are described in document [1].

The Device Services Indications AT command (+WDSI) is available under third-party FOTA service. The sent indications notify the different states of FOTA. The +WDSI command is described in document [1].

Examples

| Command |  | Responses |  |
| --- | --- | --- | --- |
| AT+WDSTPF=? |  | +WDSTPF: 0,(1-255) +WDSTPF: 1,(0-1) OK |  |
| AT+WDSTPF? |  | +WDSTPF: 0,"http://abcd.net:80/1234" +WDSTPF: 1,0 OK |  |
| AT+WDSC? |  | +WDSC: 0,1 +WDSC: 1,0 +WDSC: 2,1 +WDSC: 3,0 +WDSC: 4,15,60,240,480,1440,2880,0,0 +WDSC: 5,0 +WDSC: 6,0 |  |
| AT+WDSTPF=1 Note: Set start download |  | OK +WDSI: 9,<package Size> +WDSI: 18,1 +WDSI: 18,5 +WDSI: 18,70 +WDSI: 18,100 +WDSI: 12 +WDSI: 10 +WDSI: 14 +WDSI: 16 |  |

## The device has two services to perform a FOTA :

- 1st one :  allows the device to start session and communicate with Sierra Cloud Server, then launch a FOTA.

- 2nd one : Third party cloud server FOTA, this service allows the device to start a package download with any cloud server over HTTP  protocol and launch FOTA.

The FOTA from Sierra server must not be used simultaneously with this third party FOTA update: the cross effects are not guarantees.